3GPP TSG-RAN5 Meeting #100	R5-234039

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2818 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RRC_IDLE procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu EMEA Ltd, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In case of pc_noOf_PDNsSameConnection = 1 AND pc_noOf_PDNsNewConnection =1, connected without release is not maintained at step 1b1 in Table 4.5.2.2-4 If pc_noOf_PDNsNewConnection =1, then, RRCRelease is transmitted at step 20a1 in Table 4.5.2.2-2 even though connected without release is present. The reason for RRCRelease transmission is to allow optional new PDU establishment in Table 4.5.2.2-4. Since pc_noOf_PDNsSameConnection =1, AT command for new PDU will not be triggered at step 1a1 in Table 4.5.2.2-4 if UE does not establish new PDU autonomously. This leads to step 1b1 in Table 4.5.2.2-4 and UE stays in RRC IDLE state even though RRC CONNECTED is required at the end procedure (ie connected without release is present). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added steps 1a1 -1b8 to bring back UE into RRC CONNECTED even in case of connected without release is present AND pc_noOf_PDNsSameConnection = 1 AND pc_noOf_PDNsNewConnection =1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Affected test cases: 8.1.6.1.3.7(RRC_CONNECTED is not maintained after preamble) 9.1.6.1.1(RRC_CONNECTED is required at Step 7) And few tests requiring RRC_COINNECTED after RRC_IDLE extn procedure when UE confgured with pc_noOf_PDNsSameConnection = 1 AND pc_noOf_PDNsNewConnection =1. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

### 4.5.2	RRC_IDLE

#### 4.5.2.1	Initiation

The SS shall:

1>	if connectivity is EN-DC:

2>	use 1 E-UTRA cell and 1 NR cell, default parameters;

2>	if connected without release is not present:

3>	perform according to the table 4.5.2.2-1: E-UTRA RRC_IDLE;

1>	if connectivity is E-UTRA/EPC:

2>	use 1 E-UTRA cell, default parameters;

2>	if unrestricted nr PDN is not present:

3>	perform according to the table 4.5.2.2-1: E-UTRA RRC_IDLE;

2>	else:

3>	perform according to the table 4.5.2.2-5: E-UTRA RRC_IDLE Unrestricted nr PDN:

3>	if pc_noOf_PDNsNewConnection > 0:

4>	perform according to the table 4.5.2.2-6: E-UTRA RRC_IDLE Unrestricted nr PDN Extension;

1>	if connectivity is NR:

2>	use 1 NR cell, default parameters;

2> if sidelink is On

3>	use 1 NR-SS-UE;

3> if GNSS Sync is On

4> use 1 GNSS simulator.

2>	perform according to the table 4.5.2.2-2: NR RRC_IDLE;

2> if pc_noOf_PDUsNewConnection > 0

3> perform according to the table 4.5.2.2-4: NR RRC_IDLE Extension;

1>	if connectivity is WLAN:

2>	use 1 WLAN cell, default parameters;

2>	if connected without release is not present:

3>	perform according to the table 4.5.2.2-3: WLAN Ipsec_SA_Released;

2>	else:

3>	Not defined:

1>	if connectivity is NR-DC:

2>	use 2 NR cells, default parameters;

2>	perform according to the table 4.5.2.2-2: NR RRC_IDLE.

1>	if connectivity is NE-DC:

2>	use 1 E-UTRA cell and 1 NR cell, default parameters;

2>	perform according to the table 4.5.2.2-2: NR RRC_IDLE.

#### 4.5.2.2	Procedures

Table 4.5.2.2-1: E-UTRA RRC_IDLE

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-9a2 | Same as TS 36.508 [2] table 4.5.2.3-1, steps 1-9a2. | - | - | - | - |
| - | EXCEPTION: Steps 10a1 to 10b8 describe behaviour which depends on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 10a1-10a10 | IF Test Mode = On OR Test Loop Function = On THEN steps 10-19 as defined in TS 36.508 [2] table 4.5.2A.3-1, are performed. The ACTIVATE TEST MODE is using the associated condition for the test loop. | - | - | - | - |
| 10b1-10b8 | ELSE steps 10-17 as defined in TS 36.508 [2], table 4.5.2.3-1 are performed. | - | - | - | - |

Table 4.5.2.2-2: NR RRC_IDLE

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | - | <-- | NR RRC: SYSTEM INFORMATION (BCCH) | - | - |
| 2 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 3 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 4 | The UE transmits an RRCSetupComplete message and a REGISTRATION REQUEST message. | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | - |
| 5 | The SS transmits a DLInformationTransfer message and an AUTHENTICATION REQUEST message. | <-- | NR RRC: DLInformationTransfer 5GMM: AUTHENTICATION REQUEST | - | - |
| 6 | The UE transmits an ULInformationTransfer message and an AUTHENTICATION RESPONSE message. | --> | NR RRC: ULInformationTransfer 5GMM: AUTHENTICATION RESPONSE | - | - |
| 7 | Void | - | - | - | - |
| 8 | The SS transmits a DLInformationTransfer message and a SECURITY MODE COMMAND message. | <-- | NR RRC: DLInformationTransfer 5GMM: SECURITY MODE COMMAND | - | - |
| 9 | The UE transmits an ULInformationTransfer message and a SECURITY MODE COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: SECURITY MODE COMPLETE | - | - |
| - | EXCEPTION: Step 9Aa1 to 9Aa2 describes behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported and the procedure parameter Interworking without N26 interface supported is not present. | - | - | - | - |
| 9Aa1 | IF UE_S1_SUPPORTED the SS transmits a DLInformationTransfer message and a SECURITY MODE COMMAND message. | <-- | NR RRC: DLInformationTransfer 5GMM: SECURITY MODE COMMAND | - | - |
| 9Aa2 | The UE transmits an ULInformationTransfer message and a SECURITY MODE COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: SECURITY MODE COMPLETE | - | - |
| - | EXCEPTION: Steps 9a1 to 9a2 describe the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value | - | - | - | - |
| 9a1 | IF Test Mode = On OR Test Loop Function = On, the SS transmits an ACTIVATE TEST MODE message to activate UE radio bearer test mode procedure. The ACTIVATE TEST MODE message is using the associated condition for the test loop. | <-- | RRC: DLInformationTransfer TC: ACTIVATE TEST MODE | - | - |
| 9a2 | The UE transmits an ACTIVATE TEST MODE COMPLETE message. | --> | RRC: ULInformationTransfer TC: ACTIVATE TEST MODE COMPLETE | - | - |
| 10 | The SS transmits a SecurityModeCommand message. | <-- | NR RRC: SecurityModeCommand | - | - |
| 11 | The UE transmits a SecurityModeComplete message. | --> | NR RRC: SecurityModeComplete | - | - |
| 12 | The SS transmits a UECapabilityEnquiry message. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| 13 | The UE transmits a UECapabilityInformation message. | --> | NR RRC: UECapabilityInformation | - | - |
| 14 | The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message. | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION ACCEPT | - | - |
| 15 | The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: REGISTRATION COMPLETE | - | - |
| 16 | Void | - | - | - | - |
| 17 | Void | - | - | - | - |
| 18 | Void | - | - | - | - |
| - | EXCEPTION: Step 19a1 describes behaviour depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - | - |
| 19a1 | IF pc_noOf_PDUsSameConnection > 0 THEN the generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. | - | - | - | - |
| - | EXCEPTION: Steps 19Aa1 to 19Aa2 describe behaviour which depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 19Aa1 | IF connected without release is On AND Test Loop Function=On AND pc_noOf_PDUsNewConnection=0 THEN the SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. The CLOSE UE TEST LOOP is using the associated condition for the test loop. | <-- | NR RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 19Aa2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated. | --> | NR RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| - | EXCEPTION: Step 20a1 depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 20a1 | IF connected without release is not present THEN, the SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| - | EXCEPTION: Step 20Aa1 depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 20Aa1 | IF GNSS Sync = On THEN trigger UE to reset or clear the current UTC time that has been calculated from GNSS. NOTE: The UTC time can be reset or clear on the UE using AT command (+CUTCR). | - | - | - | - |
| - | EXCEPTION: Step 21a1 and 21b1 depend on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 21a1 | IF Sidelink = On AND Cast type = Unicast AND UE initiated unicast mode NR sidelink comunication THEN generic procedure specified in subclause 4.9.22 is performed to establish unicast mode sidelink communication between the UE and the NR-SS-UE. |  |  |  |  |
| 21b1 | ELSE IF Sidelink = On AND Cast type = Unicast AND NR-SS-UE initiated unicast mode NR sidelink comunication THEN generic procedure specified in subclause 4.9.23 is performed to establish unicast mode sidelink communication between the UE and the NR-SS-UE. |  |  |  |  |
| 22a1 | Void | - | - |  |  |

Table 4.5.2.2-3: WLAN Ipsec_SA_Released

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE associates with the WLAN AP and obtains the local IP address. | - | - | - | - |
| 2 | The UE performs a dynamic selection of N3IWF using DNS query | - | - | - | - |
| - | Exception: The UE establishes an IPsec tunnel in parallel to 5GC registration steps 3 to 7 as per the IKEv2 protocol as defined in TS 23.502 [33] clause 4.12.2.2 figure 4.12.2.2-1. | - | - | - | - |
| 3 | The UE transmits a REGISTRATION REQUEST message. | --> | 5GMM: REGISTRATION REQUEST | - | - |
| 4 | The SS transmits an AUTHENTICATION REQUEST message including EAP-Request/AKA'-Challenge or 5G AKA Challenge. | <-- | 5GMM: AUTHENTICATION REQUEST | - | - |
| 5 | The UE transmits an AUTHENTICATION RESPONSE message including EAP-Response/AKA'-Challenge or 5G AKA Response. | --> | 5GMM: AUTHENTICATION RESPONSE | - | - |
| 6 | The SS transmits a SECURITY MODE COMMAND message including EAP-Success if EAP-AKA' used. | <-- | 5GMM: SECURITY MODE COMMAND | - | - |
| 7 | The UE transmits a SECURITY MODE COMPLETE message. | --> | 5GMM: SECURITY MODE COMPLETE | - | - |
| 8 | The SS transmits a REGISTRATION ACCEPT message. | <-- | 5GMM: REGISTRATION ACCEPT | - | - |
| 9 | The UE transmits a REGISTRATION COMPLETE message. | --> | 5GMM: REGISTRATION COMPLETE | - | - |
| 10 | The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2A, takes place performing establishment of UE-requested PDU session. | - | - | - | - |
| - | EXCEPTION: Step 11a1 depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 11a1 | IF connected without release is not present THEN generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.5, takes place performing disconnection of security association. | - | - | - | - |
| Note: The current procedure assumes UE establishes a single PDU session over Non 3GPP Access. |  |  |  |  |  |

Table 4.5.2.2-4: NR RRC_IDLE Extension

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Start Wait_Timer = 10 sec to allow the UE to start PDU session establishment automatically. | - | - | - | - |
| - | EXCEPTION: Step 1a1 to 1b8 depends on the status of the UE; the "lower case letter" identifies a step sequence that take place if UE establishes PDU session autonomously. | - | - | - | - |
| 1a1 | Wait_Timer expires |  |  |  |  |
| - | EXCEPTION: Step 1a2a1 to 1a2b8 depends on the status of the UE; the "lower case letter" identifies a step sequence that take place if a PDU connection is already active | - | - | - | - |
| 1a2a1 | IF there is no PDU connection already active THEN Trigger the UE to perform PDU session establishment via AT or MMI command. NOTE: For the DNN name in the AT command, check the default PDU type from the value the ICS parameter pc_APN_Default_Configuration and take the value of the corresponding ICS parameter. If it is set to ‘none’ then use pc_APN_ID_Internet. | - | - | - | - |
| 1a2a2-1a2a6 | Steps 2-6 from Table 4.5.4.2-3 are performed. | - | - | - | - |
| 1a2a7 | The SS transmits a SERVICE ACCEPT message. | <-- | NR RRC: DLInformationTransfer 5GMM: SERVICE ACCEPT | - | - |
| 1a2a8 | The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsNewConnection. | - | - | - | - |
| 1a2b1-1a2b8 | ELSE Steps 1-8 from Table 4.5.4.2-3 are performed. (Note 2) |  |  |  |  |
| 1b1 | The UE transmits an RRCSetupRequest message. (Note 1) | --> | NR RRC: RRCSetupRequest |  |  |
| 1b2-1b7 | Steps 3-8 from Table 4.5.4.2-3 are performed |  |  |  |  |
| 1b8 | The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsNewConnection. | - | - | - | - |
| 2-8 | Void | - | - | - | - |
| - | EXCEPTION: Steps 8Aa1 to 8Aa2 describe behaviour which depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 8Aa1 | IF connected without release is On AND Test Loop Function=On THEN the SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. The CLOSE UE TEST LOOP is using the associated condition for the test loop. | <-- | NR RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 8Aa2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated. | --> | NR RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| - | EXCEPTION: Step 9a1 depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 9a1 | IF connected without release is not present THEN, the SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| Note 1: UE establishes PDU connection autonomously Note 2: PDU connection is already active AND UE does not establish PDU connection autonomously |  |  |  |  |  |

Table 4.5.2.2-5: E-UTRA RRC_IDLE Unrestricted nr PDN

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-9a2 | Steps 1 to 9a2 of the generic procedure for UE Registration (State 2) as specified in TS 36.508 [2], table 4.5.2.3-1 take place. | - | - | - | - |
| - | EXCEPTION: Steps 10a1 to 10b8 describe behaviour which depends on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 10a1-10a8 | IF Test Mode = On OR Test Loop Function = On THEN steps 10-17 of the generic procedure for UE Registration, UE Test Mode Activated (State 2A) as specified in TS 36.508 [2] table 4.5.2A.3-1, take place. The ACTIVATE TEST MODE is using the associated condition for the test loop. | - | - | - | - |
| 10b1-10b6 | ELSE steps 10-15 as defined in TS 36.508 [2], table 4.5.2.3-1 take place. | - | - | - | - |
| - | EXCEPTION: IF the 'IP address allocation' for the APN for which the PDN connection is established is set to "Yes" in Table 4.8.4-1 THEN, in parallel to the event described in step 10A below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 [2], subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. | - | - | - | - |
| - | EXCEPTION: IF the 'IMS registration' for the APN for which the PDN connection is established is set to "Yes" in Table 4.8.4-1, THEN in parallel to the event described in step 10A below the relevant generic procedure for IMS signalling in the U-plane specified in Table 4.8.4-1 takes place. | - | - | - | - |
| 10A | This message includes the ATTACH COMPLETE message. The ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message is piggybacked in ATTACH COMPLETE. | --> | RRC: ULInformationTransfer NAS: ATTACH COMPLETE NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT | - | - |
| 11 | The Generic Test Procedure to establish multiple additional PDN connections as specified in subclause 4.5A.2B takes place, ExpectedNumberOfNewPDNConnections=pc_noOf_PDNsSameConnection. | - | - | - | - |
| - | EXCEPTION: Steps 11Aa1 to 11Aa2 describe behaviour which depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 11Aa1 | IF connected without release is On AND Test Loop Function=On AND pc_noOf_PDUsNewConnection =0 THEN the SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. The CLOSE UE TEST LOOP is using the associated condition for the test loop. | <-- | RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 11Aa2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated. | --> | RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| - | EXCEPTION: Step 12a1 depends on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 12a1 | IF connected without release is not present THEN, the SS transmits an RRCConnectionRelease message to release RRC connection and move to E-UTRA RRC_IDLE (State 2). | <-- | RRC: RRCConnectionRelease | - | - |

Table 4.5.2.2-6: E-UTRA RRC_IDLE Unrestricted nr PDN Extension

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait for 10 sec to allow the UE to start PDN connection establishment automatically. IF it does THEN stop the 10 sec timer and continue with the steps from step 2 onwards. IF it does not and the 10 sec timer expires THEN go to step 1. | - | - | - | - |
| - | EXCEPTION: Step 1a1 and 1b1 depends on the status of the UE; the "lower case letter" identifies a step sequence that take place if a PDU connection is already active. | - | - | - | - |
| 1a1 | IF there is no PDN connection already active THEN Trigger the UE to perform PDN connection establishment via AT or MMI command. NOTE: For the DNN name in the AT command, check the default PDU type from the value the ICS parameter pc_APN_Default_Configuration and take the value of the corresponding ICS parameter. If it is set to ‘none’ then use pc_APN_ID_Internet. | - | - | - | - |
| 1b1 | ELSE skip the steps 2-8a1 | - | - | - | - |
| 2-6 | Steps 3 to 7 of the generic procedure specified in TS 36.508 [2], table 4.5.3.3-1 take place. | - | - | - | - |
| 7 | The Generic Test Procedure to establish multiple additional PDN connections as specified in subclause 4.5A.2B takes place, ExpectedNumberOfNewPDNConnections=pc_noOf_PDNsNewConnection. | - | - | - | - |
| - | EXCEPTION: Steps 7Aa1 to 7Aa2 describe behaviour which depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 7Aa1 | IF connected without release is On AND Test Loop Function=On THEN the SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. The CLOSE UE TEST LOOP is using the associated condition for the test loop. | <-- | RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 7Aa2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated. | --> | RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| - | EXCEPTION: Step 18a1 depends on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 8a1 | IF connected without release is not present THEN, the SS transmits an RRCConnectionRelease message to release RRC connection and move to E-UTRA RRC_IDLE (State 2). | <-- | RRC: RRCConnectionRelease | - | - |

#### 4.5.2.3	Specific message contents

All specific message contents shall be according clause 4.6 and 4.7 and TS 36.508 [2] clause 4.6 and 4.7 with the following exception(s).

Table 4.5.2.3-1: SECURITY MODE COMMAND (Step 9Aa1, Table 4.5.2.2-2)

Derivation path: Table 4.7.1-25 with condition UE_S1_SUPPORTED

Table 4.5.2.3-2: SERVICE REQUEST (Step 4, Table 4.5.2.2-4)

| Derivation Path: Table 4.7.1-16. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Service type | ‘0000’B | signalling |  |

Table 4.5.2.3-3: Message PDN CONNECTIVITY REQUEST (step 4, Table 4.5.2.2-5)

| Derivation path: TS 36.508 [2], Table 4.7.3-20. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Access point name | Not present or One of the provided APN(s) in the Table 4.8.4-1 | If present, the SS shall initialise the APN_Default=False If not present NOTE 2 |  |
| NOTE 1: Unless explicitly specified otherwise, the SS uses the Access point name value to address the entry of Table 4.8.4-1 to be used for the subsequent signalling of the PDN connectivity establishment and verifying specific UE behaviour e.g. depending on the type of the APN the UE may perform some actions. NOTE 2: The SS uses pc_APN_Default_Configuration to address the entry of Table 4.8.4-1 to be used for the subsequent signalling of the PDN connectivity establishment. |  |  |  |

Table 4.5.2.3-4: Message ESM INFORMATION RESPONSE (step 9a2, Table 4.5.2.2-5)

| Derivation path: TS 36.508 [2], Table 4.7.3-14. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Access point name | Not present or One of the provided APN(s) in the Table 4.8.4-1 | If present, the SS shall initialise the APN_Default=False If not present NOTE 2 |  |
| NOTE 1: Unless explicitly specified otherwise, the SS uses the Access point name value to address the entry of Table 4.8.4-1 to be used for the subsequent signalling of the PDN connectivity establishment and verifying specific UE behaviour e.g. depending on the type of the APN the UE may perform some actions. NOTE 2: The SS uses pc_APN_Default_Configuration to address the entry of Table 4.8.4-1 to be used for the subsequent signalling of the PDN connectivity establishment. |  |  |  |

Table 4.5.2.3-5: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (Steps 10a5, 10b7, Table 4.5.2.2-5)

| Derivation path: TS 36.508 [2], Table 4.7.3-6 with CONDITION Interworking_with_5GS. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS QoS | The EPC default bearer context of the entry in Table 4.8.4-1 which has been determined at step 1 |  |  |
| Access point name | IF the UE provided an Access point name in step 1 THEN the SS returns this name; OTHERWISE the SS includes the DNN/APN ID value of the entry in Table 4.8.4-1 which has been determined at step 1. |  |  |
| Container ID n+3 | '001C'H |  |  |
| Length of container ID n+3 contents |  |  |  |
| Container ID n+3 contents | 5GC QoS rule of the entry in Table 4.8.4-1 which has been determined at step 1 |  |  |
| Container ID n+4 | '001F'H |  |  |
| Length of container ID n+4 contents |  |  |  |
| Container ID n+4 contents | As per the relevant QoS rule (Container ID n+3) |  |  |

<End of Change>