3GPP TSG-WG5 Meeting #100	R5-234158

# Toulouse, France, August 21th -25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5462 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MPR requirments for UL 256QAM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, LTE_UL_CAP_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current test case which is introduced in R5-180770, the MPR requirements for band 22 and band 28 were found incorrect. E.g. for band 28, the lower limit is -5.2dB, which clearly do not consider lower power limit, MPR and TT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the MPR requirements of band 22 and Band 28 to consider the 5dB MPR requirements based on the lower limit and the TT. According to the formula: “Lower limit – MPR – TT”, the lower limit of the two bands are calculated as: Band 22: “-3.5(lower limit) – 5 (MPR) - 0.7(TT) = -9.2dB” Band 28: “ -2.5(lower limit) – 5 (MPR) - 0.7(TT) = -8.2dB” In addition, for band 22, the corresponding note is also need to be added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MPR requirements of band 22 and band 28 were unnecessarily tightened significantly and correctly implentened UE may fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3_5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

6.2.3_5.5	Test requirements

The maximum output power, derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.3_5.5-1.

Table 6.2.3_5.5-1: UE Power Class test requirements

| E-UTRA Band | Class 1 (dBm) | Tol. (Db) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | 256QAM partial RB allocation Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7 / -7.7 |
| 2 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 3 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 4 |  |  |  |  | 23 | +2.7 / -7.7 |
| 5 |  |  |  |  | 23 | +2.7 / -7.7 |
| 6 |  |  |  |  | 23 | +2.7 / -7.7 |
| 7 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 8 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 9 |  |  |  |  | 23 | +2.7 / -7.7 |
| 10 |  |  |  |  | 23 | +2.7 / -7.7 |
| 11 |  |  |  |  | 23 | +2.7 / -7.7 |
| 12 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 13 |  |  |  |  | 23 | +2.7 / -7.7 |
| 14 |  |  |  |  | 23 | +2.7 / -7.7 |
| … |  |  |  |  |  |  |
| 17 |  |  |  |  | 23 | +2.7 / -7.7 |
| 18 |  |  |  |  | 23 | +2.7 / 3 -7.7 |
| 19 |  |  |  |  | 23 | +2.7 / -7.7 |
| 20 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 21 |  |  |  |  | 23 | +2.7 / -7.7 |
| 22 |  |  |  |  | 23 | +3.0 / 1,2 -9.2 |
| 23 |  |  |  |  | 23 | +2.7 / -7.7 |
| 24 |  |  |  |  | 23 | +2.7 / 1,2 -8.7 |
| 25 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 26 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 27 |  |  |  |  | 23 | +2.7 / -7.7 |
| 28 |  |  |  |  | 23 | +2.7 / -8.2 |
| 31 |  |  |  |  | 23 | +2.7 / -7.7 |
| ... |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | +2.7 / -7.7 |
| 34 |  |  |  |  | 23 | +2.7 / -7.7 |
| 35 |  |  |  |  | 23 | +2.7 / -7.7 |
| 36 |  |  |  |  | 23 | +2.7 / -7.7 |
| 37 |  |  |  |  | 23 | +2.7 / -7.7 |
| 38 |  |  |  |  | 23 | +2.7 / -7.7 |
| 39 |  |  |  |  | 23 | +2.7 / -7.7 |
| 40 |  |  |  |  | 23 | +2.7 / -7.7 |
| 41 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 42 |  |  |  |  | 23 | +3.0 / -9.0 |
| 43 |  |  |  |  | 23 | +3.0 / -9.0 |
| 44 |  |  |  |  | 23 | +2.7 / [-8.7] |
| 45 |  |  |  |  | 23 | +2.7 / -7.7 |
| … |  |  |  |  |  |  |
| 54 |  |  |  |  | 23 | +2.7 / -7.7 |
| … |  |  |  |  |  |  |
| 65 |  |  |  |  | 23 | +2.7 / -7.7 |
| 66 |  |  |  |  | 23 | +2.7 / -7.7 |
| … |  |  |  |  |  |  |
| 70 |  |  |  |  | 23 | +2.7 / -7.7 |
| 72 |  |  |  |  | 23 | +2.7 / -7.7 |
| … |  |  |  |  |  |  |
| 74 |  |  |  |  | 23 | +2.7 / -7.7 |
| 85 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 87 |  |  |  |  | 23 | +2.7 / -7.7 |
| 88 |  |  |  |  | 23 | +2.7 / -7.7 |
| Note 1: For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Note 2: For the UE maximum output power modified by MPR, the power limits specified in Table 6.2.5.3-1 apply Note 3: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5dB for transmission bandwidths confined within 815 MHz and 818 MHz |  |  |  |  |  |  |

<End of Change>