3GPP TSG WG RAN5 Meeting #100	R5-234232

# Toulouse, FR, 21 – 25 Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | N/A | rev | - | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of A-MPR NS value in 6.2B.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-18 |
| Category: | N/A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Follow TS 36.101-2 definition to correction A-MPR NS-02N value with 2 in message content and completed A-MPR configuration table and test requirement table also |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of A-MPR NS value in message content 6.2B.3 Complete A-MPR test configuration table Complete A-MPR test requirement table |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of changes >>

6.2B.3.4	Test description

6.2B.3.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NB-IOT NTN operating bands specified in sub-clause 5.2B. All these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in Table 6.2B.3.4.1-1 to 6.2B.3.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in TS36.521-1[14] Annex A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in TS 36.521-1[14] Annex C.2.

Table 6.2B.3.4.1-1: Test Configuration Table (network signalled value “NS_02N”)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.2 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A for Maximum Power Reduction (MPR) test case | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 1) |  | QPSK | 3@0 | 15 |
| 2 (Note 1) |  | QPSK | 3@3 | 15 |
| 3 (Note 1) |  | QPSK | 3@9 | 15 |
| 4 (Note 1) |  | QPSK | 6@0 | 15 |
| 5 (Note 1) |  | QPSK | 6@6 | 15 |
| 6 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

Table 6.2B.3.4.1-2: Test Configuration Table (network signalled value "NS_24")

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.2 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A for Maximum Power Reduction (MPR) test case | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 1) |  | QPSK | 3@0 | 15 |
| 2 (Note 1) |  | QPSK | 3@3 | 15 |
| 3 (Note 1) |  | QPSK | 3@9 | 15 |
| 4 (Note 1) |  | QPSK | 6@0 | 15 |
| 5 (Note 1) |  | QPSK | 6@6 | 15 |
| 6 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

<< Unchanged sections omitted >>

6.2B.3.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.6 and with the following exceptions for each network signalled value.

6.2B.3.4.3.1	Message contents exceptions (network signalled value “NS_02N”)

1.	Information element additionalSpectrumEmission is set to NS_02N. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2B.3.4.3.1-1: SystemInformationBlockType2: Additional spurious emissions test requirement for “NS_02N”

| Derivation Path: TS 36.508 [12] clause 8.1.4.3, Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2(NS_02N) |  |  |

<< Unchanged sections omitted >>

6.2B.3.5	Test requirement

The maximum output power derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2B.3.5-1 to Table 6.2B.3.5-2. The allowed A-MPR values specified in Table 6.2B.3.3-1 are in addition to the allowed MPR requirements specified in clause 6.2B.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in TS.36.521-1[14] Table 6.2.5 F.3-1 and 6.2.5F.3-2 apply.

Table 6.2B.3.5-1: UE Power Class test requirements (network signalled value “NS_02N”)

| Test ID | MPR (dB) |  |  | A_MPR (dB) |  | Power class 3 |  |  |  |  | Power class 5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PCMAX,c (dBm) |  | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | PCMAX,c (dBm) |  | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| 1 | 0.5 |  | N/A |  | 22.5 |  | 2.0 | 25.7 | 19.8 | 19.5 |  | 2.0 | 22.7 |  | 16.8 |
| 2 | 0 |  | N/A |  | 23 |  | 2.0 | 25.7 | 20.3 | 20 |  | 2.0 | 22.7 |  | 17.3 |
| 3 | 0.5 |  | N/A |  | 22.5 |  | 2.0 | 25.7 | 19.8 | 19.5 |  | 2.0 | 22.7 |  | 16.8 |
| 4 | 1 |  | N/A |  | 22 |  | 2.0 | 25.7 | 19.3 | 19 |  | 2.0 | 22.7 |  | 16.3 |
| 5 | 1 |  | N/A |  | 22 |  | 2.0 | 25.7 | 19.3 | 19 |  | 2.0 | 22.7 |  | 16.3 |
| 6 | 2 |  | N/A |  | 21 |  | 2.0 | 25.7 | 18.3 | 18 |  | 2.0 | 22.7 |  | 15.3 |
|  |  | NOTE 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [7] clause 6.2.5F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2B.3.5-2: UE Power Class test requirements (network signalled value "NS_24")

| Test ID | MPR (dB) |  | A_MPR (dB) | Power class 3 |  |  |  | Power class 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0.5 |  | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 19.5 | 2.0 | 22.7 | 16.8 |
| 2 | 0 |  | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 20 | 2.0 | 22.7 | 17.3 |
| 3 | 0.5 |  | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 19.5 | 2.0 | 22.7 | 16.8 |
| 4 | 1 |  | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 19 | 2.0 | 22.7 | 16.3 |
| 5 | 1 |  | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 19 | 2.0 | 22.7 | 16.3 |
| 6 | 2 |  | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 18 | 2.0 | 22.7 | 15.3 |
|  |  | NOTE 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [7] clause 6.2.5F |  |  |  |  |  |  |  |  |  |

<< End of changes >>