3GPP TSG-RAN5 Meeting #100	R5-234463

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3922 | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MBS Broadcast for TC 14.1.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test step is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct Step67 to Step62a6 and Correct Step77 to Step62a12 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test step is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

14.1.1.1.3.2	Test procedure sequence

Table 14.1.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-21 | Steps 1 to 20a1 of the registration procedure described in TS 38.508-1 subclause 4.5.2.2-2 are performed on NR Cell 1 with condition UE TEST LOOP MODE C. NOTE: The UE performs registration and activate test mode C and the RRC connection is released. | - |  | - | - |
| 22 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 1. (Note 1) | - | - | - | - |
| 23-31 | Steps 1 to 9a2 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed on NR Cell 1 with condition UE TEST LOOP MODE C. | - | - | - | - |
| - | Exception: Step 32 is repeated 5 times | - | - | - | - |
| 32 | The SS transmits a MBS Packet on the MTCH with LCID=1 and g-RNTI = ’0001’H. | <-- | MBS Packet. | - | - |
| 33 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 34 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC:UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 35 | Check: Is the number of reported MBS Packets received on the MTCH in step 34 greater than zero? | - | - | 1,6 | P |
| 36 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 37 | The SS configures: -NR Cell 1 as the "Non-suitable cell". -NR Cell 2 as the "Serving cell". | - | - | - | - |
| 38 | Wait for 34[FR1]/130[FR2] seconds.(Note 2) | - | - | - | - |
| 39 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 2. (Note 1) | - | - | - | - |
| - | Exception: Step 40 is repeated 5 times | - | - | - | - |
| 40 | The SS transmits a MBS Packet on the MTCH with LCID= 1 and g-RNTI = ’FFF2’H. | <-- | MBS Packet | - | - |
| 41-48 | Steps 1 to 8 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed on NR Cell 2. | - | - | - | - |
| 49 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 50 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 51 | Check: Is the number of reported MBS Packets received on the MTCH in step 50 greater than the number of reported in step 34? | - | - | 2,6 | P |
| 52 | SS change NR Cell 1 system information | - | - | - | - |
| 53 | The SS configures: -NR Cell 1 as the "Serving cell ". -NR Cell 2 as the "Suitable neighbour intra-frequency cell ". |  |  |  |  |
| 54 | The SS transmits an RRCReconfiguration message including reconfigurationWithSync to order the UE to perform intra-frequency handover to NR Cell 1 | <-- | NR RRC: RRCReconfiguration | - | - |
| 55 | Check: Does the UE transmit RRCReconfigurationComplete message in NR Cell 1? | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 56 | Wait for a scheduling period for SIB20. | - | - | - | - |
| 57 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 1. (Note 3) | - | - | - | - |
| - | Exception: Step 58 is repeated 5 times | - | - | - | - |
| 58 | The SS transmits a MBS Packet on the MTCH with LCID=1 and g-RNTI = ’0002’H | <-- | MBS Packet | - | - |
| 59 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 60 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 61 | Check: Is the number of reported MBS Packets received on the MTCH in step 60 greater than the number of reported in step 50? | - | - | 4,5 | P |
| - | EXCEPTION: Steps 62a1-62a13 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place if inactiveState is configured | - | - | - | - |
| 62a1 | IF pc_inactiveState THEN the SS transmits an RRCRelease message with suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 62a2 | SS change NR Cell 2 system information | - | - | - | - |
| 62a3 | The SS configures: -NR Cell 1 as the "Non-suitable cell". -NR Cell 2 as the "Serving cell". | - | - | - | - |
| 62a4 | Wait for 8[FR1]/27[FR2] seconds.(Note 2) | - | - | - | - |
| 62a5 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 2. (Note 3) | - | - | - | - |
| - | Exception: Step 62a6 is repeated 5 times | - | - | - | - |
| 62a6 | The SS transmits a MBS Packet on the MTCH with LCID=1 and g-RNTI = ’FFF1’H. | <-- | MBS Packet | - | - |
| 62a7 | The SS transmits a Paging message including a matched identity (correct fullI-RNTI). | <-- | NR RRC: Paging | - | - |
| 62a8 | The UE transmits an RRCResumeRequest message. | --> | NR RRC: RRCResumeRequest | - | - |
| 62a9 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 62a10 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 62a11 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 62a12 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 62a13 | Check: Is the number of reported MBS Packets received on the MTCH in step 62a12 greater than the number of reported in step 60? | - | - | 3,5 | P |
| Note 1: MAC PDU for MBSBroadcastConfiguration is 15 bytes (12 bytes RLC SDU + 1 byte UMD PDU header + 2 bytes MAC sub Header) and DL assignments are set to 96 bits (LRBs & IMCS as per 38.523-3[3] annex B) so that MBSBroadcastConfiguration need to be sent in 2 slots. Note 2: The wait time at steps 38 is the cell re-selection delay to a newly detectable cell, it can be expressed as: Tdetect,NR_Intra (as per TS 38.133 [30], clause 4.2.2.3) plus the time to read the system information TSI-NR (1280ms). Note 3: MAC PDU for MBSBroadcastConfiguration is 15 bytes (12 bytes RLC SDU + 1 byte UMD PDU header + 2 bytes MAC sub Header) and DL assignment is set to larger than 120 bits (LRBs & IMCS as per 38.523-3[3] annex B) so that MBSBroadcastConfiguration could to be sent in 1 slot. Note 4: The wait time at steps 62a4 is the cell re-selection delay to an already detected cell, it can be expressed as: Tevaluate,NR_Intra (as per TS 38.133 [30], clause 4.2.2.3) plus the time to read the system information TSI-NR (1280ms). |  |  |  |  |  |

<End of modified section 1>

<Start of modified section 2>

Table 14.1.1.1.3.3-4: BWP-DownlinkCommon (Table 14.1.1.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 with condition InitialBWP_SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon with conditionMBS_Broadcast |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 2>