3GPP TSG-RAN5 Meeting #100	R5-234485

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

Source:	Huawei, HiSilicon

Title:	Discussion to split or modify BWP test case

Agenda item:	6.4.7

Document for:	Discussion and Endorsement

Background

TC 7.1.1.8.1 is approved in TTCN-status and is also validated in GCF.

This test case may have different configuration based on different PICS setting. Especially the pc_bwp-WithoutRestriction, if pc_bwp-WithoutRestriction=True, SS needs to configure 6RB dedicated BWP and send TRS to UE for time-frequency tracking because dedicated BWP does not fully include SSB. These two configuration are not used in other signaling test cases. There are two kinds of UE:

(1) UE supports both SSB outside BWP and SSB inside BWP, this UE set pc_bwp_WithoutRestriction=True (simplify it as pc_bwp_WithoutRestriction=True UE)

(2) UE does not support SSB outside BWP and only supports SSB inside BWP, this UE set pc_bwp_WithoutRestriction=False (simplify it as pc_bwp_WithoutRestriction=False UE).

SS will perform different configuration based on UE pics setting. If SS test 7.1.1.8.1 with a pc_bwp_WithoutRestriction=False UE, it does not need to configure 6RB BWP and TRS. If SS test 7.1.1.8.1 with a pc_bwp_WithoutRestriction=True UE, it needs to configure 6RB BWP and TRS.If SS validated 7.1.1.8.1 with a pc_bwp_WithoutRestriction=False UE, then a pc_bwp_WithoutRestriction=True UE also needs to test 7.1.1.8.1 when performing GCF certification. However, pc_bwp_WithoutRestriction=True UE may fail because 6RB BWP and TRS haven’t been tested.

From pics setting in R5s230312, we can find that 7.1.1.8.1 is approved based on pc_bwp_WithoutRestriction = False UE.

pc_bwp_WithoutRestriction = False

pc_bwp_SwitchingDelay_Type1 = False

pc_bwp_SwitchingDelay_Type2 = True

pc_bwp_SameNumerology_upto2_FR1_TDD = True

pc_bwp_SameNumerology_upto4_FR1_TDD = False

[38.306]

bwp-WithoutRestriction

Indicates support of BWP operation without bandwidth restriction. The Bandwidth restriction in terms of DL BWP for PCell and PSCell means that the bandwidth of a UE-specific RRC configured DL BWP may not include the bandwidth of CORESET #0 (if configured) and SSB. For SCell(s), it means that the bandwidth of DL BWP may not include SSB

Discussion

Observation 1:If pc_bwp-WithoutRestriction = True, the bandwidth for BWP#1#2#3 are 6RB which is configured in Table 7.1.1.8.1.3.3-2A.

- BWP#1#2#3 could not include entire SSB and CORESET#0, so UE needs to use the TRS configured in Table 7.1.1.8.1.3.3-5/6/7/8/10 to measure time and frequency offset instead of SSB. TRS are not used in other signaling test case, and TRS is not needed for pc_bwp-WithoutRestriction = false branch. We can not deduce “pc_bwp-WithoutRestriction = true conformant UE passing 7.1.1.8.1” from “pc_bwp-WithoutRestriction = false conformant UE passing 7.1.1.8.1” because the configuration is different and SS only validated 7.1.1.8.1 with the pc_bwp-WithoutRestriction = false UE.

- 38.523-3 7.1.2.2.3.1 assume that the minimize bandwidth for BWP is 24RB. If we want support 6RB BWP, 38.523-3 may need some update.

Table 7.1.1.8.1.3.3-2A: BWP-Downlink-BWP-N (Table 7.1.1.8.1.3.3-2 and Table 7.1.1.8.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  | BWP#1 |
|  | 2 |  | BWP#2 |
|  | 3 |  | BWP#3 |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | 1381 | Note 1 | BWP#1 and pc_bwp-WithoutRestriction = True |
|  | 1387 | Note 1 | BWP#2 and pc_bwp-WithoutRestriction = True |
|  | 1393 | Note 1 | BWP#3 and pc_bwp-WithoutRestriction = True |
|  | 6600 | Note 2 | BWP#1,2,3 and pc_bwp-WithoutRestriction = False and 5MHz |
|  | 7975 | Note 3 | BWP#1 and pc_bwp-WithoutRestriction = False |
|  | 9625 | Note 3 | BWP#2 and pc_bwp-WithoutRestriction = False |
|  | 11275 | Note 3 | BWP#3 and pc_bwp-WithoutRestriction = False |
|  | 12925 | Note 4 | BWP#1and pc_bwp-WithoutRestriction = False and 100MHz |
|  | 14575 | Note 4 | BWP#2and pc_bwp-WithoutRestriction = False and 100MHz |
|  | 16225 | Note 4 | BWP#3 and pc_bwp-WithoutRestriction = False and 100MHz |
| } |  |  |  |
| pdcch-ConfigCommon | Not present | no cell specific configuration for dedicated BWP |  |
| pdsch-ConfigCommon | Not present | no cell specific configuration for dedicated BWP |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config-BWP-N with condition BWP#N |  |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| setup | PDSCH-Config-BWP-N with condition BWP#N |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=6 and RBStart=6,12,18 for BWP#1,2,3 Note 2: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=25 and RBStart=0 for BWP#1,2,3 Note 3: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=30,36,42 and RBStart=0 for BWP#1,2,3 Note 4: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=48,54,60 and RBStart=0 for BWP#1,2,3 |  |  |  |

[38.523-3 7.1.2.2.3.1]

This scheduling scheme multiplexes the different kinds of PDSCH transmissions in the frequency domain of a single BWP by exclusively using resource allocation type 1 with non-interleaved VRB-to-PRB mapping. Assuming the resource blocks being numbered from 0 to NBWP-1 (with NBWP being the size of the BWP) the following allocation is done:

Table 7.1.2.2.3.1-1: Resource allocation for frequency domain multiplexing, RA type1, non-interleaved

| Kind of PDSCH transmission (Note 1) | CORESET# | SSB index of the cell (Note 4) | Resource block allocation (Note 2) |  |
| --- | --- | --- | --- | --- |
|  |  |  | RBstart (Note 2) | LRBs,max (Note 2) |
| System information | 0 | SSB#1 | 0 | 7 |
|  |  | SSB#0 | NBWP-7 | 7 |
| Paging (Note 3) | 0 | SSB#1 | 7 | NBWP-7 |
|  |  | SSB#0 | 0 | NBWP-7 |
| Random access response (Note 3) | 0 | SSB#1 | 7 | NBWP-7 |
|  |  | SSB#0 | 0 | NBWP-7 |
| CCCH and DCCH transmission on CSS (Note 3) | 0 | SSB#1 | 7 | NBWP-7 |
|  |  | SSB#0 | 0 | NBWP-7 |
| DCCH/DTCH transmissions (Note 3) | 1 | SSB#1 | 7+ Offset Carrier CORESET#0 [RBs] (Note 5) | NBWP-7 |
|  |  | SSB#0 | 0 | NBWP-7 |
| NOTE 1: In context of a generic 5G test model it is not relevant whether or not there is SI and Paging for a given deployment option (e.g. EN-DC). NOTE 2: LRBs ≤ LRBs,max with LRBs: number of resource blocks being eventually used for a particular transmission. LRBs and RBstart are as specified in TS 38.214 [22] clause 5.1.2.2.2. NOTE 3: In general Paging, Random access response and CCCH/DCCH/DTCH transmissions are mutual exclusive and therefore share the same allocation. NOTE 4: The SSB index used by an NR Cell is specified in TS 38.508-1 [5] Table 4.4.2-2. NOTE 5: Offset Carrier CORESET#0 [RBs] values are specified in the frequency tables in TS 38.508-1 [5]. |  |  |  |  |

In order to achieve a test case behaviour being independent from the frequency channel bandwidth NBWP is limited to the minimum value of 24 RBs in accordance to Table 5.3.2-1 of TS 38.101-1/2 [5, 6]. This implies that LRBs,max< 24 RBs for all configurations.

In general PDCCH and corresponding PDSCH transmissions are in the same slot (K0 = 0).

Observation 2: If UE support bwp-WithoutRestriction, it could test both pc_bwp-WithoutRestriction = True branch and pc_bwp-WithoutRestriction = False branch of 7.1.1.8.1. If UE does not support bwp-WithoutRestriction, it could only test pc_bwp-WithoutRestriction = False branch of 7.1.1.8.1.

Proposal

Based on the above discussion, the following is proposed:

Proposal: RAN5 to consider one of the following two possible options.

Option 1: Delete the 6RB BWP#1#2#3 configuration (used for pc_bwp-WithoutRestriction = True) from 7.1.1.8.1 and not create new test case.

Option 2: Split the 6RB BWP#1#2#3 configuration to 7.1.1.8.1a. If UE supports bwp-WithoutRestriction, it test 7.1.1.8.1 and 7.1.1.8.1a, else it only test 7.1.1.8.1.