3GPP TSG-RAN5 Meeting #100	R5-234499

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4754 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of NTN timing advance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., ROHDE & SCHWARZ, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to minimize the timing offset, the granularity of t value is suggested to use ms and keep more digits. |  |  |  |  |  |  |  |  |
| Summary of change: |  | t value and the corresponding NTA,adjUE were updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Extra offset may cause inaccurate timing in UL synchronization. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7A.14.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

7A.14.2.2	NTN timing advance

The Timing Advance (TA) formula, specified in TS 36.211 [35] clause 8.1, can be summarized as follows:

TTA = TTA,legacy + TTA,NTN

TTA,NTN = TTA,common + TTA,UE-specific

with:

-	TTA,legacy = (NTA + NTA,offset) * Ts: the (legacy) part of the TA formula that applies also to Terrestrial Networks,

-	TTA,common = NTA,adjcommon * Ts: the Common TA between the RP and the NTN payload,

-	TTA,UE-specific = NTA,adjUE * Ts: the TA corresponding to the service link delay.

The GSO test environment in TS 36.508 [3] specifies that both the serving satellite (NTN payload) and the UE under test have a static position during the lifetime of a test case execution. Therefore the corresponding TTA is also static for a given NTN cell. TTCN provides to the SS the following inputs to enable the SS to calculate TTA:

Table 7A.14.2.2-1: NTN timing advance - input parameters

| TTA element | Associated parameter(s) |  | Relationship |
| --- | --- | --- | --- |
| (in sec) | Protocol parameter | ASP field | (TS 36.213 [30] clause 16.1.2) |
| TTA,legacy |  | InitialValue integer (0..2047) | NTA = InitialValue * 16 |
| TTA,common | NTA,adjcommon | NtnCommonTA integer (0..8316827) | See below. |
| TTA,UE-specific | NTA,adjUE | NtnUeSpecificTA integer (0..8316827) | NTA,adjUE= NtnUeSpecificTA See below. |
| NOTE: The time unit TS is defined in TS 36.211 [35] clause 4. |  |  |  |

From the nta-CommonParameters-17 specified in TS 36.331 [19], only the value of nta-Common-r17 is defined in TS 36.508 [3] SIB31-NB for the GSO scenario. TTCN will forward this value to the SS as NtnCommonTA for the calculation by the SS of NTA,adjcommon according to the formula given in TS 36.213 [30] clause 16.1.2.

NOTE:	For the GSO scenario it is assumed that the SS uses the calculation: NTA,adjcommon = NtnCommonTA.

NTA,adjUE is calculated using the GSO serving satellite static position specified in TS 36.508 [3] SIB31-NB's ephemeris data and the UE static position specified in TS 36.508 [3] clause 4.13. The end result is provided to the SS by TTCN. The calculation is provided hereafter:

The two-way propagation delay in sec between the GSO serving satellite and the non-moving UE is assumed to be constant at any time and is calculated according to the formula:

t=2*d/c

where c is the speed of light (299792458 m/sec) and d is the distance in meters between the serving satellite and the UE.

For two points in Euclidian geometry d in meters is derived by:

d=a??x+a??y+a??z

with

a??x=(xsat-xUE)2

a??y= (ysat-yUE)2

a??z=(zsat-zUE)2

and

xsat, ysat, zsat being serving satellite coordinates, in m

xUE, yUE, zUE being UE coordinates, in m

To use above formulas the positioning data provided in TS 36.508 [3] is first converted to the coordinates accordingly.

Example of the calculation of a??x:

given UE latitude 25.08439333 and positionX-r17 of -16976014:

xsat= -16976014* 1.3 m = -22068818.2m

xUE= -3025296.94 m

a??x=(-22068818.2 m-(-3025296.94 m))2=(-19043521.26 m)2= 362655701980071.94 m2

Following the above calculations, the final rounded value t results in 243.3896 that yields NTA,adjUE to have a value of 7476928.

[End of change]