3GPP TSG-RAN5 Meeting #100	R5-234525

# Toulouse, France, 21st - 25th August

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0734 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of missing RMCs for RedCap PDCCH Test Case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the RedCap TDD PDCCH Test Case the required RMCs are missing in the specification. R.PDCCH.2-1.5 TDD and R.PDCCH.2-1.6 TDD need to be copied from 38.101-4 and a new additional PDSCH Reference Channel for 20 MHz bandwidth needs to be introduced |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add R.PDCCH.2-1.5 TDD and R.PDCCH.2-1.6 TDD to table A.3.3.2.2-1 Add new 20 MHz additional PDSCH to table A.3.3.2.2-3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Case cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged sections omitted >

#### A.3.3.2.2	Reference measurement channels for SCS 30 kHz FR1

Table A.3.3.2.2-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.2-1.1 TDD | R.PDCCH.2-1.2 TDD | R.PDCCH.2-1.3 TDD | R.PDCCH.2-1.4 TDD | R.PDCCH.2-1.5 TDD | R.PDCCH.2-1.6 TDD |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 | 30 |
| CORESET frequency domain allocation |  | 102 | 102 | 90 | 102 | 48 | 48 |
| CORESET time domain allocation |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Aggregation level |  | 2 | 4 | 8 | 8 | 4 | 8 |
| DCI Format |  | 1_0 | 1_1 | 1_1 | 2_6 | 1_1 | 1_1 |
| Payload (without CRC) | Bits | 41 | 53 | 53 | 12 | 53 | 53 |

Table A.3.3.2.2-2: PDCCH Reference Channel (Time domain allocation 2 symbols)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.2-2.1 TDD |  |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |  |
| CORESET frequency domain allocation |  | 48 |  |  |  |  |  |
| CORESET time domain allocation |  | 2 |  |  |  |  |  |
| Aggregation level |  | 16 |  |  |  |  |  |
| DCI Format |  | 1_0 |  |  |  |  |  |
| Payload (without CRC) | Bits | 41 |  |  |  |  |  |

Table A.3.3.2.2-3: Additional PDSCH Reference Channel TDD

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| DCI Format |  |  | 1-0 | 1-1 | 1-1 |
| TDD UL/DL pattern |  |  | FR1.30-1 | FR1.30-1 | FR1.30-1 |
| Channel bandwidth |  | MHz | 40 | 40 | 20 |
| Subcarrier spacing |  | kHz | 30 | 30 | 30 |
| Allocated resource blocks |  | PRBs | 106 | 106 | 51 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  |  | 4 | 4 | 4 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6) for i from {1,…,39} |  |  | 12 | 12 | 12 |
| Allocated slots per 2 frames |  |  | 31 | 31 | 31 |
| MCS table |  |  | 64QAM | 64QAM | 64QAM |
| MCS index |  |  | 4 | 4 | 4 |
| Modulation |  |  | QPSK | QPSK | QPSK |
| Target Coding Rate |  |  | 0.30 | 0.3 | 0.3 |
| Number of MIMO layers |  |  | 1 | 1 | 1 |
| Number of DMRS rEs |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  |  | 6 | 6 | 6 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6) for i from {1,…,39} |  |  | 12 | 12 | 12 |
| Overhead for TBS determination |  |  | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | Bits | 2280 | 2664 | 1288 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6) for i from {1,…,39} |  | Bits | 6912 | 8456 | 4032 |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | Bits | 16 | 16 | 16 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6) for i from {1,…,39} |  | Bits | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | CBs | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | CBs | 1 | 1 | 1 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6) for i from {1,…,39} |  | CBs | 1 | 2 | 1 |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | Bits | 7488 | 8904 | 4284 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6) for i from {1,…,39} |  | Bits | 22896 | 27984 | 13464 |
| Max. Throughput averaged over 2 frames |  | Mbps | 9.78 | 11.94 | 5.70 |
| Note 1: For PDSCH & PDSCH DMRS precoding the following configuration shall be used: Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity. Note 2: For tests with DRX configured, PDCCH and PDSCH shall only be scheduled during the DRX on duration of the test. |  |  |  |  |

#### < End of changes >