# 3GPP TSG-RAN5 Meeting #100	R5-234719

# Toulouse, France, Aug 21st - Aug 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3953 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 8.1.6.1.2.16 for SON_MDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add SON/MDT TC 8.1.6.1.2.16 Logged MDT / RRC_IDLE / sig-based logged MDT |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add SON/MDT TC 8.1.6.1.2.16 Logged MDT / RRC_IDLE / sig-based logged MDT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Plan is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.16 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0395 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

# 8.1.6.1.2.16	Logged MDT / RRC_IDLE / sig-based logged MDT

8.1.6.1.2.16.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state with logged measurements available for NR and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

ensure that {

when { The sigLoggedMeasType is included in VarLogMeasReport }

then { UE include the sigLogMeasConfigAvailable in the RRCSetupComplete message }

}

8.1.6.1.2.16.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.331, clause 5.3.3.4. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

……

1>	set the content of RRCSetupComplete message as follows:

……

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

3>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableBT in the RRCSetupComplete message;

3>	if WLAN measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

2>	if the sigLoggedMeasType in VarLogMeasReport is included:

3>	if T330 timer is running and the logged measurements configuration is for NR:

4>	set sigLogMeasConfigAvailable to true in the RRCSetupComplete message;

3>	else:

4>	if the UE has logged measurements available for NR:

5>	set sigLogMeasConfigAvailable to false in the RRCSetupComplete message;

……

8.1.6.1.2.16.3	Test description

8.1.6.1.2.16.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.6.1.2.16.3.2	Test procedure sequence

Table 8.1.6.1.2.16.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an LoggedMeasurementConfiguration message with sigLoggedMeasType set to ‘true’. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits a RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 30 seconds for the UE to perform the logging at regular time intervals. | - | - | - | - |
| 4-6 | Steps 1 to 3 of the generic procedure in TS 38.508-1 [4], Table 4.5.4.2-3 are performed. | - | - | - | - |
| 7 | Check: Does the UE transmit an RRCSetupComplete message including sigLogMeasConfigAvailable IE set it to true? | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | 1 | P |

8.1.6.1.2.16.3.3	Specific message contents

Table 8.1.6.1.2.16.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.16.3.2-1)

| Derivation Path: TS 38.508-1 Table 4.6.1-5AA |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |  |
| reportType CHOICE { |  |  |  |  |
| periodical SEQUENCE { |  |  |  |  |
| loggingInterval-r16 |  | ms61440 | 61.44s |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| LoggedMeasurementConfiguration-v1700-IEs |  | LoggedMeasurementConfiguration-v1700-IEs with condition SigLogMDT |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.6.1.2.16.3.3-2: RRCSetupComplete (step 7, Table 8.1.6.1.2.16.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition sig_LOG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>