3GPP TSG-RAN5 Meeting #100	R5-235047

# Toulouse , France, 21st – 25th Aug. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2427 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating RB allocations in table 6.1A-1b |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some RB allocations are incorrect in table 6.1A-1b |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correcting RB allocations in table 6.1A-1b |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Intra-band contiguous UL CA non-contiguous RB allocation are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.1A	General

Editor’s note:

Uplink RB allocations for intra-band UL CA with mixed numerology is FFS

The minimum requirements for band combinations including Band n41 also apply for the corresponding band combinations with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said band combinations with Band n90 are not listed in the tables below but are covered by this specification.

Uplink RB allocations for intra-band CA given in Table 6.1A-1a are used throughout this section, unless otherwise stated by the test case.

<Unchanged Text Skipped>

Table 6.1A-1b: Common uplink configuration for intra-band contiguous 2UL CA (non-contiguous RB allocation)

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | RB allocation (Inner) |  |  | RB allocation (Outer1) |  |  | RB allocation (Outer2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
| 20 | 15 | 5+15 | 30 | 1@25 | 1@29 | 65 | 1@18 | 1@57 | 104 | 1@0 | 1@78 |
|  |  | 10+10 | 34 | 1@35 | 1@16 | 63 | 1@15 | 1@25 | 104 | 1@0 | 1@51 |
|  |  | 15+5 | 37 | 1@52 | 1@9 | 65 | 1@19 | 1@4 | 104 | 1@0 | 1@24 |
| 25 | 15 | 5+20 | 34 | 1@25 | 1@33 | 81 | 1@22 | 1@77 | 131 | 1@0 | 1@106 |
|  |  | 10+15 | 43 | 1@44 | 1@34 | 80 | 1@21 | 1@48 | 131 | 1@0 | 1@78 |
|  |  | 15+10 | 43 | 1@43 | 1@6 | 80 | 1@21 | 1@21 | 131 | 1@0 | 1@51 |
|  |  | 20+5 | 48 | 1@67 | 1@8 | 80 | 1@28 | 1@1 | 131 | 1@0 | 1@24 |
|  | 30 | 10+15 | 40 | 1@21 | 1@16 | 37 | 1@9 | 1@21 | 62 | 1@0 | 1@37 |
|  |  | 15+10 | 40 | 1@21 | 1@2 | 37 | 1@9 | 1@7 | 62 | 1@0 | 1@23 |
| 30 | 15 | 10+20 | 54 | 1@51 | 1@52 | 98 | 1@25 | 1@70 | 158 | 1@0 | 1@105 |
|  |  | 15+15 | 51 | 1@53 | 1@24 | 95 | 1@27 | 1@41 | 158 | 1@0 | 1@78 |
|  |  | 20+10 | 54 | 1@53 | 1@0 | 98 | 1@23 | 1@14 | 158 | 1@0 | 1@51 |
|  | 30 | 10+20 | 49 | 1@24 | 1@23 | 47 | 1@13 | 1@35 | 75 | 1@0 | 1@50 |
|  |  | 15+15 | 47 | 1@25 | 1@9 | 45 | 1@12 | 1@18 | 76 | 1@0 | 1@37 |
|  |  | 20+10 | 47 | 1@29 | 1@0 | 47 | 1@12 | 1@7 | 75 | 1@0 | 1@23 |
| 35 | 15 | 15+20 | 62 | 1@60 | 1@42 | 113 | 1@31 | 1@64 | 185 | 1@0 | 1@105 |
|  |  | 20+15 | 62 | 1@61 | 1@16 | 114 | 1@31 | 1@38 | 185 | 1@0 | 1@78 |
|  | 30 | 15+20 | 57 | 1@29 | 1@18 | 54 | 1@15 | 1@30 | 89 | 1@0 | 1@50 |
|  |  | 20+15 | 57 | 1@29 | 1@5 | 54 | 1@14 | 1@16 | 89 | 1@0 | 1@37 |
| 40 | 15 | 10+30 | 53 | 1@52 | 1@52 | 130 | 1@36 | 1@113 | 212 | 1@0 | 1@159 |
|  |  | 20+20 | 70 | 1@71 | 1@34 | 129 | 1@36 | 1@58 | 212 | 1@0 | 1@105 |
|  |  | 30+10 | 52 | 1@110 | 1@1 | 130 | 1@39 | 1@8 | 212 | 1@0 | 1@51 |
|  | 30 | 10+30 | 66 | 1@34 | 1@42 | 62 | 1@17 | 1@54 | 102 | 1@0 | 1@77 |
|  |  | 20+20 | 65 | 1@34 | 1@14 | 62 | 1@17 | 1@27 | 102 | 1@0 | 1@50 |
|  |  | 30+10 | 46 | 1@56 | 1@0 | 63 | 1@18 | 1@2 | 102 | 1@0 | 1@23 |
| 45 | 15 | 15+30 | 79 | 1@79 | 1@78 | 146 | 1@44 | 1@110 | 239 | 1@0 | 1@159 |
|  | 30 | 15+30 | 74 | 1@38 | 1@36 | 70 | 1@20 | 1@51 | 116 | 1@0 | 1@77 |
| 50 | 15 | 10+40 | 57 | 1@52 | 1@56 | 165 | 1@48 | 1@160 | 268 | 1@0 | 1@215 |
|  |  | 20+30 | 87 | 1@89 | 1@69 | 161 | 1@47 | 1@101 | 266 | 1@0 | 1@159 |
|  |  | 30+20 | 87 | 1@88 | 1@14 | 161 | 1@47 | 1@47 | 266 | 1@0 | 1@105 |
|  |  | 40+10 | 55 | 1@162 | 1@0 | 165 | 1@53 | 1@1 | 268 | 1@0 | 1@51 |
|  | 30 | 10+40 | 50 | 1@24 | 1@24 | 79 | 1@21 | 1@75 | 130 | 1@0 | 1@105 |
|  |  | 20+30 | 82 | 1@43 | 1@32 | 78 | 1@22 | 1@48 | 129 | 1@0 | 1@77 |
|  |  | 30+20 | 82 | 1@43 | 1@5 | 78 | 1@22 | 1@21 | 129 | 1@0 | 1@50 |
|  |  | 40+10 | 48 | 1@83 | 1@0 | 79 | 1@28 | 1@0 | 130 | 1@0 | 1@23 |
| 55 | 15 | 15+40 | 84 | 1@79 | 1@83 | 182 | 1@54 | 1@156 | 295 | 1@0 | 1@215 |
|  | 30 | 15+40 | 78 | 1@38 | 1@38 | 89 | 1@27 | 1@77 | 144 | 1@0 | 1@105 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | RB allocation (Inner) |  |  | RB allocation (Outer1) |  |  | RB allocation (Outer2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
| 60 | 15 | 10+50 | 108 | 1@52 | 1@54 | 192 | 1@49 | 1@188 | 322 | 1@0 | 1@269 |
|  |  | 20+40 | 107 | 1@106 | 1@106 | 195 | 1@57 | 1@145 | 322 | 1@0 | 1@215 |
|  |  | 30+30 | 104 | 1@107 | 1@50 | 193 | 1@58 | 1@90 | 320 | 1@0 | 1@159 |
|  |  | 40+20 | 105 | 1@112 | 1@0 | 195 | 1@55 | 1@33 | 322 | 1@0 | 1@105 |
|  |  | 50+10 | 52 | 1@220 | 1@0 | 189 | 1@82 | 1@0 | 322 | 1@0 | 1@51 |
|  | 30 | 10+50 | 49 | 1@24 | 1@23 | 93 | 1@22 | 1@90 | 157 | 1@0 | 1@132 |
|  |  | 20+40 | 100 | 1@51 | 1@49 | 96 | 1@29 | 1@73 | 157 | 1@0 | 1@105 |
|  |  | 30+30 | 99 | 1@52 | 1@22 | 94 | 1@28 | 1@43 | 156 | 1@0 | 1@77 |
|  |  | 40+20 | 96 | 1@59 | 1@0 | 96 | 1@28 | 1@17 | 157 | 1@0 | 1@50 |
|  |  | 50+10 | 45 | 1@112 | 1@0 | 92 | 1@42 | 1@0 | 157 | 1@0 | 1@23 |
| 70 | 15 | 20+50 | 107 | 1@106 | 1@106 | 229 | 1@69 | 1@191 | 376 | 1@0 | 1@269 |
|  |  | 30+40 | 126 | 1@121 | 1@89 | 229 | 1@69 | 1@137 | 376 | 1@0 | 1@215 |
|  |  | 40+30 | 126 | 1@126 | 1@35 | 229 | 1@67 | 1@79 | 376 | 1@0 | 1@159 |
|  |  | 50+20 | 106 | 1@165 | 1@0 | 229 | 1@67 | 1@25 | 376 | 1@0 | 1@105 |
|  | 30 | 20+50 | 100 | 1@51 | 1@49 | 112 | 1@33 | 1@93 | 184 | 1@0 | 1@132 |
|  |  | 30+40 | 122 | 1@61 | 1@43 | 112 | 1@33 | 1@66 | 184 | 1@0 | 1@105 |
|  |  | 40+30 | 122 | 1@62 | 1@16 | 112 | 1@32 | 1@37 | 184 | 1@0 | 1@77 |
|  |  | 50+20 | 98 | 1@85 | 1@0 | 112 | 1@32 | 1@10 | 184 | 1@0 | 1@50 |
| 80 | 15 | 30+50 | 143 | 1@143 | 1@125 | 261 | 1@80 | 1@180 | 430 | 1@0 | 1@269 |
|  |  | 40+40 | 142 | 1@144 | 1@69 | 260 | 1@77 | 1@120 | 432 | 1@0 | 1@215 |
|  |  | 50+30 | 143 | 1@144 | 1@16 | 261 | 1@78 | 1@68 | 430 | 1@0 | 1@159 |
|  | 30 | 30+50 | 138 | 1@70 | 1@60 | 128 | 1@39 | 1@88 | 211 | 1@0 | 1@132 |
|  |  | 40+40 | 137 | 1@71 | 1@32 | 128 | 1@39 | 1@60 | 212 | 1@0 | 1@105 |
|  |  | 50+30 | 138 | 1@71 | 1@6 | 128 | 1@38 | 1@32 | 211 | 1@0 | 1@77 |
| 90 | 15 | 40+50 | 161 | 1@163 | 1@107 | 294 | 1@89 | 1@166 | 486 | 1@0 | 1@269 |
|  |  | 50+40 | 161 | 1@162 | 1@52 | 294 | 1@89 | 1@112 | 486 | 1@0 | 1@215 |
|  | 30 | 40+50 | 156 | 1@80 | 1@51 | 144 | 1@43 | 1@80 | 239 | 1@0 | 1@132 |
|  |  | 50+40 | 156 | 1@80 | 1@24 | 144 | 1@43 | 1@53 | 239 | 1@0 | 1@105 |
| 100 | 15 | 50+50 | 178 | 1@180 | 1@87 | 326 | 1@101 | 1@156 | 540 | 1@0 | 1@269 |
|  | 30 | 20+80 | 99 | 1@51 | 1@48 | 160 | 1@48 | 1@156 | 268 | 1@0 | 1@216 |
|  |  | 50+50 | 173 | 1@89 | 1@41 | 160 | 1@49 | 1@75 | 266 | 1@0 | 1@132 |
|  |  | 80+20 | 95 | 1@171 | 1@0 | 159 | 1@59 | 1@0 | 268 | 1@0 | 1@50 |
| 105 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+90 | 76 | 1@38 | 1@37 | 162 | 1@35 | 1@158 | 283 | 1@0 | 1@244 |
|  |  | 25+80 | 127 | 1@65 | 1@62 | 171 | 1@54 | 1@159 | 282 | 1@0 | 1@216 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | RB allocation (Inner) |  |  | RB allocation (Outer1) |  |  | RB allocation (Outer2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
| 110 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 10+100 | 48 | 1@24 | 1@23 | 162 | 1@22 | 1@159 | 297 | 1@0 | 1@272 |
|  |  | 20+90 | 97 | 1@51 | 1@47 | 175 | 1@49 | 1@172 | 296 | 1@0 | 1@244 |
|  |  | 30+80 | 154 | 1@78 | 1@76 | 178 | 1@56 | 1@155 | 295 | 1@0 | 1@216 |
|  |  | 40+70 | 194 | 1@99 | 1@89 | 178 | 1@55 | 1@126 | 295 | 1@0 | 1@188 |
|  |  | 50+60 | 192 | 1@98 | 1@60 | 177 | 1@55 | 1@98 | 295 | 1@0 | 1@161 |
|  |  | 60+50 | 192 | 1@99 | 1@32 | 180 | 1@56 | 1@73 | 295 | 1@0 | 1@132 |
|  |  | 70+40 | 194 | 1@98 | 1@5 | 178 | 1@55 | 1@43 | 295 | 1@0 | 1@105 |
|  |  | 80+30 | 152 | 1@142 | 1@0 | 178 | 1@55 | 1@15 | 295 | 1@0 | 1@77 |
|  |  | 90+20 | 95 | 1@199 | 1@0 | 173 | 1@74 | 1@1 | 296 | 1@0 | 1@50 |
|  |  | 100+10 | 52 | 1@248 | 1@0 | 160 | 1@115 | 1@1 | 297 | 1@0 | 1@23 |
| 115 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+100 | 76 | 1@38 | 1@37 | 176 | 1@35 | 1@172 | 311 | 1@0 | 1@272 |
|  |  | 25+90 | 127 | 1@65 | 1@62 | 186 | 1@58 | 1@178 | 310 | 1@0 | 1@244 |
|  |  | 90+25 | 123 | 1@185 | 1@0 | 187 | 1@59 | 1@0 | 310 | 1@0 | 1@64 |
|  |  | 100+15 | 74 | 1@237 | 1@0 | 176 | 1@98 | 1@0 | 311 | 1@0 | 1@37 |
| 120 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 20+100 | 99 | 1@51 | 1@48 | 189 | 1@49 | 1@186 | 324 | 1@0 | 1@272 |
|  |  | 30+90 | 154 | 1@78 | 1@76 | 195 | 1@61 | 1@177 | 323 | 1@0 | 1@244 |
|  |  | 40+80 | 208 | 1@106 | 1@103 | 195 | 1@60 | 1@148 | 323 | 1@0 | 1@216 |
|  |  | 50+70 | 211 | 1@107 | 1@78 | 194 | 1@60 | 1@120 | 322 | 1@0 | 1@188 |
|  |  | 60+60 | 211 | 1@108 | 1@50 | 194 | 1@60 | 1@91 | 324 | 1@0 | 1@161 |
|  |  | 70+50 | 211 | 1@107 | 1@22 | 194 | 1@61 | 1@65 | 322 | 1@0 | 1@132 |
|  |  | 80+40 | 206 | 1@115 | 1@0 | 195 | 1@61 | 1@38 | 323 | 1@0 | 1@105 |
|  |  | 90+30 | 152 | 1@170 | 1@0 | 195 | 1@61 | 1@10 | 323 | 1@0 | 1@77 |
|  |  | 100+20 | 95 | 1@227 | 1@0 | 187 | 1@88 | 1@1 | 324 | 1@0 | 1@50 |
|  | 15 | N/A |  |  |  |  |  |  |  |  |  |
| 125 | 30 | 25+100 | 127 | 1@65 | 1@62 | 203 | 1@63 | 1@200 | 338 | 1@0 | 1@272 |
|  |  | 100+25 | 123 | 1@213 | 1@0 | 201 | 1@74 | 1@1 | 338 | 1@0 | 1@64 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | RB allocation (Inner) |  |  | RB allocation (Outer1) |  |  | RB allocation (Outer2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
| 130 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 30+100 | 152 | 1@78 | 1@75 | 211 | 1@67 | 1@199 | 351 | 1@0 | 1@272 |
|  |  | 40+90 | 208 | 1@106 | 1@103 | 211 | 1@66 | 1@170 | 351 | 1@0 | 1@244 |
|  |  | 50+80 | 229 | 1@117 | 1@97 | 211 | 1@66 | 1@143 | 350 | 1@0 | 1@216 |
|  |  | 60+70 | 230 | 1@117 | 1@69 | 211 | 1@66 | 1@141 | 351 | 1@0 | 1@188 |
|  |  | 70+60 | 230 | 1@117 | 1@42 | 211 | 1@66 | 1@87 | 351 | 1@0 | 1@161 |
|  |  | 80+50 | 229 | 1@117 | 1@13 | 211 | 1@66 | 1@59 | 350 | 1@0 | 1@132 |
|  |  | 90+40 | 206 | 1@143 | 1@0 | 211 | 1@66 | 1@31 | 351 | 1@0 | 1@105 |
|  |  | 100+30 | 150 | 1@199 | 1@0 | 211 | 1@67 | 1@4 | 351 | 1@0 | 1@77 |
| 140 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 40+100 | 210 | 1@106 | 1@104 | 228 | 1@71 | 1@192 | 379 | 1@0 | 1@272 |
|  |  | 50+90 | 249 | 1@126 | 1@116 | 228 | 1@71 | 1@165 | 378 | 1@0 | 1@244 |
|  |  | 60+80 | 250 | 1@127 | 1@89 | 228 | 1@72 | 1@137 | 379 | 1@0 | 1@216 |
|  |  | 70+70 | 247 | 1@126 | 1@59 | 227 | 1@72 | 1@109 | 378 | 1@0 | 1@188 |
|  |  | 80+60 | 248 | 1@126 | 1@32 | 228 | 1@72 | 1@82 | 379 | 1@0 | 1@161 |
|  |  | 90+50 | 249 | 1@126 | 1@4 | 229 | 1@73 | 1@56 | 378 | 1@0 | 1@132 |
|  |  | 100+40 | 208 | 1@170 | 1@0 | 229 | 1@73 | 1@28 | 379 | 1@0 | 1@105 |
| 150 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 50+100 | 263 | 1@133 | 1@130 | 245 | 1@78 | 1@189 | 406 | 1@0 | 1@272 |
|  |  | 60+90 | 268 | 1@136 | 1@107 | 245 | 1@77 | 1@159 | 407 | 1@0 | 1@244 |
|  |  | 70+80 | 267 | 1@135 | 1@78 | 244 | 1@77 | 1@131 | 406 | 1@0 | 1@216 |
|  |  | 80+70 | 267 | 1@135 | 1@50 | 244 | 1@77 | 1@103 | 406 | 1@0 | 1@188 |
|  |  | 90+60 | 268 | 1@135 | 1@23 | 245 | 1@77 | 1@76 | 407 | 1@0 | 1@161 |
|  |  | 100+50 | 263 | 1@143 | 1@0 | 245 | 1@79 | 1@50 | 406 | 1@0 | 1@132 |
| 160 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 60+100 | 288 | 1@145 | 1@125 | 262 | 1@84 | 1@183 | 435 | 1@0 | 1@272 |
|  |  | 70+90 | 287 | 1@145 | 1@98 | 262 | 1@84 | 1@156 | 434 | 1@0 | 1@244 |
|  |  | 80+80 | 287 | 1@145 | 1@69 | 262 | 1@84 | 1@128 | 434 | 1@0 | 1@216 |
|  |  | 90+70 | 287 | 1@145 | 1@42 | 262 | 1@84 | 1@100 | 434 | 1@0 | 1@188 |
|  |  | 100+60 | 288 | 1@145 | 1@15 | 261 | 1@83 | 1@70 | 435 | 1@0 | 1@161 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | RB allocation (Inner) |  |  | RB allocation (Outer1) |  |  | RB allocation (Outer2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
| 170 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 70+100 | 305 | 1@154 | 1@116 | 278 | 1@88 | 1@176 | 462 | 1@0 | 1@272 |
|  |  | 80+90 | 304 | 1@154 | 1@88 | 277 | 1@88 | 1@147 | 462 | 1@0 | 1@244 |
|  |  | 90+80 | 304 | 1@154 | 1@60 | 278 | 1@90 | 1@122 | 462 | 1@0 | 1@216 |
|  |  | 100+70 | 305 | 1@154 | 1@32 | 278 | 1@88 | 1@92 | 462 | 1@0 | 1@188 |
| 180 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 80+100 | 323 | 1@163 | 1@106 | 295 | 1@94 | 1@171 | 490 | 1@0 | 1@272 |
|  |  | 90+90 | 322 | 1@164 | 1@79 | 294 | 1@94 | 1@142 | 490 | 1@0 | 1@244 |
|  |  | 100+80 | 325 | 1@164 | 1@52 | 295 | 1@94 | 1@115 | 490 | 1@0 | 1@216 |
| 190 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 90+100 | 342 | 1@173 | 1@98 | 311 | 1@99 | 1@164 | 518 | 1@0 | 1@272 |
|  |  | 100+90 | 342 | 1@173 | 1@70 | 311 | 1@100 | 1@137 | 518 | 1@0 | 1@244 |
| 200 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 100+100 | 360 | 1@182 | 1@88 | 328 | 1@105 | 1@159 | 546 | 1@0 | 1@272 |
| NOTE 1: BWchannel_CA is defined in clause 5.3A.3 NOTE 2: NRB_alloc for intra-band contiguous UL CA is defined in 6.2A.2.0 |  |  |  |  |  |  |  |  |  |  |  |

<End of Changes>