3GPP TSG-RAN WG5 Meeting #100	R5-235189

Toulouse, France, 21st August 2023-25th August 2023

Title:	Delays in IoT NTN communications

Source:	Keysight Technologies UK Ltd

Agenda item:	5.3.33.9

Document for:	Discussion and endorsement

# 1.	Introduction

3GPP has been discussing the delays present in Non-Terrestrial Networks Communications and the responsibilities of compensating such effect to enable the communication ([1], [2]).

This document analyses the outcome of such discussions and proposes a way forward to progress on test specifications implications related to this topic.

This document is a re-submission from R5-233757 ([13]).

# 2. 	Discussion

## 2.1 Dynamic delay in service and feeder links

According to 3GPP TS 36.300 section 23.21.2.2 ([2]), in Non-Terrestrial Networks communications there will be delays associated either to the service link as well as to the feeder link (refer to Figure 1). DL and UL are frame aligned at the uplink time synchronization reference point (RP) with an offset given by NTA,offset .

Figure 1: Timing relationship parameters

To enable the communications, pre-compensation is required by the UE:

The following parameters will play a key role on achieving proper timing and synchronization:

-	Common TA is a configured offset that corresponds to the RTT between the Reference Point (RP) and the NTN payload ([2]).

According to 3GPP TS 36.211 ([3]) section 8.1 Uplink frame number i for transmission for the UE shall start at TTA before the start of the corresponding downlink frame at the UE:

Additionally, according to 3GPP TS 36.213 ([4]) section 4.2.3:

-	Koffset is a configured scheduling offset that need to be larger or equal to the sum of the service link RTT and the Common TA ([2]).

-	kmac is a configured offset that need to be larger or equal to the RTT between the RP and the gNB ([2]).

All these values are provided by the network in the specific SIB message for NTN (SIB31).

Additionally, these parameters are somehow related to the satellite orbit emulation, i.e. to the ephemeris of the satellite. In the test cases it is important that TE emulates appropriate delays in both feeder and service links according to information in NTN SIB message (SIB31).

Proposal 1: Test equipment shall emulate the delay in both feeder and service links according to information in NTN SIB message.

## 2.2 Kmac and Common TA parameters

As described in section 2.1 in this document, DL and UL are frame aligned at the uplink time synchronization reference point (RP) with an offset given by NTA,offset .

Hence, there are two options for the definition of the reference point (RP):

- Option 1: DL and UL frame boundaries aligned at gNB (kmac=0, koffset=2xDelay)

- Option 2: DL and UL not aligned at gNB (kmac≠0, Koffset≠2xDelay)

Considering that:

- kmac needs to be larger than or equal to the RTT between the RP and the gNB (as per 3GPP TS 36.300 section 23.21.2.1 ([2])), having a value larger than RTT between the RP and the gNB would require a consensus on how larger kmac needs to be compared to the RTT between the RP and the gNB,

- According to R4-2214473 ([5]), during RRM discussions for NR NTN, it was indicated that “In order to alleviate the impact of uncertainty on UE uplink transmission timing due to quantized feeder link delay information, the feeder link delay can be set to zero.” so the agreement in RRM test cases was to use kmac as not configured, ta-Common=ta-CommonDrift=ta-CommonDriftVariant=0 (as per R4-2303147 ([6])). This is also applicable to IoT NTN.

As the same uncertainty impact due to quantized feeder link delay might impact all test cases, it is proposed to define reference point so DL and UL frame boundaries are aligned at gNB (kmac=0 or not configured) and ta-Common=ta-CommonDrift=ta-CommonDriftVariant=0. ta-Report seems to be not needed.

Proposal 2: Define Reference point so DL and UL frame boundaries are aligned at gNB (kmac=0 or not configured).

Proposal 3: Define ta-Common=ta-CommonDrift=ta-CommonDriftVariant=0.

Proposal 4: Define ta-Report as Not configured.

## 2.3 Koffset

According to 3GPP TS36.300 ([2]), koffset is a configured scheduling offset that need to be larger or equal to the sum of the service link RTT and the Common TA. Having kmac=0 implies that koffset should be equal to or greater than twice of the satellite-UE one-way delay.

koffset (= kcell,offset - kUE,offset) , where kcell,offset is provided by cellSpecificKoffset and kUE,offset is provided by a Differential koffset MAC CE command; otherwise, if not respectively provided, kcell,offset = 0 or kUE,offset = 0 according to 3GPP TS 36.213 ([4]) section 4.2.3.

During RAN4 discussions, one analysis on the Satellite-UE RTT delay and associated koffset values was carried out (R4-2213860 ([7])) as shown below:

As it can be seen in the above analysis, koffset value is dependent on the type of satellite as well as on the minimum elevation angle (α in the figure above). Even when this reference was discussed for NR NTN, it is also applicable to IoT NTN.

In RRM, according to R4-2306370 ([8]), the following agreement was achieved:

Hence, 30º minimum elevation angle applies and this leads to same conclusions as in [6]):

R4-2119586 ([9]) and R4-2218407 ([10]) describe somehow the trade-off required between satellite visibility duration and performance (SNR/RSRP/SINR) (applicable to IoT NTN too):

In demod test cases, as only PDSCH for NGSO is being defined, the koffset value agreed was also 8 slots (refer to R4-2220278 Issue 1-1-1 and Issue 1-2-4 ([11])).

Assuming kUE,offset could be agreed as 0 (same as done in NR NTN [12], , it is proposed that same kcell,offset values agreed in demod and RRM are used for all test cases: 256 slots for GSO and 8 slots for NGSO.

Value for NGSO might require further updates for NGSO satellites in case it is necessary to maximize satellite visibility duration in some RF test cases.

Proposal 5: Use kcell,offset equal to 256 slots for GSO and 8 slots for NGSO in all test cases.

# 3. 	Conclusion

This document analyses discussions in different 3GPP working groups regarding the propagation delays in Non-Terrestrial Network communications and proposes a way forward to progress on test specifications implications related to this topic.

The following proposals are made:

Proposal 1: Test equipment shall emulate the delay in both feeder and service links according to information in NTN SIB message.

Proposal 2: Define Reference point so DL and UL frame boundaries are aligned at gNB (kmac=0 or not configured).

Proposal 3: Define ta-Common=ta-CommonDrift=ta-CommonDriftVariant=0.

Proposal 4: Define ta-Report as Not configured.

Proposal 5: Use kcell,offset equal to 256 slots for GSO and 8 slots for NGSO in all test cases.

# 4. 	References

- 3GPP TS 36.306, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities (Release 17)”

- 3GPP TS 36.300, “Evolved Universal Terrestrial Radio Access (E-UTRA); and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 17)”

- 3GPP TS 36.211, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation (Release 17)”

- 3GPP TS 36.213, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures (Release 17)”

- R4-2214473, “WF on performance part for NR NTN”, Xiaomi, RAN4#104-e

- R4-2303147, “WF on Rel-17 NR NTN RRM Performance requirements”, Xiaomi, RAN4#106

- R4-2213860, “Discussion on UE NTN demod PDSCH”, Huawei, HiSilicon, RAN4#104-e

- R4-2306370, “WF on NB-IoT/eMTC RRM requirements for NTN”, Moderator (MediaTek inc.), RAN4#106bis-e

- R4-2119586, “General and RRM requirements impacts”, Qualcomm Incorporated, RAN4#101-e

- R4-2218407, “Open Issues in UE and Satellite position details”, RAN4#105

- R4-2220278, “WF on IoT-NTN UE demodulation and CQI reporting requirements”, MediaTek, RAN4#105

- R4-2300775, “Discussion on RRM test parameter configuration for NTN”, CMCC, RAN4#106

- R5-233757, “Delays in IoT NTN communications”, Keysight Technologies UK Ltd, RAN5#99