3GPP TSG-RAN5 Meeting #100	R5-235875

# Toulouse, France, Aug 21-25, 2023

Agenda item:	5.3.17.9

Source: 	Qualcomm Inc

Title: 	DL1024QAM PDSCH simulation results

Document for:	Discussion and Endorsement

## 1.	Introduction

[3] introduced a new fading propagation model TDLD-5 for DL1024QAM PDSCH Demodulation test cases based on the requirements defined in [4]. Some of the key characteristics for this model is a dominant LOS path and a very low doppler of 5Hz. This paper discusses the simulation results to derive the minimum test time for running these DL1024QAM test cases.

## 2.	Discussion

From [1] and [2], we can recall the procedure used to come up with the minimum test time for PDSCH throughput test cases.

- Run the PDSCH simulations for an extended period of time at the SNR of interest for multiple seeds.

- The minimum test time is determined by the time it took the throughput curve to settle within +/- 2% of the target throughput value across all seeds.

- If the throughput does not settle within the +/- 2% throughput limit within a reasonable amount of time for any seed, expand the threshold to +/- 3.5% of the target throughput value.

- To account for the additional 1.5% throughput variation, determine from the Throughput vs SNR curve, what is the SNR delta corresponding to this 1.5 % throughput increase.

- That SNR value is the added SNR uncertainty due to finite test time.

PDSCH simulations for all the defined DL1024QAM test cases in [3] were run.

Table 5.2.2.1.1.0-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-8 | R.PDSCH.1-17.1 FDD | 10 / 15 | 1024QAM, 0.79 | TDLD30-5 | 2x2, ULA Low | 70 | 29.5 |

Table 5.2.2.1.1_3.4-1: Test Requirements for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-8 | R.PDSCH.1-17.1 FDD | 10 / 15 | 1024QAM, 0.79 | TDLD30-5 | 2x2, ULA Low | 70 | 29.5+TT |

Table 5.2.2.2.1.0-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-12 | R.PDSCH.2-25.1 TDD | 40 / 30 | 1024QAM, 0.79 | FR1.30-1 | TDLD30-5 | 2x2, ULA Low | 70 | 29.4 |

Table 5.2.2.2.1_3.4-1: Test requirement for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-12 | R.PDSCH.2-25.1 TDD | 40 / 30 | 1024QAM, 0.79 | FR1.30-1 | TDLD30-5 | 2x2, ULA Low | 70 | 29.4+TT |

Figure1: PDSCH Throughput Simulations for FDD 2Rx at 70% target throughput SNR, 30 seeds

Figure2: PDSCH Throughput Simulations for TDD 2Rx at 70% target throughput SNR, 30 seeds

After correcting the simulation parameter error, the simulations for the 4 scenarios completed up to 50 seconds for all simulated seeds (#30) after running for over 10 days. From the convergence plot, we can see that Throughput across all seeds are slightly above the 2.5% of the target 70% throughput. If we let the simulations longer, we expect the throughput to converge to within the 2% of the target 70% throughput limit.

Due to lack of time in this meeting, we propose to keep a more conservative minimum test time of 100 seconds as the working assumption and revise it by providing more simulation results for the next meeting.

## 3.	Conclusion

Proposal 1: Adopt a conservative minimum test time of [100] seconds as the working assumption for all 4 scenarios. To be finalized by next meeting by providing more simulation results.

## 4.		References

[1]	R5-191250, Discussion paper on statistical tput calculation for demod test cases, Qualcomm

[2]	R5-194687, Discussion on minimum test time for PDSCH and PDCCH with more simulation results, Qualcomm

[3]	TS 38.521-4 v17.3.0

[4]	TS 38.101-4 v17.9.0