Source: 	Huawei, China Unicom

Title: 	Test point analysis for Minimum output power for CA (2UL CA) in FR1

Introduction

As agreed in RAN5#78, each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations) in [1] and [2].

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for Minimum output power for CA (2UL CA) test case in FR1.

Discussion

Minimum output power for CA (2UL CA) Test case is defined as follows:

Table 6.3.2A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/Afor this test | QPSK | 31 | P_6@0 | S_25@0 | - | - |
| 6 | 50 |  | QPSK | 56 | P_6@0 | S_50@0 | - | - |
| 15 | 25 |  | QPSK | 12 | S_4@0 | P_8@0 |  |  |
| 25 | 6 |  | QPSK | 13 | P_8@0 | P_5@0 |  |  |
| 25 | 15 |  | QPSK | 40 | P_25@0 | S_15@0 | - | - |
| 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 25 | 50 |  | QPSK | 75 | P_25@0 | S_50@0 | - | - |
| 25 | 75 |  | QPSK | 24 | P_8@0 | S_16@0 |  |  |
| 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 50 | 100 |  | QPSK | 150 | P_50@0 | S_100@0 | - | - |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 |  |  |
| 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |

For intra-band contiguous UL CA and non-contiguous UL CA, the test in LTE is configured as follows:

Table 6.3.2A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 25 | 50 |  | QPSK | 75 | P_25@0 | S_50@0 |  |  |
| 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 |  |  |
| 75 | 25 |  | QPSK | 100 | P_75@0 | P_25@0 |  |  |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 | - | - |
| 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 |  |  |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |

Table 6.3.2A.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Maximum Wgap |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCC RB allocation | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | N/A | N/A | QPSK | P_25@0 | QPSK | S_25@0 |
| 2 | 100 | 100 | 5 | N/A | N/A | QPSK | P_100@0 | QPSK | S_100@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-3. Note 2: The carrier centre frequency of PCC in the UL operating band is configured closer to the DL operating band. |  |  |  |  |  |  |  |  |  |

Minimum output power for FR1 SC Test case is defined in [3] as follows:

Table 6.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for minimum output power | Modulation | RB allocation (NOTE 1) |
| 1 | test case | DFT-s-OFDM QPSK | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

Test Environment

Based on the baseline agreed in [1], test environment for Minimum output power for CA (2UL CA) in FR1 can reuse the test environment defined in LTE.

Proposal 1: Define Test Environment as ‘Normal, TL/VL, TL/VH, TH/VL, TH/VH’ for Minimum output power for CA (2UL CA) in FR1.

  - Test Frequencies

In LTE, Minimum Output Power for inter-band 2UL CA is tested for Low and High range frequencies. This is reasonable to verify different behaviours of the UE transmitter for different frequencies due to non-linearity based on the frequency. For NR inter-band 2UL CA test case, it is proposed to adopt the same selection of test frequencies.

Proposal 2: Test Low and High Frequencies for Minimum Output Power for inter-band CA (2UL CA) and intra-band contiguous CA (2UL CA) in FR1. Test Low range and High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA.

Test Subcarrier Spacing

We are measuring power in a steady state condition and whereas BW might have an influence, whether it is SCS15, 30 or 60 will not. Thus SCS would not impact Minimum Output Power in any way. It is proposed to choose highest SCS for Minimum Output Power for CA (2UL CA) as the NR SC TC clarified.

Proposal 3: Select Highest SCS supported for UE for Minimum Output Power for CA (2UL CA) measurement in FR1.

Test Channel Bandwidth

In LTE, this test case is tested for Lowest and Highest channel bandwidths for both PCC and SCC. This assumption seems to be reasonable to test different behaviours of the UE transmitter related to different frequency selectivities with different channel bandwidths. We believe this is also reasonable for NR inter-band 2UL CA, although generally we consider highest BW to be the worst case because of possible frequency selectivity and BW limitation issues.

Proposal 4: Test the Lowest and Highest Channel Bandwidth for Minimum Output Power for CA (2UL CA) in FR1.

Modulations

In LTE, only QPSK is tested, which is generating a stable power for testing.

For NR SC test case, only DFT-s-OFDM QPSK is tested. CP-OFDM has higher PAPR thus is not selected. It is proposed to also test DFT-s-OFDM QPSK for Minimum output power for CA (2UL CA).

Proposal 5: Test DFT-s-OFDM QPSK in Minimum Output Power for CA (2UL CA).

Uplink and DL Configuration

In LTE, minimum output power for inter-band 2UL CA is tested with maximum uplink allocation, which will generate the lowest power spectrum density. For NR the same configuration can be adopted.

Proposal 6: Use Outer_Full RB allocation in the measurement for CA (2UL CA) in FR1.

Number of test points for General ON/OFF Time Mask measurement for CA (2UL CA)

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 5 | 2 | 2 | 1 | 1 | 20 |

Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 Minimum output power for CA (2UL CA) test case definition:

Proposal 1: Define Test Environment as ‘Normal, TL/VL, TL/VH, TH/VL, TH/VH’ for Minimum output power for CA (2UL CA) in FR1.

Proposal 2: Test Low and High Frequencies for Minimum Output Power for inter-band CA (2UL CA) and intra-band contiguous CA (2UL CA) in FR1. Test Low range and High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA.

Proposal 3: Select Highest SCS supported for UE for Minimum Output Power for CA (2UL CA) measurement in FR1.

Proposal 4: Test the Lowest and Highest Channel Bandwidth for Minimum Output Power for CA (2UL CA) in FR1.

Proposal 5: Test DFT-s-OFDM QPSK in Minimum Output Power for CA (2UL CA).

Proposal 6: Use Outer_Full RB allocation in the measurement for CA (2UL CA) in FR1.

References

[1] R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] R5-180111, Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases

[3] 38.905，NR; Derivation of test points for radio transmission and reception User Equipment (UE) conformance test cases