3GPP TSG-RAN5 Meeting #101	R5-237429

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3250 | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routine maintenance for TS 38.523-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. The MUSIM test cases require new AT commands. 2. The procedure delay with PRACH procedure defined in clause 7.3.7.3 needs to be enhanced to cover the RRC Resume procedure. 3. The RAN5 SIG action point AP#99.02 was raised at RAN5#99 to investigate the feasibility of a generic solution for intra-frequency multi-cells test cases, to prevent/mitigate the inter-cell PDCCH clash for SIB1 & Other SI. 4. The possibility to transmit an UL grant on SCell from reception of SR on PSCell is not documented. 5. Terminology different from 3GPP core specifications is used sometimes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Introduce new AT commands. 2. Added RRCResume in clause 7.3.7.3. 3. Updated SIB1 scheduling depending on the SSB index, i.e. broadcast SIB1 in slot#2 in cells configured with SSB#1 and in slot#1 in cells configured with SSB#0. Updated SI scheduling to use different occasions/slots in the SI-window depending on the SSB index of the cell. Updated UE specific SearchSpace to aggregationLevel 2. Added SIB1 in slot#2 for Half-Duplex FDD operation. 4. The possibility to transmit an UL grant on SCell from reception of SR on PSCell has been added. 5. Terminology aligned with 3GPP core specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. MUSIM test cases can not be fully implemented. 2. Test model will not be complete. 3. Intra-frequency inter-Cell clash will remain. 4.+5. Unclarity will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.2, 5.1.2.1.1, 5.1.2.1.2, 5.1.2.3.1, 5.1.2.3.2, 5.2.1.2, 5.2.1.4, 5.2.2.1, 5.2.2.2.1, 5.2.2.2.2, 5.2.2.2.3, 5.2.2.2.4, 5.2.2.2.5, 5.2.2.4.1, 5.2.2.4.2, 6.1, 7.1.2.1.2, 7.1.2.3.3, 7.3.2.2.2, 7.3.3.2, 7.3.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the outcome of 1 revision of R5-236312. |  |  |  |  |  |  |  |  |

# 5	Test models

## 5.1	EN-DC

### 5.1.1	Layer 3

#### 5.1.1.1	Single NR carrier

Figure 5.1.1.1-1: Test model for EN-DC Layer3 testing (MCG and SCG)

Figure 5.1.1.1-2: Test model for EN-DC Layer3 testing (MCG and split DRB)

Figure 5.1.1.1-3: Test model for EN-DC Layer3 testing (Split SRB(s), DRBs removed for clarity)

The UE is configured in normal mode. On the UE side Ciphering and Integrity (PDCP and NAS) are enabled and header compression is not configured.

On the SS side L1, MAC and RLC (E-UTRA/NR) and PDCP (E-UTRA/NR) are configured in normal way and shall perform all their functions. SRB0, 1 & 2 are configured only in E-UTRA. For SRB0 the DL and UL port is above RLC. For SRB1 and SRB2 the port is above/below the E-UTRA RRC and NAS emulator, which is implemented as a PTC. For DRB, the port is above PDCP. PDCP Ciphering/Integrity is enabled. NAS integrity/Ciphering is enabled.

NOTE:	PDCP (E-UTRA/NR) is always configured in the E-UTRA PTC for SRB1 & 2.

The E-UTRA RRC/NAS emulator for SRB1 and SRB2 shall provide the Ciphering and Integrity functionality for the EPS NAS messages. In UL direction, SS shall report RRC messages, still containing (where appropriate) the secure and encoded NAS message, to the RRC port. In DL, RRC and NAS messages with same timing information shall be embedded in one PDU after integrity and ciphering for NAS messages.

SRB3 is connected directly to the SRB port in the NR PTC/dummy NR RRC/NAS emulator.

The NR PDCP can be configured in either the EUTRA or NR PTC for one or more SRBs or DRBs. If NR PDCP is configured, the bearer can be split. In this case the PDCP is fully configured on the cell upon which the bearer is terminated and the other PTC is configured with a proxy PDCP. Data shall be sent/received only on the PTC upon which the bearer is terminated. The SS shall route data to/from either cell, via the routing information provided.

#### 5.1.1.2	NR carrier aggregation

Figure 5.1.1.2-1: Test model for EN-DC Layer3 CA testing

The EN-DC Layer3 CA test model builds on top of the EN-DC Layer3 test model, with the differences specified hereafter.

On the SS NR side, in the SCG there is one PSCell and one SCell configured:

-	PSCell: The associated SCell is linked to this PSCell,to enable the connection of the SCell MAC layer to the PSCell RLC/PDCP layers for DCCH/DTCH.

-	SCell: Only PHY and MAC layers are configured, and MIB is broadcast.

The UL grants and DL assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception in PSCell over system indication port, if configured.

The NR data routing between the RLC layer of PSCell and the lower layers of either PSCell or SCell shall be provided to/by SS in the common part of the data ASP using the MacBearerRouting field.

In a similar way the reception of PRACH preambles in PSCell / SCell is reported by SS over the same port, if configured.

### 5.1.2	Layer 2

#### 5.1.2.1	PDCP

##### 5.1.2.1.1	Single NR carrier

Figure 5.1.2.1.1-1: Test model for EN-DC PDCP testing (MCG and SCG)

Figure 5.1.2.1.1-2: Test model for EN-DC PDCP testing (MCG and split DRB)

The UE is configured in Test Loop Mode, to loop back the user domain data above PDCP layer. On UE side Ciphering is enabled as null algorithm and header compression is not configured.

Test Loop Mode can be active on SCG DRB as shown in Figure 5.1.2.1.1-1 or active split DRB as shown in Figure 5.1.2.1.1-2.

NOTE: Test loop Mode activation on MCG DRB with NR-PDCP is not considered in Figure 5.1.2.1.1-2.

On the SS NR, Layer 1, MAC and RLC are configured in the normal operation. The PDCP is configured in a special mode, where SS does not add any PCDP headers in DL and/or not remove any PDCP headers in UL directions respectively at DRB port on the NR PTC. The TTCN maintains sequence numbers and state variables for the PDCP layer.

On the SS side the E-UTRA MCG layer 1, MAC, RLC and MCG DRB’s PDCP are configured in normal operation. They shall perform their functions. In case of split DRB, MCG leg’s PDCP layer is configured as Proxy mode, TTCN shall configure E-UTRA for EN-DC PDCP testing only when a Test Loop Mode is active on a split DRB.

The SS shall route DL PDCP PDUs from TTCN to PCell and/or PSCell and SS shall indicate that the UL PDCP PDU is received from PCell or PSCell.

Duplication function:

-	UL SS shall include routing information for each UL PDCP PDU.

The UL grants and DL assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception over system indication port, if configured.

##### 5.1.2.1.2	NR carrier aggregation

Figure 5.1.2.1.2-1: Test model for EN-DC with NR CA duplication PDCP testing (MCG and SCG)

The UE is configured in Test Loop Mode, to loop back the user domain data above PDCP layer. On UE side Ciphering is enabled as null algorithm and header compression is not configured. Test Loop Mode is activated on SCG DRB as shown in Figure 5.1.2.1.2-1.

On the SS side the E-UTRA MCG layer 1, MAC, RLC and MCG DRB’s PDCP are configured in normal operation. They shall perform their functions.

On the NR side, the SS configures in the PSCell, Layer 1, MAC and RLC in the normal operation. The NR PDCP is configured in a special mode, where SS does not add any PDCP headers in DL and/or not remove any PDCP headers in UL directions respectively at DRB port on the NR PTC. The TTCN maintains sequence numbers and state variables for the PDCP layer.

The SS configures DRB j and DRB j+1 on the PSCell, every DRB is connected to an RLC entity. The RLC entity configured on DRB j is linked to the MAC entity on the PSCell, the RLC entity configured on DRB j+1 is linked to the MAC entity on the SCell.

The SS configures SCell with Layer 1, MAC in normal operation. The MAC entity of the SCell is linked to the RLC entity on DRB j+1 of the PSCell.

The NR data routing between the RLC layer of PSCell and the lower layers of either PSCell or SCell shall be provided to/by SS in the common part of the data ASP using the MacBearerRouting field.

The UL grants and DL assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception over system indication port, if configured.

#### 5.1.2.2	RLC

Figure 5.1.2.2-1: Test model for EN-DC RLC AM/UM testing

The PCell is an E-UTRA Cell and PSCell is an NR cell on which testing happens. The UE is registered in E-UTRA, using SRBs 0-2, and configured for EN-DC operation.

This model is suitable for testing both UM/AM mode of operation of DRBs on UE side.

The UE is configured in Test Loop Mode, to loop back the user domain data above PDCP layer. On UE side Ciphering is enabled (since mandatory) but with null ciphering algorithm, which is equivalent to not using ciphering. Header compression is not configured on UE side.

On the SS side, L1 and MAC are configured in the normal way. The RLC of the SCG DRBs is configured in transparent mode. Hence with this configuration PDUs out of SS RLC are same as the SDUs in it. There is no PDCP configured on SS NR PTC side. The ports are directly above RLC.

The PDUs, exchanged between TTCN and SS, shall be the final RLC PDUs consisting of RLC and PDCP headers. TTCN code shall take care in DL of building RLC headers and PDCP headers and in UL handle RLC and PDCP headers. TTCN code shall take care of maintaining sequence numbers and state variables for RLC and PDCP layers. If RLC on UE side is in AM mode, TTCN shall take care of generating polls in DL and responding with RLC control PDUs on reception of UL Poll.

#### 5.1.2.3	MAC

##### 5.1.2.3.1	Single NR carrier

Figure 5.1.2.3.1-1: Test model for EN-DC MAC testing

The UE is configured in Test Loop Mode, to loop back the User Plane data above PDCP layer. On UE side Ciphering is enabled (since Mandatory) but with null ciphering algorithm, which is equivalent to not using ciphering. Header compression is not configured on UE side.

On the SS side the EUTRA MCG is configured as per the EN-DC Layer 3 test model in normal operation. The EN-DC MAC test model expects no split bearers to be configured.

On the SS NR, Layer 1 is configured in the normal way. NR MAC is configured in a special mode, where it does not add any MAC headers in DL and /or not remove any MAC headers in UL directions respectively at DRB port. In this case, the TTCN shall provide the final MAC PDU, including padding. Except for this, the NR MAC layer shall perform all its other functions. SRB3 if present is configured as in Layer 3 test model in normal operation.

On DRBs the NR RLC is configured in transparent mode. Hence with this configuration PDUs out of SS RLC are same as the SDUs in it. There is no NR PDCP configured on SS side. The ports are directly above NR RLC.

There are two different test modes in which NR MAC header addition/removal can be configured:

1.	DL/UL header-transparent mode: no header addition in DL and no header removal in UL.

2.	DL only header-transparent mode: no header addition in DL; UL NR MAC is configured in normal mode to remove MAC header and de-multiplex the MAC SDUs according to the logical channel Ids.

If SS NR MAC is configured in DL/UL header-transparent mode, the PDUs, exchanged at the DRB port between TTCN and SS, shall be the final MAC PDUs consisting of MAC, RLC and PDCP headers. TTCN code shall take care in DL of building MAC header, RLC headers and PDCP headers and in UL handle MAC, RLC and PDCP headers. TTCN code shall take care of maintaining sequence numbers and state variables for RLC and PDCP layers. During testing of multiple DRBs at the UE side, it shall still be possible to configure only one DRB on SS side with configuration in the figure 5.1.2.3-1. Other DRBs are not configured, to facilitate routing of UL MAC PDUs. Multiplexing/de-multiplexing of PDUs meant/from different DRBs shall be performed in TTCN. Since the MAC layer does not evaluate the MAC headers in UL it cannot distinguish between SRB and DRB data in UL. There shall be no SRB3 traffic while MAC is configured in this test mode. The SS MAC shall take care of automatic repetitions/retransmission in UL and DL, based on normal MAC HARQ behaviour.

If SS NR MAC is configured in DL only header-transparent mode, the UL PDUs exchanged at the DRB port between TTCN and SS shall be final RLC PDUs consisting of RLC and PDCP headers. SS shall route these PDUs based on logical channel IDs. In DL, TTCN sends fully encoded MAC PDUs at the DRB port (consisting of MAC, RLC and PDCP headers). In this case TTCN needs to take care of maintaining sequence numbers and state variables for RLC and PDCP layers. Furthermore, in UL and DL the SS MAC layer shall be capable of dealing with SRB3 data (i.e. it shall handle DL RLC PDUs coming from SRB3 RLC layer or de-multiplex UL RLC PDUs to SRB3) as in normal mode. The SS MAC shall take care of automatic repetitions/retransmissions in UL and DL, based on normal MAC HARQ behaviour. TTCN shall ensure that no DL MAC SDUs in normal mode and DL MAC PDUs in test mode are mixed for the same TTI.

The UL grants and DL assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception over system indication port, if configured. In a similar way the reception of PRACH preambles is reported by SS over the same port.

When SS is configured either with pusch-AggregationFactor > 1 or with pusch-RepTypeIndicatorDCI-0-1-r16 set to pusch-RepTypeB and SS NR MAC is configured in DL/UL header-transparent mode, SS shall handle UE automatic subsequent repetitions/retransmissions in UL on the same HARQ process according to TS 38.214[22] clause 6.1.2.1, and in addition SS shall transmit to TTCN every received UL HARQ transmission/retransmission within a bundle as separate MAC PDUs.

##### 5.1.2.3.2	NR carrier aggregation

Figure 5.1.2.3.2-1: Test model for EN-DC MAC CA testing

The EN-DC MAC CA test model builds on top of the EN-DC MAC test model, with the differences specified hereafter.

On the SS NR side, in the SCG there is one PSCell and one SCell configured:

-	PSCell only: On DRBs the NR RLC is configured in transparent mode. Hence with this configuration PDUs out of SS RLC are same as the SDUs in it. There is no NR PDCP configured in the SS. The ports are directly above NR RLC.

-	PSCell / SCell: Layer 1 is configured in the normal way. NR MAC is configured in a special mode, where it does not add any MAC headers in DL and /or not remove any MAC headers in UL directions respectively at DRB port. In this case, the TTCN shall provide the final MAC PDU, including padding. Except for this, the MAC layer shall perform all of its other functions.

The UL grants and DL assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception in PSCell over system indication port, if configured. In a similar way the reception of PRACH preambles in PSCell / SCell is reported by SS over the same port, if configured.

The configuration of UL grant on SR reception (clause 7.1.2.3.1.2.2) shall be possible in SCell. If SCell is not configured as the PUCCH-cell, reception of SR on PSCell shall be the trigger for the UL grant transmission on SCell.

The NR data routing between the RLC layer of PSCell and the lower layers of either PSCell or SCell shall be provided to/by SS in the common part of the data ASP using the MacBearerRouting field.

## 5.2	NR/5GC

### 5.2.1	Layer 3

#### 5.2.1.1	Single NR carrier

Figure 5.2.1.1-1: Test model for NR/5GC Layer3 testing

The UE is configured in normal mode. On the UE side Ciphering and Integrity (PDCP and NAS) are enabled and header compression is not configured.

On the SS side L1, MAC, RLC, PDCP and SDAP are configured in the normal way and shall perform all their functions. For SRB0 the DL and UL port is above RLC. For SRB1 and SRB2 the port is above/below the NR RRC and NAS emulator, which is implemented as a PTC. For DRB, the port is above SDAP. PDCP Ciphering/Integrity is enabled. NAS integrity/Ciphering is enabled.

The RRC/NAS emulator for SRB1 and SRB2 shall provide the Ciphering and Integrity functionality for the 5GC NAS messages. In UL, the SS shall report RRC messages, still containing (where appropriate) the secure and encoded NAS message, to the RRC port. In DL, RRC and NAS messages with same timing information shall be embedded in one PDU after integrity and ciphering of the NAS messages.

#### 5.2.1.2	NR carrier aggregation

Figure 5.2.1.2-1: Test model for NR/5GC Layer3 CA testing

The NR/5GC Layer3 CA test model builds on top of the NR/5GC Layer3 test model, with the differences specified hereafter.

In the SS side, there is one PCell and one SCell configured:

-	PCell: The associated SCell is linked to this PCell to enable the connection of the SCell MAC layer to the PCell RLC/PDCP layers for DCCH/DTCH.

-	 SCell: PHY and MAC layers are configured in normal way. RLC layer is configured only for BCCH/PCCH/CCCH.

The UL grants and DL assignments are configured from TTCN over system control port. The SS reports PUCCH scheduling information reception in PCell over system indication port, if configured.

The NR data routing between the RLC layer of PCell and the lower layers of either PCell or SCell shall be provided to/by SS in the common part of the data ASP using the MacBearerRouting field.

In a similar way the reception of PRACH preambles in PCell / SCell is reported by SS over the same port, if configured.

#### 5.2.1.3	NR/5GC Inter-RAT

##### 5.2.1.3.1	NR/E-UTRA Inter-RAT

Figure 5.2.1.3.1-1: Test model for NR/E-UTRA Inter-RAT testing

This test model is only relevant for NR/5GC to/from E-UTRA/EPC inter-RAT and is not applicable to E-UTRA/5GC.  The model consists of a dual protocol stack: one for NR and one for E-UTRA. The TTCN implementation for NR and E-UTRA functionalities are in separate PTCs.

The SS NR part is the same as the model defined in clause 5.2.1.1 for NR/5GC L3 testing.

The SS E-UTRA part is the same as the model defined in TS 36.523-3 [12] clause 4.2.2 for RRC testing.

The UE is configured in normal mode. Ciphering/Integrity (PDCP and NAS) are enabled, and header compression is not configured.

##### 5.2.1.3.2	NR/UTRAN Inter-RAT

Figure 5.2.1.3.2-1: Test model for NR/UTRAN Inter-RAT testing

The model consists of a dual protocol stack: one for NR and one for UTRAN. The TTCN implementation for NR and UTRAN functionalities are in separate PTCs.

The SS NR part is the same as the model defined in clause 5.2.1.1 for NR/5GC L3 testing.

The SS UTRAN part consisting of PHY, MAC, RLC and PDCP (optionally, if PS user RB established only), are configured in normal mode. They shall perform all of their functions normally. Ciphering is enabled and shall be performed in RLC (AM/UM) and MAC (TM RLC). Integrity is enabled, and SS shall provide RRC emulator for integrity protection calculation and checking and 'Direct transfer' adaptation. Ports are above RLC (CS RAB and SRB0), PDCP (PS RAB) and RRC Emulator (SRB1 to SRB4).

The UE is configured in normal mode. Ciphering/Integrity (PDCP and NAS) are enabled.  Ciphering is enabled in UTRAN and header compression is not configured.

##### 5.2.1.3.3	NR/WLAN Inter-RAT

Figure 5.2.1.3.3-1: Test model for NR/WLAN Inter-RAT testing

The current test model is relevant for NR/5GC and is aimed to simulate solely the presence of the WLAN Access Point (AP) in the testing environment. It can be used by test cases where the UE is only required to measure the Beacon RSSI of the WLAN AP. No data transfer between the UE and the emulated WLAN AP is supported. The TTCN implementation for NR and WLAN functionalities is located in separate PTCs.

The SS NR part is the same as the model defined in clause 5.2.1.1 for NR/5GC L3 testing.

The SS WLAN part is the same as the model defined in TS 36.523-3 [12] clause 4.5 with the following restrictions:

-	WLAN AP shall be configured via SYS and SYS_IND ports. WLAN AP Address Information provided during the configuration should be treated as a bogus data needed for the configuration but not actually used.

-	SYS_EPDG and SYS_EPDG_IND ports of WLAN PTC should not be used for any configuration.

The UE is configured in normal mode. Ciphering/Integrity (PDCP and NAS) are enabled and header compression is not configured.

##### 5.2.1.3.4	NR/GERAN Inter-RAT

Figure 5.2.1.3.4-1: Test model for NR/GERAN Inter-RAT testing

The model consists of dual protocol stack one for NR and one for GERAN. The TTCN implementation for NR and GERAN functionalities are in separate PTCs.

This model may also be used in conjunction with the NR/UTRAN test model, defined in clause 5.2.1.3.2, in order to provide all 3 RATs: NR, UTRAN and GERAN. The TTCN implementation for NR, UTRAN and GERAN functionalities are in separate PTCs.

The SS NR part is the same as the model defined in clause 5.2.1.1 for NR/5GC L3 testing.

The SS GERAN model for GPRS consists of L1, MAC/ RLC and LLC, configured in normal mode. SNDCP may also be configured. If SNDCP is configured, this shall reference the LL Entity by the LLMEId. They shall perform all of their functions normally. Ciphering is enabled and shall be performed in LLC. XIDs shall be sent/received by the TTCN and the TTCN then sends the XID information to the SS using the G_CLLC_XID_Config_REQ ASP. Ports are above RLC (GRR messages), LLC (NAS and Data) and SNDCP (User Data).

The SS GERAN model for GSM consists of L1, L2 (MAC/ RLC), configured in normal mode. They shall perform all of their functions normally. Ciphering is enabled and shall be performed in L1. Ports are above L2.

The UE is configured in normal mode. Ciphering/Integrity (PDCP and NAS) are enabled and header compression is not configured in NR. Ciphering is enabled in GERAN.

#### 5.2.1.4	NR supplementary uplink

Figure 5.2.1.4-1: Test model for NR/5GC Layer3 supplementary uplink testing

The NR/5GC Layer3 supplementary uplink test model builds on top of the NR/5GC Layer3 test model, with the differences specified hereafter.

In the SS side:

-	The NR cell is configured with one uplink (NUL) carrier and one supplementary uplink (SUL) carrier. See NOTE 1.

-	The SUL routing information shall be provided by SS in the common part of the indication ASP when the data is received from the SUL carrier.

-	The SS shall monitor both NUL carrier and SUL carrier for PRACH reception. TTCN checks whether RA Msg1 and RA Msg3 are received from NUL or SUL carrier using the SUL routing information in the indication ASP.

-	The UL grants are configured from TTCN over the system control port for PUSCH transmission on the NUL carrier or the SUL carrier.

NOTE 1:	While TS 38.508-1 [5] defines an SUL carrier as a pseudo-standalone NR Cell 33, in the present test model, the SUL carrier is an additional carrier to the NR Cell operating on the NUL carrier. So NR Cell of identity 33 is not used in TTCN.

#### 5.2.1.5	NR-NR dual connectivity

Figure 5.2.1.5-1: Test model for NR/5GC NR-DC Layer3: MCG and SCG

Figure 5.2.1.5-2: Test model for NR/5GC NR-DC Layer3 testing: MCG and split DRB

Figure 5.2.1.5-3: Test model for NR/5GC NR-DC Layer3 testing: Split SRB(s), DRBs removed for clarity

The NR/5GC Layer3 NR-DC test model builds on top of the NR/5GC Layer3 test model, with the differences specified hereafter:

-	SRB1 and SRB2 are configured only in the PCell,

-	SRB3 can be configured in the PSCell,

-	The NR PDCP can be configured in split for one or more SRBs or DRBs. In this case NR PDCP is configured in one cell and a proxy PDCP is configured in the other cell. The SS shall route data to/from either cell, via the routing information provided.

#### 5.2.1.5A	NR-E-UTRA dual connectivity

Figure 5.2.1.5A-1: Test model for NR/5GC NE-DC Layer3: MCG and SCG

Figure 5.2.1.5A-2: Test model for NR/5GC NE-DC Layer3 testing: MCG and split DRB, E-UTRA terminated

Figure 5.2.1.5A-3: Test model for NR/5GC NE-DC Layer3 testing: MCG and split DRB, NR terminated

Figure 5.2.1.5A-4: Test model for NR/5GC NE-DC Layer3 testing: Split SRB(s), DRBs removed for clarity

The NR/5GC Layer3 NE-DC test model builds on top of the NR/5GC Layer3 test model, with the differences specified hereafter:

-	SRBs 0, 1 and 2 are configured in NR.

-	Only SRB0 is configured in E-UTRA and is connected directly to the SRB port in the E-UTRA RRC/NAS emulator.

-	On the SS side E-UTRA L1, MAC and RLC are configured in normal way and shall perform all their functions.

-	Only NR PDCP is configured in both NR and E-UTRA (E-UTRA PDCP is not used).  This can be configured in split for one or more SRBs or DRBs. In this case NR PDCP is configured in one cell and a proxy PDCP is configured in the other cell. The SS shall route data to/from either cell, via the routing information provided.

-	SDAP can be configured in both NR and E-UTRA.

#### 5.2.1.6	Void

#### 5.2.1.7	Multi-USIM

Figure 5.2.1.7-1: Test model for Multi-USIM testing

The test model consists of 2 instances of the NR PTC and the RRC/NAS Emulator PTC.  These PTCs operate as normal.

A Multiplex PTC receives all messages from the SS and decides, based on the cell id, to which instance of the NR PTCs to forward each message.

The existing IP PTC is extended to include an additional co-ordination port to the 2nd instance of the NR PTC.

2 instances of the IMS PTC are always created: IMS_A is connected to NR5GC_A and IMS_B to NR5GC_B.

The interface to the SS remains the same.

### 5.2.2	Layer 2

#### 5.2.2.1	SDAP

Figure 5.2.2.1-1: Test model for NR/5GC SDAP testing

The UE is configured in Test Loop Mode, to loop back the user domain data above SDAP layer. On UE side Ciphering is enabled with null algorithm and header compression is not configured.

On the SS, Layer 1, MAC, RLC and PDCP are configured in the normal operation. The SDAP is configured in a special mode, where SS does not add any SDAP header in DL and does not remove any SDAP header in UL at the DRB port on the NR PTC. The TTCN code shall take care of the SDAP header handling and of the QoS flow to DRB mapping, i.e. the SS shall route DL SDAP PDUs from TTCN to the corresponding DRB.

The UL grants and DL assignments are configured from TTCN over system control port.

#### 5.2.2.2	PDCP

##### 5.2.2.2.1	Single NR carrier

Figure 5.2.2.2.1-1: Test model for NR/5GC PDCP testing

The UE is configured in Test Loop Mode, to loop back the user domain data above PDCP layer. On UE side Ciphering is enabled as null algorithm and header compression is not configured.

On the SS NR, Layer 1, MAC and RLC is configured in the normal operation.

The PDCP is configured in a special mode, where SS does not add any PDCP headers in DL and/or not remove any PDCP headers in UL directions respectively at DRB port on the NR PTC. The TTCN maintains sequence numbers and state variables for the PDCP layer.

The UL grants and DL assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception over system indication port, if configured.

##### 5.2.2.2.2	NR-NR dual connectivity

Figure 5.2.2.2.2-1: Test model for NR/5GC NR-DC PDCP testing (split DRB)

The UE is configured in Test Loop Mode, to loop back the user domain data above PDCP layer.

On SCG DRB at UE side ciphering is enabled as null algorithm and header compression is not configured.

The SS configures in the First Cell, Layer 1, MAC and RLC in the normal operation. The NR PDCP is configured in a special mode, where SS does not add any PDCP headers in DL and/or not remove any PDCP headers in UL directions respectively at DRB port on the NR PTC. The TTCN maintains sequence numbers and state variables for the PDCP layer.

The SS configures in the Second Cell, Layer 1, MAC and RLC in the normal operation. The NR PDCP is configured as Proxy. DRB data is sent between the NR PDCP Proxy and the NR PDCP.

Note that the First Cell refers to MCG PCell and the Second Cell to SCG PSCell, or vice versa.

The UL grants and DL assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception over system indication port, if configured.

##### 5.2.2.2.3	NR-E-UTRA dual connectivity

Figure 5.2.2.2.3-1: Test model for NE-DC PDCP testing (split DRB)

The UE is configured in Test Loop Mode, to loop back the user domain data above NR PDCP layer.

On UE side Ciphering is enabled as null algorithm and header compression is not configured.

Test Loop Mode can be active on split DRB as shown in Figure 5.2.2.2.3-1.

On the SS NR, SRBs 0,1 and 2 are configured as per Layer3 test model. The Layer1, MAC and RLC of the MCG bearer are configured in normal operation. The PDCP is configured in a special mode, where SS does not add any PDCP headers in DL and/or not remove any PDCP headers in UL directions respectively at DRB port on the NR PTC. The TTCN maintains sequence numbers and state variables for the PDCP layer.

On the SS E-UTRA the Layer1, MAC and RLC of the SCG bearer are configured in normal operation. The PDCP layer of the SCG leg is configured as Proxy mode, TTCN shall configure E-UTRA for NE-DC PDCP testing only when a Test Loop Mode is active on a split DRB.

The SS shall route DL PDCP PDUs from TTCN to PCell and/or PSCell and SS shall indicate that the UL PDCP PDU is received from PCell or PSCell.

The UL grants and DL assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception over system indication port, if configured.

##### 5.2.2.2.4	NR carrier aggregation

Figure 5.2.2.2.4-1: Test model for NR CA PDCP testing

The UE is configured in Test Loop Mode, to loop back the user domain data above PDCP layer.

On UE side Ciphering is enabled as null algorithm and header compression is not configured.

The SS configures in the PCell, Layer 1, MAC and RLC in the normal operation. The NR PDCP is configured in a special mode, where SS does not add any PDCP headers in DL and/or not remove any PDCP headers in UL directions respectively at DRB port on the NR PTC. The TTCN maintains sequence numbers and state variables for the PDCP layer.

The SS is configured with one or two SCell(s), the figure 5.2.2.2.4-1 shows 2 SCells.

The SS configures DRB j, DRB j+1 and DRB j+2 on the PCell, every DRB is connected to an RLC entity. The RLC entity configured on DRB j is linked to the MAC entity on the PCell, the RLC entity configured on DRB j+1 is linked to the MAC entity on the SCell 1 and the DRB j+2 is linked to the MAC entity on the SCell 2.

The SS configures SCell 1 with Layer 1, MAC in normal operation. The MAC entity of each SCell is linked to the RLC entity on DRB j+1 of the PCell.

The SS configures SCell 2 with Layer 1, MAC in normal operation. The MAC entity of each SCell is linked to the RLC entity on DRB j+2 of the PCell.

The NR data routing between the RLC layer of PCell and the lower layers of either PCell or SCell shall be provided to/by SS in the common part of the data ASP using the MacBearerRouting field.

The UL grants and DL assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception over system indication port, if configured.

##### 5.2.2.2.5	NR-NR dual connectivity with NR carrier aggregation

Figure 5.2.2.2.5-1: Test model for NR/5GC NR-DC with NR CA duplication in PDCP testing

The UE is configured in Test Loop Mode, to loop back the user domain data above PDCP layer.

On UE side Ciphering is enabled as null algorithm and header compression is not configured.

The SS configures in the PCell, Layer 1, MAC and RLC in the normal operation.

On the NR side, the SS configures in the PSCell, Layer 1, MAC and RLC in the normal operation. The NR PDCP is configured in a special mode, where SS does not add any PDCP headers in DL and/or not remove any PDCP headers in UL directions respectively at DRB port on the NR PTC. The TTCN maintains sequence numbers and state variables for the PDCP layer.

The SS configures DRB j and DRB j+1 on the PSCell, every DRB is connected to an RLC entity. The RLC entity configured on DRB j is linked to the MAC entity on the PSCell, the RLC entity configured on DRB j+1 is linked to the MAC entity on the SCell.

The SS configure SCell with Layer 1, MAC in normal operation. The MAC entity of the SCell is linked to the RLC entity on DRB j+1 of the PSCell.

The NR data routing between the RLC layer of PSCell and the lower layers of either PSCell or SCell shall be provided to/by SS in the common part of the data ASP using the MacBearerRouting field.

The UL grants and DL assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception over system indication port, if configured.

##### 5.2.2.2.6	Ethernet header compression

EHC testing uses the PDCP test model specified in clause 5.2.2.2.1, with the following differences.

EHC is configured in both directions (DRB created in pre-test conditions).

TTCN implements and operates the EHC compressor and the EHC decompressor. The PDCP layer at the SS handles EHC compressed packets (i.e. EHC full header packets and EHC compressed header packets) to/from the TTCN. These PDUs are each associated with one PDCP SDU.

The UE is expected to be configured with an Ethernet MAC address.

##### 5.2.2.2.7	UL data compression

The test methodology, specified in TS 36.523-3 [12] clause 7.30 for E-UTRA UDC, applies to NR UDC as well, with the following differences.

NR UDC testing uses the PDCP test model specified in clause 5.2.2.2.1.

#### 5.2.2.3	RLC

Figure 5.2.2.3-1: Test model for NR/5GC RLC testing

The UE is registered in NR, using SRBs 0-2, and configured for NR/5GC operation.

This model is suitable for testing both UM/AM mode of operation of DRBs on UE side.

The UE is configured in Test Loop Mode, to loop back the user domain data above PDCP layer. On UE side Ciphering is enabled (since mandatory) but with null ciphering algorithm, which is equivalent to not using ciphering. Header compression is not configured on UE side.

On the SS side, L1 and MAC are configured in the normal way. The RLC of the DRBs is configured in transparent mode. Hence with this configuration PDUs out of SS RLC are same as the SDUs in it. There is no PDCP configured on SS side. The ports are directly above RLC.

The PDUs, exchanged between TTCN and SS, shall be the final RLC PDUs consisting of RLC and PDCP headers. TTCN code shall take care in DL of building RLC headers and PDCP headers and in UL handle RLC and PDCP headers. TTCN code shall take care of maintaining sequence numbers and state variables for RLC and PDCP layers. If RLC on UE side is in AM mode, TTCN shall take care of generating polls in DL and responding with RLC control PDUs on reception of UL Poll.

#### 5.2.2.4	MAC

##### 5.2.2.4.1	Single NR carrier

Figure 5.2.2.4.1-1: Test model for NR/5GC MAC testing

The UE is configured in Test Loop Mode A, to loop back the User Plane data above PDCP layer. On UE side Ciphering is enabled (since Mandatory) but with null ciphering algorithm, which is equivalent to not using ciphering. Header compression is not configured on UE side.

On the SS side the NR MAC test model expects no split bearers to be configured.

On the SS NR, Layer 1 is configured in the normal way. NR MAC is configured in a special mode, where it does not add any MAC headers in DL and /or not remove any MAC headers in UL directions respectively at DRB port. In this case, the TTCN shall provide the final MAC PDU, including padding. Except for this, the NR MAC layer shall perform all its other functions. SRBs are configured as in Layer 3 test model in normal operation.

On DRBs the NR RLC is configured in transparent mode. Hence with this configuration PDUs out of SS RLC are same as the SDUs in it. There is no NR PDCP and SDAP configured on SS side. The ports are directly above NR RLC.

There are two different test modes in which NR MAC header addition/removal can be configured:

1.	DL/UL header-transparent mode: no header addition in DL and no header removal in UL.

2.	DL only header-transparent mode: no header addition in DL; UL NR MAC is configured in normal mode to remove MAC header and de-multiplex the MAC SDUs according to the logical channel Ids.

If SS NR MAC is configured in DL/UL header-transparent mode, the PDUs, exchanged at the DRB port between TTCN and SS, shall be the final MAC PDUs consisting of MAC, RLC and PDCP headers. TTCN code shall take care in DL of building MAC header, RLC headers and PDCP headers and in UL handle MAC, RLC and PDCP headers. TTCN code shall take care of maintaining sequence numbers and state variables for RLC and PDCP layers. During testing of multiple DRBs at the UE side, it shall still be possible to configure only one DRB on SS side with configuration in the figure 5.2.2.4.1-1. Other DRBs are not be configured, to facilitate routing of UL MAC PDUs. Multiplexing/de-multiplexing of PDUs meant/from different DRBs shall be performed in TTCN. Since the MAC layer does not evaluate the MAC headers in UL it cannot distinguish between SRB and DRB data in UL. There shall be no SRB traffic while MAC is configured in this test mode. The SS MAC shall take care of automatic repetitions/retransmission in UL and DL, based on normal MAC HARQ behaviour.

NOTE:	There is no need to handle SDAP headers in TTCN for UL/DL as UE Test loop Mode A is above PDCP.

If SS NR MAC is configured in DL only header-transparent mode, the UL PDUs exchanged at the DRB port between TTCN and SS shall be final RLC PDUs consisting of RLC and PDCP headers. SS shall route these PDUs based on logical channel IDs. In DL, TTCN sends fully encoded MAC PDUs at the DRB port (consisting of MAC, RLC and PDCP headers). In this case TTCN needs to take care of maintaining sequence numbers and state variables for RLC and PDCP layers. Furthermore, in UL and DL the SS MAC layer shall be capable of dealing with SRB data (i.e. it shall handle DL RLC PDUs coming from SRB RLC layers or de-multiplex UL RLC PDUs to SRB's) as in normal mode. The SS MAC shall take care of automatic repetitions/retransmissions in UL and DL, based on normal MAC HARQ behaviour. TTCN shall ensure that no DL MAC SDUs in normal mode and DL MAC PDUs in test mode are mixed for the same TTI.

The UL grants and DL assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception over system indication port, if configured. In a similar way the reception of PRACH preambles is reported by SS over the same port.

When SS is configured either with pusch-AggregationFactor > 1 or with pusch-RepTypeIndicatorDCI-0-1-r16 set to pusch-RepTypeB and SS NR MAC is configured in DL/UL header-transparent mode, SS shall handle UE automatic subsequent repetitions/retransmissions in UL on the same HARQ process according to TS 38.214[22] clause 6.1.2.1, and in addition SS shall transmit to TTCN every received UL HARQ transmission/retransmission within a bundle as separate MAC PDUs.

##### 5.2.2.4.2	NR carrier aggregation

Figure 5.2.2.4.2-1: Test model for NR/5GC MAC CA testing

The NR/5GC MAC CA test model builds on top of the NR/5GC MAC test model, with the differences specified hereafter.

On the SS NR side, there is one PCell and one SCell configured:

-	PCell only: On DRBs the NR RLC is configured in transparent mode. Hence with this configuration PDUs out of SS RLC are same as the SDUs in it. There is no NR PDCP and SDAP configured in the SS. The ports are directly above NR RLC.

-	PCell / SCell: Layer 1 is configured in the normal way. NR MAC is configured in a special mode, where it does not add any MAC headers in DL and /or not remove any MAC headers in UL directions respectively at DRB port. In this case, the TTCN shall provide the final MAC PDU, including padding. Except for this, the MAC layer shall perform all of its other functions.

The UL grants and DL assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception in PCell over system indication port, if configured. In a similar way the reception of PRACH preambles in PCell / SCell is reported by SS over the same port, if configured.

The configuration of UL grant on SR reception (clause 7.1.2.3.1.2.2) shall be possible in SCell. If SCell is not configured as the PUCCH-cell, reception of SR on PSCell shall be the trigger for the UL grant transmission on SCell.

The NR data routing between the RLC layer of PCell and the lower layers of either PCell or SCell shall be provided to/by SS in the common part of the data ASP using the MacBearerRouting field.

<SECTIONS SKIPPED>

# 6	System interface

## 6.1	Upper tester interface

The Upper Tester (UT) interface is the same as specified in TS 36.523-3 [12] clause 5.

In addition, the following MMI commands are defined.

Table 6.1-1: MMI commands

| Command | Parameters |  |
| --- | --- | --- |
|  | Name | Value |
| "CAPABILIBY_ENQUIRY" | "Freq. Band List" | <charstring> |
|  | "Two-Way" | "TRUE" / "FALSE" |
| "CHANGE_RAC_ID" | (none) |  |
| "CLEAR_RSNPN" | (none) |  |
| "NO_PRECONDITIONS" | (none) |  |
| "PERIODICAL_ MEASUREMENT_PSBCH_RSRP" | "Report Interval" | <charstring> |
| "SL_CSI_RS_CONFIGURATION" | "Freq. allocation One antenna Port" | <charstring> |
|  | “First symbol" | <charstring> |
|  | "Latency Bound" | <charstring> |
| "SL_CSI_REPORT" | (none) |  |
| "SNPN_AUTOMATIC" | (none) |  |
| "SNPN_MANUAL" | "PLMN" | <PLMN ID> |
|  | "N_ID" | <N ID> |
| "SNPN_SUBSCRIBER_DATA" | "PLMN" | <PLMN ID> |
|  | "N_ID" | <N ID> |
|  | "ACCESS_ID" | "ACCESS_ID" |
| "UEAI" | "SL-QoS Flow ID" | <charstring> |
| "UNICAST_SLDRB" | "Action" | "ESTABLISH" / "MODIFY" / "RELEASE" |

The following AT commands are also applied in the TTCN.

Table 6.1-2: AT Commands

Command

AT+C5GNSSAI

AT+C5GNSSAIRDP

AT+C5GQOS

AT+CGBRRREQ

AT+CLCCS

AT+CMICO

AT+CNASCREL

AT+CPAGRES

AT+CPAGTCC

AT+CPSDO

AT+CREJPAG

AT commands are referred to TS 27.007 [49].

<SECTIONS SKIPPED>

# 7	Test methods and design considerations

## 7.1	Common aspects

### 7.1.1	Introduction

Clause 7.1 specifies test methods and design considerations that are common to all 5GS deployment options.

### 7.1.2	Physical layer aspects

#### 7.1.2.1	Search spaces and DCI

##### 7.1.2.1.1	Parameters

For each search space there are several parameters specifying the location of this search space in the time and frequency domain.

###### 7.1.2.2.1.1	Search space configuration

| PDCCH monitoring periodicity |  |
| --- | --- |
| Comment/description | slot periodicity in time domain |
| ASN.1 parameter | SearchSpace.monitoringSlotPeriodicityAndOffset |
| Core spec reference | TS 38.213 [21] clause 10.1 |
| PDCCH monitoring offset |  |
| Comment/description | slot offset |
| ASN.1 parameter | SearchSpace.monitoringSlotPeriodicityAndOffset |
| Core spec reference | TS 38.213 [21] clause 10.1 |
| PDCCH monitoring pattern |  |
| Comment/description | first symbol(s) of the control resource set within a slot for PDCCH monitoring; in general a search space starts with first symbol of a slot; the duration is given by the L1 parameter 'CORESET-time-duration' |
| ASN.1 parameter | SearchSpace.monitoringSymbolsWithinSlot |
| Core spec reference | TS 38.213 [21] clause 10.1 |
| CORESET time duration |  |
| Comment/description | Duration of a search space in time domain: 1, 2 or 3 symbols |
| ASN.1 parameter | ControlResourceSet.duration |
| Core spec reference | TS 38.213 [21] clause 10.1 |
| CORESET frequency domain allocation |  |
| Comment/description | Bitmap specifying the frequency domain allocation of a search space NOTE: The allocations needs to fit into the BWP to which the CORESET belongs |
| ASN.1 parameter | ControlResourceSet.frequencyDomainResources |
| Core spec reference | TS 38.213 [21] clause 10.1, TS 38.211 [19] clause 7.3.2.2 |

##### 7.1.2.1.2	PDCCH search spaces

For each configured DL BWP, the TTCN may configure one or several search spaces.

For each search space, TTCN configures the SS with:

-	The configuration of this search space as given to the UE, enabling the SS to determine the PDCCH transmission occasions (same as the UE PDCCH monitoring occasions) and associated CORESET.
For SearchSpaceZero the configuration according to TS 38.213 [21] clauses 10.1 and 13 is mapped to ASN.1 type SearchSpace to configure the SS; for CORESET#0 the configuration according to TS 38.211 [19] clause 7.3.2.2 is mapped to ASN.1 type ControlResourceSet to configure the SS.

-	The CCE aggregation level L that the SS shall use for PDCCH transmission on this search space.
In general an aggregation level of 4 is used for SearchSpaceZero and common search space, aggregation level of 2 is used for UE-specific search space.

-	The priority P of this search space that the SS shall consider in its PDCCH candidate selection algorithm.

-	a value of 0 represents the highest priority, a value of 1 the second highest priority and so on.

NOTE:	“Search space” is used in terms of TS 38.213 [21] clause 10.1 and a single search space configuration (ASN.1 type ‘SearchSpace’) may contain several search spaces (see NR_BWP_SearchSpaceConfig_Type in Annex D).

<SECTIONS SKIPPED>

##### 7.1.2.3.3	Default grants

In general test cases for layer 3 and above do not need test case specific grants but use default grants as listed in table 7.1.2.3.3-1.

Table 7.1.2.3.3-1: Default grants

| Purpose | Grant size (NOTE 1) or {LRBs IMCS} pair | Comment |
| --- | --- | --- |
| Scheduling of RA Msg3: for initial access | 256 bits (NOTE 2) | grant provided by RAR (RA Msg2) |
| Scheduling of RA Msg3: for handover, EN-DC or RRC re-establishment procedure | 48 bits (NOTE 2, 3) | grant provided by RAR (RA Msg2) |
| RA Msg4 for C-RNTI based CBRA: for synchronisation (e.g. EN-DC) | LRBs = 1, IMCS = 0 | minimum grant (24 or 32 bits) |
| RA Msg4 for C-RNTI based CBRA: for handover or RRC re-establishment procedure | 144 bits (NOTE 4) |  |
| Default USS grant | LRBs = 24, IMCS = 9 (NOTE 5) | default grant: e.g. for test cases with main focus on control plane signalling |
| NOTE 1: In general for a given grant size TTCN uses the {LRBs IMCS} pair according to annex B.2. NOTE 2: According to TS 38.213 [21] clause 8.3, the RAR UL grant is always interpreted according to DCI format 0_0, so contents of Table 7.1.2.3.2.2-1 apply. NOTE 3: Minimum grant which can be assigned by the RAR: The grant is sufficient to convey C-RNTI (3 bytes) and short BSR (2 bytes) or long BSR (minimum of 3 bytes). Even with short BSR there is not enough space to convey any segment of the RRCReconfigurationComplete or RRCReestablishmentComplete (at least 5 bytes: 2 bytes MAC header + 2 bytes RLC header + 1 byte payload). NOTE 4: Initial grant of RA procedure big enough to completely convey the RRCReconfigurationComplete (10 bits) or RRCReestablishmentComplete (<10 bits). This requires a minimum of 12 bytes (2 bytes MAC header + 2 bytes RLC header + 6 bytes PDCP header + 2 bytes payload). Additionally, an optional PHR MAC element (3 bytes) needs to be considered since the PHR has higher priority than the MAC SDU. Any further user data would require a minimum of 6 additional bytes for a non-RedCap UE (2 bytes MAC header + 3 bytes RLC header + 1 byte payload, it is assumed that if UE has any data to transmit that will be on AM DRB) or 5 additional bytes for a RedCap UE (2 bytes MAC header + 2 bytes RLC header + 1 byte payload, it is assumed that if UE has any data to transmit that will be on AM DRB). NOTE 5: Applicable only when MCS index table 5.1.3.1-1 or 6.1.4.1-1 is configured to be used for UL grants (e.g. in case of mcs-Table = qam256, a different IMCS value needs to be defined). |  |  |

<SECTIONS SKIPPED>

## 7.3	NR/5GC

### 7.3.1	Introduction

Clause 7.3 specifies test methods and design considerations that are specific to NR/5GC.

### 7.3.2	Physical layer aspects

#### 7.3.2.1	General

The physical layer aspects specified in clause 7.1.2 apply, unless explicitly specified differently in the present clause 7.3.2.

#### 7.3.2.2	RedCap

##### 7.3.2.2.1	BWP operation

When RedCap-specific initial downlink BWP is configured, TTCN configures both the default initial DL BWP and the RedCap-specific initial DL BWP in SS. Each DCI is assigned to either the initial DL BWP or the RedCap-specific initial DL BWP depending on the test case configuration.

NOTE: The configuration of NCD-SSB is not covered in the current test model.

When RedCap-specific initial uplink BWP is configured, TTCN configures both the default initial UL BWP and the RedCap-specific initial UL BWP in SS.

##### 7.3.2.2.2	Half-Duplex FDD operation

A RedCap UE in Half-Duplex FDD mode (HD-FDD) has limitations in receiving an UL symbol following a DL symbol, hence a blank slot is needed in between, defined as gap T_Proc,2.

NOTE:	The gap T_Proc,2 is specified in TS 38.213 [21] clause 17.2.

If the UE under test is an HD-FDD UE:

-	TTCN configures each cell in the SS to operate in HD-FDD mode.

-	When explicit TimingInfo (i.e. not "Now") is provided in any DL transmission, the SS shall ignore the FDD mode and shall follow the explicit scheduling. It is the TTCN responsibility to schedule UL/DL operations based on the UE capability.

-	Paging is always explicitly scheduled and hence SS shall follow the explicit scheduling for paging.

-	When TimingInfo is "Now", the SS shall restrict UL/DL operations as per Table 7.3.2.2.2-1 below:

Table 7.3.2.2.2-1: Data scheduling for FR1: HD-FDD, SCS=15kHz (non-explicitly scheduled)

| Subframe | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL | DCI P-RNTI / PDCCH UL grant#1 / SSBs | DCI P-RNTI / SIB1 | DCI P-RNTI / SIB1 / PDSCH#1 |  |  | PDCCH UL grant#2 / SI | SI | PDSCH#2 |  |  |
| UL |  |  |  | Gap T_Proc,2 | RA occasion / PUSCH#1 / HARQ of PDSCH#2 |  |  |  | Gap T_Proc,2 | PUSCH#2 / HARQ of PDSCH#1 |

### 7.3.3	System information

#### 7.3.3.1	General SS requirements

TTCN provides the complete system information and scheduling information to the SS as a structured ASN.1 type using a single control ASP (NR_SYSTEM_CTRL_REQ). The following rules apply:

-	The system information is sent to SS using the asn.1 types. The SS shall encode each SI message as specified in Table 8.1-1 and add the necessary padding bits as specified in TS 38.331 [16], clause 8.5.

-	The SS shall start scheduling all system information from the same SFN.

-	The scheduling information sent to SS is the same as the scheduling information sent to the UE. For each SI message (including SI messages containing posSIBs), the slotOffset list in NR_SYSTEM_CTRL_REQ indicates the exact point in time in the SI-window at which SS shall transmit the related SI to the UE.

-	SI-window shall be calculated as per TS 38.331[16] clause 5.2.2.3.2 for all SI messages. For SI messages containing posSIBs, the parameter OffsetToSI_Used indicates to SS the SI-window calculation to be applied.

-	In the present version of this specification, encryption of the content of SI messages containing posSIBs is not supported.

-	If the H-SFN value is included in SIB1 sent from the TTCN, the SS shall set the hyperSFN in this message to the current H-SFN. In this case, a dummy value is provided by TTCN.

-	If the H-SFN is not included in the SIB1 sent from the TTCN, it shall not be added by the SS.

#### 7.3.3.2	Scheduling information

The maximum number of resource blocks as defined in table 7.3.3.2-1 are used to broadcast the system information.

Table 7.3.3.2-1: Maximum number of resource blocks

|  | Maximum number of resource blocks assigned | Number of symbols assigned |
| --- | --- | --- |
| SIB1 | 7 | 12 |
| for all SIs | 7 | 12 |

The slot offset values defined in Table 7.3.3.2-1A are used for all SI messages, including SIs containing posSIBs when OffsetToSI-Used is not configured, with their respective SI-window. The slotOffset list depends on the SSB index the cell is configured with.

Table 7.3.3.2-1A: SlotOffset values

| Configuration | slotOffset list for SSB#0 | slotOffset list for SSB#1 |
| --- | --- | --- |
| FR1 FDD & TDD SCS=15kHz | {5} | {6] |
| FR1 TDD SCS=30kHz | {5} | {6] |
| FR2 TDD SCS=120kHz | {5} | {6] |

When OffetToSI-Used is configured, the slot offset values defined in Table 7.3.3.2-1B are used for all SIs containing posSIBs with their respective SI-window. The soltOffset list depends on the SSB index the cell is configured with.

Table 7.3.3.2-1B: PosSI SlotOffset values when offsetToSI-Used is configured

| Configuration | slotOffset list for SSB#0 | slotOffset list for SSB#1 |
| --- | --- | --- |
| FR1 FDD & TDD SCS=15kHz | {25} | {26} |
| FR1 TDD SCS=30kHz | {15} | {16} |
| FR2 TDD SCS=120kHz | {5} | {6} |

All System Information messages are sent only once within the SI-window and redundancy version 0 is selected.

SIB1 is broadcasted in slot#1 in frames with even SFN in cells configured with SSB#0 and in slot#2 in frames with even SFN in cells configured with SSB#1.

Tables 7.3.3.2-2 to 7.3.3.2-4 give the SFN's and subframe numbers in which the MIB, SIB1 and other SIs are actually scheduled as per default parameters for si-WindowLength s80 for FR1 and s160 for FR2, periodicity for SI are defined in TS 38.508-1 [5]. These SI schedulings are applied to all SIs, including SIs containing posSIBs when OffsetToSI_Used is not configured.

Table 7.3.3.2-2: MIB, SIB1 and SI scheduling for FR1: FDD and TDD, SCS=15kHz

| SFN | Frame |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Subframe 0 |  | Subframe 1 |  | Subframe 2 |  | Subframe 3 |  | Subframe 4 |  | Subframe 5 |  | Subframe 6 |  | Subframe 7 |  | Subframe 8 |  | Subframe 9 |  |
|  | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 | SI1 SSB#0 | Sl0 | SI1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  |
| 1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 2 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 3 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 4 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 5 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 6 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 7 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 8 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 | SI2 SSB#0 | Sl0 | SI2 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  |
| 9 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 10 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 11 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 12 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 13 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 14 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 15 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 16 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 | SI3 SSB#0 | Sl0 | SI3 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  |
| 17 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 18 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 19 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 20 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 21 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 22 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 23 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 24 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 | SI4 SSB#0 | Sl0 | SI4 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  |
| 25 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |

Table 7.3.3.2-3: MIB, SIB1 and SI scheduling for FR1 TDD, SCS=30kHz

| SFN | Frame |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Subframe 0 |  | Subframe 1 |  | Subframe 2 |  | Subframe 3 |  | Subframe 4 |  | Subframe 5 |  | Subframe 6 |  | Subframe 7 |  | Subframe 8 |  | Subframe 9 |  |
|  | Sl0 | MIB | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 | SI1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Slot Sl1 |  | Slot Sl1 | SI1 SSB#0 | Slot Sl1 |  | Slot Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 2 | Sl0 | MIB | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Slot Sl1 |  | Slot Sl1 |  | Slot Sl1 |  | Slot Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 3 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 4 | Sl0 | MIB | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 | SI2 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Slot Sl1 |  | Slot Sl1 | SI2 SSB#0 | Slot Sl1 |  | Slot Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 5 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 6 | Sl0 | MIB | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Slot Sl1 |  | Slot Sl1 |  | Slot Sl1 |  | Slot Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 7 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 8 | Sl0 | MIB | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 | SI3 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Slot Sl1 |  | Slot Sl1 | SI3 SSB#0 | Slot Sl1 |  | Slot Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 9 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 10 | Sl0 | MIB | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Slot Sl1 |  | Slot Sl1 |  | Slot Sl1 |  | Slot Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 11 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 12 | Sl0 | MIB | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 | SI4 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Slot Sl1 |  | Slot Sl1 | SI4 SSB#0 | Slot Sl1 |  | Slot Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 13 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 14 | Sl0 | MIB | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Slot Sl1 |  | Slot Sl1 |  | Slot Sl1 |  | Slot Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 15 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |

Table 7.3.3.2-4: MIB, SIB1 and SI scheduling for FR2: TDD, SCS=120kHz

| SFN | Frame |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Subframe 0 |  | Subframe 1 |  | Subframe 2 |  | Subframe 3 |  | Subframe 4 |  | Subframe 5 |  | Subframe 6 |  | Subframe 7 |  | Subframe 8 |  | Subframe 9 |  |
| 0 | Sl0 | MIB | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 | SIB1 SSB#1 | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 | SI1 SSB#0 | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 | SI1 SSB#1 | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |
| 1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |
| 2 | Sl0 | MIB | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 | SIB1 SSB#1 | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 | SI2 SSB#0 | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 | SI2 SSB#1 | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |
| 3 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |
| 4 | Sl0 | MIB | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 | SIB1 SSB#1 | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 | SI3 SSB#0 | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 | SI3 SSB#1 | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |
| 5 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |
| 6 | Sl0 | MIB | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 | SIB1 SSB#1 | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 | SI4 SSB#0 | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 | SI4 SSB#1 | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |

When SIs containing posSIBs with OffsetToSI-Used configured are scheduled, Tables 7.3.3.2-5 to 7.3.3.2-7 give the SFN's and subframe numbers in which the MIB, SIB1, other SIs are actually scheduled. Other SIs containing posSIBs are hereafter named PosSIs. Per default parameters for si-WindowLength s80 for FR1 and s160 for FR2, periodicity for SI are defined in TS 38.508-1 [5].

Table 7.3.3.2-5: MIB, SIB1, SI and PosSI with OffsetToSI-Used configured scheduling for FR1: FDD and TDD, SCS=15kHz

| SFN | Frame |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Subframe 0 |  | Subframe 1 |  | Subframe 2 |  | Subframe 3 |  | Subframe 4 |  | Subframe 5 |  | Subframe 6 |  | Subframe 7 |  | Subframe 8 |  | Subframe 9 |  |
|  | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 | SI1 SSB#0 | Sl0 | SI1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  |
| 1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 2 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 3 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 4 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 5 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 6 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 7 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 8 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 | SI2 SSB#0 | Sl0 | SI2 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  |
| 9 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 10 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 | PosSI1 SSB#0 | Sl0 | PosSI1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  |
| 11 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 12 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 13 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 14 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 15 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 16 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 | SI3 SSB#0 | Sl0 | SI3 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  |
| 17 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 18 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 | PosSI2 SSB#0 | Sl0 | PosSI2 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  |
| 19 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 20 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 21 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 22 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 23 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 24 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 | SI4 SSB#0 | Sl0 | SI4 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  |
| 25 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
| 26 | Sl0 | MIB | Sl0 | SIB1 SSB#0 | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 | PosSI3 SSB#0 | Sl0 | PosSI3 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  |
| 27 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |

Table 7.3.3.2-6: MIB, SIB1, SI and PosSI with OffsetToSI-Used configured scheduling for FR1 TDD, SCS=30kHz

| SFN | Frame |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Subframe 0 |  | Subframe 1 |  | Subframe 2 |  | Subframe 3 |  | Subframe 4 |  | Subframe 5 |  | Subframe 6 |  | Subframe 7 |  | Subframe 8 |  | Subframe 9 |  |
|  | Sl0 | MIB | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 | SI1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Slot Sl1 |  | Slot Sl1 | SI1 SSB#0 | Slot Sl1 |  | Slot Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 2 | Sl0 | MIB | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Slot Sl1 |  | Slot Sl1 |  | Slot Sl1 |  | Slot Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 3 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 4 | Sl0 | MIB | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 | SI2 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Slot Sl1 |  | Slot Sl1 | SI2 SSB#0 | Slot Sl1 |  | Slot Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 5 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 6 | Sl0 | MIB | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Slot Sl1 |  | Slot Sl1 |  | Slot Sl1 |  | Slot Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 7 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 8 | Sl0 | MIB | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 | SI3 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 | PosSI1 SSB#1 | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Slot Sl1 |  | Slot Sl1 | SI3 SSB#0 | Slot Sl1 |  | Slot Sl1 |  | Sl1 |  | Sl1 |  | Sl1 | PosSI1 SSB#0 | Sl1 |  | Sl1 |  |
| 9 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 10 | Sl0 | MIB | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Slot Sl1 |  | Slot Sl1 |  | Slot Sl1 |  | Slot Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 11 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 12 | Sl0 | MIB | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 | SI4 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 | PosSI2 SSB#1 | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Slot Sl1 |  | Slot Sl1 | SI4 SSB#0 | Slot Sl1 |  | Slot Sl1 |  | Sl1 |  | Sl1 |  | Sl1 | PosSI2 SSB#0 | Sl1 |  | Sl1 |  |
| 13 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 14 | Sl0 | MIB | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Slot Sl1 |  | Slot Sl1 |  | Slot Sl1 |  | Slot Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 15 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
| 16 | Sl0 | MIB | Sl0 | SIB1 SSB#1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 | PosSI3 SSB#1 | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Slot Sl1 |  | Slot Sl1 |  | Slot Sl1 |  | Slot Sl1 |  | Sl1 |  | Sl1 |  | Sl1 | PosSI3 SSB#0 | Sl1 |  | Sl1 |  |
| 17 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |

Table 7.3.3.2-7: MIB, SIB1, SI and PosSI with OffsetToSI-Used configured scheduling for FR2: TDD, SCS=120kHz

| SFN | Frame |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Subframe 0 |  | Subframe 1 |  | Subframe 2 |  | Subframe 3 |  | Subframe 4 |  | Subframe 5 |  | Subframe 6 |  | Subframe 7 |  | Subframe 8 |  | Subframe 9 |  |
| 0 | Sl0 | MIB | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 | SIB1 SSB#1 | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 | SI1 SSB#0 | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 | SI1 SSB#1 | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |
| 1 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |
| 2 | Sl0 | MIB | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 | SIB1 SSB#1 | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 | SI2 SSB#0 | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 | SI2 SSB#1 | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |
| 3 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |
| 4 | Sl0 | MIB | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 | SIB1 SSB#1 | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 | SI3 SSB#0 | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 | SI3 SSB#1 | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |
| 5 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |
| 6 | Sl0 | MIB | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 | SIB1 SSB#1 | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 | SI4 SSB#0 | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 | SI4 SSB#1 | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |

| SFN | Frame |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |
| 8 | Sl0 | MIB | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 | SIB1 SSB#1 | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 | PosSI1 SSB#0 | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 | PosSI1 SSB#1 | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |
| 9 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |
| 10 | Sl0 | MIB | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 | SIB1 SSB#1 | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 | PosSI2 SSB#0 | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 | PosSI2 SSB#1 | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |
| 11 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |
| 12 | Sl0 | MIB | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 | SIB1 SSB#0 | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 | SIB1 SSB#1 | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 | PosSI3 SSB#0 | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 | PosSI3 SSB#1 | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |
| 13 | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  | Sl0 |  |
|  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  | Sl1 |  |
|  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  | Sl2 |  |
|  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  | Sl3 |  |
|  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  | Sl4 |  |
|  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  | Sl5 |  |
|  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  | Sl6 |  |
|  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  | Sl7 |  |

#### 7.3.3.3	System information modification

For system information modification, the same rules as defined in clause 7.3.3.1 and 7.3.3.2 are applied.

The SFN for the start of modification period is calculated by TTCN. The modified system information and the calculated SFN are provided in the ASP NR_SYSTEM_CTRL_REQ. When the cell is switched off, ‘activateNow’ is used.

The updated SI other than ETWS and CMAS, is broadcasted in the modification period following the one where SI change indication is transmitted. The updated SI for ETWS and CMAS is broadcasted in the same modification period as the one where SI change indication is transmitted. Short message indications are transmitted as follows:

-	When UE is in RRC_IDLE or in RRC_INACTIVE, a single Short Message indication is transmitted in UE paging occasions of the modification period. With the default values provided in TS 38.508-1[5], this results in 4 Short Messages transmitted by SS during the modification period.

-	When UE is in RRC_CONNECTED, Short Message indications are transmitted in every paging occasion of each frame throughout the modification period. With the default values provided in TS 38.508-1[5], this results in 64*4*2 Short Messages transmitted by SS during the modification period.

#### 7.3.3.4	Request for on demand system information

In case PRACH preamble (RA Msg1) is used for indication of requested other SI:

-	TTCN configures SS with the SI-RequestConfig as provided to UE in SIB1 and SS shall monitor these PRACH resources.

-	TTCN configures SS to report PRACH preambles and to transmit Random Access Response (RA Msg2) as response to RA Msg1 but not to handle contention resolution.

-	TTCN reconfigures SS to stop broadcasting a specific SI from the start of a modification period and reconfigures SS to restart broadcasting the SI with activation time ‘Now’ (after the reception of PRACH preamble in TTCN). When activation time ‘Now’ is applied, SS shall start broadcasting from the next SI-window.

In case RA Msg3 is used for indication of requested other SI:

-	TTCN configures the SS to transmit a RA Msg4 with Contention Resolution Identity upon receipt of RRCSystemInfoRequest (RA Msg3).

-	TTCN reconfigures SS to stop broadcasting a specific SI from the start of a modification period and reconfigures SS to restart broadcasting the SI with activation time ‘Now’ (after the reception of RRCSystemInfoRequest in TTCN). When activation time ‘Now’ is applied, SS shall start broadcasting from the next SI-window.

<SECTIONS SKIPPED>

### 7.3.7	Processing delay testing for RRC procedures

#### 7.3.7.1	Introduction

Test cases testing RRC procedure delay (according to TS 38.331 [16] clause 12), when UE is PUCCH synchronized and can respond to UL grant, require special test method.

#### 7.3.7.2	Procedure delays in PUCCH synchronized state

The following SS and UE configurations apply:

By default in TS 38.508-1[5] DRX is not configured.

By default SS is configured to retransmit any DL MAC PDU max 4 times.

HARQ retransmissions in UL cannot be compensated, any HARQ error in UL shall result in an inconclusive verdict for the test case (otherwise a UE may get fail due to a HARQ error).

SS is configured to report UL HARQ ACK/NACK received from UE to TTCN.

NOTE:	Due to L2 signalling (e.g. RLC STATUS PDUs) it is necessary to limit the reporting of UL HARQ ACK/NACK to the time between sending of the DL RRC message and receiving the HARQ ACK.

To avoid unexpected side effects the timeAlignmentTimer is set to infinity and TTCN configures the SS to not send any Timing Advance Command MAC CE during the test case body (since this may result in additional UL HARQ ACK/NACK).

SS is configured to report HARQ errors and in the case of an UL HARQ error, an inconclusive verdict is assigned.

The following timing applies:

Let Nsf be the maximum allowed RRC procedure delay value (in ms) as specified in the test case prose. Nsf is the maximum time allowed for UE from the end of reception of the DL message on the UE physical layer up to when the UE shall be ready for the reception of UL grant for the transmission of the response message.

NOTE:	A tolerance may have been added to the Nsf value in the test case prose on top of the RRC procedure delay specified in TS 38.331 [16] clause 12.

Let Nslot = (Nsf * Nslotsubframe,μ) + 1

TTCN schedules at time T1 the transmission of the DL message to the UE.

T2 is the time HARQ ACK is received.

TTCN schedules UL grants from T3: T3 = T1 + Nslot + ∆1

∆1 is expressed in slots. T1 and T3 timing are expressed in SFN/subframe/slots.

For FD-FDD

∆1 = 0

The UL grant is repeated continuously in every slot

For FR1 TDD SCS=15kHz or HD-FDD (RedCap)

∆1 = 5 - (T1 + Nslot) mod 5. This is the possible slot delay since not all slots can be used for UL grant.

The UL grant periodicity is set to 5 slots. It is repeated in subframe#0 and subframe#5, to cope with K2=4 and the UL subframe#4 and subframe#9.

For FR1 TDD SCS=30kHz

∆1 = (4 - (T1 + Nslot) mod 10) mod10. This is the possible slot delay since not all slots can be used for UL grant.

The UL grant periodicity is set to 10 slots. It is repeated in slot#0 in subframe#2 and subframe#7, to cope with K2=4 and the UL slots: slot#0 in subframe#4 and subframe#9.

For FR2 TDD SCS=120kHz

∆1 = (5- (T1 + Nslot) mod 5) mod 5. This is the possible slot delay since not all slots can be used for UL grant.

The UL grant periodicity is set to 5 slots. It is repeated for possible transmission in every UL slots.

The UL data is received by SS at T4.

The RRC procedure delay requirements are fulfilled when: (T4-K2) - (T2 - K1) <= Nslot + ∆2

∆2 is expressed in slots. T4 and T2 timing are expressed in SFN/subframe/slots.

For FD-FDD

∆2 =0

For FR1 TDD SCS=15kHz or HD-FDD (RedCap)

∆2 = 5 - (T2-K1 + Nslot) mod 5

For FR1 TDD SCS=30kHz

∆2 = (4 - (T2-K1 + Nslot) mod 10) mod10

For FR2 TDD SCS=120kHz

∆2 = (5- (T2-K1 + Nslot) mod 5) mod 5

Figure 7.3.7.2-1 shows the RRC procedure delay check sequence that will be applied when the DL Message is retransmitted once when UE is in PUCCH synchronized state and can respond to UL grants.

Figure 7.3.7.2-1: Delays in PUCCH synchronized state

#### 7.3.7.3	Procedure delays with RACH procedure

The following SS and UE configurations apply:

SS is configured with RACH procedure with contention-based with C-RNTI based contention resolution.

By default in TS 38.508-1[5] DRX is not configured.

In TTCN, this procedure delays with RACH is implemented in parallel to the PUCCH synchronized state procedure, as defined in clause 7.3.7.2, as some UEs may use SR/PUCCH. Therefore, SS is configured to retransmit any DL MAC PDU max 4 times.

No HARQ retransmission of RRCReestablishment/RRCResume is taken into account in the delay calculation in the procedure with RACH.

HARQ retransmissions in UL cannot be compensated, any HARQ error in UL shall result in an inconclusive verdict for the test case (otherwise a UE may get fail due to a HARQ error).

SS is configured to report UL HARQ ACK/NACK received from UE to TTCN.

NOTE:	Due to L2 signalling (e.g. RLC STATUS PDUs) it is necessary to limit the reporting of UL HARQ ACK/NACK to the time between sending of the DL RRC message and receiving the HARQ ACK.

By default, the timeAlignmentTimer is set to infinity and TTCN configures the SS to not send any Timing Advance Command MAC CE during the test case body (since this may result in additional UL HARQ ACK/NACK).

SS is configured to report HARQ errors and in the case of an UL HARQ error, an inconclusive verdict is assigned.

SS is configured to report RACH preamble.

The following timing applies:

Let Nsf be the maximum allowed RRC procedure delay value (in ms) as specified in the test case prose. Nsf is the maximum time allowed for UE from the end of reception of the DL message on the UE physical layer up to when the UE shall be ready for the reception of UL grant for the transmission of the response message.

Let Nslot = (Nsf * Nslotsubframe,μ) + 1

TTCN schedules at time T1 the transmission of the DL message to the UE as specified in the prose.

T2 is the time HARQ ACK is received.

TTCN schedules UL grants from T3: T3 = T1 + Nslot + ∆1 (as per clause 7.3.7.2)

The UL RACH preamble is received at T5

T6 is the last occasion for SS to transmit RAR in message 2

The UL data is received by SS at T4

The RRC procedure delay requirements are fulfilled when: T4 - T1 <= Nslot + ∆TDelaymax

∆TDelaymax is expressed in slots. T4 and T1 timing are expressed in SFN/subframe/slots.

For FD-FDD SCS=15kHz

∆TDelaymax = 7

For HD-FDD (RedCap) SCS=15kHz

If Nsf = 10 ms, ∆TDelaymax = 2

If Nsf = 6 ms, ∆TDelaymax = 6

For FR1 TDD SCS=15kHz

If Nsf = 10 ms, ∆TDelaymax = 7

If Nsf = 6 ms, ∆TDelaymax = 11

For FR1 TDD SCS=30kHz

If Nsf = 10 ms, ∆TDelaymax = 17

If Nsf = 6 ms, ∆TDelaymax = 25

For FR2 TDD SCS=120kHz

If Nsf = 10 ms, ∆TDelaymax = 32

If Nsf = 6 ms, ∆TDelaymax = 24

Figure 7.3.7.3-1 shows the RRC procedure delay sequence with the example of RRC connection re-establishment, the same is applicable for RRC connection resume.

Figure 7.3.7.3-1: Delays with RACH procedure