3GPP TSG WG RAN5 Meeting #101	R5-237857

# Chicago, USA, 13 – 17 Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | N/A | rev | 1 | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Initial conditions update for multiple RX test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. , Keysight Technologies UK Ltd ,CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Initial conditions requires update to implement proposal 1.2 endorsed in R5-236027. Remove editor’s notes of “Initial condition and call setup procedure support satellite access are TBD” to implement proposal 4 of R5-236027 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated test procedure of RX cases and did some clean-up for editor’s note |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases are remained incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.4.1, 7.4A.4.1, 7.4B.4.1 7,5A.4.1, 7.5B.4.1 ,7.6A.2.4.1 7.6B.2.4.1, 7.9A.4.1, 7.9B.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on R5-236027 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: update initial condition final version: revised from R5-236052r1 |  |  |  |  |  |  |  |  |

<<Start of changes >>

7.3B.4	Test description

7.3B.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.3 using only the main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS36.521-1[14] Annex C0, C1 and C.3.0, and NPUSCH Format 2 is used to carry ACK/NACK on the uplink.

4.	The DL Reference Measurement channels are set according to Table 7.3B.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	UE location according to TS 36.508 [12] clause 8.2.6.1 is provided to the UE through any preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] table 8.2.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 8.2.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.2.6.3.1.

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 7.3B.4.3.

<< End of changes >>

<<Start of changes >>

7.4A	Maximum input level for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

- Test configuration is TBD

<< End of changes >>

<<Start of changes >>

7.4A.4	Test description

7.4A.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS36.521-1 [14] Annex C0, C.1 and C.3.1, and uplink signals according to Annex H.1 and H.3.1.

4.	The UL and DL Reference Measurement channels are set according to Table 7.4A.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through any preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB31 information during the duration of the test as define in TS 36.508[12] clause 5.6.3.1

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 7.4A.4.3.

<< End of changes >>

<<Start of changes >>

7.4B	Maximum input level for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

- Test configuration is TBD

<< End of changes >>

<<Start of changes >>

7.4B.4	Test description

7.4B.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.3 using only the main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS36.521-1 [14] Annex C0, C1 and C.3.0, and NPUSCH Format 2 is used to carry ACK/NACK on the uplink.

4.	The DL Reference Measurement channels are set according to Table 7.4B.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 8.2.6.1 is provided to the UE through any preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] table 8.2.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 8.2.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB31-NB information during the duration of the test as define in TS 36.508[12] clause 8.2.6.3.1

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 7.4B.4.3.

<< End of changes >>

<<Start of changes >>

7.5A	Adjacent Channel Selectivity for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

- Test configuration is TBD

<< End of changes >>

<<Start of changes >>

7.5A.4	Test description

7.5A.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.4 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS36.521-1 [14] Annex C0, C.1 and C.3.1, and uplink signals according to Annex H.1 and H.3.1.

4.	The UL and DL Reference Measurement channels are set according to Table 7.5A.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through any preconfigured means.

7.	 Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB31 information during the duration of the test as define in TS 36.508[12] clause 5.6.3.1

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 7.5A.4.3.

<< End of changes >>

7.5B	Adjacent Channel Selectivity for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

- Test configuration is TBD

<<Start of changes >>

7.5B.4	Test description

7.5B.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.4 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS36.521-1 [14] Annex C0, C.1 and C.3.1, and uplink signals according to Annex H.1 and H.3.1.

4.	The UL and DL Reference Measurement channels are set according to Table 7.5B.4.1-1.

5	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 8.2.6.1 is provided to the UE through any preconfigured means.

7.	 Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] table 8.2.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 8.2.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB31-NB information during the duration of the test as define in TS 36.508[12] clause 8.2.6.3.1

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. Message contents are defined in clause 7.5B.4.3.

<< End of changes >>

<<Start of changes >>

7.6A.2	In-band blocking for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

- Test configuration is TBD

<< End of changes >>

<<Start of changes >>

7.6A.2.4	Test Description

7.6A.2.4.1	Initial Conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.4 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [14] Annex C0, C.1 and C.3.1, and uplink signals according to TS 36.521-1 [14] Annex H.1 and H.3.1.

4.	The UL and DL Reference Measurement channels are set according to Table 7.6A.2.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1 [14] Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through any preconfigured means.

7.	 Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB31 information during the duration of the test as define in TS 36.508[12] clause 5.6.3.1

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 7.6A.2.4.3.

<< End of changes >>

<<Start of changes >>

7.6B.2	In-band blocking for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

- Test configuration is TBD

<< End of changes >>

<<Start of changes >>

7.6B.2.4	Test Description

7.6B.2.4.1	Initial Conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connector as shown in TS 36.508 [12] Annex A, Figure A.4 using only the main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [14] Annex C.0, C.1 and C.2, and NPUSCH Format 2 is used to carry ACK/NACK on the uplink.

4.	The DL Reference Measurement channel is set according to Table 7.6B.2.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1 [14] Annex B.0.

6.	UE location according to TS 36.508 [12] clause 8.2.6.1 is provided to the UE through any preconfigured means.

7.	 Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] table 8.2.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 8.2.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB31-NB information during the duration of the test as define in TS 36.508[12] clause 8.2.6.3.1

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 7.6B.2.4.3.

<< End of changes >>

<<Start of changes >>

7.9A	Spurious emissions for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

7.9A.4	Test Description

7.9A.4.1	Initial Conditions

<<Skip unchanged >>

1.	Connect a spectrum analyzer (or other suitable test equipment) to the UE antenna connectors as shown in TS 36.508 [12] Annex A, Figure A.7 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to [Annex C0, C.1 and C.3.1].

4.	The DL Reference Measurement channels are set according to Table 7.9A.4.1-1.

5.	Propagation conditions are set according to [Annex B.0].

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through any preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB31 information during the duration of the test as define in TS 36.508[12] clause 5.6.3.1

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 7.9A.4.3.

<< End of changes >>

<<Start of changes >>

7.9B	Spurious emissions for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

7.9B.4	Test Description

7.9B.4.1	Initial Conditions

<<Skip unchanged >>

1.	Connect a spectrum analyzer (or other suitable test equipment) to the UE antenna connectors as shown in TS 36.508 [12] Annex A, Figure A.7 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to [Annex C0, C.1 and C.3.1].

4.	The DL Reference Measurement channels are set according to Table 7.9B.4.1-1.

5.	Propagation conditions are set according to [Annex B.0].

6.	UE location according to TS 36.508 [12] clause 8.2.6.1 is provided to the UE through any preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] table 8.2.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 8.2.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB31-NB information during the duration of the test as define in TS 36.508[12] clause 8.2.6.3.1

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 7.9B.4.3.

<< End of changes >>