3GPP TSG-RAN WG5 Meeting #101	R5-237213

Chicago, USA, November 13th – November 17th, 2023

Agenda Item:	5.3.14.9

Source:	THALES

Title: 	GSO satellite Ephemeris file generation methodology for NTN NR UE testing

Document for:	Discussion

# Introduction

The main objective of this technical discussion document is to present an updated methodology to generate satellite Ephemeris for NTN NR UE testing that incorporates both SIB19 state vector and agreement reached at RAN4#106-bis with respect to NTN UE testing.

The data required by the Test Equipment to implement SIB19 (NTN dedicated SIB from 5G NR Rel-17) is based on two potential information: ECEF information and/or Orbital information. Effectively, it has been agreed in RAN1/RAN2 that UEs are provided with satellite ephemeris in ECI of the date (namely ECEF) in either state vector (position velocity) or orbital information (Kepler elements).

The testing methodology from this discussion document can apply for test preparation of NTN NR UE either for:

- Testing at the time of the current date using GNSS real time resources, or

- Testing at a previous date earlier than the current date (i.e. 1 day) using stored GNSS information or GNSS information to be retrieved from AT commands.

The recovery, the interpretation and processing of location information by the UE (pre-configured location, GNSS information or AT commands) is out scope of this document.

This approach is in line with maximum errors described in R1-2111122 "Considerations on UL timing and frequency synchronization in NTN" or the testing process described in R4-2302517 "RRM test cases methodology and configuration for SAN NTN assistance information parameters".

In addition to Range and Doppler effect evaluation, the delay, delay shift considered. Furthermore, the state vector (position, velocity) are given in SIB19 format using the dedicated transfer functions.

This contribution is an update of previous RAN5 contribution R5-233941 “satellite Ephemeris file generation methodology for NTN NR UE testing”, THALES.

# Recall of RRC Specification Protocol (TS 38.331) in Rel-17

The specification TS 38.331 "Technical Specification Group Radio Access Network - NR - Radio Resource Control (RRC) protocol specification" indicates the required satellite ephemeris information to be included in SIB and transmitted/broadcasted by the satellite. The UE uses this information (namely SIB19 for NTN 5GNR) to perform UL Doppler and timing advance pre-compensation.

-- ASN1START

-- TAG-EPHEMERISINFO-START

EphemerisInfo-r17 ::=          CHOICE {

positionVelocity-r17           PositionVelocity-r17,

orbital-r17                    Orbital-r17

}

PositionVelocity-r17 ::=       SEQUENCE {

positionX-r17                  PositionStateVector-r17,

positionY-r17                  PositionStateVector-r17,

positionZ-r17                  PositionStateVector-r17,

velocityVX-r17                 VelocityStateVector-r17,

velocityVY-r17                 VelocityStateVector-r17,

velocityVZ-r17                 VelocityStateVector-r17

}

Orbital-r17 ::=                SEQUENCE {

semiMajorAxis-r17              INTEGER (0..8589934591),

eccentricity-r17               INTEGER (0..1048575),

periapsis-r17                  INTEGER (0..268435455),

longitude-r17                  INTEGER (0..268435455),

inclination-r17                INTEGER (-67108864..67108863),

meanAnomaly-r17                INTEGER (0..268435455)

}

PositionStateVector-r17 ::= INTEGER (-33554432..33554431)

VelocityStateVector-r17 ::= INTEGER (-131072..131071)

-- TAG-EPHEMERISINFO-STOP

-- ASN1STOP

With the following description:

EphemerisInfo field descriptions

Eccentricity Satellite orbital parameter: eccentricity e, see NIMA TR 8350.2 [71]. Unit is radian. Step of 1.431 * 10-8. Actual value = field value * (1.431 * 10-8).

Inclination Satellite orbital parameter: inclination i, see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

Longitude Satellite orbital parameter: longitude of ascending node , see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

meanAnomaly Satellite orbital parameter: Mean anomaly M at epoch time, see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

Periapsis Satellite orbital parameter: argument of periapsis , see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

positionX, positionY, positionZ X, Y, Z coordinate of satellite position state vector in ECEF. Unit is meter. Step of 1.3 m. Actual value = field value * 1.3.

semiMajorAxis Satellite orbital parameter: semi major axis , see NIMA TR 8350.2 [71]. Unit is meter. Step of 4.249 * 10-3 m. Actual value = 6500000 + field value * (4.249 * 10-3).

velocityVX, velocityVY, velocityVZ X, Y, Z coordinate of satellite velocity state vector in ECEF. Unit is meter/second. Step of 0.06 m/s. Actual value = field value * 0.06.

Those transfer functions previously presented are directly incorporated in the GMAT script.

# Agreement on NTN UE testing

The test of NTN UE for NGSO satellite has been discussed at several occasions during RAN4#108 Toulouse.

Meanwhile, after the RAN4#108 meeting, RAN#101 decided the following Way Forward in RP-232682 “Summary of offline discussions for NTN testing”:

Way Forward:

- RAN tasks RAN4 to discuss and conclude in Oct meeting on

  - The condition for testing the frequency error requirements to verify NGSO UE pre-compensation functionality for Doppler shift, which will be implemented in the RAN5 NTN specifications

    - For example, setting an artificial fixed Doppler shift, which is randomly selected out of a range of Doppler shift values decided by RAN4, as the condition in a test

  - The condition for RRM uplink timing test cases to verify NGSO UE delay pre-compensation functionality, which can be implemented in RAN5 NTN specifications

    - For example, setting an artificial fixed delay, which is randomly selected out of a range of delay values decided by RAN4, as the condition in a test

    - Note: For all the other NTN RRM test cases (except NGSO scenario in UE timing TCs), zero doppler shift and zero timing shift are assumed to be configured for RAN5 conformance testing

- Further discuss TE-emulated channel model with delay and Doppler shifts matching the satellite propagator model in future release i.e. Rel-19

- No changes to the RAN4 core requirements

More discussion took place before the RAN4#108-bis meeting and during the meeting, which generated the following documents:

- R4-2317963, Topic summary for [108bis][332] LS_BSRF_RAN_task;

- R4-2316953, Ad-hoc meeting minutes on LS_BSRF_RAN_task.

At the end, the final agreements for NGSO/GSO testing reached at RAN4#108-bis were:

(priority) Issue 1-1: [NGSO][RF] Range of Doppler shift for RF frequency error requirements

agreement

- For one of the cases in the freq error test, the range of absolute value of Doppler is within [0.1 or 7.5ppm] greater than zero and less than or equal to 24ppm.

  - The range can be further limited, considering other factors like elevation angles and satellite height.

  - The exact value(s) and the number of values (>1) to be selected are left to RAN5.

    - At least the worst-case value needs to be selected

  - UE should derive the amount of Doppler to be pre-compensated based on the ephemeris info (SIB-19 or SIB-31) and UE location.

    - The same ephemeris info will be maintained during each single frequency error measurement of the TE

Issue 1-2: [NGSO][RF] Assumption of delay shift for RF frequency error requirements

Agreement:

- Constant value from the same ephemeris (i.e., orbit emulation) as the Doppler shift is derived

- UE is assumed to derive the amount of delay to be pre-compensated based on the ephemeris info (SIB-19 or SIB-31) and UE location.

- The requirement is defined assuming the UE pre-compensates the amount of delay

(priority) Issue 1-3: [NGSO][RRM] Range of Delay shift for RRM UL timing accuracy test cases

Agreement:

- The range of the one-way delay from UE to satellite is within 2ms (lowest value for LEO orbit 600km) to 6.67ms (highest for LEO orbit 1200 km)

  - Assuming that the absolute elevation angle is larger than 30 degree

  - The range can be further limited, considering other factors like elevation angles and satellite height.

  - The set of values and the number of values is to be introduced by RAN4.  RAN5 to select from this set according to RAN4’s test case revision.

    - At least the worst-case value needs to be selected

  - UE should derive the amount of time delay to be pre-compensated based on the ephemeris info (SIB-19 or SIB-31) and UE location.

    - The same ephemeris info will be maintained during each single timing accuracy measurement of the TE

Issue 1-4: [NGSO][RRM] Assumption of Doppler shift for RRM UL timing accuracy test cases

Agreement:

- Constant value from the same ephemeris (i.e., orbit emulation) as the delay is derived

- UE is assumed to derive the amount of Doppler to be pre-compensated based on the ephemeris info (SIB-19 or SIB-31) and UE location.

- The requirement is defined assuming the UE pre-compensates the amount of Doppler

Issue 1-5: [NGSO][RRM] Assumption of Doppler and timing shifts for RRM test cases other than UL timing accuracy

Agreement: Constant non-zero time delay, with constant non-zero Doppler value per satellite

Sub-topic 1-2: GSO testing

Agreement:  Apply the same principles for GSO as agreed for NGSO

Finally, the following Way Forward has been approved as follows at RAN4#108-bis:

- R4-2316967	WF on NTN RAN task

- Chair:  Issue 2-5 indicates non-zero Doppler for GSO.  In addition, zero Doppler shall be considered for GEO.

# GSO satellite Ephemeris Generation

The purpose is to generate an orbit for NTN testing, so design techniques of a low Earth orbit such as stabilizations techniques of the orbit by selecting eccentricity with SMA (Semi-major Axis) and inclination are not included. Furthermore, the presented model contains only gravity force J0.

For the mission preparation, a brief description of GMAT instantiation main steps is given the following sub sections through the GUI. The related script is jointly updated once any created objects and validated using the GUI.

The GUI is used for all steps 4.1 to 4.6 using "resources" tab, 4.7 using "Mission" tab for mission sequence and the script for coding range and Doppler.

## Spacecraft

The simulated orbit is for a satellite at 35786 km altitude with an orbit inclination of 7°.

Observation 1: UTC time representation is defined as the timescale to be used for earth-space communication and therefore selected.

Keplerian representation can be selected to fill initial satellite state. Excentricity is set to 1e-7.

Figure 4-1 Satellite Setting

## Ground Station simulating the UE

The ground station object is generated from GUI in resources. The NR NTN UE is modeled by a Ground Station.

Ground Station location for simulation is given in EarthFixed frame with Earth ellipsoid model selected and is considered fixed.

The used acronym for ground station is GDS.

Figure 4-2 Ground Station setting

A local topocentric frame is be attached to the ground station. This allows to get satellite coordinates in local ground station frame and then calculate satellite Elevation and then determine satellite visibility start and stop.

The UE selected location is given by TS 36.508 (and respectively by TS 38.508-1):

## Propagators & Force Models

The default propagator type is used.

Default Gravity JGM2 is kept. Degree of Earth potential is set to 70 and an order of 0.

Drag and Atmospheric model unchanged. The drag & atmospheric cannot be disabled.

The max Step Size is adjusted such timestamp allows to get Doppler shift in S-band lower ±50Hz for a carrier frequency of 2 GHz

The possible values to considered are 2n*10 ms with n=0,1,2,..8.

In this case 2.56s has been considered for GSO.

Figure 4-3 Propagator Setting

As the drag model cannot be deselected, it is however possible to mitigate this perturbation to 0 by setting spacecraft drag surfaces properties.

Set to 0

Figure 4-4 No Drag effect by setting satellite surface to 0

## UE Coordinate Systems

A local topocentric frame is be attached to the ground station. This allows to get satellite coordinates in local ground station frame and then calculate satellite Elevation and then determine satellite visibility start and stop.

Figure 4-5 Creation of topocentric frame attached to the ground station

Once this frame defined, GMAT automatically allows access of satellite vector in UE frame without any additional coding.

## Subscribers

The subscribers in this context are data representation such as plots, groundtrack view or orbit view or results file generation.

- Orbitview is let to default

- Groundtrackplot is let to default except groundstation object is enabled through the GUI.

- Result file generation

The object has to be created from GUI "resources".

The parameters to be save are selectable through the GUI. Variables can be added but need to be created first.

Figure 4-6 Reportfile setting for Sat1

Range, Delay, Doppler, Sat_El_GDS have been previously created variables and therefore can selected through the GUI.

The list of store parameters for this use-case are: Sat_El_GDS, Sat_UTC_Gregorian, One-WayDelay, Xsat_ECEF_Coded, Ysat_ECEF_Coded, Zsat_ECEF_Coded, VXsat_ECEF_Coded, VYsat_ECEF_Coded, VZsat_ECEF_Coded, sma_ECEF_Coded, Excentricit_ECEF_Coded, Inc_ECEF_Coded, RAAN_ECEF_Coded, AOP_ECEF_Coded, MeanAnom_ECEF_Coded.

## Variables, Arrays, String

Variables can be created through the GUI.

Range, Delay, Doppler, DelayRate and Sat_El_GDS are created result variables.

6 variables are created to get satellite position and velocity in SIB19 INTEGER format according to §2 above: Xsat_ECEF_Coded, Ysat_ECEF_Coded,  Zsat_ECEF_Coded, VXsat_ECEF_Coded, VYsat_ECEF_Coded, VZsat_ECEF_Coded

Intermediate variables are created for computation purpose and not stored.

This table provides the correspondence between created variables and SIB19 parameters.

|  | SIB19 Release17 | GMAT variable |
| --- | --- | --- |
| StateVector | PositionX-r17 | Xsat_ECEF_Coded |
|  | PositionY-r17 | Ysat_ECEF_Coded |
|  | PositionZ-r17 | Zsat_ECEF_Coded |
|  | VelocityVX-r17 | VXsat_ECEF_Coded |
|  | VelocityVY-r17 | VYsat_ECEF_Coded |
|  | VelocityVZ-r17 | VZsat_ECEF_Coded |
| Orbital elements | semiMajorAxis-r17 | sma_ECEF_Coded |
|  | eccentricity-r17 | Excentricit_ECEF_Coded |
|  | periapsis-r17 | AOP_ECEF_Coded |
|  | longitude-r17 | RAAN_ECEF_Coded |
|  | inclination-r17 | Inc_ECEF_Coded |
|  | meanAnomaly-r17 | MeanAnom_ECEF_Coded |

Table 4-1 SIB19 parameter vs GMAT variable

## Mission Preparation using the GUI

This mission preparation aims to define the different execution steps using the "resources" tab.

First step is to initialize the variables.

Second step is to create the logical structure While 'condition' endwhile.

The condition is related the use of the propagator:

Third step is to create a verification if satellite is in visibility of the ground station; a logical structure "if …" is created with the condition:

## Mission preparation using the script

After creation of the logical structure, Sat_El_GDS, range and Doppler equations need to be implemented.

Modified script is highlighted in yellow to compute state vector components in SIB19 decimal format.

Sat_El_GDS is calculated as the elevation of the spacecraft in GDS frame. Range and Doppler are computed in GDS frame allowing simple calculations:

- The radial range is the norm of the Sat1 position vector;

- The radial range rate is the orthogonal projection of velocity vector on axis supported normalized Sat1 position vector;

- The Doppler effect is the minus radial range rate divided by the range and by the light celerity;

- The Delay Shift is the Delay multiplied by Doppler effect.

The resulting script for the while loop should be as follows:

BeginScript 'Initialisation'

GMAT RAAN_Init = Sat1.RAAN;

EndScript;

GMAT 'Set variable GDS_longitude to actual longitude' GDS_longitude = 121.56076999;

GMAT DeltaRAAN = Sat1.EarthFixed.PlanetodeticLON-GDS_longitude;

GMAT temp = RAAN_Init-DeltaRAAN;

GMAT Sat1.RAAN = temp;

While 'While ElapsedDays < Threshold' Sat1.ElapsedDays < 1

GMAT Xsat_GDS = Sat1.GDS.X;

GMAT Ysat_GDS = Sat1.GDS.Y;

GMAT Zsat_GDS = Sat1.GDS.Z;

GMAT VXsat_GDS = Sat1.GDS.VX;

GMAT VYsat_GDS = Sat1.GDS.VY;

GMAT VZsat_GDS = Sat1.GDS.VZ;

GMAT D = sqrt(Sat1.GDS.X^2+Sat1.GDS.Y^2+Sat1.GDS.Z^2);

GMAT Dz = Zsat_GDS;

GMAT Dy = Ysat_GDS;

GMAT Dx = Xsat_GDS;

GMAT DopplerEffect = (Xsat_GDS*VXsat_GDS+Ysat_GDS*VYsat_GDS+Zsat_GDS*VZsat_GDS)/D/LightCel;

GMAT Doppler_ppm = DopplerEffect*1000000;

GMAT OneWayDelay_ms = D/LightCel*1000;

GMAT El_GDS = RadToDeg(asin(Zsat_GDS/D));

If Xsat_GDS < 0

GMAT El_GDS = 180-RadToDeg(asin(Zsat_GDS/D));

EndIf;

GMAT 'Generate Xsat_ECEF_Coded (m)' temp = Sat1.EarthFixed.X*1000/1.3;

GMAT Xsat_ECEF_Coded_m = floor(temp+0.5);

GMAT 'Generate Ysat_ECEF_Coded (m)' temp = Sat1.EarthFixed.Y*1000/1.3;

GMAT Ysat_ECEF_Coded_m = floor(temp+0.5);

GMAT 'Generate Zsat_ECEF_Coded (m)' temp = Sat1.EarthFixed.Z*1000/1.3;

GMAT Zsat_ECEF_Coded_m = floor(temp+0.5);

GMAT 'Generate VXsat_ECEF_Coded (m)' temp = Sat1.EarthFixed.VX*1000/0.06;

GMAT VXsat_ECEF_Coded_m = floor(temp+0.5);

GMAT 'Generate VYsat_ECEF_Coded (m)' temp = Sat1.EarthFixed.VY*1000/0.06;

GMAT VYsat_ECEF_Coded_m = floor(temp+0.5);

GMAT 'Generate VZsat_ECEF_Coded (m)' temp = Sat1.EarthFixed.VZ*1000/0.06;

GMAT VZsat_ECEF_Coded_m = floor(temp+0.5);

GMAT 'Generate sma_ECEF_Coded (m)' temp = (Sat1.Earth.SMA*1000-6500000)/0.004249;

GMAT sma_ECEF_Coded_m = floor(temp+0.5);

GMAT 'Generate Excentricity_ECEF_Coded ' temp = Sat1.Earth.ECC/0.00000001431;

GMAT Excentricity_ECEF_Coded = floor(temp+0.5);

GMAT 'Generate Inc_ECEF_Coded (rad)' temp = DegToRad(Sat1.EarthFixed.INC)/0.00000002341;

GMAT Inc_ECEF_Coded = floor(temp+0.5);

GMAT 'Generate RAAN_ECEF_Coded (rad)' temp = DegToRad(Sat1.EarthFixed.RAAN)/0.00000002341;

GMAT RAAN_ECEF_Coded = floor(temp+0.5);

GMAT 'Generate AOP_ECEF_Coded (rad)' temp = DegToRad(Sat1.EarthFixed.AOP)/0.00000002341;

GMAT AOP_ECEF_Coded = floor(temp+0.5);

GMAT 'Generate MeanAnom_ECEF_Coded(rad)' temp = DegToRad(Sat1.Earth.MA)/0.00000002341;

GMAT MeanAnom_ECEF_Coded = floor(temp+0.5);

Propagate 'Prop One Step' DefaultProp(Sat1);

EndWhile;

The generated report file for SIB19 then contain satellite ephemeris for each time of the propagation:

Units: Satellite elevation in UE frame in degree, one-way Delay in ms, Doppler in ppm, position for SIB19 in INTEGER, velocity component for SIB19 in INTEGER.

## Data for test

At RAN4#108-bis, it has been agreed to perform NTN UE testing for two different Doppler values:

- Null Doppler with corresponding delay as obtained from ephemeris files,

- A non-Doppler with related delay value as obtained from ephemeris files.

Using the above results and an elevation within the [30 degrees – 150 degrees], the test inputs for non-zero Doppler can then be derived as given in the following example:

Table 4-2 Minimum – Maximum – 0 Doppler summary

## GSO considerations

### UE location

Observation 2: for any given UE location, the daily elevation variation remains rather low compared to NGSO cases. However, the UE location is limited by the Earth visibility domain with the minimum angle of 10° or 30°.

As an indication, the Earth visibility domain as seen by a satellite in geostationary position and 0° inclination for a minimum elevation of 0° is given the following figure:

Figure 4-7 Visibility domain from a geostationary satellite, Minimum UE elevation=0

This table gives at satellite longitude the maximum UE latitude with its minimum elevation angle:

| Minimum UE Elevation (degree) | maximum UE latitude with satellite longitude at UE longitude (spherical Earth) |
| --- | --- |
| 0 | 81.30 |
| 5 | 76.33 |
| 10 | 71.43 |
| 15 | 66.60 |
| 20 | 61.83 |
| 25 | 57.12 |
| 30 | 52.47 |

Table 4-3 maximum UE latitude with satellite longitude at UE longitude

### Maximum Doppler

The maximum Doppler during simulation is about ±0.1 ppm. As a matter of fact, this is a common value for geostationary satellite with a fixed (or static) UE location.

Although TR 38.811 shows higher values, is important to mention that these results were obtained with UE in motion embarked a plane:

Table 4-3 Extract of TR 38.811 Rel-15

Finally, the figure below shows satellite movement and Doppler seen by the UE during the simulation for a fixed UE position (24 hours have been simulated):

Figure 4-8 Satellite orbit evolution during 24hrs with UE Doppler

# Overall Conclusion

Proposal 1. For GSO NTN UE testing, RAN5 can use the following example file: