3GPP TSG-RAN5 Meeting #101	R5-237818

# Chicago, USA, 13th – 17th Nov. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2982 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of SCG activation and deactivation configuration for NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The RRM test cases 7.5.14 for SCG activation and deactivation in NR-DC has been in place in TS 38.533, but the network configuration that indicates SCG state and related TCI state is still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding SCG activation and deactivation configuration for NR-DC RRM test case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mentioned RRM test case will not be tested properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.1, 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 CR 2783 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-236878r1 |  |  |  |  |  |  |  |  |

<Start of Changes>

RRCReconfiguration

Table 4.6.1-13: RRCReconfiguration

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |
|  |  | RadioBearerConfig with conditions SRB2 and DRB1 |  | NR, NR-DC |
|  |  | RadioBearerConfig with conditions SRB2 and REEST |  | REEST |
| secondaryCellGroup |  | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
|  |  | CellGroupConfig with condition NR-DC_SCG | OCTET STRING (CONTAINING CellGroupConfig) | NR-DC_SCG |
|  |  | CellGroupConfig with conditions EN-DC and PSCell_change | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_HO |
|  |  | CellGroupConfig with condition MEAS | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_MEAS |
|  |  | CellGroupConfig with condition SCell_add | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_SCell_add |
|  |  | Not present |  |  |
| measConfig |  | Not present |  |  |
|  |  | MeasConfig | Measurements configuration | NR_MEAS, EN-DC_MEAS, IRAT_MEAS |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | NR, SCell_add, NR_MEAS, NR-DC, SIDELINK, DAPS_HO_ReleaseSource, CHO, CPC, REEST, L2RelayUE, L2RemoteUE, SHRT304, SHRT310, SHRT312, SHRRLF, DeactivatedSCG |
| masterCellGroup |  | CellGroupConfig with condition SRB2_DRB1 | OCTET STRING (CONTAINING CellGroupConfig) | NR, NR-DC |
|  |  | CellGroupConfig with condition SCell_add | OCTET STRING (CONTAINING CellGroupConfig) | SCell_add |
|  |  | CellGroupConfig with condition MEAS | OCTET STRING (CONTAINING CellGroupConfig) | NR_MEAS |
|  |  | CellGroupConfig with condition REEST | OCTET STRING (CONTAINING CellGroupConfig) | REEST |
|  |  | CellGroupConfig with condition RELAY | OCTET STRING (CONTAINING CellGroupConfig) | RELAY |
| fullConfig |  | Not present |  |  |
| dedicatedNAS-MessageList |  | Not present |  |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message { |  | 1 entry |  | NR |
| DedicatedNAS-Message[1] |  | DedicatedNAS-Message | entry 1 A sequence of OCTET STRING (s) containing one or more DedicatedNAS-Message(s) |  |
| } |  |  |  |  |
| masterKeyUpdate |  | Not present |  |  |
| masterKeyUpdate SEQUENCE { |  |  |  | MasterKeyChange |
| keySetChangeIndicator |  | true |  |  |
| nextHopChainingCount |  | NextHopChainingCount |  |  |
| nas-Container |  | Not present |  |  |
|  |  | OCTET STRING including the 10 Octets value generated according to TS 24.501 [28] clause 9.11.2.9 |  | Inter_Sys_HO |
| } |  |  |  |  |
| dedicatedSIB1-Delivery |  | Not present |  |  |
| dedicatedSystemInformationDelivery |  | Not present |  |  |
| otherConfig |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | NR-DC, SIDELINK, DAPS_HO_ReleaseSource, CHO, CPC, NE-DC,L2RelayUE, L2RemoteUE, SHRT304, SHRT310, SHRT312, SHRRLF, DeactivatedSCG |
| otherConfig-v1540 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| mrdc-SecondaryCellGroupConfig |  | Not present |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  | NR-DC, NE-DC |
| setup SEQUENCE { |  |  |  |  |
| mrdc-ReleaseAndAdd |  | Not present |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |  |
| nr-SCG |  | RRCReconfiguration with condition NR-DC_SCG | OCTET STRING (CONTAINING RRCReconfiguration) | NR-DC |
| eutra-SCG |  | RRCConnectionReconfiguration according TS 36.508 [2], table 4.6.1-8 with condition NE-DC | OCTET STRING (CONTAINING RRCConnectionReconfiguration) | NE-DC |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| radioBearerConfig2 |  | Not present |  | NE-DC |
|  |  | RadioBearerConfig with condition DRBn and SecondaryKeys | OCTET STRING (CONTAINING RadioBearerConfig) | NR-DC |
| sk-Counter |  | SK-Counter |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | SIDELINK, DAPS_HO_ReleaseSource, CHO, CPC,L2RelayUE, L2RemoteUE, SHRT304, SHRT310, SHRT312, SHRRLF, DeactivatedSCG |
| otherConfig-v1610 |  | Not present |  |  |
| bap-Config-r16 |  | Not present |  |  |
| iab-IP-AddressConfigurationList-r16 |  | Not present |  |  |
| conditionalReconfiguration-r16 |  | Not present |  |  |
|  |  | ConditionalReconfiguration |  | CHO, CPC |
| daps-SourceRelease-r16 |  | Not present |  |  |
|  |  | true |  | DAPS_HO_ReleaseSource |
| t316-r16 |  | Not present |  |  |
| needForGapsConfigNR-r16 |  | Not present |  |  |
| onDemandSIB-Request-r16 |  | Not present |  |  |
| dedicatedPosSysInfoDelivery-r16 |  | Not present |  |  |
| sl-ConfigDedicatedNR-r16 |  | Not present |  |  |
| sl-ConfigDedicatedNR-r16 CHOICE { |  |  |  | SIDELINK |
| setup |  | SL-ConfigDedicatedNR-r16 |  |  |
| } |  |  |  |  |
| sl-ConfigDedicatedEUTRA-Info-r16 |  | Not present |  |  |
| smtc-r16 |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | L2RelayUE, L2RemoteUE, SHRT304, SHRT310, SHRT312, SHRRLF, DeactivatedSCG |
| otherConfig-v1700 |  | Not present |  |  |
| otherConfig-v1700 SEQUENCE{ |  |  |  | SHRT304, SHRT310, SHRT312, SHRRLF |
| ul-GapFR2-PreferenceConfig-r17 |  | Not present |  |  |
| musim-GapAssistanceConfig-r17 |  | Not present |  |  |
| musim-LeaveAssistanceConfig-r1 |  | Not present |  |  |
| successHO-Config-r17 |  | Not present |  |  |
| successHO-Config-r17 CHOICE { |  |  |  | SHRT304, SHRT310, SHRT312, SHRRLF |
| setup SEQUENCE { |  |  |  |  |
| thresholdPercentageT304-r17 |  | Not present |  |  |
|  |  | p40 |  | SHRT304 |
| thresholdPercentageT310-r17 |  | Not present |  |  |
|  |  | p40 |  | SHRT310 |
| thresholdPercentageT312-r17 |  | Not present |  |  |
|  |  | p20 |  | SHRT312 |
| sourceDAPS-FailureReporting-r17 |  | Not present |  |  |
|  |  | true |  | SHRRLF |
| } |  |  |  |  |
| } |  |  |  |  |
| maxBW-PreferenceConfigFR2-2-r17 |  | Not present |  |  |
| maxMIMO-LayerPreferenceConfigFR2-2-r17 |  | Not present |  |  |
| minSchedulingOffsetPreferenceConfigExt-r17 |  | Not present |  |  |
| rlm-RelaxationReportingConfig-r17 |  | Not present |  |  |
| bfd-RelaxationReportingConfig-r17 |  | Not present |  |  |
| scg-DeactivationPreferenceConfig-r17 |  | Not present |  |  |
| rrm-MeasRelaxationReportingConfig-r17 |  | Not present |  |  |
| propDelayDiffReportConfig-r17 |  | Not present |  |  |
| } |  |  |  |  |
| sl-L2RelayUE-Config-r17 |  | Not present |  |  |
| sl-L2RelayUE-Config-r17 CHOICE { |  |  |  |  |
| setup |  | SL-L2RelayUE-Config with condition SL-SRB0 and SL-SRB1 |  | L2RelayUE |
| } |  |  |  |  |
| sl-L2RemoteUE-Config-r17 |  | Not present |  |  |
| sl-L2RemoteUE-Config-r17 CHOICE { |  |  |  | L2RemoteUE |
| setup |  | SL-L2RemoteUE-Config with condition SL-SRB2 and SL-DRB1 |  |  |
| } |  |  |  |  |
| dedicatedPagingDelivery-r17 |  | Not present |  |  |
| needForGapNCSG-ConfigNR-r17 |  | Not present |  |  |
| needForGapNCSG-ConfigEUTRA-r17 |  | Not present |  |  |
| musim-GapConfig-r17 |  | Not present |  |  |
| ul-GapFR2-Config-r17 |  | Not present |  |  |
| ul-GapFR2-Config-r17 CHOICE { |  |  |  | NR-FR2- Uplink-Gaps |
| setup |  | UL-GapFR2-Config-r17 |  |  |
| } |  |  |  |  |
| scg-State-r17 |  | Not present |  |  |
|  |  | deactivated |  | DeactivatedSCG |
| appLayerMeasConfig-r17 |  | Not present |  |  |
| ue-TxTEG-RequestUL-TDOA-Config-r17 |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |
| EN-DC_MEAS | An EN-DC measurement is configured |
| IRAT_MEAS | An IRAT measurement is configured in the NR PCell |
| NR_MEAS | A NR measurement is configured |
| NR | NR connected to 5GC |
| NR-DC | NR-NR Dual Connectivity is configured |
| NR-DC_SCG | Add SCG side configuration (NR-DC) |
| EN-DC_HO | NR PSCell change (EN-DC) |
| SCell_add | Add SCell |
| EN-DC_SCell_add | Add Scell (EN-DC) |
| MasterKeyChange | MasterKeyUpdate when performing ReconfigurationWithSync and indicating a change of the AS security algorithms associated to the master key |
| Inter_Sys_HO | Used during inter-system handover to NR procedure |
| REEST | The first RRCReconfiguration message after successful completion of the RRC re-establishment procedure |
| SIDELINK | For NR sidelink dedicated configuration |
| DAPS_HO_ReleaseSource | The source cell part of DAPS operation is to be stopped and the source cell part of DAPS configuration is to be released. |
| CHO | Conditional handover |
| CPC | Conditional PSCell change |
| NE-DC | NR E-UTRA Dual Connectivity |
| L2RelayUE | For L2 U2N Relay UE test cases. |
| L2RemoteUE | For L2 U2N Remote UE test cases. |
| RELAY | Add Uu Relay RLC channel configuration |
| NR-FR2-Upink-Gaps | Configure Uplink Gaps for NR FR2 |
| SHRT304 | Report the successful handover information to the network for the elapsed time of the timer T304 over the threshold |
| SHRT310 | Report the successful handover information to the network for the elapsed time of the timer T310 over the threshold |
| SHRT312 | Report the successful handover information to the network the ror elapsed time of the timer T312 over the threshold |
| SHRRLF | Report the successful handover information to the network for radio link failure |
| DeactivatedSCG | Indicate that SCG is in deactivated state |

<Unchanged Skipped>

–	CellGroupConfig

Table 4.6.3-19: CellGroupConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| cellGroupId | CellGroupId |  |  |
|  | CellGroupId condition NR-DC_SCG |  | NR-DC_SCG |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  | EN-DC |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and DRB2 | entry 1 |  |
|  | RLC-BearerConfig with conditions AM and DRB2 and Re-establish_RLC |  | PSCell_change |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  | SRB1 |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB1 | entry 1 |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  | SRB2_DRB1 |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and DRB1 | entry 2 |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  | SRB2_DRB2 |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and DRB2 | entry 2 |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  | DRBn, NR-DC_SCG |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and DRBn | entry 1 DRBn is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 3 entries |  | PCell_change |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions SRB1 and Re-establish_RLC | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions SRB2 and Re-establish_RLC | entry 2 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM, DRB1 and Re-establish_RLC | entry 3 |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1+n entries | n is the number of DRBs established before RRC resume | RESUME |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 and RESUME | entry 1 |  |
| RLC-BearerConfig[k+1, k=1..n] | RLC-BearerConfig with condition DRBk and RESUME | entry [k+1, k=1..n] |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2+n entries | n is the number of DRBs established before RRC re-establishement | REEST |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB1 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with condition SRB2 and Re-establish_RLC | entry 2 |  |
| RLC-BearerConfig[k+2, k=1..n] | RLC-BearerConfig with condition DRBk and Re-establish_RLC | entry [k+2, k=1..n] |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  | MRBm AND (AM_PTP OR UM_bi_PTP OR UM_PTP OR UM_PTM) |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and PTP | entry 1 | AM_PTP |
|  | RLC-BearerConfig with conditions UM and PTP |  | UM_bi_PTP |
|  | RLC-BearerConfig with conditions UM_DLonly and PTP |  | UM_PTP |
|  | RLC-BearerConfig with conditions UM_DLonly and PTM |  | UM_PTM |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  | MRBm_DRBn AND (AM_PTP OR UM_bi_PTP OR UM_PTP OR UM_PTM) |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and PTP and MRBm | entry 1 | AM_PTP |
|  | RLC-BearerConfig with conditions UM and PTP and MRBm |  | UM_bi_PTP |
|  | RLC-BearerConfig with conditions UM_DLonly and PTP and MRBm |  | UM_PTP |
|  | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm |  | UM_PTM |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and DRBn | entry 2 DRBn is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  | MRBm AND (AMPTP_UMPTM OR UMPTP_UMPTM) |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and PTP and MRBm | entry 1 | AMPTP_UMPTM |
|  | RLC-BearerConfig with conditions UM_DLonly and PTP and MRBm |  | UMPTP_UMPTM |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 2 | AMPTP_UMPTM, UMPTP_UMPTM |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 3 entries |  | MRBm_DRBn AND (AMPTP_UMPTM OR UMPTP_UMPTM) |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and PTP and MRBm | entry 1 | AMPTP_UMPTM |
|  | RLC-BearerConfig with conditions UM_DLonly and PTP and MRBm |  | UMPTP_UMPTM |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 2 | AMPTP_UMPTM, UMPTP_UMPTM |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM and DRBn | entry 3 DRBn is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| rlc-BearerToAddModList | Not present |  |  |
| rlc-BearerToReleaseList | Not present |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig |  |  |
|  | MAC-CellGroupConfig with condition MBS_Multicast |  | AMPTP_UMPTM, UMPTP_UMPTM, UM_PTM |
|  | Not present |  | SRB2_DRB1, MEAS, SRB2_DRB2, SCell_add |
| physicalCellGroupConfig | PhysicalCellGroupConfig |  |  |
|  | Not present |  | SRB2_DRB1, MEAS, SRB2_DRB2, SCell_add, MRBm, MRBm_DRBn |
| spCellConfig | Not present |  | SRB2_DRB1, SRB2_DRB2, SCell_add, AM_PTP, UM_bi_PTP, UM_PTP |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present |  |  |
|  | ServCellIndex |  | EN-DC, EN-DC AND MEAS |
|  | ServCellIndex with condition NR-DC_SCG |  | NR-DC_SCG |
| reconfigurationWithSync | Not present |  |  |
| reconfigurationWithSync SEQUENCE { |  |  | EN-DC, PCell_change, PSCell_change, NR-DC_SCG |
| spCellConfigCommon | ServingCellConfigCommon |  |  |
| newUE-Identity | RNTI-Value |  |  |
|  | RNTI-Value with condition NR-DC_SCG |  | NR-DC_SCG |
| t304 | ms1000 |  |  |
| rach-ConfigDedicated | Not present |  |  |
| rach-ConfigDedicated CHOICE { |  |  | CFRA |
| uplink | RACH-ConfigDedicated |  |  |
| supplementaryUplink | RACH-ConfigDedicated |  | SUL AND SIG |
| } |  |  |  |
| } |  |  |  |
| rlf-TimersAndConstants CHOICE { |  |  |  |
| setup | RLF-TimersAndConstants |  |  |
| } |  |  |  |
| rlf-TimersAndConstants | Not present |  | MEAS, RESUME |
| rlmInSyncOutOfSyncThreshold | Not present |  |  |
| spCellConfigDedicated | ServingCellConfig |  | EN-DC, SRB1, PCell_change, PSCell_change, NR-DC_SCG, REEST |
|  | Not present |  |  |
|  | ServingCellConfig with condition MEAS |  | MEAS |
|  | ServingCellConfig with condition RESUME |  | RESUME |
|  | ServingCellConfig with condition MBS_Multicast |  | AMPTP_UMPTM, UMPTP_UMPTM, UM_PTM |
| lowMobilityEvaluationConnected-r17 | Not present |  |  |
| goodServingCellEvaluationRLM-r17 | Not present |  |  |
| goodServingCellEvaluationBFD-r17 | Not present |  |  |
| deactivatedSCG-Config-r17 | Not present |  |  |
| deactivatedSCG-Config-r17 CHOICE { |  |  | RLM_BFD_PSCell |
| setup SEQUENCE { |  |  |  |
| bfd-and-RLM-r17 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sCellToAddModList | Not present |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  | SCell_add |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | SCellIndex |  |  |
| sCellConfigCommon | ServingCellConfigCommon with condition No_UL and SCell_add |  |  |
|  | ServingCellConfigCommon with condition SCell_add |  | RF AND UL_CA |
| sCellConfigDedicated | ServingCellConfig with condition No_UL and Scell_Add |  |  |
|  | ServingCellConfig with condition Scell_Add |  | RF AND UL_CA |
| } |  |  |  |
| } |  |  |  |
| sCellToReleaseList | Not present |  |  |
| f1c-TransferPathNRDC-r17 | Not present |  |  |
| uplinkTxSwitching-2T-Mode-r17 | Not present |  |  |
| uplinkTxSwitching-DualUL-TxState-r17 | Not present |  |  |
| uu-RelayRLC-ChannelToAddModList-r17 | Not present |  |  |
| uu-RelayRLC-ChannelToAddModList-r17 SEQUENCE (SIZE (1.. maxUu-RelayRLC-ChannelID-r17)) OF Uu-RelayRLC-ChannelConfig-r17{ |  |  | RELAY |
| Uu-RelayRLC-ChannelConfig-r17[1] | Uu-RelayRLC-ChannelConfig |  |  |
| } |  |  |  |
| uu-RelayRLC-ChannelToReleaseList-r17 | Not present |  |  |
| simultaneousU-TCI-UpdateList1-r17 | Not present |  |  |
| simultaneousU-TCI-UpdateList2-r17 | Not present |  |  |
| simultaneousU-TCI-UpdateList3-r17 | Not present |  |  |
| simultaneousU-TCI-UpdateList4-r17 | Not present |  |  |
| rlc-BearerToReleaseListExt-r17 | Not present |  |  |
| iab-ResourceConfigToAddModList-r17 | Not present |  |  |
| iab-ResourceConfigToReleaseList-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |
| CFRA | This condition applies when CFRA is configured |
| SUL | Supplementary Uplink |
| SRB1 | Establishment of SRB1 |
| SRB2_DRB1 | Establishment of SRB2 and DRB1 |
| SRB2_DRB2 | Establishment of SRB2 and DRB2 |
| DRBn | Establishment of DRBn |
| PCell_change | Intra-NR PCell change NR) |
| PSCell_change | NR PSCell change (EN-DC) |
| SCell_add | Add SCell |
| MEAS | A NR or EN-DC measurement is configured |
| NR-DC_SCG | Add SCG (NR-DC) |
| RESUME | Used in RRCResume Message |
| REEST | The first RRCReconfiguration message after successful completion of the RRC re-establishment procedure. |
| UL_CA | Uplink CA |
| MRBm | Establishment of MRBm |
| MRBm_DRBn | Establishment of MRBm and DRBn |
| AM_PTP | Multicast MRB with RLC-AM entity configuration for receiving PTP transmission |
| UM_bi_PTP | Multicast MRB with bidirectional RLC-UM configuration for receiving PTP transmission |
| UM_PTP | Multicast MRB with DL only RLC-UM configuration for receiving PTP transmission |
| UM_PTM | Multicast MRB with DL only RLC-UM entity for receiving PTM transmission |
| AMPTP_UMPTM | Multicast MRB with two RLC entities, one RLC-AM entity for receiving PTP transmission and the other DL only RLC-UM entity for receiving PTM transmission |
| UMPTP_UMPTM | Multicast MRB with two RLC-UM entities, one DL only RLC-UM entity for receiving PTP transmission and the other DL only RLC-UM entity for receiving PTM transmission |
| RELAY | This condition applies when Relay UE is configured |
| RLM_BFD_PSCell | Perform RLM and BFD on the PSCell when the SCG is deactivated |

<Unchanged Skipped>

–	ServingCellConfig

Table 4.6.3-167: ServingCellConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| tdd-UL-DL-ConfigurationDedicated | Not present |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated |  |  |
|  | BWP-DownlinkDedicated with condition Scell_Add |  | Scell_Add |
|  | BWP-DownlinkDedicated with condition MBS_Multicast |  | MBS_Multicast |
|  | Not present |  | MEAS, RESUME |
| downlinkBWP-ToReleaseList | Not present |  |  |
| downlinkBWP-ToAddModList | Not present |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 1 entry |  | DLBWP_add |
| BWP-Downlink[1] | BWP-Downlink |  |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | BWP-Id |  |  |
|  | Not present |  | MEAS |
| bwp-InactivityTimer | Not present |  |  |
| defaultDownlinkBWP-Id | BWP-Id |  |  |
|  | Not present |  | MEAS, RESUME |
| uplinkConfig | Not present |  | MEAS, No_UL |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
|  | BWP-UplinkDedicated with condition SUL_NUL |  | PUSCH_PUCCH_ON_SUL |
|  | BWP-UplinkDedicated with condition RESUME |  | RESUME |
| uplinkBWP-ToReleaseList | Not present |  |  |
| uplinkBWP-ToAddModList | Not present |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 1 entry |  | ULBWP_add |
| BWP-Uplink[1] | BWP-Uplink |  |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | BWP-Id |  |  |
| pusch-ServingCellConfig | Not present |  | RESUME |
| pusch-ServingCellConfig CHOICE { |  |  |  |
| setup | PUSCH-ServingCellConfig |  |  |
| } |  |  |  |
| carrierSwitching | Not present |  |  |
| } |  |  |  |
| supplementaryUplink | Not present |  |  |
| supplementaryUplink SEQUENCE { |  |  | PUSCH_PUCCH_ON_SUL |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
|  | BWP-UplinkDedicated with condition RESUME |  | RESUME |
| uplinkBWP-ToReleaseList | Not present |  |  |
| uplinkBWP-ToAddModList | Not present |  |  |
| firstActiveUplinkBWP-Id | BWP-Id |  |  |
| pusch-ServingCellConfig CHOICE { |  |  |  |
| setup | PUSCH-ServingCellConfig |  |  |
| } |  |  |  |
| carrierSwitching | Not present |  |  |
| } |  |  |  |
| pdcch-ServingCellConfig CHOICE { |  |  |  |
| setup | PDCCH-ServingCellConfig |  |  |
| } |  |  |  |
| pdcch-ServingCellConfig | Not present |  | MEAS, RESUME |
| pdsch-ServingCellConfig CHOICE { |  |  |  |
| setup | PDSCH-ServingCellConfig |  |  |
| } |  |  |  |
| pdsch-ServingCellConfig | Not present |  | MEAS, RESUME |
| csi-MeasConfig | Not present |  |  |
| sCellDeactivationTimer | Not present |  |  |
| crossCarrierSchedulingConfig | Not present |  |  |
| tag-Id | 0 |  |  |
| dummy1 | Not present |  |  |
| pathlossReferenceLinking | Not present |  |  |
| servingCellMO | Not present |  |  |
|  | MeasObjectId |  | MEAS |
| nr-dl-PRS-PDC-Info-r17 | Not present |  |  |
| semiStaticChannelAccessConfigUE-r17 | Not present |  |  |
| mimoParam-r17 | Not present |  |  |
| channelAccessMode2-r17 | Not present |  |  |
| timeDomainHARQ-BundlingType1-r17 | Not present |  |  |
| nrofHARQ-BundlingGroups-r17 | Not present |  |  |
| fdmed-ReceptionMulticast-r17 | Not present |  |  |
| moreThanOneNackOnlyMode-r17 | Not present |  |  |
| tci-ActivatedConfig-r17 | Not present |  |  |
| tci-ActivatedConfig-r17 SEQUENCE { |  |  | SCG_Activate |
| pdcch-TCI-r17 SEQUENCE (SIZE(1..5)) OF TCI-StateId { | 1 entry |  |  |
| TCI-StateId[1] | TCI-StateId | entry 1 |  |
| } |  |  |  |
| pdsch-TCI-r17 | 1 | BIT STRING (SIZE (1..maxNrofTCI-States)) |  |
| } |  |  |  |
| directionalCollisionHandling-DC-r17 | Not present |  |  |
| lte-NeighCellsCRS-AssistInfoList-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PUSCH_PUCCH_ON_SUL | For the purpose of SUL test under condition that supplementary uplink is configured with both PUSCH and PUCCH on SUL carrier. |
| MEAS | A NR or EN-DC measurement is configured. |
| No_UL | No uplink CA |
| RESUME | Used in RRCResume Message |
| Scell_Add | Add SCell |
| MBS_Multicast | Used for MBS Multicast reception |
| DLBWP_add | A dedicated DL BWP is configured |
| ULBWP_add | A dedicated UL BWP is configured |
| SCG_Activate | When SCG is being activated |

<End of Changes>