3GPP TSG-RAN WG5 Meeting #102	R5-240970

Athens, Greece, 26th February – 3rd March 2024

Title:	Test frequencies calculation review for asymmetric channel bandwidths

Source:	Keysight Technologies UK Ltd, Nokia, Skyworks Solutions Inc., T-Mobile USA

Agenda item:	5.4.20

Document for:	Discussion and endorsement

# 1.	Introduction

RAN4 clarified that deviation to the default Tx-Rx carrier center frequency separation allowed when using asymmetric bandwidth (as defined in 3GPP TS 38.101-1 [1] clause 5.3.6) was indeed the maximum permitted deviation [2]:

In the same CR [2], for certain bands using asymmetric channel bandwidths a note 5 was added restricting the Tx-Rx carrier center frequency separation for REFSENS UL configuration when using those asymmetric channel bandwidths:

However, these changes were missed in 3GPP TS 38.508-1 Annex C.2.3 [3] when defining the criterion to choose ΔFTX-RX:

Additionally, the changes in 3GPP TS 38.101-1 [1] described above are being further clarified in RAN4#110 as per [4]:

Hence, the guidelines to calculate the test frequencies for asymmetric bandwidths in C.2.3 as well as the impacted test frequencies need to be updated in 3GPP TS 38.508-1 [3]. This document discusses how to shape the required updates.

# 2. 	Discussion

## 2.1 Impacted frequency bands, channel bandwidths and nominal/default Tx-Rx carrier center frequency separation

### 2.1.1 Frequency bands impacted by Note 5 in table 7.3.2-2 in 3GPP TS 38.101-1 [1]

In order to understand which frequency bands and which bandwidths are impacted by the issue detected, asymmetric channel bandwidths defined in 3GPP TS 38.101-1 [1], clause 5.3.6 have been analysed as per figure below:

Observation 1: Not all frequency bands using asymmetric bandwidths are impacted by Note 5 in table 7.3.2-2 in 3GPP TS 38.101-1 [1].

Observation 2: Not all asymmetric channel bandwidths combinations within a frequency band are necessarily impacted by Note 5 in table 7.3.2-2 in 3GPP TS 38.101-1 [1].

According to latest version of PRD21 [5], all the impacted asymmetric channel bandwidth combinations correspond to either Rel-17 or Rel-18:

Observation 3: Asymmetric channel bandwidths combinations impacted by Note 5 in table 7.3.2-2 in 3GPP TS 38.101-1 [1] were added either in Rel-17 or Rel-18.

### 2.1.2 Frequency bands exercising asymmetric channel bandwidths impacted by Note 5 in table 7.3.2-2 in 3GPP TS 38.101-1 in conformance testing

If channel bandwidths under test according to 3GPP TS 38.508-1 [3] sections 4.3.10A, 4.3.1.0B and 4.3.10C are analysed, it can be observed that asymmetric channel bandwidths impacted by Note 5 in table 7.3.2-2 in 3GPP TS 38.101-1 [1] are exercised in conformance testing only when testing High channel bandwidth for bands n5, n8, n25 and n71.

Observation 4: Asymmetric channel bandwidths impacted by Note 5 in table 7.3.2-2 in 3GPP TS 38.101-1 [1] are exercised in conformance testing only when testing High channel bandwidth for bands n5, n8, n25 and n71.

### 2.1.3 Frequency bands categories according to its Tx-Rx carrier centre frequency separation

When analysing 3GPP TS 38.101-1 [1] table 5.4.4-1, there are basically 3 types of frequency bands according to its nominal/default Tx-Rx carrier center frequency separation:

- Type 1: Frequency bands using 2 values for nominal/default Tx-Rx carrier center frequency separation (n24).

- Type 2: Frequency bands using a range for nominal/default Tx-Rx carrier center frequency separation (n91, n92, n93, n94, n109)

- Type 3: Frequency bands using a single value for nominal/default Tx-Rx carrier center frequency separation (others).

However, frequency bands impacted by the issue detected are always using a single value for nominal/default Tx-Rx carrier center frequency separation, as shown in the snapshot below:

Observation 5: All impacted frequency bands use a single value for nominal/default Tx-Rx frequency separation.

### 2.1.4 Proposal

Clarification added to the definition of ΔFTX-RX in 3GPP TS 38.101-1 [1] as per CR in [2] was:

Considering that actual UL/DL carrier frequency separation ΔFTX-RX has a direct impact on sensitivity performance in FDD bands due to maximum sensitivity degradation (MSD) in the receiver when the power level to be received is very low and the transmitted power level is high, multiple options on asymmetric channel bandwidths/frequency bands combinations could be considered to shape required changes in 3GPP TS 38.508-1 [3]:

- OPTION 1.1: Only test frequencies for asymmetric channel bandwidths/frequency bands combinations impacted by Note 5 in table 7.3.2-2 in 3GPP TS 38.101-1 [1] are modified (i.e. n5, n8, n25, n26, n71, n105).

- OPTION 1.2: Only test frequencies for asymmetric channel bandwidths/frequency bands combinations with a fixed default/nominal Tx-Rx frequency separation are modified (which will add n66 and n70 to the frequency band list under OPTION 1.1 + all permitted asymmetric channel bandwidths combinations for the frequency band list)

- OPTION 1.3: All frequency bands using asymmetric channel bandwidths (which will add n24, n91, n92, n93, n94 and n109 to the frequency band list under OPTION 1.2)

With the goal to minimize the impact of the changes, it is proposed that updates to 3GPP TS 38.508-1 [3] Annex C.3.2 are such that only frequency bands impacted by Note 5 in table 7.3.2-2 in 3GPP TS 38.101-1 [1] or equivalent (to make it future proof) require modification (OPTION 1.1).

Proposal 1: Updates to 3GPP TS 38.508-1 [3] Annex C.3.2 are such that only asymmetric channel bandwidths/frequency bands combinations impacted by Note 5 in table 7.3.2-2 in 3GPP TS 38.101-1 [1] will be modified at this stage.

Proposal 2: Further discuss whether actual UL/DL carrier frequency separation ΔFTX-RX used to calculate test frequencies for other asymmetric channel bandwidths/frequency bands combinations require a revision.

## 2.2 Impacted test cases

When shaping required changes in 3GPP TS 38.508-1 [3], the other aspect to be discussed is whether changes in the calculation of test frequencies should impact all test cases or just a reduced set of test cases (the ones related to Note 5 in Table 7.3.2-3 in 3GPP TS 38.101-1 [1] either directly, as in REFSENS test case , or indirectly using REFSENS value as a test parameter, as ACS or blocking tests).

Two options could be considered:

- OPTION 2.1: Redefine asymmetric channel bandwidths test frequencies calculation similarly for all test cases.

- OPTION 2.2: Redefine asymmetric channel bandwidths test frequencies calculation only for test cases impacted all test cases, while keeping existing test frequencies for other test cases.

Proposal 3: To simplify test specifications, it is proposed to redefine asymmetric channel bandwidths test frequencies calculation applicable to all test cases.

# 3. 	Conclusion

This document shows that guidelines to calculate the test frequencies for asymmetric bandwidths in C.2.3 as well as the impacted test frequencies need to be updated in 3GPP TS 38.508-1 [3]. It also discusses how to shape the required updates.

The following observations and proposals are made:

Observation 1: Not all frequency bands using asymmetric bandwidths are impacted by Note 5 in table 7.3.2-2 in 3GPP TS 38.101-1 [1].

Observation 2: Not all asymmetric channel bandwidths combinations within a frequency band are necessarily impacted by Note 5 in table 7.3.2-2 in 3GPP TS 38.101-1 [1].

Observation 3: Asymmetric channel bandwidths combinations impacted by Note 5 in table 7.3.2-2 in 3GPP TS 38.101-1 [1] were added either in Rel-17 or Rel-18.

Observation 4: Asymmetric channel bandwidths impacted by Note 5 in table 7.3.2-2 in 3GPP TS 38.101-1 [1] are exercised in conformance testing only when testing High channel bandwidth for bands n5, n8, n25 and n71

Observation 5: All impacted frequency bands use a single value for nominal/default Tx-Rx frequency separation.

Proposal 1: Updates to 3GPP TS 38.508-1 [3] Annex C.3.2 are such that only asymmetric channel bandwidths/frequency bands combinations impacted by Note 5 in table 7.3.2-2 in 3GPP TS 38.101-1 [1] will be modified at this stage.

Proposal 2: Further discuss whether actual UL/DL carrier frequency separation ΔFTX-RX used to calculate test frequencies for other asymmetric channel bandwidths/frequency bands combinations require a revision.

Proposal 3: To simplify test specifications, it is proposed to redefine asymmetric channel bandwidths test frequencies calculation applicable to all test cases.

These proposals are submitted for approval in [6-8].

# 4. 	References

- 3GPP TS 38.101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

- R4-2115101, “CR to TS 38.101-1 – Introduction of 35MHz and 45MHz channel bandwidth”, Skyworks Solutions Inc., Apple, RAN4#100-e.

- 3GPP TS 38.508-1, “5GS; User Equipment (UE) conformance specification; Part 1: Common test environment”

- R4-2400358, “CR to TS 38.101-1 Rel-17 Corrections to UL configuration for asymmetric ULDL”, Skyworks Solutions, Inc., Nokia, T-Mobile USA, Keysight, RAN4#110

- R5-236670, “3GPP RAN5 PRD 21: Permanent Reference Document (PRD); NR bands and 5G NR CADC configuration handling in RAN5”

- R5-240971, “Annex C update to asymmetric channel bandwidths test frequencies calculation”, Keysight UK Ltd, Nokia, Skyworks Solutions Inc., T-Mobile USA, RAN5#102.

- R5-240468, “Test frequencies and channel bandwidth updates for band n71 and n25 in Rel-17”, Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc., Keysight Technologies, RAN5#102

- R5-240972, “Asymmetric channel bandwidths test frequencies updates for frequency bands n5 and n8”, Keysight UK Ltd, Nokia, Skyworks Solutions Inc., T-Mobile USA, RAN5#102.