3GPP TSG-RAN5 Meeting #102	R5-241025

# Athens, Greece, February 26 – March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2698 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 6.3A.3.5 2Tx-2Tx UL switching between two bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 has introduced time mask requirement for 2Tx-2Tx UL switching between two bands. RAN5 testing is still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the test case of time mask for 2Tx-2Tx UL switching between two bands |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 2Tx-2Tx UL switching between two bands couldn’t be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.3.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.3A.3.3.5	Test requirement

The requirement for the power of carrier 1 measured in step 1.6, 1.9, 2.6, 2.9 of the test procedure and the power of carrier 2 measured in step 1.7 and 2.7 shall not exceed the values specified in table 6.3A.3.3.5-1.

Table 6.3A.3.3.5-1: Time mask for 2Tx-2Tx switching between two uplink carriers (On power)

|  | Measured output power |
| --- | --- |
| Transmit ON power | Same as Test ID 1 of Table 6.2D.2.5-1 as appropriate |
| NOTE1: TT or each frequency and channel bandwidth of Transmit ON power is specified in Table 6.2A.2.1.5-3 |  |

6.3A.3.5	Time mask for switching between two uplink bands with two transmit antenna connectors (3UL CA)

6.3A.3.5.1	Test purpose

To verify that the time mask for switching between two uplink bands with two transmitter connectors meets the requirements given in 6.3A.3.0.3.5.

The time mask for switching between NR UL carrier 1 in band A, which capable of two transmit antenna connectors, and NR UL carrier 2 and carrier 3 in band B, which are capable of two transmit antenna connectors defines the transient period(s) allowed when the capability uplinkTxSwitchingPeriod2T2T is present.

6.3A.3.5.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support 3UL inter-band CA with intra-band contiguous CA and dynamic UL 2Tx-2Tx switching.

6.3A.3.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3A.3.0.3.5.

6.3A.3.5.4	Test description

6.3A.3.5.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in table 6.3A.3.5.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3A.3.5.4.1-1: Test Configuration Table for inter-band CA with intran-band contiguous CA 2Tx-2Tx Uplink switching

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.1.3 for inter band CA in FR1 |  | Mid range for PCC and SCCs |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Highest NRB on PCC and Highest NRB_agg on SCCs |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  | Highest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation |  |
|  |  |  | PCC (NOTE 2) | SCCs (NOTE 3) |
| 1 | N/A for this test | CP-OFDM QPSK | Inner Full | Inner Full |
| NOTE 1: PCC is the component carrier on the band with single carrier configured and SCCs are the contiguous aggregated component carriers on another band. PCC is configured as Carrier 1 and both SCCs are configured as Carrier2. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: The specific configuration of each RB allocation is defined in Table 6.1A-1a. NOTE 4: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A.3-1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.6 for TE diagram and section A.3.2.3.11 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3A.3.5.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3A.3.5.4.3.

6.3A.3.5.4.2	Test procedure

In this test case, one component carrier (PCC) is configured in band A and two contiguous aggregated carriers (SCCs) are configured in band B. The carrier in band A is configured as Carrier 1, and both carriers in band B is configured as Carrer 2.

1.	Sub test 1: Switching period located in Carrier 1

1.1.	Configure SCCs according to Annex C.0, C.1, C.2 for all downlink physical channels.

1.2.	The SS shall configure SCCs as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.3A.3.5.4.3 with uplinkTxSwitchingPeriodLocation-r16 configured TRUE on carrier1 and FALSE on carrier 2.

1.3.	SS activates SCCs by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133 [19], clause 9.3).

1.3a.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.5.4.1-1 on PCC only. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PCMAX,c level on PCC.

1.3b.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.5.4.1-1 on both SCCs. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PCMAX level on the SCCs.

1.4.	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.5.4.1-1 on carrier 1 on slot n-1, where slot n is an uplink slot for carrier 2. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

1.5.	The SS sends uplink scheduling information via DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.5.4.1-1 on carrier 2 (SCCs) on slot n and slot m, with both slot n and slot m being uplink slots for carrier 2 and m ≥ n+20 when SCS=15kHz (m ≥ n+40 when SCS=30 kHz, m ≥ n+80 when SCS=60 kHz). Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

1.6.	Measure the sum of output power of UE PUSCH transmission for carrier 1 over all antenna connectors during slot n-1 excluding a transient period of 10 µs and a Switching period X µs in the end of slot n-1. The length of uplink switching period X is indicated by UE capability uplinkTxSwitchingPeriod2T2T.

1.7.	Measure the sum of output power of UE PUSCH transmission on carrier 2 (SCCs) over all antenna connectors and both component carriers during slot n and slot m excluding a transient period of 10 µs in the beginning of slot n and in the end of slot m.

1.8.	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.5.4.1-1 on carrier 1 on slot m+1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

1.9.	Measure the sum of output power of UE PUSCH transmission for carrier 1 over all antenna connectors during slot m+1 excluding a switching period X and a transient period of 10 µs in the beginning of slot m+1. The length of uplink switching period X is indicated by UE capability uplinkTxSwitchingPeriod2T2T-r17.

2.	Sub test 2: Switching period located in Carrier 2

2.1.	Configure SCCs according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.2.	The SS shall configure SCCs as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.3A.3.5.4.3 with uplinkTxSwitchingPeriodLocation-r16 configured TRUE on carrier 2 and FALSE on carrier 1.

2.3.	SS activates SCCs by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133 [19], clause 9.3).

2.3a.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.5.4.1-1 on PCC only. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PCMAX,c level on PCC.

2.3b.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.5.4.1-1 on both SCCs. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PCMAX level on SCCs.

2.4.	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.5.4.1-1 on carrier 1 on slot n-1, where slot n is an uplink slot for carrier 2. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.5.The SS sends uplink scheduling information via DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.5.4.1-1 on carrier 2 (SCCs) on slot n (n ≥1) and slot m, with both slot n and slot m being uplink slots for carrier 2 and m ≥ n+20 when SCS=15kHz (m ≥ n+40 when SCS=30 kHz, m ≥ n+80 when SCS=60 kHz). Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.6.	Measure the sum of output power of UE PUSCH transmission for carrier 1 over all antenna connectors during slot n-1, for carrier 1 excluding a transient period of 10 µs in the end of slot n-1.

2.7.	Measure the sum of output power of UE PUSCH transmission on carrier 2 (SCCs) over all antenna connectors and both component carrier s during during slot n and slot m excluding a switching period X and a transient period of 10 µs in the beginning of slot n and in the end of slot m. The length of uplink switching period X is indicated by UE capability uplinkTxSwitchingPeriod2T2T.

2.8.	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.5.4.1-1 on carrier 1 on slot m+1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.9.	Measure the sum of output power of UE PUSCH transmission for carrier 1 over all antenna connectors during slot m+1 excluding a transient period of 10 µs in the beginning of slot m+1.

6.3A.3.5.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO, with the following exceptions.

Table 6.3A.3.5.4.3-1: CellGroupConfig

| Derivation Path: TS 38.508-1[5], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| uplinkTxSwitchingOption-r16 | switchedUL |  | switchedUL OR Both |
|  | dualUL |  | dualUL |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| switchedUL | UE indicated supporting of switchedUL in uplinkTxSwitching-OptionSupport-r16 |
| dualUL | UE indicated supporting of dualUL in uplinkTxSwitching-OptionSupport-r16 |
| Both | UE indicated supporting of both in uplinkTxSwitching-OptionSupport-r16 |

Table 6.3A.3.5.4.3-2: ServingCellConfig

| Derivation Path: 38.508-1[5], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkTxSwitching-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| uplinkTxSwitchingPeriodLocation-r16 | TRUE |  | PL |
|  | FALSE |  | noPL |
| uplinkTxSwitchingCarrier-r16 | carrier1 |  | PCC |
|  | carrier2 |  | SCC |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PL | The location of UL Tx switching period is configured in this carrier |
| noPL | The location of UL Tx switching period is not configured in this carrier |

Table 6.3A.3.5.4.3-3: FrequencyInfoUL-SIB for PCC

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 23 |  |  |

Table 6.3A.3.5.4.3-4: FrequencyInfoUL for SCCs

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  |  |

6.3A.3.5.5	Test requirement

The requirement for the power of carrier 1 measured in step 1.6, 1.9, 2.6, 2.9 of the test procedures and the power of carrier 2 measured in step 1.7 and 2.7 shall not exceed the values specified in table 6.3A.3.5.5-1.

Table 6.3A.3.5.5-1: Time mask for 2Tx-2Tx switching between two bands (On power)

|  | Measured output power |
| --- | --- |
| Transmit ON power | PCC: Same as Test ID 1 of Table 6.2D.2.5-1 as appropriate SCCs: Same as Test ID 1 of Table 6.2H.1.2.5-1 or 6.2H.1.2.5-2 as appropriate |
| NOTE1: TT or each frequency and channel bandwidth of Transmit ON power is specified in Table 6.2A.2.1.5-3 |  |

6.3A.4	Power control for CA

6.3A.4.1	Absolute power tolerance for CA

6.3A.4.1.0	Minimum conformance requirements

The absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame at the start of a contiguous transmission or non-contiguous transmission with a transmission gap on each active component carriers larger than 20ms. The requirement can be tested by time aligning any transmission gaps on the component carriers.

For intra-band contiguous carrier aggregation the absolute power control tolerance per component carrier is given in Table 6.3.4.2.3-1.

For intra-band non-contiguous carrier aggregation the absolute power control tolerance per component carrier is given in Table 6.3.4.2.3-1.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3A.4.

< End of Changes >