3GPP TSG-RAN5 Meeting # 102	R5-241559

# Athens, Greece,  26th February - 1st March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1460 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definition of a TN-to-NTN sysInfo combination mapping for legacy test cases to be run in NTN environment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160, MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 09.02.2024 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In RAN5#101-agreed document R5-237126 the proposal to use in test environment for Vertical UEs NB-IoT NTN system information combinations 8 to 13 was adopted. Hower the mapping of the TN (legacy) system information combinations to the corresponding NTN ones is left undefined. - When constant ephemeris information is used for NGSO signalling test environment, UE needs to deactivate prediction of satellite trajectory, otherwise the Satellite-UE will be asynchronous in UL/DL. - The common test environments for NTN, applicable to SIG/RF/RRM, contain some pure signalling aspects. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduced Table 8.1.5B.1-1 defining a system information combinations mapping. - Prediction of satellite trajectory was deactivated for NGSO in the pre-test condition. - The pure signalling aspects of the common test environments for NTN have been moved to new clauses 6.3.6 & 8.3.8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The mapping of system informtion combinations may be ambiguously interpreted. - Signalling test cases will not be able to run in NGSO branch. - Confusion on the common test environment will remain for RF/RRM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13, 4.14.1, 6.3.6(new), 8.1.5B.1, 8.3.8(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the outcome of 1 revision of R5-240527. |  |  |  |  |  |  |  |  |

## 4.13	UE position requirements for NTN testing

Clauses 4.14 and 8.1.5B define the common test environment which applies to all test cases executed for eMTC and NB-IoT NTN UEs, unless otherwise specified.

-	UE's positioning engine (e.g., standalone GNSS receiver) shall be enabled to allow it to acquire the position. UE position during the test is stationary. Below position shall be used to pre-configure UE position:

-	Latitude: 25.08439333

-	Longitude: 121.56076999

-	Altitude: 0

## 4.14	Common test environment for Vertical UEs

### 4.14.1	eMTC NTN

This clause defines the common test environment which applies to all test cases executed for eMTC NTN UEs, unless otherwise specified.

-	Test configures following IEs with condition NTN of MasterInformationBlock in Table 4.4.3.2-1, SystemInformationBlockType1-BR in Table 4.4.3.3-3A, SystemInformationBlockType31 in Table 4.4.3.3-21 and RadioResourceConfigCommonSIB-DEFAULT in Table 4.6.3-14.

-	Test uses system information combination 32 for single cell scenario and combination 33 for intra-frequency multi cell scenario.

<SECTIONS SKIPPED>

### 6.3.5	Satellite ephemeris information for NTN

#### 6.3.5.1	GSO scenario

The satellite ephemeris information of GSO is defined in Table 4.4.3.3-21 for eMTC NTN and Table 8.1.4.3.3-10 for NB-IoT NTN with below assumption:

-	Inclination: 0

-	Eccentricity: 0

-	Latitude: 0

-	Longitude: 121.56076999

-	Altitude: 35786 km

-	Speed: 0 km/s

In case of multi-cell NTN GSO scenario, a maximum of 3 NTN cells can be used. The satellite ephemeris information is defined in Table 6.3.5.1-1 unless otherwise specified in a test case.

Table 6.3.5.1-1: Satellite ephemeris information in multi-cell NTN GSO scenario

| Cell type (Note 1) | positionX | positionY | positionZ | velocityVX/Y/Z | nta-Common | Koffset |
| --- | --- | --- | --- | --- | --- | --- |
| First | -16976014 | 27636499 | 0 | 0 | 8243100 | 550 |
| Second | -23550424 | 22301093 | 0 | 0 | 8248322 | 550 |
| Third | -14502736 | 29010891 | 0 | 0 | 8248322 | 550 |
| Note 1: First Cell is the cell defined as default serving cell in the test case at the end of the preamble. Second Cell and third Cell are used in multi-cell test cases with Second Cell being defined as the 1st neighbour cell and Third Cell being defined as the 2nd neighbour cell. |  |  |  |  |  |  |

For example, in TC 22.5.23 runing in GSO scenario, Ncell 50 is the serving cell in the preamble so it uses the satellite ephemeris information of First Cell, while Ncell 62 uses the satellite ephemeris information of Second Cell.

#### 6.3.5.2	NGSO scenario

The satellite ephemeris information of NGSO is defined in Table 4.4.3.3-21 for eMTC NTN and Table 8.1.4.3.3-10 for NB-IoT NTN with below assumption:

-	Inclination: 90

-	Eccentricity: 0

-	Longitude: 121.56076999

-	Altitude: 600 km

-	Speed: 7.56 km/s

The UE position defined in clause 4.13 is the projection position from one of the satellite information to the ground.

As the NGSO satellite is moving and to avoid varying nta-Common, nta-CommonDrift and nta-CommonDriftVariation, the Reference Point (RP) is defined as the satellite itself.

Considering the mininum elevation angle for both NTN Gateway and UE is 10°, the initial latitude of the satellite is: 25.0843933 (UE latitude) - 15.8394129 (the max intersection angle between UE and the satellite in ECEF coordinates) = 9.24498034°.

The table of the satellite ephemeris information of NGSO NTN lasts 510 seconds as the NGSO satellite can serve the UE for about 8.5 minutes with the periodicity set to the si-WindowLength-r13 = 160 ms. The satellite ephemeris information in systemInformationBlockType31 or systemInformationBlockType31-NB is defined and where appropriate is given in Excel format in .xlsx files for each time instance in the appropriate data file specified in Annex B.1.

In case of multi-cell NTN NGSO scenario, a maximum of 3 NTN cells can be used. The satellite ephemeris information of each cell has the same value as the default, but the epoch time is shifted with respect to the previous cell by 20 seconds.

Table 6.3.5.2-1: Satellite ephemeris information in multi-cell NTN NGSO scenario

| Cell type (Note 1) | Time offset (second) | positionY | positionZ | velocityVX/Y/Z | nta-Common | Koffset |
| --- | --- | --- | --- | --- | --- | --- |
| First | 0 | See Annex B.1 |  |  |  |  |
| Second | +20 | See Annex B.1 |  |  |  |  |
| Third | +40 | See Annex B.1 |  |  |  |  |
| Note 1: First Cell is the dell defined as default serving cell in the test case at the end of the preamble. Second Cell and Third Cell are used in multi-cell test cases with Second Cell being defined as the 1st neighbour cell and Third Cellbeing defined as the 2nd neighbour cell. |  |  |  |  |  |  |

For example, in TC 22.5.23 runing in NGSO scenario, Ncell 50 is the serving cell in the preamble so it uses the satellite ephemeris information of First Cell, while Ncell 62 uses the satellite ephemeris information of Second Cell.

### 6.3.6	Signalling test environment for Vertical UEs

#### 6.3.6.1	eMTC NTN

This clause defines the signalling test environment which applies to all signalling test cases executed for eMTC NTN UEs, unless otherwise specified. This is in addition to the common test environment defined in clause 4.14.1.

-	Test frequencies specified in Table 6.2.3.1-1d are used.

-	For NGSO scenario, UE prediction of satellite trajectory shall be deactivated in the pre-test conditions by any preconfigured means.

NOTE: 	As constant ephemeris information in SIB31 is used for the NGSO scenario, the frequency Doppler and transmission delay values remain same during the test execution. Therefore, prediction of satellite trajectory needs to be deactivated in the UE, so that the UE will not update the Doppler and delay pre-compensation with the assumption that the satellite is moving before the expiry of timer T317 (ul-SyncValidityDuration-r17).

<SECTIONS SKIPPED>

### 8.1.5B	Common test environment for Vertical UEs

#### 8.1.5B.1	NB-IoT NTN

This clause defines the common test environment which applies to all test cases executed for NB-IoT NTN UEs, unless otherwise specified.

-	Test configures following IEs with condition NTN of MasterInformationBlock-NB in Table 8.1.4.3.2-1, SystemInformationBlockType1-NB in Table 8.1.4.3.2-3, SystemInformationBlockType31-NB in Table 8.1.4.3.3-10 and RadioResourceConfigCommonSIB-NB-DEFAULT in Table 8.1.6.3-9.

-	Test uses system information combinations 8 to 13.

<SECTIONS SKIPPED>

### 8.3.7	NB-IoT Timer tolerances

The timer tolerances specified for the test environment in this subclause apply to all Signalling test cases defined in TS 36.523-1 [18] unless otherwise specified.

All the timers used during testing are within a tolerance margin given in Table 8.3.7.-1 below. For NB-IoT and its typically longer timer values across all protocol layers, common timer tolerances are applicable independently from protocol layer resp. test case group. If for a specific test a different tolerance value is required then this should be specified in the relevant test document (i.e. the document where the test is described).

Table 8.3.7-1: Timer tolerances for protocol timers

|  | timer value | < 500 ms | not applicable |
| --- | --- | --- | --- |
| 500 ms ≤ | timer value | < 10000 ms | timer tolerance = 10 % |
| 10000 ms ≤ | timer value | < 40000 ms | timer tolerance = 5 % |
| 40000 ms ≤ | timer value |  | timer tolerance = 2.5 % |

### 8.3.8	Signalling test environment for Vertical UEs

#### 8.3.8.1	NB-IoT NTN

This clause defines the signalling test environment which applies to all signalling test cases executed for NB-IoT NTN UEs, unless otherwise specified. This is in addition to the common test environment defined in clause 8.1.5B.1.

-	Test frequencies specified in Table 8.3.2.3.1-1 are used.

-	For NGSO scenario, UE prediction of satellite trajectory shall be deactivated in the pre-test conditions by any preconfigured means.

NOTE: 	As constant ephemeris information in SIB31-NB is used for the NGSO scenario, the frequency Doppler and transmission delay values remain same during the test execution. Therefore, prediction of satellite trajectory needs to be deactivated in the UE, so that the UE will not update the Doppler and delay pre-compensation with the assumption that the satellite is moving before the expiry of timer T317 (ul-SyncValidityDuration-r17).

-	For Rel-13 to Rel-15 NB-IoT test cases applicable to a UE supporting NTN only access in NB-IoT (pc_NB_ntn_only_Connectivity_EPC):

-	UE is pre-configured with the UE position defined in clause 4.13.

-	The mapping of the system information combination in terrestrial networks (TN) to a corresponding system information combination in non-terrestrial networks (NTN) is defined in Table 8.3.8.1-1:

Table 8.3.8.1-1: Mapping of the TN to NTN system information combinations

| System Information Combination in TN environment | System Information Combination in NTN environment |
| --- | --- |
| c1 | c8 |
| c2 | c10 |
| c3 | c11 |
| c4 | c13 |
| c6 | c12 |