3GPP TSG-RAN5 Meeting #102	R5-24xxxx

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | xxxx | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE RACS test case 8.5.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The UL grant size mentioned in the Note 1 is more than the size of 1st segment as it amounts to 17*501 = 8517 bytes while the first segment is os size 8188 bytes. This will lead to test case failure as UE will send both the segments together which is not as per the expected procedure. - The existing radio bearers are not resumed after re-establishment procedure |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modified the UL grant size in the note to be just enough to transmit the first segment - Added steps 12A and 12B to add the RRC connection reconfiguration procedure to resume the existing radio bearers |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 8.5.5.1	RACS / UL Message Segment transfer / UECapabilityInformation / Success

8.5.5.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives a UECapabilityEnquiry message with rrc-SegAllowed-r16 enabled }

then { UE successfully segments UECapabilityInformation message at RRC layer and sends ULDedicatedMessageSegment messages with rrc-MessageSegmentType set accordingly for the first and last segment and rrc-MessageSegmentContainer containing the segments of UECapabilityInformation message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and having received UECapabilityEnquiry message with rrc-segAllowed-r16 enabled and having transmitted the firstSegment of UECapabilityInformation message within the ULDedicatedMessageSegment message }

ensure that {

when { UE detects radio link failure on expiry of timer T310 }

then { UE successfully completes RRC re-establishment procedure and discards the remaining segments of UECapabilityInformation message }

}

8.5.5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clause 5.6.3, 5.6.22.2, 5.6.22.3 and 5.3.7.2. Unless otherwise stated these are Rel-16 requirements.

[TS 36.331, clause 5.6.3]

The UE shall:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 36.323 [8]:

2>	initiate the UL message segment transfer procedure as specified in clause 5.6.22;

1>	else:

2>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.6.22.2]

A UE capable of UL RRC message segmentation in RRC_CONNECTED will initiate the procedure when the following conditions are met:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and

1>	if the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 36.323 [8];

Upon initiating the procedure, the UE shall:

- initiate transmission of the ULDedicatedMessageSegment message as specified in 5.6.22.3;

[TS 36.331, clause 5.6.22.3]

The UE shall segment the encoded RRC PDU based on the maximum supported size of a PDCP SDU specified in TS 36.323 [8]. UE shall minimize the number of segments and set the contents of the ULDedicatedMessageSegment messages as follows:

1>	For each new UL DCCH message, set the segmentNumber to 0 for the first message segment and increment the segmentNumber for each subsequent RRC message segment;

1>	set rrc-MessageSegmentContainer to include the segment of the UL DCCH message corresponding to the segmentNumber;

1>	if the segment included in the rrc-MessageSegmentContainer is the last segment of the UL DCCH message:

2>	set the rrc-MessageSegmentType to lastSegment;

1>	else:

2>	set the rrc-MessageSegmentType to notLastSegment;

1>	submit all the ULDedicatedMessageSegment messages generated for the segmented RRC message to lower layers for transmission in ascending order based on the segmentNumber, upon which the procedure ends.

[TS 36.331, clause 5.3.7.2]

Upon initiation of the procedure, the UE shall:

…

1>	if UE is not configured with conditionalReconfiguration:

2>	release uplinkDataCompression, if configured;

2>	suspend all RBs, including RBs configured with NR PDCP, except SRB0;

2>	reset MAC;

8.5.5.1.3	Test description

8.5.5.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18].

8.5.5.1.3.2	Test procedure sequence

Table 8.5.5.1.3.2-1: Power levels

|  | Parameter | Unit | Cell 1 |
| --- | --- | --- | --- |
| T1 | Cell-specific RS RPRE | dBm/15kHz | “Off” |
| T2 | Cell-specific RS RPRE | dBm/15kHz | -85 |

Table 8.5.5.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a UECapabilityEnquiry message including eutra to enquire max capability and with rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| - | EXCEPTION: Steps 2a1 to 2b7 describe behaviour that depends on the size of UEs capability container; the "lower case letter" identifies a step sequence that takes place depending on the size of UE capability message. IF UE’s encoded UECapabilityInformation message >= Max PDCP SDU size then step 2a1 will be performed else steps 2b1 to 2b7 will be performed. | - | - | - | - |
| - | EXCEPTION: Step 2a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the ULDedicatedMessageSegment message indicate rrc-MessageSegmentType-r16 = lastSegment. | - | - | - | - |
| 2a1 | Check: Does the UE send segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message? NOTE: The SS shall ensure that the concatenated segments decode into a valid UECapabilityInformation message (specific capability content is not checked). | --> | ULDedicatedMessageSegment | 1 | P |
| 2b1 | The UE sends UECapabilityInformation Message. | --> | UECapabilityInformation | - | - |
| 3a1-4a11 | Void | - | - | - | - |
| 2b2 | The SS transmits a SET UL MESSAGE REQUEST message. | <-- | SET UL MESSAGE REQUEST | - | - |
| 2b3 | The UE transmits a SET UL MESSAGE RESPONSE message. | --> | SET UL MESSAGE RESPONSE | - | - |
| 2b4 | The SS transmits a UECapabilityEnquiry message with rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| - | EXCEPTION: Step 2b5 is repeated a maximum of 2 times. | - | - | - | - |
| 2b5 | Check: Does the UE send segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message? NOTE: The SS shall ensure that the concatenated segments decode into a valid UECapabilityInformation message (specific capability content is not checked). | --> | ULDedicatedMessageSegment | 1 | P |
| 2b6 | The SS transmits a SET UL MESSAGE REQUEST message. | <-- | SET UL MESSAGE REQUEST | - | - |
| 2b7 | The UE transmits a SET UL MESSAGE RESPONSE message. | --> | SET UL MESSAGE RESPONSE | - | - |
| 3 | SS stops regular grant transmission upon scheduling request. | - | - | - | - |
| 4 | The SS transmits a UECapabilityEnquiry message to enquire max capability and with rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| 5 | The SS allocates 17 UL grants periodically from 15 (FDD)/18 (TDD) subframes after successful completion of step 4 to schedule PUSCH till the ULDedicatedMessageSegment at step 6 is received. (Note 1) | - | - | - | - |
| 6 | The UE sends 1st segment of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message. | --> | ULDedicatedMessageSegment | - | - |
| 7 | The SS changes Cell 1 parameters according to the row "T1" in table 8.5.5.1.3.2-1 in order that the radio link quality of Cell 1 is degraded. | - | - | - | - |
| 8 | Wait for T310 to expire | - | - | - | - |
| 9 | The SS changes Cell 1 parameters according to the row "T2" in table 8.5.5.1.3.2-1. | - | - | - | - |
| 10 | The UE sends an RRCConnectionReestablishmentRequest message on Cell 1. | --> | RRCConnectionReestablishmentRequest | - | - |
| 11 | The SS transmits an RRCConnectionReestablishment message to resume SRB1 operation on Cell 1. | <-- | RRCConnectionReestablishment | - | - |
| 12 | The UE transmits an RRCConnectionReestablishmentComplete message on Cell 1? | --> | RRCConnectionReestablishmentComplete | - | - |
| 12A | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer. | <-- | RRCConnectionReconfiguration | - | - |
| 12B | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationtComplete | - | - |
| 13 | SS resumes regular UL grant transmission. | - | - | - | - |
| 14 | Check: Upon successful completion of RRC re-establishment procedure, does the UE discards remaining segments of UECapabilityInformation message and does not send any ULDedicatedMessageSegment message in the next 5 seconds? | - | - | 2 | P |
| NOTE 1: 16 UL grants of default size (501 bytes, as per 36.523-3[20] Table 7.2.0-1) and an additional UL grant of size 177 bytes (NPRB=20, IMCS=4, as per 36.213 Table 7.1.7.2.1) are chosen to enable UE to transmit only one ulDedicatedMessageSegment as maximum PDCP SDU size 8188 bytes. |  |  |  |  |  |

8.5.5.1.3.3	Specific message contents

Table 8.5.5.1.3.3-1: UECapabilityEnquiry (steps 1, 2b4, 4, Table 8.5.5.1.3.2-2)

| Derivation Path: TS 36.508 [7], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRequest SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF RAT TYPE { |  |  |  |
| RAT-Type [1] | eutra |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| requestedFrequencyBands-r11 SEQUENCE { | n entries, where n is equal to UE supported bands or 16 if number of UE supported bands exceeds 16. |  |  |
| FreqBandIndicator-r11 [1] | px_ePrimaryFrequencyBand | E-UTRA primary frequency band under test. |  |
| FreqBandIndicator-r11 [k] | For k=2 to n set to value different from values set in preceding entries FreqBandIndicator-r11 [1] to FreqBandIndicatorr11 [k-1] | Frequency bands supported by UE. Any value β such that pc_eBandβ_Supp is TRUE |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rrc-SegAllowed-r16 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.5.1.3.3-2: ULDedicatedMessageSegment (steps 2a1, 2b5 and step 6, Table 8.5.5.1.3.2-2)

| Derivation Path: 36.508 [18], Table 4.6.1-22A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ULDedicatedMessageSegment-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ulDedicatedMessageSegment-r16 SEQUENCE { |  |  |  |
| segmentNumber-r16 | 0 for first segment, and incremented by 1 for each subsequent segment |  |  |
| rrc-MessageSegmentContainer-r16 | Not Checked | OCTET STRING including a segment of the encoded DL DCCH message |  |
| rrc-MessageSegmentType-r16 | notLastSegment for all segments except the last segment. lastSegment for the last segment |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.5.1.3.3-3: RRCConnectionReestablishmentRequest (step 11, Table 8.5.5.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.5.1.3.3-4: UECapabilityInformation (step 2b1, Table 8.5.5.1.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| ueCapabilityInformation-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRAT-ContainerList SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { | 1 entry |  |  |
| rat-Type | eutra | E-UTRA only |  |
| ueCapabilityRAT-Container | UE-EUTRA-Capability | Encoded as per TS 36.331 [17] clause 5.6.3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |