3GPP TSG-RAN5 Meeting #102	R5-240156

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4168 | rev | - | Current version: | 17.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TC 11.4.1a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS 24.501 Cl. 5.5.1.2.2, the UE needs to set the NR-PSSI bit to "NR paging subgrouping supported" in the 5GMM capability IE in the REGISTRATION REQUEST message for this case. The UE shall set the NR-PSSI bit to "NR paging subgrouping supported" in the 5GMM capability IE if the UE supports PEIPS assistance information and the 5GS registration type IE in the REGISTRATION REQUEST message is not set to "emergency registration". The UE may include its UE paging probability information in the Requested PEIPS assistance information IE if the UE has set the NR-PSSI bit to "NR paging subgrouping supported" in the 5GMM capability IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NR-PSSI bit within 5GMM capability IE was specified in REGISTRATION REQUEST message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be misalignmeny with core specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.1a.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

11.4.1a	5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / Utilising emergency number stored on the USIM / New emergency PDU session / PEIPS assistance information

11.4.1a.1	Test Purpose (TP)

(1)

with { UE with PEIPS assistance information provided with the Paging subgroup ID in 5GMM-IDLE state with an emergency PDU Session established on a cell that support PEIPS and subgroupsNumForUEID is absent in subgroupConfig }

ensure that {

when { UE receives a Paging message with matching identity and a PEI with the bit corresponding for the subgroup the UE belongs to indicated as 0 }

then { UE responds to paging }

}

11.4.1a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501 [22], subclauses 5.3.25. Unless otherwise stated these are Rel-17 requirements.

[TS 24.501, subclause 5.6.1.2]

When an emergency PDU session is successfully established over 3GPP access after the UE received the Negotiated PEIPS assistance information IE during the last registration procedure, the UE and the AMF shall not use PEIPS assistance information until:

-	the successful completion of the PDU session release procedure of the emergency PDU;

-	the UE receives PEIPS assistance information during a registration procedure with PDU session status IE or upon successful completion of a service request procedure, if the UE or the network locally releases the emergency PDU session;

-	the successful completion of handover of emergency PDU session to non-3GPP access; or

-	the successful transfer of the emergency PDU session in 5GS to the EPS or ePDG connected to EPC.

11.4.1a.3	Test description

11.4.1a.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell

-	NR Cell 1 as defined in TS 38.508-1 [4] Table 4.4.2-3. System information combination NR-1 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2. SIB1 indicates ims-EmergencySupport.

UE:

-	The UE is equipped with a test USIM with USIM Configuration 20 as defined in TS 38.508-1 [4] Table 6.4.1-20 (USIM contains two Emergency Numbers: 144, 117).

Preamble:

-	Cells power level configuration in accordance with TS 38.508-1 [4], Table 6.2.2.1-3:

-	NR Cell 1 "Serving cell"

-	The UE is in 5GS state 0-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-0.

11.4.1a.3.2	Test procedure sequence

Table 11.4.1a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-21a1 | The UE performs steps 1-20a1 of the RRC_IDLE procedure specified in TS 38.508-1 [4], table 4.5.2.2-2. | - | - | - | - |
| 22 | Make the UE attempt an IMS emergency call dialling the number 144 which is stored on the USIM. (NOTE 1) | - | - | - | - |
| 23 | The UE performs Test Procedure for IMS Emergency call establishment with IMS emergency registration as specified in TS 38.508-1 [4], subclause 4.9.11. | - | - | - | - |
| 24 | The Test procedure for IMS MT Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12B takes place with “keep emergency PDU session”. | - | - | - | - |
| 25 | The SS releases the RRC connection. | - | - | - | - |
| 26 | The SS transmits a PEI with the bit corresponding for the subgroup the UE belongs to indicated as 0 and the Paging message including a matched identity. | <-- | NR RRC: Paging | - | - |
| 27 | Check: Does the UE transmit an RRCSetupRequest message? | --> | NR RRC: RRCSetupRequest | 1 | P |
| 28 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 29 | The UE transmits an RRCSetupComplete message including SERVICE REQUEST to confirm the successful completion of the connection establishment. | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | - | - |
| 30 | Steps 5 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 [4], Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| NOTE 1: This could be done by e.g., MMI or AT command. |  |  |  |  |  |

11.4.1a.3.3	Specific message contents

Table 11.4.1a.3.3-1: SIB1 for NR Cell 1 (preamble and all steps, Table 11.4.1a.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-28 with condition PEI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommonSIB in Table 11.4.1a.3.3-2 |  |  |
| } |  |  |  |

Table 11.4.1a.3.3-2: ServingCellConfigCommonSIB (Table 11.4.1a.3.3-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-169 with condition PEI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB in Table 11.4.1a.3.3-3 |  |  |
| } |  |  |  |

Table 11.4.1a.3.3-3: DownlinkConfigCommonSIB (Table 11.4.1a.3.3-2)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-53 with condition PEI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| pei-Config-r17 ::= SEQUENCE { |  |  |  |
| subgroupConfig-r17 ::= SEQUENCE { |  |  |  |
| subgroupsNumForUEID-r17 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 11.4.1a.3.3-3A: REGISTRATION REQUEST (step 10, Table 11.4.1a.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GMM capability |  |  |  |  |
| NR-PSSI (octet 6, bit 4) |  | '1'B | NR paging subgrouping supported |  |

Table 11.4.1a.3.3-4: REGISTRATION ACCEPT (step 15, Table 11.4.1a.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Negotiated PEIPS assistance information |  |  |  |  |
| PEIPS assistance information IEI |  | ‘33’H |  |  |
| Length of PEIPS assistance information contents |  | ‘01’H |  |  |
| PEIPS assistance information type 1 |  |  |  |  |
| Type of information |  | ‘000’B | Paging subgroup ID value |  |
| Paging subgroup ID value |  | ‘00001’B |  |  |

Table 11.4.1a.3.3-5: RRCSetupRequest (step 27, Table 11.4.1a.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupRequest ::= SEQUENCE { |  |  |  |
| rrcSetupRequest SEQUENCE { |  |  |  |
| establishmentCause | mt-Access |  |  |
| } |  |  |  |
| } |  |  |  |

[End of change]