3GPP TSG-RAN5 Meeting #102	R5-240542

# Athens, Greece, 26th February– 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4249 | rev | - | Current version: | 17.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates for NR RRC test case 8.1.5.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | With the addition of new PICS in TS 38.508-2 version 18.1.0, pc_survivalTime_r17 remain not checked. Due to R5-233295, parameter pc_mpsPriorityIndication_r16 was renamed to pc_NR_mpsPriorityIndication_r16. Field pusch-RepetitionCRC-r17 was renamed to pusch-RepetitionMsg3-r17 in TS 38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following updates in Table 8.1.5.1.1.3.3-3: Added check of feature survivalTime_r17 References to parameter pc_mpsPriorityIndication_r16 changed to pc_NR_mpsPriorityIndication_r16 Rename reference to pusch-RepetitionCRC-r17 as pusch-RepetitionMsg3-r17. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Check of field survivalTime_r17 in mac_ParametersCommon will be missing and a discrepancy between fields and parameters will remain in Audit test case 8.1.5.1.1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 8.1.5.1.1	UE capability transfer / Success

8.1.5.1.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives an UECapabilityEnquiry message }

then { UE transmits an UECapabilityInformation message including UE radio access capability information corresponding to the ue-CapabilityRequest variable }

}

8.1.5.1.1.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.331, clauses 5.6.1.3, 5.6.1.4, 5.7.7.2 and 5.7.7.3.

[TS 38.331, clause 5.6.1.3]

The UE shall set the contents of UECapabilityInformation message as follows:

1>	if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to nr:

2>	include in the ue-CapabilityRAT-ContainerList a UE-CapabilityRAT-Container of the type UE-NR-Capability and with the rat-Type set to nr;

2>	include the supportedBandCombinationList, featureSets and featureSetCombinations as specified in clause 5.6.1.4;

…

1>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends.

[TS 38.331, clause 5.6.1.4]

The UE invokes the procedures in this clause if the NR or E-UTRA network requests UE capabilities for nr, eutra-nr or eutra. This procedure is invoked once per requested rat-Type (see clause 5.6.1.3 for capability enquiry by the NR network; see TS 36.331 [10], clause 5.6.3.3 for capability enquiry by the E-UTRA network). The UE shall ensure that the feature set IDs are consistent across feature sets, feature set combinations and band combinations in all three UE capability containers that the network queries with the same fields with the same values, i.e. UE-CapabilityRequestFilterNR and fields in UECapabilityEnquiry message (i.e. requestedFreqBandsNR-MRDC, requestedCapabilityNR and eutra-nr-only flag) as defined in TS 36.331, where applicable.

NOTE 1:	Capability enquiry without frequencyBandListFilter is not supported.

[TS 38.331, clause 5.7.7.2]

A UE capable of UL RRC message segmentation in RRC_CONNECTED will initiate the procedure when the following conditions are met:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and

1>	if the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 38.323 [5];

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the ULDedicatedMessageSegment message as specified in 5.7.7.3;

[TS 38.331, clause 5.7.7.3]

The UE shall segment the encoded RRC PDU based on the maximum supported size of a PDCP SDU specified in TS 38.323 [5]. UE shall minimize the number of segments and set the contents of the ULDedicatedMessageSegment messages as follows:

1>	F or each new UL DCCH message, set the segmentNumber to 0 for the first message segment and increment the segmentNumber for each subsequent RRC message segment;

1>	set rrc-MessageSegmentContainer to include the segment of the UL DCCH message corresponding to the segmentNumber;

1>	if the segment included in the rrc-MessageSegmentContainer is the last segment of the UL DCCH message:

2>	set the rrc-MessageSegmentType to lastSegment;

1>	else:

2>	set the rrc-MessageSegmentType to notLastSegment;

1>	submit all the ULDedicatedMessageSegment messages generated for the segmented RRC message to lower layers for transmission in ascending order based on the segmentNumber, upon which the procedure ends.

8.1.5.1.1.3	Test Description

8.1.5.1.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cell.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A on NR Cell 1.

8.1.5.1.1.3.2	Test procedure sequence

Table 8.1.5.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a UECapabilityEnquiry message including nr and rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| - | EXCEPTION: The "lower case letter" identifies a step sequence that takes place depending on the size of UE capability message. IF pc_NR_UL_Segmentation and UE’s encoded UECapabilityInformation message >= Max PDCP SDU size then steps 2a1 and 2a2 will be performed else step 2b1 will be performed. | - | - | - | - |
| - | EXCEPTION: Step 2a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the UlDedicatedMessageSegment message indicates rrc-MessageSegmentType-r16 = lastSegment. | - | - | - | - |
| 2a1 | The UE sends segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message. | --> | ULDedicatedMessageSegment | - | - |
| 2a2 | Check: Is the received UE radio access capability information as per the ue-CapabilityRequest variable? | - | - | 1 | P |
| 2b1 | Check: Does the UE transmit a UECapabilityInformation message including UE radio access capability information as per the ue-CapabilityRequest variable? | --> | UECapabilityInformation | 1 | P |

8.1.5.1.1.3.3	Specific message contents

Table 8.1.5.1.1.3.3-1: UECapabilityEnquiry (step 1, Table 8.1.5.1.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-31 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityEnquiry SEQUENCE { |  |  |  |
| ue-CapabilityRAT-RequestList SEQUENCE (SIZE (1.. maxRAT-CapabilityContainers)) OF CapabilityRAT-Request { | - |  |  |
| UE-CapabilityRAT-Request[1] SEQUENCE { |  | entry 1 |  |
| rat-Type | nr |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| ue-CapabilityEnquiryExt | OCTET STRING Containing UECapabilityEnquiry-v1560-IEs (Table 8.1.5.1.1.3.3-4) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-2: UECapabilityInformation (steps2a2 and 2b1, Table 8.1.5.1.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityInformation SEQUENCE { |  |  |  |
| ue-CapabilityRAT-ContainerList SEQUENCE (SIZE (0..maxRAT-CapabilityContainers)) OF CapabilityRAT-Request { |  |  |  |
| UE-CapabilityRAT-Request[1] SEQUENCE { |  | entry 1 |  |
| rat-Type | nr |  |  |
| ueCapabilityRAT-Container | UE-NR-Capability | Encoded as per TS 38.331 [12] clause 5.6.1 |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-3: UE-NR-Capability (Table 8.1.5.1.1.3.3-2)

| Derivation Path: TS 38.331 [12], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-NR-Capability ::= SEQUENCE { |  |  |  |
| accessStratumRelease | Checked | "rel-15" or higher |  |
|  | Checked | "rel-17" or higher | pc_supportOfRedCap_r17 |
| pdcp-Parameters SEQUENCE { |  |  |  |
| supportedROHC-Profiles SEQUENCE { |  |  |  |
| profile0x0000 | Not checked |  |  |
| profile0x0001 | Not checked |  |  |
| profile0x0002 | Not checked |  |  |
| profile0x0003 | Not checked |  |  |
| profile0x0004 | Not checked |  |  |
| profile0x0006 | Not checked |  |  |
| profile0x0101 | Not checked |  |  |
| profile0x0102 | Not checked |  |  |
| profile0x0103 | Not checked |  |  |
| profile0x0104 | Not checked |  |  |
| } |  |  |  |
| maxNumberROHC-ContextSessions | Not checked |  |  |
| uplinkOnlyROHC-Profiles | Not checked |  |  |
| continueROHC-Context | Not checked |  |  |
| outOfOrderDelivery | Checked |  | pc_outOfOrderDelivery |
| shortSN | Checked |  | pc_shortSN |
| pdcp-DuplicationSRB | Checked |  | pc_pdcp_DuplicationSplitSRB |
|  | Not Present |  | pc_supportOfRedCap_r17 |
| pdcp-DuplicationMCG-OrSCG-DRB | Checked |  | pc_pdcp_DuplicationMCG_OrSCG_DRB |
|  | Not Present |  | pc_supportOfRedCap_r17 |
| drb-IAB-r16 | Not checked |  |  |
| non-DRB-IAB-r16 | Not checked |  |  |
| extendedDiscardTimer-r16 | Not checked |  |  |
| continueEHC-Context-r16 | Not checked |  |  |
| ehc-r16 | Checked |  | pc_NR_ehc_r16 |
| maxNumberEHC-Contexts-r16 | Not checked |  |  |
| jointEHC-ROHC-Config-r16 | Not checked |  |  |
| pdcp-DuplicationMoreThanTwoRLC-r16 | Checked |  | pc_pdcp_DuplicationMoreThanTwoRLC_r16 |
| longSN-RedCap-r17 | Checked |  | pc_longSN_RedCap_r17 |
| udc-r17 SEQUENCE { | Checked | TS 38.306 cl. 4.2.4. Indicates whether the UE supports the uplink data compression operation as specified in TS 38.323. Nested parameters apply on presence of udc-r17 | pc_NR_udc_r17 |
| standardDictionary-r17 | Checked |  | pc_NR_udc_standardDictionary_r17 |
| operatorDictionary-r17 | Not checked |  |  |
| continueUDC-r17 | Checked |  | pc_NR_udc_continueUDC_r17 |
| supportOfBufferSize-r17 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| rlc-Parameters SEQUENCE { |  |  |  |
| am-WithShortSN | Checked |  | pc_am_WithShortSN |
| um-WithShortSN | Checked |  | pc_um_WithShortSN |
| um-WithLongSN | Checked |  | pc_um_WIthLongSN |
| am-WithLongSN-RedCap-r17 | Checked |  | pc_supportOfRedCap_r17 and pc_am_WithLongSN |
| } |  |  |  |
| mac-Parameters SEQUENCE { |  |  |  |
| mac-ParametersCommon SEQUENCE { |  |  |  |
| lcp-Restriction | Checked |  | pc_lcp_Restriction |
| dummy | Not checked |  |  |
| lch-ToSCellRestriction | Not checked |  |  |
| recommendedBitRate | Checked |  | pc_recommendedBitRate |
| recommendedBitRateQuery | Checked |  | pc_recommendedBitRateQuery |
| recommendedBitRateMultiplier-r16 | Not checked |  |  |
| preEmptiveBSR-r16 | Not checked |  |  |
| autonomousTransmission-r16 | Not checked |  |  |
| lch-PriorityBasedPrioritization-r16 | Checked |  | pc_lch_PriorityBasedPrioritization_r16 |
| lch-ToConfiguredGrantMapping-r16 | Not checked |  |  |
| lch-ToGrantPriorityRestriction-r16 | Checked |  | pc_lch_ToGrantPriorityRestriction_r16 |
| singlePHR-P-r16 | Not checked |  |  |
| ul-LBT-FailureDetectionRecovery-r16 | Not checked |  |  |
| tdd-MPE-P-MPR-Reporting-r16 | Not checked |  |  |
| lcid-ExtensionIAB-r16 | Not checked |  |  |
| spCell-BFR-CBRA-r16 | Not checked |  |  |
| srs-ResourceId-Ext-r16 | Not checked |  |  |
| enhancedUuDRX-forSidelink-r17 | Not checked |  |  |
| mg-ActivationRequestPRS-Meas-r17 | Not checked |  |  |
| mg-ActivationCommPRS-Meas-r17 | Not checked |  |  |
| intraCG-Prioritization-r17 | Not checked |  |  |
| jointPrioritizationCG-Retx-Timer-r17 | Not checked |  |  |
| survivalTime-r17 | Checked |  | pc_survivalTime_r17 |
| lcg-ExtensionIAB-r17 | Not checked |  |  |
| harq-FeedbackDisabled-r17 | Not checked |  |  |
| uplink-Harq-ModeB-r17 | Not checked |  |  |
| sr-TriggeredBy-TA-Report-r17 | Not checked |  |  |
| extendedDRX-CycleInactive-r17 | Not checked |  |  |
| simultaneousSR-PUSCH-DiffPUCCH-groups-r17 | Not checked |  |  |
| lastTransmissionUL-r17 | Not checked |  |  |
| } |  |  |  |
| mac-ParametersXDD-Diff SEQUENCE { |  |  |  |
| skipUplinkTxDynamic | Checked (NOTE 4) |  | pc_skipUplinkTxDynamic |
| logicalChannelSR-DelayTimer | Checked (NOTE 5) |  | pc_logicalChannelSR_DelayTimer |
| longDRX-Cycle | Checked (NOTE 6) |  | pc_longDRX_Cycle |
| shortDRX-Cycle | Checked (NOTE 7) |  | pc_shortDRX_Cycle |
| multipleSR-Configurations | Not checked |  |  |
| multipleConfiguredGrants | Checked (NOTE 12) |  | pc_multipleConfiguredGrants_r16 |
| } |  |  |  |
| } |  |  |  |
| phy-Parameters SEQUENCE { |  |  |  |
| phy-ParametersCommon SEQUENCE { |  |  |  |
| csi-RS-CFRA-ForHO | Checked |  | pc_csi_RS_CFRA_ForHO |
| dynamicPRB-BundlingDL | Not checked |  |  |
| sp-CSI-ReportPUCCH | Not checked |  |  |
| sp-CSI-ReportPUSCH | Not checked |  |  |
| nzp-CSI-RS-IntefMgmt | Not checked |  |  |
| type2-SP-CSI-Feedback-LongPUCCH | Not checked |  |  |
| precoderGranularityCORESET | Not checked |  |  |
| dynamicHARQ-ACK-Codebook | supported |  |  |
| semiStaticHARQ-ACK-Codebook | Not checked |  |  |
| spatialBundlingHARQ-ACK | Checked |  | pc_spatialBundlingHARQ_ACK |
| dynamicBetaOffsetInd-HARQ-ACK-CSI | Not checked |  |  |
| pucch-Repetition-F1-3-4 | Not checked |  |  |
| ra-Type0-PUSCH | Checked |  | pc_ra_Type0_PUSCH |
| dynamicSwitchRA-Type0-1-PDSCH | Checked |  | pc_dynamicSwitchRA_Type0_1_PDSCH |
| dynamicSwitchRA-Type0-1-PUSCH | Checked |  | pc_dynamicSwitchRA_Type0_1_PUSCH |
| pdsch-MappingTypeA | supported |  | pc_pdsch_MappingTypeA |
| pdsch-MappingTypeB | Checked |  | pc_pdsch_MappingTypeB |
| interleavingVRB-ToPRB-PDSCH | Checked |  | pc_interleavingVRB_ToPRB_PDSCH |
| interSlotFreqHopping-PUSCH | Not checked |  |  |
| type1-PUSCH-RepetitionMultiSlots | Not checked |  |  |
| type2-PUSCH-RepetitionMultiSlots | Not checked |  |  |
| pusch-RepetitionMultiSlots | Checked |  | pc_pusch_RepetitionMultiSlots |
| pdsch-RepetitionMultiSlots | Checked |  | pc_pdsch_RepetitionMultiSlots |
| downlinkSPS | Checked |  | pc_downlinkSPS |
| configuredUL-GrantType1 | Checked |  | pc_configuredUL_GrantType1 |
| configuredUL-GrantType2 | Checked |  | pc_configuredUL_GrantType2 |
| pre-EmptIndication-DL | Not checked |  |  |
| cbg-TransIndication-DL | Not checked |  |  |
| cbg-TransIndication-UL | Not checked |  |  |
| cbg-FlushIndication-DL | Not checked |  |  |
| dynamicHARQ-ACK-CodeB-CBG-Retx-DL | Not checked |  |  |
| rateMatchingResrcSetSemi-Static | Not checked |  |  |
| rateMatchingResrcSetDynamic | Not checked |  |  |
| bwp-SwitchingDelay | Checked | TS 38.306 [23] clause 4.2.7.10: It is mandatory to report type 1 or type 2. This capability is not applicable to IAB-MT | pc_bwp_SwitchingDelay_Type1 or pc_bwp_SwitchingDelay_Type2 |
| twoStepRACH-r16 | Checked |  | pc_twoStepRACH_r16 |
| dci-Format1-2And0-2-r16 | Checked |  | pc_dci_Format1_2And0_2_r16 |
| monitoringDCI-SameSearchSpace-r16 | Not checked |  |  |
| type2-CG-ReleaseDCI-0-1-r16 | Not checked |  |  |
| type2-CG-ReleaseDCI-0-2-r16 | Not checked |  |  |
| sps-ReleaseDCI-1-1-r16 | Not checked |  |  |
| sps-ReleaseDCI-1-2-r16 | Not checked |  |  |
| csi-TriggerStateNon-ActiveBWP-r16 | Not checked |  |  |
| separateSMTC-InterIAB-Support-r16 | Not checked |  |  |
| separateRACH-IAB-Support-r16 | Not checked |  |  |
| ul-flexibleDL-SlotFormatSemiStatic-IAB-r16 | Not checked |  |  |
| ul-flexibleDL-SlotFormatDynamics-IAB-r16 | Not checked |  |  |
| dft-S-OFDM-WaveformUL-IAB-r16 | Not checked |  |  |
| dci-25-AI-RNTI-Support-IAB-r16 | Not checked |  |  |
| t-DeltaReceptionSupport-IAB-r16 | Not checked |  |  |
| guardSymbolReportReception-IAB-r16 | Not checked |  |  |
| harqACK-CB-SpatialBundlingPUCCH-Group-r16 | Not checked |  |  |
| crossSlotScheduling-r16 | Not checked |  |  |
| maxNumberSRS-PosPathLossEstimateAllServingCells-r16 | Not checked |  |  |
| extendedCG-Periodicities-r16 | Not checked |  |  |
| extendedSPS-Periodicities-r16 | Not checked |  |  |
| codebookVariantsList-r16 | Not checked |  |  |
| pusch-RepetitionTypeA-r16 | Not checked |  |  |
| dci-DL-PriorityIndicator-r16 | Checked |  | pc_dci_DL_PriorityIndicator_r16 |
| dci-UL-PriorityIndicator-r16 | Checked |  | pc_dci_UL_PriorityIndicator_r16 |
| maxNumberPathlossRS-Update-r16 | Not checked |  |  |
| type2-HARQ-ACK-Codebook-r16 | Not checked |  |  |
| maxTotalResourcesForAcrossFreqRanges-r16 | Not checked |  |  |
| harqACK-separateMultiDCI-MultiTRP-r16 | Not checked |  |  |
| harqACK-jointMultiDCI-MultiTRP-r16 | Not checked |  |  |
| bwp-SwitchingMultiCCs-r16 | Not checked |  |  |
| guardSymbolReportReception-IAB-r17 | Not checked |  |  |
| restricted-IAB-DU-BeamReception-r17 | Not checked |  |  |
| recommended-IAB-MT-BeamTransmission-r17 | Not checked |  |  |
| case6-TimingAlignmentReception-IAB-r17 | Not checked |  |  |
| case7-TimingAlignmentReception-IAB-r17 | Not checked |  |  |
| dl-tx-PowerAdjustment-IAB-r17 | Not checked |  |  |
| desired-ul-tx-PowerAdjustment-r17 | Not checked |  |  |
| fdm-SoftResourceAvailability-DynamicIndication-r17 | Not checked |  |  |
| updated-T-DeltaRangeReception-r17 | Not checked |  |  |
| slotBasedDynamicPUCCH-Rep-r17 | Checked |  | pc_pucch_Repetition_F0_1_2_3_4_DynamicIndication_r17 |
| sps-HARQ-ACK-Deferral-r17 | Not checked |  |  |
| unifiedJointTCI-commonUpdate-r17 | Not checked |  |  |
| mTRP-PDCCH-singleSpan-r17 | Not checked |  |  |
| supportedActivatedPRS-ProcessingWindow-r17 | Not checked |  |  |
| cg-TimeDomainAllocationExtension-r17 | Not checked |  |  |
| } |  |  |  |
| phy-ParametersXDD-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA | Checked (NOTE 17) |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| dl-SchedulingOffset-PDSCH-TypeB | Not checked |  |  |
| ul-SchedulingOffset | Not checked |  |  |
| } |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| dummy1 | Not checked |  |  |
| twoFL-DMRS | Not checked |  |  |
| dummy2 | Not checked |  |  |
| dummy3 | Not checked |  |  |
| supportedDMRS-TypeDL | Not checked |  |  |
| supportedDMRS-TypeUL | Not checked |  |  |
| semiOpenLoopCSI | Not checked |  |  |
| csi-ReportWithoutPMI | Not checked |  |  |
| csi-ReportWithoutCQI | Not checked |  |  |
| onePortsPTRS | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| pucch-F2-WithFH | supported |  |  |
| pucch-F3-WithFH | supported |  |  |
| pucch-F4-WithFH | Not checked |  |  |
| freqHoppingPUCCH-F0-2 | Not checked |  |  |
| freqHoppingPUCCH-F1-3-4 | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot | Not checked |  |  |
| uci-CodeBlockSegmentation | Not checked |  |  |
| onePUCCH-LongAndShortFormat | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot | Checked (NOTE 26) |  | pc_twoPUCCH_AnyOthersInSlot |
| intraSlotFreqHopping-PUSCH | Not checked |  |  |
| pusch-LBRM | Not checked |  |  |
| pdcch-BlindDetectionCA | Not checked |  |  |
| tpc-PUSCH-RNTI | Not checked |  |  |
| tpc-PUCCH-RNTI | Not checked |  |  |
| tpc-SRS-RNTI | Not checked |  |  |
| absoluteTPC-Command | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| pusch-HalfPi-BPSK | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK | Not checked |  |  |
| almostContiguousCP-OFDM-UL | Not checked |  |  |
| sp-CSI-RS | Not checked |  |  |
| sp-CSI-IM | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot | Not checked |  |  |
| multipleCORESET | Not checked |  |  |
| csi-RS-IM-ReceptionForFeedback | Not checked | CSI-RS-IM-ReceptionForFeedback |  |
| csi-RS-ProcFrameworkForSRS | Not checked | CSI-RS-ProcFrameworkForSRS |  |
| csi-ReportFramework | Not checked | CSI-ReportFramework |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot SEQUENCE { |  |  |  |
| sameSymbol | Not checked |  |  |
| diffSymbol | Not checked |  |  |
| } |  |  |  |
| mux-SR-HARQ-ACK-PUCCH | Not checked |  |  |
| mux-MultipleGroupCtrlCH-Overlap | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA | Checked (NOTE 18) |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| dl-SchedulingOffset-PDSCH-TypeB | Not checked |  |  |
| ul-SchedulingOffset | Not checked |  |  |
| dl-64QAM-MCS-TableAlt | Not checked |  |  |
| ul-64QAM-MCS-TableAlt | Not checked |  |  |
| cqi-TableAlt | Not checked |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-UL | Not checked |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-UL | Not checked |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-UL | Not checked |  |  |
| } |  |  |  |
| phy-ParametersFR1 SEQUENCE { |  |  |  |
| pdcchMonitoringSingleOccasion | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| pdsch-256QAM-FR1 | Checked |  | pc_pdsch_256QAM_FR1 |
| pdsch-RE-MappingFR1-PerSymbol | Not checked |  |  |
| } |  |  |  |
| phy-ParametersFR2 SEQUENCE { |  |  |  |
| dummy | Not checked |  |  |
| pdsch-RE-MappingFR2-PerSymbol | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| rf-Parameters SEQUENCE { |  |  |  |
| supportedBandListNR SEQUENCE (SIZE (1..maxBands)) OF BandNR { |  |  |  |
| BandNR[i] SEQUENCE { |  |  |  |
| bandNR | Checked | Checked for 'maxBands' entries of FreqBandIndicatorNR[i] | pc_nrBandx ('x' being the band number/type related PICS listed in TS 38.508-2 [5]) |
| modifiedMPR-Behaviour | Not checked |  |  |
| mimo-ParametersPerBand | Not checked | MIMO-ParametersPerBand[i] |  |
| extendedCP | Not checked |  |  |
| multipleTCI | supported |  |  |
| bwp-WithoutRestriction | Checked |  | pc_bwp_WithoutRestriction |
| bwp-SameNumerology | Checked (NOTE 21) |  | pc_bwp_SameNumerology_upto2_FR1_FDD or pc_bwp_SameNumerology_upto2_FR1_TDD or pc_bwp_SameNumerology_upto2_FR2 or pc_bwp_SameNumerology_upto4_FR1_FDD or pc_bwp_SameNumerology_upto4_FR1_TDD or pc_bwp_SameNumerology_upto4_FR2 |
| bwp-DiffNumerology | upto4 (NOTE 20) |  | pc_bwp_DiffNumerology_FR1_FDD or pc_bwp_DiffNumerology_FR1_TDD or pc_bwp_DiffNumerology_FR2 |
| crossCarrierScheduling-SameSCS | Not checked |  |  |
| pdsch-256QAM-FR2 | Checked (NOTE 8) |  | pc_pdsch_256QAM_FR2 |
| pusch-256QAM | Checked (NOTE 9) |  | pc_pusch_256QAM_FR1 OR pc_pusch_256QAM_FR2 |
| ue-PowerClass | Not checked |  |  |
| rateMatchingLTE-CRS | Not checked |  |  |
| channelBWs-DL-v1530 CHOICE { |  |  |  |
| fr1 SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| } |  |  |  |
| fr2 SEQUENCE { |  |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| channelBWs-UL-v1530[i] CHOICE { |  |  |  |
| fr1 SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| } |  |  |  |
| fr2 SEQUENCE { |  |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| sharedSpectrumChAccessParamsPerBand-r16 | Not checked |  |  |
| cancelOverlappingPUSCH-r16 | Not checked |  |  |
| multipleRateMatchingEUTRA-CRS-r16 | Not checked |  |  |
| overlapRateMatchingEUTRA-CRS-r16 | Not checked |  |  |
| pdsch-MappingTypeB-Alt-r16 | Not checked |  |  |
| oneSlotPeriodicTRS-r16 | Not checked |  |  |
| olpc-SRS-Pos-r16 | Not checked |  |  |
| spatialRelationsSRS-Pos-r16 | Not checked |  |  |
| simulSRS-MIMO-TransWithinBand-r16 | Not checked |  |  |
| channelBW-DL-IAB-r16 | Not checked |  |  |
| channelBW-UL-IAB-r16 | Not checked |  |  |
| rasterShift7dot5-IAB-r16 | Not checked |  |  |
| ue-PowerClass-v1610 | Not checked |  |  |
| condHandover-r16 | Checked |  | pc_condHandover_r16 |
| condHandoverFailure-r16 | Checked |  | pc_condHandoverFailure_r16 |
| condHandoverTwoTriggerEvents-r16 | Checked |  | pc_condHandoverTwoTriggerEvents_r16 |
| condPSCellChange-r16 | Checked |  | pc_condPSCellChange_r16 |
| condPSCellChangeTwoTriggerEvents-r16 | Not checked |  |  |
| mpr-PowerBoost-FR2-r16 | Not checked |  |  |
| activeConfiguredGrant-r16 | Not checked |  |  |
| jointReleaseConfiguredGrantType2-r16 | Not checked |  |  |
| sps-r16 | Checked |  | pc_multi_sps_r16 |
| jointReleaseSPS-r16 | Not checked |  |  |
| simulSRS-TransWithinBand-r16 | Not checked |  |  |
| trs-AdditionalBandwidth-r16 | Not checked |  |  |
| handoverIntraF-IAB-r16 | Not checked |  |  |
| maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 | Checked |  | pc_nrBandx_maxUplinkDutyCycle_PC1dot5_MPE_FR1_r16 (‘x’ being the band number related PICS listed in Table A.4.3.1-4h of TS 38.508-2 [2]) |
| txDiversity-r16 | Checked |  | pc_txDiversity_r16 |
| pdsch-1024QAM-FR1-r17 | Not checked |  |  |
| ue-PowerClass-v1700 | Not checked |  |  |
| fr2-2-AccessParamsPerBand-r17 | Not checked |  |  |
| rlm-Relaxation-r17 | Not checked |  |  |
| bfd-Relaxation-r17 | Not checked |  |  |
| cg-SDT-r17 | Checked |  | pc_cg_SDT_r17 |
| locationBasedCondHandover-r17 | Not checked |  |  |
| timeBasedCondHandover-r17 | Not checked |  |  |
| eventA4BasedCondHandover-r17 | Not checked |  |  |
| mn-InitiatedCondPSCellChangeNRDC-r17 | Not checked |  |  |
| sn-InitiatedCondPSCellChangeNRDC-r17 | Not checked |  |  |
| pdcch-SkippingWithoutSSSG-r17 | Not checked |  |  |
| sssg-Switching-1BitInd-r17 | Not checked |  |  |
| sssg-Switching-2BitInd-r17 | Not checked |  |  |
| pdcch-SkippingWithSSSG-r17 | Not checked |  |  |
| searchSpaceSetGrp-switchCap2-r17 | Not checked |  |  |
| uplinkPreCompensation-r17 | Not checked |  |  |
| uplink-TA-Reporting-r17 | Not checked |  |  |
| max-HARQ-ProcessNumber-r17 | Not checked |  |  |
| type2-HARQ-Codebook-r17 | Not checked |  |  |
| type1-HARQ-Codebook-r17 | Not checked |  |  |
| type3-HARQ-Codebook-r17 | Not checked |  |  |
| ue-specific-K-Offset-r17 | Not checked |  |  |
| multiPDSCH-SingleDCI-FR2-1-SCS-120kHz-r17 | Not checked |  |  |
| multiPUSCH-SingleDCI-FR2-1-SCS-120kHz-r17 | Not checked |  |  |
| parallelPRS-MeasRRC-Inactive-r17 | Not checked |  |  |
| nr-UE-TxTEG-ID-MaxSupport-r17 | Not checked |  |  |
| prs-ProcessingRRC-Inactive-r17 | Not checked |  |  |
| prs-ProcessingWindowType1A-r17 | Not checked |  |  |
| prs-ProcessingWindowType1B-r17 | Not checked |  |  |
| prs-ProcessingWindowType2-r17 | Not checked |  |  |
| srs-AllPosResourcesRRC-Inactive-r17 | Not checked |  |  |
| olpc-SRS-PosRRC-Inactive-r17 | Not checked |  |  |
| spatialRelationsSRS-PosRRC-Inactive-r17 | Not checked |  |  |
| maxNumberPUSCH-TypeA-Repetition-r17 | Checked |  | pc_maxNumberPUSCH_TypeA_Repetition_r17 |
| puschTypeA-RepetitionsAvailSlot-r17 | Checked |  | pc_puschTypeA_RepetitionsAvailSlot_r17 |
| tb-ProcessingMultiSlotPUSCH-r17 | Checked |  | pc_tb_ProcessingMultiSlotPUSCH_r17 |
| tb-ProcessingRepMultiSlotPUSCH-r17 | Checked |  | pc_tb_ProcessingRepMultiSlotPUSCH_r17 |
| maxDurationDMRS-Bundling-r17 | Not checked |  |  |
| pusch-RepetitionMsg3-r17 | Checked |  | pc_pusch_RepetitionMsg3_r17 |
| sharedSpectrumChAccessParamsPerBand-v1710 | Not checked |  |  |
| parallelMeasurementWithoutRestriction-r17 | Not checked |  |  |
| maxNumber-NGSO-SatellitesWithinOneSMTC-r17 | Not checked |  |  |
| k1-RangeExtension-r17 | Not checked |  |  |
| aperiodicCSI-RS-FastScellActivation-r17 | Not checked |  |  |
| aperiodicCSI-RS-AdditionalBandwidth-r17 | Not checked |  |  |
| bwp-WithoutCD-SSB-OrNCD-SSB-RedCap-r17 | Not checked |  |  |
| halfDuplexFDD-TypeA-RedCap-r17 | Checked |  | pc_halfDuplexFDD_TypeA_RedCap_r17 |
| posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 PosSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 | Not checked |  |  |
| channelBWs-DL-SCS-480kHz-FR2-2-r17 | Not checked |  |  |
| channelBWs-UL-SCS-480kHz-FR2-2-r17 | Not checked |  |  |
| channelBWs-DL-SCS-960kHz-FR2-2-r17 | Not checked |  |  |
| channelBWs-UL-SCS-960kHz-FR2-2-r17 | Not checked |  |  |
| ul-GapFR2-r17 | Not checked |  |  |
| oneShotHARQ-feedbackTriggeredByDCI-1-2-r17 | Not checked |  |  |
| oneShotHARQ-feedbackPhy-Priority-r17 | Not checked |  |  |
| enhancedType3-HARQ-CodebookFeedback-r17 | Not checked |  |  |
| triggeredHARQ-CodebookRetx-r17 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| supportedBandCombinationList SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination { |  |  |  |
| bandList SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters { |  |  |  |
| BandParameters CHOICE { |  |  |  |
| eutra SEQUENCE { |  |  |  |
| bandEUTRA | Not checked | FreqBandIndicatorEUTRA |  |
| ca-BandwidthClassDL-EUTRA | Not checked | CA-BandwidthClassEUTRA |  |
| ca-BandwidthClassUL-EUTRA | Not checked | CA-BandwidthClassEUTRA |  |
| } |  |  |  |
| nr SEQUENCE { |  |  |  |
| bandNR | Not checked | FreqBandIndicatorNR |  |
| ca-BandwidthClassDL-NR | Not checked | CA-BandwidthClassNR |  |
| ca-BandwidthClassUL-NR | Not checked | CA-BandwidthClassNR |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSetCombination | Not checked | FeatureSetCombinationId |  |
| ca-ParametersEUTRA | Not checked | CA-ParametersEUTRA |  |
| ca-ParametersNR | Not checked | CA-ParametersNR |  |
| mrdc-Parameters | Not checked | MRDC-Parameters |  |
| supportedBandwidthCombinationSet | Not checked |  |  |
| powerClass-v1530 | Not checked |  |  |
| } |  |  |  |
| appliedFreqBandListFilter | Not checked | FreqBandList |  |
| supportedBandCombinationList-v1610 SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1610 { |  | BandCombinationList-v1610 |  |
| BandCombination-v1610 SEQUENCE { |  |  |  |
| bandList-v1610 SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1610 { |  |  |  |
| ca-ParametersNR-v1610 | Checked | CA-ParametersNR-v1610 (Table 8.1.5.1.1.3.3-8 |  |
| ca-ParametersNRDC-v1610 | Not checked | CA-ParametersNRDC-v1610 |  |
| powerClass-v1610 | Not checked |  |  |
| powerClassNRPart-r16 | Not checked |  |  |
| featureSetCombinationDAPS-r16 | Not checked | FeatureSetCombinationId |  |
| mrdc-Parameters-v1620 | Not checked | MRDC-Parameters-v1620 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| supportedBandCombinationListSidelinkEUTRA-NR-r16 | Not checked | BandCombinationListSidelinkEUTRA-NR-r16 |  |
| supportedBandCombinationList-UplinkTxSwitch-r16 | Not checked | BandCombinationList-UplinkTxSwitch-r16 |  |
| extendedBand-n77-r16 | Checked |  | pc_extendedBand_n77_r16 |
| supportedBandCombinationList-v1700 | Not checked | BandCombinationList-v1700 |  |
| supportedBandCombinationList-UplinkTxSwitch-v1700 | Not checked | BandCombinationList-UplinkTxSwitch-v1700 |  |
| supportedBandCombinationListSL-RelayDiscovery-r17 | Not checked |  |  |
| supportedBandCombinationListSL-NonRelayDiscovery-r17 | Not checked |  |  |
| supportedBandCombinationListSidelinkEUTRA-NR-v1710 | Not checked | BandCombinationListSidelinkEUTRA-NR-v1710 |  |
| sidelinkRequested-r17 | Not checked |  |  |
| extendedBand-n77-2-r17 | Checked |  | pc_extendedBand_n77_2_r17 |
| } |  |  |  |
| measAndMobParameters SEQUENCE { |  |  |  |
| measAndMobParametersCommon SEQUENCE { |  |  |  |
| supportedGapPattern | Checked | Acc. to TS 38.306 [23] clause 4.2.9: Indicates measurement gap pattern(s) optionally supported by the UE for NR SA. The leading / leftmost bit (bit 0) corresponds to the gap pattern 2, the next bit corresponds to the gap pattern 3, as specified in TS 38.133 [10] and so on. The UE shall set the bits corresponding to the measurement gap pattern 13, 14, 17, 18 and 19 to 1 if the UE is an NR standalone capable UE that supports a band in FR2. | pc_gp2_nr, pc_gp3_nr, pc_gp4_nr, pc_gp5_nr, pc_gp6_nr, pc_gp7_nr, pc_gp8_nr, pc_gp9_nr, pc_gp10_nr, pc_gp11_nr, pc_gp12_nr, pc_gp15_nr, pc_gp16_nr, pc_gp17_nr, pc_gp18_nr, pc_gp19_nr, pc_gp20_nr, pc_gp21_nr, pc_gp22_nr, pc_gp23_nr |
| ssb-RLM | supported |  |  |
| ssb-AndCSI-RS-RLM | Not checked |  |  |
| eventB-MeasAndReport | Checked |  | pc_eventB_MeasAndReport |
| handoverFDD-TDD | Not checked |  |  |
| eutra-CGI-Reporting | Checked |  | pc_eutra_CGI_Reporting |
| nr-CGI-Reporting | Checked |  | pc_nr_CGI_Reporting |
| independentGapConfig | Checked |  | pc_independentGapConfig |
| periodicEUTRA-MeasAndReport | Checked |  | pc_periodicEUTRA_MeasAndReport |
| handoverFR1-FR2 | Not checked |  |  |
| maxNumberCSI-RS-RRM-RS-SINR | Not checked |  |  |
| reportAddNeighMeasForPeriodic-r16 | Not checked |  |  |
| condHandoverParametersCommon-r16 | Not checked |  |  |
| nr-NeedForGap-Reporting-r16 | Not checked |  |  |
| supportedGapPattern-NRonly-r16 | Not checked |  |  |
| supportedGapPattern-NRonly-NEDC-r16 | Not checked |  |  |
| maxNumberCLI-RSSI-r16 | Not checked |  |  |
| maxNumberCLI-SRS-RSRP-r16 | Not checked |  |  |
| maxNumberPerSlotCLI-SRS-RSRP-r16 | Not checked |  |  |
| mfbi-IAB-r16 | Not checked |  |  |
| dummy | Not checked |  |  |
| nr-CGI-Reporting-NPN-r16 | Checked |  | pc_nr_CGI_Reporting_NPN_r16 |
| idleInactiveEUTRA-MeasReport-r16 | Checked |  | pc_idleInactiveEUTRA_MeasReport |
| idleInactive-ValidityArea-r16 | Not checked |  |  |
| eutra-AutonomousGaps-r16 | Not checked |  |  |
| eutra-AutonomousGaps-NEDC-r16 | Not checked |  |  |
| eutra-AutonomousGaps-NRDC-r16 | Not checked |  |  |
| pcellT312-r16 | Not checked |  |  |
| supportedGapPattern-r16 | Not checked |  |  |
| concurrentMeasGap-r17 | Not checked |  |  |
| nr-NeedForGapNCSG-Reporting-r17 | Not checked |  |  |
| eutra-NeedForGapNCSG-Reporting-r17 | Not checked |  |  |
| ncsg-MeasGapPerFR-r17 | Not checked |  |  |
| ncsg-MeasGapPatterns-r17 | Not checked |  |  |
| ncsg-MeasGapNR-Patterns-r17 | Not checked |  |  |
| preconfiguredUE-AutonomousMeasGap-r17 | Not checked |  |  |
| preconfiguredNW-ControlledMeasGap-r17 | Not checked |  |  |
| handoverFR1-FR2-2-r17 | Not checked |  |  |
| handoverFR2-1-FR2-2-r17 | Not checked |  |  |
| independentGapConfigPRS-r17 | Not checked |  |  |
| rrm-RelaxationRRC-ConnectedRedCap-r17 | Checked |  | pc_ rrm_RelaxationRRC_ConnectedRedCap_r17 |
| parallelMeasurementGap-r17 | Not checked |  |  |
| condHandoverWithSCG-NRDC-r17 | Not checked |  |  |
| gNB-ID-LengthReporting-r17 | Not checked |  |  |
| gNB-ID-LengthReporting-ENDC-r17 | Not checked |  |  |
| gNB-ID-LengthReporting-NEDC-r17 | Not checked |  |  |
| gNB-ID-LengthReporting-NRDC-r17 | Not checked |  |  |
| gNB-ID-LengthReporting-NPN-r17 | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersXDD-Diff SEQUENCE { |  |  |  |
| intraAndInterF-MeasAndReport | Checked (NOTE 1) |  | pc_intraAndInterF_MeasAndReport |
| eventA-MeasAndReport | Checked (NOTE 2) |  | pc_eventA_MeasAndReport |
| handoverInterF | Checked (NOTE 15) |  | pc_ handoverInterF |
| handoverLTE-EPC | Not checked |  |  |
| handoverLTE-5GC | Not checked |  |  |
| sftd-MeasNR-Neigh | Checked (NOTE 13) |  | pc_SFTD_MeasNR_Neigh_FDD and/or pc_SFTD_MeasNR_Neigh_TDD |
| sftd-MeasNR-Neigh-DRX | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |
| ss-SINR-Meas | Checked (NOTE 10) |  | pc_ss_SINR_Meas |
| csi-RSRP-AndRSRQ-MeasWithSSB | Checked (NOTE 3) |  | pc_csi_RSRP_AndRSRQ_MeasWithSSB |
| csi-RSRP-AndRSRQ-MeasWithoutSSB | Checked (NOTE 11) |  | pc_csi_RSRP_AndRSRQ_MeasWithoutSSB |
| csi-SINR-Meas | Not checked |  |  |
| csi-RS-RLM | Not checked |  |  |
| handoverInterF | Checked (NOTE 14) |  | pc_ handoverInterF |
| handoverLTE-EPC | Not checked |  |  |
| handoverLTE-5GC | Not checked |  |  |
| nr-AutonomousGaps-r16 | Not checked |  |  |
| nr-AutonomousGaps-ENDC-r16 | Not checked |  |  |
| nr-AutonomousGaps-NEDC-r16 | Not checked |  |  |
| nr-AutonomousGaps-NRDC-r16 | Not checked |  |  |
| dummy | Not checked |  |  |
| cli-RSSI-Meas-r16 | Not checked |  |  |
| cli-SRS-RSRP-Meas-r16 | Not checked |  |  |
| interFrequencyMeas-NoGap-r16 | Not checked |  |  |
| simultaneousRxDataSSB-DiffNumerology-Inter-r16 | Not checked |  |  |
| idleInactiveNR-MeasReport-r16 | Checked (NOTE 22) |  | pc_idleInactiveNR_MeasReport |
| idleInactiveNR-MeasBeamReport-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fdd-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersXDD-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA | Checked (NOTE 17) |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| dl-SchedulingOffset-PDSCH-TypeB | Not checked |  |  |
| ul-SchedulingOffset | Not checked |  |  |
| } |  |  |  |
| mac-ParametersXDD-Diff SEQUENCE { |  |  |  |
| skipUplinkTxDynamic | Checked (NOTE 4) |  | pc_skipUplinkTxDynamic |
| logicalChannelSR-DelayTimer | Checked (NOTE 5) |  | pc_logicalChannelSR_DelayTimer |
| longDRX-Cycle | Checked (NOTE 6) |  | pc_longDRX_Cycle |
| shortDRX-Cycle | Checked (NOTE 7) |  | pc_shortDRX_Cycle |
| multipleSR-Configurations | Not checked |  |  |
| multipleConfiguredGrants | Checked (NOTE 12) |  | pc_multipleConfiguredGrants_r16 |
| } |  |  |  |
| measAndMobParametersXDD-Diff SEQUENCE { |  |  |  |
| intraAndInterF-MeasAndReport | Checked (NOTE 1) |  | pc_intraAndInterF_MeasAndReport |
| eventA-MeasAndReport | Checked (NOTE 2) |  | pc_eventA_MeasAndReport |
| handoverInterF | Checked (NOTE 15) |  | pc_ handoverInterF |
| handoverLTE-EPC | Not checked |  |  |
| handoverLTE-5GC | Not checked |  |  |
| sftd-MeasNR-Neigh | Checked (NOTE 13) |  | pc_SFTD_MeasNR_Neigh_FDD |
| sftd-MeasNR-Neigh-DRX | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| tdd-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersXDD-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA | Checked (NOTE 17) |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| dl-SchedulingOffset-PDSCH-TypeB | Not checked |  |  |
| ul-SchedulingOffset | Not checked |  |  |
| } |  |  |  |
| mac-ParametersXDD-Diff SEQUENCE { |  |  |  |
| skipUplinkTxDynamic | Checked (NOTE 4) |  | pc_skipUplinkTxDynamic |
| logicalChannelSR-DelayTimer | Checked (NOTE 5) |  | pc_logicalChannelSR_DelayTimer |
| longDRX-Cycle | Checked (NOTE 6) |  | pc_longDRX_Cycle |
| shortDRX-Cycle | Checked (NOTE 7) |  | pc_shortDRX_Cycle |
| multipleSR-Configurations | Not checked |  |  |
| multipleConfiguredGrants | Checked (NOTE 12) |  | pc_multipleConfiguredGrants_r16 |
| } |  |  |  |
| measAndMobParametersXDD-Diff SEQUENCE { |  |  |  |
| intraAndInterF-MeasAndReport | Checked (NOTE 1) |  | pc_intraAndInterF_MeasAndReport |
| eventA-MeasAndReport | Checked (NOTE 2) |  | pc_eventA_MeasAndReport |
| handoverInterF | Checked (NOTE 15) |  | pc_ handoverInterF |
| handoverLTE-EPC | Not checked |  |  |
| handoverLTE-5GC | Not checked |  |  |
| sftd-MeasNR-Neigh | Checked (NOTE 13) |  | pc_SFTD_MeasNR_Neigh_TDD |
| sftd-MeasNR-Neigh-DRX | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr1-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| dummy1 | Not checked |  |  |
| twoFL-DMRS | Not checked |  |  |
| dummy2 | Not checked |  |  |
| dummy3 | Not checked |  |  |
| supportedDMRS-TypeDL | Not checked |  |  |
| supportedDMRS-TypeUL | Not checked |  |  |
| semiOpenLoopCSI | Not checked |  |  |
| csi-ReportWithoutPMI | Not checked |  |  |
| csi-ReportWithoutCQI | Not checked |  |  |
| onePortsPTRS | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| pucch-F2-WithFH | Not checked |  |  |
| pucch-F3-WithFH | Not checked |  |  |
| pucch-F4-WithFH | Not checked |  |  |
| freqHoppingPUCCH-F0-2 | Not checked |  |  |
| freqHoppingPUCCH-F1-3-4 | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot | Not checked |  |  |
| uci-CodeBlockSegmentation | Not checked |  |  |
| onePUCCH-LongAndShortFormat | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot | Checked (NOTE 26) |  | pc_twoPUCCH_AnyOthersInSlot |
| intraSlotFreqHopping-PUSCH | Not checked |  |  |
| pusch-LBRM | Not checked |  |  |
| pdcch-BlindDetectionCA | Not checked |  |  |
| tpc-PUSCH-RNTI | Not checked |  |  |
| tpc-PUCCH-RNTI | Not checked |  |  |
| tpc-SRS-RNTI | Not checked |  |  |
| absoluteTPC-Command | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| pusch-HalfPi-BPSK | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK | Not checked |  |  |
| almostContiguousCP-OFDM-UL | Not checked |  |  |
| sp-CSI-RS | Not checked |  |  |
| sp-CSI-IM | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot | Not checked |  |  |
| multipleCORESET | Not checked |  |  |
| csi-RS-IM-ReceptionForFeedback | Not checked |  |  |
| csi-RS-ProcFrameworkForSRS | Not checked |  |  |
| csi-ReportFramework | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot SEQUENCE { |  |  |  |
| sameSymbol | Not checked |  |  |
| diffSymbol | Not checked |  |  |
| } |  |  |  |
| mux-SR-HARQ-ACK-PUCCH | Not checked |  |  |
| mux-MultipleGroupCtrlCH-Overlap | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA | Checked (NOTE 18) |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| dl-SchedulingOffset-PDSCH-TypeB | Not checked |  |  |
| ul-SchedulingOffset | Not checked |  |  |
| dl-64QAM-MCS-TableAlt | Not checked |  |  |
| ul-64QAM-MCS-TableAlt | Not checked |  |  |
| cqi-TableAlt | Not checked |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-UL | Not checked |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-UL | Not checked |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-UL | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |
| ss-SINR-Meas | Checked (NOTE 10) |  | pc_ss_SINR_Meas |
| csi-RSRP-AndRSRQ-MeasWithSSB | Checked (NOTE 3) |  | pc_csi_RSRP_AndRSRQ_MeasWithSSB |
| csi-RSRP-AndRSRQ-MeasWithoutSSB | Checked (NOTE 11) |  | pc_csi_RSRP_AndRSRQ_MeasWithoutSSB |
| csi-SINR-Meas | Not checked |  |  |
| csi-RS-RLM | Not checked |  |  |
| handoverInterF | Checked (NOTE 14) |  | pc_ handoverInterF |
| handoverLTE-EPC | Not checked |  |  |
| handoverLTE-5GC | Not checked |  |  |
| nr-AutonomousGaps-r16 | Not checked |  |  |
| nr-AutonomousGaps-ENDC-r16 | Not checked |  |  |
| nr-AutonomousGaps-NEDC-r16 | Not checked |  |  |
| nr-AutonomousGaps-NRDC-r16 | Not checked |  |  |
| dummy | Not checked |  |  |
| cli-RSSI-Meas-r16 | Not checked |  |  |
| cli-SRS-RSRP-Meas-r16 | Not checked |  |  |
| interFrequencyMeas-NoGap-r16 | Not checked |  |  |
| simultaneousRxDataSSB-DiffNumerology-Inter-r16 | Not checked |  |  |
| idleInactiveNR-MeasReport-r16 | Checked (NOTE 22) |  | pc_idleInactiveNR_MeasReport |
| idleInactiveNR-MeasBeamReport-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr2-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { | Not checked |  |  |
| dynamicSFI | Not checked |  |  |
| dummy1 | Not checked |  |  |
| twoFL-DMRS | Not checked |  |  |
| dummy2 | Not checked |  |  |
| dummy3 | Not checked |  |  |
| supportedDMRS-TypeDL | Not checked |  |  |
| supportedDMRS-TypeUL | Not checked |  |  |
| semiOpenLoopCSI | Not checked |  |  |
| csi-ReportWithoutPMI | Not checked |  |  |
| csi-ReportWithoutCQI | Not checked |  |  |
| onePortsPTRS | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| pucch-F2-WithFH | Not checked |  |  |
| pucch-F3-WithFH | Not checked |  |  |
| pucch-F4-WithFH | Not checked |  |  |
| freqHoppingPUCCH-F0-2 | Not checked |  |  |
| freqHoppingPUCCH-F1-3-4 | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot | Not checked |  |  |
| uci-CodeBlockSegmentation | Not checked |  |  |
| onePUCCH-LongAndShortFormat | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot | Checked (NOTE 26) |  | pc_twoPUCCH_AnyOthersInSlot |
| intraSlotFreqHopping-PUSCH | Not checked |  |  |
| pusch-LBRM | Not checked |  |  |
| pdcch-BlindDetectionCA | Not checked |  |  |
| tpc-PUSCH-RNTI | Not checked |  |  |
| tpc-PUCCH-RNTI | Not checked |  |  |
| tpc-SRS-RNTI | Not checked |  |  |
| absoluteTPC-Command | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| pusch-HalfPi-BPSK | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK | Not checked |  |  |
| almostContiguousCP-OFDM-UL | Not checked |  |  |
| sp-CSI-RS | Not checked |  |  |
| sp-CSI-IM | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot | Not checked |  |  |
| multipleCORESET | Not checked |  |  |
| csi-RS-IM-ReceptionForFeedback | Not checked |  |  |
| csi-RS-ProcFrameworkForSRS | Not checked |  |  |
| csi-ReportFramework | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot SEQUENCE { |  |  |  |
| sameSymbol | Not checked |  |  |
| diffSymbol | Not checked |  |  |
| } |  |  |  |
| mux-SR-HARQ-ACK-PUCCH | Not checked |  |  |
| mux-MultipleGroupCtrlCH-Overlap | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA | Checked (NOTE 18) |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| dl-SchedulingOffset-PDSCH-TypeB | Not checked |  |  |
| ul-SchedulingOffset | Not checked |  |  |
| dl-64QAM-MCS-TableAlt | Not checked |  |  |
| ul-64QAM-MCS-TableAlt | Not checked |  |  |
| cqi-TableAlt | Not checked |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-UL | Not checked |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-UL | Not checked |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-UL | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |
| ss-SINR-Meas | Checked (NOTE 10) |  | pc_ss_SINR_Meas |
| csi-RSRP-AndRSRQ-MeasWithSSB | Checked (NOTE 3) |  | pc_csi_RSRP_AndRSRQ_MeasWithSSB |
| csi-RSRP-AndRSRQ-MeasWithoutSSB | Checked (NOTE 11) |  | pc_csi_RSRP_AndRSRQ_MeasWithoutSSB |
| csi-SINR-Meas | Not checked |  |  |
| csi-RS-RLM | Not checked |  |  |
| handoverInterF | Checked (NOTE 14) |  | pc_ handoverInterF |
| handoverLTE-EPC | Not checked |  |  |
| handoverLTE-5GC | Not checked |  |  |
| nr-AutonomousGaps-r16 | Not checked |  |  |
| nr-AutonomousGaps-ENDC-r16 | Not checked |  |  |
| nr-AutonomousGaps-NEDC-r16 | Not checked |  |  |
| nr-AutonomousGaps-NRDC-r16 | Not checked |  |  |
| dummy | Not checked |  |  |
| cli-RSSI-Meas-r16 | Not checked |  |  |
| cli-SRS-RSRP-Meas-r16 | Not checked |  |  |
| interFrequencyMeas-NoGap-r16 | Not checked |  |  |
| simultaneousRxDataSSB-DiffNumerology-Inter-r16 | Not checked |  |  |
| idleInactiveNR-MeasReport-r16 | Checked (NOTE 22) |  | pc_idleInactiveNR_MeasReport |
| idleInactiveNR-MeasBeamReport-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSets SEQUENCE { |  |  |  |
| featureSetsDownlink SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink { |  |  |  |
| FeatureSetDownlink SEQUENCE { | Not checked |  |  |
| featureSetListPerDownlinkCC | Not checked |  |  |
| intraBandFreqSeparationDL | Not checked |  |  |
| scalingFactor | Not checked |  |  |
| crossCarrierScheduling-OtherSCS | Not checked |  |  |
| scellWithoutSSB | Not checked |  |  |
| csi-RS-MeasSCellWithoutSSB | Not checked |  |  |
| dummy1 | Not checked |  |  |
| type1-3-CSS | Not checked |  |  |
| pdcch-MonitoringAnyOccasions | Not checked |  |  |
| dummy2 | Not checked |  |  |
| ue-SpecificUL-DL-Assignment | Not checked |  |  |
| searchSpaceSharingCA-DL | Not checked |  |  |
| timeDurationForQCL SEQUENCE { |  |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| pdsch-ProcessingType1-DifferentTB-PerSlot SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| dummy3 | Not checked |  |  |
| dummy4 | Not checked |  |  |
| dummy5 | Not checked |  |  |
| dummy6 | Not checked |  |  |
| dummy7 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSetsDownlinkPerCC SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC { |  |  |  |
| FeatureSetDownlinkPerCCSEQUENCE { |  |  |  |
| supportedSubcarreirSpacingDL | Not checked | SubcarrierSpacing |  |
| supportedBandwidthDL | Not checked | SupportedBandwidth |  |
| channelBW-90mhz | Not checked |  |  |
| maxNumberMIMO-LayersPDSCH | Checked | MIMO-LayersDL | pc_maxNumberMIMO_LayersPDSCH_eightLayers or pc_maxNumberMIMO_LayersPDSCH_fourLayers or pc_maxNumberMIMO_LayersPDSCH_twoLayers |
| supportedModulationOrderDL | Not checked | ModulationOrder |  |
| } |  |  |  |
| } |  |  |  |
| featureSetsUplink SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink { |  |  |  |
| FeatureSetUplink SEQUENCE { |  |  |  |
| featureSetListPerUplinkCC | Not checked |  |  |
| scalingFactor | Not checked |  |  |
| crossCarrierScheduling-OtherSCS | Not checked |  |  |
| intraBandFreqSeparationUL | Not checked |  |  |
| searchSpaceSharingCA-UL | Not checked |  |  |
| dummy1 | Not checked |  |  |
| supportedSRS-Resources | Not checked | SRS-Resources |  |
| twoPUCCH-Group | Checked |  | pc_twoPUCCH_group |
| dynamicSwitchSUL | Checked |  | pc_dynamicSwitch_SUL |
| simultaneousTxSUL-NonSUL | Not checked |  |  |
| pusch-ProcessingType1-DifferentTB-PerSlot SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| dummy2 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSetsUplinkPerCC SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC { |  |  |  |
| FeatureSetUplinkPerCCSEQUENCE { |  |  |  |
| supportedSubcarrierSpacingUL | Not checked | SubcarrierSpacing |  |
| supportedBandwidthUL | Not checked | SupportedBandwidth |  |
| channelBW-90mHz | Not checked |  |  |
| mimo-CB-PUSCH SEQUENCE { |  |  |  |
| maxNumberMIMO-LayersCB-PUSCH | Not checked |  |  |
| maxNumberSRS-ResourcePerSet | Not checked |  |  |
| } |  |  |  |
| maxNumberMIMO-LayersNonCB-PUSCH | Not checked |  |  |
| supportedModulationOrderUL | Not checked | ModulationOrder |  |
| } |  |  |  |
| } |  |  |  |
| featureSetsDownlink-v1610 SEQUENCE (SIZE (1.. maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1610 | Checked | FeatureSetDownlink-v1610 (Table 8.1.5.1.1.3.3-6) |  |
| featureSetsUplink-v1610 SEQUENCE (SIZE (1.. maxUplinkFeatureSets)) OF FeatureSetUplink-v1610 | Checked | FeatureSetUplink-v1610 (Table 8.1.5.1.1.3.3-7) |  |
| featureSetDownlinkPerCC-v1620 SEQUENCE (SIZE (1.. maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1620 | Not checked |  |  |
| } |  |  |  |
| featureSetCombinations SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination { |  |  |  |
| FeatureSetCombination SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FeatureSetsPerBand { |  |  |  |
| FeatureSetsPerBand SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSet { |  |  |  |
| FeatureSetCHOICE { |  |  |  |
| eutra SEQUENCE { |  |  |  |
| downlinkSetEUTRA | Not checked | FeatureSetEUTRA-DownlinkId |  |
| uplinkSetEUTRA | Not checked | FeatureSetEUTRA-UplinkId |  |
| } |  |  |  |
| nr SEQUENCE { |  |  |  |
| downlinkSetNR | Not checked | FeatureSetDownlinkId |  |
| uplinkSetNR | Not checked | FeatureSetUpinkId |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| fdd-Add-UE-NR-Capabilities-v1530 SEQUENCE { |  |  |  |
| eutra-ParametersXDD-Diff SEQUENCE { |  |  |  |
| rsrqMeasWidebandEUTRA | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| tdd-Add-UE-NR-Capabilities-v1530 SEQUENCE { |  |  |  |
| eutra-ParametersXDD-Diff SEQUENCE { |  |  |  |
| rsrqMeasWidebandEUTRA | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| dummy | Not checked |  |  |
| interRAT-Parameters SEQUENCE { |  |  |  |
| eutra SEQUENCE { |  |  |  |
| supportedBandListEUTRA SEQUENCE (SIZE (1..maxBandsEUTRA)) OF FreqBandIndicatorEUTRA | Not checked |  |  |
| eutra-ParametersCommon SEQUENCE { |  |  |  |
| mfbi-EUTRA | Not checked |  |  |
| modifiedMRP-BehaviorEUTRA | Not checked |  |  |
| multiNS-Pmax-EUTRA | Not checked |  |  |
| rs-SINR-MeasEUTRA | Checked |  | pc_RS_SINR_MeasEUTRA |
| ne-DC | Not checked |  |  |
|  | Not Present |  | pc_supportOfRedCap_r17 |
| nr-HO-ToEN-DC-r16 | Checked |  | pc_interRAT_NR_ToENDC |
|  | Not Present |  | pc_supportOfRedCap_r17 |
| } |  |  |  |
| eutra-ParametersXDD-Diff SEQUENCE { |  |  |  |
| rsrqMeasWidebandEUTRA | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| inactiveState | Checked |  | pc_inactiveState |
| delayBudgetReporting | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sdap-Parameters SEQUENCE { |  |  |  |
| as-ReflectiveQoS | Checked |  | pc_as_ReflectiveQoS |
| } |  |  |  |
| overheatingInd | Not checked |  |  |
| ims-Parameters SEQUENCE { |  |  |  |
| ims-ParametersCommon SEQUENCE { |  |  |  |
| voiceOverEUTRA-5GC | Not checked |  |  |
| voiceOverSCG-BearerEUTRA-5GC | Not checked |  |  |
| voiceFallbackIndicationEPS-r16 | Checked |  | pc_voiceFallbackIndication |
| } |  |  |  |
| ims-ParametersFRX-Diff SEQUENCE { |  |  |  |
| voiceOverNR | Checked (NOTE 16) |  | pc_voiceOverNR |
| } |  |  |  |
| } |  |  |  |
| fr1-Add-UE-NR-Capabilities-v1540 SEQUENCE { |  |  |  |
| ims-ParametersFRX-Diff SEQUENCE { |  |  |  |
| voiceOverNR | Checked (NOTE 16) |  | pc_voiceOverNR |
| } |  |  |  |
| } |  |  |  |
| fr2-Add-UE-NR-Capabilities-v1540 SEQUENCE { |  |  |  |
| ims-ParametersFRX-Diff SEQUENCE { |  |  |  |
| voiceOverNR | Checked (NOTE 16) |  | pc_voiceOverNR |
| } |  |  |  |
| } |  |  |  |
| fr1-fr2-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| dummy1 | Not checked |  |  |
| twoFL-DMRS | Not checked |  |  |
| dummy2 | Not checked |  |  |
| dummy3 | Not checked |  |  |
| supportedDMRS-TypeDL | Not checked |  |  |
| supportedDMRS-TypeUL | Not checked |  |  |
| semiOpenLoopCSI | Not checked |  |  |
| csi-ReportWithoutPMI | Not checked |  |  |
| csi-ReportWithoutCQI | Not checked |  |  |
| onePortsPTRS | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| pucch-F2-WithFH | Not checked |  |  |
| pucch-F3-WithFH | Not checked |  |  |
| pucch-F4-WithFH | Not checked |  |  |
| freqHoppingPUCCH-F0-2 | Not checked |  |  |
| freqHoppingPUCCH-F1-3-4 | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot | Not checked |  |  |
| uci-CodeBlockSegmentation | Not checked |  |  |
| onePUCCH-LongAndShortFormat | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot | Not checked |  |  |
| intraSlotFreqHopping-PUSCH | Not checked |  |  |
| pusch-LBRM | Not checked |  |  |
| pdcch-BlindDetectionCA | Not checked |  |  |
| tpc-PUSCH-RNTI | Not checked |  |  |
| tpc-PUCCH-RNTI | Not checked |  |  |
| tpc-SRS-RNTI | Not checked |  |  |
| absoluteTPC-Command | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| pusch-HalfPi-BPSK | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK | Not checked |  |  |
| almostContiguousCP-OFDM-UL | Not checked |  |  |
| sp-CSI-RS | Not checked |  |  |
| sp-CSI-IM | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot | Not checked |  |  |
| multipleCORESET | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |
| ss-SINR-Meas | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithSSB | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithoutSSB | Not checked |  |  |
| csi-SINR-Meas | Not checked |  |  |
| csi-RS-RLM | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-NR-Capability-v1550 |  |
| reducedCP-Latency | Checked |  | pc_reducedCP_Latency |
| nonCriticalExtension SEQUENCE { |  | UE-NR-Capability-v1560 |  |
| nrdc-Parameters | Not checked | NRDC-Parameters |  |
| receivedFilters | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-NR-Capability-v1570 |  |
| nrdc-Parameters-v1570 | Not checked | NRDC-Parameters-v1570 |  |
| nonCriticalExtension SEQUENCE { |  | UE-NR-Capability-v1610 |  |
| inDeviceCoexInd-r16 | Not checked |  |  |
| dl-DedicatedMessageSegmentation-r16 | Checked |  | pc_NR_dl_DedicatedMessageSegmentation |
| nrdc-Parameters-v1610 | Not checked |  |  |
| powSav-Parameters-r16 | Checked | PowSav-Parameters-r16 (Table 8.1.5.1.1.3.3-9) |  |
| fr1-Add-UE-NR-Capabilities-v1610 SEQUENCE { |  | UE-NR-CapabilityAddFRX-Mode-v1610 |  |
| powSav-ParametersFRX-Diff-r16 | Not checked |  |  |
| mac-ParametersFRX-Diff-r16 | Checked (NOTES 19, 23, 24, 25) | MAC-ParametersFRX-Diff-r16 (Table 8.1.5.1.1.3.3-11) |  |
| } |  |  |  |
| fr2-Add-UE-NR-Capabilities-v1610 SEQUENCE { |  | UE-NR-CapabilityAddFRX-Mode-v1610 |  |
| powSav-ParametersFRX-Diff-r16 | Not checked |  |  |
| mac-ParametersFRX-Diff-r16 | Checked (NOTES 19, 23, 24, 25) | MAC-ParametersFRX-Diff-r16 (Table 8.1.5.1.1.3.3-11) |  |
| } |  |  |  |
| bh-RLF-Indication-r16 | Not checked |  |  |
| directSN-AdditionFirstRRC-IAB-r16 | Not checked |  |  |
| bap-Parameters-r16 | Not checked |  |  |
| referenceTimeProvision-r16 | Not checked |  |  |
| sidelinkParameters-r16 SEQUENCE { |  |  |  |
| sidelinkParametersNR-r16 { |  |  |  |
| rlc-ParametersSidelink-r16 | Not checked |  |  |
| mac-ParametersSidelink-r16 | Not checked |  |  |
| fdd-Add-UE-Sidelink-Capabilities-r16 | Not checked |  |  |
| tdd-Add-UE-Sidelink-Capabilities-r16 | Not checked |  |  |
| supportedBandListSidelink-r16 SEQUENCE (SIZE (1..maxBands)) OF BandSidelink-r16 { |  |  |  |
| freqBandSidelink-r16 | Not checked |  |  |
| sl-Reception-r16 | Not checked |  |  |
| sl-TransmissionMode1-r16 | Checked |  | pc_NR_sl_TransmissionMode1_r16 |
| sync-Sidelink-r16 | Not checked |  |  |
| sl-Tx-256QAM-r16 | Not checked |  |  |
| psfch-FormatZeroSidelink-r16 | Not checked |  |  |
| lowSE-64QAM-MCS-TableSidelink-r16 | Not checked |  |  |
| enb-sync-Sidelink-r16 | Not checked |  |  |
| sl-TransmissionMode2-r16 | Checked | TS 38.306 [23] clause 4.2.16.1.6: Support of this feature is mandatory if UE supports NR sidelink | pc_NR_sl_TransmissionMode2_r16 |
| congestionControlSidelink-r16 | Not checked |  |  |
| fewerSymbolSlotSidelink-r16 | Not checked |  |  |
| sl-openLoopPC-RSRP-ReportSidelink-r16 | Not checked |  |  |
| sl-Rx-256QAM-r16 | Not checked |  |  |
| } |  |  |  |
| relayParameters-r17 | Not checked |  |  |
| p0_OLPC_Sidelink_r17 | Not checked |  |  |
| } |  |  |  |
| sidelinkParametersEUTRA-r16 | Not checked |  |  |
| } |  |  |  |
| highSpeedParameters-r16 | Not checked |  |  |
| mac-Parameters-v1610 SEQUENCE { |  | MAC-Parameters-v1610 |  |
| mac-ParametersFRX-Diff-r16 | Checked (NOTES 19, 23, 24, 25) | MAC-ParametersFRX-Diff-r16 (Table 8.1.5.1.1.3.3-11) |  |
| } |  |  |  |
| mcgRLF-RecoveryViaSCG-r16 | Not checked |  |  |
| resumeWithStoredMCG-SCells-r16 | Not checked |  |  |
| resumeWithStoredSCG-r16 | Not checked |  |  |
| resumeWithSCG-Config-r16 | Checked |  | pc_resumeWithSCG_Config_r16 |
| ue-BasedPerfMeas-Parameters-r16 | Checked | UE-BasedPerfMeas-Parameters-r16 (Table 8.1.5.1.1.3.3-10) |  |
| son-Parameters-r16 SEQUENCE { |  |  |  |
| rach-Report-r16 | Checked |  | pc_rachReport_r16 |
| rlfReportCHO-r17 | Not checked |  |  |
| rlfReportDAPS-r17 | Checked |  | pc_rlfReportDAPS_r17 |
| success-HO-Report-r17 | Checked |  | pc_success_HO_Report_r17 |
| twoStepRACH-Report-r17 | Checked |  | pc_twoStepRACH_Report_r17 |
| pscell-MHI-Report-r17 | Not checked |  |  |
| onDemandSI-Report-r17 | Checked |  | pc_onDemandSI_Report_r17 |
| } |  |  |  |
| onDemandSIB-Connected-r16 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-NR-Capability-v1640 |  |
| redirectAtResumeByNAS-r16 | Not checked |  |  |
| phy-ParametersSharedSpectrumChAccess-r16 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-NR-Capability-v1650 |  |
| mpsPriorityIndication-r16 | Checked |  | pc_NR_mpsPriorityIndication_r16 |
| highSpeedParameters-v1650 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-NR-Capability-v1690 |  |
| ul-RRC-Segmentation-r16 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-NR-Capability-v1700 |  |
| inactiveStatePO-Determination-r17 | Not checked |  |  |
| highSpeedParameters-v1700 | Not checked |  |  |
| powSav-Parameters-v1700 | Not checked |  |  |
| mac-Parameters-v1700 SEQUENCE { |  |  |  |
| mac-ParametersFR2-2-r17 SEQUENCE { |  | MAC-ParametersFR2-2-r17 |  |
| directMCG-SCellActivation-r17 | Not checked |  |  |
| directMCG-SCellActivationResume-r17 | Not checked |  |  |
| directSCG-SCellActivation-r17 | Not checked |  |  |
| directSCG-SCellActivationResume-r17 | Not checked |  |  |
| drx-Adaptation-r17 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| ims-Parameters-v1700 | Not checked |  |  |
| measAndMobParameters-v1700 | Not checked |  |  |
| appLayerMeasParameters-r17 | Not checked |  |  |
| redCapParameters-r17 SEQUENCE { |  |  |  |
| supportOfRedCap-r17 | Checked |  | pc_supportOfRedCap_r17 |
| supportOf16DRB-RedCap-r17 | Checked | Acc. to TS 38.306 [23] clause 4.2.21.2 “This capability is only applicable for RedCap UEs.” | pc_16DRB_RedCap _r17 |
| } |  |  |  |
| ra-SDT-r17 | Checked |  | pc_ra_SDT_r17 |
| srb-SDT-r17 | Checked |  | pc_srb_SDT_r17 |
| gNB-SideRTT-BasedPDC-r17 | Not checked |  |  |
| bh-RLF-DetectionRecovery-Indication-r17 | Not checked |  |  |
| nrdc-Parameters-v1700 | Not checked |  |  |
| bap-Parameters-v1700 | Not checked |  |  |
| musim-GapPreference-r17 | Checked |  | pc_musim_GapPreference_r17 |
| musimLeaveConnected-r17 | Checked |  | pc_musimLeaveConnected_r17 |
| mbs-Parameters-r17 | Not checked |  |  |
| nonTerrestrialNetwork-r17 | Not checked |  |  |
| ntn-ScenarioSupport-r17 | Not checked |  |  |
| sliceInfoforCellReselection-r17 | Checked |  | pc_sliceInfoforCellReselection_r17 |
| ue-RadioPagingInfo-r17 SEQUENCE { |  |  |  |
| pei-SubgroupingSupportBandList-r17 | Checked | Checked if any frequency band is reported | pc_pei_SubgroupingSupportBandList_r17 |
| } |  |  |  |
| ul-GapFR2-Pattern-r17 | Not checked |  |  |
| ntn-Parameters-r17 | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and intraAndInterF-MeasAndReport is supported in both modes, then support of intraAndInterF-MeasAndReport will be signalled in measAndMobParameters/measAndMobParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and intraAndInterF-MeasAndReport is only supported in one mode, then support of intraAndInterF-MeasAndReport will be signalled in one of fdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff as appropriate. Note 2: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and eventA-MeasAndReport is supported in both modes, then support of eventA-MeasAndReport will be signalled in measAndMobParameters/measAndMobParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and eventA-MeasAndReport is only supported in one mode, then support of eventA-MeasAndReport will be signalled in one of fdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff as appropriate. Note 3: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and csi-RSRP-AndRSRQ-MeasWithSSB is supported in both frequency ranges, then support of csi-RSRP-AndRSRQ-MeasWithSSB will be signaled in measAndMobParameters/measAndMobParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and csi-RSRP-AndRSRQ-MeasWithSSB is only supported in one frequency range, then support of csi-RSRP-AndRSRQ-MeasWithSSB will be signaled in one of fr1-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff as appropriate. Note 4: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and skipUplinkTxDynamic is supported in both modes, then support of skipUplinkTxDynamic will be signalled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and skipUplinkTxDynamic is only supported in one mode, then skipUplinkTxDynamic will be signalled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 5: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and logicalChannelSR-DelayTimer is supported in both modes, then support of logicalChannelSR-DelayTimer will be signalled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and logicalChannelSR-DelayTimer is only supported in one mode, then support of logicalChannelSR-DelayTimer will be signalled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 6: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and longDRX_Cycle is supported in both modes, then support of longDRX_Cycle will be signalled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and longDRX_Cycle is only supported in one mode, then support of longDRX_Cycle will be signalled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 7: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and shortDRX_Cycle is supported in both modes, then support of shortDRX_Cycle will be signalled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and shortDRX_Cycle is only supported in one mode, then support of shortDRX_Cycle will be signalled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 8: If the UE supports FR2 and pc_pdsch_256QAM_FR2 is supported, then support of pdsch_256QAM_FR2 will be signalled in at least one entry of supportedBandListNR as appropriate. Note 9: If pc_pusch_256QAM_FR1 or pc_pusch_256QAM_FR2 is supported, then support of pusch-256QAM will be signalled in at least one entry of supportedBandListNR as appropriate. Note 10: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and ss-SINR-Meas is supported in both frequency ranges, then support of ss-SINR-Meas will be signaled in measAndMobParameters/measAndMobParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and ss-SINR-Meas is only supported in one frequency range, then support of ss-SINR-Meas will be signaled in one of fr1-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff as appropriate. Note 11: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and csi-RSRP-AndRSRQ-MeasWithoutSSB is supported in both frequency ranges, then support of csi-RSRP-AndRSRQ-MeasWithoutSSB will be signaled in measAndMobParameters/measAndMobParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and csi-RSRP-AndRSRQ-MeasWithoutSSB is only supported in one frequency range, then support of csi-RSRP-AndRSRQ-MeasWithoutSSB will be signaled in one of fr1-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff as appropriate. Note 12: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and multipleConfiguredGrants is supported in both modes, then support of multipleConfiguredGrants will be signaled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and multipleConfiguredGrants is only supported in one mode, then support of multipleConfiguredGrants will be signaled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 13: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and sftd-MeasNR-Neigh is supported in both modes, then support of sftd-MeasNR-Neigh will be signaled in measAndMobParameters/measAndMobParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and sftd-MeasNR-Neigh is only supported in one mode, then support of sftd-MeasNR-Neigh will be signaled in one of fdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff as appropriate. Note 14: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and handoverInterF is supported in both frequency ranges, then support of handoverInterF will be signaled in measAndMobParameters/measAndMobParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and handoverInterF is only supported in one frequency range, then support of handoverInterF will be signaled in one of fr1-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff as appropriate. Note 15: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and handoverInterF is supported in both modes, then support of handoverInterF will be signaled in measAndMobParameters/measAndMobParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and handoverInterF is only supported in one mode, then support of handoverInterF will be signaled in one of fdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff as appropriate. Note 16: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and voiceOverNR is supported in both frequency ranges, then support of voiceOverNR will be signaled in ims-Parameters/ims-ParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and voiceOverNR is only supported in one frequency range, then support of voiceOverNR will be signaled in one of fr1-Add-UE-NR-Capabilities-v1540/ims-ParametersFRX-Diff or fr2-Add-UE-NR-Capabilities-v1540/ims-ParametersFRX-Diff as appropriate. Note 17: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and dl-SchedulingOffset-PDSCH-TypeA is supported in both modes, then support of dl-SchedulingOffset-PDSCH-TypeA will be signaled in phy-Parameters/phy-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and dl-SchedulingOffset-PDSCH-TypeA is only supported in one mode, then support of dl-SchedulingOffset-PDSCH-TypeA will be signaled in one of fdd-Add-UE-NR-Capabilities/ phy-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/phy-ParametersXDD-Diff as appropriate. Note 18: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and dl-SchedulingOffset-PDSCH-TypeA is supported in both frequency ranges, then support of dl-SchedulingOffset-PDSCH-TypeA will be signaled in phy-Parameters/phy-ParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and dl-SchedulingOffset-PDSCH-TypeA is only supported in one frequency range, then support of dl-SchedulingOffset-PDSCH-TypeA will be signaled in one of fr1-Add-UE-NR-Capabilities/phy-ParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/phy-ParametersFRX-Diff as appropriate. Note 19: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and drx-Adaptation-r16 is supported in both frequency ranges, then support of drx-Adaptation-r16 will be signaled in UE-NR-Capability-v1610/mac-Parameters-v1610. If the UE supports both frequency ranges (FR1 + FR2) and drx-Adaptation-r16 is only supported in one frequency range, then support of drx-Adaptation-r16 will be signaled in one of UE-NR-Capability-v1610/fr1-Add-UE-NR-Capabilities-v1610 or UE-NR-Capability-v1610/fr2-Add-UE-NR-Capabilities-v1610 as appropriate. Note 20: If UE supports FR1 and pc_bwp_DiffNumerology_FR1_FDD or pc_bwp_DiffNumerology_FR1_TDD or supports FR2 and pc_bwp_DiffNumerology_FR2, then support of bwp-DiffNumerology will be signaled in at least one entry of supportedBandListNR as appropriate. Note 21: If UE supports FR1 and pc_bwp_SameNumerology_upto2_FR1_FDD or pc_bwp_SameNumerology_upto2_FR1_TDD or pc_bwp_SameNumerology_upto4_FR1_FDD or pc_bwp_SameNumerology_upto4_FR1_TDD or supports FR2 and pc_bwp_SameNumerology_upto2_FR2 or pc_bwp_SameNumerology_upto4_FR2 then support of bwp_SameNumerology will be signaled in at least one entry of supportedBandListNR as appropriate. Note 22: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and idleInactiveNR-MeasReport-r16 is supported in both frequency ranges, then support of idleInactiveNR-MeasReport-r16 will be signaled in measAndMobParameters/measAndMobParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and idleInactiveNR-MeasReport-r16 is only supported in one frequency range, then support of idleInactiveNR-MeasReport-r16 will be signaled in one of fr1-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff as appropriate. Note 23: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and directMCG-SCellActivation-r16 is supported in both frequency ranges, then support of directMCG-SCellActivation-r16 will be signaled in UE-NR-Capability-v1610/mac-Parameters-v1610. If the UE supports both frequency ranges (FR1 + FR2) and directMCG-SCellActivation-r16 is only supported in one frequency range, then support of directMCG-SCellActivation-r16 will be signaled in one of UE-NR-Capability-v1610/fr1-Add-UE-NR-Capabilities-v1610 or UE-NR-Capability-v1610/fr2-Add-UE-NR-Capabilities-v1610 as appropriate. Note 24: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and directMCG-SCellActivationResume-r16 is supported in both frequency ranges, then support of directMCG-SCellActivationResume-r16 will be signaled in UE-NR-Capability-v1610/mac-Parameters-v1610. If the UE supports both frequency ranges (FR1 + FR2) and directMCG-SCellActivationResume-r16 is only supported in one frequency range, then support of directMCG-SCellActivationResume-r16 will be signaled in one of UE-NR-Capability-v1610/fr1-Add-UE-NR-Capabilities-v1610 or UE-NR-Capability-v1610/fr2-Add-UE-NR-Capabilities-v1610 as appropriate. Note 25: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and directSCG-SCellActivationResume-r16 is supported in both frequency ranges, then support of directSCG-SCellActivationResume-r16 will be signaled in UE-NR-Capability-v1610/mac-Parameters-v1610. If the UE supports both frequency ranges (FR1 + FR2) and directSCG-SCellActivationResume-r16 is only supported in one frequency range, then support of directSCG-SCellActivationResume-r16 will be signaled in one of UE-NR-Capability-v1610/fr1-Add-UE-NR-Capabilities-v1610 or UE-NR-Capability-v1610/fr2-Add-UE-NR-Capabilities-v1610 as appropriate. Note 26: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and twoPUCCH-AnyOthersInSlot is supported in both frequency ranges, then support of twoPUCCH-AnyOthersInSlot will be signaled in phy-Parameters/phy-ParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and twoPUCCH-AnyOthersInSlot is only supported in one frequency range, then support of twoPUCCH-AnyOthersInSlot will be signaled in one of fr1-Add-UE-NR-Capabilities/phy-ParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/phy-ParametersFRX-Diff as appropriate. |  |  |  |

Table 8.1.5.1.1.3.3-4: UECapabilityEnquiry-v1560-IEs (Table 8.1.5.1.1.3.3-1)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| UECapabilityEnquiry-v1560-IEs ::= SEQUENCE { |  |  |  |
| capabilityRequestFilterCommon | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rrc-SegAllowed-r16 | enabled |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-5: ULDedicatedMessageSegment (step 2a1, Table 8.1.5.1.1.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-32C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ULDedicatedMessageSegment-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ulDedicatedMessageSegment-r16 SEQUENCE { |  |  |  |
| segmentNumber-r16 | 0 for first segment, and incremented by 1 for each subsequent segment |  |  |
| rrc-MessageSegmentContainer-r16 | Not Checked | OCTET STRING including segmented UECapabilityInformation message |  |
| rrc-MessageSegmentType-r16 | notLastSegment for all segments except the last segment. lastSegment for the last segment |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-6: FeatureSetDownlink-v1610 (Table 8.1.5.1.1.3.3-3)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| FeatureSetDownlink-v1610 ::= SEQUENCE { |  |  |  |
| cbgPDSCH-ProcessingType1-DifferentTB-PerSlot-r16 | Not checked |  |  |
| cbgPDSCH-ProcessingType2-DifferentTB-PerSlot-r16 | Not checked |  |  |
| intraFreqDAPS-r16 | Checked |  | pc_intraFreqDAPS_r16 |
| intraBandFreqSeparationDL-v1620 | Not checked |  |  |
| intraBandFreqSeparationDL-Only-r16 | Not checked |  |  |
| pdcch-Monitoring-r16 | Not checked |  |  |
| pdcch-MonitoringMixed-r16 | Not checked |  |  |
| crossCarrierSchedulingProcessing-DiffSCS-r16 | Not checked |  |  |
| singleDCI-SDM-scheme-r16 | Not checked |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-7: FeatureSetUplink-v1610 (Table 8.1.5.1.1.3.3-3)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| FeatureSetUplink-v1610 ::= SEQUENCE { |  |  |  |
| pusch-RepetitionTypeB-r16 | Checked |  | pc_pusch_RepetitionTypeB_r16 |
| ul-CancellationSelfCarrier-r16 | Not checked |  |  |
| ul-CancellationCrossCarrier-r16 | Not checked |  |  |
| ul-FullPwrMode2-MaxSRS-ResInSet-r16 | Not checked |  |  |
| cbgPUSCH-ProcessingType1-DifferentTB-PerSlot-r16 | Not checked |  |  |
| cbgPUSCH-ProcessingType2-DifferentTB-PerSlot-r16 | Not checked |  |  |
| supportedSRS-PosResources-r16 | Not checked |  |  |
| intraFreqDAPS-UL-r16 | Not checked |  |  |
| intraBandFreqSeparationUL-v1620 | Not checked |  |  |
| multiPUCCH-r16 | Not checked |  |  |
| twoPUCCH-Type1-r16 | Not checked |  |  |
| twoPUCCH-Type2-r16 | Not checked |  |  |
| twoPUCCH-Type3-r16 | Not checked |  |  |
| twoPUCCH-Type4-r16 | Not checked |  |  |
| mux-SR-HARQ-ACK-r16 | Not checked |  |  |
| dummy1 | Not checked |  |  |
| dummy2 | Not checked |  |  |
| twoPUCCH-Type5-r16 | Not checked |  |  |
| twoPUCCH-Type6-r16 | Not checked |  |  |
| twoPUCCH-Type7-r16 | Not checked |  |  |
| twoPUCCH-Type8-r16 | Not checked |  |  |
| twoPUCCH-Type9-r16 | Not checked |  |  |
| twoPUCCH-Type10-r16 | Not checked |  |  |
| twoPUCCH-Type11-r16 | Not checked |  |  |
| ul-IntraUE-Mux-r16 | Not checked |  |  |
| ul-FullPwrMode-r16 | Not checked |  |  |
| crossCarrierSchedulingProcessing-DiffSCS-r16 | Not checked |  |  |
| ul-FullPwrMode1-r16 | Not checked |  |  |
| ul-FullPwrMode2-SRSConfig-diffNumSRSPorts-r16 | Not checked |  |  |
| ul-FullPwrMode2-TPMIGroup-r16 | Not checked |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-8: CA-ParametersNR-v1610 (Table 8.1.5.1.1.3.3-3)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| CA-ParametersNR-v1610 ::= SEQUENCE { |  |  |  |
| parallelTxMsgA-SRS-PUCCH-PUSCH-r16 | Not checked |  |  |
| msgA-SUL-r16 | Not checked |  |  |
| jointSearchSpaceSwitchAcrossCells-r16 | Not checked |  |  |
| half-DuplexTDD-CA-SameSCS-r16 | Not checked |  |  |
| scellDormancyWithinActiveTime-r16 | Not checked |  |  |
| scellDormancyOutsideActiveTime-r16 | Checked |  | pc_scellDormancyOutsideActiveTime_r16 |
| crossCarrierA-CSI-trigDiffSCS-r16 | Not checked |  |  |
| defaultQCL-CrossCarrierA-CSI-Trig-r16 | Not checked |  |  |
| interCA-NonAlignedFrame-r16 | Not checked |  |  |
| simul-SRS-Trans-BC-r16 | Not checked |  |  |
| interFreqDAPS-r16 | Checked |  | pc_interFreqDAPS_r16 |
| codebookParametersPerBC-r16 | Not checked |  |  |
| blindDetectFactor-r16 | Not checked |  |  |
| pdcch-MonitoringCA-r16 | Not checked |  |  |
| pdcch-BlindDetectionCA-Mixed-r16 | Not checked |  |  |
| pdcch-BlindDetectionMCG-UE-r16 | Not checked |  |  |
| pdcch-BlindDetectionSCG-UE-r16 | Not checked |  |  |
| pdcch-BlindDetectionMCG-UE-Mixed-r16 | Not checked |  |  |
| pdcch-BlindDetectionSCG-UE-Mixed-r16 | Not checked |  |  |
| crossCarrierSchedulingDL-DiffSCS-r16 | Not checked |  |  |
| crossCarrierSchedulingDefaultQCL-r16 | Not checked |  |  |
| crossCarrierSchedulingUL-DiffSCS-r16 | Not checked |  |  |
| simul-SRS-MIMO-Trans-BC-r16 | Not checked |  |  |
| codebookParametersAdditionPerBC-r16 | Not checked |  |  |
| codebookComboParametersAdditionPerBC-r16 | Not checked |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-9: PowSav-Parameters-r16 (Table 8.1.5.1.1.3.3-3)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| PowSav-Parameters-r16 ::= SEQUENCE { |  |  |  |
| powSav-ParametersCommon-r16 SEQUENCE { |  |  |  |
| drx-Preference-r16 | Not checked |  |  |
| maxCC-Preference-r16 | Not checked |  |  |
| releasePreference-r16 | Checked |  | pc_releasePreference_r16 |
| minSchedulingOffsetPreference-r16 | Not checked |  |  |
| } |  |  |  |
| powSav-ParametersFRX-Diff-r16 | Not checked |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-10: UE-BasedPerfMeas-Parameters-r16 (Table 8.1.5.1.1.3.3-3)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| UE-BasedPerfMeas-Parameters-r16 ::= SEQUENCE { |  |  |  |
| barometerMeasReport-r16 | Checked |  | pc_barometer_r16 |
| immMeasBT-r16 | Not checked |  |  |
| immMeasWLAN-r16 | Checked |  | pc_immMeasWLAN_r16 |
| loggedMeasBT-r16 | Not checked |  |  |
| loggedMeasurements-r16 | Checked |  | pc_loggedMeasurements_r16 |
| loggedMeasWLAN-r16 | Checked |  | pc_loggedMeasWLAN_r16 |
| orientationMeasReport-r16 | Checked |  | pc_orientation_r16 |
| speedMeasReport-r16 | Checked |  | pc_speed_r16 |
| gnss-Location-r16 | Checked |  | pc_GNSS_location_r16 |
| ulPDCP-Delay-r16 | Checked |  | pc_PDCP_Delay_r16 |
| sigBasedLogMDT-OverrideProtect-r17 | Not checked |  |  |
| multipleCEF-Report-r17 | Checked |  | pc_multiple_CEF_Report_r17 |
| excessPacketDelay-r17 | Not checked |  |  |
| earlyMeasLog-r17 | Checked |  | pc_earlyMeasLog_r17 |
| } |  |  |  |

Table 8.1.5.1.1.3.3-11: MAC-ParametersFRX-Diff-r16 (Table 8.1.5.1.1.3.3-3)

| MAC-ParametersFRX-Diff-r16 ::= SEQUENCE { |  |  |  |
| --- | --- | --- | --- |
| directMCG-SCellActivation-r16 | Checked |  | pc_directMCG_SCellActivation_r16 |
| directMCG-SCellActivationResume-r16 | Checked |  | pc_directMCG_SCellActivationResume_r16 |
| directSCG-SCellActivation-r16 | Not checked |  |  |
| directSCG-SCellActivationResume-r16 | Checked |  | pc_directSCG_SCellActivationResume_r16 |
| drx-Adaptation-r16 | Checked | MinTimeGap-r16 Acc. to TS 38.306 [23] clause 4.2.6: when this field is reported, either of sharedSpectrumChAccess-r16 or non-SharedSpectrumChAccess-r16 shall be reported, at least. | pc_DRX_Adaptation |
| } |  |  |  |