3GPP TSG-RAN5 Meeting #102	R5-240592

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4256 | rev | - | Current version: | 17.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New testcase 10.5.1.2 UE establishing a PDN connection as a user plane resource of an already established MA PDU session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add testcase for UE establishing a PDN connection as a user plane resource of an already established MA PDU session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding testcase skeleton with test sequence. Test specific messages TBD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE establishing a PDN connection as a user plane resource of an already established MA PDU session is not tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0439 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------------- Skipped chapters ---------------------------------------------------------------

10.5.1.1.3.3	Specific message contents

TBD

------------------------------------------------------- Start of first change -----------------------------------------------------------

##### 10.5.1.2	UE-requested MA PDU session establishment / ATSSS / UE establishing a PDN connection as a user-plane resource of an already established MA PDU session / Success

10.5.1.2.1	Test Purpose (TP)

(1)

with { UE in state 5GMM-REGISTERED, and 5GMM-IDLE mode over non 3GPP access without cellular coverage }

ensure that {

when { UE is configured to request a new MA PDU session }

then { UE include the ATSSS request parameter in the Extended protocol configuration options IE of the PDU SESSION ESTABLISHMENT REQUEST message to indicate its support of establishing a PDN connection as the user plane resource of an MA PDU session }

}

(2)

with { UE is registered over non-3GPP access, and an MA PDU session has been established successfully on this access }

ensure that {

when { UE detects a suitable EPC E-UTRAN cell }

then { UE sends a PDN CONNECTIVITY REQUEST message with the request type set to "handover" and including ATSSS request PCO parameter and the PDU session ID of the already established MA PDU session }

10.5.1.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.193, clause 5.3.2. Unless otherwise stated these are Rel-17 requirements.

[TS 24.193, clause 5.3.2]

In order to establish a PDN connection as a user-plane resource of an already established MA PDU session, the UE shall initiate the UE requested PDN connectivity procedure according to 3GPP TS 24.301 [10].

In the PDN CONNECTIVITY REQUEST message or, when applicable, in the ESM INFORMATION RESPONSE message, of the UE requested PDN connectivity procedure:

a)	the UE shall set the request type to "handover" as specified in 3GPP TS 24.301 [10];

NOTE 1:	According to 3GPP TS 24.301 [10], the PDU session ID of the already established MA PDU session is included in the protocol configuration options IE or the extended protocol configuration options IE of the PDN CONNECTIVITY REQUEST message with the request type "handover".

b)	the UE shall set the PDN Type IE to "IPv4", "IPv6", "IPv4v6", "Ethernet" or "non-IP"; and

NOTE 2:	If the UE does not support Ethernet PDN type or the Ethernet PDN type is not supported in EPC, the UE sets "non-IP" PDN type when the established MA PDU session is Ethernet PDU session type.

c)	in the protocol configuration options or extended protocol configuration options IE, the UE shall include the ATSSS request PCO parameter.

Upon receipt of the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message of a default EPS bearer context activation procedure as a response to the PDN CONNECTIVITY REQUEST message as specified in 3GPP TS 24.301 [10], the UE shall consider that the PDN connection is established as a user-plane resource of the MA PDU session.

Upon receipt of a PDN CONNECTIVITY REJECT message as specified in 3GPP TS 24.301 [10], the UE shall consider that the PDN connection is not established as a user-plane resource of the MA PDU session.

10.5.1.2.3	Test description

10.5.1.2.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27 is configured according to TS 38.508-1 [4], clause 4.4.1.3.1.

-	E-UTRA Cell 1 is configured according to TS 38.508-1 [4], Table 4.4.1.1.1.

-	WLAN Cell 27 is set to “Serving cell” and E-UTRA Cell 1 is set to “Off cell”

UE:

-	None

Preamble:

-	The UE is brought to state 1W-A on WLAN Cell 27 according to TS 38.508-1 [4] clause 4.4A.

10.5.1.2.3.2	Test procedure sequence

Table 10.5.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit a PDU SESSION ESTABLISHMENT REQUEST with request type set to “handover” and including ATSSS request PCO parameter in the Extended protocol configuration options on WLAN Cell 27 | --> | 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | 1 | P |
| 2 | The SS establishes an IPSec child security association according to the IKEv2 specification in RFC 7296 [34] | - | - | - | - |
| 3 | The SS transmits an PDU SESSION ESTABLISHMENT ACCEPT | <-- | 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT ACCEPT | - | - |
| 4 | E-UTRA Cell 1 is set to “Serving cell”, WLAN Cell 27 is set to “non-suitable cell” | - | - | - | - |
| 5-7 | Steps 1 to 3 of generic procedure EUTRA RRC_IDLE described in TS 38.508-1 Table 4.5.2.2-1 is performed. | - | - | - | - |
| 8 | Check: Does the UE set request type as “handover” and include ATSSS request PCO parameter in the PDN CONNECTIVITY REQUEST message. The PDN CONNECTIVITY REQUEST message is piggybacked in ATTACH REQUEST | --> | RRC: RRCConnectionSetupComplete NAS: ATTACH REQUEST NAS: PDN CONNECTIVITY REQUEST | 2 | P |
| 9-14 | Steps 5 to 9a2 of generic procedure EUTRA RRC_IDLE described in TS 38.508-1 Table 4.5.2.2-1 is performed | - | - | - | - |
| 15-22 | Steps 10b1 to 10b8 of generic procedure EUTRA RRC_IDLE described in TS 38.508-1 Table 4.5.2.2-1 is performed | - | - | - | - |

10.5.1.2.3.3	Specific message contents

TBD

-------------------------------------------------------- End of first change -----------------------------------------------------------

# 11	Multi-layer and Services

## 11.1	5GS / EPS Fallback

### 11.1.1	MO MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / Success