3GPP TSG-RAN5 Meeting #102	R5-240602

Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2897 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing the Re-Positioning Concept for FR2 RRM TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The re-positioning concept, firmly established for UE RF test cases, has not been applied to RRM test cases even though the relevant MTSUs/TTs were derived with the re-positioning concept applied. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The re-positioning approach for 1 AoA and 2 AoA test cases were introduced and outlined in Annex I (RRM OTA procedures) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases are executed with a larger MU than the test system was assessed for. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | I.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Discussion paper in R5-240601 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

Annex I (normative):
RRM OTA procedures

Annex I defines the RRM OTA procedures for the permitted testing methodologies defined in [32].

<<< Skip Unchanged Sections >>>

# I.5	Re-Positioning Approach for RRM FR2

This clause provides guidance on the applicability of the re-positioning3 approach which was originally introduced to reduce measurement uncertainties due to positioner blocking [18]. Illustrations of the applicability in the following clauses are based on example 1-AoA and 2-AoA capable RRM test systems shown in Figure I.5-1. The probes shown in blue and red are shown for simplicity as typical microwave probes antennas; any of these probes could be implemented as probe&reflector combinations to yield applicable IFF, Enhanced IFF, IFF+DFF test setups, see Clause I.0. The number of locations of the 2 AoA probes are chosen arbitrarily, i.e., systems with fewer and different probe locations are applicable.

Figure I.5-1: Example combined-axes RRM system setup to support 1 AoA (top) and 1 AoA & 2 AoA test cases (bottom)

The re-positioning approach is applicable to RRM test cases if, during the test case, the positioner is not obstructing the beam path(s) between the active probe(s) and the DUT.

## I.5.1	1 AoA Test Cases

The 1 AoA test cases follow Setups 1 and 2, see Clauses A.9.1 and A.9.2, respectively. Here, the re-positioning approach is applicable only when the beam path between the active 1 AoA probe and the DUT is not obstructed by the positioner for all the test iterations (if applicable), i.e., the positioner shall always be in the back hemisphere of the 1 AoA probe for each test iteration.

This requirement is illustrated next using the example 1 AoA RRM systems from Figure I.5-1. For the example systems in Figure I.5-1 with P0 (shown in blue) used as active probe, the positioner shall be placed anywhere within the green shaded area (azimuthal range between 90° and 270° with respect to the 1 AoA P0 probe) in Figure I.5.1-1 for the re-positioning approach to apply and with the optimized QoQZ MU applicable to the assessed MU of the test system. The different test conditions and orientations from Clause N.2 [18] can be used for each test iteration to avoid positioner blocking. If the positioner is placed for any of the test iterations within the red shaded area, the re-positioning approach is no longer adopted and the QoQZ procedure, Annex O of [18], shall utilize the full range of rotation angles to assess the MU of the test system.

Figure I.5.1-1: Ranges of positioner locations for 1 AoA TCs that are applicable with re-positioning concept

## I.5.2	2 AoA Test Cases

The 2 AoA test cases follow Setups 3 and 4, see Clauses A.9.3 and A.9.4, respectively. Here, the re-positioning approach is applicable only when the beam path between each of the 2 active AoA probes and the DUT is not obstructed by the positioner for all the test iterations (if applicable), i.e., the positioner shall always be in the back hemisphere of each of the active AoA probes for each test iteration.

This requirement is illustrated next using the example 2 AoA RRM system from Figure I.5-1. For the 2 AoA TCs using a combination of P0 (probe shown in blue in Figure I.5-1) and either {P30, P60, P90, P120, P150} probe (shown in red in Figure I.5-1), the positioner must be placed anywhere within the overlapping green shaded areas of the respective active probes in Figure I.5.2-1 with the re-positioning approach to apply and with the optimized QoQZ MU applicable to the assessed MU of the test system. While Figure I.5.2-1 shows the acceptable positioner location ranges for the re-positioner concept separately for each active probe, Figure I.5.2-2 shows the combined/aggregate effects for each 2 AoA combination provided that the P0 probe is one of the AoAs: when the positioner is located within the green shaded area in Figure I.5.2-2 for all 2 AoA test iterations, the re-positioner concept applies. The different test conditions and orientations from Clause N.2 [18] can be used for each test iteration to avoid positioner blocking. If the positioner is placed for any of the test iterations within any of the red shaded areas of Figure I.5.2-2, the re-positioning approach is no longer adopted and the QoQZ procedure, Annex O of [18], shall utilize the full range of rotation angles to assess the MU of the test system.

Figure I.5.2-1: Ranges of positioner locations for 2 AoA TCs that are applicable with re-position concept (separately for each active AoA)

Figure I.5.2-2: Ranges of positioner locations for 2 AoA TCs that are applicable with re-position concept (separately for each 2 AoA combination with P0 active)

<<< END OF CHANGES >>>