3GPP TSG RAN WG5 Meeting #102		R5-241051

# Athens, Greece, February 26 – March 1, 2024

Agenda item:	5.4.20

Source:	Apple

Title:	On TRP test result variations adopting reduced grids

WI/SI:	TEIx_Test

Release:	Rel-17

Document for:	Information

# 1	Introduction

In previous RAN4#109 and RAN5#101 meetings, contributions questioning the feasibility of reduce grid adoption in 3GPP were presented on [1-3]. After a series of lengthy on and off-line discussions a compromised text was agreed to be included on TR 38.870 section B.1.12 [4, 5], as follows:

Table B.2.12-1: Grid Options for TRP/TRS with constant-step size grids

| Test Metric | Frequency Range | Quadrature | [°] | [°] | Min. Number of Grid Points (Note 1) | Std. Uncertainty [dB] | ¦Mean Error¦ (Note 3) [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TRP | < 3GHz | sin() | 15 | 15 | 266 | 0 | 0 |
| TRS |  |  | 30 | 30 | 62 | 0.13 | 0 |
| TRP |  | Clenshaw-Curtis | 15 | 15 | 266 | 0 | 0 |
| TRS |  |  | 30 | 30 | 62 | 0.10 | 0 |
| TRP (Note 2) |  |  | 30 | 30 | 62 | 0.10 | 0 |
| TRS (Note 2) |  |  | 30 | 60 | 32 | 0.15 | 0 |
| TRS (Note 2) |  |  | 45 | 45 | 26 | 0.19 | 0 |
| TRP | > 3GHz | sin() | 15 | 15 | 266 | 0 | 0 |
| TRS |  |  | 30 | 30 | 62 | 0.21 | 0 |
| TRP |  | Clenshaw-Curtis | 15 | 15 | 266 | 0 | 0 |
| TRS |  |  | 30 | 30 | 62 | 0.18 | 0 |
| TRP (Note 2) |  |  | 30 | 30 | 62 | 0.18 | 0 |
| TRS (Note 2) |  |  | 30 | 60 | 32 | 0.25 | 0 |
| TRS (Note 2, 4) |  |  | 45 | 45 | 27 | 0.23 | 0.08 |
| TRS (Note 2, 5) |  |  | 45 | 45 | 25 | 0.25 | 0.08 |
| Note 1: The exact number of grid points depends on how the back pole EIRP(=180°)/EIS(=180°) is approximated due to obstruction and/or blocking. Note 2: The overall MU shall not be larger than the maximum MU limits if the coarsest measurement grid is adopted. Note 3: The inclusion of the mean error into the MU template/budget is FFS. Note 4: The EIS value at 180˚ is determined from two 165º measurements. Note 5: The EIS value at 180˚ is averaged from previous cut. |  |  |  |  |  |  |  |

The mean error in Table B.2.12-1 shall be considered a systematic uncertainty that cannot be corrected and thus shall be included in the uncertainty budget table as a systematic uncertainty added to the combined expanded uncertainty.

The legacy grid with ==15° for TRP and ==30° for TRS should be considered the default measurement grid and is recommended for certification testing.

# 2	Results and discussions

The proposed alternative grids were evaluated using measured TRP data which was down sampled from a 15-degree data set (legacy grid).  TRP measurement results were collected on thirty-four devices.

The results are presented based on the delta in (dB) from the legacy TRP measurements grid taken with elevation and azimuth with 15º grid resolution.

The post-processed TRP measurement results are presented on Table 1

The results on Table 1 are presented using the following color code:

- Grid selection producing the lower TRP result, green.

- Grid selection producing the highest TRP result, red.

- Variation lower than 0.25dB, green

- Variation between 0.25 and 0.5 dB, blue

- Variation between 0.5 and 0.75dB, black

- Variation beyond 0.75dB, red

The column “Variation between data points open selection”, means that any arbitrarily grid was selected strictly based on lower and higher TRP results.

The column “Variation from legacy grids to C.C. open selection”, means that this delta is calculated between the 15 degrees Clenshaw Curtis legacy grid and the grid selection that provides the lowest TRP results.

Table 1, TRP deltas vs. grid dependency

# 3	Observations and Proposals

Observation 1: 	Despite the limited number of data-points, the Table 1 indicates that an open selection of reduced grids will cause a large distribution of reduced grids alternatives based on devices radiation pattern characteristics.

Observation 2:	The variation of reduced grid selection seems to be uniformly distributed among devices with different antenna systems radiation pattern directivities.

Observation 3:	The larger variation between reduced grid with lowest TRP vs. reduced grid with highest TRP, can be higher than 1dB. This is a conservative estimate based on the limited number of samples. It’s expected that larger devices sample might produce even larger variations.

Observation 4:	As expected, and observed on [3], the larger variation between reduced grid with lowest TRP vs. reduced grid with highest TRP seems to occur in the data points with antennas which the directivity is higher than 6dBi.

Observation 5:	The observations in this work are TRP measurement based, there’s no evidence that observation/conclusion based on TRP measurement results can be considered for TRS reduced grid proposals.

Observation 6:	As stated on Table B.2.12-1 (Note 3) [5], at this point no mean error related to grid selection is included on the MU budget.

Proposal 1:	RAN 5 to consider a revision of the text on section B.2.12 [5] to:

“The legacy grid with ==15° for TRP and ==30° for TRS should shall be considered the default measurement grid and is recommended for certification testing.

Proposal 2:	RAN 5 to reinforce with RAN4 that data collection for performance requirements pool of data shall be taken only with legacy grids, thus avoiding an open selection of reduced grids.

# 4	References

[1]		R5-237151, On TRP test time reduction adopting reduced grids, Apple, Vodafone, AT&T, ETS Lindgren, Telecom Italia, RAN5#101

[2]		R4-2320626, On usage of coarse sampling grid for TRP/TRS measurement, Telecom Italia, Vodafone, China Telecom, Orange, T-Mobile USA, RAN4#109

[3]	R4-2318429, On TRP test time reduction adopting reduced grids, Apple, Vodafone, AT&T, ETS Lindgren, Telecom Italia, RAN4#109

[4]	R5-237842, TP to TR 38.870 Annex B on forearm phantom and other MU changes, R&S, RAN5#101

[5]	TR 38.870, Enhanced Over-the-Air (OTA) test methods for NR FR1 Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS)