3GPP TSG-RAN5 Meeting #102	R5-241336

# Athens, Greece, February 26th - March 1st 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3089 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test cases 4.5.9.1, 5.5.6.3.1, and 5.5.6.4.1 to Annex F |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RRM test cases 4.5.9.1, 5.5.6.3.1 and 5.5.6.4.1 are missing in TS 38.533 Annex F. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add RRM test cases 4.5.9.1 to Table F.1.1.2-3 and Table F.1.3.2-1. Add RRM test cases 5.5.6.3.1 and 5.5.6.4.1 to Table F.1.1.2-5 and Table F.1.3.2-3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM test cases 4.5.9.1, 5.5.6.3.1 and 5.5.6.4.1 will remain missing in TS 38.533 Annex F and WI cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****Start of changes

Table F.1.1.2-3: Maximum test system uncertainty for RRM requirements for EN-DC FR1 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 4.3.2.2.1 Contention based random access test in FR1 for PSCell in EN-DC | Noc ±1.5 dB Ês2 / Noc ±0.3 dB Uplink absolute power measurement ±1.5 dB Uplink relative power measurement ±0.7 dB ±112Tc Uplink signal transmit timing relative to downlink | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 4.3.2.2.2 Non-contention based random access test in FR1 for PSCell in EN-DC | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 |
| 4.3.2.2.3 EN-DC FR1 2-step contention based random access | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 |
| 4.3.2.2.4 EN-DC FR1 2-step non-contention based random access | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 |
| 4.4.1.1 EN-DC FR1 UE transmit timing accuracy | Noc ±1.5 dB Ês2 / Noc ±0.3 dB ±112Tc Uplink signal transmit timing relative to downlink | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy | Noc ±1.5 dB Ês2 / Noc ±0.3 dB ±88Tc Timing Advance Adjustment accuracy | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 4.5.1.1 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR Meas PRB are the measurement PRB for SSB-based RLM #RBJ to RBJ+19 |
| 4.5.1.2 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.3 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.4 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.5 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.6 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.7 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.8 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.9 EN-DC FR1 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.2.1 EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | ± 0.6dB (AWGN conditions) | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 4.5.2.2 EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | Same as 4.5.2.1 | Same as 4.5.2.1 |
| 4.5.2.3 EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | Noc2 ±1.5 dB Noc3 ±1.5 dB Ês2 / Noc2 ±0.3 dB Ês3 / Noc3 ±0.3 dB | Note: Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN |
| 4.5.2.4 EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | Same as 4.5.2.3 | Same as 4.5.2.3 |
| 4.5.2.5 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | Noc1 ±1.5 dB Noc2 ±1.5 dB Noc3 ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB | Note: Noc1 is the AWGN on E-UTRAN cell 1 frequency Ês1 / Noc1 is the ratio of E-UTRAN cell 1 signal / AWGN Noc2 is the AWGN on NR cell 2 frequency Ês2 / Noc2 is the ratio of NR cell 2 signal / AWGN Noc3 is the AWGN on E-UTRAN cell 3 frequency Ês3 / Noc3 is the ratio of E-UTRAN cell 3 signal / AWGN |
| 4.5.2.6 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | Same as 4.5.2.5 | Same as 4.5.2.5 |
| 4.5.2.10 EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell | Noc ±1.5 dB Ês2 / Noc ±0.3 dB | Note: Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 4.5.3.1 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.5.3.2 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle | Same as 4.5.3.1 | Same as 4.5.3.1 |
| 4.5.3.3 EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX | Same as 4.5.3.1 | Same as 4.5.3.1 |
| 4.5.3.5 EN-DC FR1 direct SCell activation at SCell addition of known SCell | Same as 4.5.3.1 | Same as 4.5.3.1 |
| 4.5.3.6 EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês2 / Noc1 ±0.3 dB Average Ês3 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB | Note: Noc2 is the AWGN on cell 2 frequency Noc3 is the AWGN on cell 3 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 for CSI-RSRP Meas PRB are # RB0 to # RB105. |
| 4.5.3.7 EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle | Same as 4.5.3.6 | Same as 4.5.3.6 |
| 4.5.4.1 EN-DC FR1 UE UL carrier RRC reconfiguration delay | Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB Noc3 ±1.5 dB Ês3 / Noc3 ±0.3 dB | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN |
| 4.5.5.1 EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.5.5.2 EN-DC FR1 SSB-based beam failure detection and link recovery in DRX | Same as 4.5.5.1 | Same as 4.5.5.1 |
| 4.5.5.3 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |
| 4.5.5.4 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX | Same as 4.5.5.3 | Same as 4.5.5.3 |
| 4.5.5.5 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |
| 4.5.5.6 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | Same as 4.5.5.5 | Same as 4.5.5.5 |
| 4.5.5.7 EN-DC FR1 PSCell TRP specific SSB-based beam failure detection and link recovery in non-DRX | Same as 4.5.5.1 | Same as 4.5.5.1 |
| 4.5.5.8 EN-DC FR1 SCell TRP specific CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | Same as 4.5.5.5 | Same as 4.5.5.5 |
| 4.5.6.1.1 EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC | ±0.6 dB | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 4.5.6.1.2 EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC | Same as 4.5.6.1.1 | Same as 4.5.6.1.1 |
| 4.5.6.2.1 EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | ±0.6 dB | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 4.5.7.1 EN-DC FR1 addition and release delay of known PSCell | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB | Noc1 is the AWGN on NR freq1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.5.8.1 EN-DC FR1 interruptions at switching between two uplink carriers | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB | Noc is the AWGN on cell 2 frequency Ês / Noc is the ratio of cell 2 signal / AWGN |
| 4.5.9.1 UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN-DC | ±1.0 dB | The following parameters are proposed to be controlled: - AWGN flatness and signal flatness ±2 dB - Signal-to-noise ratio uncertainty ±0.3 dB Test System uncertainty on PSCell = SQRT (0.32 + (0.25 * 2)2) = 0.58 dB, rounded to 1.0 dB. |
| 4.5.10.1 EN-DC FR1 PSCell activation and deactivation delay | Same as 4.5.7.1 | Same as 4.5.7.1 |
| 4.5.11.1 EN-DC FR1 conditional PSCell addition delay | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB | Noc1 is the AWGN on NR freq1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.6.1.1 EN-DC FR1 event-triggered reporting without gap in non-DRX | Noc ±1.5 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB | Note: Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |
| 4.6.1.2 EN-DC FR1 event-triggered reporting without gap in DRX | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.3 EN-DC FR1 event-triggered reporting with gap in non-DRX | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.4 EN-DC FR1 event-triggered reporting with gap in DRX | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.5 EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.6 EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.7 EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.8 EN-DC FR1 event triggered reporting cell without SSB time index detection in DRX for UE configured with highSpeedMeasCA-Scell-r17 | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 2 signal / AWGN Ês2 / Noc2 is the ratio of cell 3 signal / AWGN Ês3 / Noc2 is the ratio of cell 4 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.6.2.1 EN-DC FR1-FR1 event-triggered reporting in non-DRX | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB | Noc2 is the AWGN on NR freq2 Noc3 is the AWGN on NR freq3 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.6.2.2 EN-DC FR1-FR1 event-triggered reporting in DRX | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 4.6.2.9 EN-DC FR1-FR1 event triggered reporting without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 4.6.4.1 EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.6.4.2 EN-DC FR1 SSB-based L1-RSRP measurement in DRX | Same as 4.6.4.1 | Same as 4.6.4.1 |
| 4.6.4.3 EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |
| 4.6.4.4 EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX | Same as 4.6.4.3 | Same as 4.6.4.3 |
| 4.6.4.5 EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 4.6.4.1 | Same as 4.6.4.1 |
| 4.6.5.1 EN-DC FR1 SRS-RSRP measurement with non-DRX | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB | Average Noc is the AWGN on virtual UE frequency (NR) on NR freq1 Ês / Noc is the ratio of virtual UE signal / AWGN over the SRS resource RBs |
| 4.6.5.2 EN-DC FR1 CLI-RSSI measurement with non-DRX | Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc2 is the AWGN on cell 2 (NR) frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 4.6.7.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on Cell 2 frequency Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |
| 4.6.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on Cell 2 frequency Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |
| 4.6.7.3 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on Cell 2 frequency Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |
| 4.7.1.1.1 EN-DC FR1 SS-RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Average Ês3 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês2 / Noc ±0.3 dB Meas PRB Ês3 / Noc ±0.3 dB | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 2 SNR Ês3 / Noc is the cell 3 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.7.1.1.2 EN-DC FR1 SS-RSRP relative measurement accuracy | Same as 4.7.1.1.1 | Same as 4.7.1.1.1 |
| 4.7.1.2.1 EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês2 / Noc1 ±0.3 dB Average Ês3 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc1 ±0.3 dB Meas PRB Ês3 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês2 / Noc1 is the cell 2 SNR Ês3 / Noc2 is the cell 3 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.7.1.2.2 EN-DC FR1-FR1 SS-RSRP relative measurement accuracy | Same as 4.7.1.2.1 | Same as 4.7.1.2.1 |
| 4.7.2.1 EN-DC FR1 SS-RSRQ measurement accuracy | Same as 4.7.1.1.1 | Same as 4.7.1.1.1 |
| 4.7.2.2.1 EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy | Same as 4.7.1.2.1 | Same as 4.7.1.2.1 |
| 4.7.2.2.2 EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy | Same as 4.7.1.2.1 | Same as 4.7.1.2.1 |
| 4.7.3.1 EN-DC FR1 SS-SINR measurement accuracy | Same as 4.7.1.1.1 | Same as 4.7.1.1.1 |
| 4.7.3.2.1 EN-DC FR1-FR1 SS-SINR absolute measurement accuracy | Same as 4.7.1.2.1 | Same as 4.7.1.2.1 |
| 4.7.3.2.2 EN-DC FR1-FR1 SS-SINR relative measurement accuracy | Same as 4.7.1.2.1 | Same as 4.7.1.2.1 |
| 4.7.4.1.1 EN-DC FR1 SSB based L1-RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 2 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.7.4.1.2 EN-DC FR1 SSB based L1-RSRP relative measurement accuracy | Same as 4.7.4.1.1 | Same as 4.7.4.1.1 |
| 4.7.4.2.1 EN-DC FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Same as 4.7.4.1.1 | Same as 4.7.4.1.1 |
| 4.7.4.2.2 EN-DC FR1 CSI-RS based L1-RSRP relative measurement accuracy | Same as 4.7.4.1.1 | Same as 4.7.4.1.1 |
| 4.7.5.1 EN-DC FR1 SFTD measurement accuracy | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.8 dB | Noc1 is the AWGN on NR freq1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 Noc2 is the AWGN on E-UTRA freq2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RB22 to RB27 |
| 4.7.6.1 EN-DC SRS-RSRP measurement accuracy with FR1 serving cell | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB | Average Noc is the AWGN on virtual UE frequency (NR) on NR freq1 Ês / Noc is the ratio of virtual UE signal / AWGN over the SRS resource RBs |
| 4.7.7.1.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB | Noc is the AWGN on NR freq1 Ês2 / Noc is the SNR for the CSI-RS#0 & CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |
| 4.7.7.1.2 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy | Same as 4.7.7.1.1 | Same as 4.7.7.1.1 |
| 4.7.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB | Noc is the AWGN on NR freq1 Ês2 / Noc is the SNR for the SSB#0 & SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |
| 4.7.7.3.1 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Same as 4.7.7.1.1 | Same as 4.7.7.1.1 |
| 4.7.7.3.2 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy | Same as 4.7.7.1.1 | Same as 4.7.7.1.1 |

****End of changes

****Start of changes

Table F.1.1.2-5: Maximum test system uncertainty for RRM requirements for EN-DC FR2 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 5.3.2.2.1 EN-DC FR2 contention based random access | Average Ês ±6.0 dB Meas PRB Ês ±6.0 dB Uplink absolute power measurement ±6.0 dB Uplink relative power measurement ±FFS dB ±[48]Tc Uplink signal transmit timing relative to downlink | Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.3.2.2.2 EN-DC FR2 non-contention based random access | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 |
| 5.4.1.1 EN-DC FR2 UE transmit timing accuracy | Noc ±5.65 dB, f<=40.8 GHz Ês2 / Noc ±0.3 dB ±48Tc Uplink signal transmit timing relative to downlink ±40Tc Relative during UE timing adjustment | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.4.3.1 EN-DC FR2 UE timing advance adjustment accuracy | Noc ±5.65 dB, f<=40.8 GHz Ês1 / Noc ±0.3 dB ±40Tc timing advance adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.5.1.1 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | Average Noc ±5.65 dB, f<=40.8 GHz Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Fading profile ±0.5 dB for 1Tx | Ês1 / Noc is the SNR of RLM-RS1 Ês2 / Noc is the SNR of RLM-RS2 |
| 5.5.1.2 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 5.5.1.3 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 5.5.1.4 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 5.5.1.5 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Average Noc ±5.65 dB, f<=40.8 GHz Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Fading profile ±0.5 dB for 1Tx | Ês1 / Noc is the SNR of RLM-RS1 Ês2 / Noc is the SNR of RLM-RS2 |
| 5.5.1.6 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 5.5.1.5 | Same as 5.5.1.5 |
| 5.5.1.7 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Same as 5.5.1.5 | Same as 5.5.1.5 |
| 5.5.1.8 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Same as 5.5.1.5 | Same as 5.5.1.5 |
| 5.5.1.10 EN-DC FR2 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 5.5.2.1 EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | Noc ±5.65 dB, f<=40.8 GHz Ês2 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 5.5.2.2 EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | Same as 5.5.2.1 | Same as 5.5.2.1 |
| 5.5.2.3 EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | Noc ±5.65 dB, f<=40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 5.5.2.4 EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | Same as 5.5.2.4 | Same as 5.5.2.4 |
| 5.5.3.1 EN-DC FR2 SCell activation and deactivation intra-band in non-DRX | Average Noc2 ±5.65 dB Average Noc3 ±5.65 dB Average Ês2 / Noc ±0.3 dB Average Ês3 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |
| 5.5.3.7 EN-DC FR2 direct SCell activation at SCell addition of known SCell | Same as 5.5.3.1 | Same as 5.5.3.1 |
| 5.5.3.8 EN-DC FR2 fast SCell Activation of SCell in FR2 intra-band | Average Noc2 ±5.65 dB Average Noc3 ±5.65 dB Average Ês2 / Noc ±0.3 dB Average Ês3 / Noc ±0.3 dB Meas PRB Average Noc2 ±5.65 dB Meas PRB Average Noc3 ±5.65 dB Meas PRB Average Ês2 / Noc ±0.3 dB Meas PRB Average Ês3 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 for CSI-RSRP Meas PRB are # RB0 to # RB274. |
| 5.5.5.1 EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX | Noc averaged over BWConfig, ±5.65 dB, f<=40.8 GHz Noc averaged over measured PRBs, ±5.65 dB, f<=40.8 GHz Ês / Noc averaged over BWConfig: ±0.3 dB Ês / Noc averaged over measured PRBs: ±0.3 dB Fading profile ±0.7 dB, 2Tx | Noc averaged over BWConfig is the AWGN absolute power averaged over BWConfig Noc averaged over measured PRBs is the AWGN absolute power averaged over measured PRBs Ês / Noc averaged over BWConfig is the Ratio of cell signal / AWGN averaged over BWConfig Ês / Noc averaged over measured PRBs is the Ratio of cell signal / AWGN averaged over measured PRBs |
| 5.5.5.2 EN-DC FR2 SSB-based beam failure detection and link recovery in DRX | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 5.5.5.3 EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Noc ±5.65 dB, f<=40.8 GHz Ês2 / Noc ±0.3 dB Fading profile ±0.7 dB for 2Tx | Note: Noc is the AWGN on cell 2 frequency Ês2 / Noc is the SNR for the CSI-RS |
| 5.5.5.4 EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX | Same as 5.5.5.3 | Same as 5.5.5.3 |
| 5.5.5.5 EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 5.5.5.6 EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX | Noc ±5.65 dB, f<=40.8 GHz Ês3 / Noc ±0.3 dB Fading profile ±0.7 dB for 2Tx | Note: Noc is the AWGN on cell 3 frequency Ês3 / Noc is the SNR for the CSI-RS |
| 5.5.5.7 EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | Same as 5.5.5.6 | Same as 5.5.5.6 |
| 5.5.5.8 EN-DC FR2 CSI-RS-based PSCell TRP specific Beam Failure Detection and Link Recovery in DRX mode | Same as 5.5.5.3 | Same as 5.5.5.3 |
| 5.5.5.9 EN-DC FR2 SSB-based beam failure detection and link recovery in DRX mode for UE fulfilling relaxed measurement criterion | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 5.5.6.2.1 EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | ±1.0 dB | Overall system uncertainty for fading conditions comprises two quantities: 1. gNB emulator Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (gNB emulator Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) gNB emulator Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±3.6 dB |
| 5.5.6.3.1 E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC | ±1.0 dB | The following parameters are proposed to be controlled: - AWGN flatness and signal flatness ±3.6 dB - Signal-to-noise ratio uncertainty ±0.3 dB Test System uncertainty on PSCell = SQRT (0.32 + (0.25 * 3.6)2) = 0.95 dB, rounded to 1.0 dB. |
| 5.5.6.4.1 E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 Scell inside active time | Same as 5.5.6.3.1 | Same as 5.5.6.3.1 |
| 5.5.9.1.1 EN-DC PSCell FR2 uplink spatial relation switch for a known spatial relation | ±1.0 dB | The following parameters are proposed to be controlled: - AWGN flatness and signal flatness ±3.6 dB - Signal-to-noise ratio uncertainty ±0.3 dB Test System uncertainty on PSCell = SQRT (0.32 + (0.25 * 3.6)2) = 0.95 dB, rounded to 1.0 dB. |
| 5.5.9.2.1 EN-DC PSCell FR2 uplink spatial relation switch associated with a known DL-RS | Same as 5.5.9.1.1 | Same as 5.5.9.1.1 |
| 5.5.12.1 EN-DC FR2 PSCell activation and deactivation delay | Average Ês2 ±5.65 dB, f <= 40.8 GHz | Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.5.13.1 EN-DC FR2 Addition and Release Delay of NR PSCell | Ês averaged over BW ±5.65 dB, f <= 40.8 GHz Ês averaged over measured PRB ±5.65 dB, f <= 40.8 GHz | Ês averaged over BW is the absolute power of cell 2 signal averaged over BW Ês averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB |
| 5.6.1.1 EN-DC FR2 event-triggered reporting without gap in non-DRX | Ês2 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês2 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz Ês3 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês3 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz | Ês2 averaged over BW is the absolute power of cell 2 signal averaged over BW Ês2 averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB Ês3 averaged over BW is the absolute power of cell 3 signal averaged over BW Ês3 averaged over measured PRB is the absolute power of cell 3 signal averaged over measured PRB |
| 5.6.1.2 EN-DC FR2 event-triggered reporting without gap in DRX | Noc ±5.65 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |
| 5.6.1.3 EN-DC FR2 event-triggered reporting without gap in non-DRX | Same as 5.6.1.1 | Same as 5.6.1.1 |
| 5.6.1.4 EN-DC FR2 event-triggered reporting with gap in DRX | Same as 5.6.1.2 | Same as 5.6.1.2 |
| 5.6.2.1 EN-DC FR2-FR2 event-triggered reporting in non-DRX | Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB Meas PRB Ês2 ±5.65 dB | Ês1 is NR cell 2 signal Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.2.2 EN-DC FR2-FR2 event-triggered reporting in DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc ±0.3 dB Ês2 / Noc over meas PRBs ±0.3 dB | Ês1 is NR cell 2 signal Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.2.3 EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.1 | Same as 5.6.2.1 |
| 5.6.2.4 EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.2 | Same as 5.6.2.2 |
| 5.6.2.5 EN-DC FR1-FR2 event-triggered reporting in non-DRX | Average Ês2 ±5.65, f <= 40.8 GHz | Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.2.6 EN-DC FR1-FR2 event-triggered reporting in DRX | Noc ±5.65 dB, f <= 40.8 GHz Ês2 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 5.6.2.7 EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.5 | Same as 5.6.2.5 |
| 5.6.2.8 EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.6 | Same as 5.6.2.6 |
| 5.6.3.1 EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.3.2 EN-DC FR2 SSB-based L1-RSRP measurement in DRX | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 5.6.3.3 EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RS-RSRP #RBJ to RBJ+65 |
| 5.6.3.4 EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX | Same as 5.6.3.3 | Same as 5.6.3.3 |
| 5.6.4.1 EN-DC FR2 SRS-RSRP measurement in non-DRX | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell2 signal/AWGN |
| 5.6.6.1 EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RS-SINR #RBJ to RBJ+275 |
| 5.6.6.2 EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-SINR#RBJ to RBJ+19 |
| 5.6.6.3 EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | Same as 5.6.6.1 | Same as 5.6.6.1 |
| 5.7.1.1 EN-DC FR2 SS-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Ês2 / Noc is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 2 signal Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.1.2 EN-DC FR2-FR2 SS-RSRP measurement accuracy | Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over measPRBs ±5.65 dB, f <= 40.8 GHz Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over measPRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on the NR freq 1 Noc2 is the AWGN on the NR freq 2 Ês1 / Noc1 is the ratio of cell 2 signal / AWGN Ês2 / Noc2 is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.1.3 EN-DC FR1-FR2 SS-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês2 / Noc is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.2.1 EN-DC FR2 SS-RSRQ measurement accuracy | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Ês2 / Noc is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.2.2 EN-DC FR2-FR2 SS-RSRQ measurement accuracy | Same as 5.7.1.2 | Same as 5.7.1.2 |
| 5.7.3.1 EN-DC FR2 SS-SINR measurement accuracy | Same as 5.7.2.1 | Same as 5.7.2.1 |
| 5.7.3.2 EN-DC FR2-FR2 SS-SINR measurement accuracy | Same as 5.7.1.2 | Same as 5.7.1.2 |
| 5.7.4.1 EN-DC FR2 SSB based L1-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.4.2 EN-DC FR2 CSI-RS based L1-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for CSI-RS-RSRP Test 2 Ês1 is NR cell 2 signal Meas PRB is the measurement PRB for CSI-RS-RSRP |
| 5.7.5.1 EN-DC FR2 SRS-RSRP measurement accuracy | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell2 signal/AWGN |
| 5.7.6.1 EN-DC FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for CSI-RS-RSRP |
| 5.7.6.2 EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SSB or CSI-RS |
| 5.7.6.3 EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for CSI-RS-RSRP |

****End of changes

****Start of changes

Table F.1.3.2-1: Derivation of test requirements for EN-DC FR1 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 4.3.2.2.1 | Absolute uplink power: Normal conditions ±9dB Relative uplink power step: Normal conditions ±2.5dB Uplink timing: 15kHz SCS Te ±12*64*Tc 30kHz SCS Te ±8*64*Tc | 2.1dB 0.7dB 112Tc 112Tc | Absolute uplink power: Normal conditions ±11.1dB Relative uplink power step: Normal conditions ±3.2dB Uplink timing: 15kHz SCS Te ±880*Tc 30kHz SCS Te ±624*Tc |
| 4.3.2.2.2 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 |
| 4.3.2.2.3 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 |
| 4.3.2.2.4 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 |
| 4.4.1.1 EN-DC FR1 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Max step size Tq: 5.5*64*Tc Min adjust rate Tp: 5.5*64*Tc Max adjust rate: 5.5*64*Tc Ês2 / Noc: +3.00dB Noc = -98 dBm/15 kHz (Config 1,2,3) Test 2 (with DRX): ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Ês2 / Noc: +3.00dB | ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.5*64Tc -3.6*64*Tc +1.1*64*Tc +0.3 dB +1.5 dB ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.3dB | Test 1 (10MHz Ch BW): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Max step size Tq: 6.0*64*Tc Min adjust rate: 1.9*64*Tc Max adjust rate: 6.6*64*Tc Ês2 / Noc: +3.30dB Noc = -98 dBm/15 kHz (Config 1,2,3) +1.5 dB Test 2 (with DRX): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Ês2 / Noc: +3.30dB |
| 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE Timing Advance Adjustment Accuracy for 15kHz SCS = ±256 Tc+ TT UE Timing Advance Adjustment Accuracy for 30kHz SCS = ±256 Tc+ TT | 0 0 0 +/- 88 Tc +/- 88 Tc | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE TAAA for 15kHz SCS = ±344 Tc UE TAAA for 30kHz SCS = ±344 Tc |
| 4.5.1.1 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | SNR during T1: 1dB T2: -7dB T3: -15dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |
| 4.5.1.2 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause D.4.1.1 |
| 4.5.1.3 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.4 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Same as 4.5.1.2 | Same as 4.5.1.2 | Same as 4.5.1.2 |
| 4.5.1.5 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.6 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 4.5.1.2 | Same as 4.5.1.2 | Same as 4.5.1.2 |
| 4.5.1.7 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.8 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Same as 4.5.1.2 | Same as 4.5.1.2 | Same as 4.5.1.2 |
| 4.5.1.9 EN-DC FR1 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion | SNR during T1: 1dB T2: 1dB T3: -15dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB | SNR during T1: 1.8dB T2: 1.8dB T3: -15.8dB |
| 4.5.2.1 EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | SNRs as specified | 0.6dB | Formula: SNR + TT |
| 4.5.2.2 EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | Same as 4.5.2.1 | Same as 4.5.2.1 | Same as 4.5.2.1 |
| 4.5.2.3 EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB | During T1: 0dB 0dB 0dB 0dB | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |
| 4.5.2.4 EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | Same as 4.5.2.3 | Same as 4.5.2.3 | Same as 4.5.2.3 |
| 4.5.2.5 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB | During T1: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |
| 4.5.2.6 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | Same as 4.5.2.5 | Same as 4.5.2.5 | Same as 4.5.2.5 |
| 4.5.2.10 EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell | During T1: Noc: -104dBm/15kHz Ês2 / Noc: +17dB | During T1: 0dB 0dB | During T1: Noc: -104dBm/15kHz Ês2 / Noc: +17dB |
| 4.5.3.1 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |
| 4.5.3.2 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle | Same as 4.5.3.1 | Same as 4.5.3.1 | Same as 4.5.3.1 |
| 4.5.3.3 EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX | Same as 4.5.3.1 | Same as 4.5.3.1 | Same as 4.5.3.1 |
| 4.5.3.5 EN-DC FR1 direct SCell activation at SCell addition of known SCell | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |
| 4.5.3.6 EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB In signaling: A3-offset = -15dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB In signaling: 0 dB | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB In signaling: A3-offset = -15dB |
| 4.5.3.7 EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 640 ms SCell measurement cycle | Same as 4.5.3.6 | Same as 4.5.3.6 | Same as 4.5.3.6 |
| 4.5.4.1 EN-DC FR1 UE UL carrier RRC reconfiguration delay | During T1: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T2: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T3: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T2: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T3: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB |
| 4.5.5.1 EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |
| 4.5.5.2 EN-DC FR1 SSB-based beam failure detection and link recovery in DRX | Same as 4.5.5.1 | Same as 4.5.5.1 | Same as 4.5.5.1 |
| 4.5.5.3 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |
| 4.5.5.4 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX | Same as 4.5.5.3 | Same as 4.5.5.3 | Same as 4.5.5.3 |
| 4.5.5.5 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in section D.4.1.1 q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |
| 4.5.5.6 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | Same as 4.5.5.5 | Same as 4.5.5.5 | Same as 4.5.5.5 |
| 4.5.5.7 EN-DC FR1 PSCell TRP specific SSB-based beam failure detection and link recovery in non-DRX | q0,0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q0,1 SSB SNR during: T1-T5: 5dB q1,0 and q1,1, SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1,0 and q1,1, SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1,0 and q1,1, SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB | q0,0, Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q0,1, Offset during: T1-T5: +0.8dB q1,0 and q1,1, SSB during T1: 0dB -0.2dB q1,0 and q1,1, SSB during T2: 0dB -0.2dB q1,0 and q1,1, SSB during T3, T4 and T5: 0dB +0.2dB | q0,0, SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q0,1, SNR during: T1-T5: +5.8dB q1,0 and q1,1, SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1,0 and q1,1, SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1,0 and q1,1, SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |
| 4.5.5.8 EN-DC FR1 SCell TRP specific CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | q0,0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q0,1 CSI-RS SNR during: T1-T5: 5dB q1,0 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1,0 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1,0 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB q1,1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1,1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1,1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q0,1, Offset during: T1-T5: +0.8dB q1,0 SSB during T1: 0dB -0.2dB q1,0 SSB during T2: 0dB -0.2dB q1,0 SSB during T3, T4 and T5: 0dB +0.2dB q1,1 SSB during T1: 0dB -0.2dB q1,1 SSB during T2: 0dB -0.2dB q1,1 SSB during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in section D.4.1.1 q0,1, SNR during: T1-T5: +5.8dB q1,0 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1,0 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1,0 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB q1,1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1,1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1,1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |
| 4.5.6.1.1 EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB | During T1: 0dB 0dB During T2: 0dB 0dB During T3: 0dB 0dB | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |
| 4.5.6.1.2 EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB |
| 4.5.6.2.1 EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB | During T1: 0dB 0dB | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |
| 4.5.7.1 EN-DC FR1 addition and release delay of known PSCell | For Test Configurations 1,2,4,5 Noc: -88dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS For Test Configurations 3,6 Noc: -88dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -93 dBm/SCS | -0.6dB 0dB -3dB -0.6dB 0dB -3dB | Noc: -88.6dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -99 dBm/SCS Noc: -88.6dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS |  |
| 4.5.8.1 EN-DC FR1 interruptions at switching between two uplink carriers | Noc: -104dBm/15kHz Ês / Noc: 17dB | 0dB 0dB | Noc: -104dBm/15kHz Ês / Noc: 17dB |  |
| 4.5.9.1 UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC | Noc: -104 dBm/15kHz Ês / Noc: 17 dB | 0dB 0dB | Noc: -104 dBm/15kHz Ês / Noc: 17 dB |  |
| 4.5.10.1 EN-DC FR1 PSCell activation and deactivation delay | For Test Configurations 1,2,4,5 Noc: -85dBm/15kHz Ês / Noc: 0.0dB For Test Configurations 3,6 Noc: -82dBm/15kHz Ês / Noc: 0.0dB | -3.6dB 0dB -3.6dB 0dB | Noc: -88.6dBm/15kHz Ês / Noc: 0.0dB Noc: -85.6dBm/15kHz Ês / Noc: 0.0dB |  |
| 4.5.11.1 EN-DC FR1 conditional PSCell addition delay | For Test Configurations 1,2,4,5 Noc: -85 dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS For Test Configurations 3,6 Noc: -85 dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -93 dBm/SCS | 0dB 0dB 0dB -3.6dB 0dB -3dB | Noc: -85 dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS Noc: -88.6dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS |  |
| 4.6.1.1 EN-DC FR1 event-triggered reporting without gap in non-DRX | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |
| 4.6.1.2 EN-DC FR1 event-triggered reporting without gap in DRX | Same as 4.6.1.1 | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.3 EN-DC FR1 event-triggered reporting with gap in non-DRX | Same as 4.6.1.1 | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.4 EN-DC FR1 event-triggered reporting with gap in DRX | Same as 4.6.1.1 | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.5 EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | Same as 4.6.1.1 | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.6 EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | Same as 4.6.1.1 | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.7 EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 4.6.1.1 | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.8 EN-DC FR1 event triggered reporting cell without SSB time index detection in DRX for UE configured with highSpeedMeasCA-Scell-r17 | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Ês2 / Noc: +4.00dB | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |
| 4.6.2.1 EN-DC FR1-FR1 event-triggered reporting in non-DRX | During T1: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +7.00dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +7.00dB |
| 4.6.2.2 EN-DC FR1-FR1 event-triggered reporting in DRX | Same as 4.6.2.1 | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Same as 4.6.2.1 | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Same as 4.6.2.1 | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 4.6.2.9 EN-DC FR1-FR1 event triggered reporting without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 | Same as 4.6.2.1 | Same as 4.6.2.1 | Same as 4.6.2.1 |  |
| 4.6.4.1 EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SSB#0 SS-RSRP values ± 10dB Reported SSB#1 SS-RSRP values ± 10dB Reported Differential SS-RSRP values ± 3dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.5dB Via mapping | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 SSB#1: RSRP_55 to RSRP_75 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 SSB#1: RSRP_58 to RSRP_78 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 |
| 4.6.4.2 EN-DC FR1 SSB-based L1-RSRP measurement in DRX | Same as 4.6.4.1 | Same as 4.6.4.1 | Same as 4.6.4.1 |
| 4.6.4.3 EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-RSRP values ± 10dB Reported Differential CSI-RSRP values ± 3dB | 0dB 0dB 0.5dB Via mapping | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 CSI-RS#1: RSRP_55 to RSRP_75 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 CSI-RS#1: RSRP_58 to RSRP_78 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 |
| 4.6.4.4 EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX | Same as 4.6.4.3 | Same as 4.6.4.3 | Same as 4.6.4.3 |
| 4.6.4.5 EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 4.6.4.1 | Same as 4.6.4.1 | Same as 4.6.4.1 |
| 4.6.5.1 EN-DC FR1 SRS-RSRP measurement with non-DRX | Test Configuration 1: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +4dB Test Configuration 2: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +4dB | Test Config 1: T1: 0dB 0dB T2: 0dB +5.25dB Test Config 2: T1: 0dB 0dB T2: 0dB +4.75dB | Test Configuration 1: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +9.25dB Test Configuration 2: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +8.75dB |  |
| 4.6.5.2 EN-DC FR1 CLI-RSSI measurement with non-DRX | During T1: Noc on CLI RSSI: -116 dBm/15kHz Io on CLI RSSI : -97.43 dBm/ 1.08 MHz During T2: Noc on CLI RSSI: -108dBm/15kHz Io on CLI RSSI : -89.43 dBm/ 1.08 Mhz | During T1: 0dB 0dB During T2: 0dB 0.5dB | During T1: Noc: -116 dBm/15kHz Io on CLI RSSI : -97.43 dBm/ 1.08 MHz During T2: Noc: -116 dBm/15kHz Io on CLI RSSI : -88.93 dBm/ 1.08 MHz |  |
| 4.6.7.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-SINR values ± 5.5dB Reported Differential CSI-SINR values ± 4.5dB | 0dB 0dB 0dB Via mapping | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 CSI-RS#1: SINR_41 to SINR_64 CSI-RS#0: DIFFSINR_0 to DIFFSINR_7 |  |
| 4.6.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SS-SINR values ±4.5dB Reported Differential SS-SINR values ±3.5dB | 0dB 0dB 0dB Via mapping | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 SSB#1: SINR_43 to SINR_62 SSB#0: DIFFSINR_0 to DIFFSINR_6 |  |
| 4.6.7.3 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-SINR values ±4dB Reported Differential CSI-SINR values ±3dB | 0dB 0.5dB 0dB Via mapping | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.50dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 CSI-RS#1: SINR_44 to SINR_61 CSI-RS#0: DIFFSINR_0 to DIFFSINR_5 |  |
| 4.7.1.1.1 EN-DC FR1 SS-RSRP absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |
|  | Test 1: Noc: -106dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported RSRP values: ±6dB | Test 1: -1.5dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping | Test 1: Noc: -107.5dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_44 to RSRP_56 Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_60 to RSRP_79 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -0.8dB RSRP_34 to RSRP_46 RSRP_34 to RSRP_46 RSRP_35 to RSRP_47 RSRP_35 to RSRP_47 RSRP_36 to RSRP_48 RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 depending on operating band |
|  | TEST CONFIGURATION 3, 6 |  |  |
|  | Test 1: Noc: -110dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -91dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported RSRP values: ±6dB | Test 1: -0.8dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping | Test 1: Noc: -110.8dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_41 to RSRP_53 Test 2: Noc: -91dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_57 to RSRP_76 Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -0.8dB RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 RSRP_38 to RSRP_50 RSRP_38 to RSRP_50 RSRP_39 to RSRP_51 RSRP_40 to RSRP_52 RSRP_40 to RSRP_52 depending on operating band |
| The derivation of the RSRP values takes into account the uncertainty in Cell 3 SS-RSRP from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. In all cases the RSRP values are 4.5dB wider at each end for extreme conditions. |  |  |  |
| 4.7.1.1.2 EN-DC FR1 SS-RSRP relative measurement accuracy | Test 1: Noc: Test configuration 1, 2, 4, 5:-106dBm/15kHz Test configuration 3, 6:-110dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported relative SS-RSRP values: ±3dB Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported relative SS-RSRP values: ±3dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported relative SS-RSRP values: ±3dB | Test 1: 0 dB 0 dB 0 dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping | Test 1: Noc: -106 dBm/15kHz Noc: -110 dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: 0dB RSRP_x-7 to RSRP_x+1 |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 2 and Cell 3 RSRP from Noc, Ês2 / Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |
| 4.7.1.2.1 EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported RSRP values: ±4.5dB | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_62 to RSRP_81 Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_32 to RSRP_45 RSRP_33 to RSRP_45 RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 RSRP_34 to RSRP_47 RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 depending on operating band |
|  | TEST CONFIGURATION 3, 6 |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported RSRP values: ±4.5dB | Test 1: -1.35dB -1.35dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -96dBm/15kHz Noc2: -96dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 RSRP_37 to RSRP_50 RSRP_38 to RSRP_51 RSRP_39 to RSRP_51 depending on operating band |
| The derivation of the RSRP values takes into account the uncertainty in Cell 3 SS-RSRP from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 3.0dB wider at each end for Test 1, and 4.5 dB wider at each for Test 2. |  |  |  |
| 4.7.1.2.2 EN-DC FR1-FR1 SS-RSRP relative measurement accuracy | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported relative RSRP values: ±4.5dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported relative RSRP values: ±4.5dB | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_x-7 to RSRP_x+7 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_x-31 to RSRP_x-18 |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 2 and Cell 3 RSRP from Noc, Ês2 / Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 1.5dB wider at each end. |  |  |  |
| 4.7.2.1 EN-DC FR1 SS-RSRQ measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |
|  | Test 1: Noc: -85dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -101dBm/15kHz Ês2 / Noc: -2.9dB Ês3 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB Reported RSRQ values: ±3.5dB | Test 1: -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB Via mapping | Test 1: Noc: -86.5dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: Noc: -101dBm/15kHz Ês2 / Noc: -2.9dB Ês3 / Noc: -2.9dB RSRQ_46 to RSRQ_60 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB RSRQ_44 to RSRQ_59 |
|  | TEST CONFIGURATION 3, 6 |  |  |
|  | Test 1: Noc: -91dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB Reported RSRQ values: ±3.5dB | Test 1: -1.6dB 0dB 0dB Via mapping Test 2: N/A Test 3: 0dB 0dB 0dB Via mapping | Test 1: Noc: -92.6dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB RSRQ_44 to RSRQ_59 |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 3 SS-RSRQ from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the SS-RSRQ values are 1.5dB wider at each end for Test 1, and 0.5 dB wider at each for Tests 2 and 3. |  |  |  |
| 4.7.2.2.1 EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |
|  | TEST CONFIGURATION 3, 6 |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 3 SS-RSRQ from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1.5dB wider at each end. |  |  |  |
| 4.7.2.2.2 EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |
|  | TEST CONFIGURATION 3, 6 |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 and Cell 3 SS-RSRQ from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1dB wider at each end. |  |  |  |
| 4.7.3.1 EN-DC FR1 SS-SINR measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -4dB Ês3 / Noc: -4dB Reported SINR values: ±3.5dB | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -3.5dB Ês3 / Noc: -3.5dB SINR_28 to SINR_45 |
|  | TEST CONFIGURATION 3, 6 |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -4dB Ês3 / Noc: -4dB Reported SINR values: ±3.5dB | Test 1: -0.2dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping | Test 1: Noc: -93.2dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -3.5dB Ês3 / Noc: -3.5dB SINR_28 to SINR_45 |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each end. |  |  |  |
| 4.7.3.2.1 EN-DC FR1-FR1 SS-SINR absolute measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB Reported SINR values: ±3dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -4dB Ês3 / Noc2: -4dB Reported SINR values: ±3.5dB | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB SINR_79 to SINR_94 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3.2dB Ês3 / Noc2: -3.2dB SINR_32 to SINR_49 |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 1dB wider at each for Tests 1 and 2 and 0.5dB wider at each end for Test 3. |  |  |  |
| 4.7.3.2.2 EN-DC FR1-FR1 SS-SINR relative measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported SINR values: ±3.5dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB Reported SINR values: ±3.5dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -4dB Ês3 / Noc2: -4dB Reported SINR values: ±4dB | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB SINR_x-10 to SINR_x+10 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB SINR_x-10 to SINR_x+10 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3.2dB Ês3 / Noc2: -3.2dB SINR_x-11 to SINR_x+11 |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each for Tests 1 and 2 and 0dB wider at each end for Test 3. |  |  |  |
| 4.7.4.1.1 EN-DC FR1 SSB based L1-RSRP absolute measurement accuracy |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB RSRP_62 to RSRP_82 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_31 to RSRP_44 RSRP_31 to RSRP_45 RSRP_32 to RSRP_45 RSRP_32 to RSRP_46 RSRP_33 to RSRP_46 RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 depending on operating band |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB RSRP_63 to RSRP_84 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 RSRP_35 to RSRP_48 RSRP_35 to RSRP_49 RSRP_36 to RSRP_49 RSRP_37 to RSRP_50 RSRP_37 to RSRP_51 depending on operating band |
| 4.7.4.1.2 EN-DC FR1 SSB based L1-RSRP relative measurement accuracy |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported differential L1-RSRP values for normal conditions: -4.6dB to 0dB Reported differential L1-RSRP values for extreme conditions: -5.6dB to 0dB Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported differential L1-RSRP values for normal conditions: -4.6dB to 0dB Reported differential L1-RSRP values for extreme conditions: -5.6dB to 0dB | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB DIFFRSRP_0 to DIFFRSRP_2 DIFFRSRP_0 to DIFFRSRP_2 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB DIFFRSRP_0 to DIFFRSRP_2 DIFFRSRP_0 to DIFFRSRP_2 |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported differential L1-RSRP values for normal conditions: -4.6dB to 0dB Reported differential L1-RSRP values for extreme conditions: -5.6dB to 0dB Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported differential L1-RSRP values for normal conditions: -4.6dB to 0dB Reported differential L1-RSRP values for extreme conditions: -5.6dB to 0dB | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB DIFFRSRP_0 to DIFFRSRP_2 DIFFRSRP_0 to DIFFRSRP_2 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB DIFFRSRP_0 to DIFFRSRP_2 DIFFRSRP_0 to DIFFRSRP_2 |
| 4.7.4.2.1 EN-DC FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Same as 4.7.4.1.1 | Same as 4.7.4.1.1 | Same as 4.7.4.1.1 |
| 4.7.4.2.2 EN-DC FR1 CSI-RS based L1-RSRP relative measurement accuracy | Same as 4.7.4.1.2 | Same as 4.7.4.1.2 | Same as 4.7.4.1.2 |
| 4.7.5.1 EN-DC FR1 SFTD measurement accuracy | Noc1: -104dBm/15kHz Ês1 / Noc1: -3.0dB Noc2: -104dBm/15kHz Ês2 / Noc2: 17.0dB | 0dB 0.3dB 0dB 0dB | Noc1: -104dBm/15kHz Ês1 / Noc1: -2.7dB Noc2: -104dBm/15kHz Ês2 / Noc2: 17.0dB |
| 4.7.6.1 EN-DC FR1 SRS-RSRP measurement accuracy | Test Configuration 1: Test 1: Noc = -106dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±3dB Test 2: Noc = -88dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±6.5dB Test 3: Noc = -114+ΔBG_offset dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±3dB Test Configuration 2: Test 1: Test 1 is not used when configuring SCS 30kHz Test 2: Noc = -91dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±7dB Test 3: Noc = -114+ ΔBG_offset dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±3.5dB | Test Config 1: Test 1: 0dB +0.5dB Via mapping Test 2: 0dB +0.5dB Via mapping Test 3: 0dB +0.75dB for BG_A +0.5dB otherwise Via mapping Test Config 2: Test 1: N/A Test 2: 0dB +0.5dB Via mapping Test 3: 0dB +1.25dB for BG_A +0.5dB otherwise Via mapping | Test Config 1: Test 1: Noc = -106dBm/15kHz Ês / Noc = +1.5dB Reported SRS-RSRP: 30 to 41 Test 2: Noc = -88dBm/15kHz Ês / Noc = +1.5dB Reported SRS-RSRP: 45 to 62 Test 3: Noc = -114+ΔBG_offset dBm/15kHz Ês / Noc = +1.75dB for BG_A Ês / Noc = +1.5dB otherwise Reported SRS-RSRP: BG_A: 23 to 33 BG_C: 23 to 34 BG_D: 24 to 34 BG_E: 24 to 35 Test Config 2: Test 1: N/A Test 2: Noc = -91dBm/15kHz Ês / Noc = +1.5dB Reported SRS-RSRP: 44 to 63 Test 3: Noc = -114+ΔBG_offset dBm/15kHz Ês / Noc = +2.25dB for BG_A Ês / Noc = +1.5dB otherwise Reported SRS-RSRP: BG_A: 26 to 37 BG_C: 26 to 37 BG_D: 26 to 38 BG_E: 27 to 38 |  |
| 4.7.7.1.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported CSI-SINR values: ±5.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported CSI-SINR values: ±5.5dB (±1dB additionally for extreme conditions) | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_28 to L1-SINR_57 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_27 to L1-SINR_58 |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported CSI-SINR values: ±5.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported CSI-SINR values: ±5.5dB (±1dB additionally for extreme conditions) | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_28 to L1-SINR_57 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_27 to L1-SINR_58 |
| 4.7.7.1.2 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy |  |  |  |
| Same as 4.7.7.1.1 | Same as 4.7.7.1.1 | Same as 4.7.7.1.1 | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB L1-SINR_x-9 to L1-SINR_x+9 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB L1-SINR_x-9 to L1-SINR_x+9 |
| Same as 4.7.7.1.1 | Same as 4.7.7.1.1 | Same as 4.7.7.1.1 | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB L1-SINR_x-9 to L1-SINR_x+9 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB L1-SINR_x-9 to L1-SINR_x+9 |
| 4.7.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported SS-SINR values: ±4.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported SS-SINR values: ±4.5dB (±1dB additionally for extreme conditions) | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_30 to L1-SINR_55 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_29 to L1-SINR_56 |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_30 to L1-SINR_55 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_29 to L1-SINR_56 |
| 4.7.7.3.1 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_30 to L1-SINR_55 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_29 to L1-SINR_56 |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_30 to L1-SINR_55 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_29 to L1-SINR_56 |
| 4.7.7.3.2 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy |  |  |  |
| Same as 4.7.7.1.2 | Same as 4.7.7.1.2 | Same as 4.7.7.1.2 | Same as 4.7.7.1.2 |
| Same as 4.7.7.1.2 | Same as 4.7.7.1.2 | Same as 4.7.7.1.2 | Same as 4.7.7.1.2 |

****End of changes

****Start of changes

Table F.1.3.2-3: Derivation of test requirements for EN-DC FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 5.3.2.2.1 EN-DC FR2 contention based random access | Absolute uplink power: Pint ≥ P ≥ Pmin ±14.0dB PUMAX ≥ P > Pint ±12.0dB Relative uplink power step: Pint ≥ P ≥ Pmin ±6.0dB PUMAX ≥ P > Pint ±4.0dB Uplink timing: 120kHz SCS Te ±3.5*64*Tc | Not applicable due to UL calibration FFS dB FFS dB [48]Tc | Absolute uplink power: ±FFSdB Relative uplink power step: ±(6+FFS)dB, either PRACH being compared ≤ (Pmax – 6dB) ±(4+FFS)dB, both PRACHs being compared > (Pmax – 6dB) Uplink timing: 120kHz SCS Te ±224±[48]*Tc |
| 5.3.2.2.2 EN-DC FR2 non-contention based random access | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 |
| 5.4.1.1 EN-DC FR2 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Max step size Tq: 2.5*64*Tc Min adjust rate Tp: 2.5*64*Tc Max adjust rate: 2.5*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB | ±0.75*64*Tc +0.625*64Tc -3.725*64*Tc +1.225*64*Tc 0dB 0dB ±0.75*64*Tc 0dB 0dB | Test 1 (no DRX): Uplink timing: ±3.75*64*Tc Max step size Tq: 3.125*64*Tc Min adjust rate: -1.225*64*Tc Max adjust rate: 3.725*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3.75*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB |
| 5.4.3.1 EN-DC FR2 UE timing advance adjustment accuracy | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE Timing Advance Adjustment Accuracy for 120kHz SCS = ±32 Tc | 0dB 0dB +/- 40 Tc | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE TAAA for 120kHz SCS = ±72 Tc |
| 5.5.1.1 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -15 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB |
| 5.5.1.2 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -4.5 dB During T5: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: +2 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB During T4: -2.7 dB 0 dB 0 dB During T5: -2.7 dB 0 dB 2.1 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -15 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -4.5 dB During T5: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: +4.1 dB |
| 5.5.1.3 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB |
| 5.5.1.4 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: -14 dB | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB During T4: 0 dB -0.4 dB -0.4 dB During T5: 0 dB 1.3 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB During T4: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.9 dB Ês2 / Noc: -15.4 dB During T5: Noc: -104.7 dBm/15kHz Ês1 / Noc: 3.3 dB Ês2 / Noc: -15.4 dB |
| 5.5.1.5 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -15 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB |
| 5.5.1.6 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -4.5 dB During T5: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: +2 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB During T4: -2.7 dB 0 dB 0 dB During T5: -2.7 dB 0 dB 2.1 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -15 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -4.5 dB During T5: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: +4.1 dB |
| 5.5.1.7 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB |
| 5.5.1.8 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -104.7dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -104.7dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: -15 dB | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB During T4: 0 dB -0.4 dB -0.4 dB During T5: 0 dB 1.3 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB During T4: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.9 dB Ês2 / Noc: -15.4 dB During T5: Noc: -104.7 dBm/15kHz Ês1 / Noc: 3.3 dB Ês2 / Noc: -15.4 dB |
| 5.5.1.10 EN-DC FR2 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T3: Noc: -104.7dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: 0 dB 1.25 dB 1.25 dB During T2: 0 dB 1.25 dB 1.25 dB During T3: 0 dB -0.5 dB -0.5 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.25 dB Ês2 / Noc: +3.25 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.25 dB Ês2 / Noc: +3.25 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.5 dB Ês2 / Noc: -15.5 dB |
| 5.5.2.3 EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | During T1: Noc1: -111.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc: +12 dB Ês2 / Noc: +5 dB | During T1: 0 dB 0 dB 0 dB 0 dB | During T1: Noc1: -111.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc: +12 dB Ês2 / Noc: +5 dB |
| 5.5.2.3 EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | During T1: Noc1: -111.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc: +12 dB Ês2 / Noc: +5 dB | During T1: 0 dB 0 dB 0 dB 0 dB | During T1: Noc1: -111.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc: +12 dB Ês2 / Noc: +5 dB |
| 5.5.3.1 EN-DC FR2 SCell activation and deactivation intra-band in non-DRX | During T1: Noc2: -104.7dBm/15kHz Noc3: -104.7dBm/15kHz Ês2 / Noc2: +7dB Ês3 / Noc3: +7dB During T2: Noc2: -104.7dBm/15kHz Noc3: -104.7dBm/15kHz Ês2 / Noc2: +7dB Ês3 / Noc3: +7dB During T3: Noc2: -104.7dBm/15kHz Noc3: -104.7dBm/15kHz Ês2 / Noc2: +7dB Ês3 / Noc3: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc2: -104.7dBm/15kHz Noc3: -104.7dBm/15kHz Ês2 / Noc2: +7dB Ês3 / Noc3: +7dB During T2: Noc2: -104.7dBm/15kHz Noc3: -104.7dBm/15kHz Ês2 / Noc2: +7dB Ês3 / Noc3: +7dB During T3: Noc2: -104.7dBm/15kHz Noc3: -104.7dBm/15kHz Ês2 / Noc2: +7dB Ês3 / Noc3: +7dB |
| 5.5.3.7 EN-DC FR2 direct SCell activation at SCell addition of known SCell | Same as 5.5.3.1 | Same as 5.5.3.1 | Same as 5.5.3.1 |
| 5.5.3.8 EN-DC FR2 fast SCell Activation for SCell in FR2 intra-band | During T1: Noc2: -104.7 dBm/15kHz Noc3: -104.7 dBm/15kHz Ês2 / Noc2: +7 dB Ês3 / Noc3: +7 dB During T2: Noc2: -104.7 dBm/15kHz Noc3: -104.7 dBm/15kHz Ês2/ Noc2: +7 dB Ês3/ Noc3: +7 dB In signalling: A3-offset = -15dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB In signalling: 0dB | During T1: Noc2: -104.7 dBm/15kHz Noc3: -104.7 dBm/15kHz Ês2 / Noc2: +7 dB Ês3 / Noc3: +7 dB During T2: Noc2: -104.7 dBm/15kHz Noc3: -104.7 dBm/15kHz Ês2/ Noc2: +7 dB Ês3/ Noc3: +7 dB In signalling: A3-offset = -15dB |
| 5.5.5.1 EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX | During T1: SNR_SSB q0: 5 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: -3 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20.2 dB In signaling: rsrp-ThresholdSSB: -95 dBm/120kHz for Config 1,2 rsrp-ThresholdSSB: -92 dBm/240kHz for Config 3,4 | +8.70 dB 0 dB +8.70 dB 0 dB 0 dB -0.2 dB -14 dB -14 dB | During T1: SNR_SSB q0: 13.7 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: 5.7 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20 dB In signaling: rsrp-ThresholdSSB: -109 dBm/120kHz for Config 1,2 srp-ThresholdSSB: -106 dBm/240kHz for Config 3,4 |
| 5.5.5.2 EN-DC FR2 SSB-based beam failure detection and link recovery in DRX | Same as 5.5.5.1 | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 5.5.5.3 EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Noc = -104.7 dBm/120 kHz Ês2 / Noc for CSI-RS set q0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês2 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdSSB: -95 dBm/SCS kHz | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês2 / Noc for CSI-RS set q0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês2 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdSSB: -109.0 dBm/SCS kHz |
| 5.5.5.4 EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX | Same as 5.5.5.3 | Same as 5.5.5.3 | Same as 5.5.5.3 |
| 5.5.5.5 EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | Same as 5.5.5.1 | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 5.5.5.6 EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX | Noc = -104.7 dBm/120 kHz Ês3 / Noc for CSI-RS set q0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês3 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdBFR: -95 dBm/SCS kHz | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês3 / Noc for CSI-RS set q0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês3 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdBFR: -109.0 dBm/SCS kHz |
| 5.5.5.7 EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | Same as 5.5.5.6 | Same as 5.5.5.6 | Same as 5.5.5.6 |
| 5.5.5.8 EN-DC FR2 PSCell CSI-RS-based TRP specific Beam Failure Detection and Link Recovery in DRX mode | Noc = -104.7 dBm/120 kHz Ês3 / Noc for CSI-RS set q0,0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês3 / Noc for CSI-RS set q0,1: T1-T5: 5 dB Ês3 / Noc for CSI-RS set q1,0 and q1,1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdBFR: -95 dBm/SCS kHz | 0 dB q0,0: 8.7 dB 8.7 dB 0 dB 0 dB 0 dB q0,1: 8.7 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz q0,0: Ês3 / Noc for CSI-RS set q0,0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês3 / Noc for CSI-RS set q0,1 T1-T5: 13.7 dB Ês3 / Noc for CSI-RS set q1,0 and q1,1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdBFR: -109.0 dBm/SCS kHz |
| 5.5.5.9 EN-DC FR2 SSB-based beam failure detection and link recovery in DRX mode for UE fulfilling relaxed measurement criterion | During T1: SNR_SSB q0: 6 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: 6 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20.2 dB In signaling: rsrp-ThresholdSSB: -95 dBm/120kHz for Config 1,2 rsrp-ThresholdSSB: -92 dBm/240kHz for Config 3,4 | +8.70 dB 0 dB +8.70 dB 0 dB 0 dB -0.2 dB -14 dB -14 dB | During T1: SNR_SSB q0: 14.70 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: 14.70 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20 dB In signaling: rsrp-ThresholdSSB: -109 dBm/120kHz for Config 1,2 srp-ThresholdSSB: -106 dBm/240kHz for Config 3,4 |
| 5.5.6.2.1 EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | During T1: Noc2: -112dBm/15kHz Ês2 / Iot2: +18dB | During T1: 0dB 0dB | During T1: Noc2: -112dBm/15kHz Ês2 / Iot2: +18dB |
| 5.5.6.3.1 E-UTRAN - NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC | Noc: -112 dBm/15kHz Ês / Iot: 18 dB | 0 dB 0 dB | Noc: -112 dBm/15kHz Ês / Iot: 18 dB |
| 5.5.6.4.1 E-UTRAN - NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time | Noc: -112 dBm/15kHz Ês / Iot: 18 dB | 0 dB 0 dB | Noc: -112 dBm/15kHz Ês / Iot: 18 dB |
| 5.5.9.1.1 EN-DC PSCell FR2 uplink spatial relation switch for a known spatial relation | During T1: Ês / Noc: 1 dB During T2: Ês / Noc: 1 dB During T3: Ês / Noc: -infinity dB During T4: Ês / Noc: 1 dB | During T1: 0 dB During T2: 0 dB During T3: 0 dB During T4: 0 dB | During T1: Ês / Noc: 1 dB During T2: Ês / Noc: 1 dB During T3: Ês / Noc: -infinity dB During T4: Ês / Noc: 1 dB |
| 5.5.9.2.1 EN-DC PSCell FR2 uplink spatial relation switch associated with a known DL-RS | During T1: Ês / Noc: 1 dB During T2: Ês / Noc: 1 dB During T3: Ês / Noc: -infinity dB During T4: Ês / Noc: 1 dB | During T1: 0 dB During T2: 0 dB During T3: 0 dB During T4: 0 dB | During T1: Ês / Noc: 1 dB During T2: Ês / Noc: 1 dB During T3: Ês / Noc: -infinity dB During T4: Ês / Noc: 1 dB |
| 5.5.12.1 EN-DC FR2 PSCell activation and deactivation delay | During T1: Ês2: - Infinity During T2-T4: Ês2: -81 dBm/120kHz | During T1: 0dB During T2-T4: 0dB | During T1: Ês2: - Infinity During T2-T4: Ês2: -81 dBm/120kHz |
| 5.5.13.1 EN-DC FR2 Addition and Release Delay of NR PSCell | During T1: Ês: -infinity During T2~T4: Ês: -81dBm/SCS | During T1: 0dB During T2~T4: 0dB | During T1: Ês: -infinity During T2~T4: Ês: -81dBm/SCS |
| 5.6.1.1 EN-DC FR2 event-triggered reporting without gap in non-DRX | During T1: Ês2: -86dBm/120kHz Ês3: -infinity During T2: Ês2: -86dBm/120kHz Ês3: -86dBm/120kHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês2: -86dBm/120kHz Ês3: -infinity During T2: Ês2: -86dBm/120kHz Ês3: -86dBm/120kHz |
| 5.6.1.2 EN-DC FR2 event-triggered reporting without gap in DRX | During T1: Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +4.00dB Signalled A3-offset: -6.0 | During T1: -3.5dB 0dB 0dB During T2: -3.5dB 0dB 0dB Signalled A3-offset: -1.0 | During T1: Noc: -101.5dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Noc: -101.5dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +4.00dB Signalled A3-offset: -7.0 |
| 5.6.1.3 EN-DC FR2 event-triggered reporting with gap in non-DRX | Same as 5.6.1.1 | Same as 5.6.1.1 | Same as 5.6.1.1 |
| 5.6.1.4 EN-DC FR2 event-triggered reporting with gap in DRX | Same as 5.6.1.2 | Same as 5.6.1.2 | Same as 5.6.1.2 |
| 5.6.2.1 EN-DC FR2-FR2 event-triggered reporting in non-DRX | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês1: -87 dBm/120kHz Ês2: -- infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz |
| 5.6.2.2 EN-DC FR2-FR2 event-triggered reporting in DRX | During T1: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: -infinity During T2: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +9.00dB Signalled A3-offset: -6.0 | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB Signalled A3-offset: -6.0 | During T1: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: -infinity During T2: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +9.00dB Signalled A3-offset: -12.0 |
| 5.6.2.3 EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.1 | Same as 5.6.2.1 | Same as 5.6.2.1 |
| 5.6.2.4 EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.2 | Same as 5.6.2.2 | Same as 5.6.2.2 |
| 5.6.2.5 EN-DC FR1-FR2 event-triggered reporting in non-DRX | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz | During T1: 0dB During T2: 0dB | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz |
| 5.6.2.6 EN-DC FR1-FR2 event-triggered reporting in DRX | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB |
| 5.6.2.7 EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.5 | Same as 5.6.2.5 | Same as 5.6.2.5 |
| 5.6.2.8 EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.6 | Same as 5.6.2.6 | Same as 5.6.2.6 |
| 5.6.3.1 EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB |
| 5.6.3.2 EN-DC FR2 SSB-based L1-RSRP measurement in DRX | Same as 5.6.3.1 | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 5.6.3.3 EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-RSRP values ± 29dB Reported Differential CSI-RSRP values ± 7dB | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: RSRP_40 to RSRP_99 CSI-RS#0: DIFFRSRP_1 to DIFFRSRP_7 |
| 5.6.3.4 EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX | Same as 5.6.3.3 | Same as 5.6.3.3 | Same as 5.6.3.3 |
| 5.6.4.1 EN-DC FR2 SRS-RSRP measurement in non-DRX | During T1: Noc: -98 dBm/15kHz Neighbour cell UE Es/Noc: -infinity During T2: Noc: -98 dBm/15kHz Neighbour cell UE Es/Noc: 4dB In signalling: i1-threshold: -103dBm/SCS | During T1: -2.5dB 3dB During T2: -2.5dB 3dB -9dB | During T1: Noc: -100.5 dBm/15kHz Neighbour cell UE Es/Noc: -infinity During T2: Noc: -100.5 dBm/15kHz Neighbour cell UE Es/Noc: 7dB i1-threshold: -112dBm/SCS |
| 5.6.6.1 EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-SINR values ± 5.5dB Reported Differential CSI-SINR values ± 4.5dB | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: SINR_51 to SINR_74 CSI-RS#0: DIFFSINR_4 to DIFFSINR_13 |
| 5.6.6.2 EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported SS-SINR values ± 4.0dB Reported Differential SS-SINR values ± 3.0dB | During T1: 0dB 1.5dB 0dB During T2: 0dB 1.5dB 0dB Via mapping | During T1: Noc: -105dBm/15kHz Ês0 / Noc: +1.50dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: +1.50dB Ês1 / Noc: +9.00dB SSB#1: SINR_54 to SINR_71 SSB#0: DIFFSINR_4 to DIFFSINR_10 |
| 5.6.6.3 EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-SINR values ± 4.5dB Reported Differential CSI-SINR values ± 3.5dB | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: SINR_53 to SINR_72 CSI-RS#0: DIFFSINR_5 to DIFFSINR_12 |
| 5.7.1.1 EN-DC FR2 SS-RSRP measurement accuracy | Test 1: Noc: -91.60dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês2: -110.0dBm/120kHz n257 Ês3: -110.0dBm/120kHz n260 Ês2: -107.4dBm/120kHz n260 Ês3: -107.4dBm/120kHz Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -5.80dB 0dB 0.4dB Via Mapping Test 2: 7.70dB 7.70dB 7.70dB 7.70dB Via Mapping | Test 1: Noc: -97.40dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB Cell 2: RSRP_50 to RSRP_108 Cell 3: RSRP_46 to RSRP_103 Cell 3: RSRP_x-12 to RSRP_X+2 Test 2: n257 Ês2: -102.3dBm/120kHz n257 Ês3: -102.3dBm/120kHz n260 Ês2: -99.7dBm/120kHz n260 Ês3: -99.7dBm/120kHz Band dependent. See test case tables in clause 5.7.1.1.5 |
| 5.7.1.2 EN-DC FR2-FR2 SS-RSRP measurement accuracy | Test 1 Config 1: Noc1: -90.60dBm/15kHz Noc2: -90.60dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 1: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 1 Config 2: Noc1: -93.70dBm/15kHz Noc2: -93.70dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 2: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1 Config 1: -5.70dB -5.70dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 1: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping Test 1 Config 2: -5.60dB -5.60dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 2: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping | Test 1 Config 1: Noc1: -96.30dBm/15kHz Noc2: -96.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 2 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 3: RSRP_52 to RSRP_109 Cell 3: n257: RSRP_x-23 to RSRP_x+26 n260: RSRP_x-23 to RSRP_x+27 Test 2 Config 1: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 2 n257: RSRP_33 to RSRP_101 n260: RSRP_34 to RSRP_104 Cell 3: n257: RSRP_32 to RSRP_87 n260: RSRP_34 to RSRP_90 Cell 3: n257: RSRP_x-37 to RSRP_x+17 n260: RSRP_x-37 to RSRP_x+19 Test 1 Config 2: Noc1: -99.30dBm/15kHz Noc2: -99.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 12 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 3: RSRP_52 to RSRP_109 Cell 3: n257: RSRP_x-23 to RSRP_x+26 n260: RSRP_x-23 to RSRP_x+27 Test 2 Config 2: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 2 n257: RSRP_36 to RSRP_104 n260: RSRP_37 to RSRP_107 Cell 3: n257: RSRP_35 to RSRP_90 n260: RSRP_37 to RSRP_93 Cell 3: n257: RSRP_x-37 to RSRP_x+17 n260: RSRP_x-37 to RSRP_x+19 |
| 5.7.1.3 EN-DC FR1-FR2 SS-RSRP measurement accuracy | Test 1: Noc: -90dBm/15kHz Ês3 / Noc: +5.0dB Reported absolute RSRP values: ±8dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês3: -98.2dBm/120kHz n260 Ês3: -93.9dBm/120kHz Reported absolute RSRP values: ±8dBFor extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -5.50dB 0dB Via Mapping Test 2: 5.50dB 5.50dB Via Mapping | Test 1: Noc: -95.50dBm/15kHz Ês3 / Noc: +5dB Band dependent. See test case tables in clause 5.7.1.3.5 Test 2: n257 Ês3: -92.7dBm/120kHz n260 Ês3: -88.4dBm/120kHz Band dependent. See test case tables in clause 5.7.1.3.5 |
| 5.7.2.1 EN-DC FR2 SS-RSRQ measurement accuracy | Test 1: Noc: -95.0dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Test 2: Noc: -95.0dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: -5.70dB 0dB 0dB Via Mapping Test 2: -1.70dB 0dB 0dB Via Mapping | Test 1: Noc: -100.70dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Cell 3: All bands: RSRQ_41 to RSRQ_73 Test 2: Noc: -96.70dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Cell 3: n257, n258, n261: RSRQ_35 to RSRQ_71 n260: RSRQ_34 to RSRQ_71 |
| 5.7.2.2 EN-DC FR2-FR2 SS-RSRQ measurement accuracy | Test 1: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Reported relative RSRQ values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Test 2: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported relative RSRQ values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally | Test 1: -1.90dB -1.90dB 0dB 0dB Via Mapping Via Mapping Test 2: -1.41dB -1.41dB 0dB 0dB Via Mapping Via Mapping | Test 1: Noc1: -95.93dBm/15kHz Noc2: -95.93dBm /15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Cell 2 and Cell 3: RSRQ_41 to RSRQ_73 Cell 3: RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -95.44dBm/15kHz Noc2: -95.44dBm /15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Cell 2 and Cell 3: RSRQ_37 to RSRQ_74 Cell 3: RSRQ_x-9 to RSRQ_x+9 |
| 5.7.3.1 EN-DC FR2 SS-SINR measurement accuracy | Test 1: Noc: -105.0dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +2.66dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc: -105.0dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB Via Mapping Test 2: 0dB 0.2dB 0.2dB Via Mapping | Test 1: Noc: -105.0dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +2.66dB Cell 3: n257, n258, n261: SINR_22 to RSRQ_58 n260: SINR_21 to RSRQ_58 Test 2: Noc: -105.0dBm/15kHz Ês2 / Noc: -2.8dB Ês3 / Noc: -2.8dB Cell 3: n257, n258, n261: SINR_18 to RSRQ_55 n260: SINR_18 to RSRQ_54 |
| 5.7.3.2 EN-DC FR2-FR2 SS-SINR measurement accuracy | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported absolute SINR values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB 0dB Via Mapping Via Mapping Test 2: -0.1dB -0.1dB 0dB 0dB Via Mapping Via Mapping Test 3: 0dB 0dB 0.9dB 0.9dB Via Mapping Via Mapping | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Cell 2 and Cell 3: n257, n258, n261: SINR_27 to SINR_62 n260: SINR_27 to SINR_61 Cell 3: SINR_x-8 to SINR_x+8 Test 2: Noc1: -105.1dBm/15kHz Noc2: -105.1dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Cell 2 and Cell 3: n257, n258, n261: SINR_48 to SINR_87 n260: SINR_48 to SINR_86 Cell 3: SINR_x-17 to SINR_x+17 Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -2.1dB Ês2 / Noc2: -2.1dB Cell 2 and Cell 3: n257, n258, n261: SINR_23 to SINR_60 n260: SINR_22 to SINR_59 Cell 3: SINR_x-9 to SINR_x+9 |
| 5.7.4.1 EN-DC FR2 SSB based L1-RSRP measurement accuracy | Test 1: Noc: -100dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: -2.0dB Reported absolute RSRP values: ±8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 SSB#0 Ês: -108.1dBm/120kHz n257 SSB#1 Ês: -105.5dBm/120kHz n260 SSB#0 Ês: -108.1dBm/120kHz n260 SSB#1 Ês: -105.5dBm/120kHz Reported absolute RSRP values: -6.5 +8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -4.1dB 0dB 0.4dB Test 2: 5.7dB | Test 1: Noc: -104.1dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: -1.6dB SSB#0: RSRP_42 to RSRP_101 SSB#1: RSRP_31 to RSRP_89 SSB#1-SSB#0: -9 to -2 Test 2: n257 SSB#0 Ês: -102.4dBm/120kHz n257 SSB#1 Ês: -99.8dBm/120kHz n260 SSB#0 Ês: -102.4dBm/120kHz n260 SSB#1 Ês: -99.8dBm/120kHz SSB#0 and SSB#1 n257: RSRP_27 to RSRP_83 n260: RSRP_30 to RSRP_86 SSB#1-SSB#0: -4 to +4 |
| 5.7.4.2 EN-DC FR2 CSI-RS based L1-RSRP measurement accuracy | Test 1: Noc: -100dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -2.0dB Reported absolute RSRP values: ±8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 CSI-RS#0 Ês: -108.1dBm/120kHz n257 CSI-RS#1 Ês: -105.5dBm/120kHz n260 CSI-RS#0 Ês: -108.1dBm/120kHz n260 CSI-RS#1 Ês: -105.5dBm/120kHz Reported absolute RSRP values: -6.5 +8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -4.1dB 0dB 0.2dB Test 2: 5.7dB | Test 1: Noc: -104.1dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -1.8dB CSI-RS#0: RSRP_42 to RSRP_101 CSI-RS#1: RSRP_30 to RSRP_89 CSI-RS#1-CSI-RS#0: -9 to -2 Test 2: n257 CSI-RS#0 Ês: -102.4dBm/120kHz n257 CSI-RS#1 Ês: -99.8dBm/120kHz n260 CSI-RS#0 Ês: -102.4dBm/120kHz n260 CSI-RS#1 Ês: -99.8dBm/120kHz CSI-RS#0 and CSI-RS#1 n257: RSRP_27 to RSRP_83 n260: RSRP_30 to RSRP_86 CSI-RS#1-CSI-RS#0: -4 to +4 |
| 5.7.5.1 EN-DC FR2 SRS-RSRP measurement accuracy | During T1: Noc: -100 dBm/15kHz SRS Es/Noc: 2dB During T2: Noc: -100 dBm/15kHz Reported absolute SRS-RSRP values: -89 ± 11.5dB | During T1: 0dB 0dB During T2: 0dB Via mapping | During T1: Noc: -100 dBm/15kHz SRS Es/Noc: 2dB During T2: Noc: -100 dBm/15kHz Reported SRS-RSRP range n257: SRS_RSRP_50 to SRS_RSRP_85 n260: SRS_RSRP_50 to SRS_RSRP_85 |
| 5.7.6.1 EN-DC FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Test 1: Noc: -100dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -2.0dB Reported absolute SINR values: ±5.5dB Reported relative SINR values: ±4.5dB For extreme conditions allow ±1dB + FFS MU additionally | Test 1: -4.1dB 0dB 0.2dB | Test 1: Noc: -104.1dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -1.8dB CSI-RS#0: SINR_53 to SINR_76 CSI-RS#1-CSI-RS#0: 6 to 15 |
| 5.7.6.2 EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Test 1: Noc: -100dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: 0.0dB CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: 0.0dB Reported absolute SINR values: ±4dB Reported relative SINR values: ±3dB For extreme conditions allow ±1dB + FFS MU additionally | Test 1: -4.1dB 0dB 0.2dB 0dB 0.2dB | Test 1: Noc: -104.1dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: 1.2dB CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: 1.2dB SSB#0: SINR_56 to SINR_73 SSB#1-SSB#0: 5 to 12 |
| 5.7.6.3 EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Test 1: Noc: -100dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -2.0dB Reported absolute SINR values: ±4.5dB Reported relative SINR values: ±3.5dB For extreme conditions allow ±1dB + FFS MU additionally | Test 1: -4.1dB 0dB 0.2dB | Test 1: Noc: -104.1dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -1.8dB CSI-RS#0: SINR_55 to SINR_74 CSI-RS#1-CSI-RS#0: 7 to 15 |

****End of changes