3GPP TSG-RAN5 Meeting #102	R5-241564

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5289 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MAC test cases for NTN UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Rohde & Schwarz, MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MAC test cases needs to be extended to cover execution with NTN device and the CR captures the changes required. NPDCCH period bundle is specified in TS 36.523-3 clause 7A.14.2. It is proposed to use this definition to define the search space for NTN and TN Networks. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated test cases to add a note and use the NPDCCH period bundle for cases where timing is specified to make them applicable for NTN UEs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TN MAC test cases cannot be executed on NTN UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.2, 22.3.1.3, 22.3.1.4, 22.3.1.6, 22.3.1.6a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-241564 is the final version of R5-240980 with below changes: 1. Included term “bundle” at missing places. 2. updated the font at step 10A of TC 22.3.1.4. 3. updated the wording in Step 2 of test case 22.3.1.3. 4. updated Step 5 for TC 22.3.1.6a. |  |  |  |  |  |  |  |  |

### 22.3.1	MAC

#### 22.3.1.1	NB-IoT / RACH Procedure / Preamble Selected by MAC / Temporary C-RNTI

##### 22.3.1.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE has need to send a CCCH UL MAC PDU and UE is in enhanced coverage level 0}

then { UE transmits a random access preamble repeated numRepetitionPerPreambleAttempt from Random Access Preambles group and the NPRACH resource corresponding to enhanced coverage level 0}

}

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE has need to send a CCCH UL MAC PDU and UE is in enhanced coverage level 1}

then { UE transmits a random access preamble repeated numRepetitionPerPreambleAttempt from Random Access Preambles group and the NPRACH resource corresponding to enhanced coverage level 1}

}

(3)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE has need to send a CCCH UL MAC PDU and UE is in enhanced coverage level 2}

then { UE transmits a random access preamble repeated numRepetitionPerPreambleAttempt from Random Access Preambles group and the NPRACH resource corresponding to enhanced coverage level 2}

}

(4)

with { UE in E-UTRA RRC_IDLE state after transmission of a NPRACH preamble as per enhanced coverage level 0 or 1}

ensure that {

when { SS does not answer with a matching Random Access Response but only non matching RAR within ra-ResponseWindowSize and PREAMBLE_TRANSMISSION_COUNTER_CE = maxNumPreambleAttemptCE for the corresponding coverage level + 1}

then { UE transmits a random access preamble repeated numRepetitionPerPreambleAttempt from Random Access Preambles group and the NPRACH resource corresponding to enhanced coverage level 1, 2 respectively}

}

(5)

with { UE in E-UTRA RRC_IDLE state after transmission of a NPRACH preamble }

ensure that {

when { SS does not answer with a matching Random Access Response but only non matching RAR within ra-ResponseWindowSize and PREAMBLE_TRANSMISSION_COUNTER <= preambleTransMax-CE }

then { UE retransmits the NPRACH preamble }

}

(6)

with { UE in E-UTRA RRC_IDLE state and transmitted NPRACH preamble }

ensure that {

when { UE receives during TTI window [RA_WINDOW_BEGIN—RA_WINDOW_END] MAC PDU containing multiple RAR’s but none of the subheaders contains a RAPID corresponding to the UE }

then { UE transmits a random access preamble in the next available Random Access occasion }

}

(7)

with { UE in E-UTRA RRC_IDLE state and transmitted NPRACH preamble }

ensure that {

when { SS transmits during RA Response window MAC PDU containing multiple RAR’s and one of the subheaders contains a RAPID corresponding to the UE }

then { UE transmits MAC PDU containing RRCConnectionRequest-NB and including Data Volume and Power Headroom Report (DPR) MAC control element }

}

(8)

with { UE in E-UTRA RRC_IDLE state and has transmitted Msg3 }

ensure that {

when { SS does not respond before contention resolution timer expiry }

then { UE transmits a random access preamble using a preamble in the same group of random access preambles as used for the previous preamble transmission }

}

(9)

with { UE in E-UTRA RRC_IDLE state and after transmitting a RACH MSG3 containing RRCConnectionRequest-NB message }
ensure that {
  when { SS transmits a valid MAC PDU including 'UE Contention Resolution Identity' MAC control element but with un-matched 'Contention Resolution Identity' } 
    then { UE reinitiates RACH procedure } 
            }

(10)

Void

(11)

with { UE in E-UTRA RRC_IDLE state and after transmitting a RACH MSG3 containing RRCConnectionRequest-NB message } 
ensure that { 
  when { SS transmits a valid MAC PDU containing RRCConnectionSetup-NB, including 'UE Contention Resolution Identity' MAC control element with matched 'Contention Resolution Identity' } 
    then { UE completes RACH procedure } 
            }

(12)

with { UE in E-UTRA RRC_IDLE state and having initiated a random access procedure }

ensure that {

when { The SS transmits a Timing Advance Command in a Random Access Response message }

then {  the UE applies the received Timing Advance value in the next transmitted MAC PDU}

}

##### 22.3.1.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.321, clauses 5.1.1, 5.1.2, 5.1.3, 5.1.4, 5.1.5, 5.4.5a, 6.1.3.10.

[TS 36.321, clause 5.1.1]

The Random Access procedure described in this subclause is initiated by a PDCCH order, by the MAC sublayer itself or by the RRC sublayer. Random Access procedure on an SCell shall only be initiated by a PDCCH order. If a MAC entity receives a PDCCH transmission consistent with a PDCCH order [5] masked with its C-RNTI, and for a specific Serving Cell, the MAC entity shall initiate a Random Access procedure on this Serving Cell. For Random Access on the SpCell a PDCCH order or RRC optionally indicate the ra-PreambleIndex and the ra-PRACH-MaskIndex, except for NB-IoT where the subcarrier index is indicated; and for Random Access on an SCell, the PDCCH order indicates the ra-PreambleIndex with a value different from 000000 and the ra-PRACH-MaskIndex. For the pTAG preamble transmission on PRACH and reception of a PDCCH order are only supported for SpCell. If the UE is an NB-IoT UE and is configured with a non-anchor carrier, perform the Random Access procedure on the anchor carrier.

Before the procedure can be initiated, the following information for related Serving Cell is assumed to be available for UEs other than NB-IoT UEs, BL UEs or UEs in enhanced coverage [8], unless explicitly stated otherwise:

-	the available set of PRACH resources for the transmission of the Random Access Preamble, prach-ConfigIndex.

-	the groups of Random Access Preambles and the set of available Random Access Preambles in each group (SpCell only):

The preambles that are contained in Random Access Preambles group A and Random Access Preambles group B are calculated from the parameters numberOfRA-Preambles and sizeOfRA-PreamblesGroupA:

If sizeOfRA-PreamblesGroupA is equal to numberOfRA-Preambles then there is no Random Access Preambles group B. The preambles in Random Access Preamble group A are the preambles 0 to sizeOfRA-PreamblesGroupA – 1 and, if it exists, the preambles in Random Access Preamble group B are the preambles sizeOfRA-PreamblesGroupA to numberOfRA-Preambles – 1 from the set of 64 preambles as defined in [7].

-	if Random Access Preambles group B exists, the thresholds, messagePowerOffsetGroupB and messageSizeGroupA, the configured UE transmitted power of the Serving Cell performing the Random Access Procedure, PCMAX, c [10], and the offset between the preamble and Msg3, deltaPreambleMsg3, that are required for selecting one of the two groups of Random Access Preambles (SpCell only).

-	the RA response window size ra-ResponseWindowSize.

-	the power-ramping factor powerRampingStep.

-	the maximum number of preamble transmission preambleTransMax.

-	the initial preamble power preambleInitialReceivedTargetPower.

-	the preamble format based offset DELTA_PREAMBLE (see subclause 7.6).

-	the maximum number of Msg3 HARQ transmissions maxHARQ-Msg3Tx (SpCell only).

-	the Contention Resolution Timer mac-ContentionResolutionTimer (SpCell only).

NOTE:	The above parameters may be updated from upper layers before each Random Access procedure is initiated.

The following information for related Serving Cell is assumed to be available before the procedure can be initiated for NB-IoT UEs, BL UEs or UEs in enhanced coverage [8]:

-	…

-	if the UE is a NB-IoT UE:

-	the available set of PRACH resources supported in the Serving Cell, nprach-ParametersList.

-	for random access resource selection and preamble transmission:

-	a PRACH resource is mapped into an enhanced coverage level.

-	each PRACH resource contains a set of nprach-NumSubcarriers subcarriers which can be partitioned into one or two groups for single/multi-tone Msg3 transmission by nprach-SubcarrierMSG3-RangeStart. Each group is referred to as a Random Access Preamble group below in the procedure text.

-	a subcarrier is identified by the subcarrier index in the range:
[nprach-SubcarrierOffset, nprach-SubcarrierOffset + nprach-NumSubcarriers -1]

-	each subcarrier of a Random Access Preamble group corresponds to a Random Access Preamble.

-	when the subcarrier index is explicitly sent from the eNB as part of a PDCCH order ra-PreambleIndex shall be set to the signalled subcarrier index.

-	the mapping of the PRACH resources into enhanced coverage levels is determined according to the following:

-	the number of enhanced coverage levels is equal to one plus the number of RSRP thresholds present in RSRP-ThresholdsPrachInfoList.

-	each enhanced coverage level has one PRACH resource present in nprach-ParametersList.

-	enhanced coverage levels are numbered from 0 and the mapping of PRACH resources to enhanced coverage levels are done in increasing numRepetitionsPerPreambleAttempt order.

-	the criteria to select PRACH resources based on RSRP measurement per enhanced coverage level supported in the Serving Cell rsrp-ThresholdsPrachInfoList.

-	the maximum number of preamble transmission attempts per enhanced coverage level supported in the Serving Cell maxNumPreambleAttemptCE.

-	the number of repetitions required for preamble transmission per attempt for each enhanced coverage level supported in the Serving Cell numRepetitionPerPreambleAttempt.

-	the configured UE transmitted power of the Serving Cell performing the Random Access Procedure, PCMAX, c [10].

-	the RA response window size ra-ResponseWindowSize and the Contention Resolution Timer mac-ContentionResolutionTimer (SpCell only) per enhanced coverage level supported in the Serving Cell.

-	the power-ramping factor powerRampingStep.

-	the maximum number of preamble transmission preambleTransMax-CE.

-	the initial preamble power preambleInitialReceivedTargetPower.

-	the preamble format based offset DELTA_PREAMBLE (see subclause 7.6). For NB-IoT the DELTA_PREAMBLE is set to 0.

The Random Access procedure shall be performed as follows:

-	Flush the Msg3 buffer;

-	set the PREAMBLE_TRANSMISSION_COUNTER to 1;

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	set the PREAMBLE_TRANSMISSION_COUNTER_CE to 1;

-	if the starting enhanced coverage level, or for NB-IoT the initial number of PRACH repetitions, has been indicated in the PDCCH order which initiated the Random Access procedure, or if the starting enhanced coverage level has been provided by upper layers:

-	the MAC entity considers itself to be in that enhanced coverage level regardless of the measured RSRP;

-	else:

-	if the RSRP threshold of enhanced coverage level 3 is configured by upper layers in rsrp-ThresholdsPrachInfoList and the measured RSRP is less than the RSRP threshold of enhanced coverage level 3 and the UE is capable of enhanced coverage level 3 then:

-	the MAC entity considers to be in enhanced coverage level 3;

-	else if the RSRP threshold of enhanced coverage level 2 configured by upper layers in rsrp-ThresholdsPrachInfoList and the measured RSRP is less than the RSRP threshold of enhanced coverage level 2 and the UE is capable of enhanced coverage level 2 then:

-	the MAC entity considers to be in enhanced coverage level 2;

-	else if the measured RSRP is less than the RSRP threshold of enhanced coverage level 1 as configured by upper layers in rsrp-ThresholdsPrachInfoList then:

-	the MAC entity considers to be in enhanced coverage level 1;

-	else:

-	the MAC entity considers to be in enhanced coverage level 0;

-	set the backoff parameter value to 0 ms;

-	for the RN, suspend any RN subframe configuration;

-	proceed to the selection of the Random Access Resource (see subclause 5.1.2).

NOTE:	There is only one Random Access procedure ongoing at any point in time in a MAC entity. If the MAC entity receives a request for a new Random Access procedure while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure.

[TS 36.321, clause 5.1.2]

The Random Access Resource selection procedure shall be performed as follows:

-	…

-	else, for NB-IoT, if ra-PreambleIndex (Random Access Preamble) and PRACH resource have been explicitly signalled:

-	the PRACH resource is that explicitly signalled;

-	if the ra-PreambleIndex signalled is not 000000:

-	the Random Access Preamble is set to nprach-SubcarrierOffset + (ra-PreambleIndex modulo nprach-NumSubcarriers), where nprach-SubcarrierOffset and nprach-NumSubcarriers are parameters in the currently used PRACH resource.

-	else:

-	select the Random Access Preamble group according to the PRACH resource and the support for multi-tone Msg3 transmission.

-	randomly select a Random Access Preamble within the selected group.

-	else the Random Access Preamble shall be selected by the MAC entity as follows:

-	If Msg3 has not yet been transmitted, the MAC entity shall, for NB-IoT UEs, BL UEs or UEs in enhanced coverage:

-	expect for NB-IoT, select the Random Access Preambles group and the PRACH resource corresponding to the selected enhanced coverage level;

-	for NB-IoT, select the PRACH resource corresponding to the selected enhanced coverage level, and select the Random Access Preambles group corresponding to the PRACH resource and the support for multi-tone Msg3 transmission;

-	…

-	except for NB-IoT, set PRACH Mask Index to 0.

-	determine the next available subframe containing PRACH permitted by the restrictions given by the prach-ConfigIndex (except for NB-IoT), the PRACH Mask Index (except for NB-IoT, see subclause 7.3), physical layer timing requirements [2] and in case of NB-IoT, the subframes occupied by PRACH resources related to a higher enhanced coverage level (a MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH subframe);

-	if the transmission mode is TDD and the PRACH Mask Index is equal to zero:

-	…

-	else:

-	determine a PRACH within the determined subframe in accordance with the requirements of the PRACH Mask Index, if any.

-	for NB-IoT UEs, BL UEs or UEs in enhanced coverage, select the ra-ResponseWindowSize and mac-ContentionResolutionTimer corresponding to the selected enhanced coverage level and PRACH.

-	proceed to the transmission of the Random Access Preamble (see subclause 5.1.3).

[TS 36.321, clause 5.1.3]

The random-access procedure shall be performed as follows:

-	set PREAMBLE_RECEIVED_TARGET_POWER to preambleInitialReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_TRANSMISSION_COUNTER – 1) * powerRampingStep;

-	if the UE is a BL UE or a UE in enhanced coverage:

-	the PREAMBLE_RECEIVED_TARGET_POWER is set to:
PREAMBLE_RECEIVED_TARGET_POWER - 10 * log10(numRepetitionPerPreambleAttempt);

-	if NB-IoT:

-	for enhanced coverage level 0, the PREAMBLE_RECEIVED_TARGET_POWER is set to:
 PREAMBLE_RECEIVED_TARGET_POWER - 10 * log10(numRepetitionPerPreambleAttempt)

-	for other enhanced coverage levels, the PREAMBLE_RECEIVED_TARGET_POWER is set corresponding to the max UE output power;

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	instruct the physical layer to transmit a preamble with the number of repetitions required for preamble transmission corresponding to the selected preamble group (i.e., numRepetitionPerPreambleAttempt) using the selected PRACH corresponding to the selected enhanced coverage level, corresponding RA-RNTI, preamble index or for NB-IoT subcarrier index, and PREAMBLE_RECEIVED_TARGET_POWER.

-	else:

-	instruct the physical layer to transmit a preamble using the selected PRACH, corresponding RA-RNTI, preamble index and PREAMBLE_RECEIVED_TARGET_POWER.

[TS 36.321, clause 5.1.4]

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap or a Sidelink Discovery Gap for Transmission or a Sidelink Discovery Gap for Reception, the MAC entity shall monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI defined below, in the RA Response window which starts at the subframe that contains the end of the preamble transmission [7] plus three subframes and has length ra-ResponseWindowSize. If the UE is a BL UE or a UE in enhanced coverage, RA Response window starts at the subframe that contains the end of the last preamble repetition plus three subframes and has length ra-ResponseWindowSize for the corresponding coverage level. If the UE is an NB-IoT UE, in case the number of NPRACH repetitions is greater than or equal to 64, RA Response window starts at the subframe that contains the end of the last preamble repetition plus 41 subframes and has length ra-ResponseWindowSize for the corresponding coverage level, and in case the number of NPRACH repetitions is less than 64, RA Response window starts at the subframe that contains the end of the last preamble repetition plus 4 subframes and has length ra-ResponseWindowSize for the corresponding coverage level.

…

For NB-IoT UEs, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as:

RA-RNTI=1+ floor(SFN_id/4)

where SFN_id is the index of the first radio frame of the specified PRACH.

The MAC entity may stop monitoring for Random Access Response(s) after successful reception of a Random Access Response containing Random Access Preamble identifiers that matches the transmitted Random Access Preamble.

-	If a downlink assignment for this TTI has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded, the MAC entity shall regardless of the possible occurrence of a measurement gap or a Sidelink Discovery Gap for Transmission or a Sidelink Discovery Gap for Reception:

-	if the Random Access Response contains a Backoff Indicator subheader:

-	set the backoff parameter value as indicated by the BI field of the Backoff Indicator subheader and Table 7.2-1, except for NB-IoT where the value from Table 7.2-2 is used.

-	else, set the backoff parameter value to 0 ms.

-	if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble (see subclause 5.1.3), the MAC entity shall:

-	consider this Random Access Response reception successful and apply the following actions for the serving cell where the Random Access Preamble was transmitted:

-	process the received Timing Advance Command (see subclause 5.2);

-	indicate the preambleInitialReceivedTargetPower and the amount of power ramping applied to the latest preamble transmission to lower layers (i.e., (PREAMBLE_TRANSMISSION_COUNTER – 1) * powerRampingStep);

-	process the received UL grant value and indicate it to the lower layers;

-	if ra-PreambleIndex was explicitly signalled and it was not 000000 (i.e., not selected by MAC):

-	consider the Random Access procedure successfully completed.

-	else, if the Random Access Preamble was selected by the MAC entity:

-	set the Temporary C-RNTI to the value received in the Random Access Response message no later than at the time of the first transmission corresponding to the UL grant provided in the Random Access Response message;

-	if this is the first successfully received Random Access Response within this Random Access procedure:

-	if the transmission is not being made for the CCCH logical channel, indicate to the Multiplexing and assembly entity to include a C-RNTI MAC control element in the subsequent uplink transmission;

-	obtain the MAC PDU to transmit from the "Multiplexing and assembly" entity and store it in the Msg3 buffer.

NOTE:	When an uplink transmission is required, e.g., for contention resolution, the eNB should not provide a grant smaller than 56 bits (or 88 bits for NB-IoT) in the Random Access Response.

NOTE:	If within a Random Access procedure, an uplink grant provided in the Random Access Response for the same group of Random Access Preambles has a different size than the first uplink grant allocated during that Random Access procedure, the UE behaviour is not defined.

If no Random Access Response is received within the RA Response window, or if none of all received Random Access Responses contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble, the Random Access Response reception is considered not successful and the MAC entity shall:

-	if the notification of power ramping suspension has not been received from lower layers:

-	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax-CE + 1:

-	if the Random Access Preamble is transmitted on the SpCell:

-	indicate a Random Access problem to upper layers;

-	if NB-IoT:

-	consider the Random Access procedure unsuccessfully completed;

-	else:

-	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

-	if the Random Access Preamble is transmitted on the SpCell:

-	indicate a Random Access problem to upper layers;

-	if the Random Access Preamble is transmitted on an SCell:

-	consider the Random Access procedure unsuccessfully completed.

-	if in this Random Access procedure, the Random Access Preamble was selected by MAC:

-	based on the backoff parameter, select a random backoff time according to a uniform distribution between 0 and the Backoff Parameter Value;

-	delay the subsequent Random Access transmission by the backoff time;

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	increment PREAMBLE_TRANSMISSION_COUNTER_CE by 1;

-	if PREAMBLE_TRANSMISSION_COUNTER_CE = maxNumPreambleAttemptCE for the corresponding enhanced coverage level + 1:

-	reset PREAMBLE_TRANSMISSION_COUNTER_CE;

-	consider to be in the next enhanced coverage level, if it is supported by the Serving Cell and the UE, otherwise stay in the current enhanced coverage level;

-	select the Random Access Preambles group, ra-ResponseWindowSize, mac-ContentionResolutionTimer, and PRACH resource corresponding to the selected enhanced coverage level;

-	if the UE is an NB-IoT UE:

-	if the Random Access Procedure was initiated by a PDCCH order:

-	consider the PRACH resource corresponding to the selected enhanced coverage level as explicitly signalled;

-	proceed to the selection of a Random Access Resource (see subclause 5.1.2).

[TS 36.321, clause 5.1.5]

Contention Resolution is based on either C-RNTI on PDCCH of the SpCell or UE Contention Resolution Identity on DL-SCH. If the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage, the MAC entity shall use the mac-ContentionResolutionTimer for the corresponding enhanced coverage level if it exists.

Once Msg3 is transmitted, the MAC entity shall:

-	start mac-ContentionResolutionTimer and restart mac-ContentionResolutionTimer at each HARQ retransmission;

-	regardless of the possible occurrence of a measurement gap or Sidelink Discovery Gap for Reception, monitor the PDCCH until mac-ContentionResolutionTimer expires or is stopped;

-	if notification of a reception of a PDCCH transmission is received from lower layers, the MAC entity shall:

-	if the C-RNTI MAC control element was included in Msg3:

-	if the Random Access procedure was initiated by the MAC sublayer itself or by the RRC sublayer and the PDCCH transmission is addressed to the C-RNTI and contains an UL grant for a new transmission; or

-	if the Random Access procedure was initiated by a PDCCH order and the PDCCH transmission is addressed to the C-RNTI:

-	consider this Contention Resolution successful;

-	stop mac-ContentionResolutionTimer;

-	discard the Temporary C-RNTI;

-	if the UE is an NB-IoT UE and is configured with a non-anchor carrier:

-	the UL grant or DL assignment contained in the PDCCH transmission on the anchor carrier is valid only for the non-anchor carrier.

-	consider this Random Access procedure successfully completed.

-	else if the CCCH SDU was included in Msg3 and the PDCCH transmission is addressed to its Temporary C-RNTI:

-	if the MAC PDU is successfully decoded:

-	stop mac-ContentionResolutionTimer;

-	if the MAC PDU contains a UE Contention Resolution Identity MAC control element; and

-	if the UE Contention Resolution Identity included in the MAC control element matches the 48 first bits of the CCCH SDU transmitted in Msg3:

-	consider this Contention Resolution successful and finish the disassembly and demultiplexing of the MAC PDU;

-	set the C-RNTI to the value of the Temporary C-RNTI;

-	discard the Temporary C-RNTI;

-	consider this Random Access procedure successfully completed.

-	else

-	discard the Temporary C-RNTI;

-	consider this Contention Resolution not successful and discard the successfully decoded MAC PDU.

-	if mac-ContentionResolutionTimer expires:

-	discard the Temporary C-RNTI;

-	consider the Contention Resolution not successful.

-	if the Contention Resolution is considered not successful the MAC entity shall:

-	flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer;

-	if the notification of power ramping suspension has not been received from lower layers:

-	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax-CE + 1:

-	indicate a Random Access problem to upper layers.

-	if NB-IoT:

-	consider the Random Access procedure unsuccessfully completed;

-	else:

-	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

-	indicate a Random Access problem to upper layers.

-	based on the backoff parameter, select a random backoff time according to a uniform distribution between 0 and the Backoff Parameter Value;

-	delay the subsequent Random Access transmission by the backoff time;

-	proceed to the selection of a Random Access Resource (see subclause 5.1.2).

[TS 36.321, clause 5.4.5a]

The Data Volume and Power Headroom reporting procedure is only applicable for NB-IoT UEs and is used to provide the serving eNB with information about the amount of data available for transmission in the UL buffers associated with the MAC entity, and to provide the serving eNB with information about the difference between the nominal UE maximum transmission power and the estimated transmission power for UL-SCH transmission for the Serving Cell. The reporting is done using the DPR MAC control element, which is sent in Msg3 together with a CCCH SDU.

[TS 36.321, clause 6.1.3.10]

The Data Volume and Power Headroom Report (DPR) MAC control element is identified by the MAC PDU subheader used for the CCCH MAC SDU, as specified in table 6.2.1-2. It does not add any additional subheader and is always placed before the CCCH MAC SDU.

It has a fixed size and consists of a single octet defined as follows (figure 6.1.3.10-1):

-	Data Volume (DV): The Data Volume field identifies the total amount of data available across all logical channels and of data not yet associated with a logical channel after all MAC PDUs for the TTI have been built. The amount of data is indicated in number of bytes. It shall include all data that is available for transmission in the RLC layer, in the PDCP layer, and in the RRC layer; the definition of what data shall be considered as available for transmission is specified in [3], [4] and [8] respectively. The size of the RLC and MAC headers are not considered in the buffer size computation. The length of this field is 4 bits. The values taken by the Data Volume field are shown in Table 6.1.3.10-1;

-	Power Headroom (PH): This field indicates the power headroom level. The length of the field is 2 bits. The reported PH and the corresponding power headroom levels are shown in Table 6.1.3.10-2 below (the corresponding measured values in dB can be found in [9]);

-	R: reserved bit, set to "0".

##### 22.3.1.1.3	Test description

22.3.1.1.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

UE:

None.

Preamble:

-	UE is in state Registered, Idle Mode (state 3-NB) according to [18] in Ncell 1.

22.3.1.1.3.2	Test procedure sequence

Table 22.3.1.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the Ncells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1, T2 and T3" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 22.3.1.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Ncell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | NRS-EPRE | dBm/15kHz | -70 | The power level values are such that enhanced coverage level 0 |
| T1 | NRS-EPRE | dBm/15kHz | -90 | The power level values are such that UE is in enhanced coverage level 1 |
| T2 | NRS-EPRE | dBm/15kHz | -110 | The power level values are such that enhanced coverage level 2 |

Table 22.3.1.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | Exception: Steps 0-17E are repeated once for each time instant specified in table 22.3.1.1.3.2-1 by applying the Ncell 1 power level as per time instance. | - | - | - | - |
| 0 | The SS waits 15s to allow time to the UE to measure Ncell power level. |  |  |  |  |
| 1 | The SS transmits a Paging message including a matched identity. | <-- | MAC PDU (Paging) | - | - |
| - | Exception: Steps 2, 4, 6, 9 & 13 is repeated for numRepetitionsPerPreambleAttempt-r13 times configured for corresponding CE level | - | - | - | - |
| 2 | Check: Does the UE transmit a preamble on NPRACH? NOTE: The RACH preamble is calculated based on same nprach-ParametersList-r13 parameters for corresponding CE level defined in SystemInformationBlockType2-NB | --> | NPRACH Preamble | 1,2,3 | P |
| 3 | The SS transmits Random Access Response with RAPID different to the transmitted Preamble in step 2, including T-CRNTI and not including Back off Indicator sub header. | <-- | Random Access Response | - | - |
| 4 | Check: Does the UE transmit same preamble group as step 2 on NPRACH? | --> | NPRACH Preamble | 1,2,3,5 | P |
| 5 | The SS transmits a MAC PDU addressed to UE RA-RNTI, containing multiple RAR’s but none of the MAC sub headers contains a matching RAPID | <-- | Random Access Response | - | - |
| 6 | Check: Does the UE transmit same preamble group as step 2 on NPRACH? | --> | NPRACH Preamble | 1,2,3,6 | P |
| 7 | The SS transmits a MAC PDU addressed to UE RA-RNTI, containing multiple RAR’s one of the MAC sub headers contains a matching RAPID | <-- | Random Access Response | - | - |
| 8 | The UE transmits an RRCConnectionRequest-NB message including Data Volume and Power Headroom Report (DPR) MAC control element. | --> | MAC PDU (RRCConnectionRequest-NB) | 7 | P |
| 9 | Check: Does the UE transmit same preamble group as step 2 on NPRACH? | --> | NPRACH Preamble | 1,2,3,8 | P |
| 10 | The SS transmits a MAC PDU addressed to UE RA-RNTI, containing multiple RAR’s one of the MAC sub headers contains a matching RAPID | <-- | Random Access Response | - | - |
| 11 | Check: Does the UE transmits an RRCConnectionRequest-NB message including Data Volume and Power Headroom Report (DPR) MAC control element. | --> | MAC PDU (RRCConnectionRequest-NB) | 7 | P |
| 12 | The SS transmits a valid MAC PDU including a ‘UE Contention Resolution Identity’ MAC control element with unmatched ‘Contention Resolution Identity’ and a CCCH message (RRCConnectionSetup-NB). | <-- | MAC PDU (RRCConnectionSetup-NB) | - | - |
| 13 | Check: Does the UE transmit same preamble group as step 2 on NPRACH? | --> | NPRACH Preamble | 9 | P |
| 14 | Void | - | - | - | - |
| - | Exception: Steps 15 & 16 are repeated for numRepetitionsPerPreambleAttempt-r13 times configured for corresponding CE level | - | - | - | - |
| 15 | Check: Does the UE transmit a preamble on NPRACH? NOTE: The RACH preamble is calculated based on same nprach-ParametersList-r13 parameters for corresponding CE level +1 (if CE level is 0,1) or CE level 2 defined in SystemInformationBlockType2-NB | --> | NPRACH Preamble | 4 | P |
| 16 | Check: Does the UE transmit same preamble group as step 15 on NPRACH(PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax-CE (=7)+ 1)? | --> | NPRACH Preamble | 4 | P |
| 17 | The SS transmits a MAC PDU addressed to UE RA-RNTI, containing a matching RAPID. | <-- | Random Access Response |  |  |
| 17A - 17D | Steps 2 to 5 of the 'Generic Test Procedure NB-IoT Control Plane CIoT MT user data transfer non-SMS transport' as described in TS 36.508 [18] clause 8.1.5A.2.3 are performed. | - | - | - | - |
| 17E | SS transmits an RRCConnectionRelease-NB message. | <-- | MAC PDU (RRCConnectionRelease-NB) | - | - |
| 18 | SS sets Ncell 1 power level as per T0 | - | - | - | - |
| 18A | The SS waits 15s to allow time to the UE to measure Ncell power level. | - | - | - | - |
| 19 | The SS transmits a Paging message including a matched identity. | <-- | MAC PDU (Paging) | - | - |
| 20 | Check: Does the UE transmit a preamble on NPRACH? NOTE: The RACH preamble is calculated based on same nprach-ParametersList-r13 parameters for corresponding CE level defined in SystemInformationBlockType2-NB | --> | NPRACH Preamble | 1 | P |
| 21 | SS respond to UE Random Access request by a Random Access Response with TA field within message set to 600 (NOTE 1) | <-- | MAC PDU (Random Access Response (TA=600)) | - | - |
| 22 | Check: Does UE send an RRCConnectionRequest-NB message? NOTE: The UL transmission is using the Timing Advance value sent by the SS in step 21 | --> | MAC PDU (RRCConnectionRequest-NB) | 12 | P |
| 23 | The SS transmits a valid MAC PDU containing a “UE Contention Resolution Identity” MAC control element with matching “Contention Resolution Identity” and a CCCH message (RRCConnectionSetup-NB containing configuration of UE-specific search space). | <-- | MAC PDU (RRCConnectionSetup-NB) | - | - |
| 24 | Void | - | - | - | - |
| 25 | Check: Does UE send an RRCConnectionSetupComplete-NB message? NOTE: The UL transmission is using the Timing Advance value sent by the SS in step 21 | --> | MAC PDU (RRCConnectionSetupComplete-NB) | 11 | P |
| NOTE 1: TA value of 600 has been chosen arbitrarily in the middle of the range 0 to 1282 and corresponds to 0.3125 ms (timing advance in ms = 1000 x NTA x where NTA = TA 16 and seconds according to TS 36.213 and TS 36.211). |  |  |  |  |  |

22.3.1.1.3.3	Specific message contents

Table 22.3.1.1.3.3-1: NPRACH-ConfigSIB-NB-DEFAULT in SystemInformationBlockType2-NB in preamble

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPRACH-ConfigSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| rsrp-ThresholdsPrachInfoList-r13::= SEQUENCE { | 2 entries |  |  |
| RSRP-Range[1] | 61 | -79 dBm |  |
| RSRP-Range[2] | 41 | -99 dBm |  |
| } |  |  |  |
| nprach-ParametersList-r13 ::= SEQUENCE { | 3 entries | 1: CE level 0 2: CE level 1 3: CE level 2 |  |
| { |  |  |  |
| nprach-Periodicity-r13 | ms640 |  |  |
| nprach-StartTime-r13 | ms8 |  |  |
| nprach-SubcarrierOffset-r13 | n12 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | oneThird |  |  |
| maxNumPreambleAttemptCE-r13 | n3 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n1 |  |  |
| npdcch-NumRepetitions-RA-r13 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v4 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| { |  |  |  |
| nprach-Periodicity-r13 | ms640 |  |  |
| nprach-StartTime-r13 | ms32 |  |  |
| nprach-SubcarrierOffset-r13 | n12 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | oneThird |  |  |
| maxNumPreambleAttemptCE-r13 | n3 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n2 |  |  |
| npdcch-NumRepetitions-RA-r13 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v4 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| { |  |  |  |
| nprach-Periodicity-r13 | ms640 |  |  |
| nprach-StartTime-r13 | ms128 |  |  |
| nprach-SubcarrierOffset-r13 | n12 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | oneThird |  |  |
| maxNumPreambleAttemptCE-r13 | n3 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n4 |  |  |
| npdcch-NumRepetitions-RA-r13 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v4 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.1.1.3.3-1A: NPRACH-ConfigSIB-NB-v1530-DEFAULT in SystemInformationBlockType2-NB in preamble

| Derivation Path: 36.508 Table 8.1.6.3-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPRACH-ConfigSIB-NB-v1530-DEFAULT ::= SEQUENCE { |  |  | TDD |
| tdd-Parameters-r15 SEQUENCE { |  |  |  |
| nprach-ParametersListTDD-r15 SEQUENCE (SIZE (1..maxNPRACH-Resources-NB-r13)) OF SEQUENCE { | 3 entries | 1: CE level 0 2: CE level 1 3: CE level 2 |  |
| { |  |  |  |
| nprach-Parameters-r15 SEQUENCE { |  |  |  |
| nprach-Periodicity-r15 | ms640 |  |  |
| nprach-StartTime-r15 | ms10 |  |  |
| nprach-SubcarrierOffset-r15 | n12 |  |  |
| nprach-NumSubcarriers-r15 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r15 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r15 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r15 | v4 |  |  |
| npdcch-Offset-RA-r15 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r15 | n8 |  |  |
| } |  |  |  |
| } |  |  |  |
| { |  |  |  |
| nprach-Parameters-r15 SEQUENCE { |  |  |  |
| nprach-Periodicity-r15 | ms640 |  |  |
| nprach-StartTime-r15 | ms40 |  |  |
| nprach-SubcarrierOffset-r15 | n12 |  |  |
| nprach-NumSubcarriers-r15 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r15 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r15 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r15 | v4 |  |  |
| npdcch-Offset-RA-r15 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r15 | n8 |  |  |
| } |  |  |  |
| } |  |  |  |
| { |  |  |  |
| nprach-Parameters-r15 SEQUENCE { |  |  |  |
| nprach-Periodicity-r15 | ms640 |  |  |
| nprach-StartTime-r15 | ms160 |  |  |
| nprach-SubcarrierOffset-r15 | n12 |  |  |
| nprach-NumSubcarriers-r15 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r15 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r15 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r15 | v4 |  |  |
| npdcch-Offset-RA-r15 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r15 | n8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.1.1.3.3-1B: NPRACH-ConfigSIB-NB-v1550-DEFAULT in SystemInformationBlockType2-NB in preamble

| Derivation Path: 36.508 Table 8.1.6.3-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPRACH-ConfigSIB-NB-v1550-DEFAULT ::= SEQUENCE { |  |  | TDD |
| tdd-Parameters-v1550 SEQUENCE { |  |  |  |
| nprach-ParametersListTDD-v1550 SEQUENCE (SIZE (1..maxNPRACH-Resources-NB-r13)) OF SEQUENCE { | 3 entries | 1: CE level 0 2: CE level 1 3: CE level 2 |  |
| { |  |  |  |
| maxNumPreambleAttemptCE-v1550 | n3 |  |  |
| numRepetitionsPerPreambleAttempt-v1550 | n1 |  |  |
| } |  |  |  |
| { |  |  |  |
| maxNumPreambleAttemptCE-v1550 | n3 |  |  |
| numRepetitionsPerPreambleAttempt-v1550 | n2 |  |  |
| } |  |  |  |
| { |  |  |  |
| maxNumPreambleAttemptCE-v1550 | n3 |  |  |
| numRepetitionsPerPreambleAttempt-v1550 | n4 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.1.1.3.3-2: RACH-ConfigCommon-NB-DEFAULT in SystemInformationBlockType2-NB in preamble

| Derivation Path: 36.508 Table 8.1.6.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| preambleTransMax-CE-r13 | n7 |  |  |
| rach-InfoList-r13 (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF SEQUENCE { | 3 entries | 1: CE level 0 2: CE level 1 3: CE level 2 |  |
| { |  |  |  |
| ra-ResponseWindowSize-r13 | pp10 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| { |  |  |  |
| ra-ResponseWindowSize-r13 | pp10 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| { |  |  |  |
| ra-ResponseWindowSize-r13 | pp10 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.1.1.3.3-3: NPUSCH-ConfigCommon-NB-DEFAULT in SystemInformationBlockType2-NB in preamble

| Derivation Path: 36.508 Table 8.1.6.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPUSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ack-NACK-NumRepetitions-Msg4-r13 (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF SEQUENCE { | 3 entries | 1: CE level 0 2: CE level 1 3: CE level 2 |  |
| ACK-NACK-NumRepetitions-NB-r13 | r8 |  |  |
| ACK-NACK-NumRepetitions-NB-r13 | r8 |  |  |
| ACK-NACK-NumRepetitions-NB-r13 | r8 |  |  |
| } |  |  |  |

Table 22.3.1.1.3.3-4: q-RxLevMin in SystemInformationBlockType1-NB in preamble

| Derivation Path: 36.508 Table 8.1.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-NB ::= SEQUENCE { |  |  |  |
| cellSelectionInfo-r13 SEQUENCE { |  |  |  |
| q-RxLevMin-r13 | -70 (-140 dBm) |  |  |
| } |  |  |  |
| } |  |  |  |

#### 22.3.1.2	NB-IoT / Correct Handling of DL MAC PDU / Assignment / HARQ process / TimeAlignmentTimer expiry

##### 22.3.1.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives downlink assignment on the NPDCCH with a C-RNTI unknown by the UE and data is available in the associated subframe }

then { UE does not send any HARQ feedback on the HARQ process }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives downlink assignment on the NPDCCH for the UE’s C-RNTI and receives a MAC PDU containing an single AMD PDU with no padding in the associated TTI in repetitions as per DL_REPETITION_NUMBER }

then { UE sends a HARQ feedback }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives a MAC PDU containing multiple MAC SDUs each containing an AMD PDU and no padding }

then { UE successfully decodes the MAC PDU and forwards the AMD PDUs to higher layer }

}

(4)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when{ UE is receiving RLC PDUs in MAC PDUs with padding greater than 2 bytes }

then { UE acknowledges reception of the RLC PDUs }

}

(5)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE is receiving RLC PDUs in MAC PDUs with padding equal to or less than 2 bytes }

then { UE acknowledges reception of the RLC PDUs }

}

(6)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { SS is transmitting a MAC control Timing Advance PDU with padding equal to or less than 2 bytes and no Data MAC PDU sub-headers followed by transmitting a RLC PDU }

then { UE acknowledges reception of the RLC PDU using the new Timing Advance }

}

(7)

with { UE in E-UTRA RRC_CONNECTED state and timeAlignmentTimer has expired }

ensure that {

when { SS sends downlink assignment on the NPDCCH with a C-RNTI assigned to UE and data is available in the associated subframe }

then { UE does not send any HARQ feedback on the HARQ process }

}

(8)

with { UE in E-UTRA RRC_CONNECTED state and timeAlignmentTimer has expired }

ensure that {

when { SS sends uplink grant on the NPDCCH with a C-RNTI assigned to UE }

then { UE does not send any MAC PDU }

}

(9)

with { UE in E-UTRA RRC_CONNECTED state and timeAlignmentTimer has expired }

ensure that {

when { SS sends NPDCCH order to the C-RNTI assigned to UE }

then { UE sends a prach preamble given in the NPDCCH Order }

}

##### 22.3.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.321 clauses: 5.3.1, 5.3.2.1, 5.3.2.2., 6.1.2 & 6.2.1

[TS 36.321, clause 5.1.2]

-	else, for NB-IoT, if ra-PreambleIndex (Random Access Preamble) and PRACH resource have been explicitly signalled:

-	the PRACH resource is that explicitly signalled;

-	if the ra-PreambleIndex signalled is not 000000:

-	the Random Access Preamble is set to nprach-SubcarrierOffset + (ra-PreambleIndex modulo nprach-NumSubcarriers), where nprach-SubcarrierOffset and nprach-NumSubcarriers are parameters in the currently used PRACH resource.

[TS 36.321, clause 5.1.4]

For NB-IoT UEs, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as:

RA-RNTI=1+ floor(SFN_id/4)

where SFN_id is the index of the first radio frame of the specified PRACH.

The MAC entity may stop monitoring for Random Access Response(s) after successful reception of a Random Access Response containing Random Access Preamble identifiers that matches the transmitted Random Access Preamble.

-	If a downlink assignment for this TTI has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded, the MAC entity shall regardless of the possible occurrence of a measurement gap or a Sidelink Discovery Gap for Transmission or a Sidelink Discovery Gap for Reception:

-	if the Random Access Response contains a Backoff Indicator subheader:

-	set the backoff parameter value as indicated by the BI field of the Backoff Indicator subheader and Table 7.2-1, except for NB-IoT where the value from Table 7.2-2 is used.

-	else, set the backoff parameter value to 0 ms.

-	if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble (see subclause 5.1.3), the MAC entity shall:

-	consider this Random Access Response reception successful and apply the following actions for the serving cell where the Random Access Preamble was transmitted:

-	process the received Timing Advance Command (see subclause 5.2);

-	indicate the preambleInitialReceivedTargetPower and the amount of power ramping applied to the latest preamble transmission to lower layers (i.e., (PREAMBLE_TRANSMISSION_COUNTER – 1) * powerRampingStep);

-	process the received UL grant value and indicate it to the lower layers;

-	if ra-PreambleIndex was explicitly signalled and it was not 000000 (i.e., not selected by MAC):

-	consider the Random Access procedure successfully completed.

[TS 36.321, clause 5.3.1]

Downlink assignments transmitted on the PDCCH indicate if there is a transmission on a DL-SCH for a particular MAC entity and provide the relevant HARQ information.

When the MAC entity has a C-RNTI, Semi-Persistent Scheduling C-RNTI, or Temporary C-RNTI, the MAC entity shall for each TTI during which it monitors PDCCH and for each Serving Cell:

-	if a downlink assignment for this TTI and this Serving Cell has been received on the PDCCH for the MAC entity’s C-RNTI, or Temporary C-RNTI:

-	if this is the first downlink assignment for this Temporary C-RNTI:

-	consider the NDI to have been toggled.

-	if the downlink assignment is for the MAC entity’s C-RNTI and if the previous downlink assignment indicated to the HARQ entity of the same HARQ process was either a downlink assignment received for the MAC entity’s Semi-Persistent Scheduling C-RNTI or a configured downlink assignment:

-	consider the NDI to have been toggled regardless of the value of the NDI.

-	indicate the presence of a downlink assignment and deliver the associated HARQ information to the HARQ entity for this TTI.

[TS 36.321, clause 5.3.2.1]

There is one HARQ entity at the MAC entity for each Serving Cell which maintains a number of parallel HARQ processes. Each HARQ process is associated with a HARQ process identifier. The HARQ entity directs HARQ information and associated TBs received on the DL-SCH to the corresponding HARQ processes (see subclause 5.3.2.2).

The number of DL HARQ processes per HARQ entity is specified in [2], clause 7.

When the physical layer is configured for downlink spatial multiplexing [2], one or two TBs are expected per TTI and they are associated with the same HARQ process. Otherwise, one TB is expected per TTI.

For NB-IoT UEs or BL UEs or UEs in enhanced coverage, the parameter DL_REPETITION_NUMBER provides the number of transmissions repeated in a bundle. For each bundle, DL_REPETITION_NUMBER is set to a value provided by lower layers. Within a bundle, after the initial (re)transmission, DL_REPETITION_NUMBER-1 HARQ retransmissions follow. The HARQ feedback is transmitted for the bundle and a downlink assignment corresponding to a new transmission or a retransmission of the bundle is received after the last repetition of the bundle. A retransmission of a bundle is also a bundle.

In addition to the broadcast HARQ process, NB-IoT has one DL HARQ process.

The MAC entity shall:

-	If a downlink assignment has been indicated for this TTI:

-	allocate the TB(s) received from the physical layer and the associated HARQ information to the HARQ process indicated by the associated HARQ information.

-	If a downlink assignment has been indicated for the broadcast HARQ process:

-	allocate the received TB to the broadcast HARQ process.

NOTE:	In case of BCCH and BR-BCCH a dedicated broadcast HARQ process is used.

[TS 36.321, clause 5.3.2.2]

For each TTI where a transmission takes place for the HARQ process, one or two (in case of downlink spatial multiplexing) TBs and the associated HARQ information are received from the HARQ entity.

For each received TB and associated HARQ information, the HARQ process shall:

-	if the NDI, when provided, has been toggled compared to the value of the previous received transmission corresponding to this TB; or

-	if the HARQ process is equal to the broadcast process and if this is the first received transmission for the TB according to the system information schedule indicated by RRC; or

-	if this is the very first received transmission for this TB (i.e. there is no previous NDI for this TB):

-	consider this transmission to be a new transmission.

-	else:

-	consider this transmission to be a retransmission.

The MAC entity then shall:

-	if this is a new transmission:

-	attempt to decode the received data.

-	else if this is a retransmission:

-	if the data for this TB has not yet been successfully decoded:

-	combine the received data with the data currently in the soft buffer for this TB and attempt to decode the combined data.

-	if the data which the MAC entity attempted to decode was successfully decoded for this TB; or

-	if the data for this TB was successfully decoded before:

-	if the HARQ process is equal to the broadcast process:

-	deliver the decoded MAC PDU to upper layers.

-	else if this is the first successful decoding of the data for this TB:

-	deliver the decoded MAC PDU to the disassembly and demultiplexing entity.

-	generate a positive acknowledgement (ACK) of the data in this TB.

-	else:

-	replace the data in the soft buffer for this TB with the data which the MAC entity attempted to decode.

-	generate a negative acknowledgement (NACK) of the data in this TB.

-	if the HARQ process is associated with a transmission indicated with a Temporary C-RNTI and the Contention Resolution is not yet successful (see subclause 5.1.5); or

-	if the HARQ process is equal to the broadcast process; or

-	if the timeAlignmentTimer, associated with the TAG containing the serving cell on which the HARQ feedback is to be transmitted, is stopped or expired:

-	do not indicate the generated positive or negative acknowledgement to the physical layer.

-	else:

-	indicate the generated positive or negative acknowledgement for this TB to the physical layer.

The MAC entity shall ignore NDI received in all downlink assignments on PDCCH for its Temporary C-RNTI when determining if NDI on PDCCH for its C-RNTI has been toggled compared to the value in the previous transmission.

NOTE:	When the MAC entity is configured with more than one serving cell, UE behaviours for storing data to the soft buffer is specified in [2].

NOTE:	If the MAC entity receives a retransmission with a TB size different from the last valid TB size signalled for this TB, the UE behaviour is left up to UE implementation.

[TS 36.321, clause 6.1.2]

A MAC PDU consists of a MAC header, zero or more MAC Service Data Units (MAC SDU), zero, or more MAC control elements, and optionally padding; as described in Figure 6.1.2-3.

Both the MAC header and the MAC SDUs are of variable sizes.

A MAC PDU header consists of one or more MAC PDU subheaders; each subheader corresponds to either a MAC SDU, a MAC control element or padding.

A MAC PDU subheader consists of the five or six header fields R/F2/E/LCID/(F)/L but for the last subheader in the MAC PDU and for fixed sized MAC control elements. The last subheader in the MAC PDU and subheaders for fixed sized MAC control elements consist solely of the four header fields R/F2/E/LCID. A MAC PDU subheader corresponding to padding consists of the four header fields R/F2/E/LCID.

Figure 6.1.2-1: R/F2/E/LCID/F/L MAC subheader

Figure 6.1.2-1a: R/F2/E/LCID/L MAC subheader

Figure 6.1.2-2: R/F2/E/LCID MAC subheader

MAC PDU subheaders have the same order as the corresponding MAC SDUs, MAC control elements and padding.

MAC control elements are always placed before any MAC SDU.

Padding occurs at the end of the MAC PDU, except when single-byte or two-byte padding is required. Padding may have any value and the MAC entity shall ignore it. When padding is performed at the end of the MAC PDU, zero or more padding bytes are allowed.

When single-byte or two-byte padding is required, one or two MAC PDU subheaders corresponding to padding are placed at the beginning of the MAC PDU before any other MAC PDU subheader.

A maximum of one MAC PDU can be transmitted per TB per MAC entity. A maximum of one MCH MAC PDU can be transmitted per TTI.

Figure 6.1.2-3: Example of MAC PDU consisting of MAC header, MAC control elements, MAC SDUs and padding

[TS 36.321, clause 6.2.1]

The MAC header is of variable size and consists of the following fields:

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC control element or padding as described in tables 6.2.1-1, 6.2.1-2 and 6.2.1-4 for the DL-SCH, UL-SCH and MCH respectively. There is one LCID field for each MAC SDU, MAC control element or padding included in the MAC PDU. In addition to that, one or two additional LCID fields are included in the MAC PDU, when single-byte or two-byte padding is required but cannot be achieved by padding at the end of the MAC PDU. A UE of Category 0 [12] shall indicate CCCH using LCID "01011", otherwise the UE shall indicate CCCH using LCID "00000". The LCID field size is 5 bits;

-	L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC control element in bytes. There is one L field per MAC PDU subheader except for the last subheader and subheaders corresponding to fixed-sized MAC control elements. The size of the L field is indicated by the F field and F2 field;

-	F: The Format field indicates the size of the Length field as indicated in table 6.2.1-3. There is one F field per MAC PDU subheader except for the last subheader and subheaders corresponding to fixed-sized MAC control elements and except for when F2 is set to 1. The size of the F field is 1 bit. If the F field is included; if the size of the MAC SDU or variable-sized MAC control element is less than 128 bytes, the value of the F field is set to 0, otherwise it is set to 1;

-	F2: The Format2 field indicates the size of the Length field as indicated in table 6.2.1-3. There is one F2 field per MAC PDU subheader. The size of the F2 field is 1 bit. If the size of the MAC SDU or variable-sized MAC control element is larger than 32767 bytes, and if the corresponding subheader is not the last subheader, the value of the F2 field is set to 1, otherwise it is set to 0.

-	E: The Extension field is a flag indicating if more fields are present in the MAC header or not. The E field is set to "1" to indicate another set of at least R/F2/E/LCID fields. The E field is set to "0" to indicate that either a MAC SDU, a MAC control element or padding starts at the next byte;

-	R: Reserved bit, set to "0".

The MAC header and subheaders are octet aligned.

Table 6.2.1-1: Values of LCID for DL-SCH

| Index | LCID values |
| --- | --- |
| 00000 | CCCH |
| 00001-01010 | Identity of the logical channel |
| 01011-10111 | Reserved |
| 11000 | Activation/Deactivation (4 octets) |
| 11001 | SC-MCCH, SC-MTCH (see note) |
| 11010 | Long DRX Command |
| 11011 | Activation/Deactivation (1 octet) |
| 11100 | UE Contention Resolution Identity |
| 11101 | Timing Advance Command |
| 11110 | DRX Command |
| 11111 | Padding |
| NOTE: Both SC-MCCH and SC-MTCH cannot be multiplexed with other logical channels in the same MAC PDU except for Padding. |  |

For NB-IoT only the following LCID values for DL-SCH are applicable: CCCH, Identity of the logical channel, UE Contention Resolution Identity, Timing Advance Command, DRX Command and Padding.

Table 6.2.1-2: Values of LCID for UL-SCH

| Index | LCID values |
| --- | --- |
| 00000 | CCCH |
| 00001-01010 | Identity of the logical channel |
| 01011 | CCCH |
| 01100-10101 | Reserved |
| 10110 | Truncated Sidelink BSR |
| 10111 | Sidelink BSR |
| 11000 | Dual Connectivity Power Headroom Report |
| 11001 | Extended Power Headroom Report |
| 11010 | Power Headroom Report |
| 11011 | C-RNTI |
| 11100 | Truncated BSR |
| 11101 | Short BSR |
| 11110 | Long BSR |
| 11111 | Padding |

For NB-IoT only the following LCID values for UL-SCH are applicable: CCCH (LCID "00000"), Identity of the logical channel, C-RNTI, Short BSR and Padding.

Table 6.2.1-3: Values of F and F2 fields

| Index of F2 | Index of F | Size of Length field (in bits) |
| --- | --- | --- |
| 0 | 0 | 7 |
|  | 1 | 15 |
| 1 | - | 16 |

Table 6.2.1-4: Values of LCID for MCH

| Index | LCID values |
| --- | --- |
| 00000 | MCCH (see note) |
| 00001-11100 | MTCH |
| 11101 | Reserved |
| 11110 | MCH Scheduling Information or Extended MCH Scheduling Information |
| 11111 | Padding |
| NOTE: If there is no MCCH on MCH, an MTCH could use this value. |  |

##### 22.3.1.2.3	Test description

22.3.1.2.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

-	RRC-Connection-Setup-NB (preamble: Table 8.1.5.2.3-1, step 3 [18]) using parameters as specified in Table 22.3.1.2.3.3-1

UE:

None.

Preamble

-	UE is in state NB-IoT UE Attach, Connected Mode, UE Test Loopback Activated (State 2B-NB) with test loop mode G and configured to return no data in UL according to [18] in Ncell 1.

22.3.1.2.3.2	Test procedure sequence

Table 22.3.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a downlink assignment to a C-RNTI different from the C-RNTI assigned to the UE on NPDCCH and transmits in the indicated downlink assignment a RLC PDU in a MAC PDU on NPDSCH. | <-- | (NPDCCH (unknown C-RNTI)) MAC PDU | - | - |
| 2 | Check: Does the UE send any HARQ ACK on PUCCH? | --> | HARQ ACK/NACK | 1 | F |
| 3 | The SS transmits DCI N1 with Irep=2 resulting in 4 repetitions. The SS transmits a MAC PDU containing a MAC SDU with a RLC SDU of 38 bytes in an AMD PDU(SN=X+1) with polling field ‘P’ set to ‘1’ and repeats it in the next 3 consecutive TTIs; the SS shall generate a CRC error for each transmission causing the UE to send a NACK (NOTE 1, 2, 4) | <-- | NPDCCH (DCI N1 )) MAC PDU (R/R/E/LCID MAC sub-header (E=’0’, LCID= ‘0011’), 40 bytes MAC SDU) | - | - |
| 4 | Check: Does the UE send HARQ NACK NPDSCH? | --> | HARQ NACK | 2 | P |
| - | EXCEPTION: Step 5 shall be repeated till HARQ ACK is received at step 6 (NOTE 11). | - | - | - | - |
| 5 | The SS indicates retransmissions on NPDCCH by transmitting DCI N1 with Irep=1 and NDI same as step 3 and transmits the same MAC PDU like step 3. |  | NPDCCH (DCI N1 )) MAC PDU (R/R/E/LCID MAC sub-header (E=’0’, LCID= ‘0011’), 40 bytes MAC SDU) |  |  |
| - | EXCEPTION: HARQ NACK from the UE should be allowed at step 6 (NOTE 11). | - | - | - | - |
| 6 | Check: Does the UE send HARQ ACK NPDSCH? | --> | HARQ ACK | 2 | P |
| 7 | The SS allocates an UL grant for UE to send the RLC status PDU (NOTE 3) | - | - | - | - |
| 8 | Check: Does the UE transmit a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU in step 3? | --> | MAC PDU (RLC STATUS PDU (ACK_SN ‘X+2’)) | 2 | P |
| 9 | The SS transmits DCI N1 with Irep=3 resulting in 8 repetitions. The SS transmits three MAC SDUs each containing a RLC SDU of 18, 19 or 21 bytes in an AMD PDU (SN=X+2 to X+4) with the polling field ‘P’ set to ‘1’ in the last AMD PDU and repeats it in the next 7 consecutive TTIs. (NOTE 1, 2, 5) | <-- | NPDCCH (DCI N1 )) MAC PDU (2 x R/R/E/LCID/L MAC sub-header (E=’1’, LCID=‘0011’, F=’0’, L=’11’), R/R/E/LCID MAC sub-header (E=’0’, LCID=‘0011’), 19 bytes MAC SDU, 19 bytes MAC SDU, 21 bytes MAC SDU | - | - |
| 10 | Check: Does the UE send HARQ ACK NPDSCH? | --> | HARQ ACK | 3 | P |
| 11 | In the third (next for NTN) NPDCCH period bundle after the transmission at step 9 the SS allocates an UL grant for the UE to send the RLC status PDU (NOTE 3, NOTE 4) | - | - | - | - |
| 12 | Check: Does the UE transmit a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDUs in step 5? | --> | MAC PDU (RLC STATUS PDU (ACK_SN ‘X+5’)) | 3 | P |
| 13 | The SS transmits DCI N1 with Irep=4 resulting in 16 repetitions. The SS transmits a MAC PDU containing an RLC SDU in an AMD PDU with polling field ‘P’ set to ‘1’and repeats it in the next 15 consecutive TTIs. (NOTE 1, 2, 6) | <-- | NPDCCH (DCI N1 )) MAC PDU(AMD PDU, 7-byte padding) | - | - |
| 14 | Check: Does the UE send HARQ ACK NPDSCH? | --> | HARQ ACK | 4 | P |
| 15 | In the third (next for NTN) NPDCCH period bundle after the transmission at step 13 the SS allocates an UL grant for the UE to send the RLC status PDU (NOTE 3, NOTE 4) | - | - | - | - |
| 16 | Check: Does the UE transmit an RLC STATUS PDU with ACK_SN field equal to X+6? | --> | RLC STATUS PDU (ACK_SN ‘X+6’) | 4 | P |
| 17 | The SS transmits DCI N1 with Irep=5 resulting in 32 repetitions. The SS transmits a MAC PDU containing an RLC SDU in an AMD PDU with polling field ‘P’ set to ‘1’ and repeats it in the next 31 consecutive TTIs. (NOTE 1, 2, 7) | <-- | NPDCCH (DCI N1 )) MACPDU(AMD PDU, one byte padding) | - | - |
| 18 | Check: Does the UE send HARQ ACK NPDSCH? | --> | HARQ ACK | 5 | P |
| 19 | In the third (next for NTN) NPDCCH period bundle after the transmission at step 17 the SS allocates an UL grant for the UE to send the RLC status PDU (NOTE 3,NOTE 4) | - | - | - | - |
| 20 | Check: Does the UE transmit an RLC STATUS PDU with ACK_SN field equal to X+7? | --> | MAC PDU(RLC STATUS PDU (ACK_SN =X+7) ) | 5 | P |
| 21 | The SS transmits a Timing Advance Command without any additional padding (i.e. TBS=16). Start Timer_1 = Time Alignment timer value. The Timing Advance Command indicates a TA index of 31 resulting in no change of timing advance (see TS 36.213 clause 4.2.3) | <-- | NPDCCH (DCI N1)) MAC Control PDU (Timing Advance) | - | - |
| 22 | Check: Does the UE send HARQ ACK NPDSCH? | --> | HARQ ACK | 6 | P |
| 23 | The SS waits for 39 (4 for NTN) NPDCCH period bundles. (NOTE 8, NOTE 4) | - | - | - | - |
| 24 | In the 40th (5th for NTN) NPDCCH period bundle after the transmission at step 21 the SS transmit another Timing Advance MAC PDU (8 bits) with 1-byte padding (i.e. TBS=24). Restart Timer_1 = Time Alignment timer value The Timing Advance Command indicates a TA index of 63 resulting in new timing advance to be applied at step 30. (NOTE 8,NOTE 4) | <-- | MAC Control Element (Timing Advance) + 1-byte padding | - | - |
| 25 | Check: Does the UE send HARQ ACK NPDSCH? | --> | HARQ ACK | 6 | P |
| 26 | The SS waits for 55 (6 for NTN) NPDCCH period bundles. (NOTE 9, NOTE 4) | - | - | - | - |
| 27 | In the 56th (7th for NTN)NPDCCH period bundle after the transmission at step 24 the SS transmits MAC PDU containing one RLC SDU in an AMD PDU with polling field ‘P’ set to ‘1’. (NOTE 9, NOTE 4) | <-- | MAC PDU(AMD PDU (SN=X+7, P=1)) | - | - |
| 28 | Check: Does the UE send HARQ ACK NPDSCH? | --> | HARQ ACK | 6 | P |
| 29 | In the 58th (8th for NTN)NPDCCH period bundle after the transmission at step 24 the SS allocates an UL grant for UE to send the RLC status PDU. (NOTE 3, NOTE 4) | - | - | - | - |
| 30 | Check: Does the UE transmit an RLC STATUS PDU acknowledging the reception of the RLC PDU in step 27 with new Timing Advance? | --> | MAC PDU(RLC STATUS PDU (ACK_SN =X+8)) | 6 | P |
| 31 | Wait for timeAlignmentTimer to expire in UE | - | - | - | - |
| 32 | In the 88th (9th for NTN) NPDCCH period bundle after the transmission at step 21 SS transmits a downlink assignment to C-RNTI assigned to the UE on NPDCCH and transmits in the indicated downlink assignment a RLC PDU with polling field ‘P’ set to ‘0’ in a MAC PDU on NPDSCH. (NOTE 10, NOTE 13, , NOTE 4) | <-- | (NPDCCH (C-RNTI)) MAC PDU | - | - |
| 33 | Check: Does the UE send any HARQ ACK on PUCCH? | --> | HARQ ACK/NACK | 7 | F |
| 34 | In the 90th (10th for NTN) NPDCCH period bundle after the transmission at step 24 the SS allocates an UL grant for the UE. (NOTE 3, NOTE 4) | - | - | - | - |
| 35 | Check: Does the UE transmit any UL MAC PDU? | --> | MAC PDU | 8 | F |
| 36 | The SS transmits a NPDCCH order to C-RNTI assigned to the UE on NPDCCH | <-- | NPDCCH order (sub-carrier index := 13) | - | - |
| 37 | Check: Does the UE transmit a preamble on NPRACH with RAPID corresponding to subcarrier index provided in step 36? | --> | NPRACH Preamble | 9 | P |
| 38 | The SS transmits a MAC PDU addressed to UE RA-RNTI, containing a matching RAPID. (NOTE 12) | <-- | Random Access Response | - | - |
| 39 | C-RNTI based contention resolution happens and causes the UE to be UL synchronised again. (NOTE 12) | - | - | - | - |
| NOTE 1: RLC SDU with size n contains a DLInformationTransfer-NB with ESM_DATA_TRANSPORT: 18 or 26 bits DLInformationTransfer-NB(PER encode: ‘0000000000’B + 8 bits ( length of DedicatedInfoNAS <= 127 bytes ) or 16 bits length ( 128 bytes <= length of DedicatedInfoNAS < 16383 bytes )) 1/2 byte Protocol discriminator ( SECURITY PROTECTED NAS MESSAGE ) 1/2 byte Security header type ( SECURITY PROTECTED NAS MESSAGE ) 4 bytes Message authentication code ( SECURITY PROTECTED NAS MESSAGE ) 1 byte Sequence number ( SECURITY PROTECTED NAS MESSAGE ) 1/2 byte Protocol discriminator ( ESM_DATA_TRANSPORT ) 1/2 byte EPS bearer identity ( ESM_DATA_TRANSPORT ) 1 byte Procedure transaction identity ( ESM_DATA_TRANSPORT ) 1 byte ESM data transport message identity ( ESM_DATA_TRANSPORT ) 2 bytes length indicator of the user data container ( ESM_DATA_TRANSPORT ) N bytes User data container contents ( UL RLC SDU ) 6 bits Aligned bits ( PER encode ) n = N+14 or N+15 bytes DL RLC SDU NOTE 2: The RLC SQN X is the last SQN used on SRB1bis before start of the test sequence. NOTE 3: NPDCCH period is 64ms according to TS 36.508 Table 8.1.6.3-3 [18]. NOTE 4: MAC PDU at step 3 and 5: 1 byte R/R/E/LCID MAC SDU sub-header 40 bytes MAC SDU containing RLC AMD PDU (2 bytes AMD header, 38 bytes RLC SDU) TBS= 328 (ISF=1, ITBS=10) NOTE 5: MAC PDU at step 9: 4 bytes 2 x R/R/E/LCID/L MAC SDU sub-header 1 byte R/R/E/LCID MAC SDU sub-header 18 bytes MAC SDU containing RLC AMD PDU (2 bytes AMD header, 16 bytes RLC SDU) 19 bytes MAC SDU containing RLC AMD PDU (2 bytes AMD header, 17 bytes RLC SDU) 21 bytes MAC SDU containing RLC AMD PDU (2 bytes AMD header, 19 bytes RLC SDU) TBS= 504 (ISF=2, ITBS=10) NOTE 6: MAC PDU at step 13: 2 bytes R/R/E/LCID/L MAC SDU sub-header 1 byte R/F2/E/LCID padding sub-header 18 bytes MAC SDU containing RLC AMD PDU (2 bytes AMD header, 16 bytes RLC SDU) 7 bytes padding TBS= 224 (ISF=1, ITBS=7) NOTE 7: MAC PDU at step 17: 1 byte R/F2/E/LCID padding sub-header 1 byte R/R/E/LCID MAC SDU sub-header 16 bytes MAC SDU containing RLC AMD PDU (2 bytes AMD header, 14 bytes RLC SDU) TBS= 144 (ISF=0, ITBS=10) NOTE 8: With an NPDCCH period of 64ms 40 NPDCCH periods are 2.56s i.e. the transmission at step 23 happens at about 50% of Timer_1 expiry (5.12s). NOTE 9: With an NPDCCH period of 64ms 56 NPDCCH periods are 3.584s i.e. the transmission at step 27 happens at about 70% of Timer_1 expiry (5.12s). NOTE 10: With an NPDCCH period of 64ms 88 NPDCCH periods are 5.632s corresponding to Timer_1 + 10%. NOTE 11: UE soft combiner implementation should have sufficient retransmissions to be able to successfully decode the data in its soft buffer. NOTE 12: The value of T-CRNTI and CRNTI are different at step 38/39. NOTE 13: The DL RLC Sequence number is incremented after step 32. NOTE 14: NPDCCH period bundle is specified in TS 36.523-3 [20] clause 7A.14.2. |  |  |  |  |  |

22.3.1.2.3.3	Specific message contents

Table 22.3.1.2.3.3-1: RRCConnectionSetup-NB

| Derivation path: 36.508 table 8.1.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetup-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetup-r13 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated-r13 SEQUENCE { |  |  |  |
| mac-MainConfig-r13 CHOICE { |  |  |  |
| explicitValue-r13 SEQUENCE { |  |  |  |
| timeAlignmentTimerDedicated-r13 | sf5120 | needs to be long enough to ensure time alignment timer not to time out before step 20 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 22.3.1.3	NB-IoT / Correct Handling of UL MAC PDU / Assignment / HARQ process / Padding

##### 22.3.1.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives for a TTI an uplink grant with valid C-RNTI }

then { UE transmits UL MAC PDU repetitions as per UL_REPETITION_NUMBER }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives an UL Grant with toggled NDI, redundancy version = 0 in DCI format and has data available for transmission}

then { UE transmits a new MAC PDU in repetitions as per UL_REPETITION_NUMBER using redundancy versions alternating between 0,2 for each repetition}

}

(3)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives an UL Grant with un toggled NDI and redundancy version = 1 in DCI format }

then { UE retransmits the  MAC PDU in repetitions as per UL_REPETITION_NUMBER using redundancy versions alternating between 2,0 for each repetition }

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE inserts a R/R/E/LCID field in the MAC header and there is a subsequent R/R/E/LCID field to be inserted }

then { UE sets E field to 1 }

}

(5)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE inserts a R/R/E/LCID field in the MAC header and a MAC SDU or a MAC control element starts at the next byte }

then { UE sets E field to 0 }

}

(6)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE inserts the last MAC sub-header in the MAC PDU }

then { UE inserts a MAC sub-header consist solely of the four header fields R/R/E/LCID }

}

(7)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE is to transmit a MAC PDU with padding exceeding 2 bytes }

then { UE inserts the last MAC sub-header as a padding MAC subheader consisting solely of the four header fields R/R/E/LCID with LCID set to Padding and Padding goes to the end of the MAC PDU }

}

(8)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE is to transmit a MAC PDU with single-byte padding and there is a data MAC PDU sub-header present }

then { UE is inserting padding MAC PDU subheader before any other MAC PDU sub-header }

}

(9)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE is to transmit a MAC PDU with two-byte padding and there is a data MAC PDU sub-header }

then { UE is inserting two padding MAC PDU subheaders before any other MAC PDU sub-header or one padding MAC PDU subheader as a last MAC PDU subheader }

}

##### 22.3.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.321, clause 5.4.1, 5.4.2.1, 5.4.2.2, 6.1.2, 6.2.1. Unless otherwise stated these are Rel-13 requirements.

[TS 36.321, clause 5.4.1]

In order to transmit on the UL-SCH the MAC entity must have a valid uplink grant (except for non-adaptive HARQ retransmissions) which it may receive dynamically on the PDCCH or in a Random Access Response or which may be configured semi-persistently. To perform requested transmissions, the MAC layer receives HARQ information from lower layers. When the physical layer is configured for uplink spatial multiplexing, the MAC layer can receive up to two grants (one per HARQ process) for the same TTI from lower layers.

If the MAC entity has a C-RNTI, a Semi-Persistent Scheduling C-RNTI, or a Temporary C-RNTI, the MAC entity shall for each TTI and for each Serving Cell belonging to a TAG that has a running timeAlignmentTimer and for each grant received for this TTI:

-	if an uplink grant for this TTI and this Serving Cell has been received on the PDCCH for the MAC entity’s C-RNTI or Temporary C-RNTI; or

-	if an uplink grant for this TTI has been received in a Random Access Response:

-	if the uplink grant is for MAC entity’s C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity’s Semi-Persistent Scheduling C-RNTI or a configured uplink grant:

-	consider the NDI to have been toggled for the corresponding HARQ process regardless of the value of the NDI.

-	deliver the uplink grant and the associated HARQ information to the HARQ entity for this TTI.

-	else, if this Serving Cell is the SpCell and if an uplink grant for this TTI has been received for the SpCell on the PDCCH of the SpCell for the MAC entity’s Semi-Persistent Scheduling C-RNTI:

-	if the NDI in the received HARQ information is 1:

-	consider the NDI for the corresponding HARQ process not to have been toggled;

-	deliver the uplink grant and the associated HARQ information to the HARQ entity for this TTI.

-	else if the NDI in the received HARQ information is 0:

-	if PDCCH contents indicate SPS release:

-	clear the configured uplink grant (if any).

-	else:

-	store the uplink grant and the associated HARQ information as configured uplink grant;

-	initialise (if not active) or re-initialise (if already active) the configured uplink grant to start in this TTI and to recur according to rules in subclause 5.10.2;

-	if UL HARQ operation is asynchronous, set the HARQ Process ID to the HARQ Process ID associated with this TTI;

-	consider the NDI bit for the corresponding HARQ process to have been toggled;

-	deliver the configured uplink grant and the associated HARQ information to the HARQ entity for this TTI.

-	else, if this Serving Cell is the SpCell and an uplink grant for this TTI has been configured for the SpCell:

-	if UL HARQ operation is asynchronous, set the HARQ Process ID to the HARQ Process ID associated with this TTI;

-	consider the NDI bit for the corresponding HARQ process to have been toggled;

-	deliver the configured uplink grant, and the associated HARQ information to the HARQ entity for this TTI.

NOTE:	The period of configured uplink grants is expressed in TTIs.

NOTE:	If the MAC entity receives both a grant in a Random Access Response and a grant for its C-RNTI or Semi persistent scheduling C-RNTI requiring transmissions on the SpCell in the same UL subframe, the MAC entity may choose to continue with either the grant for its RA-RNTI or the grant for its C-RNTI or Semi persistent scheduling C-RNTI.

NOTE:	When a configured uplink grant is indicated during a measurement gap and indicates an UL-SCH transmission during a measurement gap, the MAC entity processes the grant but does not transmit on UL-SCH. When a configured uplink grant is indicated during a Sidelink Discovery gap for reception and indicates an UL-SCH transmission during a Sidelink Discovery gap for transmission with a SL-DCH transmission, the MAC entity processes the grant but does not transmit on UL-SCH.

For configured uplink grants, the HARQ Process ID associated with this TTI is derived from the following equation for asynchronous UL HARQ operation:

HARQ Process ID = [floor(CURRENT_TTI/semiPersistSchedIntervalUL)] modulo numberOfConfUlSPS-Processes,

where CURRENT_TTI=[(SFN * 10) + subframe number] and it refers to the subframe where the first transmission of a bundle takes place.

[TS 36.321, clause 5.4.2.1]

There is one HARQ entity at the MAC entity for each Serving Cell with configured uplink, which maintains a number of parallel HARQ processes allowing transmissions to take place continuously while waiting for the HARQ feedback on the successful or unsuccessful reception of previous transmissions.

The number of parallel HARQ processes per HARQ entity is specified in [2], clause 8. NB-IoT has one UL HARQ process.

When the physical layer is configured for uplink spatial multiplexing [2], there are two HARQ processes associated with a given TTI. Otherwise there is one HARQ process associated with a given TTI.

At a given TTI, if an uplink grant is indicated for the TTI, the HARQ entity identifies the HARQ process(es) for which a transmission should take place. It also routes the received HARQ feedback (ACK/NACK information), MCS and resource, relayed by the physical layer, to the appropriate HARQ process(es).

In asynchronous HARQ operation, a HARQ process is associated with a TTI based on the received UL grant except for UL grant in RAR. Except for NB-IoT, each asynchronous HARQ process is associated with a HARQ process identifier. For UL transmission with UL grant in RAR, HARQ process identifier 0 is used. HARQ feedback is not applicable for asynchronous UL HARQ.

When TTI bundling is configured, the parameter TTI_BUNDLE_SIZE provides the number of TTIs of a TTI bundle. TTI bundling operation relies on the HARQ entity for invoking the same HARQ process for each transmission that is part of the same bundle. Within a bundle HARQ retransmissions are non-adaptive and triggered without waiting for feedback from previous transmissions according to TTI_BUNDLE_SIZE. The HARQ feedback of a bundle is only received for the last TTI of the bundle (i.e. the TTI corresponding to TTI_BUNDLE_SIZE), regardless of whether a transmission in that TTI takes place or not (e.g. when a measurement gap occurs). A retransmission of a TTI bundle is also a TTI bundle. TTI bundling is not supported when the MAC entity is configured with one or more SCells with configured uplink.

Uplink HARQ operation is asynchronous for NB-IoT UEs, BL UEs or UEs in enhanced coverage except for the repetitions within a bundle.

For NB-IoT UEs, BL UEs or UEs in enhanced coverage, the parameter UL_REPETITION_NUMBER provides the number of transmission repetitions within a bundle. For each bundle, UL_REPETITION_NUMBER is set to a value provided by lower layers. Bundling operation relies on the HARQ entity for invoking the same HARQ process for each transmission that is part of the same bundle. Within a bundle HARQ retransmissions are non-adaptive and are triggered without waiting for feedback from previous transmissions according to UL_REPETITION_NUMBER. An uplink grant corresponding to a new transmission or a retransmission of the bundle is only received after the last repetition of the bundle. A retransmission of a bundle is also a bundle.

TTI bundling is not supported for RN communication with the E-UTRAN in combination with an RN subframe configuration.

For transmission of Msg3 during Random Access (see subclause 5.1.5) TTI bundling does not apply. For NB-IoT UEs, BL UEs or UEs in enhanced coverage, uplink repetition bundling is used for transmission of Msg3.

For each TTI, the HARQ entity shall:

-	identify the HARQ process(es) associated with this TTI, and for each identified HARQ process:

-	if an uplink grant has been indicated for this process and this TTI:

-	if the received grant was not addressed to a Temporary C-RNTI on PDCCH and if the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this HARQ process; or

-	if the uplink grant was received on PDCCH for the C-RNTI and the HARQ buffer of the identified process is empty; or

-	if the uplink grant was received in a Random Access Response:

-	if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a Random Access Response:

-	obtain the MAC PDU to transmit from the Msg3 buffer.

-	else:

-	obtain the MAC PDU to transmit from the "Multiplexing and assembly" entity;

-	deliver the MAC PDU and the uplink grant and the HARQ information to the identified HARQ process;

-	instruct the identified HARQ process to trigger a new transmission.

-	else:

-	deliver the uplink grant and the HARQ information (redundancy version) to the identified HARQ process;

-	instruct the identified HARQ process to generate an adaptive retransmission.

-	else, if the HARQ buffer of this HARQ process is not empty:

-	instruct the identified HARQ process to generate a non-adaptive retransmission.

When determining if NDI has been toggled compared to the value in the previous transmission the MAC entity shall ignore NDI received in all uplink grants on PDCCH for its Temporary C-RNTI.

[TS 36.321, clause 5.4.2.2]

Each HARQ process is associated with a HARQ buffer.

For synchronous HARQ, each HARQ process shall maintain a state variable CURRENT_TX_NB, which indicates the number of transmissions that have taken place for the MAC PDU currently in the buffer, and a state variable HARQ_FEEDBACK, which indicates the HARQ feedback for the MAC PDU currently in the buffer. When the HARQ process is established, CURRENT_TX_NB shall be initialized to 0.

The sequence of redundancy versions is 0, 2, 3, 1. The variable CURRENT_IRV is an index into the sequence of redundancy versions. This variable is up-dated modulo 4. For BL UEs or UEs in enhanced coverage see subclause 8.6.1 in [2] for the sequence of redundancy versions and redundancy version determination. For NB-IoT UEs see subclause 16.5.1.2 in [2] for the sequence of redundancy versions and redundancy version determination.

For NB-IoT UEs, BL UEs or UEs in enhanced coverage for UL_REPETITION_NUMBER for Mode B operation, the same redundancy version is used multiple times before cycling to the next redundancy version as specified in Subclause 16.5.1.2, 8.6.1 and 7.1.7.1 in [2].

New transmissions are performed on the resource and with the MCS indicated on PDCCH or Random Access Response. Adaptive retransmissions are performed on the resource and, if provided, with the MCS indicated on PDCCH. Non-adaptive retransmission is performed on the same resource and with the same MCS as was used for the last made transmission attempt.

For synchronous HARQ, the MAC entity is configured with a maximum number of HARQ transmissions and a maximum number of Msg3 HARQ transmissions by RRC: maxHARQ-Tx and maxHARQ-Msg3Tx respectively. For transmissions on all HARQ processes and all logical channels except for transmission of a MAC PDU stored in the Msg3 buffer, the maximum number of transmissions shall be set to maxHARQ-Tx. For transmission of a MAC PDU stored in the Msg3 buffer, the maximum number of transmissions shall be set to maxHARQ-Msg3Tx.

When the HARQ feedback is received for this TB, the HARQ process shall:

-	set HARQ_FEEDBACK to the received value.

If the HARQ entity requests a new transmission, the HARQ process shall:

-	if UL HARQ operation is synchronous:

-	set CURRENT_TX_NB to 0;

-	set HARQ_FEEDBACK to NACK;

-	set CURRENT_IRV to 0;

-	store the MAC PDU in the associated HARQ buffer;

-	store the uplink grant received from the HARQ entity;

-	generate a transmission as described below.

If the HARQ entity requests a retransmission, the HARQ process shall:

-	if UL HARQ operation is synchronous:

-	increment CURRENT_TX_NB by 1;

-	if the HARQ entity requests an adaptive retransmission:

-	store the uplink grant received from the HARQ entity;

-	set CURRENT_IRV to the index corresponding to the redundancy version value provided in the HARQ information;

-	if UL HARQ operation is synchronous:

-	set HARQ_FEEDBACK to NACK;

-	generate a transmission as described below.

-	else if the HARQ entity requests a non-adaptive retransmission:

-	if UL HARQ operation is asynchronous or HARQ_FEEDBACK = NACK:

-	generate a transmission as described below.

NOTE:	When receiving a HARQ ACK alone, the MAC entity keeps the data in the HARQ buffer.

NOTE:	When no UL-SCH transmission can be made due to the occurrence of a measurement gap or a Sidelink Discovery Gap for Transmission, no HARQ feedback can be received and a non-adaptive retransmission follows.

NOTE:	For asynchronous HARQ operation, UL retransmissions are triggered only by adaptive retransmission grants, except for retransmissions within a bundle.

To generate a transmission, the HARQ process shall:

-	if the MAC PDU was obtained from the Msg3 buffer; or

-	if Sidelink Discovery Gaps for Transmission are not configured by upper layers, and there is no measurement gap at the time of the transmission and, in case of retransmission, the retransmission does not collide with a transmission for a MAC PDU obtained from the Msg3 buffer in this TTI; or

-	if Sidelink Discovery Gaps for Transmission are configured by upper layers, and there is no measurement gap at the time of the transmission and, in case of retransmission, the retransmission does not collide with a transmission for a MAC PDU obtained from the Msg3 buffer, and there is no Sidelink Discovery Gap for Transmission in this TTI; or

-	if Sidelink Discovery Gaps for Transmission are configured by upper layers, and there is no measurement gap at the time of the transmission and, in case of retransmission, the retransmission does not collide with a transmission for a MAC PDU obtained from the Msg3 buffer, and there is a Sidelink Discovery Gap for Transmission, and there is no configured grant for transmission on SL-DCH in this TTI:

-	instruct the physical layer to generate a transmission according to the stored uplink grant with the redundancy version corresponding to the CURRENT_IRV value;

-	increment CURRENT_IRV by 1;

-	if UL HARQ operation is synchronous and there is a measurement gap or Sidelink Discovery Gap for Reception at the time of the HARQ feedback reception for this transmission and if the MAC PDU was not obtained from the Msg3 buffer:

-	set HARQ_FEEDBACK to ACK at the time of the HARQ feedback reception for this transmission.

After performing above actions, if UL HARQ operation is synchronous the HARQ process then shall:

-	if CURRENT_TX_NB = maximum number of transmissions – 1:

-	flush the HARQ buffer;

[TS 36.321, clause 6.1.2]

A MAC PDU consists of a MAC header, zero or more MAC Service Data Units (MAC SDU), zero, or more MAC control elements, and optionally padding; as described in Figure 6.1.2-3.

Both the MAC header and the MAC SDUs are of variable sizes.

A MAC PDU header consists of one or more MAC PDU subheaders; each subheader corresponds to either a MAC SDU, a MAC control element or padding.

A MAC PDU subheader consists of the five or six header fields R/F2/E/LCID/(F)/L but for the last subheader in the MAC PDU and for fixed sized MAC control elements. The last subheader in the MAC PDU and subheaders for fixed sized MAC control elements consist solely of the four header fields R/F2/E/LCID. A MAC PDU subheader corresponding to padding consists of the four header fields R/F2/E/LCID.

Figure 6.1.2-1: R/F2/E/LCID/F/L MAC subheader

Figure 6.1.2-1a: R/F2/E/LCID/L MAC subheader

Figure 6.1.2-2: R/F2/E/LCID MAC subheader

MAC PDU subheaders have the same order as the corresponding MAC SDUs, MAC control elements and padding.

MAC control elements are always placed before any MAC SDU.

Padding occurs at the end of the MAC PDU, except when single-byte or two-byte padding is required. Padding may have any value and the MAC entity shall ignore it. When padding is performed at the end of the MAC PDU, zero or more padding bytes are allowed.

When single-byte or two-byte padding is required, one or two MAC PDU subheaders corresponding to padding are placed at the beginning of the MAC PDU before any other MAC PDU subheader.

A maximum of one MAC PDU can be transmitted per TB per MAC entity. A maximum of one MCH MAC PDU can be transmitted per TTI.

Figure 6.1.2-3: Example of MAC PDU consisting of MAC header, MAC control elements, MAC SDUs and padding

[TS 36.321, clause 6.2.1]

The MAC header is of variable size and consists of the following fields:

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC control element or padding as described in tables 6.2.1-1, 6.2.1-2 and 6.2.1-4 for the DL-SCH, UL-SCH and MCH respectively. There is one LCID field for each MAC SDU, MAC control element or padding included in the MAC PDU. In addition to that, one or two additional LCID fields are included in the MAC PDU, when single-byte or two-byte padding is required but cannot be achieved by padding at the end of the MAC PDU. A UE of Category 0 [12] shall indicate CCCH using LCID "01011", otherwise the UE shall indicate CCCH using LCID "00000". The LCID field size is 5 bits;

-	L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC control element in bytes. There is one L field per MAC PDU subheader except for the last subheader and subheaders corresponding to fixed-sized MAC control elements. The size of the L field is indicated by the F field and F2 field;

-	F: The Format field indicates the size of the Length field as indicated in table 6.2.1-3. There is one F field per MAC PDU subheader except for the last subheader and subheaders corresponding to fixed-sized MAC control elements and except for when F2 is set to 1. The size of the F field is 1 bit. If the F field is included; if the size of the MAC SDU or variable-sized MAC control element is less than 128 bytes, the value of the F field is set to 0, otherwise it is set to 1;

-	F2: The Format2 field indicates the size of the Length field as indicated in table 6.2.1-3. There is one F2 field per MAC PDU subheader. The size of the F2 field is 1 bit. If the size of the MAC SDU or variable-sized MAC control element is larger than 32767 bytes, and if the corresponding subheader is not the last subheader, the value of the F2 field is set to 1, otherwise it is set to 0.

-	E: The Extension field is a flag indicating if more fields are present in the MAC header or not. The E field is set to "1" to indicate another set of at least R/F2/E/LCID fields. The E field is set to "0" to indicate that either a MAC SDU, a MAC control element or padding starts at the next byte;

-	R: Reserved bit, set to "0".

The MAC header and subheaders are octet aligned.

Table 6.2.1-1: Values of LCID for DL-SCH

| Index | LCID values |
| --- | --- |
| 00000 | CCCH |
| 00001-01010 | Identity of the logical channel |
| 01011-10111 | Reserved |
| 11000 | Activation/Deactivation (4 octets) |
| 11001 | SC-MCCH, SC-MTCH (see note) |
| 11010 | Long DRX Command |
| 11011 | Activation/Deactivation (1 octet) |
| 11100 | UE Contention Resolution Identity |
| 11101 | Timing Advance Command |
| 11110 | DRX Command |
| 11111 | Padding |
| NOTE: Both SC-MCCH and SC-MTCH cannot be multiplexed with other logical channels in the same MAC PDU except for Padding |  |

For NB-IoT only the following LCID values for DL-SCH are applicable: CCCH, Identity of the logical channel, UE Contention Resolution Identity, Timing Advance Command, DRX Command and Padding.

Table 6.2.1-2: Values of LCID for UL-SCH

| Index | LCID values |
| --- | --- |
| 00000 | CCCH |
| 00001-01010 | Identity of the logical channel |
| 01011 | CCCH |
| 01100-10101 | Reserved |
| 10110 | Truncated Sidelink BSR |
| 10111 | Sidelink BSR |
| 11000 | Dual Connectivity Power Headroom Report |
| 11001 | Extended Power Headroom Report |
| 11010 | Power Headroom Report |
| 11011 | C-RNTI |
| 11100 | Truncated BSR |
| 11101 | Short BSR |
| 11110 | Long BSR |
| 11111 | Padding |

For NB-IoT only the following LCID values for UL-SCH are applicable: CCCH (LCID "00000"), Identity of the logical channel, C-RNTI, Short BSR and Padding.

Table 6.2.1-3: Values of F and F2 fields:

| Index of F2 | Index of F | Size of Length field (in bits) |
| --- | --- | --- |
| 0 | 0 | 7 |
|  | 1 | 15 |
| 1 | - | 16 |

Table 6.2.1-4: Values of LCID for MCH

| Index | LCID values |
| --- | --- |
| 00000 | MCCH (see note) |
| 00001-11100 | MTCH |
| 11101 | Reserved |
| 11110 | MCH Scheduling Information or Extended MCH Scheduling Information |
| 11111 | Padding |
| NOTE: If there is no MCCH on MCH, an MTCH could use this value. |  |

##### 22.3.1.3.3	Test description

22.3.1.3.3.1	Pre-test conditions

System Simulator:

-	NCell 1.

UE:

-	None.

Preamble:

-	The UE shall be in State 2B-NB with test loop mode G (CP CIoT Optimisation) according to TS 36.508 [18].

22.3.1.3.3.2	Test procedure sequence

NOTE:	In the Table 22.3.1.3.3.2-1 the LCID=’00011’ maps to SIB1bis in CP mode.

If the start RLC UL and DL sequence numbers to be used at start of test body are non zero, but X and Y respectively due to transmission/reception of RLC PDU's in preamble on bearer to be used, then any sequence number 'n' in test procedure maps as:

-	UL SQN n maps to SQN X+n MOD 1024 &

-	DL SQN n maps to SQN Y+n MOD 1024.

Table 22.3.1.3.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a MAC PDU containing RLC PDU with polling field ‘P’ set to '0' and an RLC SDU | <-- | MAC PDU(RLC SN=0) | - | - |
| 2 | The SS shall schedule UL grants at the beginning of the next 10 NPDCCH bundle, allowing the UE to return the RLC SDU as received in step 1, on NPDCCH, but with the C-RNTI different from the C-RNTI assigned to the UE.(Note 8) | <-- | (UL Grant (unknown C-RNTI)) | - | - |
| 3 | Check: Does the UE transmit a MAC PDU corresponding to grant in step 2? | --> | MAC PDU | 1 | F |
| 4 | SS transmits one UL Grant, allowing the UE to return the RLC SDU as received in step 1, on NPDCCH with the C-RNTI assigned to the UE. | <-- | (UL Grant (C-RNTI)) | - | - |
| 5 | Check: Does the UE transmit a MAC PDU corresponding to grant in step 4? | --> | MAC PDU(RLC SN=0) | 1 | P |
| 6 | SS transmits an RLC STATUS PDU to acknowledge correctly received data。 | <-- | RLC STATUS PDU(RLC SN=1) | - | - |
| 7 | The SS Transmits a MAC PDU containing RLC PDU with polling field ‘P’ set to '0' and an RLC SDU of 16 bytes | <-- | MAC PDU(RLC SN=1) | - | - |
| 8 | In the third NPDCCH period bundle after DL transmission of step 7 the SS allocates one UL Grant of size 208 bits (Note 7), sufficient for one RLC SDU to be loop backed in slots within a repetition, and NDI indicates new transmission, and redundancy version = 0. (Note 8) | <-- | Uplink Grant | - | - |
| 9 | Check: Does the UE transmit a MAC PDU including one RLC SDU, with redundancy version alternating between 0, 2 for each repetition? (Note 1) | --> | MAC PDU(RLC SN=1) | 2 | P |
| 10 | The SS allocates one UL Grant of size 208 bits (Note 7), sufficient for one RLC SDU to be loop backed in slots within a repetition, and NDI is the same with which in step 8, and redundancy version = 1. | <-- | Uplink Grant |  |  |
| 11 | Check: Does the UE retransmit the MAC PDU in step 9, with redundancy version alternating between 2, 0 for each repetition? (Note 1) | --> | MAC PDU(RLC SN=1) | 3 | P |
| 12 | SS transmits an RLC STATUS PDU to acknowledge correctly received data. | <-- | RLC STATUS PDU(RLC SN=2) | - | - |
| 13 | The SS Transmits a MAC PDU containing RLC PDU with polling field ‘P’ set to '1' and an RLC SDU of 15 bytes | <-- | MAC PDU(RLC SN=2) | - | - |
| 14 | In the third NPDCCH period bundle after DL transmission of step 13 the SS allocates one UL Grant of size 176 bits. (Note 2, Note 8) | <-- | (UL grant) | - | - |
| 15 | Check: Does the UE return a MAC PDU of length 176 bits containing two MAC sub-headers where the first MAC sub-header have the Expansion bit ‘E’ set to ‘1’ and the second MAC sub-header has the Expansion bit ‘E’ set to ‘0’ and no length field? (Note 3) | --> | MAC PDU (MAC sub-header (E=’1’, LCID=’00011’, L=’ 17’), MAC sub-header (E=’0’, LCID=’00011’, no Length field present), AMD PDU(RLC SN=2), Status PDU) Or MAC PDU (MAC sub-header (E=’1’, LCID=’00011’, L=’ 2’), MAC sub-header (E=’0’, LCID=’00011’, no Length field present), Status PDU, AMD PDU(RLC SN=2)) | 4,5,6 | P |
| 16 | SS transmits an RLC STATUS PDU to acknowledge correctly received data | <-- | RLC STATUS PDU(RLC SN=3) | - | - |
| 17 | Void |  |  |  |  |
| 18 | The SS transmits a MAC PDU containing RLC PDU with polling field ‘P’ set to '0' and an RLC SDU of 8 bytes | <-- | MAC PDU(RLC SN=3) | - | - |
| 19 | In the third NPDCCH period bundle after DL transmission of step 18 the SS transmits one uplink grant of size 176 bits. (Note 4, Note 8) | <-- | (UL grant) | - | - |
| 20 | Check: Does the UE transmit a MAC PDU with a MAC SDU of length 10 bytes and where the last MAC sub-header has the Extension field ‘E’ set to ‘0’ and the Logical Channel ID field ‘LCID’ set to ‘11111’? | --> | MAC PDU (BSR sub-header, MAC SDU sub-header, Padding MAC sub-header (E=’0’, LCID=’11111’),Short BSR, MAC SDU(RLC SN=3), padding) | 7 | P |
| 20A | SS transmits an RLC STATUS PDU to acknowledge correctly received data. | <-- | RLC STATUS PDU(RLC SN=4) |  |  |
| 21 | The SS transmits a MAC PDU containing RLC PDU with polling field ‘P’ set to '0' and an RLC SDU of 11 bytes | <-- | MAC PDU(RLC SN=4) | - | - |
| 22 | In the third NPDCCH period bundle after DL transmission of step 21 the SS transmits one uplink grant of size 120 bits. (Note 5, Note 8) | <-- | (UL grant) | - | - |
| 23 | Check: Does the UE transmit a MAC PDU with a MAC SDU of length 13 bytes and with a padding MAC sub-header, with Extension field ‘E’ is set to ‘1’ and the Logical Channel ID field ‘LCID’ is set to ‘11111’, inserted before the MAC SDU sub-header? | --> | MAC PDU (Padding MAC-sub-header (E=’1’, LCID=’11111’), MAC SDU sub-header, MAC SDU)(RLC SN=4) | 8 | P |
| 23A | SS transmits an RLC STATUS PDU to acknowledge correctly received data. | <-- | RLC STATUS PDU(RLC SN=5) |  |  |
| 24 | The SS transmits a MAC PDU containing RLC PDU with polling field ‘P’ set to '0' and an RLC SDU of 8 bytes | <-- | MAC PDU(RLC SN=5) | - | - |
| 25 | In the third NPDCCH period bundle after DL transmission of step 24 the SS transmits one uplink grant of size 120 bits. (Note 6, Note 8) | <-- | (UL grant) | - | - |
| 26 | Check: Does the UE transmit a MAC PDU with two padding MAC sub-headers, with Extension field ‘E’ is set to ‘1’ and the Logical Channel ID field ‘LCID’ is set to ‘11111’, inserted before the BSR sub-header and the MAC SDU sub-header. Or a MAC PDU with BSR sub-header with Extension field ‘E’ is set to ‘1’ and MAC SDU sub-header (R/R/E/LCID/F/L) inserted before the Padding MAC sub-header? | --> | MAC PDU (Padding MAC-sub-header#1 (E=’1’, LCID=’11111’), Padding MAC-sub-header#2 (E=’1’, LCID=’11111’), BSR sub-header, MAC SDU sub-header, Short BSR, MAC-SDU(RLC SN=5)) Or MAC PDU(BSR sub-header, MAC SDU sub-header, Padding MAC-sub-header(E=’0’, LCID=’11111’), Short BSR, MAC-SDU(RLC SN=5)) | 9 | P |
| 27 | SS transmits an RLC STATUS PDU to acknowledge correctly received data | <-- | RLC STATUS PDU(RLC SN=6) |  |  |
| NOTE 1: Transmission of a UL MAC PDU with a specific redundancy version by the UE is implicitly tested by receiving the UL MAC PDU correctly at SS. NOTE 2: UL grant of 176 bits (ITBS=3, IRU=2, see TS 36.213 Table 16.5.1.2-2) is chosen to enable UE to transmit two MAC SDUs, one of size 17 and one of size 2 bytes, in a MAC PDU (15 bytes RLC SDU + 2 bytes AMD PDU header + 2 bytes RLC Status PDU + 2 bytes MAC sub-header (7 bit LI) + one byte MAC sub-header (R/R/E/LCID) = 22 bytes = 176 bits). NOTE 3: MAC SDUs can come in any order NOTE 4: UL grant of 176 bits (ITBS=3, IRU=2, see TS 36.213 Table 16.5.1.2-2) is chosen such that the MAC PDU padding will be larger than 2 bytes. RLC SDU size is 8 bytes, size of AMD PDU header is 2 bytes, size of MAC header is 4 bytes (2 bytes for MAC SDU sub-header using 7-bit LI, 1 byte for BSR sub-header and 1 byte for padding MAC sub-header) and size of Short BSR is 1 byte, equals to 120 bits (15 bytes) and resulting into 56 bits padding. NOTE 5: UL grant of 120 bits (ITBS=0, IRU=4, see TS 36.213 Table 16.5.1.2-2) is chosen such that the MAC PDU padding will be a single byte. RLC SDU size is 11 bytes, size of AMD PDU header is 2 bytes and size of MAC header is 1 byte for MAC SDU sub-header, equals to 112 bits (14 bytes) and resulting into 1 single byte padding. NOTE 6: UL grant of 120 bits (ITBS=0, IRU=4, see TS 36.213 Table 16.5.1.2-2) is chosen such that the MAC PDU padding will be equal to 2 bytes. RLC SDU size is 8 bytes, size of AMD PDU header is 2 bytes, size of MAC header is 4 bytes (1 bytes for MAC SDU sub-header, 1 byte for Short BSR sub-header and 2 bytes for padding MAC sub-header) and size of Short BSR is 1 byte, equals to 120 bits (15 bytes) and resulting no padding at the end of the MAC PDU. NOTE 7: UL grant of 208 bits (ITBS=4, IRU=2, see TS 36.213 Table 16.5.1.2-2) is chosen. RLC SDU size is 16 bytes, size of AMD PDU header is 2 bytes and MAC header is 4 bytes (2 bytes MAC sub-header (7 bit LI), 1 byte for BSR sub-header and 1 byte for padding MAC sub-header) and size of Short BSR is 1 byte, equals to 23 bytes = 184 bits NOTE 8: NPDCCH period bundle is specified in TS 36.523-3 [20] clause 7A.14.2. |  |  |  |  |  |

22.3.1.3.3.3	Specific message contents

Table 22.3.1.3.3.3-1: Void

#### 22.3.1.4	NB-IoT / Correct handling of MAC control information / Buffer status

##### 22.3.1.4.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UL data arrives in the UE transmission buffer and no BSR has been triggered }

then { UE triggers a regular BSR and Reports a Short Buffer Status Reporting (BSR) }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when{RETX_BSR_TIMER expires and UE has pending data for transmission }

then { UE transmits a random access preamble }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { a Regular BSR has been triggered and UE has pending data for transmission and UE has only resources to send either BSR report or partial data }

then { UE transmits the BSR report }

}

(4)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { a Regular BSR has been triggered and UE has pending data for transmission and UE has only UL resources to send all pending data available for transmission, but UL grant is not sufficient to additionally accommodate the BSR MAC control element}

then { UE cancels the triggered BSR report and transmits the UL data}

}

(5)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { a Regular BSR has been triggered and UE has pending data for transmission and UE has UL resources to send all pending data including BSR }

then { UE transmits the UL data and reports buffer status reporting (BSR) that indicates there is no more data in the buffer}

}

(6)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE transmits a MAC PDU and the number of padding bits is equal to or larger than the size of a Short BSR plus its subheader and the UE has available data for transmission form in the TTI where the BSR is transmitted }

then { UE triggers padding BSR and reports a Short BSR }

}

(7)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { periodicBSR-Timer expires and UE has buffered data in a TTI }

then { UE triggers a Periodic BSR and reports Short BSR and restarts the periodicBSR-Timer}

}

##### 22.3.1.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.321, clause 5.4.3.1, 5.4.5, 6.1.2, 6.1.3.1 and 6.2.1. Unless otherwise stated these are Rel-13 requirements.

[TS 36.321 clause 5.4.3.1]

For the Logical Channel Prioritization procedure, the MAC entity shall take into account the following relative priority in decreasing order:

-	MAC control element for C-RNTI or data from UL-CCCH;

-	MAC control element for BSR, with exception of BSR included for padding;

-	MAC control element for PHR, Extended PHR, or Dual Connectivity PHR;

-	MAC control element for Sidelink BSR, with exception of Sidelink BSR included for padding;

-	data from any Logical Channel, except data from UL-CCCH;

-	MAC control element for BSR included for padding;

-	MAC control element for Sidelink BSR included for padding.

NOTE:	When the MAC entity is requested to transmit multiple MAC PDUs in one TTI, steps 1 to 3 and the associated rules may be applied either to each grant independently or to the sum of the capacities of the grants. Also the order in which the grants are processed is left up to UE implementation. It is up to the UE implementation to decide in which MAC PDU a MAC control element is included when MAC entity is requested to transmit multiple MAC PDUs in one TTI. When the UE is requested to generate MAC PDU(s) in two MAC entities in one TTI, it is up to UE implementation in which order the grants are processed.

[TS 36.321 clause 5.4.5]

The Buffer Status reporting procedure is used to provide the serving eNB with information about the amount of data available for transmission in the UL buffers associated with the MAC entity. RRC controls BSR reporting by configuring the three timers periodicBSR-Timer, retxBSR-Timer and logicalChannelSR-ProhibitTimer and by, for each logical channel, optionally signalling logicalChannelGroup which allocates the logical channel to an LCG [8].

For the Buffer Status reporting procedure, the MAC entity shall consider all radio bearers which are not suspended and may consider radio bearers which are suspended.

For NB-IoT the Long BSR is not supported and all logical channels belong to one LCG.

A Buffer Status Report (BSR) shall be triggered if any of the following events occur:

-	UL data, for a logical channel which belongs to a LCG, becomes available for transmission in the RLC entity or in the PDCP entity (the definition of what data shall be considered as available for transmission is specified in [3] and [4] respectively) and either the data belongs to a logical channel with higher priority than the priorities of the logical channels which belong to any LCG and for which data is already available for transmission, or there is no data available for transmission for any of the logical channels which belong to a LCG, in which case the BSR is referred below to as "Regular BSR";

-	UL resources are allocated and number of padding bits is equal to or larger than the size of the Buffer Status Report MAC control element plus its subheader, in which case the BSR is referred below to as "Padding BSR";

-	retxBSR-Timer expires and the MAC entity has data available for transmission for any of the logical channels which belong to a LCG, in which case the BSR is referred below to as "Regular BSR";

-	periodicBSR-Timer expires, in which case the BSR is referred below to as "Periodic BSR".

For Regular BSR:

-	if the BSR is triggered due to data becoming available for transmission for a logical channel for which logicalChannelSR-ProhibitTimer is configured by upper layers:

-	start or restart the logicalChannelSR-ProhibitTimer;

-	else:

-	if running, stop the logicalChannelSR-ProhibitTimer.

For Regular and Periodic BSR:

-	if more than one LCG has data available for transmission in the TTI where the BSR is transmitted: report Long BSR;

-	else report Short BSR.

For Padding BSR:

-	if the number of padding bits is equal to or larger than the size of the Short BSR plus its subheader but smaller than the size of the Long BSR plus its subheader:

-	if more than one LCG has data available for transmission in the TTI where the BSR is transmitted: report Truncated BSR of the LCG with the highest priority logical channel with data available for transmission;

-	else report Short BSR.

-	else if the number of padding bits is equal to or larger than the size of the Long BSR plus its subheader, report Long BSR.

If the Buffer Status reporting procedure determines that at least one BSR has been triggered and not cancelled:

-	if the MAC entity has UL resources allocated for new transmission for this TTI:

-	instruct the Multiplexing and Assembly procedure to generate the BSR MAC control element(s);

-	start or restart periodicBSR-Timer except when all the generated BSRs are Truncated BSRs;

-	start or restart retxBSR-Timer.

-	else if a Regular BSR has been triggered and logicalChannelSR-ProhibitTimer is not running:

-	if an uplink grant is not configured or the Regular BSR was not triggered due to data becoming available for transmission for a logical channel for which logical channel SR masking (logicalChannelSR-Mask) is setup by upper layers:

-	a Scheduling Request shall be triggered.

A MAC PDU shall contain at most one MAC BSR control element, even when multiple events trigger a BSR by the time a BSR can be transmitted in which case the Regular BSR and the Periodic BSR shall have precedence over the padding BSR.

The MAC entity shall restart retxBSR-Timer upon indication of a grant for transmission of new data on any UL-SCH.

All triggered BSRs shall be cancelled in case the UL grant(s) in this TTI can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC control element plus its subheader. All triggered BSRs shall be cancelled when a BSR is included in a MAC PDU for transmission.

The MAC entity shall transmit at most one Regular/Periodic BSR in a TTI. If the MAC entity is requested to transmit multiple MAC PDUs in a TTI, it may include a padding BSR in any of the MAC PDUs which do not contain a Regular/Periodic BSR.

All BSRs transmitted in a TTI always reflect the buffer status after all MAC PDUs have been built for this TTI. Each LCG shall report at the most one buffer status value per TTI and this value shall be reported in all BSRs reporting buffer status for this LCG.

NOTE:	A Padding BSR is not allowed to cancel a triggered Regular/Periodic BSR, except for NB-IoT. A Padding BSR is triggered for a specific MAC PDU only and the trigger is cancelled when this MAC PDU has been built.

[TS 36.321 clause 6.1.2]

A MAC PDU consists of a MAC header, zero or more MAC Service Data Units (MAC SDU), zero, or more MAC control elements, and optionally padding; as described in Figure 6.1.2-3.

Both the MAC header and the MAC SDUs are of variable sizes.

A MAC PDU header consists of one or more MAC PDU subheaders; each subheader corresponds to either a MAC SDU, a MAC control element or padding.

A MAC PDU subheader consists of the five or six header fields R/F2/E/LCID/(F)/L but for the last subheader in the MAC PDU and for fixed sized MAC control elements. The last subheader in the MAC PDU and subheaders for fixed sized MAC control elements consist solely of the four header fields R/F2/E/LCID. A MAC PDU subheader corresponding to padding consists of the four header fields R/F2/E/LCID.

Figure 6.1.2-1: R/F2/E/LCID/F/L MAC subheader

Figure 6.1.2-1a: R/F2/E/LCID/L MAC subheader

Figure 6.1.2-2: R/F2/E/LCID MAC subheader

MAC PDU subheaders have the same order as the corresponding MAC SDUs, MAC control elements and padding.

MAC control elements are always placed before any MAC SDU.

Padding occurs at the end of the MAC PDU, except when single-byte or two-byte padding is required. Padding may have any value and the MAC entity shall ignore it. When padding is performed at the end of the MAC PDU, zero or more padding bytes are allowed.

When single-byte or two-byte padding is required, one or two MAC PDU subheaders corresponding to padding are placed at the beginning of the MAC PDU before any other MAC PDU subheader.

A maximum of one MAC PDU can be transmitted per TB per MAC entity. A maximum of one MCH MAC PDU can be transmitted per TTI.

Figure 6.1.2-3: Example of MAC PDU consisting of MAC header, MAC control elements, MAC SDUs and padding

[TS 36.321 clause 6.1.3.1]

Buffer Status Report (BSR) MAC control elements consist of either:

-	Short BSR and Truncated BSR format: one LCG ID field and one corresponding Buffer Size field (figure 6.1.3.1-1); or

-	Long BSR format: four Buffer Size fields, corresponding to LCG IDs #0 through #3 (figure 6.1.3.1-2).

The BSR formats are identified by MAC PDU subheaders with LCIDs as specified in table 6.2.1-2.

The fields LCG ID and Buffer Size are defined as follow:

-	LCG ID: The Logical Channel Group ID field identifies the group of logical channel(s) which buffer status is being reported. The length of the field is 2 bits;

-	Buffer Size: The Buffer Size field identifies the total amount of data available across all logical channels of a logical channel group after all MAC PDUs for the TTI have been built. The amount of data is indicated in number of bytes. It shall include all data that is available for transmission in the RLC layer and in the PDCP layer; the definition of what data shall be considered as available for transmission is specified in [3] and [4] respectively. The size of the RLC and MAC headers are not considered in the buffer size computation. The length of this field is 6 bits. If extendedBSR-Sizes is not configured, the values taken by the Buffer Size field are shown in Table 6.1.3.1-1. If extendedBSR-Sizes is configured, the values taken by the Buffer Size field are shown in Table 6.1.3.1-2.

Figure 6.1.3.1-1: Short BSR and Truncated BSR MAC control element

Figure 6.1.3.1-2: Long BSR MAC control element

Table 6.1.3.1-1: Buffer size levels for BSR

| Index | Buffer Size (BS) value [bytes] | Index | Buffer Size (BS) value [bytes] |
| --- | --- | --- | --- |
| 0 | BS = 0 | 32 | 1132 < BS <= 1326 |
| 1 | 0 < BS <= 10 | 33 | 1326 < BS <= 1552 |
| 2 | 10 < BS <= 12 | 34 | 1552 < BS <= 1817 |
| 3 | 12 < BS <= 14 | 35 | 1817 < BS <= 2127 |
| 4 | 14 < BS <= 17 | 36 | 2127 < BS <= 2490 |
| 5 | 17 < BS <= 19 | 37 | 2490 < BS <= 2915 |
| 6 | 19 < BS <= 22 | 38 | 2915 < BS <= 3413 |
| 7 | 22 < BS <= 26 | 39 | 3413 < BS <= 3995 |
| 8 | 26 < BS <= 31 | 40 | 3995 < BS <= 4677 |
| 9 | 31 < BS <= 36 | 41 | 4677 < BS <= 5476 |
| 10 | 36 < BS <= 42 | 42 | 5476 < BS <= 6411 |
| 11 | 42 < BS <= 49 | 43 | 6411 < BS <= 7505 |
| 12 | 49 < BS <= 57 | 44 | 7505 < BS <= 8787 |
| 13 | 57 < BS <= 67 | 45 | 8787 < BS <= 10287 |
| 14 | 67 < BS <= 78 | 46 | 10287 < BS <= 12043 |
| 15 | 78 < BS <= 91 | 47 | 12043 < BS <= 14099 |
| 16 | 91 < BS <= 107 | 48 | 14099 < BS <= 16507 |
| 17 | 107 < BS <= 125 | 49 | 16507 < BS <= 19325 |
| 18 | 125 < BS <= 146 | 50 | 19325 < BS <= 22624 |
| 19 | 146 < BS <= 171 | 51 | 22624 < BS <= 26487 |
| 20 | 171 < BS <= 200 | 52 | 26487 < BS <= 31009 |
| 21 | 200 < BS <= 234 | 53 | 31009 < BS <= 36304 |
| 22 | 234 < BS <= 274 | 54 | 36304 < BS <= 42502 |
| 23 | 274 < BS <= 321 | 55 | 42502 < BS <= 49759 |
| 24 | 321 < BS <= 376 | 56 | 49759 < BS <= 58255 |
| 25 | 376 < BS <= 440 | 57 | 58255 < BS <= 68201 |
| 26 | 440 < BS <= 515 | 58 | 68201 < BS <= 79846 |
| 27 | 515 < BS <= 603 | 59 | 79846 < BS <= 93479 |
| 28 | 603 < BS <= 706 | 60 | 93479 < BS <= 109439 |
| 29 | 706 < BS <= 826 | 61 | 109439 < BS <= 128125 |
| 30 | 826 < BS <= 967 | 62 | 128125 < BS <= 150000 |
| 31 | 967 < BS <=1132 | 63 | BS > 150000 |

Table 6.1.3.1-2: Extended Buffer size levels for BSR

| Index | Buffer Size (BS) value [bytes] | Index | Buffer Size (BS) value [bytes] |
| --- | --- | --- | --- |
| 0 | BS = 0 | 32 | 4940 < BS <= 6074 |
| 1 | 0 < BS <= 10 | 33 | 6074 < BS <= 7469 |
| 2 | 10 < BS <= 13 | 34 | 7469 < BS <= 9185 |
| 3 | 13 < BS <= 16 | 35 | 9185 < BS <= 11294 |
| 4 | 16 < BS <= 19 | 36 | 11294 < BS <= 13888 |
| 5 | 19 < BS <= 23 | 37 | 13888 < BS <= 17077 |
| 6 | 23 < BS <= 29 | 38 | 17077 < BS <= 20999 |
| 7 | 29 < BS <= 35 | 39 | 20999 < BS <= 25822 |
| 8 | 35 < BS <= 43 | 40 | 25822 < BS <= 31752 |
| 9 | 43 < BS <= 53 | 41 | 31752 < BS <= 39045 |
| 10 | 53 < BS <= 65 | 42 | 39045 < BS <= 48012 |
| 11 | 65 < BS <= 80 | 43 | 48012 < BS <= 59039 |
| 12 | 80 < BS <= 98 | 44 | 59039 < BS <= 72598 |
| 13 | 98 < BS <= 120 | 45 | 72598 < BS <= 89272 |
| 14 | 120 < BS <= 147 | 46 | 89272 < BS <= 109774 |
| 15 | 147 < BS <= 181 | 47 | 109774 < BS <= 134986 |
| 16 | 181 < BS <= 223 | 48 | 134986 < BS <= 165989 |
| 17 | 223 < BS <= 274 | 49 | 165989 < BS <= 204111 |
| 18 | 274 < BS <= 337 | 50 | 204111 < BS <= 250990 |
| 19 | 337 < BS <= 414 | 51 | 250990 < BS <= 308634 |
| 20 | 414 < BS <= 509 | 52 | 308634 < BS <= 379519 |
| 21 | 509 < BS <= 625 | 53 | 379519 < BS <= 466683 |
| 22 | 625 < BS <= 769 | 54 | 466683 < BS <= 573866 |
| 23 | 769 < BS <= 945 | 55 | 573866 < BS <= 705666 |
| 24 | 945 < BS <= 1162 | 56 | 705666 < BS <= 867737 |
| 25 | 1162 < BS <= 1429 | 57 | 867737 < BS <= 1067031 |
| 26 | 1429 < BS <= 1757 | 58 | 1067031 < BS <= 1312097 |
| 27 | 1757 < BS <= 2161 | 59 | 1312097 < BS <= 1613447 |
| 28 | 2161 < BS <= 2657 | 60 | 1613447 < BS <= 1984009 |
| 29 | 2657 < BS <= 3267 | 61 | 1984009 < BS <= 2439678 |
| 30 | 3267 < BS <= 4017 | 62 | 2439678 < BS <= 3000000 |
| 31 | 4017 < BS <=4940 | 63 | BS > 3000000 |

[TS 36.321 clause 6.2.1]

The MAC header and subheaders are octet aligned.

Table 6.2.1-1: Values of LCID for DL-SCH

| Index | LCID values |
| --- | --- |
| 00000 | CCCH |
| 00001-01010 | Identity of the logical channel |
| 01011-10111 | Reserved |
| 11000 | Activation/Deactivation (4 octets) |
| 11001 | SC-MCCH, SC-MTCH (see note) |
| 11010 | Long DRX Command |
| 11011 | Activation/Deactivation (1 octet) |
| 11100 | UE Contention Resolution Identity |
| 11101 | Timing Advance Command |
| 11110 | DRX Command |
| 11111 | Padding |
| NOTE: Both SC-MCCH and SC-MTCH cannot be multiplexed with other logical channels in the same MAC PDU except for Padding |  |

For NB-IoT only the following LCID values for DL-SCH are applicable: CCCH, Identity of the logical channel, UE Contention Resolution Identity, Timing Advance Command, DRX Command and Padding.

Table 6.2.1-2: Values of LCID for UL-SCH

| Index | LCID values |
| --- | --- |
| 00000 | CCCH |
| 00001-01010 | Identity of the logical channel |
| 01011 | CCCH |
| 01100-10101 | Reserved |
| 10110 | Truncated Sidelink BSR |
| 10111 | Sidelink BSR |
| 11000 | Dual Connectivity Power Headroom Report |
| 11001 | Extended Power Headroom Report |
| 11010 | Power Headroom Report |
| 11011 | C-RNTI |
| 11100 | Truncated BSR |
| 11101 | Short BSR |
| 11110 | Long BSR |
| 11111 | Padding |

For NB-IoT only the following LCID values for UL-SCH are applicable: CCCH (LCID "00000"), Identity of the logical channel, C-RNTI, Short BSR and Padding.

Table 6.2.1-3: Values of F and F2 fields:

| Index of F2 | Index of F | Size of Length field (in bits) |
| --- | --- | --- |
| 0 | 0 | 7 |
|  | 1 | 15 |
| 1 | - | 16 |

Table 6.2.1-4: Values of LCID for MCH

| Index | LCID values |
| --- | --- |
| 00000 | MCCH (see note) |
| 00001-11100 | MTCH |
| 11101 | Reserved |
| 11110 | MCH Scheduling Information or Extended MCH Scheduling Information |
| 11111 | Padding |
| NOTE: If there is no MCCH on MCH, an MTCH could use this value. |  |

##### 22.3.1.4.3	Test description

22.3.1.4.3.1	Pre-test conditions

System Simulator:

-	Ncell 1

-	RRC Connection Setup (preamble: Table 8.1.5.2.3-1, step 3 [18]) using parameters as specified in Table 22.3.1.4.3.3-1

UE:

None.

Preamble:

-	The UE is in Loopback Activated State 2B-NB with test loop mode G according to [18] on Ncell 1.

22.3.1.4.3.2	Test procedure sequence

Table 22.3.1.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Void | - | - | - | - |
| 2 | The SS transmits a MAC PDU containing two RLC SDUs of RLC UL SDU size 10 bytes. (Note 0) | <-- | MAC PDU (2 RLC SDUs) | - | - |
| 3 | In the third (next for NTN) NPDCCH period bundle after DL transmission of step 2 the SS allocates an UL Grant of 32 bits. (Note 1, Note 8) | <-- | (UL Grant, 32 bits) | - | - |
| 4 | Check: Does the UE transmit a Short BSR with ‘Buffer size’ field set to value ‘6’ or bigger? (Note 2) | --> | MAC PDU (MAC Short BSR (Buffer Size=’6’ or bigger)) | 1,3 | P |
| 5 | Wait for retxBSR-Timer expiry on UE side. | - | - | - | - |
| 5A | Check: Does the UE transmit a preamble on NPRACH? | --> | NPRACH Preamble | 2 | P |
| 5B | The SS transmits a Random Access Response containing a matching RAPID. (Note 7) | <-- | Random Access Response | - | - |
| 5C | The UE transmit a MAC PDU containing C-RNTI, BSR and a portion of loop backed first RLC SDU. (Note 2a) | --> | MAC PDU(C-RNTI, MAC Short BSR(Buffer Size=’4’ or bigger), portion of loop backed first RLC SDU) | - | - |
| 6 | The SS allocates an UL Grant of 176 bits before mac-ContentionResolutionTimer expiry. (Notes 2b, 7) | <-- | (UL Grant, 176 bits) | - | - |
| 7 | Check: Does the UE transmit a MAC PDU including a portion of loop backed first RLC SDU and second RLC SDUs? (Note 2b) | --> | MAC PDU (RLC AMD PDU with header, 2 RLC SDU) |  |  |
| 7A | SS transmits an RLC STATUS PDU to acknowledge correctly received data. | <-- | RLC STATUS PDU (ACK_SN=1) | - | - |
| 8 | The SS transmits a MAC PDU containing one RLC UL SDU of size 10 bytes. (Note 0) | <-- | MAC PDU (1 RLC SDU) | - | - |
| 9 | In the third (next for NTN) NPDCCH period bundle after DL transmission of step 8 the SS allocates an UL Grant of 32 bits. (Note 1, Note 8) | <-- | (UL Grant, 32 bits) | - | - |
| 10 | Check: Does the UE transmit a Short BSR with Buffer size#1’ field set to value '1’ or bigger? (Note 2) | --> | MAC PDU (MAC Short BSR (Buffer Size=’1’ or bigger)) | 3 | P |
| 10A | In the third (next for NTN) NPDCCH period bundle after the transmission at step 9 the SS allocates an UL Grant of 104 bits. (Note 2c, Note 8) | <-- | (UL Grant, 104 bits) | - | - |
| 10B | Check: Does the UE transmit a MAC PDU including the RLC SDU sent at step 8? (Note 2c) | --> | MAC PDU (SDU subheader, RLC AMD PDU with header, 1 RLC SDU) | - | - |
| 10C | SS transmits an RLC STATUS PDU to acknowledge correctly received data | <-- | RLC STATUS PDU (ACK_SN=2) | - | - |
| 11 | The SS transmits a MAC PDU containing three RLC UL SDUs of size 10 bytes. (Note 0) | <-- | MAC PDU (3 RLC SDUs) | - | - |
| 12 | In the third (next for NTN) NPDCCH period bundle after the transmission at step 11 the SS allocates an UL Grant of 296 bits. (Note 3, Note 8) | <-- | (UL Grant, 296 bits) | - | - |
| 13 | Check: Does the UE transmit a MAC PDU including three RLC SDUs and not including any BSR? (Note 3) | --> | MAC PDU (padding, SDU subheader, RLC AMD PDU with header, 2 LIs, 3 RLC SDUs) | 4 | P |
| 14 | SS transmits an RLC STATUS PDU to acknowledge correctly received data | <-- | RLC STATUS PDU (ACK_SN=3) | - | - |
| 15 | The SS transmits a MAC PDU containing two MAC SDUs, the first containing a 8 byte RLC UL SDU and the second containing a 7 byte RLC UL SDU. (Note 0) | <-- | MAC PDU | - | - |
| 16 | In the third (next for NTN) NPDCCH period bundle after DL transmission of step 15 the SS allocates an uplink grant of size 256 bits. (Note 4, Note 8) | <-- | (UL grant, 256 bits) | - | - |
| 17 | Check: Does the UE return a MAC PDU of length 256 bits including 2 RLC SDUs, Padding and Short BSR with Buffer size(s) set to ‘0’? (Note 4) | --> | MAC PDU (Short BSR subheader, SDU subheader, padding subheader, short BSR, RLC AMD PDU with header, 1 LI, 2 RLC SDUs, padding) | 5 | P |
| 18 | SS transmits an RLC STATUS PDU to acknowledge correctly received data | <-- | RLC STATUS PDU (ACK_SN=4) | - | - |
| 19 | The SS transmits a MAC PDU including an RLC SDU of RLC UL SDU size 12 bytes. (Note 0) | <-- | MAC PDU | - | - |
| 20 | In the third (next for NTN) NPDCCH period bundle after DL transmission of step 19 the SS transmits an uplink grant of size 136 bits. (Note 4a, Note 8) | <-- | (UL grant, 136 bits) | - | - |
| 21 | Check: Does UE transmit a MAC PDU containing an RLC SDU and with a Short BSR indicating pending data (‘Buffer size’ field = ‘0’) for ‘LCG ID’ field set to ‘0’? (Note 4a) | --> | MAC PDU (Short BSR header, Short BSR(LCG ID=’00’, Buffer size=’0’), MAC SDU) | 6 | P |
| 21A | SS transmits an RLC STATUS PDU to acknowledge correctly received data. | <-- | RLC STATUS PDU (ACK_SN=3) |  |  |
| 21B | The SS transmits an RRCConnectionRelease-NB message. | <-- | RRC: RRCConnectionRelease-NB |  |  |
| 21C | 'Generic Test Procedure NB-IoT Control Plane CIoT MT user data transfer non-SMS transport' as described in TS 36.508 [18], clause 8.1.5A.2.3 are performed | - | - | - | - |
| 22 | The SS transmits a MAC PDU containing an RLC PDU, which contains 1 RLC UL SDU of size 14 bytes. (Note 0) | <-- | MAC PDU (RLC PDU) | - | - |
| 23 | The SS sends an uplink grant of size 32 bits. | <-- | (UL grant, 32 bits) | - | - |
| 24 | The UE transmits a short BSR report. (Note 6) | --> | MAC PDU (Short BSR header, Short BSR(LCG ID='00', Buffer size index > 0)) | - | - |
| - | EXCEPTION: Steps 25 to 27 shall be repeated two times (Note 5). | - | - | - | - |
| 25 | Wait for periodicBSR-Timer expiry. | - | - | - | - |
| 26 | In the next NPDCCH period bundle after periodicBSR -Timer expiry the SS sends an uplink grant of size 32 bits (Note 1, Note 8) | - | (UL Grant, 32 bits) | - | - |
| 27 | Check: Does UE transmit a MAC PDU containing a Short BSR with ‘LCG ID’ field set to ‘00’ and Buffer Size Index > 0? | --> | MAC PDU (Short BSR header, Short BSR(LCG ID=’00’, Buffer Size index > 0)) | 7 | P |
| Note 0: RLC SDU with size n contains a DLInformationTransfer-NB with ESM_DATA_TRANSPORT: 18 or 26 bits DLInformationTransfer-NB(PER encode: ‘0000000000’B + 8 bits ( length of DedicatedInfoNAS <= 127 bytes ) or 16 bits length ( 128 bytes <= length of DedicatedInfoNAS < 16383 bytes )) 1/2 byte Protocol discriminator ( SECURITY PROTECTED NAS MESSAGE ) 1/2 byte Security header type ( SECURITY PROTECTED NAS MESSAGE ) 4 bytes Message authentication code ( SECURITY PROTECTED NAS MESSAGE ) 1 byte Sequence number ( SECURITY PROTECTED NAS MESSAGE ) 1/2 byte Protocol discriminator ( ESM_DATA_TRANSPORT ) 1/2 byte EPS bearer identity ( ESM_DATA_TRANSPORT ) 1 byte Procedure transaction identity ( ESM_DATA_TRANSPORT ) 1 byte ESM data transport message identity ( ESM_DATA_TRANSPORT ) 2 bytes length indicator of the user data container ( ESM_DATA_TRANSPORT ) N bytes User data container contents ( UL RLC SDU ) 6 bits Aligned bits ( PER encode ) n = N+14 or N+15 bytes DL RLC SDU Note 1: 32 bits enables UE to transmit a MAC PDU with a MAC BSR header and a Short BSR (1 bytes). Note 2: UE triggers a Short BSR of type "Regular BSR" to report buffer status for one LCG for that TTI. The UE should not send any of the received RLC SDUs (segmented) due to Regular BSR has higher priority than U-plane logical channels. Note 2a: 88 bits for msg3 enable UE to transmit the CRNTI, BSR and user data (received in step 2), where 3 bytes belong first RLC SDU with FI=01 in its AMD PUD header, in a MAC PDU with 3 bytes MAC subheader: 1 byte CRNTI, 1 byte BSR, 1 byte SDU subheader 3 bytes MAC CE: 2 bytes CRNTI, 1 byte BSR 5 bytes MAC SDU: AMD PDU with 2 bytes header and a portion of 1 RLC SDU (3 bytes) 11 bytes (TBS = 88) Note 2b: The UE has 17 bytes of user data (received in step 2) in the transmission buffer, where 17 bytes belong RLC SDU with FI=10 in its AMD PUD header. 176 bits enables UE to transmit the user data in a MAC PDU with 1 byte MAC subheader: 1 byte SDU subheader 21 bytes MAC SDU: AMD PDU with 4 bytes header and a portion of 1 RLC SDU (7 bytes) and 1 RLC SDUs (10 bytes) 22 bytes (TBS = 176)Note 2c: The UE has 10 bytes of user data (received in step 8) in the transmission buffer. 104 bits enables UE to transmit the user data in a MAC PDU with 1 byte MAC subheader: 1 byte SDU subheader 12 bytes MAC SDU: AMD PDU with 2 bytes header and 1 RLC SDU (10 bytes) 13 bytes (TBS =104) Note 3: The UE has 30 bytes of user data (received in steps 11) in the transmission buffer. 296 bits enables UE to transmit the user data in a MAC PDU with 2 bytes MAC subheader: 1 byte padding, 1 byte SDU subheader 35 bytes MAC SDU: AMD PDU with 5 bytes header (2 LIs) and 3 RLC SDUs (10 bytes each) 37 bytes (TBS = 296) Note 4: The UE has 15 bytes of user data (received in steps 15) in the transmission buffer. 256 bits enables UE to transmit the user data in a MAC PDU with 4 bytes MAC subheader (1 byte BSR subheader, 2 bytes SDU subheader, 1 byte paddingr) 1 byte short BSR 19 bytes MAC SDU: AMD PDU with 4 bytes header (1 LI) and 2 RLC SDUs (7 and 8 bytes) 8 bytes padding 32 bytes (TBS=256). Note 4a: The UE has 12 bytes of user data (received in steps 19) in the transmission buffer. 136 bits enables UE to transmit the user data in a MAC PDU with 2 bytes MAC subheader: 1 byte BSR subheader, 1 byte SDU subheader 1 byte short BSR 14 bytes MAC SDU: AMD PDU with 2 byte header and 12 bytes RLC SDU 17 bytes (TBS=136). Note 5: One short BSR due to first expiry of periodicBSR-Timer and one short BSR due to second expire of periodicBSR-Timer. Note 6: The UE starts periodicBSR-Timer. Note 7: The value of T-CRNTI and CRNTI are different at step 5B/6. Note 8: NPDCCH period bundle is specified in TS 36.523-3 [20] clause 7A.14.2. |  |  |  |  |  |

22.3.1.4.3.3	Specific Message Contents

Table 22.3.1.4.3.3-1: RRCConnectionSetup-NB (Preamble)

| Derivation path: 36.508 table 8.1.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetup-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetup-r13 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated-r13 SEQUENCE { |  |  |  |
| mac-MainConfig-r13 CHOICE { |  |  |  |
| explicitValue-r13 SEQUENCE { |  |  |  |
| ul-SCH-Config-r13 SEQUENCE { |  |  |  |
| periodicBSR-Timer-r13 | pp64 |  |  |
| retxBSR-Timer-r13 | pp16 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.1.4.3.3-2: RRCConnectionSetup-NB (Step 21C, Table 22.3.1.4.3.2-1)

| Derivation path: 36.508 table 8.1.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetup-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetup-r13 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated-r13 SEQUENCE { |  |  |  |
| mac-MainConfig-r13 CHOICE { |  |  |  |
| explicitValue-r13 SEQUENCE { |  |  |  |
| ul-SCH-Config-r13 SEQUENCE { |  |  |  |
| periodicBSR-Timer-r13 | pp16 |  |  |
| retxBSR-Timer-r13 | pp64 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### [Sections skipped here]

#### 22.3.1.6	NB-IoT / DL-SCH / UL-SCH transport block size selection / DCI format N1/ N0

##### 22.3.1.6.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE on NPDCCH receives DCI format N1 indicating a resource assignment corresponding to  number of subframes and a modulation and coding scheme  }

then { UE decodes the received transport block of size corresponding to the read  and  and forwards it to higher layers }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE has pending data for transmission and receives a Resource Block Assignment corresponding to  number of subframes and a modulation and coding scheme  for NPUSCH scheduling }

then { UE transmits MAC PDU on NPUSCH on the granted resources using a transport block size corresponding to the read  and  }

##### 22.3.1.6.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.212, clause 6.4.3.1, 6.4.3.2; TS 36.213, clauses 16.4.3.1, 16.4.1.5, 16.4.1.5.1; and TS 36.306 clause 4.1C.

[TS 36.212, clause 6.4.3.1]

DCI format N0 is used for the scheduling of NPUSCH in one UL cell.

The following information is transmitted by means of the DCI format N0:

-	Flag for format N0/format N1 differentiation – 1 bit, where value 0 indicates format N0 and value 1 indicates format N1

-	Subcarrier indication – 6 bits as defined in section 16.5.1.1 of [3]

-	Resource assignment – 3 bits as defined in section 16.5.1.2 of [3]

-	Scheduling delay – 2 bits as defined in section 16.5.1 of [3]

-	Modulation and coding scheme – 4 bits as defined in section 16.5.1.2 of [3]

-	Redundancy version – 1 bit as defined in section 16.5.1.2 of [3]

-	Repetition number – 3 bits as defined in section 16.5.1.2 of [3]

-	New data indicator – 1 bit

-	DCI subframe repetition number – 2 bits as defined in section 16.6 in [3]

[TS 36.212, clause 6.4.3.2]

DCI format N1 is used for the scheduling of one NPDSCH codeword in one cell and random access procedure initiated by a NPDCCH order. The DCI corresponding to a NPDCCH order is carried by NPDCCH.

The following information is transmitted by means of the DCI format N1:

-	Flag for format N0/format N1 differentiation – 1 bit, where value 0 indicates format N0 and value 1 indicates format N1

-	NPDCCH order indicator – 1 bit

Format N1 is used for random access procedure initiated by a NPDCCH order only if NPDCCH order indicator is set to ‘1’, format N1 CRC is scrambled with C-RNTI, and all the remaining fields are set as follows:

-	Starting number of NPRACH repetitions – 2 bits as defined in section 16.3.1 of [3]

-	Subcarrier indication of NPRACH – 6 bits as defined in section 16.3.1 of [3]

-	All the remaining bits in format N1 are set to one

Otherwise,

-	Scheduling delay – 3 bits as defined in section 16.4.1 of [3]

-	Resource assignment – 3 bits as defined in section 16.4.1.3 of [3]

-	Modulation and coding scheme – 4 bits as defined in section 16.4.1.5 of [3]

-	Repetition number – 4 bits as defined in section 16.4.1.3 of [3]

-	New data indicator – 1 bit

-	HARQ-ACK resource – 4 bits as defined in section 16.4.2 of [3].

-	DCI subframe repetition number – 2 bits as defined in section 16.6 in [3]

When the format N1 CRC is scrambled with a RA-RNTI, then the following fields among the fields above are reserved:

-	New data indicator

-	HARQ-ACK resource

If the number of information bits in format N1 is less than that of format N0, zeros shall be appended to format N1 until the payload size equals that of format N0.

[TS 36.306, clause 4.1C]

The field ue-Category-NB defines a combined uplink and downlink capability in NB-IoT. The parameters set by the UE Category are defined in subclause 4.2. Tables 4.1C-1 and 4.1C-2 define the downlink and, respectively, uplink physical layer parameter values for each UE Category.

Table 4.1C-1: Downlink physical layer parameter values set by the field ue-Category-NB

| UE Category | Maximum number of DL-SCH transport block bits received within a TTI | Maximum number of bits of a DL-SCH transport block received within a TTI | Total number of soft channel bits |
| --- | --- | --- | --- |
| Category NB1 | 680 | 680 | 2112 |

Table 4.1C-2: Uplink physical layer parameter values set by the field ue-Category-NB

| UE Category | Maximum number of UL-SCH transport block bits transmitted within a TTI | Maximum number of bits of an UL-SCH transport block transmitted within a TTI |
| --- | --- | --- |
| Category NB1 | 1000 | 1000 |

[TS 36.213, clause 16.4.1.3]

The resource allocation information in DCI format N1, N2 (paging) for NPDSCH indicates to a scheduled UE

-	a number of subframes () determined by the resource assignment field () in the corresponding DCI according to Table 16.4.1.3-1.

-	a repetition number () determined by the repetition number field () in the corresponding DCI according to Table 16.4.1.3-2.

Table 16.4.1.3-1: Number of subframes () for NPDSCH

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |
| 4 | 5 |
| 5 | 6 |
| 6 | 8 |
| 7 | 10 |

Table 16.4.1.3-2: Number of repetitions () for NPDSCH

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |
| 8 | 192 |
| 9 | 256 |
| 10 | 384 |
| 11 | 512 |
| 12 | 768 |
| 13 | 1024 |
| 14 | 1536 |
| 15 | 2048 |

The number of repetitions for the NPDSCH carrying SystemInformationBlockType1-NB is determined based on the parameter schedulingInfoSIB1 configured by higher-layers and according to Table 16.4.1.3-3.

[TS 36.213, clause 16.4.1.5]

The UE shall use modulation order, = 2.

To determine the transport block size in the NPDSCH, the UE shall first,

-	if NPDSCH carries SystemInformationBlockType1-NB

-	set  to the value of the parameter schedulingInfoSIB1 configured by higher-layers

-	otherwise

-	read the 4-bit "modulation and coding scheme" field () in the DCI and set .

and second,

-	if NPDSCH carries SystemInformationBlockType1-NB

-	use subclause 16.4.1.5.2 for determining its transport block size.

-	otherwise,

-	read the 3-bit "resource assignment" field () in the DCI and determine its TBS by the procedure in subclause 16.4.1.5.1.

The NDI as signalled on NPDCCH, and the TBS, as determined above, shall be delivered to higher layers.

[TS 36.213, clause 16.4.1.5.1]

The TBS is given by the (,) entry of Table 16.4.1.5.1-1. For the value of the higher layer parameter operationModeInfo set to ‘00’ or ‘01’, .

Table 16.4.1.5.1-1: Transport block size (TBS) table

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 |  |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 |  |  |
| 7 | 104 | 224 | 328 | 472 | 584 | 680 |  |  |
| 8 | 120 | 256 | 392 | 536 | 680 |  |  |  |
| 9 | 136 | 296 | 456 | 616 |  |  |  |  |
| 10 | 144 | 328 | 504 | 680 |  |  |  |  |
| 11 | 176 | 376 | 584 |  |  |  |  |  |
| 12 | 208 | 440 | 680 |  |  |  |  |  |

[TS 36.213, clause 16.5.1.1]

The resource allocation information in uplink DCI format N0 for NPUSCH transmission indicates to a scheduled UE

- a set of contiguously allocated subcarriers () of a resource unit determined by the Subcarrier indication field in the corresponding DCI,

- a number of resource units () determined by the resource assignment field in the corresponding DCI according to Table 16.5.1.1-2,

- a repetition number () determined by the repetition number field in the corresponding DCI according to Table 16.5.1.1-3.

The subcarrier spacing  of NPUSCH transmission is determined by the uplink subcarrier spacing field in the Narrowband Random Access Response Grant according to subclause 16.3.3.

For NPUSCH transmission with subcarrier spacing, where  is the subcarrier indication field in the DCI. is reserved.

For NPUSCH transmission with subcarrier spacing, the subcarrier indication field () in the DCI determines the set of contiguously allocated subcarriers () according to Table 16.5.1.1-1.

Table 16.5.1.1-1: Allocated subcarriers for NPUSCH with

| Subcarrier indication field () | Set of Allocated subcarriers () |
| --- | --- |
| 0 – 11 |  |
| 12-15 |  |
| 16-17 |  |
| 18 |  |
| 19-63 | Reserved |

Table 16.5.1.1-2: Number of resource units () for NPUSCH

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |
| 4 | 5 |
| 5 | 6 |
| 6 | 8 |
| 7 | 10 |

Table 16.5.1.1-3: Number of repetitions () for NPUSCH.

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |

[TS 36.213, clause 16.5.1.2]

To determine the modulation order, redundancy version and transport block size for the NPUSCH, the UE shall first

-	read the “modulation and coding scheme” field () in the DCI, and

-	read the “redundancy version” field () in the DCI, and

-	read the "resource assignment" field () in the DCI, and

-	compute the total number of allocated subcarriers (), number of resource units (), and repetition number () according to subclause 16.5.1.1.

The UE shall use modulation order, = 2 if . The UE shall useand Table 16.5.1.2-1 to determine the modulation order to use for NPUSCH if .

Table 16.5.1.2-1: Modulation and TBS index table for NPUSCH with

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 2 |
| 2 | 2 | 1 |
| 3 | 2 | 3 |
| 4 | 2 | 4 |
| 5 | 2 | 5 |
| 6 | 2 | 6 |
| 7 | 2 | 7 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |

NPUSCH is transmitted in N consecutive NB-IoT UL slots, ni , i=0,1,…,N-1. The redundancy version  of the NPUSCH transmission in jth block of B consecutive NB-IoT UL slots ni , is determined by, , where  if ,  otherwise. Portion of NPUSCH codeword with  as defined in clause 6.3.2 in [4] mapped to slot  of allocated  resource unit(s) is transmitted in NB-IoT UL slots ni. for and   for .

The UE shall use (,) and Table 16.5.1.2-2 to determine the TBS to use for the NPUSCH. is given in Table 16.5.1.2.1-1 if ,  otherwise.

Table 16.5.1.2-2: Transport block size (TBS) table for NPUSCH

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 |  |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 |  |  |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 |  |  |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 |  |  |
| 11 | 176 | 376 | 584 | 776 | 1000 |  |  |  |
| 12 | 208 | 440 | 680 | 1000 |  |  |  |  |

Table 16.4.1.5.1-1: Transport block size (TBS) table

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 |  |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 |  |  |
| 7 | 104 | 224 | 328 | 472 | 584 | 680 |  |  |
| 8 | 120 | 256 | 392 | 536 | 680 |  |  |  |
| 9 | 136 | 296 | 456 | 616 |  |  |  |  |
| 10 | 144 | 328 | 504 | 680 |  |  |  |  |
| 11 | 176 | 376 | 584 |  |  |  |  |  |
| 12 | 208 | 440 | 680 |  |  |  |  |  |

The NDI as signalled on NPDCCH, and the RV and TBS, as determined above, shall be delivered to higher layers.

##### 22.3.1.6.3	Test description

22.3.1.6.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

-	MasterInformationBlock-NB / MasterInformationBlock-TDD-NB using parameters as specified in TS 36.508 [18] Table 8.1.4.3.2-1 with condition "Standalone"
NOTE: According to TS 36.213 [30] clause 16.4.1.5.1 for Inband Operation Mode ITBS is restricted to 0 ≤ ITBS ≤ 10 i.e. ISF/IMCS combinations with IMCS = 11, 12 could not be tested

UE:

None.

Preamble:

-	UE is in state NB-IoT UE Attach, Connected Mode, UE Test Loopback Activated (State 2B-NB) with test loop mode G and according to [18] in Ncell 1.

22.3.1.6.3.2	Test procedure sequence

Table 22.3.1.6.3.2-1: RLC SDU size used as test data

| TBSUL [bits] | UL RLC SDU size [bits] |
| --- | --- |
| 56 < TBsize ≤1000 (Note) | TBSUL – 56 (Note) |
| Note: UL MAC PDU contains 2 MAC SDUs (RLC STATUS PDU, RLC AMD PDU with a single RLC SDU): 2 bytes R/R/E/LCID/L header for 1st MAC SDU 1 byte R/R/E/LCID header for 2nd MAC SDU 2 bytes RLC STATUS PDU 2 bytes RLC AMD header N bytes UL RLC SDU N = (TBSUL – 56) / 8 |  |

Table 22.3.1.6.3.2-2: TB sizes and ISF/ITBS combinations used for UL and DL

|  | DL |  |  | UL |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| S.No | ISF | IMCS = ITBS | TBSDL | IRU | IMCS / ITBS | TBSUL |
| 1 | 0 | 0 | 16 | 0 | 0/0 | 16 |
| 2 | 0 | 1 | 24 | 0 | 2/1 | 24 |
| 3 | 0 | 2 | 32 | 0 | 1/2 | 32 |
| 4 | 0 | 3 | 40 | 0 | 3/3 | 40 |
| 5 | 0 | 4 | 56 | 0 | 4/4 | 56 |
| 6 | 0 | 5 | 72 | 0 | 5/5 | 72 |
| 7 | 0 | 6 | 88 | 0 | 6/6 | 88 |
| 8 | 0 | 7 | 104 | 0 | 7/7 | 104 |
| 9 | 0 | 8 | 120 | 0 | 8/8 | 120 |
| 10 | 0 | 9 | 136 | 0 | 9/9 | 136 |
| 11 | 0 | 10 | 144 | 0 | 10/10 | 144 |
| 12 | 0 | 11 | 176 | 0 | 10/10 | 144 |
| 13 | 0 | 12 | 208 | 0 | 10/10 | 144 |
| 14 | 1 | 0 | 32 | 1 | 0/0 | 32 |
| 15 | 1 | 1 | 56 | 1 | 2/1 | 56 |
| 16 | 1 | 2 | 72 | 1 | 1/2 | 72 |
| 17 | 1 | 3 | 104 | 1 | 3/3 | 104 |
| 18 | 1 | 4 | 120 | 1 | 4/4 | 120 |
| 19 | 1 | 5 | 144 | 1 | 5/5 | 144 |
| 20 | 1 | 6 | 176 | 1 | 6/6 | 176 |
| 21 | 1 | 7 | 224 | 1 | 7/7 | 224 |
| 22 | 1 | 8 | 256 | 1 | 8/8 | 256 |
| 23 | 1 | 9 | 296 | 1 | 9/9 | 296 |
| 24 | 1 | 10 | 328 | 1 | 10/10 | 328 |
| 25 | 1 | 11 | 376 | 1 | 10/10 | 328 |
| 26 | 1 | 12 | 440 | 1 | 10/10 | 328 |
| 27 | 2 | 0 | 56 | 2 | 0/0 | 56 |
| 28 | 2 | 1 | 88 | 2 | 2/1 | 88 |
| 29 | 2 | 2 | 144 | 2 | 1/2 | 144 |
| 30 | 2 | 3 | 176 | 2 | 3/3 | 176 |
| 31 | 2 | 4 | 208 | 2 | 4/4 | 208 |
| 32 | 2 | 5 | 224 | 2 | 5/5 | 224 |
| 33 | 2 | 6 | 256 | 2 | 6/6 | 256 |
| 34 | 2 | 7 | 328 | 2 | 7/7 | 328 |
| 35 | 2 | 8 | 392 | 2 | 8/8 | 392 |
| 36 | 2 | 9 | 456 | 2 | 9/9 | 456 |
| 37 | 2 | 10 | 504 | 2 | 10/10 | 504 |
| 38 | 2 | 11 | 584 | 2 | 10/10 | 504 |
| 39 | 2 | 12 | 680 | 2 | 10/10 | 504 |
| 40 | 3 | 0 | 88 | 3 | 0/0 | 88 |
| 41 | 3 | 1 | 144 | 3 | 2/1 | 144 |
| 42 | 3 | 2 | 176 | 3 | 1/2 | 176 |
| 43 | 3 | 3 | 208 | 3 | 3/3 | 208 |
| 44 | 3 | 4 | 256 | 3 | 4/4 | 256 |
| 45 | 3 | 5 | 328 | 3 | 5/5 | 328 |
| 46 | 3 | 6 | 392 | 3 | 6/6 | 392 |
| 47 | 3 | 7 | 472 | 3 | 7/7 | 472 |
| 48 | 3 | 8 | 536 | 3 | 8/8 | 536 |
| 49 | 3 | 9 | 616 | 3 | 9/9 | 616 |
| 50 | 3 | 10 | 680 | 3 | 10/10 | 680 |
| 51 | 4 | 0 | 120 | 4 | 0/0 | 120 |
| 52 | 4 | 1 | 176 | 4 | 2/1 | 176 |
| 53 | 4 | 2 | 208 | 4 | 1/2 | 208 |
| 54 | 4 | 3 | 256 | 4 | 3/3 | 256 |
| 55 | 4 | 4 | 328 | 4 | 4/4 | 328 |
| 56 | 4 | 5 | 424 | 4 | 5/5 | 424 |
| 57 | 4 | 6 | 504 | 4 | 6/6 | 504 |
| 58 | 4 | 7 | 584 | 4 | 7/7 | 584 |
| 59 | 4 | 8 | 680 | 4 | 8/8 | 680 |
| 60 | 4 | 8 | 680 | 4 | 9/9 | 776 |
| 61 | 4 | 8 | 680 | 4 | 10/10 | 872 |
| 62 | 5 | 0 | 152 | 5 | 0/0 | 152 |
| 63 | 5 | 1 | 208 | 5 | 2/1 | 208 |
| 64 | 5 | 2 | 256 | 5 | 1/2 | 256 |
| 65 | 5 | 3 | 328 | 5 | 3/3 | 328 |
| 66 | 5 | 4 | 408 | 5 | 4/4 | 408 |
| 67 | 5 | 5 | 504 | 5 | 5/5 | 504 |
| 68 | 5 | 6 | 600 | 5 | 6/6 | 600 |
| 69 | 5 | 7 | 680 | 5 | 7/7 | 712 |
| 70 | 5 | 7 | 680 | 5 | 8/8 | 808 |
| 71 | 5 | 7 | 680 | 5 | 9/9 | 936 |
| 72 | 5 | 7 | 680 | 5 | 10/10 | 1000 |
| 73 | 6 | 0 | 208 | 6 | 0/0 | 208 |
| 74 | 6 | 1 | 256 | 6 | 2/1 | 256 |
| 75 | 6 | 2 | 328 | 6 | 1/2 | 328 |
| 76 | 6 | 3 | 440 | 6 | 3/3 | 440 |
| 77 | 6 | 4 | 552 | 6 | 4/4 | 552 |
| 78 | 6 | 5 | 680 | 6 | 5/5 | 680 |
| 79 | 6 | 5 | 680 | 6 | 6/6 | 808 |
| 80 | 6 | 5 | 680 | 6 | 7/7 | 1000 |
| 81 | 7 | 0 | 256 | 7 | 0/0 | 256 |
| 82 | 7 | 1 | 344 | 7 | 2/1 | 344 |
| 83 | 7 | 2 | 424 | 7 | 1/2 | 424 |
| 84 | 7 | 3 | 568 | 7 | 3/3 | 568 |
| 85 | 7 | 4 | 680 | 7 | 4/4 | 680 |
| 86 | 7 | 4 | 680 | 7 | 5/5 | 872 |
| 87 | 7 | 4 | 680 | 7 | 6/6 | 1000 |

Table 22.3.1.6.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | Void. | - | - | - | - |
| - | EXCEPTION: Steps 1 to 8 are repeated as per table 22.3.1.6.3.2-2 with values of ISF, DL IMCS, IRU and UL IMCS | - | - | - | - |
| 2 | SS looks up for TBSUL in table 22.3.1.6.3.2-1 as per the execution counter | - | - | - | - |
| - | EXCEPTION: Steps 3 to 6 are performed for TBSUL>56 | - | - | - | - |
| 3 | SS creates one UL RLC SDU of size TBSUL, embeds it in an ESM DATA TRANSPORT and DLInformationTransfer-NB and splits the resulting DL RLC SDU into three RLC AMD PDUs ( if TBSDL < 100 bits ) or two equal sized RLC AMD PDUs ( if TBSDL > 100 bits ) (NOTE 1) | - | - | - | - |
| 3A | SS gets current timing and NPDCCH period N is the next NPDCCH period bundle which can be used by the SS for a DL transmission. (NOTE 3) | - | - | - | - |
| 4 | At NPDCCH period bundle N+1 the SS sends a DCI format N1 and a MAC PDU containing an RLC AMD PDU with polling field ‘P’ set to '0' and containing the 1st AMD PDU of the RLC SDU created at step 3. (NOTE 3) | <-- | DCI N1 format with ISF, DL IMCS MAC PDU with 1st AMD PDU of the DL RLC SDU | - | - |
| 4A | IF TBSDL < 100 bits At NPDCCH period bundle N+2 the SS sends a DCI format N1 and a MAC PDU containing an RLC AMD PDU with polling field ‘P’ set to '0' and containing the 2nd AMD PDU of the RLC SDU created at step 3. (NOTE 3) | <-- | DCI N1 format with ISF, DL IMCS MAC PDU with 2nd AMD PDU of the DL RLC SDU | - | - |
| 4B | At NPDCCH period bundle N+i the SS sends a DCI format N1 and a MAC PDU containing an RLC AMD PDU with polling field ‘P’ set to '1' and containing the last AMD PDU of the RLC SDU created at step 3. Where i=3 if TBSDL < 100 bits, otherwise i=2. (NOTE 2, (NOTE 3)) | <-- | DCI N1 format with ISF, DL IMCS MAC PDU with last AMD PDU of the DL RLC SDU | - | - |
| 5 | In the NPDCCH period bundle N+3 (N+1 for NTN) after the transmission at step 4B the SS allocates an uplink grant for the UL TBS according to 22.3.1.6.3.2-2. (NOTE3) | <-- | (UL Grant) DCI N0 format with IRU and UL IMCS | - | - |
| 6 | CHECK: Does UE return an RLC SDU with the same content as created and embedded in the ESM DATA TRANSPORT at step 3 using the Resource unit Assignment and modulation and coding scheme as configured by the SS in step 5? | --> | MAC PDU containing RLC STATUS PDU and RLC AMD PDU with UL RLC SDU | 1,2 | P |
| 7 | The SS transmits an RLC STATUS PDU to acknowledge correctly received data | <-- | RLC STATUS PDU | - | - |
| 8 | Void. |  |  | - | - |
| NOTE 1: DL RLC SDU with size n contains a DLInformationTransfer-NB with ESM_DATA_TRANSPORT:18 or 26 bits DLInformationTransfer-NB (PER encode: ‘0000000000’B + 8 bits ( length of DedicatedInfoNAS <= 127 bytes ) or 16 bits length ( 128 bytes <= length of DedicatedInfoNAS < 16383 bytes )) 1/2 byte Protocol discriminator ( SECURITY PROTECTED NAS MESSAGE ) 1/2 byte Security header type ( SECURITY PROTECTED NAS MESSAGE ) 4 bytes Message authentication code ( SECURITY PROTECTED NAS MESSAGE ) 1 byte Sequence number ( SECURITY PROTECTED NAS MESSAGE ) 1/2 byte Protocol discriminator ( ESM_DATA_TRANSPORT ) 1/2 byte EPS bearer identity ( ESM_DATA_TRANSPORT ) 1 byte Procedure transaction identity ( ESM_DATA_TRANSPORT ) 1 byte ESM data transport message identity ( ESM_DATA_TRANSPORT ) 2 bytes length indicator of the user data container ( ESM_DATA_TRANSPORT ) N bytes User data container contents ( UL RLC SDU ) 6 bits Aligned bits ( PER encode ) n = N+14 or N+15 bytes DL RLC SDU to achieve the maximum TBS in UL (1000 bits) the DL RLC SDU needs to be segmented NOTE 2: setting of the poll bit causes the UE to insert an RLC Status PDU in the UL message of step 6 NOTE 3: NPDCCH period bundle is specified in TS 36.523-3 [20] clause 7A.14.2. |  |  |  |  |  |

22.3.1.6.3.3	Specific message contents

None.

#### 22.3.1.6a	NB-IoT / DL-SCH / UL-SCH transport block size selection / DCI format N1 / N0 / Category NB2

##### 22.3.1.6a.1	Test Purpose (TP)

Same as 22.3.1.6.

##### 22.3.1.6a.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.212, clause 6.4.3.1, 6.4.3.2; TS 36.213, clauses 16.4.3.1, 16.4.1.5, 16.4.1.5.1; and TS 36.306 clause 4.1C.

[TS 36.212, clause 6.4.3.1]

DCI format N0 is used for the scheduling of NPUSCH in one UL cell.

The following information is transmitted by means of the DCI format N0:

-	Flag for format N0/format N1 differentiation – 1 bit, where value 0 indicates format N0 and value 1 indicates format N1

-	Subcarrier indication – 6 bits as defined in section 16.5.1.1 of [3]

-	Resource assignment – 3 bits as defined in section 16.5.1.2 of [3]

-	Scheduling delay – 2 bits as defined in section 16.5.1 of [3]

-	Modulation and coding scheme – 4 bits as defined in section 16.5.1.2 of [3]

-	Redundancy version – 1 bit as defined in section 16.5.1.2 of [3]

-	Repetition number – 3 bits as defined in section 16.5.1.2 of [3]

-	New data indicator – 1 bit

-	DCI subframe repetition number – 2 bits as defined in section 16.6 in [3]

-	HARQ process number – 1 bit. This field can only be present if 2 HARQ processes are configured and the corresponding DCI format is mapped onto the UE specific search space given by the C-RNTI as defined in [3].

[TS 36.212, clause 6.4.3.2]

DCI format N1 is used for the scheduling of one NPDSCH codeword in one cell, random access procedure initiated by a NPDCCH order, and notifying SC-MCCH change. The DCI corresponding to a NPDCCH order is carried by NPDCCH.

The following information is transmitted by means of the DCI format N1:

-	If the format N1 CRC is scrambled by C-RNTI or RA-RNTI:

-	Flag for format N0/format N1 differentiation – 1 bit, where value 0 indicates format N0 and value 1 indicates format N1

-	NPDCCH order indicator – 1 bit

-	Else if the format N1 CRC is scrambled by a G-RNTI:

-	Information for SC-MCCH change notification – 2 bits as defined in section 5.8a of [6]

Format N1 is used for random access procedure initiated by a NPDCCH order only if NPDCCH order indicator is set to '1', format N1 CRC is scrambled with C-RNTI, and all the remaining fields are set as follows:

-	Starting number of NPRACH repetitions – 2 bits as defined in section 16.3.2 of [3]

-	Subcarrier indication of NPRACH – 6 bits as defined in section 16.3.2 of [3]

-	Carrier indication of NPRACH – 4 bits as defined in section 16.3.2 of [3]. This field is only present if ul-ConfigList is configured and the UE indicates the multiCarrier-NPRACH capability.

-	All the remaining bits in format N1 are set to one

Otherwise,

-	Scheduling delay – 3 bits as defined in section 16.4.1 of [3]

-	Resource assignment – 3 bits as defined in section 16.4.1.3 of [3]

-	Modulation and coding scheme – 4 bits as defined in section 16.4.1.5 of [3]

-	Repetition number – 4 bits as defined in section 16.4.1.3 of [3]

-	New data indicator – 1 bit

-	HARQ-ACK resource – 4 bits as defined in section 16.4.2 of [3].

-	DCI subframe repetition number – 2 bits as defined in section 16.6 in [3]

-	HARQ process number – 1 bit. This field can only be present if 2 HARQ processes are configured and the corresponding DCI format is mapped onto the UE specific search space given by the C-RNTI as defined in [3].

When the format N1 CRC is scrambled with a RA-RNTI or a G-RNTI, then the following fields among the fields above are reserved for RA-RNTI and not present for G-RNTI:

-	New data indicator

-	HARQ-ACK resource

If the number of information bits in format N1 is less than that of format N0 and the format N1 CRC is not scrambled by G-RNTI, zeros shall be appended to format N1 until the payload size equals that of format N0.

[TS 36.306, clause 4.1C]

The field ue-Category-NB defines a combined uplink and downlink capability in NB-IoT. The parameters set by the UE Category are defined in subclause 4.2. Tables 4.1C-1 and 4.1C-2 define the downlink and, respectively, uplink physical layer parameter values for each UE Category. A UE indicating Category NB2 shall also indicate Category NB1.

Table 4.1C-1: Downlink physical layer parameter values set by the field ue-Category-NB

| UE Category | Maximum number of DL-SCH transport block bits received within a TTI | Maximum number of bits of a DL-SCH transport block received within a TTI | Total number of soft channel bits |
| --- | --- | --- | --- |
| Category NB1 | 680 | 680 | 2112 |
| Category NB2 | 2536 | 2536 | 6400 |

Table 4.1C-2: Uplink physical layer parameter values set by the field ue-Category-NB

| UE Category | Maximum number of UL-SCH transport block bits transmitted within a TTI | Maximum number of bits of an UL-SCH transport block transmitted within a TTI |
| --- | --- | --- |
| Category NB1 | 1000 | 1000 |
| Category NB2 | 2536 | 2536 |

[TS 36.213, clause 16.4.1.3]

The resource allocation information in DCI format N1, N2 (paging) for NPDSCH indicates to a scheduled UE

-	a number of subframes () determined by the resource assignment field () in the corresponding DCI according to Table 16.4.1.3-1.

-	a repetition number () determined by the repetition number field () in the corresponding DCI according to Table 16.4.1.3-2.

Table 16.4.1.3-1: Number of subframes () for NPDSCH

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |
| 4 | 5 |
| 5 | 6 |
| 6 | 8 |
| 7 | 10 |

Table 16.4.1.3-2: Number of repetitions () for NPDSCH

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |
| 8 | 192 |
| 9 | 256 |
| 10 | 384 |
| 11 | 512 |
| 12 | 768 |
| 13 | 1024 |
| 14 | 1536 |
| 15 | 2048 |

The number of repetitions for the NPDSCH carrying SystemInformationBlockType1-NB is determined based on the parameter schedulingInfoSIB1 configured by higher-layers and according to Table 16.4.1.3-3.

[TS 36.213, clause 16.4.1.5]

The UE shall use modulation order, = 2.

To determine the transport block size in the NPDSCH, the UE shall first,

-	if NPDSCH carriers SystemInformationBlockType1-NB

-	set  to the value of the parameter schedulingInfoSIB1 configured by higher-layers

-	otherwise

-	read the 4-bit "modulation and coding scheme" field () in the DCI and set .

and second,

-	if NPDSCH carriers SystemInformationBlockType1-NB

-	use Subclause 16.4.1.5.2 for determining its transport block size.

-	otherwise,

-	read the 3-bit "resource assignment" field () in the DCI and determine its TBS by the procedure in Subclause 16.4.1.5.1.

For a NPDCCH UE-specific search space, if the UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-	the NDI and HARQ process ID as signalled on NPDCCH, and the TBS, as determined above, shall be delivered to higher layers,

otherwise

-	the NDI as signalled on NPDCCH, and the TBS, as determined above, shall be delivered to higher layers. HARQ process ID of 0 shall be assumed.

[TS 36.213, clause 16.4.1.5.1]

The TBS is given by the (,) entry of Table 16.4.1.5.1-1. For the value of the higher layer parameter operationModeInfo set to '00' or '01', .

Table 16.4.1.5.1-1: Transport block size (TBS) table

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1032 |
| 7 | 104 | 224 | 328 | 472 | 584 | 680 | 968 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1352 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1032 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 904 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |

[TS 36.213, clause 16.5.1.1]

The resource allocation information in uplink DCI format N0 for NPUSCH transmission indicates to a scheduled UE

-	a set of contiguously allocated subcarriers () of a resource unit determined by the Subcarrier indication field in the corresponding DCI,

-	a number of resource units () determined by the resource assignment field in the corresponding DCI according to Table 16.5.1.1-2,

-	a repetition number () determined by the repetition number field in the corresponding DCI according to Table 16.5.1.1-3.

The subcarrier spacing  of NPUSCH transmission is determined by the uplink subcarrier spacing field in the Narrowband Random Access Response Grant according to Subclause 16.3.3.

For NPUSCH transmission with subcarrier spacing, where  is the subcarrier indication field in the DCI. is reserved.

For NPUSCH transmission with subcarrier spacing, the subcarrier indication field () in the DCI determines the set of contiguously allocated subcarriers () according to Table 16.5.1.1-1.

Table 16.5.1.1-1: Allocated subcarriers for NPUSCH with

| Subcarrier indication field () | Set of Allocated subcarriers () |
| --- | --- |
| 0 – 11 |  |
| 12-15 |  |
| 16-17 |  |
| 18 |  |
| 19-63 | Reserved |

Table 16.5.1.1-2: Number of resource units () for NPUSCH

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |
| 4 | 5 |
| 5 | 6 |
| 6 | 8 |
| 7 | 10 |

Table 16.5.1.1-3: Number of repetitions () for NPUSCH

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |

[TS 36.213, clause 16.5.1.2]

To determine the modulation order, redundancy version and transport block size for the NPUSCH, the UE shall first

-	read the "modulation and coding scheme" field () in the DCI, and

-	read the "redundancy version" field () in the DCI, and

-	read the "resource assignment" field () in the DCI, and

-	compute the total number of allocated subcarriers (), number of resource units (), and repetition number () according to Subclause 16.5.1.1.

The UE shall use modulation order, = 2 if . The UE shall useand Table 16.5.1.2-1 to determine the modulation order to use for NPUSCH if .

Table 16.5.1.2-1: Modulation and TBS index table for NPUSCH with

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 2 |
| 2 | 2 | 1 |
| 3 | 2 | 3 |
| 4 | 2 | 4 |
| 5 | 2 | 5 |
| 6 | 2 | 6 |
| 7 | 2 | 7 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |

NPUSCH is transmitted in N consecutive NB-IoT UL slots, ni , i=0,1,…,N-1. The redundancy version  of the NPUSCH transmission in jth block of B consecutive NB-IoT UL slots ni , is determined by, , where  if ,  otherwise. Portion of NPUSCH codeword with  as defined in clause 6.3.2 in [4] mapped to slot  of allocated  resource unit(s) is transmitted in NB-IoT UL slots ni for and  for

The UE shall use (,) and Table 16.5.1.2-2 to determine the TBS to use for the NPUSCH. is given in Table 16.5.1.2-1 if ,  otherwise.

Table 16.5.1.2-2: Transport block size (TBS) table for NPUSCH

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1384 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 1000 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |

For a NPDCCH UE-specific search space, if the UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-	the NDI and HARQ process ID as signalled on NPDCCH, and the RV and TBS, as determined above, shall be delivered to higher layers,

otherwise

-	the NDI as signalled on NPDCCH, and the RV and TBS, as determined above, shall be delivered to higher layers.

##### 22.3.1.6a.3	Test description

22.3.1.6a.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

-	MasterInformationBlock-NB / MasterInformationBlock-TDD-NB using parameters as specified in TS 36.508 [18] Table 8.1.4.3.2-1 with condition "Standalone"
NOTE: According to TS 36.213 [30] clause 16.4.1.5.1 for Inband Operation Mode ITBS is restricted to 0 ≤ ITBS ≤ 10 i.e. ISF/IMCS combinations with IMCS = 11, 12 could not be tested

UE:

None.

Preamble:

-	UE is in state NB-IoT UE Attach, Connected Mode, UE Test Loopback Activated (State 2B-NB) with test loop mode G and according to [18] in Ncell 1.

22.3.1.6a.3.2	Test procedure sequence

Table 22.3.1.6a.3.2-1: RLC SDU size used as test data

| UE Category | TBSUL [bits] | UL RLC SDU size [bits] |
| --- | --- | --- |
| Category NB2 | 56 < TBsize ≤2536 (Note) | TBSUL – 56 (Note) |
| Note: UL MAC PDU contains 2 MAC SDUs (RLC STATUS PDU, RLC AMD PDU with a single RLC SDU): 2 bytes R/R/E/LCID/L header for 1st MAC SDU 1 byte R/R/E/LCID header for 2nd MAC SDU 2 bytes RLC STATUS PDU 2 bytes RLC AMD header N bytes UL RLC SDU N = (TBSUL – 56) / 8 |  |  |

Table 22.3.1.6a.3.2-2: TB sizes and ISF/ITBS combinations used for UL and DL for NB2

|  | DL |  |  | UL |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| S.No | ISF | IMCS = ITBS | TBSDL | IRU | IMCS / ITBS | TBSUL |
| 1 | 0 | 13 | 224 | 0 | 10/10 | 144 |
| 2 | 1 | 13 | 488 | 1 | 10/10 | 328 |
| 3 | 2 | 13 | 744 | 2 | 10/10 | 504 |
| 4 | 3 | 11 | 776 | 3 | 10/10 | 680 |
| 5 | 3 | 12 | 904 | 3 | 10/10 | 680 |
| 6 | 3 | 13 | 1032 | 3 | 10/10 | 680 |
| 7 | 4 | 9 | 776 | 4 | 8/8 | 680 |
| 8 | 4 | 10 | 872 | 4 | 9/9 | 776 |
| 9 | 4 | 11 | 1000 | 4 | 10/10 | 872 |
| 10 | 4 | 12 | 1128 | 4 | 10/10 | 872 |
| 11 | 4 | 13 | 1256 | 4 | 10/10 | 872 |
| 12 | 5 | 8 | 808 | 5 | 7/7 | 712 |
| 13 | 5 | 9 | 936 | 5 | 8/8 | 808 |
| 14 | 5 | 10 | 1032 | 5 | 9/9 | 936 |
| 15 | 5 | 11 | 1192 | 5 | 10/10 | 1000 |
| 16 | 5 | 12 | 1352 | 5 | 10/10 | 1000 |
| 17 | 5 | 13 | 1544 | 5 | 10/10 | 1000 |
| 18 | 6 | 6 | 808 | 6 | 5/5 | 680 |
| 19 | 6 | 7 | 968 | 6 | 6/6 | 808 |
| 20 | 6 | 8 | 1096 | 6 | 7/7 | 1000 |
| 21 | 6 | 9 | 1256 | 6 | 8/8 | 1096 |
| 22 | 6 | 10 | 1384 | 6 | 9/9 | 1256 |
| 23 | 6 | 11 | 1608 | 6 | 10/10 | 1384 |
| 24 | 6 | 12 | 1800 | 6 | 10/10 | 1384 |
| 25 | 6 | 13 | 2024 | 6 | 10/10 | 1384 |
| 26 | 7 | 5 | 872 | 7 | 4/4 | 680 |
| 27 | 7 | 6 | 1032 | 7 | 5/5 | 872 |
| 28 | 7 | 7 | 1224 | 7 | 6/6 | 1000 |
| 29 | 7 | 8 | 1352 | 7 | 7/7 | 1224 |
| 30 | 7 | 9 | 1544 | 7 | 8/8 | 1384 |
| 31 | 7 | 10 | 1736 | 7 | 9/9 | 1544 |
| 32 | 7 | 11 | 2024 | 7 | 10/10 | 1736 |
| 33 | 7 | 12 | 2280 | 7 | 10/10 | 1736 |
| 34 | 7 | 13 | 2536 | 7 | 10/10 | 1736 |

Table 22.3.1.6a.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| - | Steps 1 to 7 are repeated as per table 22.3.1.6a.3.2-2 with values of ISF, DL IMCS, IRU and UL IMCS | - | - | - | - |
| 1 | SS looks up for TBSUL in table 22.3.1.6a.3.2-1 as per the execution counter | - | - | - | - |
| 2 | SS creates one UL RLC SDU of size TBSUL, embeds it in an ESM DATA TRANSPORT and DLInformationTransfer-NB and puts the resulting DL RLC SDU into one RLC AMD PDU (NOTE 1) | - | - | - | - |
| 3 | SS gets current timing and NPDCCH period M is the next NPDCCH period bundle which can be used by the SS for a DL transmission. (NOTE 3) | - | - | - | - |
| 4 | At NPDCCH period bundle M+1 the SS sends a DCI format N1 and a MAC PDU containing an RLC AMD PDU of the DL RLC SDU created at step 10 with polling field ‘P’ set to '1'. ( NOTE 2, NOTE 3) | <-- | DCI N1 format with ISF, DL IMCS MAC PDU with AMD PDU of the DL RLC SDU | - | - |
| 5 | In the NPDCCH period bundle M+3(M+2 for NTN) after the transmission at step 4 the SS allocates an uplink grant for the UL TBS according to 22.3.1.6a.3.2-2. (NOTE 3) | <-- | (UL Grant) DCI N0 format with IRU and UL IMCS | - | - |
| 6 | CHECK: Does UE return an RLC SDU with the same content as created and embedded in the ESM DATA TRANSPORT at step 2 using the Resource unit Assignment and modulation and coding scheme as configured by the SS in step 5? | --> | MAC PDU containing RLC STATUS PDU and RLC AMD PDU with UL RLC SDU | 1,2 | P |
| 7 | The SS transmits an RLC STATUS PDU to acknowledge correctly received data | <-- | RLC STATUS PDU | - | - |
| NOTE 1: DL RLC SDU with size n contains a DLInformationTransfer-NB with ESM_DATA_TRANSPORT:18 or 26 bits DLInformationTransfer-NB (PER encode: ‘0000000000’B + 8 bits ( length of DedicatedInfoNAS <= 127 bytes ) or 16 bits length ( 128 bytes <= length of DedicatedInfoNAS < 16383 bytes )) 1/2 byte Protocol discriminator ( SECURITY PROTECTED NAS MESSAGE ) 1/2 byte Security header type ( SECURITY PROTECTED NAS MESSAGE ) 4 bytes Message authentication code ( SECURITY PROTECTED NAS MESSAGE ) 1 byte Sequence number ( SECURITY PROTECTED NAS MESSAGE ) 1/2 byte Protocol discriminator ( ESM_DATA_TRANSPORT ) 1/2 byte EPS bearer identity ( ESM_DATA_TRANSPORT ) 1 byte Procedure transaction identity ( ESM_DATA_TRANSPORT ) 1 byte ESM data transport message identity ( ESM_DATA_TRANSPORT ) 2 bytes length indicator of the user data container ( ESM_DATA_TRANSPORT ) N bytes User data container contents ( UL RLC SDU ) 6 bits Aligned bits ( PER encode ) n = N+14 or N+15 bytes DL RLC SDU to achieve the maximum TBS in UL (1000 bits) the DL RLC SDU needs to be segmented NOTE 2: setting of the poll bit causes the UE to insert an RLC Status PDU in the UL message of step 6 NOTE 3: NPDCCH period bundle is specified in TS 36.523-3 [20] clause 7A.14.2. |  |  |  |  |  |

22.3.1.6a.3.3	Specific message contents

None.

#### 22.3.1.7	NB-IoT / RACH Procedure / Contention free random access (CFRA)

##### 22.3.1.7.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and ra-CFRA-Config is configured and timeAlignmentTimer has expired }

ensure that {

when { SS sends NPDCCH order to the C-RNTI assigned to UE and ra-PreambleIndex signalled is not 000000 }

then { UE sends a prach preamble calculated by given ra-PreambleIndex in the NPDCCH Order and parameters in currently used PRACH resource }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and ra-CFRA-Config is configured, has transmitted PRACH Preamble after reception of PDCCH order with ra-PreambleIndex not set to 000000 }

ensure that {

when { UE receives a Random Access response containing a RAPID corresponding to the UE in RA Response window }

then { UE considers the Random Access procedure successfully completed }

}

##### 22.3.1.7.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.321, clause 5.1.1, 5.1.2, 5.1.4; TS 36.312, clause 6.4.3.2.

[TS 36.321, clause 5.1.1]

The Random Access procedure described in this subclause is initiated by a PDCCH order, by the MAC sublayer itself or by the RRC sublayer. Random Access procedure on an SCell shall only be initiated by a PDCCH order. If a MAC entity receives a PDCCH transmission consistent with a PDCCH order [5] masked with its C-RNTI, and for a specific Serving Cell, the MAC entity shall initiate a Random Access procedure on this Serving Cell. For Random Access on the SpCell a PDCCH order or RRC optionally indicate the ra-PreambleIndex and the ra-PRACH-MaskIndex, except for NB-IoT where the subcarrier index is indicated; and for Random Access on an SCell, the PDCCH order indicates the ra-PreambleIndex with a value different from 000000 and the ra-PRACH-MaskIndex. For the pTAG preamble transmission on PRACH and reception of a PDCCH order are only supported for SpCell. If the UE is an NB-IoT UE, the Random Access procedure is performed on the anchor carrier or one of the non-anchor carriers for which PRACH resource has been configured in system information.

…

The following information for related Serving Cell is assumed to be available before the procedure can be initiated for NB-IoT UEs, BL UEs or UEs in enhanced coverage [8]:

…

-	for NB-IoT, the use of contention free random access ra-CFRA-Config.

[TS 36.321, clause 5.1.2]

The Random Access Resource selection procedure shall be performed as follows:

-	For BL UEs or UEs in enhanced coverage, select the PRACH resource set corresponding to the selected enhanced coverage level.

-	If, except for NB-IoT, ra-PreambleIndex (Random Access Preamble) and ra-PRACH-MaskIndex (PRACH Mask Index) have been explicitly signalled and ra-PreambleIndex is not 000000:

-	the Random Access Preamble and the PRACH Mask Index are those explicitly signalled;

-	else, for NB-IoT, if ra-PreambleIndex (Random Access Preamble) and PRACH resource have been explicitly signalled:

-	the PRACH resource is that explicitly signalled;

-	if the ra-PreambleIndex signalled is not 000000:

-	if ra-CFRA-Config is configured:

-	the Random Access Preamble is set to nprach-SubcarrierOffset + nprach-NumCBRA-StartSubcarriers + (ra-PreambleIndex modulo (nprach-NumSubcarriers - nprach-NumCBRA-StartSubcarriers)), where nprach-SubcarrierOffset, nprach-NumCBRA-StartSubcarriers and nprach-NumSubcarriers are parameters in the currently used PRACH resource.

-	else:

-	the Random Access Preamble is set to nprach-SubcarrierOffset + (ra-PreambleIndex modulo nprach-NumSubcarriers), where nprach-SubcarrierOffset and nprach-NumSubcarriers are parameters in the currently used PRACH resource.

-	else:

-	select the Random Access Preamble group according to the PRACH resource and the support for multi-tone Msg3 transmission. A UE supporting multi-tone Msg3 shall only select the single-tone Msg3 Random Access Preambles group if there is no multi-tone Msg3 Random Access Preambles group.

-	randomly select a Random Access Preamble within the selected group.

-	else the Random Access Preamble shall be selected by the MAC entity as follows:

…

[TS 36.321, clause 5.1.4]

The MAC entity may stop monitoring for Random Access Response(s) after successful reception of a Random Access Response containing Random Access Preamble identifiers that matches the transmitted Random Access Preamble.

-	If a downlink assignment for this TTI has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded, the MAC entity shall regardless of the possible occurrence of a measurement gap or a Sidelink Discovery Gap for Transmission or a Sidelink Discovery Gap for Reception, and regardless of the prioritization of V2X sidelink communication described in subclause 5.14.1.2.2:

-	if the Random Access Response contains a Backoff Indicator subheader:

-	set the backoff parameter value as indicated by the BI field of the Backoff Indicator subheader and Table 7.2-1, except for NB-IoT where the value from Table 7.2-2 is used.

-	else, set the backoff parameter value to 0 ms.

-	if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble (see subclause 5.1.3), the MAC entity shall:

-	consider this Random Access Response reception successful and apply the following actions for the serving cell where the Random Access Preamble was transmitted:

-	process the received Timing Advance Command (see subclause 5.2);

-	indicate the preambleInitialReceivedTargetPower and the amount of power ramping applied to the latest preamble transmission to lower layers (i.e., (PREAMBLE_TRANSMISSION_COUNTER – 1) * powerRampingStep);

-	if the SCell is configured with ul-Configuration-r14, ignore the received UL grant otherwise process the received UL grant value and indicate it to the lower layers;

-	if, except for NB-IoT, ra-PreambleIndex was explicitly signalled and it was not 000000 (i.e., not selected by MAC):

-	consider the Random Access procedure successfully completed.

-	else if, the UE is an NB-IoT UE, ra-PreambleIndex was explicitly signalled and it was not 000000 (i.e., not selected by MAC) and ra-CFRA-Config is configured:

-	consider the Random Access procedure successfully completed.

-	the UL grant provided in the Random Access Response message is valid only for the configured carrier.

-	else:

-	if the Random Access Preamble was selected by the MAC entity; or

-	if the UE is an NB-IoT UE, the ra-PreambleIndex was explicitly signalled and it was not 000000 and ra-CFRA-Config is not configured:

-	set the Temporary C-RNTI to the value received in the Random Access Response message no later than at the time of the first transmission corresponding to the UL grant provided in the Random Access Response message;

-	if this is the first successfully received Random Access Response within this Random Access procedure:

-	if the transmission is not being made for the CCCH logical channel, indicate to the Multiplexing and assembly entity to include a C-RNTI MAC control element in the subsequent uplink transmission;

-	obtain the MAC PDU to transmit from the "Multiplexing and assembly" entity and store it in the Msg3 buffer.

NOTE:	When an uplink transmission is required, e.g., for contention resolution, the eNB should not provide a grant smaller than 56 bits (or 88 bits for NB-IoT) in the Random Access Response.

NOTE:	If within a Random Access procedure, an uplink grant provided in the Random Access Response for the same group of Random Access Preambles has a different size than the first uplink grant allocated during that Random Access procedure, the UE behaviour is not defined.

[TS 36.312, clause 6.4.3.2]

DCI format N1 is used for the scheduling of one NPDSCH codeword in one cell, random access procedure initiated by a NPDCCH order, and notifying SC-MCCH change. The DCI corresponding to a NPDCCH order is carried by NPDCCH.

The following information is transmitted by means of the DCI format N1:

-	If the format N1 CRC is scrambled by C-RNTI or RA-RNTI:

-	Flag for format N0/format N1 differentiation – 1 bit, where value 0 indicates format N0 and value 1 indicates format N1

-	NPDCCH order indicator – 1 bit

-	Else if the format N1 CRC is scrambled by a G-RNTI:

-	Information for SC-MCCH change notification – 2 bits as defined in section 5.8a of [6]

Format N1 is used for random access procedure initiated by a NPDCCH order only if NPDCCH order indicator is set to '1', format N1 CRC is scrambled with C-RNTI, and all the remaining fields are set as follows:

-	Starting number of NPRACH repetitions – 2 bits as defined in section 16.3.2 of [3]

-	Subcarrier indication of NPRACH – 6 bits as defined in section 16.3.2 of [3]

-	Carrier indication of NPRACH – 4 bits as defined in section 16.3.2 of [3]. This field is only present if ul-ConfigList is configured and the UE indicates the multiCarrier-NPRACH capability.

-	All the remaining bits in format N1 are set to one

Otherwise,

-	Scheduling delay – 3 bits as defined in section 16.4.1 of [3]

-	Resource assignment – 3 bits as defined in section 16.4.1.3 of [3]

-	Modulation and coding scheme – 4 bits as defined in section 16.4.1.5 of [3]

-	Repetition number – 4 bits as defined in section 16.4.1.3 of [3]

-	New data indicator – 1 bit

-	HARQ-ACK resource – 4 bits as defined in section 16.4.2 of [3].

-	DCI subframe repetition number – 2 bits as defined in section 16.6 in [3]

-	HARQ process number – 1 bit. This field can only be present if 2 HARQ processes are configured and the corresponding DCI format is mapped onto the UE specific search space given by the C-RNTI as defined in [3].

When the format N1 CRC is scrambled with a RA-RNTI or a G-RNTI, then the following fields among the fields above are reserved for RA-RNTI and not present for G-RNTI:

-	New data indicator

-	HARQ-ACK resource

If the number of information bits in format N1 is less than that of format N0 and the format N1 CRC is not scrambled by G-RNTI, zeros shall be appended to format N1 until the payload size equals that of format N0.

##### 22.3.1.7.3	Test description

22.3.1.7.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

UE:

None.

Preamble:

-	The UE shall be in Connected Mode State 2-NB according to TS 36.508 [18].

22.3.1.7.3.2	Test procedure sequence

Table 22.3.1.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The SS does not transmit Timing Advance command. | - | - | - | - |
| 2 | The SS waits for 3s (TA Timer expiry). | - | - | - | - |
| 3 | The SS transmits a NPDCCH order to C-RNTI assigned to the UE on NPDCCH | <-- | NPDCCH order (sub-carrier index := 5) | - | - |
| 4 | Check: Does the UE transmit a preamble on NPRACH with RAPID corresponding to subcarrier index provided in step 3? (NOTE 1) | --> | NPRACH Preamble | 1 | P |
| 5 | The SS transmits a MAC PDU addressed to UE RA-RNTI, containing a matching RAPID. | <-- | Random Access Response | - | - |
| 6 | Check: Does the test result of generic test procedure in TS 36.508 subclause 8.1.5A.8 indicate that the UE is in RRC_CONNECTED state? | - | - | 2 | P |
| NOTE 1: The Random Access Preamble is set to nprach-SubcarrierOffset + nprach-NumCBRA-StartSubcarriers + (ra-PreambleIndex modulo (nprach-NumSubcarriers - nprach-NumCBRA-StartSubcarriers)), i.e. RAPID = 12+8+(5mod(12-8)) = 21. |  |  |  |  |  |

22.3.1.7.3.3	Specific message contents

Table 22.3.1.7.3.3-1: RadioResourceConfigCommonSIB-NB-DEFAULT in SystemInformationBlockType2-NB (preamble, Table 22.3.1.7.3.2-1)

| Derivation Path: 36.508 Table 8.1.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nprach-Config-v1330 ::= SEQUENCE { |  |  |  |
| nprach-ParametersList-v1330 ::= SEQUENCE { |  |  |  |
| { |  |  |  |
| nprach-NumCBRA-StartSubcarriers-r13 | n8 |  |  |
| } |  |  |  |
| } |  |  |  |
| nprach-Config-v1530 | NPRACH-ConfigSIB-NB-v1530-DEFAULT |  | TDD |
| } |  |  |  |

Table 22.3.1.7.3.3-2: RRCConnectionSetup-NB (preamble, Table 22.3.1.7.3.2-1)

| Derivation path: 36.508 table 8.1.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetup-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetup-r13 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated-r13 SEQUENCE { |  |  |  |
| mac-MainConfig-r13 CHOICE { |  |  |  |
| explicitValue-r13 SEQUENCE { |  |  |  |
| timeAlignmentTimerDedicated-r13 | sf10240 | needs to be long enough to ensure time alignment timer does not time out before the end of testcase |  |
| ra-CFRA-Config-r14 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 22.3.1.8	NB-IoT / RACH Procedure / Non-anchor carrier

##### 22.3.1.8.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state and SystemInformationBlockType22-NB with IE nprach-ParametersList-r14 is broadcast }

ensure that {

when { UE has need to send a CCCH UL MAC PDU and UE is in enhanced coverage level 0}

then { UE transmits a random access preamble repeated numRepetitionsPerPreambleAttempt from Random Access Preambles group using the non-anchor NPRACH resource corresponding to enhanced coverage level 0}

}

##### 22.3.1.8.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.321, clauses 5.1.1.

[TS 36.321, clause 5.1.1]

…

-	if the UE is an NB-IoT UE:

-	the available set of PRACH resources supported in the Serving Cell on the anchor carrier, nprach-ParametersList, and on the non-anchor carriers, in ul-ConfigList.

-	for EDT, the available set of PRACH resources associated with EDT on anchor carrier, nprach-ParametersList-EDT, and on the non-anchor carriers, in ul-ConfigList.

-	for random access resource selection and preamble transmission:

-	a PRACH resource is mapped into an enhanced coverage level.

-	each PRACH resource contains a set of nprach-NumSubcarriers subcarriers which can be partitioned into one or two groups for single/multi-tone Msg3 transmission by nprach-SubcarrierMSG3-RangeStart and nprach-NumCBRA-StartSubcarriers as specified in TS 36.211 [7, 10.1.6.1]. Each group is referred to as a Random Access Preamble group below in the procedure text.

-	a subcarrier is identified by the subcarrier index in the range:
[nprach-SubcarrierOffset, nprach-SubcarrierOffset + nprach-NumSubcarriers -1]

-	each subcarrier of a Random Access Preamble group corresponds to a Random Access Preamble.

-	when the subcarrier index is explicitly sent from the eNB as part of a PDCCH order ra-PreambleIndex shall be set to the signalled subcarrier index.

-	the mapping of the PRACH resources into enhanced coverage levels is determined according to the following:

-	the number of enhanced coverage levels is equal to one plus the number of RSRP thresholds present in rsrp-ThresholdsPrachInfoList.

-	each enhanced coverage level has one anchor carrier PRACH resource present in nprach-ParametersList and zero or one PRACH resource for each non-anchor carrier signalled in ul-ConfigList.

-	for EDT, each enhanced coverage level has zero or one anchor carrier PRACH resource present in nprach-ParametersList-EDT and zero or one PRACH resource for each non-anchor carrier signalled in ul-ConfigList.

-	enhanced coverage levels are numbered from 0 and the mapping of PRACH resources to enhanced coverage levels are done in increasing numRepetitionsPerPreambleAttempt order.

-	when multiple carriers provide PRACH resources for the same enhanced coverage level, the UE will randomly select one of them using the following selection probabilities:

-	the selection probability of the anchor carrier PRACH resource for the given enhanced coverage level, nprach-ProbabilityAnchor, is given by the corresponding entry in nprach-ProbabilityAnchorList

-	the selection probability is equal for all non-anchor carrier PRACH resources and the probability of selecting one PRACH resource on a given non-anchor carrier is (1- nprach-ProbabilityAnchor)/(number of non-anchor NPRACH resources)

##### 22.3.1.8.3	Test description

22.3.1.8.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

NB-IoT Operation Mode set to standalone

-	System information combination 6 as defined in TS 36.508[18] clause 8.1.4.3.1.1.

- 	SystemInformationBlockType22-NB as specified in Table 22.3.1.8.3.3-1.

UE:

None.

Preamble:

-	UE is in state Registered, Idle Mode (state 3-NB) according to [18] in Ncell 1.

22.3.1.8.3.2	Test procedure sequence

Table 22.3.1.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0A | SS adjusts SystemInformationBlockType22-NB to indicate the NPRACH Probablity Anchor is set to zero and notifies the UE of change of System Information | <-- | Paging-NB | - | - |
| 0B | Wait for 90 seconds for UE to read updated SystemInformationBlockType22-NB | - | - | - | - |
| 1 | The SS transmits a Paging message including a matched identity. | <-- | Paging-NB | - | - |
| 2 | Check: Does the UE transmit a preamble on NPRACH on non-anchor carrier in ul-ConfigList in SystemInformationBlockType22-NB? | --> | NPRACH Preamble | 1 | P |
| 3 | The SS transmits a MAC PDU addressed to UE RA-RNTI, containing a matching RAPID. | <-- | Random Access Response | - | - |
| 4 | UE sends an RRCConnectionRequest-NB message? | --> | MAC PDU (RRCConnectionRequest-NB) | - | - |
| 5 | The SS transmits a valid MAC PDU containing “UE Contention Resolution Identity” MAC control element with matching “Contention Resolution Identity” and CCCH message (RRCConnectionSetup-NB containing configuration of UE-specific search space). | <-- | MAC PDU (UE Contention Resolution Identity, RRCConnectionSetup-NB) | - | - |
| 6 | UE sends an RRCConnectionSetupComplete-NB message ? | --> | RRCConnectionSetupComplete-NB | - | - |

22.3.1.8.3.3	Specific message contents

Table 22.3.1.8.3.3-1: SystemInformationBlockType22-NB (preamble)

| Derivation Path: 36.508 Table 8.1.4.3.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType22-NB-r14 ::= SEQUENCE { |  |  |  |
| dl-ConfigList-r14 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF DL-ConfigCommon-NB-r14 SEQUENCE { | 1 entry |  |  |
| DL-ConfigCommon-NB-r14[1] SEQUENCE { |  |  |  |
| dl-CarrierConfig-r14 | DL-CarrierConfigCommon-NB-DEFAULT1 |  |  |
| pcch-Config-r14 SEQUENCE { |  |  |  |
| npdcch-NumRepetitionPaging-r14 | Not present |  |  |
| pagingWeight-r14 | w1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-ConfigList-r14 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF UL-ConfigCommon-NB-r14 SEQUENCE { | 1 entry |  | FDD |
| UL-ConfigCommon-NB-r14[1] SEQUENCE { |  |  |  |
| ul-CarrierFreq-r14 | Uplink EARFCN for non-anchor carrier under test. For Signalling test cases, see 36.508 clause 8.3.2.3. Use the anchor carrier EARFCN + 2. |  |  |
| nprach-ParametersList-r14 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-r14 SEQUENCE { | 1 entry |  |  |
| nprach-Parameters-r14 SEQUENCE { |  |  |  |
| nprach-Periodicity-r14 | ms640 |  |  |
| nprach-StartTime-r14 | ms8 |  |  |
| nprach-SubcarrierOffset-r14 | n12 |  |  |
| nprach-NumSubcarriers-r14 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r14 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r14 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r14 | v4 |  |  |
| npdcch-Offset-RA-r14 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r14 | n8 |  |  |
| npdcch-CarrierIndex-r14 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| pagingWeightAnchor-r14 | w1 |  |  |
| nprach-ProbabilityAnchorList-r14 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-ProbabilityAnchor-NB-r14 SEQUENCE { |  |  |  |
| nprach-ProbabilityAnchor-r14 | oneHalf |  |  |
| } |  |  |  |
| ul-ConfigList-r15 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF UL-ConfigCommonTDD-NB-r15 SEQUENCE { | 1 entry |  | TDD |
| tdd-UL-DL-AlignmentOffset-r15 | khz0 |  |  |
| nprach-ParametersListTDD-r15 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-ParametersTDD-NB-r15 SEQUENCE { | 1 entry |  |  |
| nprach-Parameters-r15 SEQUENCE { |  |  |  |
| nprach-Periodicity-r15 | ms640 |  |  |
| nprach-StartTime-r15 | ms10 |  |  |
| nprach-SubcarrierOffset-r15 | n12 |  |  |
| nprach-NumSubcarriers-r15 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r15 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r15 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r15 | v4 |  |  |
| npdcch-Offset-RA-r15 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r15 | n8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.1.8.3.3-2: DL-CarrierConfigCommon-NB-DEFAULT1

| Derivation Path: 36.508 clause 8.1.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DL-CarrierConfigCommon-NB -DEFAULT::= SEQUENCE { |  |  |  |
| dl-CarrierFreq-r14 | Downlink EARFCN for non-anchor carrier under test. For Signalling test cases, see 36.508 clause 8.3.2.3. Use the anchor carrier EARFCN + 2. |  |  |
| } |  |  |  |

Table 22.3.1.8.3.3-3: SystemInformationBlockType22-NB (Step 0A)

| Derivation Path: 22.3.1.8.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType22-NB-r14 ::= SEQUENCE { |  |  |  |
| nprach-ProbabilityAnchorList-r14 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-ProbabilityAnchor-NB-r14 SEQUENCE { |  |  |  |
| nprach-ProbabilityAnchor-r14 | zero |  |  |
| } |  |  |  |
| } |  |  |  |

#### 22.3.1.9	NB-IoT / Correct HARQ process / 2 HARQ processes

##### 22.3.1.9.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and twoHARQ-ProcessesConfig configured }

ensure that {

when { the UE receives MAC PDUs for 2 HARQ processes and results in successful decode }

then { the UE transmits ACKs for the corresponding HARQ processes }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and twoHARQ-ProcessesConfig configured }

ensure that {

when { UE receives uplink grants on NPDCCH for 2 HARQ processes respectively }

then { UE transmits uplink MAC PDUs on the resources indicated by the UL grants addressed to the corresponding HARQ processes }

}

##### 22.3.1.9.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.321, clauses 5.3.2.1and clauses 5.4.2.1.

[TS 36.321, clause 5.3.2.1]

…

In addition to the broadcast HARQ process, NB-IoT has one or two DL HARQ processes.

[TS 36.321, clause 5.4.2.1]

There is one HARQ entity at the MAC entity for each Serving Cell with configured uplink, which maintains a number of parallel HARQ processes allowing transmissions to take place continuously while waiting for the HARQ feedback on the successful or unsuccessful reception of previous transmissions.

The number of parallel HARQ processes per HARQ entity is specified in [2], clause 8. NB-IoT has one or two UL HARQ processes.

##### 22.3.1.9.3	Test description

22.3.1.9.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

UE:

None.

Preamble:

-	The UE shall be in State 2B-NB with test loop mode G (CP CIoT Optimisation) according to TS 36.508 [18].

22.3.1.9.3.2	Test procedure sequence

If the start RLC UL and DL sequence numbers to be used at start of test body are non zero, but X and Y respectively due to transmission/reception of RLC PDU’s in preamble on bearer to be used, then any sequence number ‘n’ in test procedure maps as:

UL SQN n maps to SQN X+n MOD 1024 &

DL SQN n maps to SQN Y+n MOD 1024.

Table 22.3.1.9.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1 to 6 run 2 times using process X for each iteration (X=0,1). | - | - | - | - |
| 1 | The SS transmits a MAC PDU containing a MAC SDU with a RLC SDU of 38 bytes in an AMD PDU with polling field ‘P’ set to ‘0’ on HARQ process X. (NOTE 1) | <-- | NPDCCH (DCI N1) MAC PDU (R/R/E/LCID MAC sub-header (E=’0’, LCID= ‘0011’), 40 bytes MAC SDU) (RLC SN=0) | - | - |
| 2 | Check: Does the UE send a HARQ ACK? | --> | HARQ ACK | 1 | P |
| 3 | The SS allocates an UL grant on HARQ process X, allowing the UE to return the RLC SDU as received in step 1. | <-- | Uplink Grant | - | - |
| 4 | Check: Does the UE transmit a MAC PDU corresponding to UL grant in step 3? | --> | MAC PDU(RLC SN=0) | 2 | P |
| 5 | SS transmits an RLC STATUS PDU on HARQ process X to acknowledge correctly received data. | <-- | MAC PDU (RLC STATUS PDU(RLC SN=1)) | - | - |
| 6 | The UE sends HARQ ACK for the RLC STATUS PDU of step 5. | --> | HARQ ACK | - | - |
| NOTE 1: RLC SDU with size n contains a DLInformationTransfer-NB with ESM_DATA_TRANSPORT: 18 or 26 bits DLInformationTransfer-NB(PER encode: ‘0000000000’B + 8 bits ( length of DedicatedInfoNAS <= 127 bytes ) or 16 bits length ( 128 bytes <= length of DedicatedInfoNAS < 16383 bytes )) 1/2 byte Protocol discriminator ( SECURITY PROTECTED NAS MESSAGE ) 1/2 byte Security header type ( SECURITY PROTECTED NAS MESSAGE ) 4 bytes Message authentication code ( SECURITY PROTECTED NAS MESSAGE ) 1 byte Sequence number ( SECURITY PROTECTED NAS MESSAGE ) 1/2 byte Protocol discriminator ( ESM_DATA_TRANSPORT ) 1/2 byte EPS bearer identity ( ESM_DATA_TRANSPORT ) 1 byte Procedure transaction identity ( ESM_DATA_TRANSPORT ) 1 byte ESM data transport message identity ( ESM_DATA_TRANSPORT ) 2 bytes length indicator of the user data container ( ESM_DATA_TRANSPORT ) N bytes User data container contents ( UL RLC SDU ) 6 bits Aligned bits ( PER encode ) n = N+14 or N+15 bytes DL RLC SDU |  |  |  |  |  |

22.3.1.9.3.3	Specific message contents

Table 22.3.1.9.3.3-1: RRCConnectionSetup-NB (preamble, Table 22.3.1.9.3.2-1)

| Derivation path: 36.508 table 8.1.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetup-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetup-r13 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated-r13 SEQUENCE { |  |  |  |
| srb-ToAddModList-r13 SEQUENCE (SIZE (1)) OF SEQUENCE { | 1 entry |  |  |
| SRB-ToAddMod-NB-r13 SEQUENCE { |  |  |  |
| rlc-Config-v1430 SEQUENCE { |  |  |  |
| t-Reordering-r14 | ms1600-v1310 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| physicalConfigDedicated-r13 SEQUENCE { |  |  |  |
| twoHARQ-ProcessesConfig-r14 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 22.3.1.10	NB-IoT / RACH Procedure / Early contention resolution

##### 22.3.1.10.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE has sent a CCCH UL MAC PDU with earlyContentionResolution is set to TRUE and UE receive a PDCCH transmission addressed to its Temporary C-RNTI contains a UE Contention Resolution Identity MAC control element matches the 48 first bits of the CCCH SDU transmitted in Msg3}

then { UE considers this Contention Resolution successful }

}

##### 22.3.1.10.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.300, clause 10.1.5.1; TS 36.321, clauses 5.1.5, TS 36.331, clauses 5.3.3.3.

[TS 36.300, clause 10.1.5.1]

Contention Resolution on DL:

-	Early contention resolution shall be used i.e. eNB does not wait for NAS reply before resolving contention;

-	For NB-IoT, for initial access, RRC connection resume procedure and RRC Connection Re-establishment procedure, eNB may transmit MAC PDU containing the UE contention resolution identity MAC control element without RRC response message;

NOTE:	In Release 13, NB-IoT UEs do not support the MAC PDU containing the UE contention resolution identity MAC control element without RRC response message for initial access, RRC connection resume procedure and RRC Connection Re-establishment procedure.

-	Not synchronised with message 3;

-	HARQ is supported;

-	Addressed to:

-	The Temporary C-RNTI on PDCCH for initial access and after radio link failure;

-	The C-RNTI on PDCCH for UE in RRC_CONNECTED.

-	HARQ feedback is transmitted only by the UE which detects its own UE identity, as provided in message 3, echoed in the Contention Resolution message;

-	For initial access, RRC Connection Re-establishment procedure and EDT for Control Plane CIoT EPS Optimizations, no segmentation is used (RLC-TM).

[TS 36.321, clause 5.1.5]

Contention Resolution is based on either C-RNTI on PDCCH of the SpCell or UE Contention Resolution Identity on DL-SCH.

Once Msg3 is transmitted, the MAC entity shall:

-	except for a BL UE or a UE in enhanced coverage, or an NB-IoT UE, start mac-ContentionResolutionTimer and restart mac-ContentionResolutionTimer at each HARQ retransmission;

-	for a BL UE or a UE in enhanced coverage, or an NB-IoT UE, start mac-ContentionResolutionTimer and restart mac-ContentionResolutionTimer at each HARQ retransmission of the bundle in the subframe containing the last repetition of the corresponding PUSCH transmission;

-	regardless of the possible occurrence of a measurement gap or Sidelink Discovery Gap for Reception, monitor the PDCCH until mac-ContentionResolutionTimer expires or is stopped;

-	if notification of a reception of a PDCCH transmission is received from lower layers, the MAC entity shall:

-	if the C-RNTI MAC control element was included in Msg3:

-	if the Random Access procedure was initiated by the MAC sublayer itself or by the RRC sublayer and the PDCCH transmission is addressed to the C-RNTI and contains an UL grant for a new transmission; or

-	if the Random Access procedure was initiated by a PDCCH order and the PDCCH transmission is addressed to the C-RNTI:

-	consider this Contention Resolution successful;

-	stop mac-ContentionResolutionTimer;

-	discard the Temporary C-RNTI;

-	if the UE is an NB-IoT UE:

-	the UL grant or DL assignment contained in the PDCCH transmission is valid only for the configured carrier.

-	consider this Random Access procedure successfully completed.

-	else if the CCCH SDU was included in Msg3 and the PDCCH transmission is addressed to its Temporary C-RNTI:

-	if the MAC PDU is successfully decoded:

-	stop mac-ContentionResolutionTimer;

-	if the MAC PDU contains a UE Contention Resolution Identity MAC control element; and

-	if the UE Contention Resolution Identity included in the MAC control element matches the 48 first bits of the CCCH SDU transmitted in Msg3:

-	consider this Contention Resolution successful and finish the disassembly and demultiplexing of the MAC PDU;

-	set the C-RNTI to the value of the Temporary C-RNTI;

-	discard the Temporary C-RNTI;

-	consider this Random Access procedure successfully completed.

-	else

-	discard the Temporary C-RNTI;

-	consider this Contention Resolution not successful and discard the successfully decoded MAC PDU.

[TS 36.331, clause 5.3.3.3]

The UE shall set the contents of RRCConnectionRequest message as follows:

…

1>	if the UE is a NB-IoT UE:

2>	if the UE supports multi-tone transmission, include multiToneSupport;

2>	if the UE supports multi-carrier operation, include multiCarrierSupport;

2>	if the UE supports DL channel quality reporting and cqi-Reporting is present in SystemInformationBlockType2-NB:

3>	set the cqi-NPDCCH to include the latest results of the downlink channel quality measurements of the serving cell as specified in TS 36.133 [16];

NOTE 2:	The downlink channel quality measurements may use measurement period T1 or T2, as defined in TS 36.133 [16]. In case period T2 is used the RRC-MAC interactions are left to UE implementation.

2>	set earlyContentionResolution to TRUE;

##### 22.3.1.10.3	Test description

22.3.1.10.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

UE:

None.

Preamble:

-	UE is in state Registered, Idle Mode (state 3-NB) according to [18] in Ncell 1.

22.3.1.10.3.2	Test procedure sequence

Table 22.3.1.10.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message including a matched identity. | <-- | Paging-NB | - | - |
| 2 | The UE transmit a preamble on NPRACH | --> | NPRACH Preamble | - | - |
| 3 | The SS transmits a MAC PDU addressed to UE RA-RNTI, containing a matching RAPID. | <-- | Random Access Response | - | - |
| 4 | Check: Does UE send an RRCConnectionRequest-NB message with earlyContentionResolution set to TRUE? | --> | MAC PDU (RRCConnectionRequest-NB) | - | - |
| 5 | The SS transmits a valid MAC PDU containing “UE Contention Resolution Identity” MAC control element with matching “Contention Resolution Identity”. | <-- | MAC PDU (UE Contention Resolution Identity) | - | - |
| 6 | The SS transmits RRCConnectionSetup-NB message. | <-- | MAC PDU (RRCConnectionSetup-NB) | - | - |
| 7 | Check: Does UE send an RRCConnectionSetupComplete-NB message? | --> | RRCConnectionSetupComplete-NB | 1 | P |

22.3.1.10.3.3	Specific message contents

Table 22.3.1.10.3.3-1: RRCConnectionRequest-NB (step 4, Table 22.3.1.10.3.2-1)

| Derivation Path: 36.331 clause 8.1.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r13 SEQUENCE { |  |  |  |
| ue-Identity-r13 | Any allowed value |  |  |
| earlyContentionResolution-r14 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |