3GPP TSG-RAN5 Meeting #102	R5-241599

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3041 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update test procedure for registration of NR sidelink U2N Relay / Relay UE side |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Procedure for 5G ProSe U2N Relay Discovery & NAS message hasn’t be clarified in the procedure for registration of NR sidelink U2N Relay / Relay UE side according to TS 23.304 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding it in the procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for NR sidelink U2N Relay / Relay UE side will be not implementable |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.41 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.9.41	Test procedure for 5G ProSe Layer-2 U2N Relay initial access / Relay UE side

4.9.41.1	Scope

The purpose of this procedure is to test relay UE’s behaviour during 5G ProSe Layer-2 U2N Relay Initial Access procedure.

4.9.41.2	Procedure description

4.9.41.2.1	Initial conditions

The UE is in state 3N-A.

4.9.41.2.2	Procedure sequence

Table 4.9.41.2.2-1: Procedure for 5G ProSe Layer-2 U2N Relay initial access / Relay UE side

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Procedure for The UE initiated 5G ProSe U2N Relay Discovery, procedure specified in subclause 4.9.43 is performed. |  |  |  |  |
| - | EXCEPTION: Steps 2a1 and 2b1 describe the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 2a1 | IF Cast type = Unicast AND UE initiated unicast mode ProSe Direct communication THEN generic procedure specified in subclause 4.9.39 is performed to establish unicast mode sidelink communication between the UE and the NR-SS-UE. | - | - | - | - |
| 2b1 | ELSE IF Cast type = Unicast AND NR-SS-UE initiated unicast mode ProSe Direct communication THEN generic procedure specified in subclause 4.9.40 is performed to establish unicast mode sidelink communication between the UE and the NR-SS-UE. | - | - | - | - |
| 3 | The NR-SS-UE1 transmits an RRCSetupRequest message. | <-- | PC5: SRAP PDU (including NR RRC: RRCSetupRequest) | - | - |
| 4 | The UE transmits a SidelinkUEInformationNR message. | --> | NR RRC: SidelinkUEInformationNR | - | - |
| 5 | The SS transmits an RRCReconfiguration message. | <-- | NR RRC: RRCReconfiguration | - | - |
| 6 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - |  |
| 7 | The UE relays the RRCSetupRequest message received at step 3. | --> | PC5: SRAP PDU (including NR RRC: RRCSetupRequest) | - | - |
| 8 | The SS transmits an RRCSetup message. | <-- | PC5: SRAP PDU (including NR RRC: RRCSetup) | - | - |
| 9 | The UE relays the RRCSetup message received at step 8. | --> | PC5: SRAP PDU (including NR RRC: RRCSetup) | - | - |
| 10 | The NR-SS-UE1 transmits an RRCSetupComplete message and a REGISTRATION REQUEST message. | <-- | PC5: SRAP PDU (including NR RRC: RRCSetupComplete) | - | - |
| 11 | The UE relays the RRCSetupComplete message received at step 10 and a REGISTRATION REQUEST message. | --> | SRAP PDU (including NR RRC: RRCSetupComplete) | - | - |

4.9.41.3	Specific message contents

Table 4.9.41.3-1: RRCSetupRequest (steps 3 and 7, Table 4.9.41.2.2-1)

Derivation Path: TS 38.508-1 [4] Table 4.6.1-23 with condition L2RemoteUE

Table 4.9.41.3-2: SidelinkUEInformationNR (Step 4, Table 4.9.41.2.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-28A with condition L2RelayUE

Table 4.9.41.3-3: RRCReconfiguration (step 5, Table 4.9.41.2.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-13 with condition L2RelayUE and RELAY

Table 4.9.41.3-4: RRCReconfiguration (steps 16 and 17, Table 4.9.41.2.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-13 with condition L2RemoteUE and RELAY