3GPP TSG-RAN5 Meeting #102	R5-241613

# Athens, Greece, 26 February-1 March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5277 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of SENSE eMTC TC 6.1.1.14-Periodic SENSE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SENSE-UEContest |  |  |  |  |  | Date: |  |  | 2024-2-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 6.1.1.14 is added to test UE perform periodic PLMN selection for SENSE(signal level enhanced network selection) when signal level for RPLMN is lower than threshold over a period of time in the USIM, even if RPLMN is HPLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of SENSE eMTC TC 6.1.1.14 eMTC / SENSE/ Periodic attempts for signal level enhanced network selection / Automatic mode |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SENSE-UEContest will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.14(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.523-2 CR 1437 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | revised from R5-240420r2 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.1.1.14	eMTC / SENSE/ Periodic attempts for signal level enhanced network selection / Automatic mode

6.1.1.14.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode and RPLMN, HPLMN, UPLMN and OPLMN cells available and signal level enhanced network selection is applicable and the received signal quality of RPLMN is lower than the "Operator controlled signal threshold per access technology" stored in the USIM and the received signal quality of HPLMN and OPLMN are equal to or greater than the "Operator controlled signal threshold per access technology" stored in the USIM }

ensure that {

when { UE is switched on }

then { UE selects a cell of a PLMN, if received signal quality of this PLMN is equal to or greater than the "Operator controlled signal threshold per access technology" stored in the USIM }

}

(2)

with { UE in Automatic network selection mode and UE in RRC_IDLE and UE camped on a HPLMN cell }

ensure that {

when { The received signal quality of the HPLMN is lower than the "Operator controlled signal threshold per access technology" over an averaging window }

then { UE shall periodically attempt to obtain service on an allowable PLMN/access technology combination for which the received signal quality of the candidate PLMN/access technology combination is equal to or greater than the "Operator controlled signal threshold per access technology" }

}

(3)

with { UE in Automatic network selection mode and UE camps on a cell of RPLMN }

ensure that {

when { The received signal quality of the registered PLMN and all other available and allowable PLMN/access technology combinations are lower than the "Operator controlled signal threshold per access technology" }

then { UE stops applying signal level enhanced network selection and selects a cell of the highest priority PLMN after higher priority PLMN search timer T expires. }

}

6.1.1.14.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clauses 4.4.3.1, 4.4.3.5. Unless otherwise stated these are Rel-18 requirements.

[TS 23.122, clause 4.4.3.1]

a)	if

-	signal level enhanced network selection is not applicable (see clause 3.11); or

-	the MS has stopped applying signal level enhanced network selection according to requirement v) of clause 4.4.3.1.1;

then the MS selects the registered PLMN or equivalent PLMN (if it is available) using all access technologies that the MS is capable of without considering the "Operator controlled signal threshold per access technology" stored in the USIM; or

b)	if:

-	signal level enhanced network selection is applicable (see clause 3.11); and

-	the received signal quality from a candidate PLMN/access technology combination comprising of a registered PLMN or an equivalent PLMN (if it is available) is equal to or greater than the "Operator controlled signal threshold per access technology" of the access technology configured in the USIM.

the MS shall select the registered PLMN or equivalent PLMN (if it is available) and the access technology for which the received signal quality is equal to or greater than the "Operator controlled signal threshold per access technology" stored in the USIM. If for the registered PLMN or equivalent PLMN there are two or more access technologies for which the received signal quality is equal to or greater than the "Operator controlled signal threshold per access technology" stored in the USIM, it is up to implementation which access technology is selected by the MS.

[TS 23.122, clause 4.4.3.5]

If signal level enhanced network selection is applicable (see clause 3.11) and the received signal quality of registered PLMN observed over an averaging window is lower than the "Operator controlled signal threshold per access technology" the MS shall periodically attempt to obtain service on an allowable PLMN/access technology combination for which the received signal quality of the candidate PLMN/access technology combination is equal to or greater than the "Operator controlled signal threshold per access technology" in accordance with the requirements that are applicable to i), ii), iii), iv) and v) as defined in the Automatic Network Selection Mode in clause 4.4.3.1.1. For this purpose, the value of the timer TSENSE is configured with an MS implementation specific value with a minimum value of 2 min and a maximum value set to the value applicable for timer T as defined in clause 4.4.3.3.1.1.

The averaging window shall be shorter than the value of the timer TSENSE.

…

The attempts to obtain service on an allowable PLMN shall be as specified below:

a)	The periodic attempts shall only be performed in automatic mode, and not while the MS is attached for emergency bearer services, is registered for emergency services, has a PDN connection for emergency bearer services or has a PDU session for emergency services.

b)	When the MS detects that the received signal quality of the current cell of the registered PLMN is below the "Operator controlled signal threshold per access technology" either upon registration or any time later, the MS shall start timer TSENSE, if not already running.

c)	If upon expiry of timer TSENSE the received signal quality of the registered PLMN observed over an averaging window is equal to or greater than the "Operator controlled signal threshold per access technology" the MS shall stay on the current selected PLMN. If timer T defined in clause 4.4.3.3.1.1 expires while timer TSENSE is running and the received signal quality of registered PLMN observed over an averaging window is lower than the "Operator controlled signal threshold per access technology", the MS shall stop timer TSENSE and shall perform the actions defined in this clause instead of the action defined for timer T expiry defined in clause 4.4.3.3.1.1.

d)	If the received signal quality of the registered PLMN and all other available and allowable PLMN/access technology combinations are lower than the "Operator controlled signal threshold per access technology", the MS shall stop applying signal level enhanced network selection and repeat the network selection procedure as specified in clause 4.4.3.3.1.1.

e)	The attempts shall only be performed by the MS while in idle mode or 5GMM-CONNECTED mode with RRC inactive indication (see 3GPP TS 24.501 [64]).

…

g)	The MS shall limit its attempts to access allowable PLMN/access technology combinations of the same country as the current serving VPLMN, as defined in Annex B.

EXCEPTION: If the MS is in a VPLMN through satellite NG-RAN access or satellite E-UTRAN access with a shared MCC, the MS may attempt to access higher priority PLMN/access technology combinations irrespective of their MCC values.

EXCEPTION: If the MS is in a VPLMN through non-satellite access, the MS may attempt to access higher priority PLMNs with a shared MCC with satellite NG-RAN access technology or satellite E-UTRAN access technology.

6.1.1.14.3	Test description

6.1.1.14.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency multi-PLMN cells as specified in TS 36.508 clause 4.4.1.2 are configured broadcasting PLMNs as indicated in Table 6.1.1.14.3.1-1.

-	The PLMNs are identified in the test by the identifiers in Table 6.1.1.14.3.1-1.

Table 6.1.1.14.3.1–1: PLMN identifiers

| Cell | PLMN name | MCC | MNC |
| --- | --- | --- | --- |
| 1 | PLMN4 | 001 | 01 |
| 12 | PLMN1 | 001 | 11 |
| 13 | PLMN2 | 001 | 21 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is registered to PLMN1 before it is switched off.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 6.1.1.14.3.1-2.

-     The UE is configured for using SENSE as specified in 3GPP TS 24.368 which could be done by MMI or AT command.

Table 6.1.1.14.3.1–2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN4. |  |
| EFPLMNwAcT | 1 | PLMN1 Remaining mandatory entries use default values | E-UTRAN |
| EFOPLMNwACT | 1 | PLMN2 Remaining defined entries use default values | E-UTRAN |
| EFHPLMNwAcT | 1 | PLMN4 | E-UTRAN |
| EFUST |  | Services 20, 42, 43, 74 and 148 are supported. Service 71 is not supported (there is no EHPLMN list). |  |
| EFHPPLMN |  | 1 (2 hours) | Category M1 of E-UTRAN enhanced-MTC |
| EFOCST |  | 1’H(SENSE enabled by operator) 58’H(Operator controlled signal threshold for E-UTRAN is -91dBm) | E-UTRAN |

Preamble:

-	The UE is made to camp on Cell 12 and then Switched OFF (State 1).

6.1.1.14.3.2	Test procedure sequence

Table 6.1.1.14.3.2-1 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1”, “T2” etc are applied at the points indicated in the Main behaviour description in Table 6.1.1.14.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508 Table 6.2.2.1-1.

Table 6.1.1.14.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 12 | Cell 13 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | RS EPRE | dBm/15kHz | “Off” | -85 | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T1 | RS EPRE | dBm/15kHz | -85 | -97 | -85 |  |
| T2 | RS EPRE | dBm/15kHz | -97 | -97 | -85 |  |
| T3 | RS EPRE | dBm/15kHz | -97 | -97 | -97 |  |

Table 6.1.1.14.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.1.1.14.3.2-1 | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3 | Check: Does the UE send an RRCConnectionRequest on Cell 1? | --> | RRCConnectionRequest | 1 | P |
| 4-18 | Steps 3 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 19 | SS adjusts cell levels according to row T2 of table 6.1.1.14.3.2-1 | - | - | - | - |
| 20 | Check: Does the UE send an RRCConnectionRequest on Cell 13 before 7500 seconds (Note 1, Note 3) from power on? | --> | RRCConnectionRequest | 2 | P |
| 21-25 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 13. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| - | EXCEPTION: Step 26a1 describes optional behaviour that depends on the UE capability | - | - | - | - |
| 26a1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS VPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| 27 | SS adjusts cell levels according to row T3 of table 6.1.1.14.3.2-1 | - | - | - | - |
| 28 | Check: Does the UE send an RRCConnectionRequest on Cell 1 after 2min but before 7500 seconds (Note 2, Note 3) from power on? | --> | RRCConnectionRequest | 3 | P |
| 29-33 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| - | EXCEPTION: Step 34a1 describes optional behaviour that depends on the UE capability | - | - | - | - |
| 34a1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS HPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| Note 1: The received signal quality of registered PLMN observed over an averaging window is lower than the "Operator controlled signal threshold per access technology", and UE shall periodically attempt to obtain service on an allowable PLMN/access technology combination for which the received signal quality of the candidate PLMN/access technology combination is equal to or greater than the "Operator controlled signal threshold per access technology" (Refer to TS 23.122 Rel-18). TSENSE is between 120s to T, and averaging window is shorter than TSENSE. Hence, UE attempt to PLMN search before T expire. T is the high priority PLMN search timer defined by EFHPPLMN. Note 2: Following attempts to access the HPLMN/EHPLMN/higher priority PLMN in VPLMN is operator specific setting (Refer to TS 23.122 Rel-12). Hence, window between 120s to T+Tolerance is being used, where the high priority PLMN search timer T defined by EFHPPLMN. Note 3: Tolerance of 5min is added to allow time for the UE to find the proper PLMN |  |  |  |  |  |

6.1.1.14.3.3	Specific message contents

None

<End of modified section 1>