3GPP TSG-RAN5 Meeting #102	R5-241719

# Athens, Greece, 26th Feb – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3082 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to SUL configuration messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For the UE does not support pc_dynamicSwitch_SUL, RF SUL TC may not be operated because SS could not switch UE from NUL to SUL based on UL/SUL indicator field in DCI 0_1 and then start the SUL test. - RF SS may flexibly configure SUL after registration in an RRCReconfiguration message or during registration in an RRCSetup message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - If pc_dynamicSwitch_SUL=False, PUSCH on NUL is released(or not configured) and PUSCH on SUL is configured. - Add new conditions for BWP-UplinkDedicated: For PUCCH: - If PUCCH-Config has been configured previously on the NUL carrier before supplementary carrier is configured, then PUCCH-Config is configured in SUL and PUCCH-Config is released in NUL when supplementary carrier is configured. - If PUCCH-Config has not been configured previously on the NUL carrier before supplementary carrier is configured, then PUCCH-Config is configured in SUL and PUCCH-Config is not configured in NUL when supplementary carrier is configured. For PUSCH & pc_dynamicSwitch_SUL=False: - If PUSCH-Config has been configured previously on the NUL carrier before supplementary carrier is configured, then PUSCH-Config is configured in SUL and PUSCH-Config is released in NUL when supplementary carrier is configured. - If PUSCH-Config has not been configured previously on the NUL carrier before supplementary carrier is configured, then PUSCH-Config is configured in SUL and PUSCH-Config is not configured in NUL when supplementary carrier is configured. For PUSCH & pc_dynamicSwitch_SUL=True: - No change |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SUL teastcases will not works correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-240996 |  |  |  |  |  |  |  |  |

<Start of Changes>

–	BWP-UplinkDedicated

Table 4.6.3-15: BWP-UplinkDedicated

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| setup | PUCCH-Config |  |  |
| release | NULL |  | SUL_NUL AND PucchOnNul |
| } |  |  |  |
| pucch-Config | Not present |  | SUL_NUL AND NoPucchOnNul RF AND Scell_Add |
| pusch-Config | Not present |  | RESUME, SUL_NUL AND NoPuschOnNul AND pc_dynamicSwitch_SUL=False |
| pusch-Config CHOICE { |  |  |  |
| setup | PUSCH-Config |  |  |
| release | NULL |  | SUL_NUL AND PuschOnNul AND pc_dynamicSwitch_SUL=False |
| } |  |  |  |
| configuredGrantConfig | Not present |  |  |
|  | ConfiguredGrantConfig |  | CG_Config |
| srs-Config | Not present |  | Short_DCI |
|  | SRS-Config |  |  |
| beamFailureRecoveryConfig | Not present |  |  |
| sl-PUCCH-Config-r16 | Not present |  |  |
| sl-PUCCH-Config-r16 CHOICE { |  |  | SIDELINK |
| setup | PUCCH-Config |  |  |
| } |  |  |  |
| cp-ExtensionC2-r16 | Not present |  |  |
| cp-ExtensionC3-r16 | Not present |  |  |
| useInterlacePUCCH-PUSCH-r16 | Not present |  |  |
| pucch-ConfigurationList-r16 | Not present |  |  |
| lbt-FailureRecoveryConfig-r16 | Not present |  |  |
| lbt-FailureRecoveryConfig-r16 CHOICE { |  |  | SharedSpectrum |
| setup | LBT-FailureRecoveryConfig |  |  |
| } |  |  |  |
| configuredGrantConfigToAddModList-r16 | Not present |  |  |
| configuredGrantConfigToReleaseList-r16 | Not present |  |  |
| configuredGrantConfigType2DeactivationStateList-r16 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Short_DCI | Used in test scenarios requiring DCI formats 0-0 and 1-0 on USS |
| SUL_NUL | On the NUL carrier when supplementary carrier is configured |
| RESUME | Used in RRCResume Message |
| SIDELINK | Used for sidelink communication |
| SCell_add | Add Scell |
| SharedSpectrum | Operation with shared spectrum channel access |
| CG_Config | Configured Grant Configuration |
| PucchOnNul | PUCCH-Config has been configured previously on the NUL carrier before supplementary carrier is configured |
| NoPucchOnNul | PUCCH-Config has not been configured previously on the NUL carrier before supplementary carrier is configured |
| PuschOnNul | PUSCH-Config has been configured previously on the NUL carrier before supplementary carrier is configured |
| NoPuschOnNul | PUSCH-Config has not been configured previously on the NUL carrier before supplementary carrier is configured |

‘

<End of Changes>