# Title:	Test Tolerances for NR SA FR2 event triggered reporting tests with concurrent measurement gaps with partially overlapping scenario for SSB-based measurements and PRS-based measurement

# Source:	Anritsu

# 1. Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for Inter-frequency event triggered reporting tests with partially overlapping scenario in FR2.

The test cases in question, from TS 38.533, are:

7.6.15.3, NR SA FR2 event triggered reporting tests with concurrent measurement gaps with partially overlapping scenario for SSB-based measurements and PRS-based measurement

The applicability of the analysis is:

UE Power class 3, f ≤40.8GHz

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Test case in Annex A of TS 38.133

The Test Requirements are defined in the following extracts from TS 38.133 v17.13.0 Annex A.

Proposed changes to the specifications are highlighted in yellow.

<< Extract from TS 38.133 >>

#### A.7.6.15.3	SA event triggered reporting tests for FR2 concurrent gap with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement

##### A.7.6.15.3.1	Test Purpose and Environment

The purpose of this test is to verify that the concurrent gap capable UE makes correct reporting of events. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4 and PRS-RSRP measurement delay requirements specified in clause 9.9.3.5.

In this test, there are three cells: NR cell 1 as PCell in FR2 on NR RF channel 1, NR cell 2 as neighbour cell in FR2 on NR RF channel 2, and NR cell 3 as neighbour cell in FR2 on NR RF channel 1.  The test parameters are given in Tables A.7.6.15.3.1-1, A.7.6.15.3.1-2 and A.7.6.15.3.1-3.

Two measurement gap patterns (MeasGapId #0 and MeasGapId #1) are configured with the gap pattern ID #0 and #1 as defined in Table A.7.6.15.3.1-2. MeasGapId #1 is configured with a higher priority than MeasGapId #0. MeasGapId #0 and MeasGapId #1 are associated with the MOs for RF channel numbers #1 and #2, respectively.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used for carrier 2. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2 and NR cell 3. Cell 1 and cell 3 transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance of MeasGapId #1 containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

Table A.7.6.15.3.1-1: SA event triggered reporting tests for FR2

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.6.15.3.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 concurrent gap with partially partial overlapping scenario for SSB-based measurements and PRS measurement

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1: Cell 1 and Cell 3 2: Cell 2 | Two TDD carrier frequencies are used for the NR cells. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | Config 1 | NR cell 2, NR cell 3 | Cell 2 is an inter-frequency cell neighbor cell Cell 3 is a neighbour cell in the positioning assistance data. |
| Gap Pattern Id |  | Config 1 | 0 for MeasGapId #0 1 for MeasGapId #1 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 7 for MeasGapId #0 11 for MeasGapId #1 |  |
| A3-Offset | dB | Config 1 | -6 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between cell 1 and neighbour cell 2 |  | Config 1 | 3s | Synchronous cells. |
| Time offset between cell 1 and neighbour cell 3 |  | Config 1 | 3s | Synchronous cells. |
| Expected RSTD between cell 1 and cell 3 | s | Config 1 | 3 |  |
| Expected RSTD uncertainty between cell 1 and cell 3 | s | Config 1 | 5 |  |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 11 |  |

Table A.7.6.15.3.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gap with partially-partial overlapping scenario for SSB-based measurements and PRS measurement

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |  | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 1 as specified in clause A.3.15 |  |  |  |  |  |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  |  | Rough |  |  | Rough |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  | TDDConf.3.1 |  |
| Duplex mode |  |  | Config 1 | TDD |  |  | TDD |  |  | TDD |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  |  | N/A |  |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | N/A |  |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | N/A |  |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | N/A |  |  |  |  |
| OCNG Patterns |  |  | Config 1 | OP.1 |  |  | OP.1 |  |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  |  | - |  |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  |  | - |  |  | - |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.3.1 TDD |  |  | - |  |  | - |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  |  | - |  |  | - |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  |  | 120 |  |  | 120 |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR2 |  |  | SSB.1 FR2 |  |  | SSB.1 FR2 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.1 |  |  | SMTC.4 |  |  | SMTC.1 |  |
| PRS configuration |  |  | Config 1 | PRS.1.1 FR2 |  |  | N/A |  |  | PRS.1.2 FR2 |  |
| PRS muting configuration |  |  | Config 1 | ‘10’ |  |  | N/A |  |  | ‘01’ |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  |  | 0 |  |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  | -102 |  |  | -104.7 |  |  | -102 |  |
| Note2 |  | dBm/SCS Note4 | Config 1 | -93 |  |  | -95.7 |  |  | -93 |  |
| SSB_-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -89.7 | -89.7 |  | -Infinity |  | -86.7 | -Infinity | -86.7 |
| SSB |  | dB | Config 1 | 3.3 | 3.3 |  | -Infinity |  | 6.3 -> 9.0 | -Infinity | 6.3 |
| PRS-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -Infinity | -96 |  | N/A |  | N/A | -Infinity | -103 |
| PRS |  | dB | Config 1 | -Infinity | -3 |  | N/A |  | N/A | -Infinity | -10 |
| PRS |  | dB | Config 1 | -Infinity | -3 |  | N/A |  | N/A | -Infinity | -10 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -58.56 -> -59.05 |  | -55.32 | -66.7 |  | -57.2 | -57.2 -> -59.05 | -55.32 |
| Propagation Condition |  |  | Config 1 | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |  |  |

##### A.7.6.15.3.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for cell 2, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

10240 for UE supporting power class 1 and 5, or

6400 for UE supporting other power class.

The PRS RSRP measurement time fulfils the requirements specified in Clause 9.9.3.5. The UE shall perform and report the PRS RSRP measurements for Cell 3 with respect to the reference cell in the DL-AoD assistance data, Cell 1, within the time duration specified in section 9.9.3.5 starting from the beginning of time interval T2.

The rate of the correct events for the neighbour cell observed during repeated tests shall be at least 90%, where the reported PRS RSRP measurement for each correct event shall be within the PRS RSRP reporting range specified in Clause 10.1.24.3, i.e., between PRS RSRP_0 and PRS RSRP_126.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

IUE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

# 3. Discussion

The test has 2 time periods, T1 and T2. To analyse the test case, it is helpful to show the values of SSB_RP for Cells 1 and 2 versus time.

Observations

During T1 only NR Cell 1 is present and the nominal Ês/Iot value is +3.3 dB.

During T2, the SSB_RP of NR Cell 2 is 3dB higher than NR Cell 1. The “Event A3 (Neighbour becomes offset (-6dB) better than SpCell)” condition is contained in is easily met, but the effect of uncertainties on the test verdict needs to be checked. A decision can then be made on whether Test Tolerances need to be applied.

A detailed analysis of the test case is given in section 4 of this document.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Tables A.7.6.15.3.1-1, A.7.6.15.3.1-2 and A.7.6.15.3.1-3 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test. The angle of arrival of the radiated signals is also recorded.

A number of UE core requirement values and their associated parameters are also entered in this section, taking into account the angle of arrival of the radiated signals. The requirements are entered because they affect the radiated signal received by the UE, or affect how the UE sees the received signals at baseband.

The key parameters appear in section a) of the accompanying spreadsheet.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

Some UE core requirements in section a) are converted into UE-related derived parameters, in a form that allows them to be used in later sections of the spreadsheet. The rationale and method for UE gain G midpoint is given in TR 38.903 clause A.2.6, and for UE Spherical coverage gain midpoint in TR 38.903 clause A.2.1.

In this test case at the start of T2, Cell 2 becomes detectable and the UE is expected to detect and send an event triggered measurement report. The criterion is Event A3: Neighbour becomes amount of offset better than SpCell. The UE makes a measurement of NR Cell 2 (neighbour) relative to NR Cell 1 (PSCell) and compares it to the signalled A3 offset. A further step is done to calculate SS-RSRP power difference including the offset:

- (Cell 2 SS-RSRP – Cell 1 SS-RSRP) – Ofn (A3 offset)

c) Uncertainties

The Test system provides 3 cells, cell 1/cell 3 on carrier frequency 1 and cell 2 on carrier frequency 2, with AWGN. We propose to control the following parameters:

- AWGN absolute power on carrier frequency 1, Noc1 averaged over BWConfig uncertainty: ±5.65* dB

- AWGN absolute power on carrier frequency 1, Noc1 for PRBs #RBJ-RBJ+19 uncertainty: ±5.65* dB

- AWGN absolute power on carrier frequency 2, Noc2 averaged over BWConfig uncertainty: ±5.65* dB

- AWGN absolute power on carrier frequency 2, Noc2 for PRBs #RBJ-RBJ+19 uncertainty: ±5.65* dB

- Ratio of cell 1 signal / AWGN, Ês1 / Noc1 averaged over BWConfig uncertainty: ±0.3 dB

- Ratio of cell 1 signal / AWGN, Ês1 / Noc1 for PRBs #RBJ-RBJ+19 uncertainty: ±0.3 dB

- Ratio of cell 2 signal / AWGN, Ês2 / Noc2 averaged over BWConfig uncertainty: ±0.3 dB

- Ratio of cell 2 signal / AWGN, Ês2 / Noc2 for PRBs #RBJ-RBJ+19 uncertainty: ±0.3 dB

- Ratio of cell 3 signal / AWGN, Ês3 / Noc3 averaged over BWConfig uncertainty: ±0.3 dB

- Ratio of cell 3 signal / AWGN, Ês3 / Noc3 for PRBs #RBJ-RBJ+19 uncertainty: ±0.3 dB

*These values are applicable for frequencies <= 40.8 GHz.

In this test the UE measures the SS-RSRP over specific Physical Resource Block (PRB) numbers #RBJ to #RBJ+19. In addition, this test has separate constraints on the overall power in the configured bandwidth Io.

This choice forms a minimum set, so the superposition principle can be applied if necessary.

UE-related uncertainties are considered, together with selected UE core requirements that have an allowed range, and are therefore handled as a UE uncertainty:

- UE SS-RSRP relative accuracy: Value taken from TS 38.133

- Spherical coverage gain variation: Value derived, using method defined in TR 38.903 A.2.1

- UE gain G variation: Value derived, using method defined in TR 38.903 A.2.6

<< Extracts from TS 38.133 v17.13.0 >>

##### 10.1.5.1.2	Relative SS-RSRP Accuracy

The relative accuracy of SS-RSRP is defined as the SS-RSRP measured from one cell on a frequency in FR2 compared to the SS-RSRP measured from another cell on another frequency in FR2.

The accuracy requirements in Table 10.1.5.1.2-1 are valid under the following conditions:

-	Conditions defined in 38.101-2 [19] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27dB

-	| Channel 1_Io -Channel 2_Io |  20 dB

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.5.1.2-1: SS-RSRP Inter frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 6 | 9 | ≥-4 | Same value as SSB_RP in Table B.2.3-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. |  |  |  |  |  |

<< clauses skipped >>

Table B.2.3-2: Conditions for inter-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 1, 2, 3, 4, 5 |  |
| Conditions | Rx Beam Peak | n257 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 | -121.4+Y5 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 | -121.6+Y5 |  |  |
|  |  | n259 |  |  | -106.5 |  | -118.5+Y5 |  |  |
|  |  | n260 | -123.3+Y1 |  | -107.5 | -123.8+Y4 |  |  |  |
|  |  | n261 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  |  |  |
|  |  | n262 | -121.3+Y1 | -106.6 | -104.6 | -119.8+Y4 |  |  |  |
|  | Spherical coverage Note 1 | n257 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 | -113.4+Z5 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 | -113.6+Z5 |  |  |
|  |  | n259 |  |  | -93.7 |  |  |  |  |
|  |  | n260 | -115.3+Z1 |  | -94.9 | -111.8+Z4 |  |  |  |
|  |  | n261 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 |  |  |  |
|  |  | n262 | -113.1+Z1 | -94.7 | -91.5 | -107.7+Z4 |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |

Normal conditions are selected.

In addition, Cell 2 needs to be detectable so that the requirement of inter-frequency measurement can apply. The condition that an inter-frequency cell is detectable is defined in TS 38.133 clause 9.3.2.

### 9.3.2	Requirements applicability

The requirements in clause 9.3 apply, provided:

-	The cell being identified or measured is detectable.

An inter-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in clauses 10.1.4 and 10.1.5 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in clauses 10.1.9 and 10.1.10 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in clauses 10.1.14 and 10.1.15 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.3 for a corresponding Band.

The side conditions mentioned above refer to Annex B.2.3, which is already considered.

d) Controlled parameters critical to verdict

In many RRM test cases there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict, and also the side conditions that should be met for the UE to be tested correctly. The 15 controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given in the “Requirement” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB.

Where there is a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict, the sensitivity factors are derived by inspection as one, or if no effect, as zero.

Where there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict, the calculation of sensitivity factors is done using a method selected from TR 38.903. Cell 1, Cell 2 and Cell 3 Es/Iot at UE baseband is calculated according to TR 38.903 A.2.4, and applied Io calculated according to TR 38.903 A.2.5.

The test system uncertainties are combined root-sum-square as they are uncorrelated. More detailed explanation is given in the accompanying spreadsheet, and in TR 38.903 A.2.7.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment, and the UE-related uncertainties where relevant. This process follows the superposition principle. More details and explanation can be found in section 4 of TR 38.903.

e) Determine parameters to offset

During T1 the UE is required to select Cell 1 as PCell. The uncertainties do not take Cell 1 Ês/IotBB or SSB_RP power outside the allowed range, therefore no offsets are required.

During T2 the Cell 2 becomes detective. UE is expected to perform inter-frequency measurement. The nominal conditions in TS 38.133 Table A.7.6.15.3.1-3 give SSB_RP values, as measured by the UE, of -89.7 dBm/120kHz and -86.7 dBm/120kHz, which are 3 dB apart nominally.

The Event A3 measurement report trigger requirement in TS 38.331 clause 5.5.4.4 is “Event A3 (Neighbour becomes offset better than SpCell), inequality A3-1 (Entering condition) is: Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off”.

The values in this equation are derived as follows:

- Mn is evaluated for Cell 2 at T2 and is nominally -86.7 dBm/120kHz

- Ofn is the measurement object specific offset offsetMO of the reference signal of the neighbour cell. The default value is specified as 0dB in TS 38.508-1 Table 4.6.3-76.

- Ocn is the cell specific offset cellIndividualOffset of the neighbour cell and set to zero if not configured for the neighbour cell.

- Hys is the hysteresis parameter for this event, specified as 0dB in TS 38.133 Table A.7.6.15.3.1-2.

- Mp is evaluated for Cell 1 at T2 and is nominally -89.7 dBm/120kHz

- Ofp is the measurement object specific offset offsetMO of the SpCell. The default value is specified as 0dB in TS 38.508-1 Table 4.6.3-76.

- Ocp is the cell specific offset cellIndividualOffset of the serving cell and set to zero if not configured for the neighbour cell.

- Off is the offset parameter for this event (i.e. A3-Offset), specified as -6dB in TS 38.133 Table A.7.6.15.3.1-2.

The equation therefore reduces to:

Mn > Mp + Off. Putting in values and rearranging gives -86.7dBm - (-89.7)dBm > -6dB.

This inequality is satisfied by 9dB under nominal conditions. The overall uncertainty for UE Measured (Cell 2 RSRP - Cell 1 RSRP), taking into account both test system and UE measurement uncertainties, is ±14.00dB as calculated in step d) of the accompanying spreadsheet. The variation due to uncertainties would mean that the entering condition might not be met, and Event A3 would not occur. The signalled value of A3-Offset is therefore decreased by 6dB to ensure the entering condition is met.

During T2, since Cell 3 becomes detective with relatively larger SSB Es/Iot compared to Cell 1, also both since SSBs for Cell 1 and Cell 3 are configured with the same condition along with the same frequency channel, SSBs for both Cell 1 and Cell 3 become an interferer each other. Therefore, to fulfil the side condition of minimum SSB Es/Iot at UE baseband in TS 38.133 B.2.3 and the maximum Io in TS 38.133 10.1.5.1 after taking into account of UE measurement uncertainties, it is proposed to decrease the SSB_RP of Cell 3 by 0.6 dB.

During T2, PRS for Cell 3 falls below the minimum side condition of PRS-based measurement in TS 38.133 B.2.14. To fulfil the side condition, it is proposed to increase the PRS Es for cell 3 by 0.2 dB.

The check that all other controlled parameters meet their required range is done in step g). In theory it is possible for steps e) to g) to be iterative, or possibly even steps c) to g) to be iterative, but this test case can be done without such iteration.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b).

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated, as some will have changed from the original due to the application of the Test Tolerances in step f).

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (Es/IotBB, Applied SSB_RP, Applied SSB_RP difference, Applied Io, Applied Io difference and Event A3 entering condition). The critical requirement for each parameter is given in the “Requirement” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all the requirements are met, then the chosen offsets are acceptable. It can be seen that with the uncertainty values and Test Tolerances proposed, the requirements are met.

# 5. Treatment of other configurations within the same test case

Not applicable since there is only one test configuration in this test case.

# 6. Treatment of related test cases

Not applicable.