3GPP TSG-RAN5 Meeting #103 	R5-243013

# Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2799 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update for EVM including symbols with transient period |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As clarified in R5-226794, TC 6.4.2.1a EVM including symbols with transient period is incomplete due to several issues. - Test configuration for RB allocation change is missing. - Figure of transient period may mislead that the test is like on/off time mask although the actual test is on-to-on transient. - Test procedure is described to change the output power by TPC command instead of the RB allocation change. - Baseline of the output power levels to transit is not defined. - UL/DL configuration pattern needs to be decided. - Clarification is necessary on a way to calculate EVM whether to include only rising edge of transient or falling edge, or both. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 6.4.2.1a was updated to correct the above issues and completed. A.2.1 was updated to include the UL RMCs for 6.4.2.1a. MTSU and TT for 6.4.2.1a were defined in Table F.1.2-1 and Table F.3.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC 6.4.2.1a will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.1a, A.2.1, F.1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.4.2.1a	Error Vector Magnitude including symbols with transient period

6.4.2.1a.1	Test Purpose

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM.

6.4.2.1a.2	Test applicability

This test case applies to all types of NR Power Class 1 UE release 16 and forward that support short transient period capability.

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 16 and forward that support short transient period capability and don’t support Tx diversity..

6.4.2.1a.3	Minimum conformance requirements

In 6.4.2.1, EVM has been defined by excluding the symbols which have a transient period. In this section, measurement interval is defined for the symbols with a transient period to include these symbols in the RMS average EVM computation when the UE reports a transient period capability other than the default. Before calculating the EVM, the measured waveform is corrected for sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM. The symbols with transient period should not be used for equalization. Only CP-OFDM waveform is used for conformance testing.

In the case of PUSCH or PUCCH transmissions when the mean power, modulation or RB allocation across slot or subslot boundaries is expected to change the EVM result over the symbols where the transient occurs is calculated according to Table 6.4.2.1a.3-1.

Table 6.4.2.1a.3-1: EVM definition for reported transient period

| Reported transient capability (us) | EVM definition | tpstart (µs) | SCS4 |
| --- | --- | --- | --- |
| 2 | EVMafter=max⁡(EVMl_tp,EVMh) EVMbefore=max⁡(EVMl,EVMh_tp) | -0.5 | 15kHz or 30kHz5 |
| 4 | EVMafter=max⁡(EVMl_tp,EVMh) EVMbefore=max⁡(EVMl,EVMh_tp) | -1 | 15kHz |
| 7 | EVMafter=min⁡(EVMl_tp,EVMh) EVMbefore=max⁡(EVMl,EVMh_tp) | -2.7 | 15kHz |
| NOTE 1: EVMl , EVMh, EVMl_tp,and EVMh_tp are defined in Annex E.4.7 NOTE 2: EVMafter is the EVM for a symbol right after a transition; EVMbefore is the EVM for a symbol right before a transition NOTE 3: tpstart denotes the start position of the EVM exclusion window as shown in Annex E.4.7 NOTE 4: SCS denotes the SCS that can be used in the conformance test NOTE 5: 30kHz shall be used in the conformance test unless the UE signals in supportedSubCarrierSpacingUL in FeatureSetPerCC that it only supports 15kHz in the corresponding band |  |  |  |

The RMS average of the basic EVM measurements over 108 subframes calculated only on the symbols where the transient occurs for the different modulation schemes shall not exceed the values specified in Table 6.4.2.1a.3-2 for the parameters defined in Table 6.4.2.1a.3-3.  This requirement can be verified with 64 QAM and 256 QAM modulation.

Table 6.4.2.1a.3-2: Requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| 64 QAM | % | 10 |
| 256 QAM | % | 8 |

Table 6.4.2.1a.3-3: Parameters for Error Vector Magnitude

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | Table 6.3.1-1 |
| UE Output Power for 256 QAM | dBm | Table 6.3.1-1 + 10 dB |
| Operating conditions |  | Normal conditions |

The normative reference for this requirement is TS 38.101 [2] clause 6.4.2.1a.

6.4.2.1a.4	Test description

6.4.2.1a.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.4.2.1a.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4.2.1a.4.1-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range (NOTE 4) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | 15 kHz (Note 3) |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM 64 QAM | 24@05 and 1@06 |
| 2 |  | CP-OFDM 256 QAM | 24@05 and 1@06 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each NR band, which applicable channel bandwidths are specified in Table 5.3.5-1. NOTE 3: For UE supporting 2 us transient period, 30kHz shall be used in the conformance test unless the UE signals in supportedSubCarrierSpacingUL in FeatureSetPerCC that it only supports 15kHz in the corresponding band. NOTE 4: For NR band n28, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. NOTE 5: Applicable to slots 3 and 9 for SCS 15 kHz, and slots 8 and 19 for SCS 30 kHz. NOTE 6: Applicable to slots 4 and 8 for SCS 15 kHz, and slots 9 and 18 for SCS 30 kHz. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.4.2.1a.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.4.2.1a.4.3

6.4.2.1a.4.2	Test procedure

Symbol after transient

Symbol before transient

Assigned UL slot

Transient period

Transient period

Figure 6.4.2.1a.4.2-1: Error Vector Magnitude including symbols with transient period

Figure 6.4.2.1a.4.2-2: Test power patterns for Error Vector Magnitude including symbols with transient period

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.4.2.1a.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The UL assignment is such that the UE transmits on slots 3, 4, and 8, 9 for 15 kHz SCS, on slots 8, 9, and 18, 19 for 30 kHz SCS.

2.	Set Uplink RB allocation as 24@0 as defined in Table 6.4.2.1a.4.1-1. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level +11.5 dBm, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.2-1 for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

3.	Schedule the UE's PUSCH data transmission as described in Figure 6.4.2.1a.4.2-2 and Uplink RB allocation pattern as defined in Table 6.4.2.1a.4.1-1 while transmitting 0 dB TPC command for PUSCH via the PDCCH.

4.	Measure the EVMbefore and EVMafter using Global In-Channel Tx-Test (Annex E) applying tpstart of Table 6.4.2.1a.3-1 according to the declared enhanced transient capability.

5.	Repeat test steps 3~4 with UL RMC according to Table 6.4.2.1a.4.1-1 until the SS collect enough EVMbefore and EVMafter.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.3.

6.4.2.1a.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exceptions:

Table 6.4.2.1a.4.3-1: TDD-UL-DL-Config

| Derivation Path: TS 38.508-1[5], Table 4.6.3-192 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigCommon ::= SEQUENCE { |  |  |  |
| referenceSubcarrierSpacing | SubcarrierSpacing |  |  |
| pattern1 SEQUENCE { |  |  |  |
| dl-UL-TransmissionPeriodicity | ms5 |  | FR1_15kHz |
|  | ms5 |  | FR1_30kHz |
| nrofDownlinkSlots | 2 |  | FR1_15kHz |
|  | 7 |  | FR1_30kHz |
| nrofDownlinkSymbols | 10 |  | FR1_15kHz |
|  | 6 |  | FR1_30kHz |
| nrofUplinkSlots | 2 |  | FR1_15kHz |
|  | 2 |  | FR1_30kHz |
| nrofUplinkSymbols | 2 |  | FR1_15kHz |
|  | 4 |  | FR1_30kHz |
| } |  |  |  |
| pattern2 | Not present |  |  |
| } |  |  |  |

6.4.2.1a.5	Test requirement

The EVMbefore and EVMafter derived in Table 6.4.2.1a.3-1 shall not exceed the values in Table 6.4.2.1a.5-1. Both rising and falling edges of transient occurring at the respective slot boundaries are considered to have the same EVM requirements.

Table 6.4.2.1a.5-1: Test requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| 64 QAM | % | 10 + TT |
| 256 QAM | % | 8 + TT |
| Note 1: TT is defined in Table 6.4.2.1a.5-2. |  |  |

Table 6.4.2.1a.5-2: Test Tolerance

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| 64 QAM | % | 0 |
| 256 QAM | % | 0 |

<<Unchanged sections skipped>>

A.2.1	General

The measurement channels in the following subclauses are defined to derive the requirements in clause 6 (Transmitter Characteristics) and clause 7 (Receiver Characteristics). The measurement channels represent example configurations of physical channels for different data rates.

The measurement channels in the following clauses are applicable to both FDD and TDD.

The active uplink slots for TDD configurations are specified in table A.2.1-1. TDD slot patterns defined for reference sensitivity tests will be used for TDD UL RMCs, unless otherwise stated. The active uplink slots for additional TDD configuration specified in Table A.2.1-3, with the TDD pattern defined in Table A.2.1-4, is used for the phase continuity tests for DMRS bundling with 15 kHz SCS. The active uplink slots configuration specified in Table A.2.1-5 and the additional TDD pattern in Table A.2.1-6 are used for shorter transient period capability EVM tests at 15 kHz SCS.

Table A.2.1-1: TDD active uplink slots

| SCS | Active Uplink slots |
| --- | --- |
| 15 kHz | 4, 9 |
| 30 kHz | 8, 9, 18, 19 |
| 60 kHz | 16, 17, 18, 19, 36, 37, 38, 39 |

Table A.2.1-2: TDD active uplink slots for PC1.5 UE with maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16≤20%

| SCS | Active Uplink slots |
| --- | --- |
| 15 kHz | 4 |
| 30 kHz | 8, 9 |
| 60 kHz | 16, 17, 18, 19 |

Table A.2.1-3: TDD active uplink slots for additional TDD configuration

| SCS | Active Uplink slots |
| --- | --- |
| 15 kHz | 8, 9 |

Table A.2.1-4: TDD pattern for additional TDD configuration

| Parameter |  | Value |
| --- | --- | --- |
|  |  | SCS 15 kHz (µ0) |
| TDD Slot Configuration pattern (Note 1) |  | 7DS2U |
| Special Slot Configuration (Note 2) |  | 6D+4G+4U |
| referenceSubcarrierSpacing |  | 15 kHz |
| UL-DL configuration | dl-UL-TransmissionPeriodicity | 10 ms |
|  | nrofDownlinkSlots | 7 |
|  | nrofDownlinkSymbols | 6 |
|  | nrofUplinkSlot | 2 |
|  | nrofUplinkSymbols | 4 |
| NOTE 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. NOTE 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. |  |  |

Table A.2.1-5: TDD active uplink slots for shorter transient period capability

| SCS | Active Uplink slots |
| --- | --- |
| 15 kHz | 3,4 |

Table A.2.1-6: Additional TDD pattern for shorter transient period capability

| Parameter |  | Value |
| --- | --- | --- |
|  |  | SCS 15 kHz (µ0) |
| TDD Slot Configuration pattern (Note 1) |  | 2DS2U |
| Special Slot Configuration (Note 2) |  | 10D+2G+2U |
| referenceSubcarrierSpacing |  | 15 kHz |
| UL-DL configuration | dl-UL-TransmissionPeriodicity | 5 ms |
|  | nrofDownlinkSlots | 2 |
|  | nrofDownlinkSymbols | 10 |
|  | nrofUplinkSlot | 2 |
|  | nrofUplinkSymbols | 2 |
| NOTE 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. NOTE 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. |  |  |

<<Unchanged sections skipped>>

F.1.2	Measurement of transmitter

- MU and TT for >6GHz (band n96) are working assumption based on analysis of single TE vendor. Values will be revisited once analysis from other TE vendors is available.

- MU and TT for spurious emission for intra-band UL contiguous CA with UL-MIMO test cases are working assumption. Values will be revisited once more analysis is available.

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| <<Unchanged rows skipped>> |  |  |
| 6.4.2.1a Error Vector Magnitude including symbols with transient period | ±2.28 % Absolute Uplink power measurement same as 6.3.1. Relative Uplink power measurement same as 6.3.4.3. |  |
| <<Unchanged rows skipped>> |  |  |

<<Unchanged sections skipped>>

F.3.2	Measurement of transmitter

- MU and TT for >6GHz (band n96) are working assumption based on analysis of single TE vendor. Values will be revisited once analysis from other TE vendors is available.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| <<Unchanged rows skipped>> |  |  |
| 6.4.2.1a Error Vector Magnitude including symbols with transient period | 0% | Same as 6.4.2.1 |
| <<Unchanged rows skipped>> |  |  |

<<End of change>>