3GPP TSG-RAN5 Meeting #103	R5-243431

# Fukuoka City, Japan, May 20-24, 2024

Agenda item:	5.3.8.9

Source: 	Qualcomm Inc

Title: 	Simulation results to derive min test time for nr-ntn pdsch demod test cases

Document for:	Discussion and Endorsement

## 1.	Introduction

TS 38.521-5 [3] introduced NTN-TDL channel models for PDSCH Demodulation test cases with satellite access based on the requirements defined in [4]. This paper discusses the simulation results to derive the minimum test time for these NR-NTN test cases.

## 2.	Discussion

From [1] and [2], we can recall the procedure used to come up with the minimum test time for PDSCH throughput test cases.

- Run the PDSCH simulations for an extended period of time at the SNR of interest for multiple seeds.

- The minimum test time is determined by the time it took the throughput curve to settle within +/- 2% of the target throughput value across all seeds.

- If the throughput does not settle within the +/- 2% throughput limit within a reasonable amount of time for any seed, expand the threshold to +/- 3.5% of the target throughput value.

- To account for the additional 1.5% throughput variation, determine from the Throughput vs SNR curve, what is the SNR delta corresponding to this 1.5 % throughput increase.

- That SNR value is the added SNR uncertainty due to finite test time.

PDSCH simulations for all the defined NR-NTN PDSCH demod test cases in [3] were run.

Table 8.2.1.2.2.1.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | 0.3 |
| 1-2 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | 7.6 |
| 1-3 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | -0.4 |
| 1-4 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | 1.1 |

Figure1: PDSCH Throughput Simulations for test 1-1 (NTN-TDLA100-200) at 70% target throughput SNR, 30 seeds

Figure2: PDSCH Throughput Simulations for test 1-2 (NTN-TDLC5-200) at 70% target throughput SNR, 30 seeds

Figure3: PDSCH Throughput Simulations for test 1-3 (NTN-TDLC5-200) at 70% target throughput SNR, 30 seeds

Figure4: PDSCH Throughput Simulations for test 1-4 (NTN-TDLA100-200) at 70% target throughput SNR, 30 seeds

## 3.	Conclusion

Proposal 1: min test time to be updated based on the above simulation results

## 4.		References

[1]	R5-191250, Discussion paper on statistical tput calculation for demod test cases, Qualcomm

[2]	R5-194687, Discussion on minimum test time for PDSCH and PDCCH with more simulation results, Qualcomm

[3]	TS 38.521-5 v18.1.0

[4]	TS 38.101-5 v18.5.0