3GPP TSG-RAN5 Meeting #104	R5-245219r2

# Maastricht, Netherlands, 19th Aug 2024 – 23th Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-2 | CR | 0509 | rev |  | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add test applicability for new multi-cell scheduling with a single DCI test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Single DCI test case 7.1.1.2.8.X, 7.1.1.2.9.X, 7.1.1.7.2.X, 7.1.1.7.3.X, 7.1.1.4.X, 7.1.1.7.5.X do not have test applicability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add test applicability for new multi-cell scheduling with a single DCI test case 7.1.1.2.8.X, 7.1.1.2.9.X, 7.1.1.7.2.X, 7.1.1.7.3.X, 7.1.1.4.X, 7.1.1.7.5.X |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_MC_enh-UEConTest WI is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-3, 38.508-2 CR 3507,0678 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | A.4.3.2-1/XXHS1- A.4.3.2-1/XXHS8 are fixed in 38.508-2 CR 0678 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - There are two CXXXHS1, the second should be updated to CXXXHS11 - Add “A.4.3.2A.1-1/1” in Test case Selection Expression column for “DL NR CA with 2 carriers” in comment column. 2nd revision: Delete the ENDC test applicability. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Table 4.1-2a: Applicability of Protocol conformance Layer 2 test cases, ref. TS 38.523-1 [2]

| Clause | TC Title | Release | Applicability Condition | Applicability Comment |
| --- | --- | --- | --- | --- |
| 7 | Layer 2 |  |  |  |
| 7.1 | NR Layer 2 |  |  |  |
| 7.1.1 | MAC |  |  |  |
| 7.1.1.1 | Random Access Procedures |  |  |  |
| 7.1.1.1.1 | Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by RRC / contention free random access procedure | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.1.1a | Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by PDCCH Order / contention free random access procedure | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.1.2 | Random access procedure / Successful / C-RNTI Based / Preamble selected by MAC itself | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.1.3 | Random access procedure / Successful / SI request | Rel-15 | C21 | UEs supporting 5G Core |
| 7.1.1.1.4 | Random access procedure / Successful / Beam Failure / Preamble selected by MAC itself / non-Contention Free RACH procedure | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.1.5 | Random access procedure / Successful / Supplementary Uplink | Rel-15 | C28 | UEs supporting 5GS and NR SUL and supplemental uplink with dynamic switch |
| 7.1.1.1.6 | Random access procedure / Successful / Temporary C-RNTI Based / Preamble selected by MAC itself | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.1.7 | Random access procedure / 2-step RACH / RA_TYPE selection | Rel-16 | C135 | UEs supporting 2-Step RACH |
| 7.1.1.1.8 | Correct selection of RACH parameters / 2-step RACH/MSGA and PRACH resource explicitly signalled to the UE by RRC / contention free random access procedure | Rel-16 | C135 | UEs supporting 2-Step RACH |
| 7.1.1.1.9 | Random access procedure / Successful / 2-step RACH/C-RNTI Based / Preamble selected by MAC itself | Rel-16 | C135 | UEs supporting 2-Step RACH |
| 7.1.1.1.9a | Random access procedure / 2-step RACH / Successful / RRC_IDLE | Rel-16 | C135A | UEs supporting 5G Core and 2-Step RACH |
| 7.1.1.1.10 | Random access procedure / 2-step RACH/not complete/ RA_TYPE to 4-stepRA | Rel-16 | C135 | UEs supporting 2-Step RACH |
| 7.1.1.1.10a | Random access procedure / 2-step RACH/ Fallback for CBRA | Rel-16 | C135A | UEs supporting 5G Core and 2-Step RACH |
| 7.1.1.1.11 | Random access procedure / Successful / Slice specific RACH configuration | Rel-17 | C262 | UEs supporting slice-based RACH partitioning and slice-based RACH prioritisation |
| 7.1.1.1.12 | Random access procedure / Successful / ra-PrioritizationForSlicing | Rel-17 | C263 | UEs supporting slice-based RACH partitioning, slice-based RACH prioritisation and RACH prioritisation for Access Identity 1 |
| 7.1.1.1.13 | Random access procedure / Successful / Slice specific RACH configuration / 2-step RACH | Rel-17 | C264 | UEs supporting 2-Step RACH, slice-based RACH partitioning and slice-based RACH prioritisation |
| 7.1.1.1.14 | Random access procedure / Successful / ra-PrioritizationForSlicingTwoStep / 2-step RACH | Rel-17 | C265 | UEs supporting 2-Step RACH, slice-based RACH partitioning, slice-based RACH prioritisation and RACH prioritisation for Access Identity 1 |
| 7.1.1.1.15 | Random access procedure / RedCap UE / SI request | Rel-17 | C212 | UEs supporting 5G Core and RedCap |
| 7.1.1.1.16 | Random access procedure / RedCap UE identification / Msg3-based / CCCH1 | Rel-17 | C212a | UEs supporting 5G Core and RedCap and RRC_INACTIVE |
| 7.1.1.1.17 | Random access procedure / RedCap UE identification | Rel-17 | C212 | UEs supporting 5G Core and RedCap |
| 7.1.1.1.18 | Random access procedure / Msg3 repetition indication / Random access resources selection | Rel-17 | C211 | UEs supporting repetition of Message 3 PUSCH |
| 7.1.1.1.19 | Random access procedure / Successful / Beam Failure / Unified TCI | Rel-17 | C311 | UEs supporting 5GS and unified TCI state operation with joint DL/UL TCI update for intra-cell beam management |
| 7.1.1.2 | Downlink Data Transfer |  |  |  |
| 7.1.1.2.1 | Correct Handling of DL MAC PDU / Assignment / HARQ process | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.2.2 | Correct Handling of DL HARQ process PDSCH Aggregation | Rel-15 | C20 | UEs supporting 5GS and PDSCH aggregation |
| 7.1.1.2.3 | Correct HARQ process handling / CCCH | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.2.4 | Correct HARQ process handling / BCCH | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.2.5 | Correct HARQ process handling / DL grant prioritization | Rel-16 | C179 | UEs supporting DCI DL Priority Indicator |
| 7.1.1.2.6 | Correct HARQ process handling / dynamic PUCCH repetition indication | Rel-17 | C287 | UEs supporting dynamic indication of PUCCH repetition |
| 7.1.1.2.7 | Correct HARQ process handling / Unified TCI Activation | Rel-17 | C312 | UEs supporting 5GS and unified separate TCI with multi-MAC-CE |
| 7.1.1.2.8 | Multi-cell PDSCH scheduling with a single DCI / FDRA field based |  |  |  |
| 7.1.1.2.8.1 | Multi-cell PDSCH scheduling with a single DCI / FDRA field based / Intra-band Contiguous CA | Rel-18 | CXXXHS1 | UEs supporting 5G Core and intra-band contiguous CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set and co-scheduled cell indication scheme based on FDRA field of DCI format 1_3 |
| 7.1.1.2.8.2 | Multi-cell PDSCH scheduling with a single DCI / FDRA field based / Inter-band CA | Rel-18 | CXXXHS3 | UEs supporting 5G Core and inter-band CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set and co-scheduled cell indication scheme based on FDRA field of DCI format 1_3 |
| 7.1.1.2.8.3 | Multi-cell PDSCH scheduling with a single DCI / FDRA field based / Intra-band non-Contiguous CA | Rel-18 | CXXXHS5 | UEs supporting 5G Core and intra-band non-contiguous CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set and co-scheduled cell indication scheme based on FDRA field of DCI format 1_3 |
| 7.1.1.2.9 | Multi-cell PDSCH scheduling with a single DCI / Scheduled cell indicator field based |  |  |  |
| 7.1.1.2.9.1 | Multi-cell PDSCH scheduling with a single DCI / Scheduled cell indicator field based / Intra-band Contiguous CA | Rel-18 | CXXXHS7 | UEs supporting 5G Core and intra-band contiguous CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set and co-scheduled cell indication scheme based on Scheduled cells indicator field of DCI format 1_3 |
| 7.1.1.2.9.2 | Multi-cell PDSCH scheduling with a single DCI / Scheduled cell indicator field based / Inter-band CA | Rel-18 | CXXXHS9 | UEs supporting 5G Core and inter-band CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set and co-scheduled cell indication scheme based on Scheduled cells indicator field of DCI format 1_3 |
| 7.1.1.2.9.3 | Multi-cell PDSCH scheduling with a single DCI / Scheduled cell indicator field based / Intra-band non-Contiguous CA | Rel-18 | CXXXHS11 | UEs supporting 5G Core and intra-band non-contiguous CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set and co-scheduled cell indication scheme based on Scheduled cells indicator field of DCI format 1_3 |
| 7.1.1.3 | Uplink Data Transfer |  |  |  |
| 7.1.1.3.1 | Correct Handling of UL MAC PDU / Assignment / HARQ process | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.3.2 | Logical channel prioritization handling | Rel-15 | C02 | UEs supporting 5GS and RLC UM Mode |
| 7.1.1.3.2b | Logical channel prioritization handling with Mapping restrictions | Rel-15 | C175 | UEs supporting 5GS and selection of logical channels for each UL grant based on RRC configured restriction |
| 7.1.1.3.3 | Correct handling of MAC control information / Scheduling requests | Rel-15 | C53 | UEs supporting 5GS and Logical Channel SR-Delay Timer |
| 7.1.1.3.4 | Correct handling of MAC control information / Buffer status / UL data arrive in the UE Tx buffer / Regular BSR | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.3.5 | Correct handling of MAC control information / Buffer Status / UL resources are allocated / Padding BSR | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.3.6 | Correct handling of MAC control information / Buffer status / Periodic BSR timer expires | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.3.7 | UE power headroom reporting / Periodic reporting / DL pathloss change reporting | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.3.8 | UE power headroom reporting / SCell activation / DL pathloss change reporting |  |  |  |
| 7.1.1.3.8.1 | UE power headroom reporting / SCell activation / DL pathloss change reporting / Intra-band Contiguous CA | Rel-15 | C81 | UEs supporting 5GCore and intra-band contiguous CA and UL NR CA with 2 carriers |
|  |  |  | C81A | UEs supporting EN-DC and intra-band contiguous CA and EN-DC with 2 NR UL carriers |
| 7.1.1.3.8.2 | UE power headroom reporting / SCell activation / DL pathloss change reporting / Inter-band CA | Rel-15 | C82 | UEs supporting 5GCore and inter-band CA and UL NR CA with 2 carriers |
|  |  |  | C82A | UEs supporting EN-DC and inter-band CA and EN-DC with 2 NR UL carriers |
| 7.1.1.3.8.3 | UE power headroom reporting / SCell activation / DL pathloss change reporting / Intra-band non Contiguous CA | Rel-15 | C83 | UEs supporting 5GCore and intra-band non-contiguous CA and UL NR CA with 2 carriers |
|  |  |  | C83A | UEs supporting EN-DC and intra-band non-contiguous CA and EN-DC with 2 NR UL carriers |
| 7.1.1.3.9 | Correct Handling of UL HARQ process / PUSCH Repetition Type A / PUSCH Aggregation | Rel-15 | C51 | UEs supporting 5GS and PUSCH aggregation |
| 7.1.1.3.10 | Correct Handling of HARQ process / Multiple CORESETPoolIndex | Rel-16 | C107 | UEs supporting 5GS and multi-DCI based Multi-TRP |
| 7.1.1.3.11 | Correct handling of UL grant prioritization | Rel-16 | C114 | UEs supporting 5GS and LCH-based UL grant prioritization |
| 7.1.1.3.12 | Correct Handling of UL HARQ process / PUSCH Repetition Type B | Rel-16 | C134 | UEs supporting PUSCH repetition type B |
| 7.1.1.3.13 | Logical channel prioritization handling with Mapping restrictions / physical layer priority | Rel-16 | C180 | UEs supporting DCI UL Priority Indicator and LCH grant prioritisation |
| 7.1.1.3.14 | Correct Handling of UL HARQ process / PUSCH Repetition Type A enhancement |  |  |  |
| 7.1.1.3.14.1 | Correct Handling of UL HARQ process / PUSCH Repetition Type A enhancement / Increased maximum repetition number / dynamic grant | Rel-17 | C288 | UEs supporting increased maximum number of PUSCH Type A repetitions and dynamic indication of the number of repetitions for PUSCH |
| 7.1.1.3.14.2 | Correct Handling of UL HARQ process / PUSCH Repetition Type A enhancement / Increased maximum repetition number / configured grant | Rel-17 | C289 | UEs supporting increased maximum number of PUSCH Type A repetitions and PUSCH transmissions with configured grant |
| 7.1.1.3.14.3 | Correct Handling of UL HARQ process / PUSCH Repetition Type A enhancement / repetition based on available slots / dynamic grant | Rel-17 | C290 | UEs supporting PUSCH repetitions based on available slots and dynamic indication of the number of repetitions for PUSCH |
| 7.1.1.3.14.4 | Correct Handling of UL HARQ process / PUSCH Repetition Type A enhancement / repetition based on available slots / configured grant | Rel-17 | C291 | UEs supporting PUSCH repetitions based on available slots and PUSCH transmissions with configured grant |
| 7.1.1.3.15 | Correct Handling of UL HARQ process / TBoMS procedure |  |  |  |
| 7.1.1.3.15.1 | Correct Handling of UL HARQ process / TBoMS procedure / DG and CG based transmission | Rel-17 | C292 | UEs supporting TB processing over multi-slot PUSCH |
| 7.1.1.3.15.2 | Correct Handling of UL HARQ process / TBoMS procedure / Repetition of TBoMS | Rel-17 | C293 | UEs supporting repetition of TB processing over multi-slot PUSCH |
| 7.1.1.3.16 | Correct Handling of UL grant / DRB configured with survival time |  |  |  |
| 7.1.1.3.16.1 | Correct Handling of UL grant / DRB configured with survival time / Split DRB | Rel-17 | C256 | UEs supporting services with survival time and NR-DC and PDCP-duplication over split DRB |
| 7.1.1.3.16.2 | Correct Handling of UL grant / DRB configured with survival time / MCG or SCG DRB / Intra-band contiguous CA | Rel-17 | C257 | UEs supporting services with survival time and intra-band contiguous CA and CA-based PDCP duplication over MCG or SCG DRB |
| 7.1.1.3.16.3 | Correct Handling of UL grant / DRB configured with survival time / MCG or SCG DRB / Intra-band non-contiguous CA | Rel-17 | C258 | UEs supporting services with survival time and intra-band non-contiguous CA and CA-based PDCP duplication over MCG or SCG DRB |
| 7.1.1.3.16.4 | Correct Handling of UL grant / DRB configured with survival time / MCG or SCG DRB / Inter-band CA | Rel-17 | C259 | UEs supporting services with survival time and inter-band CA and CA-based PDCP duplication over MCG or SCG DRB |
| 7.1.1.3.17 | Correct Handling of UL HARQ process / PUSCH Repetition / Multi-TRP |  |  |  |
| 7.1.1.3.17.1 | Correct Handling of UL HARQ process / PUSCH Repetition / Multi-TRP / PUSCH Repetition Type A / dynamic grant | Rel-17 | C330 | UEs supporting 5GS and multi-TRP PUSCH repetition type A |
| 7.1.1.3.17.2 | Correct Handling of UL HARQ process / PUSCH Repetition / Multi-TRP / PUSCH Repetition Type B / dynamic grant | Rel-17 | C331 | UEs supporting 5GS and multi-TRP PUSCH repetition type B |
| 7.1.1.4 | Transport Size Selection |  |  |  |
| 7.1.1.4.1 | DL-SCH Transport Block Size Selection |  |  |  |
| 7.1.1.4.1.1 | DL-SCH Transport Block Size selection / DCI format 1_0 | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.4.1.2 | Void |  |  |  |
| 7.1.1.4.1.3 | DL-SCH transport block size selection / DCI format 1_1 / RA type 0/RA Type 1 / 2 Codewords enabled | Rel-15 | C64 | UEs supporting 5GS and The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 8 Layers. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier |
| 7.1.1.4.1.4 | DL-SCH transport block size selection / DCI format 1_1 / RA type 0/RA Type 1 / 2 Codewords enabled / 256QAM | Rel-15 | C65 | UEs supporting 5GS and The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 8 Layers. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier and 256QAM for PUSCH |
| 7.1.1.4.1.5 | DL-SCH transport block size selection / DCI format 1_2 | Rel-16 | C146 | Ues supporting monitoring DCI format 1_2 for DL scheduling and monitoring DCI format 0_2 for UL scheduling |
| 7.1.1.4.2 | UL-SCH Transport Block Size Selection |  |  |  |
| 7.1.1.4.2.1 | UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding disabled | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.4.2.2 | Void |  |  |  |
| 7.1.1.4.2.3 | UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / Transform precoding disabled | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.4.2.4 | UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled | Rel-15 | C11 | UEs supporting 5GS and 256QAM for PDSCH for FR1/FR2 |
| 7.1.1.4.2.5 | UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding and 64QAM | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.4.2.6 | UL-SCH Transport Block Size selection / DCI format 0_2 | Rel-16 | C146 | UEs supporting monitoring DCI format 1_2 for DL scheduling and monitoring DCI format 0_2 for UL scheduling |
| 7.1.1.4.2.7 | UL-SCH Transport Block Size selection / TBoMS procedure | Rel-17 | C292 | UEs supporting TB processing over multi-slot PUSCH |
| 7.1.1.5 | Discontinuous reception |  |  |  |
| 7.1.1.5.1 | DRX operation / Short cycle not configured / Parameters configured by RRC | Rel-15 | C03 | UEs supporting 5GS and long DRX cycle |
| 7.1.1.5.2 | DRX operation / Short cycle not configured / Long DRX command MAC control element reception | Rel-15 | C03 | UEs supporting 5GS and long DRX cycle |
| 7.1.1.5.3 | DRX operation / Short cycle configured / Parameters configured by RRC | Rel-15 | C04 | UEs supporting 5GS and short DRX cycle |
| 7.1.1.5.4 | DRX operation / Short cycle configured / DRX command MAC control element reception | Rel-15 | C04 | UEs supporting 5GS and short DRX cycle |
| 7.1.1.5.5 | DRX operation / Short cycle configured / Long DRX command MAC control element reception | Rel-15 | C70 | UEs supporting 5GS and long DRX cycle and short DRX cycle |
| 7.1.1.5.6 | DRX operation / Short cycle not configured / NTN / HARQ RTT | Rel-17 | C309 | UEs supporting 5G Core and NR NTN access |
| 7.1.1.6 | Semi-Persistent Scheduling |  |  |  |
| 7.1.1.6.1 | Correct handling of DL assignment / Semi-persistent case | Rel-15 | C17 | UEs supporting 5GS and PDSCH reception based on semi-persistent scheduling |
| 7.1.1.6.2 | Correct handling of UL grant / configured grant Type 1 | Rel-15 | C18 | UEs supporting 5GS and Type 1 PUSCH transmissions with configured grant |
| 7.1.1.6.3 | Correct handling of UL grant / configured grant Type 2 | Rel-15 | C19 | UEs supporting 5GS and Type 2 PUSCH transmissions with configured grant |
| 7.1.1.6.4 | Correct handling of DL assignment / Multi Semi-persistent configuration | Rel-16 | C113 | UEs supporting 5G Core and PDSCH reception based on semi-persistent scheduling and up to 8 configured SPS configurations in a BWP of a serving cell and up to 32 configured SPS configurations in a cell group |
| 7.1.1.6.5 | Correct handling of UL grant / Multi configured uplink grants | Rel-16 | C142 | UEs supporting 5G Core and PUSCH transmissions on multiple configured uplink grants |
| 7.1.1.7 | Activation/Deactivation of SCells |  |  |  |
| 7.1.1.7.1 | Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer |  |  |  |
| 7.1.1.7.1.1 | Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / Intra-band Contiguous CA | Rel-15 | C44 | UEs supporting 5GS and intra-band contiguous CA |
|  |  |  | C44A | UEs supporting EN-DC and intra-band contiguous CA and EN-DC with 2 NR DL carriers |
| 7.1.1.7.1.2 | Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / Inter-band CA | Rel-15 | C45 | UEs supporting 5GS and inter-band CA |
|  |  |  | C45A | UEs supporting EN-DC and inter-band CA and EN-DC with 2 NR DL carriers |
| 7.1.1.7.1.3 | Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / Intra-band non-Contiguous CA | Rel-15 | C46 | UEs supporting 5GS and intra-band non-contiguous CA |
|  |  |  | C46A | UEs supporting EN-DC and intra-band non-contiguous CA and EN-DC with 2 NR DL carriers |
| 7.1.1.7.2 | Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer |  |  |  |
| 7.1.1.7.2.1 | Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer / Intra-band Contiguous CA | Rel-18 | CXXXHS13 | UEs supporting 5G Core and intra-band contiguous CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set |
| 7.1.1.7.2.2 | Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer / Inter-band CA | Rel-18 | CXXXHS15 | UEs supporting 5G Core and inter-band CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set |
| 7.1.1.7.2.3 | Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer / Intra-band non-Contiguous CA | Rel-18 | CXXXHS17 | UEs supporting 5G Core and intra-band non-contiguous CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set |
| 7.1.1.7.3 | Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer |  |  |  |
| 7.1.1.7.3.1 | Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer / Intra-band Contiguous CA | Rel-18 | CXXXHS19 | UEs supporting 5G Core and intra-band contiguous CA and UL NR CA with 2 carriers and DCI format 0_3 with same SCS between scheduling cell and cells in the set |
| 7.1.1.7.3.2 | Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer/ Inter-band CA | Rel-18 | CXXXHS21 | UEs supporting 5G Core and inter-band CA and UL NR CA with 2 carriers and DCI format 0_3 with same SCS between scheduling cell and cells in the set |
| 7.1.1.7.3.3 | Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer / Intra-band non-Contiguous CA | Rel-18 | CXXXHS23 | UEs supporting 5G Core and intra-band non-contiguous CA and UL NR CA with 2 carriers and DCI format 0_3 with same SCS between scheduling cell and cells in the set |
| 7.1.1.7.4 | Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer |  |  |  |
| 7.1.1.7.4.1 | Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer / Intra-band Contiguous CA | Rel-18 | CXXXHS25 | UEs supporting 5G Core and intra-band contiguous CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set and search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells. |
| 7.1.1.7.4.2 | Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer/ Inter-band CA | Rel-18 | CXXXHS27 | UEs supporting 5G Core and inter-band CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set and search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells. |
| 7.1.1.7.4.3 | Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer / Intra-band non-Contiguous CA | Rel-18 | CXXXHS29 | UEs supporting 5G Core and intra-band non-contiguous CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set and search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells. |
| 7.1.1.7.5 | Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer |  |  |  |
| 7.1.1.7.5.1 | Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer/ Intra-band Contiguous CA | Rel-18 | CXXXHS31 | UEs supporting 5G Core and intra-band contiguous CA and UL NR CA with 2 carriers and DCI format 0_3 with same SCS between scheduling cell and cells in the set and search space set configurations for DCI format 0_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells. |
| 7.1.1.7.5.2 | Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer/ Inter-band CA | Rel-18 | CXXXHS33 | UEs supporting 5G Core and inter-band CA and UL NR CA with 2 carriers and DCI format 0_3 with same SCS between scheduling cell and cells in the set and search space set configurations for DCI format 0_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells. |
| 7.1.1.7.5.3 | Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer / Intra-band non-Contiguous CA | Rel-18 | CXXXHS35 | UEs supporting 5G Core and intra-band non-contiguous CA and UL NR CA with 2 carriers and DCI format 0_3 with same SCS between scheduling cell and cells in the set and search space set configurations for DCI format 0_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells. |

<End of modified section 1>

<Start of modified section 2>

4.2	Protocol conformance test cases applicability conditions

Table 4.2-1: Applicability of Protocol conformance test cases Conditions

| Condition | Test case Selection Expression | Comment |
| --- | --- | --- |
| C01 | IF A.4.1-3/2 THEN R ELSE N/A | UEs supporting EN-DC |
| C02 | IF A.4.3.4-1/2 OR A.4.3.4-1/3 THEN R ELSE N/A | UEs supporting 5GS and RLC UM Mode |
| C03 | IF A.4.3.5-1/1 THEN R ELSE N/A | UEs supporting 5GS and Long DRX Cycle |
| C04 | IF A.4.3.5-1/2 THEN R ELSE N/A | UEs supporting 5GS and short DRX cycle |
| C05 | IF A.4.3.4-1/3 THEN R ELSE N/A | UEs supporting 5GS and RLC UM with 6-bit length of RLC sequence number |
| C06 | IF A.4.3.4-1/2 THEN R ELSE N/A | UEs supporting 5GS and RLC UM with 12-bit length of RLC sequence number |
| C07 | IF A.4.3.4-1/1 THEN R ELSE N/A | UEs supporting 5GS and RLC AM with 12-bit length of RLC sequence number |
| C07A | IF A.4.3.4-1/1A THEN R ELSE N/A | UEs supporting 5GS and RLC AM with 18-bit length of RLC sequence number |
| C08 | IF A.4.3.3-1/1 THEN R ELSE N/A | UEs supporting 5GS and 12-bit length of PDCP sequence number |
| C08A | IF A.4.3.3-1/1A THEN R ELSE N/A | UEs supporting 5GS and 18-bit length of PDCP sequence number |
| C09 | IF [10] A.4.4-1/99 THEN R ELSE N/A | UEs supporting 5GS and ZUC Algorithm |
| C10 | IF A.4.1-3/2 AND A.4.3.7-1/2 THEN R ELSE N/A | UEs supporting EN-DC and UL transmission via both MCG path and SCG path for the split DRB |
| C11 | IF A.4.3.2-1/2 OR A.4.3.2-1/3 THEN R ELSE N/A | UEs supporting 5GS and 256QAM for PDSCH for FR1/FR2 |
| C12 | IF A.4.3.2-1/4 THEN R ELSE N/A | UEs supporting 5GS and 256QAM for PUSCH |
| C13 | IF A.4.1-3/2 AND A.4.3.6-1/1 THEN R ELSE N/A | UEs supporting EN-DC and NR measurements and Event A triggered reporting |
| C14 | IF A.4.1-3/2 AND A.4.3.6-1/1 AND A.4.3.6-1/3 THEN R ELSE N/A | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and NR-Inter frequency measurements and at least periodical reporting) |
| C15 | IF A.4.1-3/2 AND A.4.3.6-1/1 AND A.4.3.6-1/3 AND (A.4.3.6-1/4 OR A.4.3.6-1/40) THEN R ELSE N/A | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and Inter frequency measurements and at least periodical reporting) and CSI-RSRP and CSI-RSRQ measurement |
| C16 | IF A.4.1-3/2 AND [10] A.4.4-1/18 AND [10] A.4.4-1/19 THEN R ELSE N/A | UEs supporting EN-DC and UE requested bearer resource allocation and modification procedures |
| C17 | IF A.4.3.2-1/1 THEN R ELSE N/A | UEs supporting 5GS and PDSCH reception based on semi-persistent scheduling |
| C18 | IF A.4.3.2-1/10 THEN R ELSE N/A | UEs supporting 5GS and Type 1 PUSCH transmissions with configured grant |
| C19 | IF A.4.3.2-1/11 THEN R ELSE N/A | UEs supporting 5GS and Type 2 PUSCH transmissions with configured grant |
| C20 | IF A.4.3.2-1/12 THEN R ELSE N/A | UEs supporting 5GS and PDSCH aggregation |
| C21 | IF A.4.1-5/1 THEN R ELSE N/A | UEs supporting 5G Core |
| C21A | IF A.4.1-5/1 AND A.4.3.7-1/4 THEN R ELSE N/A | UEs supporting 5G Core and reflective QoS |
| C22 | IF A.4.1-3/2 AND A.4.3.7-1/3 THEN R ELSE N/A | UEs supporting EN-DC and SRB3 |
| C23 | IF A.4.1-3/2 AND A.4.3.7-1/3 AND A.4.3.7-1/1 THEN R ELSE N/A | UEs supporting EN-DC and SRB3 and (UL transmission via either MCG path or SCG path for the split SRB) |
| C24 | IF A.4.1-3/2 AND A.4.3.6-1/3 AND A.4.3.6-1/2 AND A.4.1-4/3 THEN R ELSE N/A | UEs supporting EN-DC and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and (two independent measurement gap configurations for FR1 and FR2) and Inter-Band EN-DC within FR1 |
| C25 | IF A.4.1-3/2 AND A.4.3.6-1/3 AND A.4.3.6-1/2 AND A.4.1-4/4 THEN R ELSE N/A | UEs supporting EN-DC and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and (two independent measurement gap configurations for FR1 and FR2) and Inter-Band EN-DC including FR2 |
| C26 | IF [10] A.4.1-1/1 OR [10] A.4.1-1/2 THEN R ELSE N/A | UEs supporting 5GS and E-UTRA |
| C27 | IF A.4.1-5/1 AND A.4.3.6-1/1 THEN R ELSE N/A | UEs supporting 5G Core and NR measurements and Event A triggered reporting |
| C28 | IF A.4.3.2-1/13 THEN R ELSE N/A | UEs supporting 5GS and supplemental uplink with dynamic switch |
| C29 | IF A.4.1-5/2 AND [10] A.4.1-1/5 THEN R ELSE N/A | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| C30 | IF A.4.1-5/2 AND A.4.3.7-1/6 AND [10] A.4.1-1/5 THEN R ELSE N/A | UEs supporting 5G core over non-3GPP Access Network and SMS over NAS and WLAN |
| C31 | IF A.4.1-5/1 AND A.4.3.6-1/5 THEN R ELSE N/A | UEs supporting 5G Core and Inter-RAT E-UTRA measurements and Event B triggered reporting |
| C32 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA |
| C32a | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.4-1/6 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and logged MDT |
| C33 | IF A.4.1-5/1 AND A.4.3.7-1/6 AND NOT [10] A.4.4-2/32 THEN R ELSE N/A | UEs supporting 5G Core and SMS over NAS and UE configured to not use SMSoIP |
| C34 | IF A.4.1-5/1 AND [10] A.4.4-1/84 THEN R ELSE N/A | UEs supporting 5G Core and MinimumPeriodicSearchTimer |
| C35 | IF A.4.1-5/1 AND (A.4.3.7-1/8 OR A.4.3.7-1/7) THEN R ELSE N/A | UEs supporting 5G Core and (ETWS reception or CMAS reception) |
| C36 | IF A.4.1-5/1 AND [10] A.4.4-1/69 THEN R ELSE N/A | UEs supporting 5G Core and user initiated PLMN reselection in automatic mode on NR |
| C37 | IF A.4.1-5/1 AND (A.4.1-2/1 OR A.4.1-2/2) THEN R ELSE N/A | UEs supporting 5G Core and more than 1 FDD or TDD NR band |
| C38 | IF A.4.1-5/1 AND A.4.1-1/1 AND A.4.1-1/2 THEN R ELSE N/A | UEs supporting 5G Core and NR FDD and NR TDD |
| C39 | IF A.4.1-5/1 AND A.4.3.7-1/9 THEN R ELSE N/A | UEs supporting 5G Core and additional UE-requested PDU establishment |
| C39A | IF A.4.1-5/1 AND A.4.3.7-1/9 AND A.4.3.7-1/54 THEN R ELSE N/A | UEs supporting 5G Core and additional UE-requested PDU establishment and support of EAP-AKA’ as EAP method for PDU session authentication and authorization |
| C40 | IF A.4.1-5/1 AND A.4.3.6-1/6 THEN R ELSE N/A | UEs supporting 5G Core and SS-SINR measurements |
| C41 | IF A.4.1-5/1 AND (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A | UEs supporting 5G Core and intra-band contiguous CA |
| C42 | IF A.4.1-5/1 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A | UEs supporting 5G Core and inter-band CA |
| C43 | IF A.4.1-5/1 AND (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A | UEs supporting 5G Core and intra-band non-contiguous CA |
| C44 | IF (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A | UEs supporting 5GS and intra-band contiguous CA |
| C44A | IF A.4.1-3/2 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.2B.2.0-1A/2 THEN R ELSE N/A | UEs supporting EN-DC and intra-band contiguous CA and EN-DC with 2 NR DL carriers |
| C45 | IF (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A | UEs supporting 5GS and inter-band CA |
| C45A | IF A.4.1-3/2 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.2B.2.0-1A/2 THEN R ELSE N/A | UEs supporting EN-DC and inter-band contiguous CA and EN-DC with 2 NR DL carriers |
| C46 | IF (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A | UEs supporting 5GS and intra-band non-contiguous CA |
| C46A | IF A.4.1-3/2 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.2B.2.0-1A/2 THEN R ELSE N/A | UEs supporting EN-DC and intra-band non-contiguous CA and EN-DC with 2 NR DL carriers |
| C47 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.2.1.1-1/4 AND A.4.3.7-1/11 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and Emergency Services Fallback in NR connected to 5GCN |
| C48 | Void |  |
| C49 | IF A.4.1-5/1 AND A.4.3.6-1/2 THEN R ELSE N/A | UE supporting 5G Core and two independent measurement gap configurations for FR1 and FR2 |
| C50 | IF A.4.1-5/1 AND A.4.3.6-1/5 AND A.4.3.6-1/42 THEN R ELSE N/A | UEs supporting 5G Core and Inter-RAT E-UTRA measurements and Event B triggered reporting and E-UTRA RS-SINR measurements |
| C51 | IF A.4.3.2-1/21 THEN R ELSE N/A | UEs supporting 5GS and PUSCH aggregation |
| C52 | IF A.4.1-5/1 AND A.4.3.6-1/1 AND A.4.3.6-1/3 AND (A.4.3.6-1/4 OR A.4.3.6-1/40) THEN R ELSE N/A | UEs supporting 5G Core and NR measurements and Event A triggered reporting and (NR Intra-frequency and Inter frequency measurements and at least periodical reporting) and CSI-RSRP and CSI-RSRQ measurement |
| C53 | IF A.4.3.5-1/4 THEN R ELSE N/A | UEs supporting 5GS and Logical Channel SR-Delay Timer |
| C54 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.4-1/33 AND A.4.3.7-1/12 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback |
| C55 | IF A.4.1-3/2 AND A.4.3.6-1/1 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.2B.2.0-1A/2 AND A.4.3.7-1/3 THEN R ELSE N/A | UEs supporting EN-DC and NR measurements and Event A triggered reporting and intra-band contiguous CA and EN-DC with 2 NR DL carriers and SRB3 |
| C56 | IF A.4.1-3/2 AND A.4.3.6-1/1 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.2B.2.0-1A/2 AND A.4.3.7-1/3 THEN R ELSE N/A | UEs supporting EN-DC and NR measurements and Event A triggered reporting and inter-band CA and EN-DC with 2 NR DL carriers and SRB3 |
| C57 | IF A.4.1-3/2 AND A.4.3.6-1/1 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.2B.2.0-1A/2 AND A.4.3.7-1/3 THEN R ELSE N/A | UEs supporting EN-DC and NR measurements and Event A triggered reporting and intra-band non-contiguous CA and EN-DC with 2 NR DL carriers and SRB3 |
| C58 | IF A.4.1-5/2 AND [10] A.4.1-1/5.AND A.4.4-1/1 | UEs supporting 5G core over non-3GPP Access Network, WLAN and (ICMP or ICMP IPv6) |
| C59 | IF A.4.1-5/1 AND A.4.3.6-1/8 THEN R ELSE N/A | UEs supporting 5G Core and Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when EN-DC is not configured |
| C60 | IF A.4.1-5/1 AND A.4.3.6-1/7 THEN R ELSE N/A | UEs supporting 5G Core and Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is not configured |
| C61 | IF A.4.1-3/2 AND A.4.3.3-1/6 THEN R ELSE N/A | UEs supporting EN-DC and PDCP duplication over split SRB1/2 |
| C62 | IF A.4.1-3/2 AND A.4.3.3-1/4 THEN R ELSE N/A | UEs supporting EN-DC and PDCP duplication over split DRB |
| C63 | IF A.4.1-5/1 AND A.4.3.7-1/13 THEN R ELSE N/A | UEs supporting 5G Core and UE requested PDU session modification procedure |
| C64 | IF A.4.3.2-1/23 THEN R ELSE N/A | UEs supporting 5GS and The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 8 Layers. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier |
| C65 | IF A.4.3.2-1/23 AND A.4.3.2-1/4 THEN R ELSE N/A | UEs supporting 5GS and The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 8 Layers. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier |
| C66 | IF (A.4.3.2-1/24 OR A.4.3.2-1/24A) AND (A.4.3.2-1/42 OR A.4.3.2-1/42a OR A.4.3.2-1/42b OR A.4.3.2-1/43 OR A.4.3.2-1/43a OR A.4.3.2-1/43b) THEN R ELSE N/A | UEs supporting 5GS and (DCI and timer based active BWP switching delay type1 or type2) and ((BWP adaptation up to 2 NR FR1 FDD or NR FR1 TDD or NR FR2) or (BWP adaptation up to 4 NR FR1 FDD or NR FR1 TDD or NR FR2)) |
| C67 | IF A.4.1-3/2 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.2B.2.0-1A/2 THEN R ELSE N/A | UEs supporting EN-DC and Intra-Band Contiguous CA and EN-DC with 2 NR DL carriers |
| C68 | IF A.4.1-3/2 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.2B.2.0-1A/2 THEN R ELSE N/A | UEs supporting EN-DC and Intra-Band Non-Contiguous CA and EN-DC with 2 NR DL carriers |
| C69 | IF A.4.1-3/2 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.2B.2.0-1A/2 THEN R ELSE N/A | UEs supporting EN-DC and Inter-Band CA and EN-DC with 2 NR DL carriers |
| C70 | IF A.4.3.5-1/1 AND A.4.3.5-1/2 THEN R ELSE N/A | UEs supporting 5GS and Long DRX Cycle and Short DRX Cycle |
| C71 | IF A.4.1-3/2 AND A.4.3.7-1/3 AND A.4.3.6-1/3 THEN R ELSE N/A | UEs supporting EN-DC and SRB3 and NR intra-frequency and inter-frequency measurements and at least periodical reporting |
| C72 | IF A.4.1-5/1 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting 5G Core and intra-band contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |
| C73 | IF A.4.1-5/1 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting 5G Core and inter-band CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |
| C74 | IF A.4.1-5/1 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting 5G Core and intra-band non-contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |
| C75 | IF A.4.1-3/2 AND A.4.3.7-1/3 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.3-1/3 AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A | UEs supporting EN-DC and SRB3 and intra-band contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and EN-DC with 2 NR UL carriers |
| C76 | IF A.4.1-3/2 AND A.4.3.7-1/3 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.3-1/3 AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A | UEs supporting EN-DC and SRB3 and inter-band CA and CA-based PDCP duplication over MCG or SCG DRB and EN-DC with 2 NR UL carriers |
| C77 | IF A.4.1-3/2 AND A.4.3.7-1/3 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.3-1/3 AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A | UEs supporting EN-DC and SRB3 and intra-band non-contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and EN-DC with 2 NR UL carriers |
| C78 | IF A.4.1-5/1 AND A.4.3.7-1/32 AND [9] A.3A/50 AND [9] A.4/2B AND [9] A.15/1 AND [9] A.3A/61 THEN R ELSE N/A | UEs supporting 5G Core and IMS voice over NR and Initiating session and MTSI speech and SMS over IP |
| C79 | IF A.4.1-5/1 AND [9] A.3A/50 AND [9] A.4/2B AND [9] A.15/3 AND [9] A.15/11 AND [9] A.15/12 AND [9] A.15/13 AND [9] A.21/1 THEN R ELSE N/A | UEs supporting 5G Core and Initiating session and MTSI video and MTSI video H.265 MP MT Level 3.1 and MTSI video H.264 CHP Level 3.1 and H.264 CBP Level 3.1 and NG114 v1.0 |
| C80 | IF A.4.1-4/6 THEN R ELSE N/A | UEs supporting NR-DC |
| C81 | IF A.4.1-5/1 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting 5G Core and intra-band contiguous CA and UL NR CA with 2 carriers |
| C81A | IF A.4.1-3/2 AND A.4.1-4/1 AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A | UEs supporting EN-DC and intra-band contiguous CA and EN-DC with 2 NR UL carriers |
| C82 | IF A.4.1-5/1 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting 5G Core and inter-band CA and UL NR CA with 2 carriers |
| C82A | IF A.4.1-3/2 AND (A.4.1-4/3 OR A.4.1-4/4 OR A.4.1-4/5) AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A | UEs supporting EN-DC and inter-band CA and EN-DC with 2 NR UL carriers |
| C83 | IF A.4.1-5/1 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting 5G Core and intra-band non-contiguous CA and UL NR CA with 2 carriers |
| C83A | IF A.4.1-3/2 AND A.4.1-4/2 AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A | UEs supporting EN-DC and intra-band non-contiguous CA and EN-DC with 2 NR UL carriers |
| C84 | IF A.4.1-5/1 AND [10] A.4.4-1/99 THEN R ELSE N/A | UEs supporting 5G Core and ZUC algorithm |
| C85 | Void |  |
| C85A | IF (A.4.1-5/1 AND A.4.4-2/9) AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.2.1.1-1/4 AND A.4.3.7-1/14 THEN R ELSE N/A | UEs supporting 5G core and Emergency PDN connection transfer from S1 mode to N1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and emergency services in NR connected to 5GCN |
| C85B | IF (A.4.1-5/1 AND A.4.4-2/8) AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.2.1.1-1/4 AND A.4.3.7-1/32 THEN R ELSE N/A | UEs supporting 5G core and Emergency PDU session transfer from N1 mode to S1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and IMS voice over NR |
| C86 | IF A.4.1-4/6 AND A.4.3.7-1/3 THEN R ELSE N/A | UEs supporting NR-DC and SRB3 |
| C87 | IF A.4.1-4/6 AND A.4.3.7-1/3 AND A.4.3.6-1/3 THEN R ELSE N/A | UEs supporting NR-DC and SRB3 and NR intra-frequency and inter-frequency measurements and at least periodical reporting |
| C88 | IF A.4.1-4/6 AND A.4.3.7-1/3 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting NR-DC and SRB3 and intra-band contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |
| C89 | IF A.4.1-4/6 AND A.4.3.7-1/3 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting NR-DC and SRB3 and inter-band CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |
| C90 | IF A.4.1-4/6 AND A.4.3.7-1/3 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting NR-DC and SRB3 and intra-band non-contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |
| C91 | IF A.4.1-5/1 AND [10] A.4.4-1/98 THEN R ELSE N/A | UEs supporting 5G Core and ManualModeNetworkSelectionException |
| C92 | IF A.4.1-5/1 AND A.4.3.7-1/14 THEN R ELSE N/A | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| C93 | IF A.4.1-3/2 AND A.4.3.6-1/1 AND A.4.3.6-1/3 AND (A.4.1-2/1 OR A.4.1-2/2 OR (A.4.1-1/1 AND A.4.1-1/2)) THEN R ELSE N/A | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and NR-Inter frequency measurements and at least periodical reporting) and multiple NR bands |
| C94 | IF A.4.1-5/1 AND (A.4.1-2/1 OR A.4.1-2/2 OR (A.4.1-1/1 AND A.4.1-1/2)) THEN R ELSE N/A | UEs supporting 5G Core and multiple NR bands |
| C95 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.4-1/33 AND A.4.3.7-1/12 AND A.4.3.7-1/15 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback and voiceFallbackIndication |
| C96 | IF A.4.1-5/1 AND A.4.1-3/2 AND A.4.3.8-1/10 THEN R ELSE N/A | UEs supporting 5G Core and EN-DC and inter-RAT Handover from NR to EN-DC |
| C97 | IF A.4.1-4/6 AND A.4.3.7-1/2 THEN R ELSE N/A | UEs supporting NR-DC and UL transmission via both MCG path and SCG path for the split DRB |
| C98 | IF A.4.1-4/6 AND A.4.3.3-1/4 THEN R ELSE N/A | UEs supporting NR-DC and PDCP duplication over split DRB |
| C99 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND (A.4.3.8-1/6 OR A.4.3.8-1/7 OR A.4.3.8-1/8)THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and (inter-RAT Handover to NR FR1 TDD from EUTRA connected to EPC or inter-RAT Handover to NR FR1 FDD from EUTRA connected to EPC or inter-RAT Handover to NR FR2 TDD from EUTRA connected to EPC) |
| C100 | IF A.4.1-5/1 AND [9] A.15/1 AND A.4.3.5-1/9 THEN R ELSE N/A | UEs supporting 5G Core and MTSI speech and bit rate recommendation query message |
| C101 | IF A.4.1-5/1 AND A.4.3.8-1/9 THEN R ELSE N/A | UEs supporting 5G Core and intra-frequency DAPS handover |
| C102 | Void |  |
| C103 | IF A.4.3.5-1/1 AND A.4.3.5-1/5 THEN R ELSE N/A | UEs supporting 5GS and Long DRX Cycle and DRX adaptation |
| C104 | IF (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.2A.1-1/2 AND A.4.3.2A.1-2/2 AND A.4.3.3-1/5 THEN R ELSE N/A | UEs supporting 5GC and Intra-band contiguous CA and DL and UL NR CA with 3 carriers and PDCP duplication with more than two RLC entities |
| C105 | IF (A.4.3.4-1/2 OR A.4.3.4-1/3) AND A.4.3.3-1/7 THEN R ELSE N/A | UEs supporting 5GS and RLC UM mode and PDCP ethernet header compression |
| C106 | IF A.4.1-5/1 AND A.4.3.10-1/1 THEN R ELSE N/A | UE supporting 5G core and NR sidelink mode 1 transmission |
| C107 | IF A.4.3.2-1/32 THEN R ELSE N/A | UE’s supporting multi-DCI based multi-TRP |
| C108 | IF A.4.1-5/1 AND A.4.3.7-1/17 THEN R ELSE N/A | UEs supporting 5G Core and RACS |
| C109 | IF A.4.1-5/1 AND A.4.3.7-1/19 THEN R ELSE N/A | UEs supporting 5G Core and RRC_INACTIVE |
| C109A | IF A.4.1-5/1 AND A.4.3.7-1/19 AND A.4.4-2/10 THEN R ELSE N/A | UEs supporting 5G Core and RRC_INACTIVE and UE’s usage setting as data centric |
| C110 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.3.7-1/19 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and RRC_INACTIVE |
| C111 | IF A.4.1-5/1 AND (A.4.3.7-1/8 OR A.4.3.7-1/7) AND A.4.3.7-1/19 THEN R ELSE N/A | UEs supporting 5G Core and (ETWS reception or CMAS reception) and RRC_INACTIVE |
| C112 | Void |  |
| C113 | IF A.4.1-5/1 AND A.4.3.2-1/1 AND A.4.3.2-1/121 THEN R ELSE N/A | UEs supporting 5G Core and PDSCH reception based on semi-persistent scheduling and up to 8 configured SPS configurations in a BWP of a serving cell and up to 32 configured SPS configurations in a cell group |
| C114 | IF A.4.1-5/1 AND A.4.3.5-1/6 THEN R ELSE N/A | UEs supporting 5GS and LCH-based UL grant prioritization |
| C115 | IF A.4.1-5/1 AND A.4.3.8-1/11 THEN R ELSE N/A | UEs supporting 5G Core and conditional handover |
| C116 | IF A.4.1-5/1 AND A.4.3.8-1/11 AND A.4.3.8-1/13 THEN R ELSE N/A | UEs supporting 5G Core and conditional handover and supporting 2 trigger events for same execution condition |
| C117 | IF A.4.1-5/1 AND A.4.3.8-1/11 AND A.4.3.8-1/12 THEN R ELSE N/A | UEs supporting 5G Core and conditional handover and conditional handover during re-establishment procedure when the selected cell is configured as candidate cell for condition handover |
| C118 | IF A.4.3.5-1/1 AND A.4.3.5-1/5 AND A.4.3.2-1/35 AND (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A | UEs supporting 5GS and Long DRX Cycle and DRX adaptation and SCell Dormancy indication outside active time and intra-band contiguous CA |
| C119 | IF A.4.3.5-1/1 AND A.4.3.5-1/5 AND A.4.3.2-1/35 AND (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A | UEs supporting 5GS and Long DRX Cycle and DRX adaptation and SCell Dormancy indication outside active time and intra-band non-contiguous CA |
| C120 | IF A.4.3.5-1/1 AND A.4.3.5-1/5 AND A.4.3.2-1/35 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A | UEs supporting 5GS and Long DRX Cycle and DRX adaptation and SCell Dormancy indication outside active time and inter-band CA |
| C121 | Void |  |
| C122 | IF A.4.1-5/1 AND A.4.4-1/5 THEN R ELSE N/A | UEs supporting 5G Core and UL PDCP Packet Delay per DRB |
| C123 | IF A.4.1-5/1 AND A.4.4-1/6 THEN R ELSE N/A | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE |
| C124 | IF A.4.1-5/1 AND A.4.4-1/4 AND A.4.4-1/6 THEN R ELSE N/A | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE and equipped with a GNSS receiver to provide detailed location information |
| C125 | IF A.4.1-5/1 AND A.4.4-1/6 AND A.4.3.7-1/19 THEN R ELSE N/A | UEs supporting 5G core and RRC_INACTIVE and logged measurements in RRC_IDLE and RRC_INACTIVE |
| C126 | IF A.4.1-5/1 AND A.4.4-1/4 THEN R ELSE N/A | UEs supporting 5G Core and equipped with a GNSS or A-GNSS receiver to provide detailed location information |
| C127 | IF A.4.1-5/1 AND [10] A.4.1-1/6 AND A.4.3.8-1/11 THEN R ELSE N/A | UEs supporting 5G Core and UTRA and NR to UTRA-FDD CELL_DCH CS handover |
| C128 | IF A.4.1-5/1 AND A.4.1-1/3 THEN R ELSE N/A | UE supporting 5G core and NR sidelink |
| C129 | IF A.4.1-5/1 AND A.4.3.7-1/18 THEN R ELSE N/A | UEs supporting 5G Core and RRC message Segmentation in the UL |
| C130 | IF A.4.1-5/1 AND A.4.3.8-1/15 THEN R ELSE N/A | UEs supporting 5G Core and inter-frequency DAPS handover |
| C131 | IF A.4.1-5/1 AND A.4.3.7-1/24 THEN R ELSE N/A | UEs supporting 5G Core and SNPN |
| C132 | IF A.4.1-5/1 AND A.4.3.7-1/23 THEN R ELSE N/A | UEs supporting 5G Core and CAG |
| C133 | IF A.4.1-5/1 AND A.4.3.7-1/21 THEN R ELSE N/A | UEs supporting 5G Core and RRC connection release with Deprioritisation |
| C134 | IF A.4.3.2-1/45 THEN R ELSE N/A | UEs supporting PUSCH repetition type B |
| C135 | IF A.4.3.2-1/46 THEN R ELSE N/A | UEs supporting 2-Step RACH |
| C135A | IF A.4.1-5/1 AND A.4.3.2-1/46 THEN R ELSE N/A | UEs supporting 5G Core and 2-Step RACH |
| C136 | IF A.4.1-5/1 AND A.4.4-1/3 THEN R ELSE N/A | UEs supporting 5G Core and delivery of rachReport upon request from the network |
| C137 | IF A.4.1-5/1 AND A.4.4-1/6 AND A.4.4-1/12 THEN R ELSE N/A | UEs supporting 5G core and logged MDT and Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state |
| C138 | IF A.4.1-5/1 AND A.4.4-1/6 AND A.4.4-1/13 THEN R ELSE N/A | UEs supporting 5G core and logged MDT and WLAN measurements in RRC_IDLE and RRC_INACTIVE state |
| C139 | IF A.4.1-5/1 AND (A.4.4-1/7 OR A.4.4-1/8 OR A.4.4-1/9) THEN R ELSE N/A | UEs supporting 5G Core and collection of sensor information such as Barometric pressure, UE speed, and UE orientation information as defined in TS 37.355 |
| C139a | IF A.4.1-5/1 AND (A.4.4-1/7 OR A.4.4-1/8 OR A.4.4-1/9) AND A.4.4-1/6 THEN R ELSE N/A | UEs supporting 5G Core and collection of sensor information such as Barometric pressure, UE speed, and UE orientation information as defined in TS 37.355 and logged MDT |
| C140 | IF A.4.1-5/1 AND A.4.4-1/10 THEN R ELSE N/A | UEs supporting 5G core and Bluetooth Measurement Collection in Immediate MDT |
| C141 | IF A.4.1-5/1 AND A.4.4-1/11 THEN R ELSE N/A | UEs supporting 5G core and WLAN Measurement Collection in Immediate MDT |
| C142 | IF A.4.1-5/1 AND A.4.3.5-1/10 THEN R ELSE N/A | UEs supporting 5G Core and PUSCH transmissions on multiple configured uplink grants |
| C143 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.4-1/4 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and standalone GNSS receiver to provide detailed location information |
| C144 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.4-1/6 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and logged measurements in RRC_IDLE and RRC_INACTIVE |
| C145 | IF A.4.1-5/1 AND A.4.3.7-1/29 THEN R ELSE N/A | UEs supporting 5G Core and release preference assistance information |
| C146 | IF A.4.3.2-1/52 THEN R ELSE N/A | UEs supporting monitoring DCI format 1_2 for DL scheduling and monitoring DCI format 0_2 for UL scheduling |
| C146a | Void | C |
| C147 | IF A.4.1-5/1 AND A.4.3.7-1/26 AND A.4.3.7-1/27 THEN R ELSE N/A | UEs supporting 5G Core and NSSAA and EAP-AKA’ for NSSAA |
| C148 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.3.7-1/21 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and RRC connection release with Deprioritisation |
| C149 | IF A.4.1-4/6 AND A.4.3.6-1/2 THEN R ELSE N/A | UEs supporting NR-DC and two independent measurement gap configurations for FR1 and FR2 |
| C150 | IF A.4.1-5/1 AND (A.4.3.6-1/48 OR A.4.3.6-1/49) THEN R ELSE N/A | UEs supporting 5G Core and SFTD measurements between NR PCell and NR neighbour cell |
| C151 | IF A.4.1-3/2 AND (A.4.3.6-1/43 OR A.4.3.6-1/44) AND (A.4.3.6-1/46 OR A.4.3.6-1/47) THEN R ELSE N/A | UEs supporting EN-DC and SFTD measurement between E-UTRA PCell and an NR neighbour cell, and SFTD measurement between E-UTRA PCell and NR PSCell |
| C152 | IF A.4.1-4/6 AND (A.4.3.6-1/48 OR A.4.3.6-1/49) AND (A.4.3.6-1/50 OR A.4.3.6-1/51) THEN R ELSE N/A | UEs supporting NR-DC and SFTD measurement between NR PCell and an NR neighbour cell, and SFTD measurement between NR PCell and NR PSCell |
| C153 | IF A.4.1-3/2 AND A.4.3.8-1/19 THEN R ELSE N/A | UEs supporting EN-DC and conditional PSCell change |
| C153A | IF A.4.1-3/2 AND (A.4.3.8-1/29 OR A.4.3.8-1/30 OR A.4.3.8-1/31) | UEs supporting EN-DC and MN initiated conditional PSCell change |
| C153B | IF A.4.1-3/1 AND A.4.3.8-1/28 | UEs supporting NR-DC and MN initiated conditional PSCell change |
| C154 | IF A.4.1-5/1 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.7-1/19 AND A.4.3.5-1/14 THEN R ELSE N/A | UEs supporting 5G Core and intra-band contiguous CA and RRC_INACTIVE and direct NR MCG SCell activation |
| C155 | IF A.4.1-5/1 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.7-1/19 AND A.4.3.5-1/14 THEN R ELSE N/A | UEs supporting 5G Core and intra-band non-contiguous CA and RRC_INACTIVE and direct NR MCG SCell activation |
| C156 | IF A.4.1-5/1 AND (A.4.1-4A/5 OR A.4.1-4A/6) AND A.4.3.7-1/19 AND A.4.3.5-1/14 THEN R ELSE N/A | UEs supporting 5G Core and inter-band CA and RRC_INACTIVE- and direct NR MCG SCell activation |
| C157 | IF A.4.1-4/6 AND A.4.3.7-1/3 AND A.4.3.7-1/1 THEN R ELSE N/A | UEs supporting NR-DC and SRB3 and (UL transmission via either MCG path or SCG path for the split SRB) |
| C158 | IF A.4.1-5/1 AND A.4.1-4/6 AND A.4.3.7-1/19 THEN R ELSE N/A | UEs supporting 5G Core and NR-DC and RRC_INACTIVE |
| C159 | IF A.4.1-5/2 AND [10] A.4.1-1/5 THEN R ELSE N/A | UEs supporting 5G core over non-3GPP Access Network and WLAN and additional UE-requested PDU establishment |
| C160 | IF A.4.1-3/3 THEN R ELSE N/A | UEs supporting NE-DC |
| C161 | IF A.4.1-5/1 AND A.4.3.7-1/21 AND [10] A.4.4-1/98 THEN R ELSE N/A | UEs supporting 5G Core and RRC connection release with Deprioritisation and ManualModeNetworkSelectionException |
| C162 | IF A.4.1-5/1 AND [9] A.22/8 AND A.4.3.7-1/36 AND [9] A.3A/50 AND [9] A.15/1 THEN R ELSE N/A | UEs supporting 5G Core and NG.114 v1.0 default configuration voice exempt and 3GPP PS data off and Initiating session and MTSI speech |
| C162A | IF A.4.1-5/1 AND [9] A.22/8 AND A.4.3.7-1/36 AND [9] A.3A/50 AND [9] A.3A/61 THEN R ELSE N/A | UEs supporting 5G Core and NG.114 v1.0 default configuration voice exempt and 3GPP PS data off and Initiating session and SMS over IP |
| C163 | IF A.4.1-5/1 AND A.4.1-1/3 AND A.4.3.10-1/3 THEN R ELSE N/A | UE supporting 5G core and NR sidelink and Sidelink CSI report |
| C164 | IF A.4.1-5/1 AND A.4.3.10-1/1 AND A.4.3.10-1/3 THEN R ELSE N/A | UE supporting 5G core and NR sidelink mode 1 transmission and Sidelink CSI report |
| C165 | IF A.4.1-5/1 AND A.4.3.7-1/33 THEN R ELSE N/A | UE supporting 5G Core and V2X communication |
| C166 | IF A.4.1-5/1 AND A.4.3.7-1/34 THEN R ELSE N/A | UE supporting 5G Core and V2X communication over NR-PC5 |
| C167 | IF A.4.1-5/1 AND A.4.3.7-1/24 AND A.4.3.7-1/30 THEN R ELSE N/A | UEs supporting 5G Core and SNPN and user initiated SNPN reselection in automatic mode on NR |
| C168 | IF A.4.1-5/1 AND A.4.3.7-1/23 AND A.4.3.7-1/31 THEN R ELSE N/A | UEs supporting 5G Core and CAG and Autonomous search function on NR |
| C169 | IF A.4.1-5/1 AND A.4.3.7-1/23 AND A.4.3.6-1/52 THEN R ELSE N/A | UEs supporting 5G Core and CAG and acquisition of CGI information from neighbour NR NPN cell |
| C170 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [9]A.12/64 AND [11]A.10/16 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation |
| C171 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [9]A.12/64 AND [11]A.10/16 AND [11]A.10/19 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and capable of triggering a Test eCall |
| C172 | IF A.4.1-5/1 AND [9] A.22/9 AND A.4.3.7-1/36 AND [9] A.3A/50 AND [9] A.15/3 AND [9] A.15/11 AND [9] A.15/12 AND [9] A.15/13 THEN R ELSE N/A | UEs supporting 5G Core and NG.114 v2.0 default configuration video exempt and 3GPP PS data off and Initiating session and MTSI video and MTSI video H.265 MP MT Level 3.1 and MTSI video H.264 CHP Level 3.1 and H.264 CBP Level 3.1 |
| C173 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [9] A.21/2 | UEs supporting 5G Core and E-UTRA and NG.114 v2.0 |
| C174 | IF A.4.1-5/1 AND [9]A.12/64 AND [11] A.10/16 AND [11] A.10/19 THEN R ELSE N/A | UEs supporting 5G Core and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and capable of triggering a Test eCall |
| C175 | IF A.4.3.5-1/11 THEN R ELSE N/A | UEs supporting 5GS and selection of logical channels for each UL grant based on RRC configured restriction |
| C176 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.2.1.1-1/4 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") |
| C177 | IF A.4.1-5/1 AND A.4.3.7-1/17 AND A.4.3.7-1/35 THEN R ELSE N/A | UEs supporting 5G Core and RACS and Manufacturer assigned Radio Capability ID |
| C178 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.3.7-1/17 AND [10]A.4.4-1/215THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and RACS |
| C179 | IF A.4.3.2-1/80 THEN R ELSE N/A | UEs supporting DCI DL Priority Indicator |
| C180 | IF A.4.3.2-1/81 AND A.4.3.2-1/82 THEN R ELSE N/A | UEs supporting DCI UL Priority Indicator and LCH grant prioritisation |
| C181 | IF (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.2A.1-1/2 AND A.4.3.2A.1-2/2 AND A.4.3.3-1/5 THEN R ELSE N/A | UEs supporting 5GC and Intra-band non-contiguous CA and DL and UL NR CA with 3 carriers and PDCP duplication with more than two RLC entities |
| C182 | IF A.4.1-3/3 AND A.4.3.6-1/1 AND A.4.3.6-1/3 THEN R ELSE N/A | UEs supporting NE-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting). |
| C183 | IF A.4.1-3/3 AND A.4.3.6-1/1 AND A.4.3.6-1/3 AND (A.4.1-2/1 OR A.4.1-2/2 OR (A.4.1-1/1 AND A.4.1-1/2)) THEN R ELSE N/A | UEs supporting NE-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and multiple NR bands |
| C184 | IF A.4.1-5/1 AND [9]A.12/64 AND [11]A.10/17 THEN R ELSE N/A | UEs supporting 5G Core and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| C185 | IF A.4.1-5/1 AND [10] A.4.1-1/6 AND [9] A.12/63 AND [11] A.10/17 AND A.4.3.7-1/14 THEN R ELSE N/A | UEs supporting 5G Core and UTRA and IMS eCall type of emergency services over 5GS and Automatic type of eCall initiation and emergency services in NR connected to 5GCN |
| C186 | IF A.4.1-5/1 AND [10] A.4.1-1/6 AND [9] A.12/64 AND [11] A.10/16 AND A.4.3.8-1/14 THEN R ELSE N/A | UEs supporting 5G Core and UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and NR to UTRA-FDD CELL_DCH CS handover |
| C187 | IF A.4.1-5/1 AND ([10] A.4.1-1/6 OR [10] A.4.1-1/7) AND [9] A.12/64 AND [11] A.10/16 THEN R ELSE N/A | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation |
| C188 | IF A.4.1-5/1 AND ([10] A.4.1-1/6 OR [10] A.4.1-1/7) AND [9] A.12/64 AND [11] A.10/17 THEN R ELSE N/A | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| C189 | IF A.4.1-5/1 AND ([10] A.4.1-1/6 OR [10] A.4.1-1/7) AND [9] A.12/63 AND [11] A.10/16 THEN R ELSE N/A | UEs supporting 5G Core and (UTRA OR GERAN) and eCall type of emergency services over 5GS and Manual type of eCall initiation |
| C190 | IF A.4.1-5/1 AND A.4.3.6-1/54 THEN R ELSE N/A | UEs supporting 5G Core and Idle/Inactive Measurements |
| C191 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.3.6-1/55 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and Idle/Inactive Measurements |
| C192 | IF A.4.1-5/1 AND A.4.3.7-1/19 AND A.4.3.6-1/54 THEN R ELSE N/A | UEs supporting 5G Core and RRC_INACTIVE and Idle/Inactive Measurements |
| C193 | IF A.4.1-5/1 AND A.4.3.7-1/19 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.3.6-1/55 THEN R ELSE N/A | UEs supporting 5G Core and RRC_INACTIVE and E-UTRA and Idle/Inactive Measurements |
| C194 | IF A.4.1-3/3 AND A.4.3.7-1/2 THEN R ELSE N/A | UEs supporting NE-DC and UL transmission via both MCG path and SCG path for the split DRB |
| C195 | IF A.4.1-4/6 AND A.4.3.3-1/6 THEN R ELSE N/A | UEs supporting NR-DC and PDCP duplication over split SRB1/2 |  |
| C196 | IF A.4.1-3/3 AND A.4.3.3-1/6 THEN R ELSE N/A | UEs supporting NE-DC and PDCP duplication over split SRB1/2 |  |
| C197 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [9]A.12/64 AND [11]A.10/17 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| C198 | IF A.4.1-5/1 AND [9] A.6a/2 THEN R ELSE N/A | UEs supporting 5G Core and IMS security |  |
| C199 | IF A.4.1-3/2 AND A.4.3.5-1/12 AND (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A | UEs supporting EN-DC, direct NR SCG SCell activation and Intra-Band Contiguous CA |  |
| C200 | IF A.4.1-3/2 AND A.4.3.5-1/12 AND (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A | UEs supporting EN-DC, direct NR SCG SCell activation and Intra-Band Non-Contiguous CA |  |
| C201 | IF A.4.1-3/2 AND A.4.3.5-1/12 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A | UEs supporting EN-DC, direct NR SCG SCell activation and Inter-Band CA |  |
| C202 | IF A.4.1-4/6 AND A.4.3.5-1/12 AND (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A | UEs supporting NR-DC, direct NR SCG SCell activation and intra-band contiguous CA |  |
| C203 | IF A.4.1-4/6 AND A.4.3.5-1/12 AND (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A | UEs supporting NR-DC, direct NR SCG SCell activation and intra-band non-contiguous CA |  |
| C204 | IF A.4.1-4/6 AND A.4.3.5-1/12 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7)THEN R ELSE N/A | UEs supporting NR-DC, direct NR SCG SCell activation and inter-band CA |  |
| C205 | Void |  |  |
| C206 | IF A.4.1-3/3 AND A.4.3.6-1/5 THEN R ELSE N/A | UEs supporting NE-DC and Inter-RAT E-UTRA measurements and Event B triggered reporting |  |
| C207 | IF A.4.1-5/1 AND A.4.3.7-1/39 THEN R ELSE N/A | UEs supporting 5G core and reception of segmented DL RRC messages. |  |
| C208 | IF A.4.1-5/1 AND A.4.4-1/2 AND A.4.3.8-1/20 AND [10] A.4.1-1/5 AND [10] A.4.4-1/117 THEN R ELSE N/A | UEs supporting 5G Core and IMS and handover from 5G Core to EPC over non-3GPP Access Network and GSMA PRD IR.51: "IMS Profile for Voice, Video and SMS over Wi-Fi" and WLAN. |  |
| C209 | IF A.4.1-5/1 AND A.4.3.12-1/2 AND A.4.3.12-1/6 AND A.4.3.12-1/7 THEN R ELSE N/A | UEs supporting 5G Core and RedCap and relaxed RRM measurements in RRC_CONNECTED and initiating UE Assistance Information procedure immediately upon change of its fulfilment status for RRM measurement relaxation criterion for connected mode. |  |
| C210 | IF A.4.1-5/1 AND A.4.3.7-1/43 THEN R ELSE N/A | UEs supporting 5G Core and eDRX |  |
| C210A | IF A.4.1-5/1 AND A.4.3.7-1/43 AND A.4.3.7-1/19 THEN R ELSE N/A | UEs supporting 5G Core and eDRX and RRC_INACTIVE |  |
| C211 | IF A.4.3.2-1/85 THEN R ELSE N/A | UEs supporting repetition of Message 3 PUSCH |  |
| C212 | IF A.4.1-5/1 AND A.4.3.12-1/2 THEN R ELSE N/A | UEs supporting 5G Core and RedCap |  |
| C212a | IF A.4.1-5/1 AND A.4.3.12-1/2 AND A.4.3.7-1/19 THEN R ELSE N/A | UEs supporting 5G Core and RedCap and RRC_INACTIVE |  |
| C213 | IF A.4.1-5/1 AND A.4.3.14-1/1 THEN R ELSE N/A | UE supporting 5G Core and broadcast reception |  |
| C214 | IF A.4.1-5/1 AND A.4.3.14-1/2 THEN R ELSE N/A | UE supporting 5G Core and dynamic scheduling for multicast for PCell |  |
| C215 | IF A.4.1-5/1 AND A.4.3.14-1/2 AND A.4.3.14-1/3 THEN R ELSE N/A | UE supporting 5G Core and dynamic scheduling for multicast for PCell and ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast |  |
| C216 | IF A.4.1-5/1 AND A.4.3.14-1/2 AND A.4.3.14-1/3 AND A.4.3.14-1/4 THEN R ELSE N/A | UE supporting 5G Core and dynamic scheduling for multicast for PCell and ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast and PTP retransmission for multicast on the same cell as multicast initial transmission |  |
| C217 | IF A.4.1-5/1 AND A.4.3.2-2/3 THEN R ELSE N/A | UEs supporting 5G Core and NR standalone shared spectrum channel access |  |
| C218 | IF A.4.1-5/1 AND A.4.3.2-2/3 AND A.4.3.2-2/19 THEN R ELSE N/A | UEs supporting 5G Core and NR standalone shared spectrum channel access and RSSI measurements and channel occupancy reporting |  |
| C219 | IF A.4.1-5/1 AND A.4.3.13-1/1 THEN R ELSE N/A | UEs supporting 5G Core and Multi-SIM features |  |
| C220 | IF A.4.1-5/1 AND A.4.3.13-1/4 THEN R ELSE N/A | UEs supporting 5G Core and Multi-SIM Reject paging request |  |
| C221 | IF A.4.1-5/1 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.7-1/19 AND A.4.3.5-1/15 AND A.4.1-4/6 THEN R ELSE N/A | UEs supporting 5G Core and intra-band contiguous CA and RRC_INACTIVE and direct NR SCG SCell activation and NR-DC |  |
| C222 | IF A.4.1-5/1 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.7-1/19 AND A.4.3.5-1/15 AND A.4.1-4/6 THEN R ELSE N/A | UEs supporting 5G Core and intra-band non-contiguous CA and RRC_INACTIVE and direct NR SCG SCell activation and NR-DC |  |
| C223 | IF A.4.1-5/1 AND (A.4.1-4A/5 OR A.4.1-4A/6) AND A.4.3.7-1/19 AND A.4.3.5-1/15 AND A.4.1-4/6 THEN R ELSE N/A | UEs supporting 5G Core and inter-band CA and RRC_INACTIVE and direct NR SCG SCell activation and NR-DC |  |
| C224 | IF A.4.1-5/1 AND A.4.3.7-1/42 THEN R ELSE N/A | UEs supporting 5G Core and PEI |  |
| C224A | IF A.4.1-5/1 AND A.4.3.7-1/42 AND A.4.3.7-1/61 THEN R ELSE N/A | UEs supporting 5G Core and PEI and PEIPS |  |
| C225 | IF A.4.1-3/2 AND (A.4.3.6-1/61 OR A.4.3.6-1/62) THEN R ELSE N/A | UEs supporting EN-DC and Idle/Inactive Measurements |  |
| C226 | IF A.4.1-5/1 AND A.4.3.5-1/13 AND (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A | UEs supporting 5G Core and direct NR MCG SCell activation and intra-band contiguous CA |  |
| C227 | IF A.4.1-5/1 AND A.4.3.5-1/13 AND (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A | UEs supporting 5G Core and direct NR MCG SCell activation and intra-band non-contiguous CA |  |
| C228 | IF A.4.1-5/1 AND A.4.3.5-1/13 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A | UEs supporting 5G Core and direct NR MCG SCell activation and inter-band CA |  |
| C229 | IF A.4.1-5/1 AND A.4.1-4/6 AND A.4.3.7-1/19 AND A.4.3.7-1/44 THEN R ELSE N/A | UEs supporting 5G Core and NR-DC and RRC_INACTIVE and (re-)configuration of an SCG during the resume procedure. |  |
| C230 | IF A.4.1-5/1 AND A.4.3.7-1/37 THEN R ELSE N/A | UEs supporting 5G Core and NSSRG |  |
| C231 | IF A.4.1-5/1 AND A.4.3.7-1/24 AND A.4.3.7-1/40 THEN R ELSE N/A | UEs supporting 5G Core and SNPN and configuration of access identities in the list of subscriber data |  |
| C232 | IF A.4.3.2-1/46 AND A.4.4-1/14 THEN R ELSE N/A | UEs Supporting 2-Step RACH and Random access SDT |  |
| C233 | IF A.4.4-1/14 THEN R ELSE N/A | UEs Supporting Random access SDT |  |
| C234 | IF [9] A.18/5 AND A.4.3.7-1/32 AND [9] A.15/1 AND [9] A.4/16 AND [9] A.21/1 AND [9] A.22/11 THEN R ELSE N/A | NR and IMS voice over NR and MTSI and MTSI speechand preconditions and NG.114 v1.0 and NG.114 v1.0 default configuration EVS/Br and NG.114 v1.0 default configuration EVS/Bw |  |
| C235 | IF A.4.3.3-1/8 THEN R ELSE N/A | UEs supporting 5GS and uplink data compression operation |  |
| C236 | IF A.4.3.3-1/8 and A.4.3.3-1/9 THEN R ELSE N/A | UEs supporting 5GS and uplink data compression operation and UL data compression with SIP static dictionary |  |
| C237 | IF [10] A.4.4-1/117 AND [10] A.4.1-1/5 AND A.4.3.8-1/21 AND A.4.4-1/2 AND A.4.1-5/1 THEN R ELSE N/A | UEs supporting WLAN and GSMA PRD IR.51: "IMS Profile for Voice, Video and SMS over Wi-Fi" and handover from EPC over non-3GPP Access Network to 5G Core and IMS and 5G Core.. |  |
| C238 | IF A.4.1-5/1 AND A.4.3.7-1/14 AND [11] A.20/90 THEN R ELSE N/A | UEs supporting 5G Core and emergency services in NR connected to 5GCN and test execution with No USIM |  |
| C239 | IF A.4.1-5/1 AND A.4.3.7-1/42 AND A.4.3.7-1/19 THEN R ELSE N/A | UEs supporting 5G Core and RRC_INACTIVE and PEI |  |
| C240 | IF A.4.1-5/1 AND A.4.3.7-1/38 THEN R ELSE N/A | UEs supporting 5G Core and slice based cell reselection |
| C241 | IF A.4.1-5/1 AND A.4.3.7-1/19 AND A.4.3.7-1/38 THEN R ELSE N/A | UEs supporting 5G Core and RRC_INACTIVE and slice based cell reselection |
| C242 | IF A.4.1-5/1 AND A.4.3.13-1/2 THEN R ELSE N/A | UEs supporting 5G Core and Multi-SIM N1 NAS signalling connection release |
| C243 | IF A.4.1-5/1 AND A.4.3.2-2/2 THEN R ELSE N/A | UEs supporting 5G Core and EN-DC with NR shared spectrum channel access |
| C244 | IF A.4.1-5/1 AND A.4.3.2-2/5 THEN R ELSE N/A | UEs supporting 5G Core and NR-DC with NR shared spectrum channel access |
| C245 | IF A.4.1-5/1 AND A.4.3.13-1/1 AND A.4.3.7-1/29 THEN R ELSE N/A | UEs supporting 5G Core and Multi-SIM features and release preference assistance information |
| C246 | IF A.4.1-5/1 AND A.4.3.13-1/6 THEN R ELSE N/A | UEs supporting 5G Core and MUSIM gap feature. |
| C247 | IF A.4.1-5/1 AND A.4.3.2-2/3 AND A.4.3.7-1/19 THEN R ELSE N/A | UEs supporting 5G Core and NR standalone shared spectrum channel access and RRC_INACTIVE |
| C248 | IF A.4.1-5/1 AND A.4.3.8-1/24 AND [10] A.4.1-1/5 AND THEN R ELSE N/A | UEs supporting 5G Core and handover from 5G Core over non-3GPP Access Network to 5G Core Network and WLAN |
| C249 | IF A.4.1-5/1 AND A.4.3.8-1/23 AND [10] A.4.1-1/5 THEN R ELSE N/A | UEs supporting 5G Core and handover from 5G Core over non-3GPP Access Network to EPC Network and WLAN |
| C250 | IF A.4.1-5/1 AND A.4.4-1/21 THEN R ELSE N/A | UEs supporting 5G Core and storage and delivery of multiple CEF report upon request from the network |
| C251 | IF A.4.1-5/1 AND A.4.1-5/2 AND [10] A.4.1-1/5 AND A.4.3.7-1/9 and A.4.3.7-1/48 THEN R ELSE N/A | UEs supporting 5G Core and 5G core over non-3GPP Access Network and WLAN and additional UE-requested PDU establishment and ATSSS |
| C252 | IF A.4.1-5/1 AND A.4.3.14-1/2 AND A.4.3.14-1/5 THEN R ELSE N/A | UE supporting 5G Core and dynamic scheduling for multicast for PCell and NACK-only based HARQ-ACK feedback for multicast with ACK/NACK transforming |
| C253 | IF A.4.1-5/1 AND A.4.3.14-1/2 AND A.4.3.14-1/3 AND A.4.3.14-1/7 THEN R ELSE N/A | UE supporting 5G Core and dynamic scheduling for multicast for PCell and ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast and multiplexing HARQ-ACK for unicast and for multicast with the same priority and different HARQ-ACK codebook types in a PUCCH or in a PUSCH |
| C254 | IF A.4.1-5/1 AND A.4.3.14-1/2 AND A.4.3.7-1/19 THEN R ELSE N/A | UE supporting 5G Core and dynamic scheduling for multicast for PCell and RRC_INACTIVE |
| C255 | IF A.4.1-3/3 AND A.4.3.7-1/19 AND A.4.3.7-1/44 THEN R ELSE N/A | UEs supporting NE-DC and RRC_INACTIVE and (re-)configuration of an SCG during the resume procedure. |
| C256 | IF A.4.3.5-1/16 AND A.4.1-4/6 AND A.4.3.3-1/4 THEN R ELSE N/A | UEs supporting services with survival time and NR-DC and PDCP-duplication over split DRB |
| C257 | IF A.4.3.5-1/16 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.3-1/3 THEN R ELSE N/A | UEs supporting services with survival time and intra-band contiguous CA and CA-based PDCP duplication over MCG or SCG DRB |
| C258 | IF A.4.3.5-1/16 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.3-1/3 THEN R ELSE N/A | UEs supporting services with survival time and intra-band non-contiguous CA and CA-based PDCP duplication over MCG or SCG DRB |
| C259 | IF A.4.3.5-1/16 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.3-1/3 THEN R ELSE N/A | UEs supporting services with survival time and inter-band CA and CA-based PDCP duplication over MCG or SCG DRB |
| C260 | IF ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.3.7-1/37 THEN R ELSE N/A | UEs supporting 5GS and E-UTRA and NSSRG. |
| C261 | IF A.4.1-5/1 AND A.4.3.7-1/9 AND A.4.3.7-1/14 THEN R ELSE N/A | UEs supporting 5G Core and additional UE-requested PDU establishment and emergency services in NR connected to 5GCN |
| C262 | IF A.4.3.7-1/45 AND A.4.3.7-1/46 THEN R ELSE N/A | UEs supporting slice-based RACH partitioning and slice-based RACH prioritisation |
| C263 | IF A.4.3.7-1/45 AND A.4.3.7-1/46 AND A.4.3.7-1/47 THEN R ELSE N/A | UEs supporting slice-based RACH partitioning, slice-based RACH prioritisation and RACH prioritisation for Access Identity 1 |
| C264 | IF A.4.3.2-1/46 AND A.4.3.7-1/45 AND A.4.3.7-1/46 THEN R ELSE N/A | UEs supporting 2-Step RACH, slice-based RACH partitioning and slice-based RACH prioritisation |
| C265 | IF A.4.3.2-1/46 AND A.4.3.7-1/45 AND A.4.3.7-1/46 AND A.4.3.7-1/47 THEN R ELSE N/A | UEs supporting 2-Step RACH, slice-based RACH partitioning, slice-based RACH prioritisation and RACH prioritisation for Access Identity 1 |
| C266 | IF A.4.1-5/1 AND A.4.4-1/6 AND A.4.4-1/23 AND A.4.3.1-2/1b THEN R ELSE N/A | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE and IDC mechanism and FR1 Band n40 |
| C267 | IF A.4.1-5/1 AND A.4.4-1/6 AND A.4.4-1/22 THEN R ELSE N/A | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE and early measurements |
| C268 | IF A.4.1-3/3 AND A.4.3.6-1/75 THEN R ELSE N/A | UEs supporting NE-DC and SFTD measurement between NR PCell and E-UTRA PSCell |
| C269 | IF A.4.1-5/1 AND A.4.4-1/16 THEN R ELSE N/A | UEs supporting 5G Core and SDT via Configured Grant Type 1 in RRC_INACTIVE state |
| C270 | IF A.4.1-5/1 AND A.4.4-1/15 AND A.4.4-1/16 THEN R ELSE N/A | UEs supporting 5G Core and SRB SDT and SDT via Configured Grant Type 1 in RRC_INACTIVE state |
| C271 | IF A.4.3.3-1/8 and A.4.3.3-1/10 THEN R ELSE N/A | UEs supporting 5GS and uplink data compression operation and continuation of uplink data compression protocol operation |
| C272 | IF A.4.1-4/6 and A.4.3.3-1/8 THEN R ELSE N/A | UEs supporting NR-DC and uplink data compression operation |
| C273 | IF A.4.1-3/3 and A.4.3.3-1/8 THEN R ELSE N/A | UEs supporting NE-DC and uplink data compression operation |
| C274 | IF A.4.1-5/1 AND A.4.4-1/26 THEN R ELSE N/A | UEs supporting 5G Core and RRC Connection release with MPS priority indication |
| C274A | IF A.4.1-5/1 AND A.4.4-1/26 AND A.4.3.7-1/19 THEN R ELSE N/A | UEs supporting 5G Core and RRC Connection release with MPS priority indication AND RRC_INACTIVE |
| C275 | IF A.4.1-5/1 AND A.4.1-5/2 AND [10] A.4.1-1/5 AND A.4.3.7-1/13 and A.4.3.7-1/48 THEN R ELSE N/A | UEs supporting 5G Core and 5G core over non-3GPP Access Network and WLAN and UE-requested PDU modification and ATSSS |
| C276 | IF A.4.1-5/1 AND A.4.3.8-1/25 AND [10] A.4.1-1/5 AND THEN R ELSE N/A | UEs supporting 5G Core and handover from 5G Core Network to 5G Core over non-3GPP Access Network and WLAN |
| C277 | IF A.4.1-5/1 AND A.4.3.8-1/22 AND [10] A.4.1-1/5 THEN R ELSE N/A | UEs supporting 5G Core and handover from EPC Network to 5G Core over non-3GPP Access Network and WLAN |
| C278 | IF A.4.1-5/1 AND A.4.4-1/24 THEN R ELSE N/A | UEs supporting 5G Core and delivery of delivery of 2-step RACH related information upon request from the network |
| C279 | IF A.4.1-5/1 AND A.4.4-1/25 THEN R ELSE N/A | UEs supporting 5G Core and delivery of delivery of 2-step RACH related information upon request from the network. |
| C280 | IF A.4.1-5/1 AND A.4.3.14-1/11 AND (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A | UE supporting 5G Core and broadcast reception on SCell and Intra-band Contiguous CA |
| C281 | IF A.4.1-5/1 AND A.4.3.14-1/11 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A | UE supporting 5G Core and broadcast reception on SCell and Inter-band CA |
| C282 | IF A.4.1-5/1 AND A.4.3.14-1/11 AND (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A | UE supporting 5G Core and broadcast reception on SCell and Intra-band non Contiguous CA |
| C283 | IF A.4.1-5/1 AND A.4.3.14-1/2 AND A.4.3.14-1/8 THEN R ELSE N/A | UE supporting 5G Core and dynamic scheduling for multicast for PCell and DCI formate 4_2 |
| C284 | IF A.4.1-5/1 AND A.4.3.14-1/2 AND A.4.3.14-1/8 AND A.4.3.14-1/9 THEN R ELSE N/A | UE supporting 5G Core and dynamic scheduling for multicast for PCell and DCI formate 4-2 and DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signalling via DCI format 4_2 |
| C285 | IF A.4.1-5/1 AND A.4.3.14-1/2 AND A.4.3.14-1/8 AND A.4.3.14-1/10 THEN R ELSE N/A | UE supporting 5G Core and dynamic scheduling for multicast for PCell and DCI formate 4-2 and DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signalling via DCI format 4_2 |
| C286 | IF A.4.1-3/2 AND A.4.3.7-1/50 THEN R ELSE N/A | UEs supporting EN-DC and user plane integrity protection with EPS |
| C287 | IF A.4.3.2-1/114 THEN R ELSE N/A | UEs supporting dynamic indication of PUCCH repetition |
| C288 | IF A.4.3.2-1/115 AND A.4.3.2-1/113 THEN R ELSE N/A | UEs supporting increased maximum number of PUSCH Type A repetitions and dynamic indication of the number of repetitions for PUSCH |
| C289 | IF A.4.3.2-1/115 AND (A.4.3.2-1/111 OR A.4.3.2-1/112) THEN R ELSE N/A | UEs supporting increased maximum number of PUSCH Type A repetitions and PUSCH transmissions with configured grant |
| C290 | IF A.4.3.2-1/116 AND A.4.3.2-1/113 THEN R ELSE N/A | UEs supporting PUSCH repetitions based on available slots and dynamic indication of the number of repetitions for PUSCH |
| C291 | IF A.4.3.2-1/116 AND (A.4.3.2-1/111 OR A.4.3.2-1/112) THEN R ELSE N/A | UEs supporting PUSCH repetitions based on available slots and PUSCH transmissions with configured grant |
| C292 | IF A.4.3.2-1/117 THEN R ELSE N/A | UEs supporting TB processing over multi-slot PUSCH |
| C293 | IF A.4.3.2-1/118 THEN R ELSE N/A | UEs supporting repetition of TB processing over multi-slot PUSCH |
| C294 | IF A.4.1-5/1 AND A.4.3.13-1/1 AND A.4.3.13-1/6 AND A.4.3.13-1/7 THEN R ELSE N/A | UEs supporting 5G Core and Multi-SIM features and MUSIM related assistance information |
| C295 | IF A.4.1-5/1 AND A.4.3.14-1/2 AND A.4.3.14-1/16 THEN R ELSE N/A | UE supporting 5G Core and dynamic scheduling for multicast for PCell and Multiple G-RNTIs. |
| C296 | IF A.4.1-5/1 AND A.4.3.14-1/2 AND A.4.3.14-1/12 THEN R ELSE N/A | UE supporting 5G Core and dynamic scheduling for multicast for PCell and SPS group-common PDSCH for multicast on PCell. |
| C297 | IF A.4.1-5/1 AND A.4.3.14-1/2 AND A.4.3.14-1/12 AND A.4.3.14-1/13 THEN R ELSE N/A | UE supporting 5G Core and dynamic scheduling for multicast for PCell and SPS group-common PDSCH for multicast on PCell and ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast. |
| C298 | IF A.4.1-5/1 AND A.4.3.14-1/2 AND A.4.3.14-1/12 AND A.4.3.14-1/13 AND A.4.3.14-1/14 THEN R ELSE N/A | UE supporting 5G Core and dynamic scheduling for multicast for PCell and SPS group-common PDSCH for multicast on PCell and ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast and PTP retransmission associated with CS-RNTI for SPS multicast on the cell same as multicast initial transmission. |
| C299 | IF A.4.1-5/1 AND A.4.3.14-1/2 AND A.4.3.14-1/12 AND A.4.3.14-1/15 THEN R ELSE N/A | UE supporting 5G Core and dynamic scheduling for multicast for PCell and SPS group-common PDSCH for multicast on PCell and unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH. |
| C300 | IF A.4.1-5/1 AND A.4.3.2-2/1 AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting 5G Core and NR CA with NR shared spectrum channel access and UL NR CA with 2 carriers |
| C301 | IF A.4.1-5/1 AND A.4.4-1/27 THEN R ELSE N/A | UEs supporting 5G Core and RLF-Report for conditional handover |
| C302 | IF A.4.1-5/1 AND A.4.4-1/28 THEN R ELSE N/A | UEs supporting 5G Core and RLF-Report for DAPS handover. |
| C303 | IF A.4.1-5/1 AND A.4.4-1/29 THEN R ELSE N/A | UEs supporting 5G Core and the storage and delivery of Successful Handover Report. |
| C304 | IF A.4.1-5/1 AND A.4.3.7-1/52 THEN R ELSE N/A | UEs supporting 5G Core and access SNPN using credentials assigned by a Credentials Holder separate from the SNPN |
| C305 | IF A.4.1-5/1 AND A.4.3.7-1/53 THEN R ELSE N/A | UEs supporting 5G Core and onboarding services in SNPN(hence supports Default UE Credentials) |
| C306 | IF A.4.1-5/1 AND A.4.3.7-1/56 THEN R ELSE N/A | UEs supporting 5G Core and emergency services in SNPN |
| C307 | IF A.4.1-5/1 AND A.4.3.7-1/52 AND A.4.3.7-1/30 THEN R ELSE N/A | UEs supporting 5G Core and accessing SNPN using credentials from a Credentials Holder and user initiated SNPN reselection in automatic mode on NR. |
| C308 | IF A.4.1-5/1 AND A.4.3.7-1/56 AND A.4.3.7-1/57 AND A.4.3.7-1/32 THEN R ELSE N/A | UEs supporting 5G Core and PLMN access in SNPN Access mode and emergency services in NR connected to 5GCN in SNPN Access mode And IMS voice over NR |
| C309 | IF A.4.1-5/1 AND A.4.4-1/17 THEN R ELSE N/A | UEs supporting 5G Core and NR NTN access |
| C310 | IF A.4.1-5/1 AND A.4.3.7-1/51 THEN R ELSE N/A | UEs supporting 5G Core and UAS |
| C311 | IF A.4.3.2-1/123 THEN R ELSE N/A | UEs supporting 5GS and unified TCI state operation with joint DL/UL TCI update for intra-cell beam management |
| C312 | IF A.4.3.2-1/133 THEN R ELSE N/A | UEs supporting 5GS and unified separate TCI with multi-MAC-CE |
| C313 | IF A.4.1-5/1 AND A.4.3.7-1/59 THEN R ELSE N/A | UEs supporting 5G Core and MICO mode |
| C314 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.4-1/26 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and RRC Connection release with MPS priority indication |
| C315 | IF A.4.1-5/1 AND A.4.3.2-1/134 THEN R ELSE N/A | UEs supporting 5G Core and partial frequency sounding for SRS with frequency hopping. |
| C316 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.4-1/33 AND A.4.3.7-1/12 AND A.4.3.7-1/58 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback and being configured for No E-UTRA Disabling In 5GS |
| C317 | IF A.4.1-5/1 AND A.4.4-1/16 AND A.4.3.7-1/6 THEN R ELSE N/A | UEs supporting 5G Core and SDT via Configured Grant Type 1 in RRC_INACTIVE state and SMS over NAS |
| C318 | IF A.4.1-5/1 AND A.4.3.2-2/1 THEN R ELSE N/A | UEs supporting 5G Core and NR CA with NR shared spectrum channel access |
| C319 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.1-5/2 AND [10] A.4.1-1/5 AND A.4.3.7-1/13 AND A.4.3.7-1/60 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and 5G core over non-3GPP Access Network and WLAN and R17 ATSSS of establishing a PDN connection as the user plane resource of an MA PDU session in 5GS |
| C320 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.1-5/2 AND [10] A.4.1-1/5 AND A.4.3.7-1/13 AND A.4.3.7-1/48 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and 5G core over non-3GPP Access Network and WLAN and R16 ATSSS |
| C321 | IF A.4.1-5/1 AND A.4.4-1/17 AND A.4.3.2-1/138 THEN R ELSE N/A | UEs supporting 5G Core and NR NTN access and UE reporting of information related to TA pre-compensation |
| C322 | IF A.4.1-5/1 AND A.4.4-1/17 AND A.4.3.4-1/3 THEN R ELSE N/A | UEs supporting 5G Core and NR NTN access and RLC UM Mode |
| C323 | IF A.4.1-5/1 AND A.4.4-1/17 AND A.4.3.6-1/76 THEN R ELSE N/A | UE supporting 5G Core and NR NTN access and UE supporting location-based triggered measurement reporting (i.e., event D1) |
| C324 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.3.7-1/58 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and No E-UTRA Disabling In 5GS |
| CXXXHS1 | IF (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.2A.1-1/1 AND A.4.3.2-1/XXHS1 AND A.4.3.2-1/XXHS3 THEN R ELSE N/A | UEs supporting 5G Core and intra-band contiguous CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set and co-scheduled cell indication scheme based on FDRA field of DCI format 1_3 |
| CXXXHS3 | IF (A.4.1-4A/5 OR A.4.1-4A/6) AND A.4.3.2A.1-1/1 AND A.4.3.2-1/XXHS1 AND A.4.3.2-1/XXHS3 THEN R ELSE N/A | UEs supporting 5G Core and inter-band CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set and co-scheduled cell indication scheme based on FDRA field of DCI format 1_3 |
| CXXXHS5 | IF (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.2A.1-1/1 AND A.4.3.2-1/XXHS1 AND A.4.3.2-1/XXHS3 THEN R ELSE N/A | UEs supporting 5GCore and intra-band non-contiguous CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set and co-scheduled cell indication scheme based on FDRA field of DCI format 1_3 |
| CXXXHS7 | IF (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.2A.1-1/1 AND A.4.3.2-1/XXHS1 AND A.4.3.2-1/XXHS4 THEN R ELSE N/A | UEs supporting 5G Core and intra-band contiguous CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set and co-scheduled cell indication scheme based on Scheduled cells indicator field of DCI format 1_3 |
| CXXXHS9 | IF (A.4.1-4A/5 OR A.4.1-4A/6) AND A.4.3.2A.1-1/1 AND A.4.3.2-1/XXHS1 AND A.4.3.2-1/XXHS4 THEN R ELSE N/A | UEs supporting 5G Core and inter-band CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set and co-scheduled cell indication scheme based on Scheduled cells indicator field of DCI format 1_3 |
| CXXXHS11 | IF (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.2A.1-1/1 AND A.4.3.2-1/XXHS1 AND A.4.3.2-1/XXHS4 THEN R ELSE N/A | UEs supporting 5G Core and intra-band non-contiguous CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set and co-scheduled cell indication scheme based on Scheduled cells indicator field of DCI format 1_3 |
| CXXXHS13 | IF (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.2A.1-1/1 AND A.4.3.2-1/XXHS1 THEN R ELSE N/A | UEs supporting 5G Core and intra-band contiguous CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set |
| CXXXHS15 | IF (A.4.1-4A/5 OR A.4.1-4A/6) AND A.4.3.2A.1-1/1 AND A.4.3.2-1/XXHS1 THEN R ELSE N/A | UEs supporting 5G Core and inter-band CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set |
| CXXXHS17 | IF (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.2A.1-1/1 AND A.4.3.2-1/XXHS1 THEN R ELSE N/A | UEs supporting 5G Core and intra-band non-contiguous CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set |
| CXXXHS19 | IF (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.2A.1-2/1 AND A.4.3.2-1/XXHS2 THEN R ELSE N/A | UEs supporting 5G Core and intra-band contiguous CA and UL NR CA with 2 carriers and DCI format 0_3 with same SCS between scheduling cell and cells in the set |
| CXXXHS21 | IF (A.4.1-4A/5 OR A.4.1-4A/6) AND A.4.3.2A.1-2/1 AND A.4.3.2-1/XXHS2 THEN R ELSE N/A | UEs supporting 5G Core and inter-band CA and UL NR CA with 2 carriers and DCI format 0_3 with same SCS between scheduling cell and cells in the set |
| CXXXHS23 | IF (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.2A.1-2/1 AND A.4.3.2-1/XXHS2 THEN R ELSE N/A | UEs supporting 5G Core and intra-band non-contiguous CA and UL NR CA with 2 carriers and DCI format 0_3 with same SCS between scheduling cell and cells in the set |
| CXXXHS25 | IF (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.2A.1-1/1 AND A.4.3.2-1/XXHS1 AND A.4.3.2-1/XXHS7 THEN R ELSE N/A | UEs supporting 5G Core and intra-band contiguous CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set and search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells. |
| CXXXHS27 | IF (A.4.1-4A/5 OR A.4.1-4A/6) AND A.4.3.2A.1-1/1 AND A.4.3.2-1/XXHS1 AND A.4.3.2-1/XXHS7 THEN R ELSE N/A | UEs supporting 5G Core and inter-band CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set and search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells. |
| CXXXHS29 | IF (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.2A.1-1/1 AND A.4.3.2-1/XXHS1 AND A.4.3.2-1/XXHS7 THEN R ELSE N/A | UEs supporting 5G Core and intra-band non-contiguous CA and DL NR CA with 2 carriers and DCI format 1_3 with same SCS between scheduling cell and cells in the set and search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells. |
| CXXXHS31 | IF (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.2A.1-2/1 AND A.4.3.2-1/XXHS2 AND A.4.3.2-1/XXHS8 THEN R ELSE N/A | UEs supporting 5GCore and intra-band contiguous CA and UL NR CA with 2 carriers and DCI format 0_3 with same SCS between scheduling cell and cells in the set and search space set configurations for DCI format 0_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells. |
| CXXXHS33 | IF (A.4.1-4A/5 OR A.4.1-4A/6) AND A.4.3.2A.1-2/1 AND A.4.3.2-1/XXHS2 AND A.4.3.2-1/XXHS8 THEN R ELSE N/A | UEs supporting 5GCore and inter-band CA and UL NR CA with 2 carriers and DCI format 0_3 with same SCS between scheduling cell and cells in the set and search space set configurations for DCI format 0_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells. |
| CXXXHS35 | IF (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.2A.1-2/1 AND A.4.3.2-1/XXHS2 AND A.4.3.2-1/XXHS8 THEN R ELSE N/A | UEs supporting 5GCore and intra-band non-contiguous CA and UL NR CA with 2 carriers and DCI format 0_3 with same SCS between scheduling cell and cells in the set and search space set configurations for DCI format 0_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells. |

<End of modified section 2>

<Start of modified section 3>

Table 4.1-2b: Additional Information of Applicability of Protocol conformance Layer 2 test cases, ref. TS 38.523-1 [2]

| Clause | Specific ICS | Specific IXIT | Number of TC Executions | Release other RAT |
| --- | --- | --- | --- | --- |
| 7 |  |  |  |  |
| 7.1 |  |  |  |  |
| 7.1.1 |  |  |  |  |
| 7.1.1.1 |  |  |  |  |
| 7.1.1.1.4 | pc_csi_RS_CFRA_ForHO, pc_maxNumberCSI_RS_BFD, pc_maxNumberCSI_RS_SSB_CBD |  |  |  |
| 7.1.1.2 |  |  |  |  |
| 7.1.1.2.8 |  |  |  |  |
| 7.1.1.2.8.1 | pc_dl_SchedulingOffset_PDSCH_TypeA | px_harqFeedbackType_DCI_1_3 |  |  |
| 7.1.1.2.8.2 | pc_dl_SchedulingOffset_PDSCH_TypeA | px_harqFeedbackType_DCI_1_3 |  |  |
| 7.1.1.2.8.3 | pc_dl_SchedulingOffset_PDSCH_TypeA | px_harqFeedbackType_DCI_1_3 |  |  |
| 7.1.1.2.9 |  |  |  |  |
| 7.1.1.2.9.1 | pc_dl_SchedulingOffset_PDSCH_TypeA | px_harqFeedbackType_DCI_1_3 |  |  |
| 7.1.1.2.9.2 | pc_dl_SchedulingOffset_PDSCH_TypeA | px_harqFeedbackType_DCI_1_3 |  |  |
| 7.1.1.2.9.3 | pc_dl_SchedulingOffset_PDSCH_TypeA | px_harqFeedbackType_DCI_1_3 |  |  |
| 7.1.1.3 |  |  |  |  |
| 7.1.1.3.2b | pc_configuredUL_GrantType1 |  |  |  |
| 7.1.1.4 |  |  |  |  |
| 7.1.1.4.1 |  |  |  |  |
| 7.1.1.4.1.3 | pc_dynamicSwitchRA_Type0_1_PDSCH |  |  |  |
| 7.1.1.4.1.4 | pc_dynamicSwitchRA_Type0_1_PDSCH |  |  |  |
| 7.1.1.4.2 |  |  |  |  |
| 7.1.1.4.2.3 | pc_dynamicSwitchRA_Type0_1_PUSCH |  |  |  |
| 7.1.1.4.2.4 | pc_dynamicSwitchRA_Type0_1_PUSCH |  |  |  |
| 7.1.1.6 |  |  |  |  |
| 7.1.1.6.4 | pc_um_WithShortSN |  |  |  |
| 7.1.1.7 |  |  |  |  |
| 7.1.1.7.1 |  |  |  |  |
| 7.1.1.7.1.1 | pc_UL_NR_CA_2CC or pc_EN_DC_NR_UL_2CC |  |  |  |
| 7.1.1.7.1.2 | pc_UL_NR_CA_2CC or pc_EN_DC_NR_UL_2CC |  |  |  |
| 7.1.1.7.1.3 | pc_UL_intra_non_contiguous_CA_NR_FR1_Class_(2A) or pc_UL_intra_non_contiguous_CA_NR_FR2_Class_(2A) |  |  |  |
| 7.1.1.7.2 |  |  |  |  |
| 7.1.1.7.2.1 |  | px_harqFeedbackType_DCI_1_3 px_coScheduledCellIndicationScheme_DCI_1_3 |  |  |
| 7.1.1.7.2.2 |  | px_harqFeedbackType_DCI_1_3 px_coScheduledCellIndicationScheme_DCI_1_3 |  |  |
| 7.1.1.7.2.3 |  | px_harqFeedbackType_DCI_1_3 px_coScheduledCellIndicationScheme_DCI_1_3 |  |  |
| 7.1.1.7.3 |  |  |  |  |
| 7.1.1.7.3.1 | pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18 | px_harqFeedbackType_DCI_1_3 px_coScheduledCellIndicationScheme_DCI_1_3 px_coScheduledCellIndicationScheme_DCI_0_3 |  |  |
| 7.1.1.7.3.2 | pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18 | px_harqFeedbackType_DCI_1_3 px_coScheduledCellIndicationScheme_DCI_1_3 px_coScheduledCellIndicationScheme_DCI_0_3 |  |  |
| 7.1.1.7.3.3 | pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18 | px_ harqFeedbackType_DCI_1_3 px_coScheduledCellIndicationScheme_DCI_1_3 px_coScheduledCellIndicationScheme_DCI_0_3 |  |  |
| 7.1.1.7.4 |  |  |  |  |
| 7.1.1.7.4.1 |  | px_ harqFeedbackType_DCI_1_3 px_coScheduledCellIndicationScheme_DCI_1_3 |  |  |
| 7.1.1.7.4.2 |  | px_ harqFeedbackType_DCI_1_3 px_coScheduledCellIndicationScheme_DCI_1_3 |  |  |
| 7.1.1.7.4.3 |  | px_ harqFeedbackType_DCI_1_3 px_coScheduledCellIndicationScheme_DCI_1_3 |  |  |
| 7.1.1.7.5 |  |  |  |  |
| 7.1.1.7.5.1 | pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18 | px_harqFeedbackType_DCI_1_3 px_coScheduledCellIndicationScheme_DCI_1_3 px_coScheduledCellIndicationScheme_DCI_0_3 |  |  |
| 7.1.1.7.5.2 | pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18 | px_harqFeedbackType_DCI_1_3 px_coScheduledCellIndicationScheme_DCI_1_3 px_coScheduledCellIndicationScheme_DCI_0_3 |  |  |
| 7.1.1.7.5.3 | pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18 | px_harqFeedbackType_DCI_1_3 px_coScheduledCellIndicationScheme_DCI_1_3 px_coScheduledCellIndicationScheme_DCI_0_3 |  |  |
| 7.1.1.12 |  |  |  |  |
| 7.1.1.12.3 |  |  |  | Rel-15 E-UTRA |
| 7.1.1.13 |  |  |  |  |
| 7.1.1.13.1 | pc_logicalChannelSR_DelayTimer |  |  |  |
| 7.1.1.13.2 | pc_logicalChannelSR_DelayTimer |  |  |  |
| 7.1.1.13.5 | pc_ra_SDT_r17 |  |  |  |
| 7.1.2 |  |  |  |  |
| 7.1.2.2 |  |  |  |  |
| 7.1.2.2.5 | pc_um_WithShortSN |  |  |  |
| 7.1.2.2.6 | pc_um_WithShortSN |  |  |  |
| 7.1.3 |  |  |  |  |
| 7.1.3.2.1 | pc_srb3 |  |  |  |

<End of modified section 3>