3GPP TSG-RAN5 Meeting #106	R5-250317

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3428 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interpretation of UL NAS IE presence and values |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 4.7 specifies the default content of NAS messages. Subclause 4.7.0.1 specifies special values for the UL NAS messages IEs. The following inconsistencies exist: - Two terms with the exact same meaning are used interchangeably and randomly throughout clause 4.7: - “If present: contents not checked”: 117 hits, - “Not checked”: 128 hits. For better readibility, only one terminology should be used consistently. - “Any allowed value” is used 9 times, whereas it should be “Present but contents not checked” according to subclause 4.7.0.1. - Subclause 4.7.0.1 terms "Present if <condition>: contents not checked“ and "Present if <condition>: <specific values>" are never used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - It is proposed to only use “Not checked”, as it is the most commonly used term, as it is a shorter piece of text, and as it is also used extensively for the default content of RRC messages. - “Any allowed value” has been replaced by “Present but contents not checked”. - The definition of those two terms has been deleted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.7	Default 5GC NAS message and information elements contents

### 4.7.0	General

#### 4.7.0.1	Interpretation of IE presence and values

For Uplink NAS messages, the following terms and their meanings shall be used to determine how to test specific IEs:

-	"Not present": test cases fail if IE is present.

-	"Present but contents not checked": test cases fail if IE is not present. No requirements regarding contents of the IE.

-	"Not checked":

-	Optional IE: IE may or may not be present. No requirements regarding contents of the IE.

-	Mandatory IE: IE shall be present. No requirements regarding contents of the IE.

-	"If present: <specific values>": IE may or may not be present. If present, its contents shall be as specified.

-	"<specific values>": test cases fail if IE is not present. Its contents shall be as specified.

#### 4.7.0.2	Security protected 5GS NAS messages

In subclause 4.7.1, all 5GS NAS messages are described in the plain 5GS NAS message format.

When a 5GS NAS message is security protected, the message shall be contained by SECURITY PROTECTED 5GS NAS MESSAGE unless contained by another NAS message.

The default contents of SECURITY PROTECTED 5GS NAS MESSAGE message is defined in table 4.7.1-28.

### 4.7.1	Contents of 5GMM messages

#### –	Authentication request

Table 4.7.1-1: AUTHENTICATION REQUEST

| Derivation Path: 24.501 clause 8.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Authentication request message identity |  | ‘0101 0110’B |  |  |
| ngKSI |  |  |  |  |
| NAS key set identifier |  | An arbitrarily selected value between '000'B and '110'B, different from the valid NAS key set identifier of the UE if such a value exists. |  |  |
| TSC |  | '0'B | native security context (for KSIAMF) |  |
| Spare half octet |  | '0000'B |  |  |
| ABBA |  | ‘0000 0000 0000 0000’B |  |  |
| Authentication parameter RAND (5G authentication challenge) |  | Not Present |  | EAP-AKA |
|  |  | An arbitrarily selected 128 bits value |  | 5G-AKA |
| Authentication parameter AUTN (5G authentication challenge) |  | Not Present |  | EAP-AKA |
|  |  | 128 bits value generated according to TS 24.501 [28] subclause 9.11.3.15 |  | 5G-AKA |
| EAP message |  | Not Present |  | 5G-AKA |
| EAP message |  | EAP-request/AKA'-challenge | See Table 4.7.3.2-01 | EAP-AKA |

| Condition | Explanation |
| --- | --- |
| EAP_AKA | EAP based primary authentication and key agreement procedure |
| 5G-AKA | 5G AKA based primary authentication and key agreement procedure |

NOTE:	Within a test execution this message is sent without integrity protection before NAS security mode control procedure has been successfully completed; and sent integrity protected and ciphered within SECURITY PROTECTED 5GS NAS MESSAGE message after 5GS NAS security mode control procedure has been successfully completed. SS does not maintain information for 5GS NAS security mode control procedure after a TC is completed.

#### –	Authentication response

Table 4.7.1-2: AUTHENTICATION RESPONSE

| Derivation Path: 24.501 clause 8.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | 5GMM |  |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Authentication response message identity |  | ‘0101 0111’B |  |  |
| Authentication response parameter |  | 16 octets RES* value calculated according to TS 24.501 [28] subclause 9.11.3.17 |  | 5G-AKA |
|  |  | Not Present |  | EAP-AKA |
| EAP message |  | EAP-response/AKA'-challenge | See Table 4.7.3.2-02 | EAP-AKA |

| Condition | Explanation |
| --- | --- |
| EAP-AKA | EAP based primary authentication and key agreement procedure |
| 5G-AKA | 5G AKA based primary authentication and key agreement procedure |

NOTE:	When sent in response to an AUTHENTICATION REQUEST message which is not integrity protected and not ciphered, the AUTHENTICATION RESPONSE message is sent integrity protected when a valid security context exists and without integrity protection and ciphering otherwise.

#### –	Authentication result

Table 4.7.1-3: AUTHENTICATION RESULT

| Derivation Path: 24.501 clause 8.2.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | 5GMM |  |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Authentication result message identity |  | ‘0101 1010’B |  |  |
| ngKSI |  | The same value as the last AUTHENTICATION REQUEST message |  |  |
| Spare half octet |  | '0000'B |  |  |
| EAP message |  | EAP-Success | See Table 4.7.3.2-03 |  |
| ABBA |  | ‘0000 0000 0000 0000’B |  |  |
| AUN3 device security key |  | Not present |  |  |

NOTE:	The security protection of this message is the same as the previous AUTHENTICATION REQUEST message.

#### –	Authentication failure

Table 4.7.1-4: AUTHENTICATION FAILURE

| Derivation Path: 24.501 clause 8.2.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | 5GMM |  |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Authentication failure message identity |  | ‘0101 1001’B |  |  |
| 5GMM cause |  | Present but contents not checked |  |  |
| Authentication failure parameter |  | Not checked |  |  |

NOTE:	The security protection of this message is the same as the previous AUTHENTICATION REQUEST message.

#### –	Authentication reject

Table 4.7.1-5: AUTHENTICATION REJECT

| Derivation Path: 24.501 clause 8.2.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | 5GMM |  |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Authentication reject message identity |  | ‘0101 1000’B |  |  |
| EAP message |  | Not present |  |  |
| EAP message |  | EAP-Response/AKA-Authentication-Reject | See Table 4.7.3.2-04 | EAP-AKA |

| Condition | Explanation |
| --- | --- |
| EAP-AKA | EAP based primary authentication and key agreement procedure |

NOTE:	This message is sent without integrity protection.

#### –	Registration request

Table 4.7.1-6: REGISTRATION REQUEST

| Derivation Path: 24.501 clause 8.2.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Registration request message identity |  | ‘0100 0001’B |  |  |
| 5GS registration type |  |  |  |  |
| 5GS registration type value |  | ‘001’B | Initial registration |  |
|  |  | ‘010’B |  | MOBILITY |
|  |  | ‘011’B |  | PERIODIC |
|  |  | ‘100’B |  | EMERGENCY |
|  |  | '101'B |  | SNPN_ONBOARDING |
| FOR |  | Present but contents not checked |  |  |
|  |  | '1'B | Follow-on request pending | EMERGENCY |
| ngKSI |  | Present but contents not checked |  |  |
| 5GS mobile identity |  | Present but contents not checked |  |  |
| Non-current native NAS key set identifier |  | Not checked |  | NON_CLEARTEXT_IE |
| 5GMM capability |  | Not checked |  | NON_CLEARTEXT_IE |
| UE security capability |  | Not checked |  |  |
| Requested NSSAI |  | Not checked |  | NON_CLEARTEXT_IE |
| Last visited registered TAI |  | Not checked |  | NON_CLEARTEXT_IE |
| S1 UE network capability |  | Not checked |  | NON_CLEARTEXT_IE |
| Uplink data status |  | Not checked |  | NON_CLEARTEXT_IE |
| PDU session status |  | Not checked |  | NON_CLEARTEXT_IE |
| MICO indication |  | Not checked |  | NON_CLEARTEXT_IE |
| UE status |  | Not checked |  |  |
| Additional GUTI |  | Not checked |  |  |
| Allowed PDU session status |  | Not checked |  | NON_CLEARTEXT_IE |
| UE's usage setting |  | Not checked |  | NOT pc_IMS AND NON_CLEARTEXT_IE |
|  |  | Present but contents not checked |  | NON_CLEARTEXT_IE |
| Requested DRX parameters |  | Not checked |  | NON_CLEARTEXT_IE |
| EPS NAS message container |  | Not checked |  |  |
| LADN indication |  | Not checked |  | NON_CLEARTEXT_IE |
| Payload container type |  | If present: ‘0101’B | UE policy container | NON_CLEARTEXT_IE |
| Payload container |  | Not checked |  | NON_CLEARTEXT_IE |
| Network slicing indication |  | Not checked |  | NON_CLEARTEXT_IE |
| 5GS update type |  | Not checked |  | NON_CLEARTEXT_IE |
| Mobile station classmark 2 |  | Not checked |  | NON_CLEARTEXT_IE |
| Supported codecs |  | Not checked |  | NON_CLEARTEXT_IE |
| NAS message container |  | The complete, ciphered, REGISTRATION REQUEST message including all IEs. |  | CIPHERED_MESSAGE |
| EPS bearer context status |  | Not checked |  | NON_CLEARTEXT_IE |
| Requested extended DRX parameters |  | Not checked |  | NON_CLEARTEXT_IE |
| T3324 value |  | Not checked |  | NON_CLEARTEXT_IE |
| UE radio capability ID |  | Not checked |  | NON_CLEARTEXT_IE |
| Requested mapped NSSAI |  | Not checked |  | NON_CLEARTEXT_IE |
| Additional information requested |  | Not checked |  | NON_CLEARTEXT_IE |
| Requested WUS assistance information |  | Not checked |  | NON_CLEARTEXT_IE |
| N5GC indication |  | Not checked |  | NON_CLEARTEXT_IE |
| Requested NB-N1 mode DRX parameters |  | Not checked |  | NON_CLEARTEXT_IE |
| UE request type |  | Not checked |  | NON_CLEARTEXT_IE |
| Paging restriction |  | Not checked |  | NON_CLEARTEXT_IE |
| Service-level-AA container |  | Not checked |  | NON_CLEARTEXT_IE |
| NID |  | Not checked |  | NON_CLEARTEXT_IE |
| MS determined PLMN with disaster condition |  | Not checked |  | NON_CLEARTEXT_IE |
| Requested PEIPS assistance information |  | Not checked |  | NON_CLEARTEXT_IE |
| Requested T3512 value |  | Not checked |  | NON_CLEARTEXT_IE |
| Unavailability information |  | Not checked |  | NON_CLEARTEXT_IE |
| Non-3GPP path switching information |  | Not checked |  | NON_CLEARTEXT_IE |
| AUN3 indication |  | Not checked |  | NON_CLEARTEXT_IE |

| Condition | Explanation |
| --- | --- |
| INITIAL | Initial registration |
| MOBILITY | Mobility registration updating |
| PERIODIC | Periodic registration updating |
| EMERGENCY | Emergency registration |
| NON_CLEARTEXT_IE | An information element that is not allowed to be sent in cleartext and shall only be included in the complete REGISTRATION REQUEST message in the NAS message container IE. |
| CIPHERED_MESSAGE | If any of the IEs marked with the condition NON_CLEARTEXT_IE is present, and the UE has a valid 5G NAS security context, this condition applies. |
| SNPN_ONBOARDING | SNPN onboarding registration |

NOTE:	This message is sent integrity protected when a valid security context exists otherwise sent without integrity protection, including only cleartext IEs.

#### –	Registration accept

Table 4.7.1-7: REGISTRATION ACCEPT

| Derivation Path: 24.501 clause 8.2.7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |  |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |  |
| Spare half octet |  | '0000'B |  |  |  |
| Registration accept message identity |  | ‘0100 0010’B |  |  |  |
| 5GS registration result |  |  |  |  |  |
| 5GS registration result value |  | ‘001’B | 3GPP access |  |  |
| SMS allowed |  | ‘0’B | SMS over NAS not allowed |  |  |
| 5G-GUTI |  | See Table 4.4.2-3 | For 5GC NAS test cases see Table 6.3.2.2-1 |  |  |
| Equivalent PLMNs |  | Not Present |  |  |  |
| TAI list |  |  |  |  |  |
| Length of tracking area identity list contents |  | '0000 0111'B | 7 octets |  |  |
| Partial tracking area identity list 1 |  |  |  |  |  |
| Number of elements |  | '0 0000'B | 1 element |  |  |
| Type of list |  | '00'B | list of TACs belonging to one PLMN, with non-consecutive TAC values |  |  |
| MCC |  | See Table 4.4.2-3 | For 5GC NAS test cases see Table 6.3.2.2-1 |  |  |
| MNC |  | See Table 4.4.2-3 | For 5GC NAS test cases see Table 6.3.2.2-1 |  |  |
| TAC 1 |  | See Table 4.4.2-3 | For 5GC NAS test cases see Table 6.3.2.2-1 |  |  |
| Allowed NSSAI |  |  |  |  |  |
| Length of NSSAI contents |  | 4 entries | Equal to the number of S-NSSAI values included |  |  |
| S-NSSAI |  |  |  |  |  |
| Length of S-NSSAI contents |  | ‘0000 0001’B | SST |  |  |
| SST |  | ‘0000 0001’B | SST value 1 (eMBB) |  |  |
| S-NSSAI |  |  |  |  |  |
| Length of S-NSSAI contents |  | ‘0000 0001’B | SST |  |  |
| SST |  | ‘0000 0010’B | SST value 2 (URLLC) |  |  |
| S-NSSAI |  |  |  |  |  |
| Length of S-NSSAI contents |  | ‘0000 0001’B | SST |  |  |
| SST |  | ‘0000 0011’B | SST value 3 (MIoT) |  |  |
| S-NSSAI |  |  |  |  |  |
| Length of S-NSSAI contents |  | ‘0000 0001’B | SST |  |  |
| SST |  | ‘0000 0100’B | SST value 4 (V2X) |  |  |
| Rejected NSSAI |  | Not Present |  |  |  |
| Configured NSSAI |  | Not Present |  |  |  |
| 5GS network feature support |  | ‘0000 1101 0000 0000 0000 0000’B | IMS voice over PS session supported over 3GPP access, Emergency services supported in NR connected to 5GCN and E-UTRA connected to 5GCN. All other features set to "not supported" including the 'Interworking without N26 interface not supported'. |  |  |
| 5GS network feature support |  | Not Present |  | SST_URLLC OR SST_V2X OR SST_MIOT |  |
| PDU session status |  | The same value as the PDU session status IE of the most recently received REGISTRATION REQUEST message |  |  |  |
| PDU session reactivation result |  | Not Present |  |  |  |
| PDU session reactivation result error cause |  | Not Present |  |  |  |
| LADN information |  | Not Present |  |  |  |
| MICO indication |  | Not Present |  |  |  |
| Network slicing indication |  | Not Present |  |  |  |
| Service area list |  | Not Present |  |  |  |
| T3512 value |  |  |  | INITIAL |  |
| Timer value |  | '0 0000'B |  |  |  |
| Unit |  | '111'B | value indicates that the timer is deactivated |  |  |
| T3512 value |  | Not Present |  |  |  |
| Non-3GPP de-registration timer value |  | Not Present |  |  |  |
| T3502 value |  | Not Present |  |  |  |
| Emergency number list |  | Not Present |  |  |  |
| Extended emergency number list |  | Not Present |  |  |  |
| SOR Transparent container |  | Not Present |  |  |  |
| EAP message |  | Not Present |  |  |  |
| NSSAI inclusion mode |  | Not Present |  |  |  |
| Operator-defined access category definitions |  | Not Present |  |  |  |
| Negotiated DRX parameters |  | Not Present |  |  |  |
| Non-3GPP NW policies |  | Not Present |  |  |  |
| EPS bearer context status |  | The same value as the EPS bearer context status IE of the most recently received REGISTRATION REQUEST message |  |  |  |
| Negotiated extended DRX parameters |  | Not Present |  |  |  |
| T3447 value |  | Not Present |  |  |  |
| T3448 value |  | Not Present |  |  |  |
| T3324 value |  | Not Present |  |  |  |
| UE radio capability ID |  | Not Present |  |  |  |
| UE radio capability ID |  | The same value as received in UE radio capability ID; if any of the REGISTRATION REQUEST message |  | pc_5GC_RACS |  |
| UE radio capability ID deletion indication |  | Not Present |  |  |  |
| Pending NSSAI |  | Not Present |  |  |  |
| Ciphering key data |  | Not Present |  |  |  |
| CAG information list |  | Not Present |  |  |  |
| Truncated 5G-S-TMSI configuration |  | Not Present |  |  |  |
| Negotiated WUS assistance information |  | Not Present |  |  |  |
| Negotiated NB-N1 mode DRX parameters |  | Not Present |  |  |  |
| Extended rejected NSSAI |  | Not Present |  |  |  |
| Service-level-AA container |  | Not Present |  |  |  |
| Negotiated PEIPS assistance information |  | Not Present |  |  |
| Negotiated PEIPS assistance information |  |  |  | pc_PEIPS_assistance_information |  |
| Length of tracking area identity list contents |  | '0000 0001'B | 1 octet |  |
| PEIPS assistance information type 1 |  |  |  |  |
| Type of information |  | '000'B | Paging subgroup ID |  |
| Paging subgroup ID value |  | '00000'B |  |  |
| 5GS additional request result |  | Not Present |  |  |  |
| NSSRG information |  | Not Present |  |  |  |
| Disaster roaming wait range |  | Not Present |  |  |  |
| Disaster return wait range |  | Not Present |  |  |  |
| List of PLMNs to be used in disaster condition |  | Not Present |  |  |  |
| Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming" |  | Not Present |  |  |  |
| Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service" |  | Not Present |  |  |  |
| Extended CAG information list |  | Not Present |  |  |  |
| NSAG information |  | Not Present |  |  |  |
| Equivalent SNPNs |  | Not Present |  |  |  |
| NID |  | Not Present |  |  |  |
| Registration accept type 6 IE container |  | Not Present |  |  |  |
| RAN timing synchronization |  | Not Present |  |  |  |
| Alternative NSSAI |  | Not Present |  |  |  |
| Maximum time offset |  | Not Present |  |  |  |
| S-NSSAI time validity information |  | Not Present |  |  |  |
| Unavailability configuration |  | Not Present |  |  |  |
| Feature authorization indication |  | Not Present |  |  |  |
| On-demand NSSAI |  | Not Present |  |  |  |

| Condition | Explanation |
| --- | --- |
| INITIAL | Initial registration |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Registration complete

Table 4.7.1-8: REGISTRATION COMPLETE

| Derivation Path: 24.501 clause 8.2.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Registration complete message identity |  | ‘0100 0011’B |  |  |
| SOR transparent container |  | Not checked |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Registration reject

Table 4.7.1-9: REGISTRATION REJECT

| Derivation Path: 24.501 clause 8.2.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Registration reject message identity |  | ‘0100 0100’B |  |  |
| 5GMM cause |  | Set according to specific message content |  |  |
| T3346 value |  | Not Present |  |  |
| T3502 value |  | Not Present |  |  |
| EAP message |  | Not Present |  |  |
| Rejected NSSAI |  | Not Present |  |  |
| CAG information list |  | Not Present |  |  |
| Extended rejected NSSAI |  | Not Present |  |  |
| Disaster return wait range |  | Not Present |  |  |
| Extended CAG information list |  | Not Present |  |  |
| Lower bound timer value |  | Not Present |  |  |
| Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming" |  | Not Present |  |  |
| Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service" |  | Not Present |  |  |
| N3IWF identifier |  | Not Present |  |  |
| TNAN information |  | Not Present |  |  |

NOTE:	This message is sent without integrity protection before the secure exchange of NAS messages has been established and sent within SECURITY PROTECTED 5GS NAS MESSAGE message after the secure exchange of NAS messages has been established.

#### –	UL NAS transport

Table 4.7.1-10: UL NAS TRANSPORT

| Derivation Path: 24.501 clause 8.2.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| UL NAS TRANSPORT message identity |  | ‘0110 0111’B |  |  |
| Payload container type |  | Set according to specific message content |  |  |
| Payload container type |  | ‘0001’B | N1 SM information | INITIAL_PDU_REQUEST |
| Payload container type |  | ‘0100’B | SOR transparent container | 5GS_SOR_CMCI |
| Spare half octet |  | '0000'B |  |  |
| Payload container |  | Set according to specific message content |  |  |
| Payload container |  | Present | The SOR transparent container in the payload container IE carries steering of roaming information. | 5GS_SOR_CMCI |
| Length of payload container contents |  |  | 2 octet |  |
| Payload container contents |  |  |  |  |
| SOR transparent container |  | Present | The SOR transparent container carries steering of roaming information. |  |
| Length of SOR transparent container contents |  |  | 2 octet |  |
| SOR header |  |  | 1 octet |  |
| SOR data type |  | 1 | The SOR transparent container carries acknowledgement of successful reception of the steering of roaming information. |  |
| ME support of SOR-CMCI indicator (MSSI) value |  | Not checked |  |  |
| ME support of SOR-CMCI indicator (MSSI) value |  | 1 | SOR-CMCI supported by the ME |  |
| ME support of SOR-SNPN-SI indicator (MSSNPNSI) value |  | Not checked |  |  |
| SOR-MAC-IAUSF |  | Set to match the calculated SoR-MAC-IAUS as the way defined in TS 33.501 A.17 |  |  |
| Payload container |  | PDU SESSION ESTABLISHMENT REQUEST message |  | INITIAL_PDU_REQUEST |
| PDU session ID |  | Not checked |  |  |
| PDU session ID |  | Same PDU session ID as defined in the PDU SESSION ESTABLISHMENT REQUEST message in the Payload container |  | INITIAL_PDU_REQUEST |
| Old PDU session ID |  | Not checked |  |  |
| Request type |  | Not checked |  |  |
| Request type |  | ‘001’B | initial request | INITIAL_PDU_REQUEST |
| S-NSSAI |  | Not checked |  |  |
| DNN |  | Not checked | (NOTE 1) |  |
| Additional information |  | Not checked |  |  |
| MA PDU session information |  | Not checked |  |  |
| Release assistance indication |  | Not checked |  |  |
| Non-3GPP access path switching indication |  | Not checked |  |  |
| Alternative S-NSSAI |  | Not checked |  |  |
| Payload container information |  | Not checked |  |  |
| NOTE 1: Although the contents of the IE is not required to be verified for PASS/FAIL purposes, the provided information shall be taken into account e.g. for the building the content of messages and allowing for specific UE behaviour as specified in Table 4.8.4-1. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| INITIAL_PDU_REQUEST | The UL NAS TRANSPORT message is used to transport a PDU SESSION ESTABLISHMENT REQUEST message to establish a new PDU session. |
| 5GS_SOR_CMCI | Used in Rel-17 SOR-CMCI test cases (TS 38.523-1 [12]) |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	DL NAS transport

Table 4.7.1-11: DL NAS TRANSPORT

| Derivation Path: 24.501 clause 8.2.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| DL NAS TRANSPORT message identity |  | ‘0110 1000’B |  |  |
| Payload container type |  | Set according to specific message content |  |  |
| Payload container type |  | ‘0001’B | N1 SM information | 5GSM_MESSAGE |
| Payload container type |  | ‘0100’B | SOR transparent container | 5GS_SOR_CMCI |
| Spare half octet |  | '0000'B |  |  |
| Payload container |  | Set according to specific message content |  |  |
| Payload container |  | 5GSM message |  | 5GSM_MESSAGE |
| Payload container |  | Present | The SOR transparent container in the payload container IE carries steering of roaming information. | 5GS_SOR_CMCI |
| Length of payload container contents |  |  | 2 octet |  |
| Payload container contents |  |  |  |  |
| SOR transparent container |  | Present | The SOR transparent container carries steering of roaming information. |  |
| Length of SOR transparent container contents |  |  | 2 octet |  |
| SOR header |  |  | 1 octet |  |
| SOR data type |  | 0 | The SOR transparent container carries steering of roaming information. |  |
| List indication value |  | 1 | List of preferred PLMN/access technology combinations is provided |  |
| List type |  | 1 | The list type is a "PLMN ID and access technology list" |  |
| Acknowledgement (ACK) value |  | 1 | Acknowledgement requested |  |
| Additional parameters (AP) value |  | 0 | Additional parameters not included |  |
| Additional parameters (AP) value |  | 1 | Additional parameters included |  |
| SOR-MAC-IAUSF |  | Set to match the calculated SoR-MAC-IAUS as the way defined in TS 33.501 A.17 |  |  |
| CounterSOR |  | Value generated as per TS 33.501 Cl 6.14.2.3 |  |  |
| Length of PLMN ID and access technology list |  |  | 1 octet |  |
| PLMN ID and access technology list |  |  | Values determined by test implementation |  |
| SOR-CMCI indicator (SI) value |  | 0 | SOR-CMCI absent |  |
| SOR-CMCI indicator (SI) value |  | 1 | SOR-CMCI present |  |
| Store SOR-CMCI in ME indicator (SSCMI) value |  |  | Value determined by test implementation |  |
| SOR-SNPN-SI indicator (SSSI) value |  | 0 | subscribed SNPN or HPLMN indication that 'no change of the SOR-SNPN-SI stored in the UE is needed and thus no SOR-SNPN-SI is provided' |  |
| SOR-CMCI |  | Present |  |  |
| Length of SOR-CMCI contents |  |  | Length value determined by test implementation |  |
| SOR-CMCI rule 1 |  |  | Value determined by test implementation |  |
| Length of SOR-CMCI rule contents |  |  | Length value determined by test implementation |  |
| Tsor-cm timer value |  | 00100001 | 60s |  |
| Criterion type |  |  | Value determined by test implementation |  |
| Criterion value |  |  | Value determined by test implementation |  |
| SOR-SNPN-SI |  | Not Present |  |  |
| PDU session ID |  | Not Present |  |  |
| PDU session ID |  | Set to the same value as the PDU session ID of the 5GSM message in the Payload container. |  | 5GSM_MESSAGE |
| Additional information |  | Not Present |  |  |
| 5GMM cause |  | Not Present |  |  |
| Back-off timer value |  | Not Present |  |  |
| Lower bound timer value |  | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| 5GSM_MESSAGE | The DL NAS TRANSPORT message is used to transport a 5GSM message |
| 5GS_SOR_CMCI | Used in Rel-17 SOR-CMCI test cases (TS 38.523-1 [12]) |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	De-registration request (UE originating de-registration)

Table 4.7.1-12: DEREGISTRATION REQUEST_1

| Derivation Path: 24.501 clause 8.2.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| De-registration request message identity |  | ‘0100 0101’B |  |  |
| De-registration type |  |  |  |  |
| Switch off |  | ‘0’B |  | NORMAL |
|  |  | ‘1’B |  | SWITCH_OFF |
| Re-registration required |  | ‘0’B |  |  |
| Access type |  | ‘01’B | 3GPP access |  |
| ngKSI |  | Present but contents not checked |  |  |
| 5GS mobile identity |  | Present but contents not checked |  |  |
| Unavailability information |  | Not checked |  | NON_CLEARTEXT_IE |
| NAS message container |  | The complete, ciphered, DEREGISTRATION REQUEST message including all IEs. |  | CIPHERED_MESSAGE |

| Condition | Explanation |
| --- | --- |
| NORMAL | Normal de-registration |
| SWITCH_OFF | Switch off |
| NON_CLEARTEXT_IE | An information element that is not allowed to be sent in cleartext. |
| CIPHERED_MESSAGE | If any of the IEs marked with the condition NON_CLEARTEXT_IE is present, this condition applies. NOTE: This condition is only applicable if the DEREGISTRATION REQUEST message is sent as an initial NAS message. |

NOTE:	If this message is sent as an initial NAS message, it is sent with integrity protection but without ciphering. Otherwise it is sent without integrity protection and ciphering before SS has started the ciphering and integrity and ciphered protected after SS has started the ciphering.

#### –	De-registration accept (UE originating de-registration)

Table 4.7.1-13: DEREGISTRATION ACCEPT_1

| Derivation Path: 24.501 clause 8.2.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| De-registration accept message identity |  | ‘0100 0110’B |  |  |

NOTE:	This message is sent using the same security protection as in the previous DETACH REQUEST message received from the UE.

#### –	De-registration request (UE terminated de-registration)

Table 4.7.1-14: DEREGISTRATION REQUEST_2

| Derivation Path: 24.501 clause 8.2.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| De-registration request message identity |  | ‘0100 0111’B |  |  |
| De-registration type |  | Set according to specific message content |  |  |
| Spare half octet |  | '0000'B |  |  |
| 5GMM cause |  | Not Present |  |  |
| T3346 value |  | Not Present |  |  |
| Rejected NSSAI |  | Not Present |  |  |
| CAG information list |  | Not Present |  |  |
| Extended rejected NSSAI |  | Not Present |  |  |
| Disaster return wait range |  | Not Present |  |  |
| Extended CAG information list |  | Not Present |  |  |
| Lower bound timer value |  | Not Present |  |  |
| Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming" |  | Not Present |  |  |
| Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service" |  | Not Present |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	De-registration accept (UE terminated de-registration)

Table 4.7.1-15: DEREGISTRATION ACCEPT_2

| Derivation Path: 24.501 clause 8.2.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| De-registration accept message identity |  | ‘0100 1000’B |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Service request

Table 4.7.1-16: SERVICE REQUEST

| Derivation Path: 24.501 clause 8.2.16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Service request message identity |  | ‘0100 1100’B |  |  |
| ngKSI |  |  |  |  |
| NAS key set identifier |  | The valid NAS key set identifier of the UE |  |  |
| TSC |  | '0'B | native security context (for KSIAMF) |  |
| Service type |  | ‘0010’B | mobile terminated services |  |
| 5G-S-TMSI |  | The valid 5G-S-TMSI of the UE |  |  |
| Uplink data status |  | Not checked |  | NON_CLEARTEXT_IE |
| PDU session status |  | Not checked |  | NON_CLEARTEXT_IE |
| Allowed PDU session status |  | Not checked |  | NON_CLEARTEXT_IE |
| NAS message container |  | If present, the complete, ciphered, SERVICE REQUEST message including all IEs. |  | CIPHERED_MESSAGE |
| UE request type |  | Not checked |  | NON_CLEARTEXT_IE |
| Paging restriction |  | Not checked |  | NON_CLEARTEXT_IE |

| Condition | Explanation |
| --- | --- |
| NON_CLEARTEXT_IE | An information element that is not allowed to be sent in cleartext and shall only be included in the complete SERIVICE REQUEST message in the NAS message container IE. NOTE: This condition is only applicable if the SERVICE REQUEST message is sent as an initial NAS message. |
| CIPHERED_MESSAGE | If any of the IEs marked with the condition NON_CLEARTEXT_IE is present, this condition applies. NOTE: This condition is only applicable if the SERVICE REQUEST message is sent as an initial NAS message. |

NOTE:	This message is sent without integrity protection, including only cleartext IEs, before NAS security mode control procedure has been successfully completed and sent within SECURITY PROTECTED 5GS NAS MESSAGE message after NAS security mode control procedure has been successfully completed

#### –	Service accept

Table 4.7.1-17: SERVICE ACCEPT

| Derivation Path: 24.501 clause 8.2.17 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Service accept message identity |  | ‘0100 1110’B |  |  |
| PDU session status |  | The same value as the PDU session status IE of the most recently received SERVICE REQUEST message |  |  |
| PDU session reactivation result |  | Not Present |  |  |
| PDU session reactivation result error cause |  | Not Present |  |  |
| EAP message |  | Not Present |  |  |
| T3448 value |  | Not Present |  |  |
| 5GS additional request result |  | Not Present |  |  |
| Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming" |  | Not Present |  |  |
| Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service" |  | Not Present |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Service reject

Table 4.7.1-18: SERVICE REJECT

| Derivation Path: 24.501 clause 8.2.18 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Service reject message identity |  | ’0100 1101’B |  |  |
| 5GMM cause |  | The value is set according to specific message content |  |  |
| PDU session status |  | Not Present |  |  |
| T3346 value |  | Not Present |  |  |
| EAP message |  | Not Present |  |  |
| T3448 value |  | Not Present |  |  |
| CAG information list |  | Not Present |  |  |
| Disaster return wait range |  | Not Present |  |  |
| Extended CAG information list |  | Not Present |  |  |
| Lower bound timer value |  | Not Present |  |  |
| Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming" |  | Not Present |  |  |
| Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service" |  | Not Present |  |  |

NOTE:	This message is sent without integrity protection before NAS security mode control procedure has been successfully completed and sent within SECURITY PROTECTED 5GS NAS MESSAGE message after NAS security mode control procedure has been successfully completed

#### –	Configuration update command

Table 4.7.1-19: CONFIGURATION UPDATE COMMAND

| Derivation Path: 24.501 clause 8.2.19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Configuration update command message identity |  | ‘0101 0100’B |  |  |
| Configuration update indication |  | Not Present |  |  |
| 5G-GUTI |  | Not Present |  |  |
| TAI list |  | Not Present |  |  |
| Allowed NSSAI |  | Not Present |  |  |
| Service area list |  | Not Present |  |  |
| Full name for network |  | Not Present |  |  |
| Short name for network |  | Not Present |  |  |
| Local time zone |  | Not Present |  |  |
| Universal time and local time zone |  | Not Present |  |  |
| Network daylight saving time |  | Not Present |  |  |
| LADN information |  | Not Present |  |  |
| MICO indication |  | Not Present |  |  |
| Network slicing indication |  | Not Present |  |  |
| Configured NSSAI |  | Not Present |  |  |
| Rejected NSSAI |  | Not Present |  |  |
| Operator-defined access category definitions |  | Not Present |  |  |
| SMS indication |  | Not Present |  |  |
| T3447 value |  | Not Present |  |  |
| CAG information list |  | Not Present |  |  |
| UE radio capability ID |  | Not Present |  |  |
| UE radio capability ID deletion indication |  | Not Present |  |  |
| 5GS registration result |  | Not Present |  |  |
| Truncated 5G-S-TMSI configuration |  | Not Present |  |  |
| Additional configuration indication |  | Not Present |  |  |
| Extended rejected NSSAI |  | Not Present |  |  |
| Service-level-AA container |  | Not Present |  |  |
| NSSRG information |  | Not Present |  |  |
| Disaster roaming wait range |  | Not Present |  |  |
| Disaster return wait range |  | Not Present |  |  |
| List of PLMNs to be used in disaster condition |  | Not Present |  |  |
| Extended CAG information list |  | Not Present |  |  |
| Updated PEIPS assistance information |  | Not Present |  |  |
| NSAG information |  | Not Present |  |  |
| Priority indicator |  | Not Present |  |  |
| RAN timing synchronization |  | Not Present |  |  |
| Extended LADN information |  | Not Present |  |  |
| Alternative NSSAI |  | Not Present |  |  |
| S-NSSAI location validity information |  | Not Present |  |  |
| S-NSSAI time validity information |  | Not Present |  |  |
| Maximum time offset |  | Not Present |  |  |
| Partially allowed NSSAI |  | Not Present |  |  |
| Partially rejected NSSAI |  | Not Present |  |  |
| Feature authorization indication |  | Not Present |  |  |
| On-demand NSSAI |  | Not Present |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Configuration update complete

Table 4.7.1-20: CONFIGURATION UPDATE COMPLETE

| Derivation Path: 24.501 clause 8.2.20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Configuration update complete message identity |  | ‘0101 0101’B |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Identity request

Table 4.7.1-21: IDENTITY REQUEST

| Derivation Path: 24.501 clause 8.2.21 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Identity request message identity |  | ‘0101 1011’B |  |  |
| Identity type |  | Set according to specific message contents |  |  |
| Spare half octet |  | '0000'B |  |  |

NOTE:	This message is sent without integrity protection before 5GS NAS security mode control procedure has been successfully completed and sent within SECURITY PROTECTED 5GS NAS MESSAGE message after 5GS NAS security mode control procedure has been successfully completed.

#### –	Identity response

Table 4.7.1-22: IDENTITY RESPONSE

| Derivation Path: 24.501 clause 8.2.22 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Identity response message identity |  | 0101 1100’B |  |  |
| Mobile identity |  | Present but contents not checked |  |  |

NOTE:	This message is sent without integrity protection before 5GS NAS security mode control procedure has been successfully completed and sent within SECURITY PROTECTED 5GS NAS MESSAGE message after 5GS NAS security mode control procedure has been successfully completed.

#### –	Notification

Table 4.7.1-23: NOTIFICATION

| Derivation Path: 24.501 clause 8.2.23 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Notification message identity |  | ‘0110 0101’B |  |  |
| Access type |  | ‘01’B | 3GPP access |  |
| Spare half octet |  | '0000'B |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Notification response

Table 4.7.1-24: NOTIFICATION RESPONSE

| Derivation Path: 24.501 clause 8.2.24 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Notification response message identity |  | ‘0110 0110’B |  |  |
| PDU session status |  | Not checked |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Security mode command

Table 4.7.1-25: SECURITY MODE COMMAND

| Derivation Path: 24.501 clause 8.2.25 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Security mode command message identity |  | ‘0101 1101’B |  |  |
| Selected NAS security algorithms |  |  |  |  |
| Type of ciphering algorithm |  | Set according to PIXIT px_NAS_5GC_CipheringAlgorithm for default ciphering algorithm |  |  |
| Type of ciphering algorithm |  | ‘0000’B | 5G encryption algorithm 5G EA0 (null ciphering algorithm) | For RF |
| Type of integrity protection algorithm |  | Set according to PIXIT px_NAS_5GC_IntegrityProtAlgorithm for default integrity protection algorithm | This value should not be equal to the null integrity algorithm. |  |
| ngKSI |  |  |  |  |
| NAS key set identifier |  | The valid NAS key set identifier |  |  |
| TSC |  | '0'B | native security context (for KSIAMF) |  |
| Spare half octet |  | '0000'B |  |  |
| Replayed UE security capabilities |  | Set according to the received UE security capabilities |  |  |
| IMEISV request |  | Not Present |  |  |
| Selected EPS NAS security algorithms |  | Not Present |  |  |
| Selected EPS NAS security algorithms |  |  |  | UE_S1_SUPPORTED |
| Type of ciphering algorithm |  | Set according to PIXIT px_NAS_CipheringAlgorithm for default ciphering algorithm | The px_NAS_CipheringAlgorithm PIXIT is defined in TS 36.523-3 [x] |  |
| Type of integrity protection algorithm |  | Set according to PIXIT px_NAS_IntegrityProtAlgorithmfor default integrity protection algorithm | The px_NAS_IntegrityProtAlgorithm is defined in TS 36.523-3 [x] |  |
| Additional 5G security information |  | Not Present |  |  |
| Additional 5G security information |  |  |  | NO_VALID_SS_SECURITY_CONTEXT |
| RINMR |  | ‘1’B | Retransmission of the initial NAS message requested |  |
| HDP |  | ‘0’B | KAMF derivation is not required |  |
| EAP message |  | Not Present |  |  |
| EAP message |  | EAP-Success | See Table 4.7.3.2-03 | EAP-AKA |
| ABBA |  | ‘0000 0000 0000 0000’B |  | EAP-AKA |
| Replayed S1 UE security capabilities |  | Not Present |  |  |
| Replayed S1 UE security capabilities |  | Set according to the received UE security capabilities in the last REGISTRATION REQUEST message |  | UE_S1_SUPPORTED |
| AUN3 device security key |  | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| NO_VALID_SS_SECURITY_CONTEXT | If the SS doesn’t have a valid security context |
| EAP_AKA | EAP based primary authentication and key agreement procedure |
| For RF | Used for RF/RRM test cases |
| UE_S1_SUPPORTED | The UE indicated support of S1 in the last REGISTRATION REQUEST message |

NOTE:	This message is always sent integrity protected with new 5GS NAS security context.

#### –	Security mode complete

Table 4.7.1-26: SECURITY MODE COMPLETE

| Derivation Path: 24.501 clause 8.2.26 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Security mode complete message identity |  | ‘0101 1110’B |  |  |
| IMEISV |  | Not present |  |  |
| NAS message container |  | Not present |  |  |
|  |  | Complete initial NAS message |  | RINMR_INDICATED |
| non-IMEISV PEI |  | Not present |  |  |

| Condition | Explanation |
| --- | --- |
| RINMR_INDICATED | The SS requested retransmission of the initial NAS message in the last SECURITY MODE COMMAND |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message with new 5GS NAS security context.

#### –	Security mode reject

Table 4.7.1-27: SECURITY MODE REJECT

| Derivation Path: 24.501 clause 8.2.27 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Security mode reject message identity |  | ‘0101 1111’B |  |  |
| 5GMM cause |  | The value is set according to specific message content |  |  |

NOTE:	This message is sent without integrity protection before 5GS NAS security mode control procedure has been successfully completed and sent within SECURITY PROTECTED 5GS NAS MESSAGE message after 5GS NAS security mode control procedure has been successfully completed.

#### –	Security protected 5GS NAS message

Table 4.7.1-28: SECURITY PROTECTED 5GS NAS MESSAGE

| Derivation Path: 24.501 clause 8.2.28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | 5GMM |  |  |
| Security header type |  | '0001'B | Integrity protected | UNCIPHERED |
|  |  | '0010'B | Integrity protected and ciphered | CIPHERED |
|  |  | '0011'B | Integrity protected with new 5G NAS security context | UNCIPHERED-NEW |
|  |  | '0100'B | Integrity protected and ciphered with new 5G NAS security context | CIPHERED-NEW |
| Spare half octet |  | '0000'B |  |  |
| Message authentication code |  | The calculated value of MAC-I for this message. | The value of MAC-I is calculated by SS using Sequence number sent by UE. | SENT-BY-SS |
|  |  | The same value as the XMAC-I value calculated by SS. |  | SENT-BY-UE |
| Sequence number |  | The internal counter of the SS |  | SENT-BY-SS |
|  |  | Present but contents not checked |  | SENT-BY-UE |
| Plain 5GS NAS message |  | Set according to specific message content |  |  |

| Condition | Explanation |
| --- | --- |
| UNCIPHERED | This condition applies to unciphered NAS message exchange |
| CIPHERED | This condition applies to ciphered NAS message exchange |
| UNCIPHERED-NEW | This condition applies to unciphered NAS message exchange with new 5G NAS security context |
| CIPHERED-NEW | This condition applies to ciphered NAS message exchange with new 5G NAS security context |
| SENT-BY-SS | Use for the message sent from SS to UE |
| SENT-BY-UE | Use for the message sent from UE to SS |

#### –	5GMM status

Table 4.7.1-29: 5GMM STATUS

| Derivation Path: 24.501 clause 8.2.29 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | 5GMM |  |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| 5GMM STATUS message identity |  | ‘0110 0100’B |  |  |
| 5GMM cause |  | ‘0110 1111’B | Protocol error, unspecified | SENT-BY-SS |
|  |  | Present but contents not checked |  | SENT-BY-UE |

| Condition | Explanation |
| --- | --- |
| SENT-BY-SS | Use for the message sent from SS to UE |
| SENT-BY-UE | Use for the message sent from UE to SS |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Control plane service request

Table 4.7.1-30: CONTROL PLANE SERVICE REQUEST

| Derivation Path: 24.501 clause 8.2.30 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Control plane service request message identity |  | ‘0100 1111’B |  |  |
| Control plane service type |  | ‘001’B | mobile terminated request |  |
| ngKSI |  |  |  |  |
| NAS key set identifier |  | The valid NAS key set identifier of the UE |  |  |
| TSC |  | '0'B | native security context (for KSIAMF) |  |
| CIoT small data container |  | Not checked |  | NON_CLEARTEXT_IE |
| Payload container type |  | Not checked |  | NON_CLEARTEXT_IE |
| Payload container |  | Not checked |  | NON_CLEARTEXT_IE |
| PDU session ID |  | Not checked |  | NON_CLEARTEXT_IE |
| PDU session status |  | Not checked |  | NON_CLEARTEXT_IE |
| Release assistance indication |  | Not checked |  | NON_CLEARTEXT_IE |
| Uplink data status |  | Not checked |  | NON_CLEARTEXT_IE |
| NAS message container |  | Not checked |  | CIPHERED_MESSAGE |
| Additional information |  | Not checked |  | NON_CLEARTEXT_IE |
| Allowed PDU session status |  | Not checked |  | NON_CLEARTEXT_IE |
| UE request type |  | Not checked |  | NON_CLEARTEXT_IE |
| Paging restriction |  | Not checked |  | NON_CLEARTEXT_IE |

| Condition | Explanation |
| --- | --- |
| NON_CLEARTEXT_IE | An information element that is not allowed to be sent in cleartext and shall only be included in the complete CONTROL PLANE SERIVICE REQUEST message in the NAS message container IE. NOTE: This condition is only applicable if the CONTROL PLANE SERVICE REQUEST message is sent as an initial NAS message. |
| CIPHERED_MESSAGE | If any of the IEs marked with the condition NON_CLEARTEXT_IE is present, this condition applies. NOTE: This condition is only applicable if the CONTROL PLANE SERVICE REQUEST message is sent as an initial NAS message. |

NOTE:	This message is sent without integrity protection, including only cleartext IEs, before NAS security mode control procedure has been successfully completed and sent within SECURITY PROTECTED 5GS NAS MESSAGE message after NAS security mode control procedure has been successfully completed

#### –	Network slice-specific authentication command

Table 4.7.1-31: NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND

| Derivation Path: 24.501 clause 8.2.31 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND message identity |  | ‘0101 0000’B |  |  |
| S-NSSAI |  | Set according to specific message content |  |  |
| EAP message |  | Set according to Table 4.7.3.2-1 | See TS 24.501 [25] subclause 9.11.2.2 |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Network slice-specific authentication complete

Table 4.7.1-32: NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE

| Derivation Path: 24.501 clause 8.2.32 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE message identity |  | ‘0101 0001’B |  |  |
| S-NSSAI |  | Set according to specific message content |  |  |
| EAP message |  | Set according to Table 4.7.3.2-2 | See TS 24.501 [25] subclause 9.11.2.2 |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Network slice-specific authentication result

Table 4.7.1-33: NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT

| Derivation Path: 24.501 clause 8.2.33 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE message identity |  | ‘0101 0010’B |  |  |
| S-NSSAI |  | Set according to specific message content |  |  |
| EAP message |  | EAP-Success | See Table 4.7.3.2-3 | NSSAA Success |
|  |  | EAP-Failure | See Table 4.7.3.2-6 | NSSAA Failure |

| Condition | Explanation |
| --- | --- |
| NSSAA Success | Network slice-specific authentication succeeds |
| NSSAA Failure | Network slice-specific authentication fails |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Relay key request

Table 4.7.1-34: RELAY KEY REQUEST

| Derivation Path: 24.501 clause 8.2.34 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Relay key request message identity |  | ‘0110 1001’B |  |  |
| PRTI |  | FFS |  |  |
| Relay key request parameters |  | FFS |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Relay key accept

Table 4.7.1-35: RELAY KEY ACCEPT

| Derivation Path: 24.501 clause 8.2.35 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Relay key accept message identity |  | ‘0110 1010’B |  |  |
| PRTI |  | FFS |  |  |
| Relay key response parameters |  | FFS |  |  |
| EAP message |  | FFS |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Relay key reject

Table 4.7.1-36: RELAY KEY REJECT

| Derivation Path: 24.501 clause 8.2.36 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Relay key reject message identity |  | ‘0110 1011’B |  |  |
| PRTI |  | FFS |  |  |
| EAP message |  | FFS |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Relay authentication request

Table 4.7.1-37: RELAY AUTHENTICATION REQUEST

| Derivation Path: 24.501 clause 8.2.37 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Relay authentication request message identity |  | ‘0110 1100’B |  |  |
| PRTI |  | FFS |  |  |
| EAP message |  | FFS |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Relay authentication response

Table 4.7.1-38: RELAY AUTHENTICATION RESPONSE

| Derivation Path: 24.501 clause 8.2.37 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Relay authentication response message identity |  | ‘0110 1101’B |  |  |
| PRTI |  | FFS |  |  |
| EAP message |  | FFS |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

### 4.7.2	Contents of 5GSM messages

#### –	PDU session establishment request

Table 4.7.2-1: PDU SESSION ESTABLISHMENT REQUEST

| Derivation Path: 24.501 clause 8.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | Any value according to TS 24.501 [25] subclause 9.4 |  |  |
| PTI |  | Any value from 1 to 254 |  |  |
| PDU SESSION ESTABLISHMENT REQUEST message identity |  | ‘1100 0001’B |  |  |
| Integrity protection maximum data rate |  | Present but contents not checked |  |  |
| PDU session type |  | Any value between '001'B, '010'B, '011'B and ‘101’B | The allowed values are respectively IPv4, IPv6, IPv4v6 and Ethernet (EtherType as defined in IEEE 802.3) |  |
| SSC mode |  | Not checked |  |  |
| 5GSM capability |  | Not checked |  |  |
| Maximum number of supported packet filters |  | Not checked |  |  |
| Always-on PDU session requested |  | Not checked |  |  |
| Always-on PDU session requested |  |  |  | SST_URLLC |
| APSR |  | ‘1’B | Always-on PDU session requested |  |
| SM PDU DN request container |  | Not checked |  |  |
| Extended protocol configuration options |  | Not checked | The SS shall remember if this IE is present and its contents because this affects subsequent SS behaviour, e.g. coding of PDU SESSION ESTABLISHMENT ACCEPT. |  |
| Extended protocol configuration options |  | Present including at least the following container |  | DATA_OFF |
| Container ID n |  | ‘0017’H | 3GPP PS data off UE status |  |
| Length of container ID n contents |  |  | 1 octet |  |
| Container ID n contents |  | ‘01’H | ‘deactivated’ |  |
| IP header compression configuration |  | Not checked |  |  |
| DS-TT Ethernet port MAC address |  | Not checked |  |  |
| DS-TT Ethernet port MAC address |  | Any value according to TS 24.501 [25] subclause 9.11.4.25 |  | Ethernet |
| UE-DS-TT residence time |  | Not checked |  |  |
| Port management information container |  | Not checked |  |  |
| Ethernet header compression configuration |  | Not checked |  |  |
| Ethernet header compression configuration |  | Any value according to TS 24.501 [25] subclause 9.11.4.28 |  | Ethernet |
| Suggested interface identifier |  | Not checked |  |  |
| Service-level-AA container |  | Not checked |  |  |
| Requested MBS container |  | Not checked |  |  |
| PDU session pair ID |  | Not checked |  |  |
| RSN |  | Not checked |  |  |
| URSP rule enforcement reports |  | Not checked |  |  |

| Condition | Explanation |
| --- | --- |
| DATA_OFF | If the UE supports 3GPP PS data off as specified in TS 38.508-2 [10], Table A.4.3.7-1/xx |
| Ethernet | If the PDU session type = '101'B. |

#### –	PDU session establishment accept

Table 4.7.2-2: PDU SESSION ESTABLISHMENT ACCEPT

| Derivation Path: 24.501 clause 8.3.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Extended protocol discriminator | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID | The same value as the value set in PDU SESSION ESTABLISHMENT REQUEST message |  |  |
| PTI | The same value as the value set in PDU SESSION ESTABLISHMENT REQUEST message |  |  |
| PDU SESSION ESTABLISHMENT ACCEPT message identity | ‘1100 0010’B |  |  |
| Selected PDU session type | '001'B |  | IPv4 |
|  | '010'B |  | IPv6 |
|  | '011'B |  | IPv4v6 |
|  | '101'B |  | Ethernet |
| Selected SSC mode | ‘001’B | SSC mode 1 |  |
| Authorized QoS rules | 5GC QoS rule of the entry in Table 4.8.4-1 which has been determined by the DNN IE in the UL NAS TRANSPORT message which carried the corresponding PDU SESSION ESTABLISHMENT REQUEST or by pc_APN_Default_Configuration if the DNN IE was not present |  |  |
| Session AMBR |  |  |  |
| Unit for Session-AMBR for downlink | ‘000 00101’ | Value is incremented in multiples of 256 Kbps |  |
| Session-AMBR for downlink | ‘0000 0000 0000 0100’B | 1024 Kbps |  |
| Unit for Session-AMBR for uplink | ‘000 00101’ | Value is incremented in multiples of 256 Kbps |  |
| Session-AMBR for uplink | ‘0000 0000 0000 0100’B | 1024 Kbps |  |
| 5GSM cause | Not Present |  |  |
| PDU address |  |  | IPv4 |
| Length of PDU address contents | 5 octets |  |  |
| PDU type value | ‘001’B | IPv4 |  |
| PDU address information | IPv4 address | The SS provides a valid IPv4 address | NOT IPv4-DHCP |
|  | 0.0.0.0 | DHCPv4 is to be used to allocate the IPv4 address | IPv4-DHCP |
| PDU address |  |  | IPv6 |
| Length of PDU address contents | 9 octets |  |  |
| PDU type value | ‘010’B | IPv6 |  |
| PDU address information | IPv6 interface identifier | The SS provides a valid IPv6 interface identifier |  |
| PDU address |  |  | IPv4v6 |
| Length of PDU address contents | 13 octets |  |  |
| PDU type value | ‘011’B | IPv4v6 |  |
| PDU address information (Octets 4 to 11) | IPv6 interface identifier | The SS provides a valid IPv6 interface identifier |  |
| PDU address information (Octets 12 to 15) | IPv4 address | The SS provides a valid IPv4 address | NOT IPv4-DHCP |
|  | 0.0.0.0 | DHCPv4 is to be used to allocate the IPv4 address | IPv4-DHCP |
| RQ timer value | Not Present |  |  |
| S-NSSAI |  |  |  |
| Length of S-NSSAI contents | ‘0000 0001’B | SST |  |
| SST | ‘0000 0001’B | SST value 1 (eMBB) | SST_eMBB |
| SST | ‘0000 0010’B | SST value 2 (URLLC) | SST_URLLC |
| SST | ‘0000 0011’B | SST value 3 (MIoT) | SST_MIoT |
| SST | ‘0000 0100’B | SST value 4 (V2X) | SST_V2X |
| Always-on PDU session indication | Not Present |  |  |
| Always-on PDU session indication |  |  |  |
| APSI | ‘0’B | Always-on PDU session not allowed | Always_On_Requested |
| APSI | ‘1’B | Always-on PDU session required | Always_On_Requested AND SST_URLLC |
| Mapped EPS bearer contexts | Not Present |  |  |
| Mapped EPS bearer contexts |  |  | Interworking_with_EPS |
| Mapped EPS bearer context |  |  |  |
| EPS bearer identity | The same value as the one specified in the Reference QoS flow referred to from the Reference QoS rule indicated in the IE Authorized QoS rules |  |  |
| Operation code | ‘001’B | Create new EPS bearer |  |
| E bit | '1'B | Parameters list is included |  |
| Number of EPS parameters | ’0001’B | 1 parameter |  |
| Mapped EPS QoS parameters | EPC default bearer context of the entry in Table 4.8.4-1 which has been determined by the DNN IE in the UL NAS TRANSPORT message which carried the corresponding PDU SESSION ESTABLISHMENT REQUEST or by pc_APN_Default_Configuration if the DNN IE was not present |  |  |
| EAP message | Not Present |  |  |
| Authorized QoS flow descriptions | The QoS flow referred to in the relevant Authorized QoS rules IE |  |  |
| Extended protocol configuration options | Not Present |  |  |
| Extended protocol configuration options |  |  | P-CSCF_IPv6 OR P-CSCF_IPv4 |
| Container ID 1 | ‘0001’H |  | P-CSCF_IPv6 |
| Length of container ID 1 contents |  | Length value determined by test implementation |  |
| Container ID 1 contents | IPv6 address | P-CSCF IPv6 Address |  |
| Container ID 2 | ‘000C’H |  | P-CSCF_IPv4 |
| Length of container ID 2 contents |  | Length value determined by test implementation |  |
| Container ID 2 contents | IPv4 address | P-CSCF IPv4 Address |  |
| Container ID 3 | ‘0001’H |  | Additional_P-CSCF_IPv6 |
| Length of container ID 3 contents |  | Length value determined by test implementation |  |
| Container ID 3 contents | IPv6 address | Additional P-CSCF IPv6 Address |  |
| Container ID 4 | ‘000C’H |  | Additional_P-CSCF_IPv4 |
| Length of container ID 4 contents |  | Length value determined by test implementation |  |
| Container ID 4 contents | IPv4 address | Additional P-CSCF IPv4 Address |  |
| Container ID n | ‘0003’H | n assigned to next available number | DNS IPv6 AND XCAP |
| Length of container ID n contents |  | Length value determined by the TTCN implementation |  |
| Container ID n contents | IPv6 address | DNS IPv6 address |  |
| Container ID n+1 | ‘000D’H | n assigned to next available number | DNS IPv4 AND XCAP |
| Length of container ID n contents |  | Length value determined by the TTCN implementation |  |
| Container ID n contents | IPv4 address | DNS IPv4 address |  |
| DNN | The DNN/APN ID of the entry in Table 4.8.4-1 which has been determined by the DNN IE in the UL NAS TRANSPORT message which carried the corresponding PDU SESSION ESTABLISHMENT REQUEST or by pc_APN_Default_Configuration if the DNN IE was not present |  |  |
| 5GSM network feature support | Not Present |  |  |
| Serving PLMN rate control | Not Present |  |  |
| ATSSS container | Not Present |  |  |
| Control plane only indication | Not Present |  |  |
| IP header compression configuration | Not Present |  |  |
| Ethernet header compression configuration | Not Present |  |  |
| Ethernet header compression configuration | Any value according to TS 24.501 [25] subclause 9.11.4.28 |  | Ethernet |
| Service-level-AA container | Not Present |  |  |
| Received MBS container | Not Present |  |  |
| N3QAI | Not Present |  |  |
| Protocol description | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| IPv4 | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message, the PDU session type = '001'B |
| IPv6 | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message, the PDU session type = '010'B |
| IPv4v6 | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message, the PDU session type = '011'B |
| IPv4-DHCP | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message, the IE Extended protocol configuration options contains a configuration protocol option = '000BH' ("IPv4 address allocation via DHCPv4", length of contents = 0). Note: This condition is used in conjunction with IPv4 or IPv4v6 as indicated in the "PDU address information" just above. |
| Ethernet | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message, the PDU session type = '101'B. |
| Always_On_Requested | If the last PDU SESSION ESTABLISHMENT REQUEST message included the Always-on PDU session requested IE |
| P-CSCF_IPv6 | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message the IE Extended protocol configuration options contains a configuration protocol option = '0001H' (“P-CSCF IPv6 Address Request", length of contents = 0) |
| P-CSCF_IPv4 | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message the IE Extended protocol configuration options contains a configuration protocol option = '000CH' (“P-CSCF IPv4 Address Request", length of contents = 0) |
| Interworking_with_EPS | If the UE has indicated support of S1, then the SS shall include this IE to provide details for the interworking with EPS being supported for a PDU session. This requirement is set up for the purpose of facilitating the test description. It is not mandatory for the Network to support Mapped EPS bearer contexts. |
| Additional_P-CSCF_IPv6 | P-CSCF_IPv6 AND SS requires to be configured with second P-CSCF Ipv6 address |
| Additional _P-CSCF_IPv4 | P-CSCF_IPv6 AND SS requires to be configured with second P-CSCF Ipv4 address |
| DNS IPv6 | If in the last PDU Session Establishment REQUEST the Extended protocol configuration options was included with a “DNS IPv6 Address Request”. |
| DNS IPv4 | If in the last PDU Session Establishment REQUEST the Extended protocol configuration options was included with a “DNS IPv4 Address Request”. |
| XCAP | Used for test cases (TS 34.229-5[47]) involving signalling between UE and XCAP server to activate or deactivate a supplementary service. |

#### –	PDU session establishment reject

Table 4.7.2-3: PDU SESSION ESTABLISHMENT REJECT

| Derivation Path: 24.501 clause 8.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The same value as the value set in PDU SESSION ESTABLISHMENT REQUEST message |  |  |
| PTI |  | The same value as the value set in PDU SESSION ESTABLISHMENT REQUEST message |  |  |
| PDU SESSION ESTABLISHMENT REJECT message identity |  | ‘1100 0011’B |  |  |
| 5GSM cause |  | The value is set according to specific message content |  |  |
| Back-off timer value |  | Not Present |  |  |
| Allowed SSC mode |  | Not Present |  |  |
| EAP message |  | Not Present |  |  |
| 5GSM congestion re-attempt indicator |  | Not Present |  |  |
| Extended protocol configuration options |  | Not Present |  |  |
| Re-attempt indicator |  | Not Present |  |  |
| Service-level-AA container |  | Not Present |  |  |

#### –	PDU session authentication command

Table 4.7.2-4: PDU SESSION AUTHENTICATION COMMAND

| Derivation Path: 24.501 clause 8.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | Set according to specific message content |  |  |
| PTI |  | '0000 0000'B | No procedure transaction identity assigned |  |
| PDU SESSION AUTHENTICATION COMMAND message identity |  | ‘1100 0101’B |  |  |
| EAP message |  | Set according to TS 38.508 Table 4.7.3.2-1 | See TS 24.501 [25] subclause 9.11.2.2 |  |
| Extended protocol configuration options |  | Not Present |  |  |

#### –	PDU session authentication complete

Table 4.7.2-5: PDU SESSION AUTHENTICATION COMPLETE

| Derivation Path: 24.501 clause 8.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The value indicated in PDU SESSION AUTHENTICATION COMMAND message |  |  |
| PTI |  | '0000 0000'B | No procedure transaction identity assigned |  |
| PDU SESSION AUTHENTICATION COMPLETE message identity |  | ‘1100 0110’B |  |  |
| EAP message |  | Set according to TS 38.508 Table 4.7.3.2-2 | See TS 24.501 [25] subclause 9.11.2.2 |  |
| Extended protocol configuration options |  | Not checked |  |  |

#### –	PDU session authentication result

Table 4.7.2-6: PDU SESSION AUTHENTICATION RESULT

| Derivation Path: 24.501 clause 8.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The value indicated in PDU SESSION AUTHENTICATION COMMAND message |  |  |
| PTI |  | '0000 0000'B | No procedure transaction identity assigned |  |
| PDU SESSION AUTHENTICATION RESULT message identity |  | ‘1100 0111’B |  |  |
| EAP message |  | Set according to specific message content | See TS 24.501 [25] subclause 9.11.2.2 |  |
| Extended protocol configuration options |  | Not Present |  |  |

#### –	PDU session modification request

Table 4.7.2-7: PDU SESSION MODIFICATION REQUEST

| Derivation Path: 24.501 clause 8.3.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The value indicated in PDU SESSION ESTABLISHMENT REQUEST message |  |  |
| PTI |  | Any value from 1 to 254 |  |  |
| PDU SESSION MODIFICATION REQUEST message identity |  | ‘1100 1001’B |  |  |
| 5GSM capability |  | Not checked |  |  |
| 5GSM cause |  | Not checked |  |  |
| Maximum number of supported packet filters |  | Not checked |  |  |
| Always-on PDU session requested |  | Not checked |  |  |
| Integrity protection maximum data rate |  | Not checked |  |  |
| Requested QoS rules |  | Not checked |  |  |
| Requested QoS flow descriptions |  | Not checked |  |  |
| Mapped EPS bearer contexts |  | Not checked |  |  |
| Extended protocol configuration options |  | Not checked |  |  |
| Port management information container |  | Not checked |  |  |
| IP header compression configuration |  | Not checked |  |  |
| Ethernet header compression configuration |  | Not checked |  |  |
| Requested MBS container |  | Not checked |  |  |
| Service-level-AA container |  | Not checked |  |  |
| Non-3GPP delay budget |  | Not checked |  |  |
| URSP rule enforcement reports |  | Not checked |  |  |

#### –	PDU session modification reject

Table 4.7.2-8: PDU SESSION MODIFICATION REJECT

| Derivation Path: 24.501 clause 8.3.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The value indicated in PDU SESSION MODIFICATION REQUEST message. |  |  |
| PTI |  | The value indicated in PDU SESSION MODIFICATION REQUEST message. |  |  |
| PDU SESSION MODIFICATION REJECT message identity |  | ‘1100 1010’B |  |  |
| 5GSM cause |  | Set according to specific message content. |  |  |
| Back-off timer value |  | Not Present |  |  |
| 5GSM congestion re-attempt indicator |  | Not Present |  |  |
| Extended protocol configuration options |  | Not Present |  |  |
| Re-attempt indicator |  | Not Present |  |  |

#### –	PDU session modification command

Table 4.7.2-9: PDU SESSION MODIFICATION COMMAND

| Derivation Path: 24.501 clause 8.3.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | Set according to specific message content. |  |  |
| PDU session ID |  | The value indicated in PDU SESSION MODIFICATION REQUEST message. |  | UE_Initiated_Modification |
| PTI |  | '0000 0000'B | No procedure transaction identity assigned |  |
| PTI |  | The value indicated in PDU SESSION MODIFICATION REQUEST message. |  | UE_Initiated_Modification |
| PDU SESSION MODIFICATION COMMAND message identity |  | ‘1100 1011’B |  |  |
| 5GSM cause |  | Not Present |  |  |
| Session AMBR |  | Not Present |  |  |
| RQ timer value |  | Not Present |  |  |
| Always-on PDU session indication |  | Not Present |  |  |
| Always-on PDU session indication |  |  |  |  |
| APSI |  | ‘0’B | Always-on PDU session not allowed | Always_On_Requested |
| APSI |  | ‘1’B | Always-on PDU session required | Always_On_Requested AND SST_URLLC |
| Authorized QoS rules |  | Not Present |  |  |
| Mapped EPS bearer contexts |  | Not Present |  |  |
| Authorized QoS flow descriptions |  | Not Present |  |  |
| Extended protocol configuration options |  | Not Present |  |  |
| ATSSS container |  | Not Present |  |  |
| IP header compression configuration |  | Not Present |  |  |
| Port management information container |  | Not Present |  |  |
| Serving PLMN rate control |  | Not Present |  |  |
| Ethernet header compression configuration |  | Not Present |  |  |
| Received MBS container |  | Not Present |  |  |
| Service-level-AA container |  | Not Present |  |  |
| Alternative S-NSSAI |  | Not Present |  |  |
| N3QAI |  | Not Present |  |  |
| Protocol description |  | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| Always_On_Requested | If the last PDU SESSION MODIFICATION REQUEST message included the Always-on PDU session requested IE |
| UE_Initiated_Modification | If this message was triggered by a PDU SESSION MODIFICATION REQUEST message sent by the UE |

#### –	PDU session modification complete

Table 4.7.2-10: PDU SESSION MODIFICATION COMPLETE

| Derivation Path: 24.501 clause 8.3.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The value indicated in PDU SESSION MODIFICATION COMMAND message |  |  |
| PTI |  | '0000 0000'B | No procedure transaction identity assigned |  |
| PTI |  | The value indicated in PDU SESSION MODIFICATION REQUEST message. |  | UE_Initiated_Modification |
| PDU SESSION MODIFICATION COMPLETE message identity |  | ‘1100 1100’B |  |  |
| Extended protocol configuration options |  | Not checked |  |  |
| Port management information container |  | Not checked |  |  |

| Condition | Explanation |
| --- | --- |
| UE_Initiated_Modification | If this message was triggered by a PDU SESSION MODIFICATION REQUEST message sent by the UE |

#### –	PDU session modification command reject

Table 4.7.2-11: PDU SESSION MODIFICATION COMMAND REJECT

| Derivation Path: 24.501 clause 8.3.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The value indicated in PDU SESSION MODIFICATION COMMAND message |  |  |
| PTI |  | '0000 0000'B | No procedure transaction identity assigned |  |
| PDU SESSION MODIFICATION COMMAND REJECT message identity |  | ‘1100 1101’B |  |  |
| 5GSM cause |  | Not checked |  |  |
| Extended protocol configuration options |  | Not checked |  |  |

#### –	PDU session release request

Table 4.7.2-12: PDU SESSION RELEASE REQUEST

| Derivation Path: 24.501 clause 8.3.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The value indicated in PDU SESSION ESTABLISHMENT REQUEST message |  |  |
| PTI |  | Any value from 1 to 254 |  |  |
| PDU SESSION RELEASE REQUEST message identity |  | ‘1101 0001’B |  |  |
| 5GSM cause |  | Not checked |  |  |
| Extended protocol configuration options |  | Not checked |  |  |

#### –	PDU session release reject

Table 4.7.2-13: PDU SESSION RELEASE REJECT

| Derivation Path: 24.501 clause 8.3.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The value indicated in PDU SESSION RELEASE REQUEST message. |  |  |
| PTI |  | The value indicated in PDU SESSION RELEASE REQUEST message. |  |  |
| PDU SESSION RELEASE REJECT message identity |  | ‘1101 0010’B |  |  |
| 5GSM cause |  | Set according to specific message content. |  |  |
| Extended protocol configuration options |  | Not Present |  |  |

#### –	PDU session release command

Table 4.7.2-14: PDU SESSION RELEASE COMMAND

| Derivation Path: 24.501 clause 8.3.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | Set according to specific message content. |  |  |
| PTI |  | '0000 0000'B | No procedure transaction identity assigned |  |
| PDU SESSION RELEASE COMMAND message identity |  | ‘1101 0011’B |  |  |
| 5GSM cause |  | ‘0001 1010’B | Insufficient resources |  |
| Back-off timer value |  | Not Present |  |  |
| EAP message |  | Not Present |  |  |
| 5GSM congestion re-attempt indicator |  | Not Present |  |  |
| Extended protocol configuration options |  | Not Present |  |  |
| Access type |  | Not Present |  |  |
| Service-level-AA container |  | Not Present |  |  |
| Alternative S-NSSAI |  | Not Present |  |  |

#### –	PDU session release complete

Table 4.7.2-15: PDU SESSION RELEASE COMPLETE

| Derivation Path: 24.501 clause 8.3.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The value indicated in PDU SESSION RELEASE COMMAND message. |  |  |
| PTI |  | '0000 0000'B | No procedure transaction identity assigned |  |
| PDU SESSION RELEASE COMPLETE message identity |  | ‘1101 0100’B |  |  |
| 5GSM cause |  | Not checked |  |  |
| Extended protocol configuration options |  | Not checked |  |  |

#### –	5GSM status

Table 4.7.2-16: 5GSM STATUS

| Derivation Path: 24.501 clause 8.3.16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | Set according to specific message content. |  |  |
| PTI |  | Set according to specific message content. |  |  |
| 5GSM STATUS message identity |  | ‘1101 0110’B |  |  |
| 5GSM cause |  | Set according to specific message content. |  |  |

#### –	Service-level authentication command

Table 4.7.2-17: SERVICE-LEVEL AUTHENTICATION COMMAND

| Derivation Path: 24.501 clause 8.3.17 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | Set according to specific message content. |  |  |
| PTI |  | Set according to specific message content. |  |  |
| SERVICE-LEVEL AUTHENTICATION COMMAND message identity |  | ‘1101 1000’B |  |  |
| Service-level-AA container |  | FFS |  |  |

#### –	Service-level authentication complete

Table 4.7.2-18: SERVICE-LEVEL AUTHENTICATION COMPLETE

| Derivation Path: 24.501 clause 8.3.18 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | Set according to specific message content. |  |  |
| PTI |  | Set according to specific message content. |  |  |
| SERVICE-LEVEL AUTHENTICATION COMPLETE message identity |  | ‘1101 1001’B |  |  |
| Service-level-AA container |  | FFS |  |  |

#### –	Remote UE report

Table 4.7.2-19: REMOTE UE REPORT

| Derivation Path: 24.501 clause 8.3.19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | Set according to specific message content. |  |  |
| PTI |  | Set according to specific message content. |  |  |
| REMOTE UE REPORT message identity |  | ‘1101 1010’B |  |  |
| Remote UE context connected |  | FFS |  |  |
| Remote UE context disconnected |  | FFS |  |  |

#### –	Remote UE report response

Table 4.7.2-20: REMOTE UE REPORT RESPONSE

| Derivation Path: 24.501 clause 8.3.20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | Set according to specific message content. |  |  |
| PTI |  | Set according to specific message content. |  |  |
| REMOTE UE REPORT RESPONSE message identity |  | ‘1101 1011’B |  |  |

<SECTIONS SKIPPED>

### 4.7C.4	Contents of 5G ProSe discovery and security messages over PC3a

#### –	DISCOVERY_REQUEST

This message is sent by the UE to the SS.

Table 4.7C.4-1: DISCOVERY_REQUEST

| Derivation path: 24.554 [58], clause 10.5.4.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| discovery-request [1] { |  |  |  |
| transaction-ID | Present but contents not checked |  |  |
| command | 1 |  | Announce |
|  | 2 |  | Monitor |
|  | 3 | Query | Discoverer |
|  | 4 | Response | Discoveree |
| ProSe-Application-ID | Present but contents not checked |  | Open |
| RPAUID | Present but contents not checked |  | Restricted |
| application-identity | Present but contents not checked |  |  |
| discovery-type | 2 |  | Restricted |
| announcing-type | 0 |  | Restricted |
| discovery-model | 1 |  | Model_A AND Restricted |
|  | 2 |  | Model_B AND Restricted |
| Discovery-Entry-ID | Present but contents not checked |  | Open |
| discovery-entry-id | Present but contents not checked |  | Restricted |
| } |  |  |  |
| ... |  |  |  |
| discovery-request[N] {} | Not checked | A UE can include one or multiple transactions in one DISCOVERY_REQUEST message for one or more ProSe Application IDs/ RPAUIDs |  |

| Condition | Explanation |
| --- | --- |
| Announce | Announce request procedure for open or restricted 5G ProSe direct discovery. |
| Monitor | Monitor request procedure for open or restricted 5G ProSe direct discovery. |
| Discoveree | Discoveree request procedure for restricted 5G ProSe direct discovery. |
| Discoverer | Discoverer request procedure for restricted 5G ProSe direct discovery. |
| Open | 5G ProSe direct discovery is set to open. |
| Restricted | 5G ProSe direct discovery is set to restricted. |
| Model_A | Direct discovery model A is used. |
| Model_B | Direct discovery model B is used. |

#### -	DISCOVERY_RESPONSE

This message is sent by the SS to the UE.

Table 4.7C.4-2: DISCOVERY_RESPONSE

| Derivation path: 24.554 [58], clause 10.5.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Current-Time | Current UTC time |  |  |
| Max-Offset | 32 | 32 seconds |  |
| response-announce [1] { |  |  | Announce AND Open |
| transaction-ID | the transaction ID received in the DISCOVERY_REQUEST message |  |  |
| ProSe-Application-Code [1] { |  | 184 bits See TS 23.003 [2], section 24.3. |  |
| PLMN ID { |  | 24 bits The PLMN ID of the ProSe Function that assigned the ProSe Application Code, i.e. Mobile Country Code (MCC) and Mobile Network Code (MNC). |  |
| Scope | '10' | country-specific scope |  |
| Spare | '0' |  |  |
| E | '1' | MCC and MNC included |  |
| MCC | The MCC part of the The PLMN ID of the ProSe Function that assigned the ProSe Application Code (the relevant PLMN will be set in the TC) |  |  |
| MNC | The MNC part of the The PLMN ID of the ProSe Function that assigned the ProSe Application Code (the relevant PLMN will be set in the TC) |  |  |
| } |  |  |  |
| TemporaryID | 1111111100000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000011111111 | 160 bits - see setting of bit 'E' above. A temporary identity that corresponds to the ProSe Application ID Name. |  |
| } |  |  |  |
| validity-timer-T5060 | 5 | Indicating the time during which the Application Code is valid Integer in the 1-525600 range representing the timer value in unit of minutes |  |
| discovery-key | 1000100000001000 0100010000001000 0010001000001000 0001000100001000 0000100010001000 0000010001001000 0000001000101000 0000000100011000 | 128-bit As defined in TS 33.303 [60] and TS 33.220 [61] |  |
| discovery-entry-ID | the discovery-entry-ID received in the DISCOVERY_REQUEST message |  |  |
| } |  |  |  |
| restricted-announce-response [1] { |  |  | Announce AND Restricted |
| transaction-ID | the transaction ID received in the DISCOVERY_REQUEST message |  |  |
| ProSe-Restricted-Code [1] { |  |  |  |
| PLMN ID | Set to the PLMN value used in the test case | The PLMN ID of the ProSe Function that assigned the ProSe Restricted Code. |  |
| TemporaryID | 1111111100000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000011111111 | A temporary identity that corresponds to the RPAUID. |  |
| } |  |  |  |
| validity-timer-T5062 | 3 | Indicating the time during which the Application Code is valid Integer in the 1-525600 range representing the timer value in unit of minutes |  |
| discovery-entry-id | the discovery-entry-id received in the DISCOVERY_REQUEST message |  |  |
| } |  |  |  |
| response-monitor [1] { |  |  | Monitor AND Open |
| transaction-ID | the transaction ID received in the DISCOVERY_REQUEST message |  |  |
| discovery-filter [1] { |  |  |  |
| ProSe Application code { |  | 184 bits See TS 23.003 [2], section 24.3. |  |
| PLMN ID { |  | 24 bits The PLMN ID of the ProSe Function that assigned the ProSe Application Code, i.e. Mobile Country Code (MCC) and Mobile Network Code (MNC). |  |
| Scope | '10' | country-specific scope |  |
| Spare | '0' |  |  |
| E | '1' | MCC and MNC included |  |
| MCC | The MCC part of the PLMN ID of the ProSe Function that assigned the ProSe Application Code (the relevant PLMN will be set in the TC) |  |  |
| MNC | The MNC part of the PLMN ID of the ProSe Function that assigned the ProSe Application Code (the relevant PLMN will be set in the TC) |  |  |
| } |  |  |  |
| TemporaryID | 1111111100000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000011111111 | 160 bits - see setting of bit 'E' above. A temporary identity that corresponds to the ProSe Application ID Name. |  |
| } |  |  |  |
| ProSe application mask [1] | 0000111111111100 00000000 1111111111111111 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 | 184 bits - The length of the ProSe Application mask is the same as the length of ProSe Application code Full matching for the MCC part is required, as well as for the first 16 bits of the Temporary ID in order a received ProSe Application code to result in a match |  |
| ProSe application mask [2] | 0000111111111100 00000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 1111111111111111 | 184 bits - The length of the ProSe Application mask is as same as the length of ProSe Application code Full matching for the MCC part is required, as well as for the last 16 bits of the Temporary ID |  |
| TTLTimer T5064 | 3 | Indicating the time during which the filter is valid Integer in the 1-525600 range representing the timer value in unit of minutes |  |
| } |  |  |  |
| } |  |  |  |
| restricted-discoveree-response [1] { |  |  | Monitor AND Restricted |
| transaction-ID | the transaction ID received in the DISCOVERY_REQUEST message |  |  |
| restricted-discovery-filter [1] { |  |  |  |
| filter | Set according to specific message content |  |  |
| TTL Timer T5066 | 3 | Indicating the time during which the filter is valid Integer in the 1-525600 range representing the timer value in unit of minutes |  |
| RPAUID | Set according to specific message content |  |  |
| metadata indicator | 0 |  |  |
| metadata | Not present |  |  |
| } |  |  |  |
| discovery-entry-id | the discovery-entry-id received in the DISCOVERY_REQUEST message |  |  |
| } |  |  |  |
| restricted-discoveree-response [1] { |  |  | Discoveree |
| transaction-ID | the transaction ID received in the DISCOVERY_REQUEST message |  |  |
| ProSe-Response-Code { |  |  |  |
| PLMN ID | Set to the PLMN value used in the test case |  |  |
| TemporaryID | 1111111100000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000011111111 |  |  |
| } |  |  |  |
| validity-timer-T5068 | 3 | Indicating the time during which the filter is valid Integer in the 1-525600 range representing the timer value in unit of minutes |  |
| discovery-entry-id | the discovery-entry-id received in the DISCOVERY_REQUEST message |  |  |
| } |  |  |  |
| restricted-discoverer-response [1] { |  |  | Discoverer |
| transaction-ID | the transaction ID received in the DISCOVERY_REQUEST message |  |  |
| subquery-result [1] { |  |  |  |
| ProSe query code { |  |  |  |
| PLMN ID | Set to the PLMN value used in the test case |  |  |
| TemporaryID | 1111111100000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000011111111 |  |  |
| } |  |  |  |
| discovery response filter | Set according to specific message content |  |  |
| validity timer T5070 | 3 | Indicating the time during which the filter is valid Integer in the 1-525600 range representing the timer value in unit of minutes |  |
| code-sending security parameter | Set according to specific message content |  |  |
| code-receiving security parameter | Set according to specific message content |  |  |
| RPAUID | Set according to specific message content |  |  |
| metadata | Not present |  |  |
| } |  |  |  |
| discovery-entry-id | the discovery-entry-id received in the DISCOVERY_REQUEST message |  |  |
| } |  |  |  |
| response-reject | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| Announce | Announce request procedure for open or restricted 5G ProSe direct discovery. |
| Monitor | Monitor request procedure for open or restricted 5G ProSe direct discovery. |
| Discoveree | Discoveree request procedure for restricted 5G ProSe direct discovery. |
| Discoverer | Discoverer request procedure for restricted 5G ProSe direct discovery. |
| Open | 5G ProSe direct direct discovery is set to open. |
| Restricted | 5G ProSe direct direct discovery is set to restricted. |

#### -	MATCH_REPORT

This message is sent by the UE to the SS.

Table 4.7C.4-3: MATCH_REPORT

| Derivation path: 24.554 [58], clause 10.5.4.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| transaction-ID | a new transaction ID |  |  |
| ProSe-PC5-discovery-message | PROSE PC5 DISCOVERY message for open 5G ProSe direct discovery announcement as defined in Table 4.7C.2-1 |  |  |
| Monitored-PLMN-id | The expected value will be set in the relevant TC. |  |  |
| VPLMN-id | Not included |  |  |
| UTC-based Counter | Generated by UE | This parameter is used to indicate the UTC time associated with the discovery transmission opportunity in which a PROSE PC5 DISCOVERY message is sent. It is expressed in unit of seconds and coded in binary format as the 32 least significant bits of the Coordinated Universal Time as defined in 3GPP TS 38.331 [13]. |  |
| metadata-flag | Present but contents not checked |  |  |