3GPP TSG-RAN5 Meeting #106	R5-250362

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0074 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modify transmit power related test cases test frequency rang requirements to be aligned with regular LTE UE requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | While LTE NB-IoT NTN is used in smart phones for mission critical services, LTE NB-IoT NTN transmit power test procedures should be tested the same frequency ranges as regular LTE UEs. For test procedures with NS values, instead of following Criteria 2, test low, mid, and high frequency ranges |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following test configuration tables are modified: - Table 6.2B.1.4.1-1 - Table 6.2B.2.4.1-1 - Table 6.2B.3.4.1-1 - Table 6.2B.3.4.1-2 - Table 6.2B.4.4.1-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The existing test frequency ranges are not aligned with regular LTE UE test frequency range requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

Table 6.2B.1.4.1-1: Test Configuration Initial Conditions for FDD

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Low range, Mid range, High range |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 2) |  | BPSK | 1@0 | 3.75 |
| 2 (Note 3) |  | BPSK | 1@47 | 3.75 |
| 3 (Note 2) |  | QPSK | 1@0 | 15 |
| 4 (Note 3) |  | QPSK | 1@11 | 15 |
| 5 (Note 1) |  | QPSK | 3@3 | 15 |
| NOTE 1: Applicable to UE supporting UL multi-tone transmissions. NOTE 2: only applicable for low range. NOTE 3: only applicable for high range. |  |  |  |  |

<skipped unchanged sections>

Table 6.2B.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Low range, Mid range, High range |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A for Maximum Power Reduction (MPR) test case | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 |  | QPSK | 1@0 | 3.75 |
| 2 |  | QPSK | 1@47 | 3.75 |
| 3 |  | BPSK | 1@0 | 15 |
| 4 |  | BPSK | 1@11 | 15 |
| 5 (Note 1) |  | QPSK | 3@0 | 15 |
| 6 (Note 1) |  | QPSK | 3@3 | 15 |
| 7 (Note 1) |  | QPSK | 3@9 | 15 |
| 8 (Note 1) |  | QPSK | 6@0 | 15 |
| 9 (Note 1) |  | QPSK | 6@6 | 15 |
| 10 (Note 1) |  | QPSK | 12@0 | 15 |
| NOTE 1: Applicable to UE supporting UL multi-tone transmissions. |  |  |  |  |

<skipped unchanged sections>

Table 6.2B.3.4.1-1: Test Configuration Table (network signalled value “NS_02N”)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | NC |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Low range, Mid range, High range |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A for Maximum Power Reduction (MPR) test case | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 1) |  | QPSK | 3@0 | 15 |
| 2 (Note 1) |  | QPSK | 3@3 | 15 |
| 3 (Note 1) |  | QPSK | 3@9 | 15 |
| 4 (Note 1) |  | QPSK | 6@0 | 15 |
| 5 (Note 1) |  | QPSK | 6@6 | 15 |
| 6 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

Table 6.2B.3.4.1-2: Test Configuration Table (network signalled value "NS_24")

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | NC |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Low range, Mid range, High range |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A for Maximum Power Reduction (MPR) test case | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 1) |  | QPSK | 3@0 | 15 |
| 2 (Note 1) |  | QPSK | 3@3 | 15 |
| 3 (Note 1) |  | QPSK | 3@9 | 15 |
| 4 (Note 1) |  | QPSK | 6@0 | 15 |
| 5 (Note 1) |  | QPSK | 6@6 | 15 |
| 6 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

<skipped unchanged sections>

Table 6.2B.4.4.1-1: Test Configuration Table for FDD & TDD

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Mid range |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 |  | QPSK | 1@0 | 3.75 |
| 2 |  | QPSK | 1@47 | 3.75 |
| 3 |  | QPSK | 1@0 | 15 |
| 4 |  | QPSK | 1@11 | 15 |
| 5 (Note 1) |  | QPSK | 12@0 | 15 |
| NOTE 1: Applicable to UE supporting UL multi-tone transmissions. |  |  |  |  |

<End of changes>