3GPP TSG-RAN5 Meeting #106	R5-250624

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3200 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of 7.3I.3 Reference sensitivity TC for eRedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For UE supporting IE supportOfERedCap-r18 but not supporting eRedCapNotReducedBB-BW-r18，the minimum conformance requirement of UL configuration is: Both Tx RBs in UL configuration and Rx RBs in FRC, when applicable, shall be allocated within the range from RBlow = ceil(NRB/2 - NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 -1, where RBlow and RBhigh are the lowest and highest available RB position and NRB_PR3 is 12 RBs for 30 kHz SCS. The “NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1)” in UL configuration table is from general refsens TC for single carrier in 7.3.2, which is not applicable for eRedCap UE, should be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the NOTE 1 in Table 7.3I.3.4.1-2. Fixed some format issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reference sensitivity for eRedCap is stilling confused. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3I.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

7.3I.3	Reference sensitivity power level for eRedCap

7.3I.3.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

7.3I.3.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR eRedCap.

7.3I.3.3	Minimum conformance requirements

For UE supporting IE supportOfERedCap-r18 and IE eRedCapNotReducedBB-BW-r18, the REFSENS requirements for RedCap UE in clause 7.3I.2 apply.

For UE supporting IE supportOfERedCap-r18 but not supporting eRedCapNotReducedBB-BW-r18, the reference sensitivity requirements for 5 MHz channel bandwidth with 15 kHz SCS defined in clause 7.3I.2 apply. These reference sensitivity requirements for 5 MHz channel bandwidth apply also for 10, 15 and 20 MHz channel bandwidths with 15 kHz SCS. In case the reference sensitivity requirements for 5 MHz channel bandwidth are not defined, the reference sensitivity requirements for 10 MHz channel bandwidth apply with the reference sensitivity level reduced by 3.2 dB, and the UL configuration shall be less than or equal to minimum between RB number specified in Table 7.3I.2-4  and 25 RBs for HD-FDD operation, and minimum between RB number specified in Table 7.3.2-3 and 25RBs otherwise. These reference sensitivity requirements for 10 MHz channel bandwidth apply also for 15 and 20 MHz channel bandwidths. Both Tx RBs in UL configuration and Rx RBs in FRC, when applicable, shall be allocated within the range from RBlow = ceil(NRB/2 - NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 -1, where RBlow and RBhigh are the lowest and highest available RB position and NRB_PR3 is 25 RBs for 15 kHz SCS.

For UE supporting IE supportOfERedCap-r18 but not supporting eRedCapNotReducedBB-BW-r18, for 30 kHz SCS, the reference sensitivity requirements defined for 10 MHz channel bandwidth in clause 7.3I.2 apply with reference sensitivity level reduced by 3.0 dB and the UL configuration shall be less than or equal to minimum between RB number specified in Table 7.3I.2-4 and 12 RBs for HD-FDD operation, and minimum between RB number specified in Table 7.3.2-3 and 12RBs otherwise. These reference sensitivity requirements for 10 MHz channel bandwidth apply also for 15 and 20 MHz channel bandwidths with 30 kHz SCS. Both Tx RBs in UL configuration and Rx RBs in FRC, when applicable, shall be allocated within the range from RBlow = ceil(NRB/2 - NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 -1, where RBlow and RBhigh are the lowest and highest available RB position and NRB_PR3 is 12 RBs for 30 kHz SCS.

NOTE:	It is not necessary to repeat verification when same requirement applies for multiple channel bandwidths.

The normative reference for this requirement is TS 38.101-1 [2] clause 7.3I.3.

7.3I.3.4	Test description

7.3I.3.4.1	Initial conditions

For eRedCap UE supporting IE eRedCapNotReducedBB-BW-r18, same initial conditions as in 7.3I.2.4.1 with following exception:

For eRedCap UE not supporting eRedCapNotReducedBB-BW-r18, same initial conditions as in 7.3I.2.4.1 with following exception:

-	The test channel bandwidth are specified in TS 38.508-1 [5] subclause 4.3.1 for RedCap.

-	The downlink RB allocation in test configuration table refers to Table 7.3I.3.4.1-1

-	The uplink RB allocation in test configuration table refers to Table 7.3I.3.4.1-2 for FDD and TDD eRedCap UE, and Table 7.3I.3.4.1-3 for HD-FDD eRedCap UE respectively.

Table 7.3I.3.4.1-1: Downlink Configuration of each RB allocation of eRedCap UE not supporting eRedCapNotReducedBB-BW-r18

| Channel Bandwidth | SCS(kHz) | LCRBmax | Outer RB allocation / Normal RB allocation |
| --- | --- | --- | --- |
| 5MHz | 15 | 25 | 25@0 |
|  | 30 | 11 | 11@0 |
| 10MHz | 15 | 25 | 25@14 |
|  | 30 | 12 | 12@6 |
| 15MHz | 15 | 25 | 25@27 |
|  | 30 | 12 | 12@13 |
| 20MHz | 15 | 25 | 25@41 |
|  | 30 | 12 | 12@20 |

Table 7.3I.3.4.1-2: Uplink configuration for reference sensitivity of FDD or TDD eRedCap UE not supporting IE eRedCapNotReducedBB-BW-r18, LCRB @ RBstart format

| Operating Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
| n1 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n2 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 | 10@1 | 12@6 | 12@13 | 12@20 |  |
| n3 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n5 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 10@8 | 10@15 | 10@22 |  |
| n7 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n8 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 10@8 | 10@15 | 10@22 |  |
| n12 | 15 | 20@5 | 20@19 | 20@32 |  | FDD |
|  | 30 |  | 10@8 | 10@15 |  |  |
| n13 | 15 | 20@0 | 20@14 |  |  | FDD |
|  | 30 |  | 10@6 |  |  |  |
| n14 | 15 | 20@0 | 20@14 |  |  | FDD |
|  | 30 |  | 10@6 |  |  |  |
| n20 | 15 | 25@0 | 20@14 | 20@27 | 20@41 | FDD |
|  | 30 |  | 10@6 | 10@13 | 10@20 |  |
| n24 | 15 | 25@0 | 25@14 |  |  | FDD |
|  | 30 |  | 12@6 |  |  |  |
| n25 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n26 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n28 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 10@8 | 10@15 | 10@22 |  |
| n30 | 15 | 20@5 | 20@19 |  |  | FDD |
|  | 30 |  | 10@8 |  |  |  |
| n34 | 15 | 25@0 | 25@14 | 25@27 |  | TDD |
|  | 30 |  | 12@6 | 12@13 |  |  |
| n38 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n39 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n40 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n41 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n48 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n50 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n51 | 15 | 25@0 |  |  |  | TDD |
|  | 30 |  |  |  |  |  |
| n53 | 15 | 25@0 | 25@14 |  |  | TDD |
|  | 30 |  | 12@6 |  |  |  |
| n65 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n66 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n70 | 15 | 25@0 | 25@14 | 25@27 | NOTE 3 | FDD |
|  | 30 |  | 12@6 | 12@13 | NOTE 3 |  |
| n71 | 15 | 25@0 | 25@14 | 20@32 | 20@41 | FDD |
|  | 30 |  | 12@6 | 10@15 | 10@20 |  |
| n74 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 10@8 | 10@15 | 10@22 |  |
| n77 | 15 |  | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n78 | 15 |  | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n79 | 15 |  | 25@14 |  | 25@41 | TDD |
|  | 30 |  | 12@6 |  | 12@20 |  |
| n85 | 15 | 20@5 | 20@19 | 20@32 |  | FDD |
|  | 30 |  | 10@8 | 10@15 |  |  |
| n91 | 15 | 25@04 | 20@19 4 |  |  | FDD |
|  | 30 |  |  |  |  |  |
| n92 | 15 | 20@5 | 20@19 | 20@32 | 20@41 | FDD |
|  | 30 |  | 10@8 | 10@15 | 10@20 |  |
| n93 | 15 | 25@04 | 25@144 |  |  | FDD |
|  | 30 |  |  |  |  |  |
| n94 | 15 | 25@0 | 25@14 | 20@32 | 20@41 | FDD |
|  | 30 |  | 12@6 | 10@15 | 10@20 |  |
| n100 | 15 | 25@0 |  |  |  | FDD |
|  | 30 |  |  |  |  |  |
| n101 | 15 | 25@0 | 25@14 |  |  | TDD |
|  | 30 |  | 12@6 |  |  |  |
| NOTE 1: FFS. NOTE 2: For band n20; for 15kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart =11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart =16; for 30kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart =6 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart =8. NOTE 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest duplex distance shall be used. NOTE 4: For band n91 and n93, largest supported UL bandwidth configuration shall be used. |  |  |  |  |  |  |

Table 7.3I.3.4.1-3: Uplink configuration for reference sensitivity of HD-FDD eRedCap UE not supporting IE eRedCapNotReducedBB-BW-r18, LCRB @ RBstart format

| Operating Band |  | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n2 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n3 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n5 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n7 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n8 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n12 |  | 15 | 25@0 | 25@14 | 25@27 |  | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 |  |  |
| n13 |  | 15 | 20@0 | 25@14 |  |  | HD-FDD |
|  |  | 30 |  | 12@6 |  |  |  |
| n14 |  | 15 | 25@0 | 25@14 |  |  | HD-FDD |
|  |  | 30 |  | 12@6 |  |  |  |
| n20 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n24 |  | 15 | 25@0 | 25@14 |  |  | HD-FDD |
|  |  | 30 |  | 12@6 |  |  |  |
| n25 |  | 15 | 25@0 | 50@2 | 75@4 | 25@41 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 | 12@20 |  |
| n26 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n28 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n30 |  | 15 | 25@0 | 25@14 |  |  | HD-FDD |
|  |  | 30 |  | 12@6 |  |  |  |
| n65 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n66 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n70 |  | 15 | 25@0 | 25@14 | 25@27 | NOTE 1 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | NOTE 1 |  |
| n71 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n74 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n85 | 15 | 25@0 | 25@14 | 25@27 |  | HD-FDD |
|  | 30 |  | 12@6 | 12@13 |  |  |
| n91 |  | 15 | 25@0 |  |  |  | HD-FDD |
| n92 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n93 |  | 15 | 25@0 |  |  |  | HD-FDD |
| n94 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| NOTE 1: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. |  |  |  |  |  |  |  |

7.3I.3.4.2	Test procedure

For eRedCap UE supporting IE eRedCapNotReducedBB-BW-r18, same test procedure as steps 1~4 of clause 7.3I.2.4.2.

For eRedCap UE not supporting IE eRedCapNotReducedBB-BW-r18, same test procedure as steps 1~4 of clause 7.3.2.4.2 with the following exceptions on step 3.

-	Set the Downlink signal level to the appropriate REFSENS value defined in Tables 7.3I.3.5-1, 7.3I.3.5-2 and 7.3I.3.5-5 if 2Rx antennas connected or Tables 7.3I.3.5-3, 7.3I.3.5-4 and 7.3I.3.5-6 if 1Rx antennas connected.

7.3I.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6 ensuring Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

7.3I.3.5	Test requirement

For eRedCap UE supporting IE eRedCapNotReducedBB-BW-r18, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with reference receive power level specified in Table 7.3I.2.5-1 and Table 7.3I.2.5-2 for eRedCap UE with 2 Rx antenna port, Tables 7.3I.2.5-3 and Table 7.3I.2.5-4 for eRedCap UE with single antenna port, Table 7.3I.2.5-5 for HD-FDD eRedCap UE with 2 Rx antenna port, Table 7.3I.2.5-6 for HD-FDD eRedCap UE with single antenna port, and parameters specified in Table 7.3.2.4.1-1, Table 7.3.2.4.1-2 and Table 7.3I.2.4.1-1.

For eRedCap UE not supporting IE eRedCapNotReducedBB-BW-r18, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with reference receive power level specified in Table 7.3I.3.5-1 and Table 7.3I.3.5-2 for eRedCap UE with 2 Rx antenna port, Tables 7.3I.3.5-3 and Table 7.3I.3.5-4 for eRedCap UE with single antenna port, Table 7.3I.3.5-5 for HD-FDD eRedCap UE with 2 Rx antenna port, Table 7.3I.3.5-6 for HD-FDD eRedCap UE with single antenna port, and parameters specified in Table 7.3I.3.4.1-1, Table 7.3I.3.4.1-2 and Table 7.3I.3.4.1-3.

<<End of changes>>