3GPP TSG-RAN5 Meeting #106 	R5-250944

# Athens, Greece, 17th Feb. 2025 - 21st Feb. 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0083 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of statistical testing of receiver characteristics for LTE NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Annex G, early pass/fail limit values for statistical testing of receiver characteristics are incorrect. For statistical testing which assumes the parameters of ER = 0.05, M = 1.5, CL = 95%, concrete values of early pass/fail limit were agreed in R5-221639 based on the simulation using a parameter set of {d_r = 0.006, cl_p = 0.9945}. However, the current Annex G does not follow the agreed values. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Early pass/fail limit values for statistical testing of receiver characteristics were update according to Annex H of TS 38.521-1 (V18.5.0) which is consistent with the decision of R5-221639. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Statistical testing of receiver characteristics will not be performed correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | G.2.4, G.2.5, G.7.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

# G.2	Statistical testing of receiver characteristics

## G.2.1	General

The test of receiver characteristics is two fold.

1.	A signal or a combination of signals is offered to the RX port(s) of the receiver.

2.	The ability of the receiver to demodulate /decode this signal is verified by measuring the throughput.

In (2) is the statistical aspect of the test and is treated here.

The minimum requirement for all receiver tests is >95% of the maximum throughput.

All receiver tests are performed in static propagation conditions. No fading conditions are applied.

## G.2.2	Mapping throughput to error ratio

a)	The measured information bit throughput R is defined as the sum (in kilobits) of the information bit payloads successfully received during the test interval, divided by the duration of the test interval (in seconds).

b)	In measurement practice the UE indicates successfully received information bit payload by signalling an ACK to the SS.
If payload is received, but damaged and cannot be decoded, the UE signals a NACK.

c)	Only the ACK and NACK signals, not the data bits received, are accessible to the SS.
The number of bits is known in the SS from knowledge of what payload was sent.

d)	For the reference measurement channel, applied for testing, the number of bits is different in different subframes, however in a radio frame it is fixed during one test.

e)	The time in the measurement interval is composed of successfully received subframes (ACK), unsuccessfully received subframes (NACK) and no reception at all (DTX-subframes).

f)	DTX-subframes may occur regularly according the applicable reference measurement channel (regDTX).
In real live networks this is the time when other UEs are served. In TDD these are the UL and special subframes.
regDTX vary from test to test but are fixed within the test.

g)	Additional DTX-subframes occur statistically when the UE is not responding ACK or NACK where it should. (statDTX)
This may happen when the UE was not expecting data or decided that the data were not intended for it.

The pass / fail decision is done by observing the:

-	number of NACKs

-	number of ACKs and

-	number of statDTXs (regDTX is implicitly known to the SS)

The ratio (NACK + statDTX)/(NACK+ statDTX + ACK)is the Error Ratio (ER). Taking into account the time consumed by the ACK, NACK, and DTX-TTIs (regular and statistical), ER can be mapped unambiguously to throughput for any single reference measurement channel test.

## G.2.3	Design of the test

The test is defined by the following design principles (see clause G.7, Theory….):

1.	The early decision concept is applied.

2.	A second limit is introduced: Bad DUT factor M>1

3.	To decide the test pass:

Supplier risk is applied based on the Bad DUT quality

To decide the test fail

Customer Risk is applied based on the specified DUT quality

The test is defined by the following parameters:

1.	Limit ER = 0.05 (Throughput limit = 95%)

2.	Bad DUT factor M=1.5 (selectivity)

3.	Confidence level CL = 95% (for specified DUT and Bad DUT-quality)

## G.2.4	Numerical definition of the pass fail limits

Table G.2.4-1: pass fail limits

| ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 67 | NA | 37 | 715 | 477 | 74 | 1290 | 1093 | 111 | 1847 | 1739 |
| 1 | 67 | NA | 38 | 731 | 493 | 75 | 1306 | 1110 | 112 | 1862 | 1756 |
| 2 | 95 | NA | 39 | 747 | 509 | 76 | 1321 | 1128 | 113 | 1877 | 1774 |
| 3 | 119 | NA | 40 | 763 | 525 | 77 | 1336 | 1145 | 114 | 1891 | 1792 |
| 4 | 141 | NA | 41 | 779 | 541 | 78 | 1351 | 1162 | 115 | 1906 | 1809 |
| 5 | 162 | NA | 42 | 795 | 557 | 79 | 1366 | 1179 | 116 | 1921 | 1827 |
| 6 | 183 | NA | 43 | 810 | 573 | 80 | 1382 | 1197 | 117 | 1936 | 1845 |
| 7 | 203 | NA | 44 | 826 | 590 | 81 | 1397 | 1214 | 118 | 1951 | 1863 |
| 8 | 222 | NA | 45 | 842 | 606 | 82 | 1412 | 1231 | 119 | 1966 | 1880 |
| 9 | 241 | 67 | 46 | 858 | 622 | 83 | 1427 | 1248 | 120 | 1981 | 1898 |
| 10 | 259 | 80 | 47 | 873 | 639 | 84 | 1442 | 1266 | 121 | 1995 | 1916 |
| 11 | 278 | 92 | 48 | 889 | 655 | 85 | 1457 | 1283 | 122 | 2010 | 1934 |
| 12 | 296 | 105 | 49 | 905 | 672 | 86 | 1472 | 1300 | 123 | 2025 | 1951 |
| 13 | 314 | 118 | 50 | 920 | 688 | 87 | 1487 | 1318 | 124 | 2040 | 1969 |
| 14 | 332 | 131 | 51 | 936 | 705 | 88 | 1503 | 1335 | 125 | 2055 | 1987 |
| 15 | 349 | 145 | 52 | 952 | 721 | 89 | 1518 | 1353 | 126 | 2069 | 2005 |
| 16 | 367 | 159 | 53 | 967 | 738 | 90 | 1533 | 1370 | 127 | 2084 | 2023 |
| 17 | 384 | 173 | 54 | 983 | 755 | 91 | 1548 | 1387 | 128 | 2099 | 2041 |
| 18 | 401 | 187 | 55 | 998 | 771 | 92 | 1563 | 1405 | 129 | 2114 | 2058 |
| 19 | 419 | 201 | 56 | 1014 | 788 | 93 | 1578 | 1422 | 130 | 2128 | 2076 |
| 20 | 436 | 216 | 57 | 1029 | 805 | 94 | 1593 | 1440 | 131 | 2143 | 2094 |
| 21 | 453 | 230 | 58 | 1045 | 822 | 95 | 1608 | 1457 | 132 | 2158 | 2112 |
| 22 | 469 | 245 | 59 | 1060 | 838 | 96 | 1623 | 1475 | 133 | 2173 | 2130 |
| 23 | 486 | 260 | 60 | 1076 | 855 | 97 | 1638 | 1492 | 134 | 2187 | 2148 |
| 24 | 503 | 275 | 61 | 1091 | 872 | 98 | 1653 | 1510 | 135 | 2202 | 2166 |
| 25 | 520 | 290 | 62 | 1107 | 889 | 99 | 1668 | 1527 | 136 | 2217 | 2183 |
| 26 | 536 | 305 | 63 | 1122 | 906 | 100 | 1683 | 1545 | 137 | 2232 | 2201 |
| 27 | 553 | 320 | 64 | 1137 | 923 | 101 | 1698 | 1562 | 138 | 2246 | 2219 |
| 28 | 569 | 335 | 65 | 1153 | 940 | 102 | 1713 | 1580 | 139 | 2261 | 2237 |
| 29 | 585 | 351 | 66 | 1168 | 957 | 103 | 1728 | 1598 | 140 | 2276 | 2255 |
| 30 | 602 | 366 | 67 | 1184 | 974 | 104 | 1742 | 1615 | 141 | 2291 | 2273 |
| 31 | 618 | 382 | 68 | 1199 | 991 | 105 | 1757 | 1633 | 142 | 2305 | 2291 |
| 32 | 634 | 398 | 69 | 1214 | 1008 | 106 | 1772 | 1650 | 143 | 2320 | 2309 |
| 33 | 651 | 413 | 70 | 1229 | 1025 | 107 | 1787 | 1668 | 144 | 2335 | 2327 |
| 34 | 667 | 429 | 71 | 1245 | 1042 | 108 | 1802 | 1686 | 145 | 2349 | 2345 |
| 35 | 683 | 445 | 72 | 1260 | 1059 | 109 | 1817 | 1703 | 146 | 2364 | 2363 |
| 36 | 699 | 461 | 73 | 1275 | 1076 | 110 | 1832 | 1721 | *) note 2 in G.2.5 |  |  |

NOTE 1:	The first column is the number of errors (ne = number of NACK + statDTX)

NOTE 2:	The second column is the number of samples for the pass limit (nsp, ns=Number of Samples= number of NACK + statDTX + ACK)

NOTE 3:	The third column is the number of samples for the fail limit (nsf)

NOTE 4:	The UE could be decided as early pass/fail when at least 67 samples are received. The nsf is set to NA for ne less than 9.

## G.2.5	Pass fail decision rules

The pass fail decision rules apply for a single test, comprising one component in the test vector. The over all Pass /Fail conditions are defined in clause G.2.6and G.2A.6

Having observed   0  errors, pass the test at      67+    samples,                                                 otherwise continue

Having observed   1  error,   pass the test at      67+    samples,                                                 otherwise continue

Having observed   2  errors,  pass the test at      95+    samples,                                                 otherwise continue

Etc. etc.

Having observed 145 errors,  pass the test at  2349+  samples, fail the test at 2345- samples,  otherwise continue

Having observed 146 errors,  pass the test at  2364+  samples, fail the test at 2363- samples.

Where x+ means: x or more,        x- means x or less

NOTE 1:	an ideal DUT passes after 67 samples. The maximum test time is 2364 samples.

NOTE 2:	It is allowed to deviate from the early decision concept  by  postponing  the decision (pass/fail or continue). Postponing the decision to or beyond the end of Table G.2.4-1 requires a pass fail decision against the test limit: pass the DUT for ER<0.0618, otherwise fail.

<<Unchanged sections skipped>>

# G.7	Theory to derive the numbers in Table G.2.4-1 (Informative)

Editor's note:	This clause of the Annex G is for information only and it described the background theory and information to derive the entries in the table G.2.4-1.

## G.7.1	Error Ratio (ER)

The Error Ratio (ER) is defined as the ratio of number of errors (ne) to all results, number of samples (ns).

(1-ER is the success ratio).

## G.7.2	Test Design

A statistical test is characterised by:

Test-time, Selectivity and Confidence level.

## G.7.3	Confidence level

The outcome of a statistical test is a decision. This decision may be correct or in-correct. The Confidence Level CL describes the probability that the decision is a correct one. The complement is the wrong decision probability (risk) D = 1-CL

## G.7.4	Introduction: Supplier Risk versus Customer Risk

There are two targets of decision:

(a)	A measurement on the pass-limit shows, that the DUT has the specified quality or is better with probability CL (CL e.g.95%) This shall lead to a "pass decision"

The pass-limit is on the good side of the specified DUT-quality. A more stringent CL (CL e.g.99%) shifts the pass-limit farer into the good direction. Given the quality of the DUTs is distributed, a greater CL passes less and better DUTs.

A measurement on the bad side of the pass-limit is simply “not pass” (undecided or artificial fail).

(aa)	Complementary:

A measurement on the fail-limit shows, that the DUT is worse than the specified quality with probability CL.

The fail-limit is on the bad side of the specified DUT-quality. A more stringent CL shifts the fail-limit farer into the bad direction. Given the quality of the DUTs is distributed, a greater CL fails less and worse DUTs.

A measurement on the good side of the fail-limit is simply “not fail”.

(b)	A DUT, known to have the specified quality, shall be measured and decided pass with probability CL. This leads to the test limit.

For CL e.g. 95%, the test limit is on the bad side of the specified DUT-quality. CL e.g.99% shifts the pass-limit farer into the bad direction. Given the DUT-quality is distributed, a greater CL passes more and worse DUTs.

(bb)	A DUT, known to be an (ε0) beyond the specified quality, shall be measured and decided fail with probability CL.

For CL e.g.95%, the test limit is on the good side of the specified DUT-quality.

NOTE 1:	The different sense for CL in (a), (aa) versus (b), (bb).

NOTE 2:	For constant CL in all 4 bullets (a) is equivalent to (bb) and (aa) is equivalent to (b).

## G.7.5	Supplier Risk versus Customer Risk

The table below summarizes the different targets of decision.

Table G.X.5-1: Equivalent statements

|  | Equivalent statements, using different cause-to-effect-directions, and assuming CL = constant >1/2 |  |
| --- | --- | --- |
| cause-to-effect-directions | Known measurement result estimation of the DUT’s quality | Known DUT’s quality estimation of the measurement’s outcome |
| Supplier Risk | A measurement on the pass-limit shows, that the DUT has the specified quality or is better (a) | A DUT, known to have an (ε0) beyond the specified DUT-quality, shall be measured and decided fail (bb) |
| Customer Risk | A measurement on the fail-limit shall shows, that the DUT is worse than the specified quality (aa) | A DUT, known to have the specified quality, shall be measured and decided pass (b) |

The shaded area shown the direct interpretation of Supplier Risk and Customer Risk.

The same statements can be based on other DUT-quality-definitions.

## G.7.6	Introduction: Standard test versus early decision concept

In standard statistical tests, a certain number of results (ns) is predefined in advance to the test. After ns results the number of bad results (ne) is counted and the error ratio (ER) is calculated by ne/ns.

Applying statistical theory, a decision limit can be designed, against which the calculated ER is compared to derive the decision. Such a limit is one decision point and is characterised by:

-	D: the wrong decision probability (a predefined parameter)

-	ns: the number of results (a fixed predefined parameter)

-	ne: the number of bad results (the limit based on just ns)

In the formula for the limit, D and ns can be understood as variable parameter and variable. However the standard test execution requires fixed ns and D. The property of such a test is: It discriminate between two states only, depending on the test design:

-	pass (with CL) / undecided           (undecided in the sense: finally undecided)

-	fail (with CL) / undecided            (undecided in the sense: finally undecided)

-	pass(with CL)  / fail (with CL)     (however against two limits).

In contrast to the standard statistical tests, the early decision concept predefines a set of (ne,ns) co-ordinates, representing the limit-curve for decision. After each result a preliminary ER is calculated and compared against the limit-curve. After each result one may make the decision or not (undecided for later decision). The parameters and variables in the limit-curve for the early decision concept have a similar but not equal meaning:

-	D: the wrong decision probability (a predefined parameter)

-	ns: the number of results (a variable parameter)

-	ne: the number of bad results (the limit. It varies together with ns)

To avoid a “final undecided” in the standard test, a second limit must be introduced and the single decision co-ordinate (ne,ns) needs a high ne, leading to a fixed (high) test time. In the early decision concept, having the same selectivity and the same confidence level an “undecided” need not to be avoided, as it can be decided later. A perfect DUT will hit the decision coordinate (ne,ns) with ne=0. This test time is short.

## G.7.7	Standard test versus early decision concept

For Supplier Risk:

The wrong decision probability D in the standard test is the probability, to decide a DUT in-correct in the single decision point. In the early decision concept there is a probability of in-correct decisions d at each point of the limit-curve. The sum of all those wrong decision probabilities accumulate to D. Hence d<D

For Customer Risk:

The correct decision probability CL in the standard test is the probability, to decide a DUT correct in the single decision point. In the early decision concept there is a probability of correct decisions cl at each point of the limit-curve. The sum of all those correct decision probabilities accumulate to CL. Hence cl<CL or d>D

## G.7.8	Selectivity

There is no statistical test which can discriminate between a limit DUT and a DUT which is an (ε0) apart from the limit in finite time and high confidence level CL. Either the test discriminates against one limit with the results pass (with CL)/undecided or fail (with CL)/undecided, or the test ends in a result pass (with CL)/fail (with CL) but this requires a second limit.

For CL>1/2, a (measurement-result = specified-DUT-quality), generates undecided in test “supplier risk against pass limit” (a, from above) and also in the test “customer risk against the fail limit “ (aa)

For CL>1/2, a DUT, known to be on the limit, will be decided pass for the test “customer risk against pass limit” (b) and also “supplier risk against fail limit” (bb).

This overlap or undecided area is not a fault or a contradiction, however it can be avoided by introducing a Bad or a Good DUT quality according to:

-	Bad DUT quality: specified DUT-quality * M (M>1)

-	Good DUT quality: specified DUT-quality * m (m<1)

Using e.g. M>1 and CL=95% the test for different DUT qualities yield different pass probabilities:

Figure G.X.8-1: Pass probability versus DUT quality

## G.7.9	Design of the test

The receiver characteristic test are defined by the following design principles:

1.	The early decision concept is applied.

2.	A second limit is introduced: Bad DUT factor M>1

3.	To decide the test pass:

Supplier risk is applied based on the Bad DUT quality

To decide the test fail

Customer Risk is applied based on the specified DUT quality

The receiver characteristic test are defined by the following parameters:

1.	Limit ER = 0.05

2.	Bad DUT factor M=1.5 (selectivity)

3.	Confidence level CL = 95% (for specified DUT and Bad DUT-quality)

This has the following consequences:

1.	A measurement on the fail limit is connected with 2 equivalent statements:

| A measurement on the fail-limit shows, that the DUT is worse than the specified DUT-quality | A DUT, known have the specified quality, shall be measured and decided pass |
| --- | --- |

2.	A measurement on the pass limit is connected with the complementary statements:

| A measurement on the pass limit shows, that the DUT is better than the Bad DUT-quality. | A DUT, known to have the Bad DUT quality, shall be measured and decided fail |
| --- | --- |

The left column is used to decide the measurement.

The right column is used to verify the design of the test by simulation.

The simulation is based on the two fulcrums A and B only in Figure G.x.8-1

3.	Test time

The minimum and maximum test time is fixed.

The average test time is a function of the DUT’s quality.

The individual test time is not predictable.

4.	The number of decision co-ordinates (ne,ns) in the early decision concept is responsible for the selectivity of the test and the maximum test time. Having fixed the number of decision co-ordinates there is still freedom to select the individual decision co-ordinates in many combinations, all leading to the same confidence level.

## G.7.10	Simulation to derive the pass fail limits in Table G.2.4-1

There is freedom to design the decision co-ordinates (ne,ns).

The binomial distribution and its inverse is used to design the pass and fail limits. Note that this method is not unique and that other methods exist.

Where

-	fail(..) is the error ratio for the fail limit

-	pass(..) is the error ratio for the pass limit

-	ER is the specified error ratio 0.05

-	ne is the number of bad results. This is the variable in both equations

-	M is the Bad DUT factor M=1.5

-	df is the wrong decision probability of a single (ne,ns) co-ordinate for the fail limit. 
	It is found by simulation to be df = 0.006

-	clp is the confidence level of a single (ne,ns) co-ordinate for the pass limit.
It is found by simulation to be clp = 0.9945

-	qnbinom(..): The inverse cumulative function of the negative binomial distribution

The simulation works as follows:

-	A large population of limit DUTs with true ER = 0.05 is decided against the pass and fail limits.

-	clp    and df   are tuned such that CL (95%) of the population passes and D (5%) of the population fails.

-	A population of Bad DUTs with true ER = M*0.05 is decided against the same pass and fail limits.

-	clp    and df   are tuned such that CL (95%) of the population fails and D (5%) of the population passes.

-	This procedure and the relationship to the measurement is justified in clause G.x.9. The number of DUTs decrease during the simulation, as the decided DUTs leave the population. That number decreases with an approximately exponential characteristics. After 146 bad results all DUTs of the population are decided.

NOTE:	The exponential decrease of the population is an optimal design goal for the decision co-ordinates (ne,ns), which can be achieved with other formulas or methods as well.

<<End of change>>