3GPP TSG-RAN5 Meeting #106	R5-251134

# Athens, Greece, Feburary 17 – Feburary 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3257 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to general RB allocation for CA with non-contiguous allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are some mistakes in the general RB allocation configuration table for CA with non-contiguous allocation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating the general RB allocation for CA with non-contiguous allocation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The relevant test cases can’t be excuted correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.1A	General

Editor’s note:

Uplink RB allocations for intra-band UL CA with mixed numerology is FFS

The minimum requirements for band combinations including Band n41 also apply for the corresponding band combinations with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said band combinations with Band n90 are not listed in the tables below but are covered by this specification.

Uplink RB allocations for intra-band CA given in Table 6.1A-1a are used throughout this section, unless otherwise stated by the test case.

Table 6.1A-1a: Common uplink configuration for intra-band contiguous 2UL CA (contiguous RB allocation)

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | 5+15 | CP | 52 | 0@0 | 52@1 | 104 | 25@0 | 79@0 |
|  |  |  | DFT-s | 50 | 0@0 | 50@0 | 100 | 25@0 | 75@0 |
| 20 |  | 10+10 | CP | 52 | 26@26 | 26@0 | 104 | 52@0 | 52@0 |
|  |  |  | DFT-s | 50 | 25@27 | 25@0 | 100 | 50@2 | 50@0 |
|  |  | 15+5 | CP | 52 | 52@26 | 0@0 | 104 | 79@0 | 25@0 |
|  |  |  | DFT-s | 50 | 50@29 | 0@0 | 100 | 75@4 | 25@0 |
|  | 15 | 5+20 | CP | 654 | 0@0 | 65@7 | 131 | 25@0 | 106@0 |
|  |  |  | DFT-s | 64 | 0@0 | 64@7 | 125 | 25@0 | 100@0 |
|  |  | 10+15 | CP | 654 | 20@32 | 45@0 | 131 | 52@0 | 79@0 |
|  |  |  | DFT-s | 65 | 20@32 | 45@0 | 125 | 50@2 | 75@0 |
|  |  | 15+10 | CP | 654 | 47@32 | 18@0 | 131 | 79@0 | 52@0 |
|  |  |  | DFT-s | 65 | 45@34 | 20@0 | 125 | 75@4 | 50@0 |
|  |  | 20+5 | CP | 654 | 65@32 | 0@0 | 131 | 106@0 | 25@0 |
| 25 |  |  | DFT-s | 64 | 64@35 | 0@0 | 125 | 100@6 | 25@0 |
|  | 30 | 10+15 | CP | 604 | 9@15 | 21@0 | 124 | 24@0 | 38@0 |
|  |  |  | DFT-s | 58 | 9@15 | 20@0 | 120 | 24@0 | 36@0 |
|  |  | 15+10 | CP | 604 | 23@15 | 7@0 | 124 | 38@0 | 24@0 |
|  |  |  | DFT-s | 58 | 20@18 | 9@0 | 120 | 36@2 | 24@0 |
|  | 15 | 10+20 | CP | 79 | 13@39 | 66@0 | 158 | 52@0 | 106@0 |
|  |  |  | DFT-s | 76 | 12@40 | 64@0 | 150 | 50@2 | 100@0 |
|  |  | 15+15 | CP | 79 | 40@39 | 39@0 | 158 | 79@0 | 79@0 |
|  |  |  | DFT-s | 76 | 40@39 | 36@0 | 150 | 75@4 | 75@0 |
|  |  | 20+10 | CP | 79 | 67@39 | 12@0 | 158 | 106@0 | 52@0 |
|  |  |  | DFT-s | 76 | 64@42 | 12@0 | 150 | 100@6 | 50@0 |
|  | 30 | 10+20 | CP | 74 | 5@19 | 32@0 | 150 | 24@0 | 51@0 |
|  |  |  | DFT-s | 74 | 5@19 | 32@0 | 148 | 24@0 | 50@0 |
| 30 |  | 15+15 | CP | 76 | 19@19 | 19@0 | 152 | 38@0 | 38@0 |
|  |  |  | DFT-s | 72 | 20@18 | 16@0 | 144 | 36@2 | 36@0 |
|  |  | 20+10 | CP | 74 | 32@19 | 5@0 | 150 | 51@0 | 24@0 |
|  |  |  | DFT-s | 74 | 32@19 | 5@0 | 148 | 50@1 | 24@0 |
|  | 15 | 15+20 | CP | 93 | 33@46 | 60@0 | 185 | 79@0 | 106@0 |
|  |  |  | DFT-s | 92 | 32@47 | 60@0 | 175 | 75@4 | 100@0 |
|  |  | 20+15 | CP | 93 | 60@46 | 33@0 | 185 | 106@0 | 79@0 |
|  |  |  | DFT-s | 92 | 60@46 | 32@0 | 175 | 100@6 | 75@0 |
|  | 30 | 15+20 | CP | 88 | 16@22 | 28@0 | 178 | 38@0 | 51@0 |
|  |  |  | DFT-s | 86 | 16@22 | 27@0 | 172 | 36@2 | 50@0 |
| 35 |  | 20+15 | CP | 88 | 29@22 | 15@0 | 178 | 51@0 | 38@0 |
|  |  |  | DFT-s | 86 | 27@24 | 16@0 | 172 | 50@1 | 36@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | 10+30 | CP | 106 | 0@0 | 106@1 | 212 | 52@0 | 160@0 |
|  |  |  | DFT-s | 100 | 0@52 | 100@0 | 210 | 50@2 | 160@0 |
| 40 |  | 20+20 | CP | 106 | 53@53 | 53@0 | 212 | 106@0 | 106@0 |
|  |  |  | DFT-s | 104 | 54@52 | 50@0 | 200 | 100@6 | 100@0 |
|  |  | 30+10 | CP | 106 | 106@53 | 0@0 | 212 | 160@0 | 52@0 |
|  |  |  | DFT-s | 100 | 100@60 | 0@0 | 210 | 160@0 | 50@0 |
|  | 30 | 10+30 | CP | 1004 | 0@0 | 50@1 | 204 | 24@0 | 78@0 |
|  |  |  | DFT-s | 100 | 0@0 | 50@1 | 198 | 24@0 | 75@0 |
|  |  | 20+20 | CP | 1004 | 26@25 | 24@0 | 204 | 51@0 | 51@0 |
|  |  |  | DFT-s | 100 | 25@26 | 25@0 | 200 | 50@1 | 50@0 |
|  |  | 30+10 | CP | 1004 | 50@25 | 0@0 | 204 | 78@0 | 24@0 |
|  |  |  | DFT-s | 100 | 50@27 | 0@0 | 198 | 75@3 | 24@0 |
|  | 15 | 15+30 | CP | 1064 | 26@53 | 80@0 | 239 | 79@0 | 160@0 |
|  |  |  | DFT-s | 118 | 18@61 | 100@0 | 235 | 75@4 | 160@0 |
| 45 | 30 | 15+30 | CP | 116 | 9@29 | 49@0 | 232 | 38@0 | 78@0 |
|  |  |  | DFT-s | 114 | 9@29 | 48@0 | 222 | 36@2 | 75@0 |
|  | 15 | 10+40 | CP | 134 | 0@0 | 134@15 | 268 | 52@0 | 216@0 |
|  |  |  | DFT-s | 128 | 0@0 | 128@12 | 266 | 50@2 | 216@0 |
|  |  | 20+30 | CP | 133 | 40@66 | 93@0 | 266 | 106@0 | 160@0 |
|  |  |  | DFT-s | 130 | 40@66 | 90@0 | 260 | 100@6 | 160@0 |
|  |  | 30+20 | CP | 133 | 94@66 | 39@0 | 266 | 160@0 | 106@0 |
|  |  |  | DFT-s | 130 | 90@70 | 40@0 | 260 | 160@0 | 100@0 |
| 50 |  | 40+10 | CP | 134 | 134@67 | 0@0 | 268 | 216@0 | 52@0 |
|  |  |  | DFT-s | 128 | 128@76 | 0@0 | 266 | 216@0 | 50@0 |
|  | 30 | 10+40 | CP | 1284 | 0@0 | 64@8 | 260 | 24@0 | 106@0 |
|  |  |  | DFT-s | 128 | 0@0 | 64@8 | 248 | 24@0 | 100@0 |
|  |  | 20+30 | CP | 128 | 19@32 | 45@0 | 258 | 51@0 | 78@0 |
|  |  |  | DFT-s | 126 | 18@33 | 45@0 | 250 | 50@1 | 75@0 |
|  |  | 30+20 | CP | 128 | 46@32 | 18@0 | 258 | 78@0 | 51@0 |
|  |  |  | DFT-s | 126 | 45@33 | 18@0 | 250 | 75@3 | 50@0 |
|  |  | 40+10 | CP | 1284 | 64@32 | 0@0 | 260 | 106@0 | 24@0 |
|  |  |  | DFT-s | 128 | 64@34 | 0@0 | 248 | 100@6 | 24@0 |
|  | 15 | 15+40 | CP | 1474 | 6@73 | 141@0 | 295 | 79@0 | 216@0 |
|  |  |  | DFT-s | 143 | 8@71 | 135@0 | 291 | 75@4 | 216@0 |
| 55 | 30 | 15+40 | CP | 144 | 2@36 | 70@0 | 288 | 38@0 | 106@0 |
|  |  |  | DFT-s | 136 | 4@34 | 64@0 | 272 | 36@2 | 100@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | 10+50 | CP | 161 | 0@0 | 161@28 | 322 | 52@0 | 270@0 |
|  |  |  | DFT-s | 160 | 0@0 | 160@28 | 320 | 50@2 | 270@0 |
|  |  | 20+40 | CP | 161 | 26@80 | 135@0 | 322 | 106@0 | 216@0 |
|  |  |  | DFT-s | 160 | 25@81 | 135@0 | 316 | 100@6 | 216@0 |
|  |  | 30+30 | CP | 160 | 80@80 | 80@0 | 320 | 160@0 | 160@0 |
|  |  |  | DFT-s | 160 | 80@80 | 80@0 | 320 | 160@0 | 160@0 |
|  |  | 40+20 | CP | 161 | 136@80 | 25@0 | 322 | 216@0 | 106@0 |
|  |  |  | DFT-s | 160 | 135@81 | 25@0 | 316 | 216@0 | 100@0 |
|  |  | 50+10 | CP | 161 | 161@80 | 0@0 | 322 | 270@0 | 52@0 |
|  |  |  | DFT-s | 160 | 160@82 | 0@0 | 320 | 270@0 | 50@0 |
|  | 30 | 10+50 | CP | 156 | 0@0 | 78@15 | 314 | 24@0 | 133@0 |
|  |  |  | DFT-s | 150 | 0@0 | 75@14 | 304 | 24@0 | 128@0 |
| 60 |  | 20+40 | CP | 156 | 12@39 | 66@0 | 314 | 51@0 | 106@0 |
|  |  |  | DFT-s | 152 | 12@39 | 64@0 | 300 | 50@1 | 100@0 |
|  |  | 30+30 | CP | 156 | 39@39 | 39@0 | 312 | 78@0 | 78@0 |
|  |  |  | DFT-s | 152 | 40@38 | 36@0 | 300 | 75@3 | 75@0 |
|  |  | 40+20 | CP | 156 | 67@39 | 11@0 | 314 | 106@0 | 51@0 |
|  |  |  | DFT-s | 152 | 64@42 | 12@0 | 300 | 100@6 | 50@0 |
|  |  | 50+10 | CP | 156 | 78@39 | 0@0 | 314 | 133@0 | 24@0 |
|  |  |  | DFT-s | 150 | 75@44 | 0@0 | 304 | 128@5 | 24@0 |
|  | 15 | 20+50 | CP | 188 | 12@94 | 176@0 | 376 | 106@0 | 270@0 |
|  |  |  | DFT-s | 181 | 1@105 | 180@0 | 370 | 100@6 | 270@0 |
|  |  | 30+40 | CP | 188 | 66@94 | 122@0 | 376 | 160@0 | 216@0 |
|  |  |  | DFT-s | 184 | 64@96 | 120@0 | 376 | 160@0 | 216@0 |
|  |  | 40+30 | CP | 188 | 122@94 | 66@0 | 376 | 216@0 | 160@0 |
|  |  |  | DFT-s | 184 | 120@96 | 64@0 | 376 | 216@0 | 160@0 |
|  |  | 50+20 | CP | 188 | 176@94 | 12@0 | 376 | 270@0 | 106@0 |
|  |  |  | DFT-s | 181 | 180@90 | 1@0 | 370 | 270@0 | 100@0 |
|  | 30 | 20+50 | CP | 184 | 5@46 | 87@0 | 368 | 51@0 | 133@0 |
|  |  |  | DFT-s | 174 | 6@45 | 81@0 | 356 | 50@1 | 128@0 |
|  |  | 30+40 | CP | 184 | 32@46 | 60@0 | 368 | 78@0 | 106@0 |
| 70 |  |  | DFT-s | 184 | 32@46 | 60@0 | 350 | 75@3 | 100@0 |
|  |  | 40+30 | CP | 184 | 60@46 | 32@0 | 368 | 106@0 | 78@0 |
|  |  |  | DFT-s | 184 | 60@46 | 32@0 | 350 | 100@6 | 75@0 |
|  |  | 50+20 | CP | 184 | 87@46 | 5@0 | 368 | 133@0 | 51@0 |
|  |  |  | DFT-s | 174 | 81@52 | 6@0 | 356 | 128@5 | 50@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | 30+50 | CP | 215 | 53@107 | 162@0 | 430 | 160@0 | 270@0 |
|  |  |  | DFT-s | 210 | 50@110 | 160@0 | 430 | 160@0 | 270@0 |
|  |  | 40+40 | CP | 216 | 108@108 | 108@0 | 432 | 216@0 | 216@0 |
|  |  |  | DFT-s | 216 | 108@108 | 108@0 | 432 | 216@0 | 216@0 |
|  |  | 50+30 | CP | 215 | 163@107 | 52@0 | 430 | 270@0 | 160@0 |
|  |  |  | DFT-s | 210 | 162@108 | 48@0 | 430 | 270@0 | 160@0 |
|  | 30 | 30+50 | CP | 210 | 25@53 | 80@0 | 422 | 78@0 | 133@0 |
| 80 |  |  | DFT-s | 210 | 25@53 | 80@0 | 406 | 75@3 | 128@0 |
|  |  | 40+40 | CP | 212 | 53@53 | 53@0 | 424 | 106@0 | 106@0 |
|  |  |  | DFT-s | 208 | 54@52 | 50@0 | 400 | 100@6 | 100@0 |
|  |  | 50+30 | CP | 210 | 80@53 | 25@0 | 422 | 133@0 | 78@0 |
|  |  |  | DFT-s | 210 | 80@53 | 25@0 | 406 | 128@5 | 75@0 |
|  | 15 | 40+50 | CP | 243 | 95@121 | 148@0 | 486 | 216@0 | 270@0 |
| 90 |  |  | DFT-s | 240 | 96@120 | 144@0 | 486 | 216@0 | 270@0 |
|  |  | 50+40 | CP | 243 | 149@121 | 94@0 | 486 | 270@0 | 216@0 |
|  |  |  | DFT-s | 240 | 150@120 | 90@0 | 486 | 270@0 | 216@0 |
|  | 30 | 40+50 | CP | 238 | 46@60 | 73@0 | 478 | 106@0 | 133@0 |
|  |  |  | DFT-s | 234 | 45@61 | 72@0 | 456 | 100@6 | 128@0 |
|  |  | 50+40 | CP | 238 | 73@60 | 46@0 | 478 | 133@0 | 106@0 |
|  |  |  | DFT-s | 234 | 72@61 | 45@0 | 456 | 128@5 | 100@0 |
|  | 15 | 50+50 | CP | 270 | 135@135 | 135@0 | 540 | 270@0 | 270@0 |
|  |  |  | DFT-s | 270 | 135@135 | 135@0 | 540 | 270@0 | 270@0 |
| 100 | 30 | 50+50 | CP | 2644 | 67@66 | 65@0 | 532 | 133@0 | 133@0 |
|  |  |  | DFT-s | 256 | 64@69 | 64@0 | 512 | 128@5 | 128@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 15+90 | CP | 282 | 0@0 | 141@33 | 566 | 38@0 | 245@0 |
|  |  |  | DFT-s | 270 | 0@0 | 135@30 | 558 | 36@2 | 243@0 |
| 105 |  | 25+80 | CP | 2804 | 0@0 | 140@5 | 564 | 65@0 | 217@0 |
|  |  |  | DFT-s | 270 | 0@0 | 135@3 | 560 | 64@1 | 216@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 10+100 | CP | 296 | 0@0 | 148@50 | 594 | 24@0 | 273@0 |
|  |  |  | DFT-s | 288 | 0@0 | 144@48 | 588 | 24@0 | 270@0 |
|  |  | 20+90 | CP | 296 | 0@0 | 148@23 | 592 | 51@0 | 245@0 |
|  |  |  | DFT-s | 288 | 0@0 | 144@21 | 586 | 50@1 | 243@0 |
|  |  | 30+80 | CP | 294 | 4@74 | 143@0 | 590 | 78@0 | 217@0 |
|  |  |  | DFT-s | 294 | 3@74 | 144@0 | 582 | 75@3 | 216@0 |
|  |  | 40+70 | CP | 294 | 32@74 | 115@0 | 590 | 106@0 | 189@0 |
|  |  |  | DFT-s | 280 | 32@74 | 108@0 | 560 | 100@6 | 180@0 |
|  |  | 50+60 | CP | 294 | 59@74 | 88@0 | 590 | 133@0 | 162@0 |
|  |  |  | DFT-s | 282 | 60@73 | 81@0 | 580 | 128@5 | 162@0 |
|  |  | 60+50 | CP | 294 | 88@74 | 59@0 | 590 | 162@0 | 133@0 |
|  |  |  | DFT-s | 282 | 81@81 | 60@0 | 580 | 162@0 | 128@0 |
|  |  | 70+40 | CP | 294 | 115@74 | 32@0 | 590 | 189@0 | 106@0 |
|  |  |  | DFT-s | 294 | 108@74 | 32@0 | 560 | 180@9 | 100@0 |
|  |  | 80+30 | CP | 294 | 143@74 | 4@0 | 590 | 217@0 | 78@0 |
|  |  |  | DFT-s | 292 | 144@73 | 2@0 | 582 | 216@1 | 75@0 |
| 110 |  | 90+20 | CP | 296 | 148@74 | 0@0 | 592 | 245@0 | 51@0 |
|  |  |  | DFT-s | 288 | 144@80 | 0@0 | 586 | 243@2 | 50@0 |
|  |  | 100+10 | CP | 296 | 148@74 | 0@0 | 594 | 273@0 | 24@0 |
|  |  |  | DFT-s | 288 | 144@81 | 0@0 | 588 | 270@3 | 24@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 15+100 | CP | 310 | 0@0 | 155@40 | 622 | 38@0 | 273@0 |
|  |  |  | DFT-s | 300 | 0@0 | 150@37 | 612 | 36@2 | 270@0 |
|  |  | 25+90 | CP | 3084 | 0@0 | 154@12 | 620 | 65@0 | 245@0 |
|  |  |  | DFT-s | 300 | 0@0 | 150@10 | 614 | 64@1 | 243@0 |
|  |  | 90+25 | CP | 3084 | 154@77 | 0@0 | 620 | 245@0 | 65@0 |
| 115 |  |  | DFT-s | 300 | 150@85 | 0@0 | 614 | 243@2 | 64@0 |
|  |  | 100+15 | CP | 310 | 155@78 | 0@0 | 622 | 273@0 | 38@0 |
|  |  |  | DFT-s | 300 | 150@86 | 0@0 | 612 | 270@3 | 36@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 20+100 | CP | 324 | 0@0 | 162@30 | 648 | 51@0 | 273@0 |
|  |  |  | DFT-s | 324 | 0@0 | 162@30 | 640 | 50@1 | 270@0 |
|  |  | 30+90 | CP | 322 | 0@0 | 161@3 | 646 | 78@0 | 245@0 |
|  |  |  | DFT-s | 320 | 0@0 | 160@2 | 636 | 75@3 | 243@0 |
|  |  | 40+80 | CP | 322 | 25@81 | 136@0 | 646 | 106@0 | 217@0 |
|  |  |  | DFT-s | 320 | 25@81 | 135@0 | 632 | 100@6 | 216@0 |
|  |  | 50+70 | CP | 3204 | 53@80 | 107@0 | 644 | 133@0 | 189@0 |
|  |  |  | DFT-s | 320 | 50@81 | 108@0 | 616 | 128@5 | 180@0 |
|  |  | 60+60 | CP | 324 | 81@81 | 81@0 | 648 | 162@0 | 162@0 |
|  |  |  | DFT-s | 324 | 81@81 | 81@0 | 648 | 162@0 | 162@0 |
|  |  | 70+50 | CP | 3204 | 109@80 | 51@0 | 644 | 189@0 | 133@0 |
|  |  |  | DFT-s | 316 | 108@81 | 50@0 | 616 | 180@9 | 128@0 |
|  |  | 80+40 | CP | 322 | 136@81 | 25@0 | 646 | 217@0 | 106@0 |
|  |  |  | DFT-s | 318 | 135@82 | 24@0 | 632 | 216@1 | 100@0 |
|  |  | 90+30 | CP | 322 | 161@81 | 0@0 | 646 | 245@0 | 78@0 |
|  |  |  | DFT-s | 320 | 160@83 | 0@0 | 636 | 243@2 | 75@0 |
| 120 |  | 100+20 | CP | 324 | 162@81 | 0@0 | 648 | 273@0 | 51@0 |
|  |  |  | DFT-s | 324 | 162@81 | 0@0 | 640 | 270@3 | 50@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 25+100 | CP | 3364 | 0@0 | 168@19 | 676 | 65@0 | 273@0 |
|  |  |  | DFT-s | 324 | 0@0 | 162@16 | 668 | 64@1 | 270@0 |
|  |  | 100+25 | CP | 3364 | 168@84 | 0@0 | 676 | 273@0 | 65@0 |
| 125 |  |  | DFT-s | 324 | 162@95 | 0@0 | 668 | 270@3 | 64@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 30+100 | CP | 350 | 0@0 | 175@10 | 702 | 78@0 | 273@0 |
|  |  |  | DFT-s | 324 | 0@0 | 162@3 | 690 | 75@3 | 270@0 |
|  |  | 40+90 | CP | 350 | 18@88 | 157@0 | 702 | 106@0 | 245@0 |
|  |  |  | DFT-s | 340 | 20@86 | 150@0 | 686 | 100@6 | 243@0 |
|  |  | 50+80 | CP | 3484 | 46@87 | 128@0 | 700 | 133@0 | 217@0 |
|  |  |  | DFT-s | 346 | 45@88 | 128@0 | 688 | 128@5 | 216@0 |
|  |  | 60+70 | CP | 350 | 74@88 | 101@0 | 702 | 162@0 | 189@0 |
|  |  |  | DFT-s | 348 | 72@88 | 100@0 | 684 | 162@0 | 180@0 |
|  |  | 70+60 | CP | 350 | 101@88 | 74@0 | 702 | 189@0 | 162@0 |
|  |  |  | DFT-s | 346 | 100@88 | 72@0 | 684 | 180@9 | 162@0 |
|  |  | 80+50 | CP | 3484 | 130@87 | 44@0 | 700 | 217@0 | 133@0 |
|  |  |  | DFT-s | 346 | 128@89 | 45@0 | 688 | 216@1 | 128@0 |
| 130 |  | 90+40 | CP | 350 | 157@88 | 18@0 | 702 | 245@0 | 106@0 |
|  |  |  | DFT-s | 340 | 160@85 | 10@0 | 686 | 243@2 | 100@0 |
|  |  | 100+30 | CP | 350 | 175@88 | 0@0 | 702 | 273@0 | 78@0 |
|  |  |  | DFT-s | 324 | 162@108 | 0@0 | 690 | 270@3 | 75@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 40+100 | CP | 378 | 11@95 | 178@0 | 758 | 106@0 | 273@0 |
|  |  |  | DFT-s | 372 | 6@100 | 180@0 | 740 | 100@6 | 270@0 |
|  |  | 50+90 | CP | 3764 | 39@94 | 149@0 | 756 | 133@0 | 245@0 |
|  |  |  | DFT-s | 372 | 36@97 | 150@0 | 742 | 128@5 | 243@0 |
|  |  | 60+80 | CP | 378 | 67@95 | 122@0 | 758 | 162@0 | 217@0 |
|  |  |  | DFT-s | 368 | 64@98 | 120@0 | 756 | 162@0 | 216@0 |
|  |  | 70+70 | CP | 3764 | 95@94 | 93@0 | 756 | 189@0 | 189@0 |
|  |  |  | DFT-s | 368 | 90@95 | 90@0 | 720 | 180@9 | 180@0 |
|  |  | 80+60 | CP | 378 | 122@95 | 67@0 | 758 | 217@0 | 162@0 |
|  |  |  | DFT-s | 368 | 120@97 | 64@0 | 756 | 216@1 | 162@0 |
|  |  | 90+50 | CP | 3764 | 151@94 | 37@0 | 756 | 245@0 | 133@0 |
| 140 |  |  | DFT-s | 372 | 150@95 | 36@0 | 742 | 243@2 | 128@0 |
|  |  | 100+40 | CP | 378 | 178@95 | 11@0 | 758 | 273@0 | 106@0 |
|  |  |  | DFT-s | 372 | 180@93 | 6@0 | 740 | 270@3 | 100@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 50+100 | CP | 4044 | 32@101 | 170@0 | 812 | 133@0 | 273@0 |
|  |  |  | DFT-s | 388 | 32@101 | 162@0 | 796 | 128@5 | 270@0 |
|  |  | 60+90 | CP | 406 | 60@102 | 143@0 | 814 | 162@0 | 245@0 |
|  |  |  | DFT-s | 396 | 54@108 | 144@0 | 810 | 162@0 | 243@0 |
|  |  | 70+80 | CP | 4044 | 88@101 | 114@0 | 812 | 189@0 | 217@0 |
|  |  |  | DFT-s | 374 | 81@102 | 100@0 | 792 | 180@9 | 216@0 |
|  |  | 80+70 | CP | 4044 | 116@101 | 86@0 | 812 | 217@0 | 189@0 |
|  |  |  | DFT-s | 392 | 100@102 | 81@0 | 792 | 216@1 | 180@0 |
|  |  | 90+60 | CP | 406 | 143@102 | 60@0 | 814 | 245@0 | 162@0 |
|  |  |  | DFT-s | 396 | 144@101 | 54@0 | 810 | 243@2 | 162@0 |
|  |  | 100+50 | CP | 4044 | 172@101 | 30@0 | 812 | 273@0 | 133@0 |
| 150 |  |  | DFT-s | 388 | 162@111 | 32@0 | 796 | 270@3 | 128@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 60+100 | CP | 434 | 53@109 | 164@0 | 870 | 162@0 | 273@0 |
|  |  |  | DFT-s | 432 | 54@108 | 162@0 | 864 | 162@0 | 270@0 |
|  |  | 70+90 | CP | 4324 | 81@108 | 135@0 | 868 | 189@0 | 245@0 |
|  |  |  | DFT-s | 430 | 80@109 | 135@0 | 846 | 180@9 | 243@0 |
|  |  | 80+80 | CP | 4324 | 109@108 | 107@0 | 864 | 216@1 | 216@0 |
|  |  |  | DFT-s | 432 | 108@109 | 108@0 | 864 | 216@1 | 216@0 |
|  |  | 90+70 | CP | 4324 | 137@108 | 79@0 | 868 | 245@0 | 189@0 |
|  |  |  | DFT-s | 432 | 135@109 | 80@0 | 846 | 243@2 | 180@0 |
| 160 |  | 100+60 | CP | 434 | 164@109 | 53@0 | 870 | 273@0 | 162@0 |
|  |  |  | DFT-s | 432 | 162@111 | 54@0 | 864 | 270@3 | 162@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 70+100 | CP | 4604 | 74@115 | 156@0 | 924 | 189@0 | 273@0 |
|  |  |  | DFT-s | 446 | 72@116 | 150@0 | 900 | 180@9 | 270@0 |
|  |  | 80+90 | CP | 4604 | 102@115 | 128@0 | 924 | 217@0 | 245@0 |
|  |  |  | DFT-s | 456 | 100@117 | 128@0 | 918 | 216@1 | 243@0 |
|  |  | 90+80 | CP | 4604 | 130@115 | 100@0 | 924 | 245@0 | 217@0 |
|  |  |  | DFT-s | 456 | 128@117 | 100@0 | 918 | 243@2 | 216@0 |
|  |  | 100+70 | CP | 4604 | 158@115 | 72@0 | 924 | 273@0 | 189@0 |
| 170 |  |  | DFT-s | 458 | 150@116 | 72@0 | 900 | 270@3 | 180@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 80+100 | CP | 4884 | 95@122 | 149@0 | 980 | 217@0 | 273@0 |
|  |  |  | DFT-s | 480 | 96@121 | 144@0 | 972 | 216@1 | 270@0 |
|  |  | 90+90 | CP | 4884 | 123@122 | 121@0 | 980 | 245@0 | 245@0 |
|  |  |  | DFT-s | 480 | 120@125 | 120@0 | 972 | 243@2 | 243@0 |
|  |  | 100+80 | CP | 4884 | 151@122 | 93@0 | 980 | 273@0 | 217@0 |
| 180 |  |  | DFT-s | 480 | 150@123 | 90@0 | 972 | 270@3 | 216@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 90+100 | CP | 516 | 116@129 | 142@0 | 1036 | 245@0 | 273@0 |
|  |  |  | DFT-s | 504 | 108@137 | 144@0 | 1026 | 243@2 | 270@0 |
|  |  | 100+90 | CP | 5164 | 144@129 | 114@0 | 1036 | 273@0 | 245@0 |
| 190 |  |  | DFT-s | 504 | 144@129 | 108@0 | 1026 | 270@3 | 243@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 100+100 | CP | 544 | 137@136 | 135@0 | 1092 | 273@0 | 273@0 |
| 200 |  |  | DFT-s | 540 | 135@138 | 135@0 | 1080 | 270@3 | 270@0 |
| NOTE 1: BWchannel_CA is defined in clause 5.3A.3 NOTE 2: NRB_alloc for intra-band contiguous UL CA is defined in 6.2A.2.0 NOTE 3: Void NOTE 4: The allocated RB number NRB_alloc is ceil(NRB_agg/2) -1∙2^μ in order to meet Inner RB allocation definition (RBStart,Low ≤ RBStart ≤ RBStart,High) described in subclause 6.2A.2.0. |  |  |  |  |  |  |  |  |  |

Table 6.1A-1b: Common uplink configuration for intra-band contiguous 2UL CA (non-contiguous RB allocation) for NR operating bands with 100 kHz channel raster

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | RB allocation (Inner) |  |  | RB allocation (Outer1) |  |  | RB allocation (Outer2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
| 20 | 15 | 5+15 | 25 | 1@24 | 1@23 | 64 | 1@20 | 1@58 | 104 | 1@0 | 1@78 |
|  |  | 10+10 | 33 | 1@34 | 1@14 | 62 | 1@19 | 1@28 | 104 | 1@0 | 1@51 |
|  |  | 15+5 | 24 | 1@56 | 1@0 | 64 | 1@20 | 1@4 | 104 | 1@0 | 1@24 |
|  | 30 | 10+10 | 28 | 1@16 | 1@5 | 58 | 1@9 | 1@13 | 96 | 1@0 | 1@23 |
| 25 | 15 | 5+20 | 26 | 1@24 | 1@24 | 81 | 1@24 | 1@79 | 131 | 1@0 | 1@105 |
|  |  | 10+15 | 43 | 1@44 | 1@34 | 79 | 1@25 | 1@51 | 131 | 1@0 | 1@78 |
|  |  | 15+10 | 43 | 1@43 | 1@6 | 79 | 1@24 | 1@23 | 131 | 1@0 | 1@51 |
|  |  | 20+5 | 25 | 1@82 | 1@0 | 81 | 1@26 | 1@0 | 131 | 1@0 | 1@24 |
|  | 30 | 10+15 | 38 | 1@20 | 1@14 | 76 | 1@12 | 1@25 | 124 | 1@0 | 1@37 |
|  |  | 15+10 | 38 | 1@20 | 1@0 | 76 | 1@12 | 1@11 | 124 | 1@0 | 1@23 |
| 30 | 15 | 5+25 | 25 | 1@24 | 1@23 | 93 | 1@23 | 1@90 | 158 | 1@0 | 1@132 |
|  |  | 10+20 | 54 | 1@51 | 1@52 | 97 | 1@28 | 1@72 | 158 | 1@0 | 1@105 |
|  |  | 15+15 | 51 | 1@53 | 1@24 | 95 | 1@31 | 1@46 | 158 | 1@0 | 1@78 |
|  |  | 20+10 | 53 | 1@54 | 1@0 | 97 | 1@29 | 1@19 | 158 | 1@0 | 1@51 |
|  |  | 25+5 | 24 | 1@110 | 1@0 | 93 | 1@41 | 1@0 | 158 | 1@0 | 1@24 |
|  | 30 | 10+20 | 46 | 1@23 | 1@21 | 92 | 1@13 | 1@34 | 150 | 1@0 | 1@50 |
|  |  | 15+15 | 48 | 1@26 | 1@11 | 92 | 1@15 | 1@22 | 152 | 1@0 | 1@37 |
|  |  | 20+10 | 46 | 1@29 | 1@0 | 92 | 1@13 | 1@7 | 150 | 1@0 | 1@23 |
| 35 | 15 | 5+30 | 27 | 1@24 | 1@25 | 109 | 1@24 | 1@107 | 185 | 1@0 | 1@159 |
|  |  | 10+25 | 52 | 1@51 | 1@50 | 113 | 1@35 | 1@95 | 185 | 1@0 | 1@132 |
|  |  | 15+20 | 62 | 1@60 | 1@42 | 113 | 1@34 | 1@67 | 185 | 1@0 | 1@105 |
|  |  | 20+15 | 62 | 1@61 | 1@16 | 113 | 1@35 | 1@41 | 185 | 1@0 | 1@78 |
|  |  | 25+10 | 51 | 1@83 | 1@0 | 113 | 1@35 | 1@14 | 185 | 1@0 | 1@51 |
|  |  | 30+5 | 26 | 1@135 | 1@0 | 109 | 1@52 | 1@0 | 185 | 1@0 | 1@24 |
|  | 30 | 10+25 | 46 | 1@23 | 1@21 | 108 | 1@16 | 1@45 | 178 | 1@0 | 1@64 |
|  |  | 15+20 | 58 | 1@29 | 1@19 | 110 | 1@17 | 1@33 | 178 | 1@0 | 1@50 |
|  |  | 20+15 | 58 | 1@30 | 1@7 | 110 | 1@17 | 1@20 | 178 | 1@0 | 1@37 |
|  |  | 25+10 | 44 | 1@44 | 1@0 | 108 | 1@16 | 1@4 | 178 | 1@0 | 1@23 |
| 40 | 15 | 10+30 | 52 | 1@51 | 1@50 | 130 | 1@41 | 1@118 | 212 | 1@0 | 1@159 |
|  |  | 15+25 | 71 | 1@69 | 1@60 | 130 | 1@40 | 1@90 | 212 | 1@0 | 1@132 |
|  |  | 20+20 | 69 | 1@70 | 1@32 | 128 | 1@41 | 1@62 | 212 | 1@0 | 1@105 |
|  |  | 25+15 | 71 | 1@70 | 1@7 | 130 | 1@41 | 1@37 | 212 | 1@0 | 1@78 |
|  |  | 30+10 | 51 | 1@110 | 1@0 | 130 | 1@41 | 1@10 | 212 | 1@0 | 1@51 |
|  | 30 | 10+30 | 44 | 1@23 | 1@20 | 124 | 1@18 | 1@55 | 204 | 1@0 | 1@77 |
|  |  | 15+25 | 68 | 1@34 | 1@29 | 126 | 1@19 | 1@43 | 206 | 1@0 | 1@64 |
|  |  | 20+20 | 64 | 1@34 | 1@14 | 124 | 1@20 | 1@30 | 204 | 1@0 | 1@50 |
|  |  | 25+15 | 68 | 1@34 | 1@2 | 126 | 1@20 | 1@17 | 206 | 1@0 | 1@37 |
|  |  | 30+10 | 44 | 1@57 | 1@0 | 124 | 1@18 | 1@1 | 204 | 1@0 | 1@23 |
| 45 | 15 | 5+40 | 27 | 1@24 | 1@25 | 136 | 1@23 | 1@133 | 241 | 1@0 | 1@215 |
|  |  | 15+30 | 78 | 1@78 | 1@76 | 145 | 1@45 | 1@110 | 239 | 1@0 | 1@159 |
|  |  | 20+25 | 78 | 1@79 | 1@50 | 144 | 1@46 | 1@83 | 239 | 1@0 | 1@132 |
|  |  | 25+20 | 79 | 1@79 | 1@24 | 144 | 1@45 | 1@55 | 239 | 1@0 | 1@105 |
|  |  | 30+15 | 77 | 1@84 | 1@0 | 145 | 1@45 | 1@29 | 239 | 1@0 | 1@78 |
|  |  | 40+5 | 26 | 1@191 | 1@0 | 136 | 1@81 | 1@0 | 241 | 1@0 | 1@24 |
|  | 30 | 15+30 | 72 | 1@37 | 1@34 | 142 | 1@22 | 1@54 | 232 | 1@0 | 1@77 |
|  |  | 20+25 | 74 | 1@38 | 1@23 | 140 | 1@22 | 1@40 | 232 | 1@0 | 1@64 |
|  |  | 25+20 | 74 | 1@38 | 1@9 | 140 | 1@22 | 1@26 | 232 | 1@0 | 1@50 |
|  |  | 30+15 | 70 | 1@44 | 1@0 | 142 | 1@22 | 1@14 | 232 | 1@0 | 1@37 |
| 50 | 15 | 10+40 | 56 | 1@51 | 1@54 | 164 | 1@49 | 1@160 | 268 | 1@0 | 1@215 |
|  |  | 20+30 | 87 | 1@89 | 1@69 | 160 | 1@52 | 1@105 | 266 | 1@0 | 1@159 |
|  |  | 25+25 | 87 | 1@89 | 1@42 | 160 | 1@51 | 1@77 | 266 | 1@0 | 1@132 |
|  |  | 30+20 | 87 | 1@88 | 1@14 | 160 | 1@51 | 1@50 | 266 | 1@0 | 1@105 |
|  |  | 40+10 | 54 | 1@163 | 1@0 | 164 | 1@53 | 1@0 | 268 | 1@0 | 1@51 |
|  | 30 | 10+40 | 48 | 1@23 | 1@22 | 158 | 1@22 | 1@76 | 260 | 1@0 | 1@105 |
|  |  | 20+30 | 82 | 1@43 | 1@32 | 156 | 1@25 | 1@51 | 258 | 1@0 | 1@77 |
|  |  | 25+25 | 82 | 1@43 | 1@18 | 156 | 1@25 | 1@37 | 260 | 1@0 | 1@64 |
|  |  | 30+20 | 82 | 1@43 | 1@5 | 156 | 1@25 | 1@24 | 258 | 1@0 | 1@50 |
|  |  | 40+10 | 46 | 1@84 | 1@0 | 158 | 1@28 | 1@0 | 260 | 1@0 | 1@23 |
| 55 | 15 | 5+50 | 28 | 1@24 | 1@26 | 165 | 1@24 | 1@163 | 295 | 1@0 | 1@269 |
|  |  | 15+40 | 83 | 1@78 | 1@81 | 181 | 1@54 | 1@155 | 295 | 1@0 | 1@215 |
|  |  | 25+30 | 96 | 1@98 | 1@60 | 177 | 1@58 | 1@101 | 293 | 1@0 | 1@159 |
|  |  | 30+25 | 96 | 1@97 | 1@32 | 177 | 1@57 | 1@73 | 293 | 1@0 | 1@132 |
|  |  | 40+15 | 82 | 1@135 | 1@0 | 181 | 1@57 | 1@21 | 295 | 1@0 | 1@78 |
|  |  | 50+5 | 27 | 1@244 | 1@0 | 165 | 1@106 | 1@0 | 295 | 1@0 | 1@24 |
|  | 30 | 15+40 | 76 | 1@37 | 1@36 | 178 | 1@27 | 1@77 | 288 | 1@0 | 1@105 |
|  |  | 25+30 | 92 | 1@48 | 1@28 | 172 | 1@27 | 1@47 | 286 | 1@0 | 1@77 |
|  |  | 30+25 | 92 | 1@48 | 1@15 | 172 | 1@27 | 1@34 | 286 | 1@0 | 1@64 |
|  |  | 40+15 | 76 | 1@69 | 1@0 | 178 | 1@28 | 1@10 | 288 | 1@0 | 1@37 |
| 60 | 15 | 10+50 | 54 | 1@51 | 1@52 | 191 | 1@50 | 1@188 | 322 | 1@0 | 1@269 |
|  |  | 20+40 | 106 | 1@105 | 1@104 | 195 | 1@62 | 1@150 | 322 | 1@0 | 1@215 |
|  |  | 30+30 | 104 | 1@107 | 1@50 | 192 | 1@62 | 1@93 | 320 | 1@0 | 1@159 |
|  |  | 40+20 | 104 | 1@113 | 1@0 | 195 | 1@63 | 1@41 | 322 | 1@0 | 1@105 |
|  |  | 50+10 | 53 | 1@218 | 1@0 | 191 | 1@80 | 1@0 | 322 | 1@0 | 1@51 |
|  | 30 | 10+50 | 46 | 1@23 | 1@21 | 186 | 1@23 | 1@91 | 314 | 1@0 | 1@132 |
|  |  | 20+40 | 98 | 1@50 | 1@47 | 190 | 1@30 | 1@73 | 314 | 1@0 | 1@105 |
|  |  | 30+30 | 100 | 1@53 | 1@24 | 188 | 1@30 | 1@45 | 312 | 1@0 | 1@77 |
|  |  | 40+20 | 96 | 1@59 | 1@0 | 190 | 1@30 | 1@18 | 314 | 1@0 | 1@50 |
|  |  | 50+10 | 46 | 1@111 | 1@0 | 186 | 1@41 | 1@0 | 314 | 1@0 | 1@23 |
| 65 | 15 | 15+50 | 82 | 1@78 | 1@80 | 213 | 1@65 | 1@198 | 349 | 1@0 | 1@269 |
|  |  | 25+40 | 116 | 1@115 | 1@97 | 212 | 1@67 | 1@145 | 349 | 1@0 | 1@215 |
|  |  | 40+25 | 116 | 1@116 | 1@15 | 212 | 1@69 | 1@64 | 349 | 1@0 | 1@132 |
|  |  | 50+15 | 80 | 1@191 | 1@0 | 213 | 1@67 | 1@9 | 349 | 1@0 | 1@78 |
|  | 30 | 15+50 | 76 | 1@37 | 1@36 | 210 | 1@33 | 1@99 | 342 | 1@0 | 1@132 |
|  |  | 25+40 | 112 | 1@57 | 1@47 | 208 | 1@33 | 1@71 | 342 | 1@0 | 1@105 |
|  |  | 40+25 | 112 | 1@57 | 1@6 | 208 | 1@33 | 1@30 | 342 | 1@0 | 1@64 |
|  |  | 50+15 | 74 | 1@97 | 1@0 | 210 | 1@33 | 1@4 | 342 | 1@0 | 1@37 |
| 70 | 15 | 20+50 | 106 | 1@105 | 1@104 | 228 | 1@73 | 1@194 | 376 | 1@0 | 1@269 |
|  |  | 30+40 | 125 | 1@123 | 1@87 | 228 | 1@71 | 1@138 | 376 | 1@0 | 1@215 |
|  |  | 40+30 | 125 | 1@125 | 1@33 | 228 | 1@73 | 1@84 | 376 | 1@0 | 1@159 |
|  |  | 50+20 | 104 | 1@167 | 1@0 | 228 | 1@73 | 1@30 | 376 | 1@0 | 1@105 |
|  | 30 | 10+60 | 48 | 1@23 | 1@22 | 214 | 1@22 | 1@104 | 372 | 1@0 | 1@161 |
|  |  | 20+50 | 98 | 1@50 | 1@47 | 224 | 1@36 | 1@96 | 368 | 1@0 | 1@132 |
|  |  | 30+40 | 120 | 1@60 | 1@41 | 224 | 1@35 | 1@68 | 368 | 1@0 | 1@105 |
|  |  | 40+30 | 120 | 1@61 | 1@14 | 224 | 1@36 | 1@41 | 368 | 1@0 | 1@77 |
|  |  | 50+20 | 96 | 1@86 | 1@0 | 224 | 1@36 | 1@14 | 368 | 1@0 | 1@50 |
|  |  | 60+10 | 48 | 1@139 | 1@0 | 214 | 1@56 | 1@0 | 372 | 1@0 | 1@23 |
| 75 | 15 | 25+50 | 132 | 1@132 | 1@130 | 244 | 1@78 | 1@188 | 403 | 1@0 | 1@269 |
|  |  | 50+25 | 130 | 1@141 | 1@0 | 244 | 1@79 | 1@52 | 403 | 1@0 | 1@132 |
|  | 30 | 15+60 | 76 | 1@37 | 1@36 | 242 | 1@37 | 1@119 | 400 | 1@0 | 1@161 |
|  |  | 25+50 | 126 | 1@64 | 1@61 | 240 | 1@38 | 1@92 | 396 | 1@0 | 1@132 |
|  |  | 50+25 | 124 | 1@72 | 1@0 | 240 | 1@38 | 1@24 | 396 | 1@0 | 1@64 |
|  |  | 60+15 | 74 | 1@126 | 1@0 | 242 | 1@42 | 1@0 | 400 | 1@0 | 1@37 |
| 80 | 15 | 30+50 | 142 | 1@142 | 1@123 | 261 | 1@84 | 1@184 | 430 | 1@0 | 1@269 |
|  |  | 40+40 | 142 | 1@144 | 1@69 | 260 | 1@85 | 1@128 | 432 | 1@0 | 1@215 |
|  |  | 50+30 | 142 | 1@143 | 1@14 | 261 | 1@85 | 1@75 | 430 | 1@0 | 1@159 |
|  | 30 | 10+70 | 48 | 1@23 | 1@22 | 242 | 1@23 | 1@119 | 426 | 1@0 | 1@188 |
|  |  | 20+60 | 98 | 1@50 | 1@47 | 258 | 1@41 | 1@118 | 426 | 1@0 | 1@161 |
|  |  | 30+50 | 138 | 1@70 | 1@60 | 256 | 1@41 | 1@90 | 422 | 1@0 | 1@132 |
|  |  | 40+40 | 138 | 1@71 | 1@33 | 256 | 1@42 | 1@63 | 424 | 1@0 | 1@105 |
|  |  | 50+30 | 138 | 1@71 | 1@6 | 256 | 1@41 | 1@35 | 422 | 1@0 | 1@77 |
|  |  | 60+20 | 98 | 1@114 | 1@0 | 258 | 1@42 | 1@8 | 426 | 1@0 | 1@50 |
|  |  | 70+10 | 46 | 1@167 | 1@0 | 242 | 1@69 | 1@0 | 426 | 1@0 | 1@23 |
| 85 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+70 | 74 | 1@37 | 1@35 | 268 | 1@36 | 1@131 | 454 | 1@0 | 1@188 |
|  |  | 25+60 | 128 | 1@64 | 1@62 | 274 | 1@43 | 1@114 | 454 | 1@0 | 1@161 |
|  |  | 60+25 | 126 | 1@100 | 1@0 | 274 | 1@44 | 1@18 | 454 | 1@0 | 1@64 |
|  |  | 70+15 | 72 | 1@154 | 1@0 | 268 | 1@56 | 1@0 | 454 | 1@0 | 1@37 |
| 90 | 15 | 40+50 | 160 | 1@162 | 1@105 | 293 | 1@96 | 1@172 | 486 | 1@0 | 1@269 |
|  |  | 50+40 | 160 | 1@161 | 1@50 | 293 | 1@95 | 1@117 | 486 | 1@0 | 1@215 |
|  | 30 | 10+80 | 46 | 1@23 | 1@21 | 268 | 1@22 | 1@131 | 482 | 1@0 | 1@216 |
|  |  | 20+70 | 98 | 1@50 | 1@47 | 290 | 1@47 | 1@140 | 480 | 1@0 | 1@188 |
|  |  | 30+60 | 152 | 1@77 | 1@74 | 290 | 1@46 | 1@112 | 480 | 1@0 | 1@161 |
|  |  | 40+50 | 156 | 1@80 | 1@51 | 288 | 1@47 | 1@84 | 478 | 1@0 | 1@132 |
|  |  | 50+40 | 156 | 1@80 | 1@24 | 288 | 1@46 | 1@56 | 478 | 1@0 | 1@105 |
|  |  | 60+30 | 152 | 1@87 | 1@0 | 290 | 1@47 | 1@29 | 480 | 1@0 | 1@77 |
|  |  | 70+20 | 96 | 1@142 | 1@0 | 290 | 1@47 | 1@2 | 480 | 1@0 | 1@50 |
|  |  | 80+10 | 46 | 1@195 | 1@0 | 268 | 1@84 | 1@0 | 482 | 1@0 | 1@23 |
| 95 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+80 | 76 | 1@37 | 1@36 | 298 | 1@37 | 1@147 | 510 | 1@0 | 1@216 |
|  |  | 25+70 | 126 | 1@64 | 1@61 | 308 | 1@50 | 1@138 | 508 | 1@0 | 1@188 |
|  |  | 70+25 | 124 | 1@128 | 1@0 | 308 | 1@50 | 1@14 | 508 | 1@0 | 1@64 |
|  |  | 80+15 | 74 | 1@181 | 1@0 | 298 | 1@69 | 1@0 | 510 | 1@0 | 1@37 |
| 100 | 15 | 50+50 | 178 | 1@180 | 1@87 | 325 | 1@106 | 1@160 | 540 | 1@0 | 1@269 |
|  | 30 | 10+90 | 46 | 1@23 | 1@21 | 296 | 1@23 | 1@146 | 538 | 1@0 | 1@244 |
|  |  | 20+80 | 96 | 1@50 | 1@46 | 320 | 1@50 | 1@158 | 536 | 1@0 | 1@216 |
|  |  | 30+70 | 150 | 1@77 | 1@73 | 322 | 1@52 | 1@134 | 534 | 1@0 | 1@188 |
|  |  | 40+60 | 176 | 1@90 | 1@71 | 324 | 1@53 | 1@108 | 536 | 1@0 | 1@161 |
|  |  | 50+50 | 174 | 1@89 | 1@42 | 320 | 1@52 | 1@78 | 532 | 1@0 | 1@132 |
|  |  | 60+40 | 176 | 1@90 | 1@15 | 324 | 1@53 | 1@52 | 536 | 1@0 | 1@105 |
|  |  | 70+30 | 150 | 1@115 | 1@0 | 322 | 1@52 | 1@23 | 534 | 1@0 | 1@77 |
|  |  | 80+20 | 96 | 1@170 | 1@0 | 320 | 1@58 | 1@0 | 536 | 1@0 | 1@50 |
|  |  | 90+10 | 44 | 1@224 | 1@0 | 296 | 1@98 | 1@0 | 538 | 1@0 | 1@23 |
| 105 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+90 | 74 | 1@37 | 1@35 | 324 | 1@36 | 1@159 | 566 | 1@0 | 1@244 |
|  |  | 25+80 | 126 | 1@64 | 1@61 | 340 | 1@55 | 1@159 | 564 | 1@0 | 1@216 |
|  |  | 80+25 | 124 | 1@156 | 1@0 | 340 | 1@55 | 1@7 | 564 | 1@0 | 1@64 |
|  |  | 90+15 | 74 | 1@209 | 1@0 | 324 | 1@84 | 1@0 | 566 | 1@0 | 1@37 |
| 110 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 10+100 | 46 | 1@23 | 1@21 | 324 | 1@23 | 1@160 | 594 | 1@0 | 1@272 |
|  |  | 20+90 | 96 | 1@50 | 1@46 | 348 | 1@50 | 1@172 | 592 | 1@0 | 1@244 |
|  |  | 30+80 | 152 | 1@77 | 1@74 | 356 | 1@58 | 1@157 | 590 | 1@0 | 1@216 |
|  |  | 40+70 | 194 | 1@99 | 1@89 | 356 | 1@58 | 1@129 | 590 | 1@0 | 1@188 |
|  |  | 50+60 | 192 | 1@98 | 1@60 | 356 | 1@58 | 1@102 | 590 | 1@0 | 1@161 |
|  |  | 60+50 | 192 | 1@99 | 1@32 | 356 | 1@59 | 1@74 | 590 | 1@0 | 1@132 |
|  |  | 70+40 | 194 | 1@98 | 1@5 | 356 | 1@58 | 1@46 | 590 | 1@0 | 1@105 |
|  |  | 80+30 | 150 | 1@143 | 1@0 | 356 | 1@58 | 1@18 | 590 | 1@0 | 1@77 |
|  |  | 90+20 | 94 | 1@199 | 1@0 | 348 | 1@72 | 1@0 | 592 | 1@0 | 1@50 |
|  |  | 100+10 | 44 | 1@252 | 1@0 | 324 | 1@112 | 1@0 | 594 | 1@0 | 1@23 |
| 115 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+100 | 74 | 1@37 | 1@35 | 352 | 1@37 | 1@174 | 622 | 1@0 | 1@272 |
|  |  | 25+90 | 124 | 1@64 | 1@60 | 374 | 1@61 | 1@182 | 620 | 1@0 | 1@244 |
|  |  | 90+25 | 124 | 1@184 | 1@0 | 374 | 1@61 | 1@2 | 620 | 1@0 | 1@64 |
|  |  | 100+15 | 72 | 1@238 | 1@0 | 352 | 1@98 | 1@0 | 622 | 1@0 | 1@37 |
| 120 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 20+100 | 98 | 1@50 | 1@47 | 376 | 1@50 | 1@186 | 648 | 1@0 | 1@272 |
|  | 30 | 30+90 | 152 | 1@77 | 1@74 | 390 | 1@64 | 1@180 | 646 | 1@0 | 1@244 |
|  | 30 | 40+80 | 206 | 1@105 | 1@101 | 390 | 1@64 | 1@152 | 646 | 1@0 | 1@216 |
|  | 30 | 50+70 | 210 | 1@107 | 1@78 | 388 | 1@63 | 1@123 | 644 | 1@0 | 1@188 |
|  | 30 | 60+60 | 212 | 1@108 | 1@51 | 390 | 1@64 | 1@96 | 648 | 1@0 | 1@161 |
|  | 30 | 70+50 | 210 | 1@107 | 1@22 | 388 | 1@63 | 1@67 | 644 | 1@0 | 1@132 |
|  | 30 | 80+40 | 206 | 1@115 | 1@0 | 390 | 1@63 | 1@40 | 646 | 1@0 | 1@105 |
|  | 30 | 90+30 | 150 | 1@171 | 1@0 | 390 | 1@64 | 1@13 | 646 | 1@0 | 1@77 |
|  | 30 | 100+20 | 96 | 1@226 | 1@0 | 376 | 1@86 | 1@0 | 648 | 1@0 | 1@50 |
| 125 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 25+100 | 124 | 1@64 | 1@60 | 404 | 1@64 | 1@200 | 676 | 1@0 | 1@272 |
|  | 30 | 100+25 | 122 | 1@213 | 1@0 | 404 | 1@72 | 1@0 | 676 | 1@0 | 1@64 |
| 130 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 30+100 | 150 | 1@77 | 1@73 | 422 | 1@69 | 1@201 | 702 | 1@0 | 1@272 |
|  | 30 | 40+90 | 206 | 1@105 | 1@101 | 422 | 1@69 | 1@173 | 702 | 1@0 | 1@244 |
|  | 30 | 50+80 | 230 | 1@117 | 1@98 | 422 | 1@69 | 1@146 | 700 | 1@0 | 1@216 |
|  | 30 | 60+70 | 230 | 1@117 | 1@69 | 422 | 1@69 | 1@117 | 702 | 1@0 | 1@188 |
|  | 30 | 70+60 | 230 | 1@117 | 1@42 | 422 | 1@69 | 1@90 | 702 | 1@0 | 1@161 |
|  | 30 | 80+50 | 230 | 1@117 | 1@14 | 422 | 1@69 | 1@62 | 700 | 1@0 | 1@132 |
|  | 30 | 90+40 | 204 | 1@144 | 1@0 | 422 | 1@68 | 1@33 | 702 | 1@0 | 1@105 |
|  | 30 | 100+30 | 150 | 1@199 | 1@0 | 422 | 1@69 | 1@6 | 702 | 1@0 | 1@77 |
| 140 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 40+100 | 208 | 1@105 | 1@102 | 458 | 1@75 | 1@197 | 758 | 1@0 | 1@272 |
|  | 30 | 50+90 | 250 | 1@126 | 1@117 | 456 | 1@74 | 1@168 | 756 | 1@0 | 1@244 |
|  | 30 | 60+80 | 250 | 1@127 | 1@89 | 456 | 1@75 | 1@140 | 758 | 1@0 | 1@216 |
|  | 30 | 70+70 | 248 | 1@126 | 1@60 | 454 | 1@74 | 1@111 | 756 | 1@0 | 1@188 |
|  | 30 | 80+60 | 248 | 1@126 | 1@32 | 456 | 1@75 | 1@85 | 758 | 1@0 | 1@161 |
|  | 30 | 90+50 | 250 | 1@126 | 1@5 | 456 | 1@74 | 1@56 | 756 | 1@0 | 1@132 |
|  | 30 | 100+40 | 206 | 1@171 | 1@0 | 458 | 1@75 | 1@30 | 758 | 1@0 | 1@105 |
| 150 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 50+100 | 260 | 1@132 | 1@128 | 490 | 1@80 | 1@191 | 812 | 1@0 | 1@272 |
|  | 30 | 60+90 | 268 | 1@136 | 1@107 | 490 | 1@81 | 1@163 | 814 | 1@0 | 1@244 |
|  | 30 | 70+80 | 268 | 1@136 | 1@80 | 488 | 1@80 | 1@134 | 812 | 1@0 | 1@216 |
|  | 30 | 80+70 | 268 | 1@136 | 1@52 | 488 | 1@80 | 1@106 | 812 | 1@0 | 1@188 |
|  | 30 | 90+60 | 268 | 1@135 | 1@23 | 490 | 1@80 | 1@79 | 814 | 1@0 | 1@161 |
|  | 30 | 100+50 | 260 | 1@144 | 1@0 | 490 | 1@80 | 1@51 | 812 | 1@0 | 1@132 |
| 160 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 60+100 | 286 | 1@145 | 1@125 | 524 | 1@86 | 1@185 | 870 | 1@0 | 1@272 |
|  | 30 | 70+90 | 286 | 1@145 | 1@98 | 522 | 1@86 | 1@157 | 868 | 1@0 | 1@244 |
|  | 30 | 80+80 | 286 | 1@145 | 1@70 | 522 | 1@86 | 1@129 | 868 | 1@0 | 1@216 |
|  | 30 | 90+70 | 286 | 1@145 | 1@42 | 522 | 1@86 | 1@101 | 868 | 1@0 | 1@188 |
|  | 30 | 100+60 | 288 | 1@145 | 1@15 | 524 | 1@86 | 1@74 | 870 | 1@0 | 1@161 |
| 170 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 70+100 | 306 | 1@154 | 1@117 | 556 | 1@91 | 1@179 | 924 | 1@0 | 1@272 |
|  | 30 | 80+90 | 304 | 1@154 | 1@88 | 556 | 1@92 | 1@152 | 924 | 1@0 | 1@244 |
|  | 30 | 90+80 | 304 | 1@154 | 1@60 | 556 | 1@92 | 1@124 | 924 | 1@0 | 1@216 |
|  | 30 | 100+70 | 306 | 1@154 | 1@33 | 556 | 1@91 | 1@95 | 924 | 1@0 | 1@188 |
| 180 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 80+100 | 324 | 1@164 | 1@108 | 590 | 1@97 | 1@174 | 980 | 1@0 | 1@272 |
|  | 30 | 90+90 | 322 | 1@164 | 1@79 | 588 | 1@97 | 1@145 | 980 | 1@0 | 1@244 |
|  | 30 | 100+80 | 324 | 1@164 | 1@52 | 590 | 1@97 | 1@118 | 980 | 1@0 | 1@216 |
| 190 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 90+100 | 342 | 1@173 | 1@98 | 622 | 1@102 | 1@167 | 1036 | 1@0 | 1@272 |
|  | 30 | 100+90 | 342 | 1@173 | 1@70 | 622 | 1@102 | 1@139 | 1036 | 1@0 | 1@244 |
| 200 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 100+100 | 360 | 1@182 | 1@88 | 656 | 1@108 | 1@162 | 1092 | 1@0 | 1@272 |
| NOTE 1: BWchannel_CA is defined in clause 5.3A.3 NOTE 2: NRB_alloc for intra-band contiguous UL CA is defined in 6.2A.2.0 |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1A-1c: Common uplink configuration for intra-band contiguous 2UL CA (non-contiguous RB allocation) for NR operating bands with 15 kHz channel raster

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | RB allocation (Inner) |  |  | RB allocation (Outer1) |  |  | RB allocation (Outer2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
| 20 | 15 | 5+15 | 24 | 1@24 | 1@22 | 63 | 1@19 | 1@56 | 104 | 1@0 | 1@78 |
|  |  | 10+10 | 33 | 1@34 | 1@14 | 62 | 1@19 | 1@28 | 104 | 1@0 | 1@51 |
|  |  | 15+5 | 23 | 1@57 | 1@0 | 63 | 1@19 | 1@2 | 104 | 1@0 | 1@24 |
|  | 30 | 10+10 | 28 | 1@16 | 1@5 | 58 | 1@9 | 1@13 | 96 | 1@0 | 1@23 |
| 25 | 15 | 5+20 | 25 | 1@24 | 1@23 | 80 | 1@23 | 1@77 | 131 | 1@0 | 1@105 |
|  |  | 10+15 | 42 | 1@44 | 1@33 | 79 | 1@26 | 1@52 | 131 | 1@0 | 1@78 |
|  |  | 15+10 | 42 | 1@43 | 1@5 | 79 | 1@25 | 1@24 | 131 | 1@0 | 1@51 |
|  |  | 20+5 | 24 | 1@83 | 1@0 | 80 | 1@27 | 1@0 | 131 | 1@0 | 1@24 |
|  | 30 | 10+15 | 38 | 1@21 | 1@15 | 74 | 1@11 | 1@23 | 124 | 1@0 | 1@37 |
|  |  | 15+10 | 38 | 1@21 | 1@1 | 74 | 1@11 | 1@9 | 124 | 1@0 | 1@23 |
| 30 | 15 | 5+25 | 25 | 1@24 | 1@23 | 93 | 1@23 | 1@90 | 158 | 1@0 | 1@132 |
|  |  | 10+20 | 53 | 1@51 | 1@51 | 97 | 1@30 | 1@74 | 158 | 1@0 | 1@105 |
|  |  | 15+15 | 51 | 1@53 | 1@24 | 95 | 1@31 | 1@46 | 158 | 1@0 | 1@78 |
|  |  | 20+10 | 51 | 1@56 | 1@0 | 97 | 1@31 | 1@21 | 158 | 1@0 | 1@51 |
|  |  | 25+5 | 24 | 1@110 | 1@0 | 93 | 1@41 | 1@0 | 158 | 1@0 | 1@24 |
|  | 30 | 10+20 | 46 | 1@23 | 1@21 | 92 | 1@14 | 1@35 | 150 | 1@0 | 1@50 |
|  |  | 15+15 | 48 | 1@26 | 1@11 | 92 | 1@15 | 1@22 | 152 | 1@0 | 1@37 |
|  |  | 20+10 | 44 | 1@30 | 1@0 | 92 | 1@14 | 1@8 | 150 | 1@0 | 1@23 |
| 35 | 15 | 5+30 | 26 | 1@24 | 1@24 | 108 | 1@23 | 1@105 | 185 | 1@0 | 1@159 |
|  |  | 10+25 | 52 | 1@51 | 1@50 | 113 | 1@35 | 1@95 | 185 | 1@0 | 1@132 |
|  |  | 15+20 | 62 | 1@60 | 1@42 | 113 | 1@34 | 1@67 | 185 | 1@0 | 1@105 |
|  |  | 20+15 | 61 | 1@61 | 1@15 | 113 | 1@36 | 1@42 | 185 | 1@0 | 1@78 |
|  |  | 25+10 | 51 | 1@83 | 1@0 | 113 | 1@35 | 1@14 | 185 | 1@0 | 1@51 |
|  |  | 30+5 | 25 | 1@136 | 1@0 | 108 | 1@53 | 1@0 | 185 | 1@0 | 1@24 |
|  | 30 | 10+25 | 46 | 1@23 | 1@21 | 108 | 1@16 | 1@45 | 178 | 1@0 | 1@64 |
|  |  | 15+20 | 58 | 1@29 | 1@19 | 108 | 1@16 | 1@31 | 178 | 1@0 | 1@50 |
|  |  | 20+15 | 58 | 1@30 | 1@7 | 108 | 1@16 | 1@18 | 178 | 1@0 | 1@37 |
|  |  | 25+10 | 44 | 1@44 | 1@0 | 108 | 1@16 | 1@4 | 178 | 1@0 | 1@23 |
| 40 | 15 | 10+30 | 51 | 1@51 | 1@49 | 129 | 1@40 | 1@116 | 212 | 1@0 | 1@159 |
|  |  | 15+25 | 70 | 1@69 | 1@59 | 129 | 1@39 | 1@88 | 212 | 1@0 | 1@132 |
|  |  | 20+20 | 69 | 1@71 | 1@33 | 128 | 1@42 | 1@63 | 212 | 1@0 | 1@105 |
|  |  | 25+15 | 70 | 1@70 | 1@6 | 129 | 1@40 | 1@35 | 212 | 1@0 | 1@78 |
|  |  | 30+10 | 50 | 1@111 | 1@0 | 129 | 1@40 | 1@8 | 212 | 1@0 | 1@51 |
|  | 30 | 10+30 | 44 | 1@23 | 1@20 | 124 | 1@19 | 1@56 | 204 | 1@0 | 1@77 |
|  |  | 15+25 | 66 | 1@33 | 1@27 | 126 | 1@19 | 1@43 | 206 | 1@0 | 1@64 |
|  |  | 20+20 | 64 | 1@34 | 1@14 | 124 | 1@20 | 1@30 | 204 | 1@0 | 1@50 |
|  |  | 25+15 | 66 | 1@34 | 1@1 | 126 | 1@20 | 1@17 | 206 | 1@0 | 1@37 |
|  |  | 30+10 | 42 | 1@58 | 1@0 | 124 | 1@19 | 1@2 | 204 | 1@0 | 1@23 |
| 45 | 15 | 5+40 | 26 | 1@24 | 1@24 | 136 | 1@24 | 1@134 | 241 | 1@0 | 1@215 |
|  |  | 15+30 | 78 | 1@78 | 1@76 | 145 | 1@45 | 1@110 | 239 | 1@0 | 1@159 |
|  |  | 20+25 | 78 | 1@80 | 1@51 | 144 | 1@47 | 1@84 | 239 | 1@0 | 1@132 |
|  |  | 25+20 | 78 | 1@80 | 1@24 | 144 | 1@46 | 1@56 | 239 | 1@0 | 1@105 |
|  |  | 30+15 | 77 | 1@84 | 1@0 | 145 | 1@45 | 1@29 | 239 | 1@0 | 1@78 |
|  |  | 40+5 | 25 | 1@192 | 1@0 | 136 | 1@81 | 1@0 | 241 | 1@0 | 1@24 |
|  | 30 | 15+30 | 72 | 1@37 | 1@34 | 142 | 1@22 | 1@54 | 232 | 1@0 | 1@77 |
|  |  | 20+25 | 74 | 1@39 | 1@24 | 140 | 1@22 | 1@40 | 232 | 1@0 | 1@64 |
|  |  | 25+20 | 74 | 1@39 | 1@10 | 140 | 1@22 | 1@26 | 232 | 1@0 | 1@50 |
|  |  | 30+15 | 70 | 1@44 | 1@0 | 142 | 1@22 | 1@14 | 232 | 1@0 | 1@37 |
| 50 | 15 | 10+40 | 55 | 1@51 | 1@53 | 164 | 1@49 | 1@160 | 268 | 1@0 | 1@215 |
|  |  | 20+30 | 86 | 1@89 | 1@68 | 160 | 1@52 | 1@105 | 266 | 1@0 | 1@159 |
|  |  | 25+25 | 86 | 1@88 | 1@40 | 160 | 1@52 | 1@78 | 266 | 1@0 | 1@132 |
|  |  | 30+20 | 86 | 1@88 | 1@13 | 160 | 1@51 | 1@50 | 266 | 1@0 | 1@105 |
|  |  | 40+10 | 54 | 1@163 | 1@0 | 164 | 1@53 | 1@0 | 268 | 1@0 | 1@51 |
|  | 30 | 10+40 | 48 | 1@23 | 1@22 | 158 | 1@22 | 1@76 | 260 | 1@0 | 1@105 |
|  |  | 20+30 | 82 | 1@44 | 1@33 | 156 | 1@25 | 1@51 | 258 | 1@0 | 1@77 |
|  |  | 25+25 | 82 | 1@43 | 1@18 | 156 | 1@25 | 1@37 | 260 | 1@0 | 1@64 |
|  |  | 30+20 | 82 | 1@43 | 1@5 | 156 | 1@25 | 1@24 | 258 | 1@0 | 1@50 |
|  |  | 40+10 | 46 | 1@84 | 1@0 | 158 | 1@28 | 1@0 | 260 | 1@0 | 1@23 |
| 55 | 15 | 5+50 | 27 | 1@24 | 1@25 | 164 | 1@23 | 1@161 | 295 | 1@0 | 1@269 |
|  |  | 15+40 | 82 | 1@78 | 1@80 | 181 | 1@55 | 1@156 | 295 | 1@0 | 1@215 |
|  |  | 25+30 | 95 | 1@98 | 1@59 | 176 | 1@57 | 1@99 | 293 | 1@0 | 1@159 |
|  |  | 30+25 | 95 | 1@97 | 1@31 | 176 | 1@56 | 1@71 | 293 | 1@0 | 1@132 |
|  |  | 40+15 | 81 | 1@136 | 1@0 | 181 | 1@58 | 1@22 | 295 | 1@0 | 1@78 |
|  |  | 50+5 | 26 | 1@245 | 1@0 | 164 | 1@107 | 1@0 | 295 | 1@0 | 1@24 |
|  | 30 | 15+40 | 76 | 1@37 | 1@36 | 176 | 1@26 | 1@75 | 288 | 1@0 | 1@105 |
|  |  | 25+30 | 90 | 1@48 | 1@27 | 172 | 1@28 | 1@48 | 286 | 1@0 | 1@77 |
|  |  | 30+25 | 92 | 1@48 | 1@15 | 172 | 1@27 | 1@34 | 286 | 1@0 | 1@64 |
|  |  | 40+15 | 74 | 1@70 | 1@0 | 176 | 1@27 | 1@8 | 288 | 1@0 | 1@37 |
| 60 | 15 | 10+50 | 54 | 1@51 | 1@52 | 191 | 1@50 | 1@188 | 322 | 1@0 | 1@269 |
|  |  | 20+40 | 106 | 1@105 | 1@104 | 195 | 1@62 | 1@150 | 322 | 1@0 | 1@215 |
|  |  | 30+30 | 104 | 1@107 | 1@50 | 192 | 1@62 | 1@93 | 320 | 1@0 | 1@159 |
|  |  | 40+20 | 104 | 1@113 | 1@0 | 195 | 1@63 | 1@41 | 322 | 1@0 | 1@105 |
|  |  | 50+10 | 53 | 1@218 | 1@0 | 191 | 1@80 | 1@0 | 322 | 1@0 | 1@51 |
|  | 30 | 10+50 | 46 | 1@23 | 1@21 | 186 | 1@23 | 1@91 | 314 | 1@0 | 1@132 |
|  |  | 20+40 | 98 | 1@50 | 1@47 | 190 | 1@30 | 1@73 | 314 | 1@0 | 1@105 |
|  |  | 30+30 | 100 | 1@53 | 1@24 | 188 | 1@30 | 1@45 | 312 | 1@0 | 1@77 |
|  |  | 40+20 | 96 | 1@59 | 1@0 | 190 | 1@30 | 1@18 | 314 | 1@0 | 1@50 |
|  |  | 50+10 | 46 | 1@111 | 1@0 | 186 | 1@41 | 1@0 | 314 | 1@0 | 1@23 |
| 65 | 15 | 15+50 | 81 | 1@78 | 1@79 | 213 | 1@66 | 1@199 | 349 | 1@0 | 1@269 |
|  |  | 25+40 | 116 | 1@115 | 1@97 | 212 | 1@68 | 1@146 | 349 | 1@0 | 1@215 |
|  |  | 40+25 | 116 | 1@117 | 1@16 | 212 | 1@69 | 1@64 | 349 | 1@0 | 1@132 |
|  |  | 50+15 | 80 | 1@191 | 1@0 | 213 | 1@68 | 1@10 | 349 | 1@0 | 1@78 |
|  | 30 | 15+50 | 74 | 1@37 | 1@35 | 208 | 1@32 | 1@97 | 342 | 1@0 | 1@132 |
|  |  | 25+40 | 112 | 1@57 | 1@47 | 208 | 1@33 | 1@71 | 342 | 1@0 | 1@105 |
|  |  | 40+25 | 112 | 1@57 | 1@6 | 208 | 1@33 | 1@30 | 342 | 1@0 | 1@64 |
|  |  | 50+15 | 74 | 1@97 | 1@0 | 208 | 1@32 | 1@2 | 342 | 1@0 | 1@37 |
| 70 | 15 | 20+50 | 104 | 1@105 | 1@102 | 228 | 1@74 | 1@195 | 376 | 1@0 | 1@269 |
|  |  | 30+40 | 125 | 1@124 | 1@88 | 228 | 1@72 | 1@139 | 376 | 1@0 | 1@215 |
|  |  | 40+30 | 125 | 1@126 | 1@34 | 228 | 1@74 | 1@85 | 376 | 1@0 | 1@159 |
|  |  | 50+20 | 103 | 1@168 | 1@0 | 228 | 1@74 | 1@31 | 376 | 1@0 | 1@105 |
|  | 30 | 10+60 | 48 | 1@23 | 1@22 | 214 | 1@23 | 1@105 | 372 | 1@0 | 1@161 |
|  |  | 20+50 | 96 | 1@50 | 1@46 | 224 | 1@36 | 1@96 | 368 | 1@0 | 1@132 |
|  |  | 30+40 | 120 | 1@61 | 1@42 | 224 | 1@35 | 1@68 | 368 | 1@0 | 1@105 |
|  |  | 40+30 | 120 | 1@62 | 1@15 | 224 | 1@36 | 1@41 | 368 | 1@0 | 1@77 |
|  |  | 50+20 | 96 | 1@86 | 1@0 | 224 | 1@36 | 1@14 | 368 | 1@0 | 1@50 |
|  |  | 60+10 | 46 | 1@140 | 1@0 | 214 | 1@56 | 1@0 | 372 | 1@0 | 1@23 |
| 75 | 15 | 25+50 | 132 | 1@132 | 1@130 | 244 | 1@78 | 1@188 | 403 | 1@0 | 1@269 |
|  |  | 50+25 | 130 | 1@141 | 1@0 | 244 | 1@79 | 1@52 | 403 | 1@0 | 1@132 |
|  | 30 | 15+60 | 76 | 1@37 | 1@36 | 242 | 1@37 | 1@119 | 400 | 1@0 | 1@161 |
|  |  | 25+50 | 126 | 1@64 | 1@61 | 240 | 1@38 | 1@92 | 396 | 1@0 | 1@132 |
|  |  | 50+25 | 124 | 1@72 | 1@0 | 240 | 1@38 | 1@24 | 396 | 1@0 | 1@64 |
|  |  | 60+15 | 74 | 1@126 | 1@0 | 242 | 1@42 | 1@0 | 400 | 1@0 | 1@37 |
| 80 | 15 | 30+50 | 142 | 1@142 | 1@123 | 260 | 1@83 | 1@182 | 430 | 1@0 | 1@269 |
|  |  | 40+40 | 142 | 1@144 | 1@69 | 260 | 1@86 | 1@129 | 432 | 1@0 | 1@215 |
|  |  | 50+30 | 142 | 1@143 | 1@14 | 260 | 1@84 | 1@73 | 430 | 1@0 | 1@159 |
|  | 30 | 10+70 | 46 | 1@23 | 1@21 | 240 | 1@22 | 1@117 | 426 | 1@0 | 1@188 |
|  |  | 20+60 | 98 | 1@50 | 1@47 | 258 | 1@42 | 1@119 | 426 | 1@0 | 1@161 |
|  |  | 30+50 | 138 | 1@70 | 1@60 | 256 | 1@41 | 1@90 | 422 | 1@0 | 1@132 |
|  |  | 40+40 | 138 | 1@71 | 1@33 | 256 | 1@42 | 1@63 | 424 | 1@0 | 1@105 |
|  |  | 50+30 | 138 | 1@71 | 1@6 | 256 | 1@41 | 1@35 | 422 | 1@0 | 1@77 |
|  |  | 60+20 | 96 | 1@115 | 1@0 | 258 | 1@42 | 1@8 | 426 | 1@0 | 1@50 |
|  |  | 70+10 | 44 | 1@168 | 1@0 | 240 | 1@70 | 1@0 | 426 | 1@0 | 1@23 |
| 85 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+70 | 74 | 1@37 | 1@35 | 268 | 1@36 | 1@131 | 454 | 1@0 | 1@188 |
|  |  | 25+60 | 126 | 1@64 | 1@61 | 274 | 1@44 | 1@115 | 454 | 1@0 | 1@161 |
|  |  | 60+25 | 124 | 1@101 | 1@0 | 274 | 1@45 | 1@19 | 454 | 1@0 | 1@64 |
|  |  | 70+15 | 72 | 1@154 | 1@0 | 268 | 1@56 | 1@0 | 454 | 1@0 | 1@37 |
| 90 | 15 | 40+50 | 160 | 1@163 | 1@106 | 293 | 1@97 | 1@173 | 486 | 1@0 | 1@269 |
|  |  | 50+40 | 160 | 1@162 | 1@51 | 293 | 1@96 | 1@118 | 486 | 1@0 | 1@215 |
|  | 30 | 10+80 | 46 | 1@23 | 1@21 | 268 | 1@23 | 1@132 | 482 | 1@0 | 1@216 |
|  |  | 20+70 | 96 | 1@50 | 1@46 | 290 | 1@47 | 1@140 | 480 | 1@0 | 1@188 |
|  |  | 30+60 | 152 | 1@77 | 1@74 | 290 | 1@46 | 1@112 | 480 | 1@0 | 1@161 |
|  |  | 40+50 | 154 | 1@80 | 1@50 | 288 | 1@47 | 1@84 | 478 | 1@0 | 1@132 |
|  |  | 50+40 | 156 | 1@80 | 1@24 | 288 | 1@47 | 1@57 | 478 | 1@0 | 1@105 |
|  |  | 60+30 | 150 | 1@88 | 1@0 | 290 | 1@47 | 1@29 | 480 | 1@0 | 1@77 |
|  |  | 70+20 | 96 | 1@142 | 1@0 | 290 | 1@47 | 1@2 | 480 | 1@0 | 1@50 |
|  |  | 80+10 | 44 | 1@196 | 1@0 | 268 | 1@84 | 1@0 | 482 | 1@0 | 1@23 |
| 95 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+80 | 74 | 1@37 | 1@35 | 296 | 1@37 | 1@146 | 510 | 1@0 | 1@216 |
|  |  | 25+70 | 124 | 1@64 | 1@60 | 306 | 1@49 | 1@136 | 508 | 1@0 | 1@188 |
|  |  | 70+25 | 124 | 1@128 | 1@0 | 306 | 1@49 | 1@12 | 508 | 1@0 | 1@64 |
|  |  | 80+15 | 72 | 1@182 | 1@0 | 296 | 1@70 | 1@0 | 510 | 1@0 | 1@37 |
| 100 | 15 | 50+50 | 177 | 1@180 | 1@86 | 325 | 1@107 | 1@161 | 540 | 1@0 | 1@269 |
|  | 30 | 10+90 | 46 | 1@23 | 1@21 | 296 | 1@23 | 1@146 | 538 | 1@0 | 1@244 |
|  |  | 20+80 | 96 | 1@50 | 1@46 | 320 | 1@50 | 1@158 | 536 | 1@0 | 1@216 |
|  |  | 30+70 | 150 | 1@77 | 1@73 | 322 | 1@52 | 1@134 | 534 | 1@0 | 1@188 |
|  |  | 40+60 | 174 | 1@89 | 1@69 | 322 | 1@52 | 1@106 | 536 | 1@0 | 1@161 |
|  |  | 50+50 | 172 | 1@89 | 1@41 | 320 | 1@52 | 1@78 | 532 | 1@0 | 1@132 |
|  |  | 60+40 | 174 | 1@89 | 1@13 | 322 | 1@52 | 1@50 | 536 | 1@0 | 1@105 |
|  |  | 70+30 | 150 | 1@115 | 1@0 | 322 | 1@52 | 1@23 | 534 | 1@0 | 1@77 |
|  |  | 80+20 | 94 | 1@171 | 1@0 | 320 | 1@58 | 1@0 | 536 | 1@0 | 1@50 |
|  |  | 90+10 | 44 | 1@224 | 1@0 | 296 | 1@98 | 1@0 | 538 | 1@0 | 1@23 |
| 105 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+90 | 74 | 1@37 | 1@35 | 324 | 1@37 | 1@160 | 566 | 1@0 | 1@244 |
|  |  | 25+80 | 124 | 1@64 | 1@60 | 340 | 1@55 | 1@159 | 564 | 1@0 | 1@216 |
|  |  | 80+25 | 124 | 1@156 | 1@0 | 340 | 1@55 | 1@7 | 564 | 1@0 | 1@64 |
|  |  | 90+15 | 72 | 1@210 | 1@0 | 324 | 1@84 | 1@0 | 566 | 1@0 | 1@37 |
| 110 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 10+100 | 46 | 1@23 | 1@21 | 324 | 1@23 | 1@160 | 594 | 1@0 | 1@272 |
|  | 30 | 20+90 | 96 | 1@50 | 1@46 | 348 | 1@50 | 1@172 | 592 | 1@0 | 1@244 |
|  | 30 | 30+80 | 150 | 1@77 | 1@73 | 356 | 1@58 | 1@157 | 590 | 1@0 | 1@216 |
|  | 30 | 40+70 | 194 | 1@99 | 1@89 | 356 | 1@58 | 1@129 | 590 | 1@0 | 1@188 |
|  | 30 | 50+60 | 192 | 1@98 | 1@60 | 354 | 1@57 | 1@100 | 590 | 1@0 | 1@161 |
|  | 30 | 60+50 | 192 | 1@99 | 1@32 | 354 | 1@58 | 1@72 | 590 | 1@0 | 1@132 |
|  | 30 | 70+40 | 192 | 1@98 | 1@4 | 356 | 1@58 | 1@46 | 590 | 1@0 | 1@105 |
|  | 30 | 80+30 | 150 | 1@143 | 1@0 | 356 | 1@58 | 1@18 | 590 | 1@0 | 1@77 |
|  | 30 | 90+20 | 94 | 1@199 | 1@0 | 348 | 1@72 | 1@0 | 592 | 1@0 | 1@50 |
|  | 30 | 100+10 | 44 | 1@252 | 1@0 | 324 | 1@112 | 1@0 | 594 | 1@0 | 1@23 |
| 115 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+100 | 74 | 1@37 | 1@35 | 352 | 1@37 | 1@174 | 622 | 1@0 | 1@272 |
|  | 30 | 25+90 | 124 | 1@64 | 1@60 | 372 | 1@60 | 1@180 | 620 | 1@0 | 1@244 |
|  | 30 | 90+25 | 124 | 1@184 | 1@0 | 372 | 1@60 | 1@0 | 620 | 1@0 | 1@64 |
|  | 30 | 100+15 | 72 | 1@238 | 1@0 | 352 | 1@98 | 1@0 | 622 | 1@0 | 1@37 |
| 120 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 20+100 | 96 | 1@50 | 1@46 | 376 | 1@50 | 1@186 | 648 | 1@0 | 1@272 |
|  | 30 | 30+90 | 150 | 1@77 | 1@73 | 388 | 1@63 | 1@178 | 646 | 1@0 | 1@244 |
|  | 30 | 40+80 | 206 | 1@105 | 1@101 | 390 | 1@64 | 1@152 | 646 | 1@0 | 1@216 |
|  | 30 | 50+70 | 210 | 1@107 | 1@78 | 388 | 1@63 | 1@123 | 644 | 1@0 | 1@188 |
|  | 30 | 60+60 | 212 | 1@108 | 1@51 | 390 | 1@64 | 1@96 | 648 | 1@0 | 1@161 |
|  | 30 | 70+50 | 210 | 1@107 | 1@22 | 388 | 1@63 | 1@67 | 644 | 1@0 | 1@132 |
|  | 30 | 80+40 | 204 | 1@116 | 1@0 | 390 | 1@64 | 1@41 | 646 | 1@0 | 1@105 |
|  | 30 | 90+30 | 150 | 1@171 | 1@0 | 388 | 1@63 | 1@11 | 646 | 1@0 | 1@77 |
|  | 30 | 100+20 | 94 | 1@227 | 1@0 | 376 | 1@86 | 1@0 | 648 | 1@0 | 1@50 |
| 125 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 25+100 | 124 | 1@64 | 1@60 | 404 | 1@64 | 1@200 | 676 | 1@0 | 1@272 |
|  | 30 | 100+25 | 122 | 1@213 | 1@0 | 404 | 1@72 | 1@0 | 676 | 1@0 | 1@64 |
| 130 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 30+100 | 150 | 1@77 | 1@73 | 422 | 1@69 | 1@201 | 702 | 1@0 | 1@272 |
|  | 30 | 40+90 | 206 | 1@105 | 1@101 | 422 | 1@69 | 1@173 | 702 | 1@0 | 1@244 |
|  | 30 | 50+80 | 230 | 1@117 | 1@98 | 422 | 1@69 | 1@146 | 700 | 1@0 | 1@216 |
|  | 30 | 60+70 | 230 | 1@118 | 1@70 | 422 | 1@70 | 1@118 | 702 | 1@0 | 1@188 |
|  | 30 | 70+60 | 230 | 1@117 | 1@42 | 422 | 1@69 | 1@90 | 702 | 1@0 | 1@161 |
|  | 30 | 80+50 | 230 | 1@117 | 1@14 | 422 | 1@69 | 1@62 | 700 | 1@0 | 1@132 |
|  | 30 | 90+40 | 204 | 1@144 | 1@0 | 422 | 1@68 | 1@33 | 702 | 1@0 | 1@105 |
|  | 30 | 100+30 | 148 | 1@200 | 1@0 | 422 | 1@69 | 1@6 | 702 | 1@0 | 1@77 |
| 140 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 40+100 | 206 | 1@105 | 1@101 | 456 | 1@74 | 1@195 | 758 | 1@0 | 1@272 |
|  | 30 | 50+90 | 248 | 1@126 | 1@116 | 456 | 1@75 | 1@169 | 756 | 1@0 | 1@244 |
|  | 30 | 60+80 | 250 | 1@127 | 1@89 | 456 | 1@75 | 1@140 | 758 | 1@0 | 1@216 |
|  | 30 | 70+70 | 248 | 1@127 | 1@61 | 454 | 1@74 | 1@111 | 756 | 1@0 | 1@188 |
|  | 30 | 80+60 | 248 | 1@126 | 1@32 | 456 | 1@75 | 1@85 | 758 | 1@0 | 1@161 |
|  | 30 | 90+50 | 248 | 1@126 | 1@4 | 456 | 1@75 | 1@57 | 756 | 1@0 | 1@132 |
|  | 30 | 100+40 | 206 | 1@171 | 1@0 | 456 | 1@74 | 1@28 | 758 | 1@0 | 1@105 |
| 150 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 50+100 | 260 | 1@132 | 1@128 | 488 | 1@79 | 1@189 | 812 | 1@0 | 1@272 |
|  | 30 | 60+90 | 268 | 1@136 | 1@107 | 490 | 1@81 | 1@163 | 814 | 1@0 | 1@244 |
|  | 30 | 70+80 | 266 | 1@135 | 1@78 | 488 | 1@80 | 1@134 | 812 | 1@0 | 1@216 |
|  | 30 | 80+70 | 266 | 1@135 | 1@50 | 488 | 1@80 | 1@106 | 812 | 1@0 | 1@188 |
|  | 30 | 90+60 | 268 | 1@136 | 1@24 | 490 | 1@80 | 1@79 | 814 | 1@0 | 1@161 |
|  | 30 | 100+50 | 260 | 1@144 | 1@0 | 488 | 1@79 | 1@49 | 812 | 1@0 | 1@132 |
| 160 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 60+100 | 286 | 1@145 | 1@125 | 524 | 1@86 | 1@185 | 870 | 1@0 | 1@272 |
|  | 30 | 70+90 | 286 | 1@145 | 1@98 | 522 | 1@86 | 1@157 | 868 | 1@0 | 1@244 |
|  | 30 | 80+80 | 284 | 1@145 | 1@69 | 522 | 1@86 | 1@129 | 868 | 1@0 | 1@216 |
|  | 30 | 90+70 | 286 | 1@145 | 1@42 | 522 | 1@86 | 1@101 | 868 | 1@0 | 1@188 |
|  | 30 | 100+60 | 288 | 1@145 | 1@15 | 524 | 1@86 | 1@74 | 870 | 1@0 | 1@161 |
| 170 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 70+100 | 304 | 1@154 | 1@116 | 556 | 1@91 | 1@179 | 924 | 1@0 | 1@272 |
|  | 30 | 80+90 | 304 | 1@154 | 1@88 | 556 | 1@92 | 1@152 | 924 | 1@0 | 1@244 |
|  | 30 | 90+80 | 304 | 1@154 | 1@60 | 556 | 1@92 | 1@124 | 924 | 1@0 | 1@216 |
|  | 30 | 100+70 | 304 | 1@154 | 1@32 | 556 | 1@91 | 1@95 | 924 | 1@0 | 1@188 |
| 180 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 80+100 | 322 | 1@163 | 1@106 | 588 | 1@96 | 1@172 | 980 | 1@0 | 1@272 |
|  | 30 | 90+90 | 322 | 1@164 | 1@79 | 588 | 1@97 | 1@145 | 980 | 1@0 | 1@244 |
|  | 30 | 100+80 | 322 | 1@163 | 1@50 | 588 | 1@96 | 1@116 | 980 | 1@0 | 1@216 |
| 190 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 90+100 | 342 | 1@173 | 1@98 | 622 | 1@102 | 1@167 | 1036 | 1@0 | 1@272 |
|  | 30 | 100+90 | 342 | 1@173 | 1@70 | 622 | 1@102 | 1@139 | 1036 | 1@0 | 1@244 |
| 200 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 100+100 | 360 | 1@182 | 1@88 | 656 | 1@108 | 1@162 | 1092 | 1@0 | 1@272 |
| NOTE 1: BWchannel_CA is defined in clause 5.3A.3 NOTE 2: NRB_alloc for intra-band contiguous UL CA is defined in 6.2A.2.0 |  |  |  |  |  |  |  |  |  |  |  |

< End of Changes >