3GPP TSG-RAN5 Meeting #106	R5-251276

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1587 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to SIP INFO messages for CEN alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Default message contents of SIP INFO message for A.2.19 and A.2.20 needs update to align with CEN requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated To header of A.2.19 and From header of A.2.20. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eCallCEN-UEConTest WI will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.19, A.2.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

## A.2.19	MO INFO for eCall over IMS / PSAP callback over IMS

| Header/param |  | Cond |  | Value/remark |  | Rel |  | Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request-Line |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| Method |  |  |  | INFO |  |  |  | RFC 6086 [139] |  |
| Request-URI |  |  |  | same URI as SS sent in its Contact header before when it sent 200 OK for INVITE |  |  |  |  |  |
| SIP-Version |  |  |  | SIP/2.0 |  |  |  |  |  |
| Via |  |  |  |  |  |  |  | RFC 3261 [15] RFC 3581 [96] |  |
| sent-protocol |  |  |  | SIP/2.0/UDP when using UDP or SIP/2.0/TCP when using TCP |  |  |  |  |  |
| sent-by |  |  |  | IP address or FQDN and protected server port of the UE |  |  |  |  |  |
| via-branch |  |  |  | value starting with ‘z9hG4bK’ |  |  |  |  |  |
| Route |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| route-param |  |  |  | MO Call (eCall): URIs of the Record-Route header field of the dialog creating response in reverse order (200 OK for eCall, 183 Session Progress for test eCall) MT Call (PSAP callback): URIs of the Record-Route header field as specified for the INVITE in A.2.9 |  |  |  |  |  |
| From |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | SIP URI of the UE URI must be the same as used for the endpoint in the earlier requests within the dialog. |  |  |  | TS 24.229 [10] |  |
| tag |  |  |  | local tag of the dialog ID |  |  |  |  |  |
| To |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | SIP URI of the SS. URI must be the same as used for the endpoint in the earlier requests within the dialog. |  |  |  |  |  |
| tag |  |  |  | remote tag of the dialog ID |  |  |  |  |  |
| Call-ID |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| callid |  |  |  | same as value received in INVITE message |  |  |  |  |  |
| CSeq |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | value of CSeq sent by the UE within its previous request in the same dialog but increased by one |  |  |  |  |  |
| method |  |  |  | INFO |  |  |  |  |  |
| Call-Info |  |  |  |  |  |  |  |  |  |
| info |  |  |  |  |  |  |  |  |  |
| absoluteURI |  |  |  | any cid URL |  | Rel-14 |  | RFC 8147 [149] |  |
| info-param |  | A3 |  | purpose=EmergencyCallData.eCall.MSD |  | Rel-14 |  | RFC 8147 [149] |  |
|  |  | A4 |  | purpose=EmergencyCallData.Control |  | Rel-14 |  | RFC 8147 [149] |  |
| Info-Package |  |  |  | EmergencyCallData.eCall.MSD |  | Rel-14 |  | RFC 8147 [149] |  |
| Content-Type |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| media-type |  |  |  | multipart/mixed;boundary=any value |  |  |  | TS 24.229 [10] |  |
| Content-Length |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | length of message-body |  |  |  |  |  |
| Content-Disposition |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| disp-type |  |  |  | Info-Package |  | Rel-14 |  | RFC 8147 [149] |  |
| Message-body |  |  |  |  |  |  |  |  |  |
|  |  | A3 |  | --boundary value (as provided in Content-Type) Content-Type: application/EmergencyCallData.eCall.MSD Content-ID: converted cid URL from Call-Info header. Note that this is done by removing the cid: prefix. Content-Disposition: by-reference MSD in ASN.1 PER encoding --boundary value (as provided in Content-Type) |  | Rel-14 |  | RFC 8147 [149] |  |
|  |  | A4 |  | --boundary value (as provided in Content-Type) Content-Type: application/EmergencyCallData.Control+xml Content-ID: converted cid URL from Call-Info header. Note that this is done by removing the cid: prefix.Content-ID: same cid as in Call-Info header Content-Disposition: by-reference <?xml version=”1.0” encoding=”UTF-8”?> <EmergencyCallData.Control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <ack ref=cid of the body part of corresponding INFO request from SS> <actionResult action=”send-data” success=”false” reason=any value/> </ack> </EmergencyCallData.Control> --boundary value (as provided in Content-Type) |  | Rel-14 |  | RFC 8147 [149] |  |

| Condition | Explanation |
| --- | --- |
| A1 | eCall over IMS was started manually |
| A2 | eCall over IMS was started automatically |
| A3 | UE able to provide an updated MSD |
| A4 | UE not able to provide an updated MSD |

## A.2.20	MT INFO for eCall over IMS / PSAP callback over IMS

| Header/param |  | Cond |  | Value/remark |  | Rel |  | Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request-Line |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| Method |  |  |  | INFO |  |  |  | RFC 6086 [139] |  |
| Request-URI |  |  |  | same URI as UE sent in Contact header of INVITE |  |  |  | RFC 8147 [149] |  |
| SIP-Version |  |  |  | SIP/2.0 |  |  |  |  |  |
| Via |  |  |  | As specified for the INVITE in A.2.9 for conditions A11 and A12 |  |  |  |  |  |
|  |  | A1 |  | As specified for the INVITE in A.2.9 |  |  |  |  |  |
| From |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | SIP URI of the SS URI must be the same as used for the endpoint in the earlier requests within the dialog. |  |  |  |  |  |
| tag |  |  |  | remote tag of the dialog ID |  |  |  |  |  |
| To |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | SIP URI of UE URI must be the same as used for the endpoint in the earlier requests within the dialog. |  |  |  | TS 24.229 [10] RFC 8147 [149] |  |
| tag |  |  |  | local tag of the dialog ID |  |  |  |  |  |
| Call-ID |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| callid |  |  |  | same as value received in INVITE message |  |  |  |  |  |
| CSeq |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | value of CSeq sent by the SS within its previous request in the same dialog but increased by one. If this is first request sent by SS, any value is used (e.g. 4711). |  |  |  |  |  |
| method |  |  |  | INFO |  |  |  |  |  |
| Call-Info |  |  |  |  |  |  |  |  |  |
| info |  |  |  |  |  |  |  |  |  |
| absoluteURI |  |  |  | cid:test-info@3gpp.org |  | Rel-14 |  | RFC 8147 [149] |  |
| info-param |  |  |  | purpose=EmergencyCallData.Control |  | Rel-14 |  | RFC 8147 [149] |  |
| Info-Package |  |  |  | EmergencyCallData.eCall.MSD |  | Rel-14 |  | RFC 8147 [149] |  |
| Content-Type |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| media-type |  |  |  | multipart/mixed;boundary=boundaryXXX |  | Rel-14 |  | TS 24.229 [10] |  |
| Content-Length |  |  |  |  |  |  |  |  |  |
| value |  |  |  | length of message-body |  |  |  |  |  |
| Content-Disposition |  |  |  |  |  |  |  |  |  |
| disp-type |  |  |  | Info-Package |  | Rel-14 |  | RFC 8147 [149] |  |
| Message-body |  |  |  | --boundaryXXX Content-Type: application/EmergencyCallData.Control+xml Content-ID: <test-info@3gpp.org> Content-Disposition: by-reference <?xml version="1.0" encoding="UTF-8"?> <EmergencyCallData.Control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <request action="send-data" datatype="eCall.MSD"/> </EmergencyCallData.Control> --boundaryXXX |  | Rel-14 |  | RFC 8147 [149] |  |

| Condition | Explanation |
| --- | --- |
| A1 | SIP INFO during/after call to URI for test service |

<End of modified section>