3GPP TSG-RAN5 Meeting #106	R5-251311

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4901 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 12.3.1.2.14 Sidelink Relay / L2 U2N / Relay UE / Radio link failure related actions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of new test case 12.3.1.2.14 Sidelink Relay / L2 U2N / Relay UE / Radio link failure related actions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 12.3.1.2.14 Sidelink Relay / L2 U2N / Relay UE / Radio link failure related actions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete compared to work plan coverage. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3.1.2.14(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0565 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

12.3.1.2.14	Sidelink Relay / L2 U2N / Relay UE / Radio link failure related actions

12.3.1.2.14.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED and AS security has been activated and SRB2 and at least one DRB have been setup and acting as a L2 U2N Relay UE }

ensure that {

when { UE detects radio link failure on expiry of timer T310 }

then { UE indicates to upper layers to trigger PC5 unicast link release or sends NotificationMessageSidelink to the connected L2 U2N Remote UE and initiates the RRC connection re-establishment procedure }

}

(2)

with { UE in NR RRC_CONNECTED and acting as a L2 U2N Relay UE }

ensure that {

when { UE initiates transmission of the NotificationMessageSidelink message due to Uu RLF }

then { UE sets the indicationType as relayUE-Uu-RLF in NotificationMessageSidelink }

}

12.3.1.2.14.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.3.10.3, 5.8.9.10.2, 5.8.9.10.3, 5.3.7.2 and TS 24.554 clause 7.2.6.1. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.3.10.3]

The UE shall:

…

1>	else:

2>	during a DAPS handover: the following only applies for the target PCell;

2>	upon T310 expiry in PCell; or

…

...

3>	else:

...

4>	if AS security has not been activated:

5>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';-

4>	else if AS security has been activated but SRB2 and at least one DRB or multicast MRB or, for IAB, SRB2, have not been setup:

5>	store the radio link failure information in the VarRLF-Report as described in clause 5.3.10.5;

5>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

4>	else:

5>	store the radio link failure information in the VarRLF-Report as described in clause 5.3.10.5;

5>	if T316 is configured; and

5>	if SCG transmission is not suspended; and

5>	if the SCG is not deactivated; and

5>	if neither PSCell change nor PSCell addition is ongoing (i.e. timer T304 for the NR PSCell is not running in case of NR-DC or timer T307 of the E-UTRA PSCell is not running as specified in TS 36.331 [10], clause 5.3.10.10, in NE-DC):

6>	initiate the MCG failure information procedure as specified in 5.7.3b to report MCG radio link failure.

5>	else:

6>	initiate the connection re-establishment procedure as specified in 5.3.7.

A L2/L3 U2N Relay UE shall:

1>	upon detecting radio link failure:

2>	either indicate to upper layers (to trigger PC5 unicast link release) or send NotificationMessageSidelink to the connected L2/L3 U2N Remote UE(s) in accordance with 5.8.9.10.

[TS 38.331, clause 5.8.9.10.2]

The U2N Relay UE may initiate the procedure when one of the following conditions is met:

1>	upon Uu RLF as specified in 5.3.10;

1>	upon reception of an RRCReconfiguration including the reconfigurationWithSync;

1>	upon cell reselection;

1>	upon L2 U2N Relay UE's RRC connection failure including RRC connection reject as specified in 5.3.3.5 and 5.3.13.10, and T300 expiry as specified in 5.3.3.7, and RRC resume failure as specified in 5.3.13.5;

[TS 38.331, clause 5.8.9.10.3]

The U2N Relay UE shall set the indication type as follows:

1>	if the UE initiates transmission of the NotificationMessageSidelink message due to Uu RLF:

2>	set the indicationType as relayUE-Uu-RLF;

1>	else if the UE initiates transmission of the NotificationMessageSidelink message due to reconfiguration with sync:

2>	set the indicationType as relayUE-HO;

1>	else if the UE initiates transmission of the NotificationMessageSidelink message due to cell reselection:

2>	set the indicationType as relayUE-CellReselection;

1>	if the UE initiates transmission of the NotificationMessageSidelink message due to Uu RRC connection establishment/Resume failure:

2>	set the indicationType as relayUE-Uu-RRC-Failure;

1>	submit the NotificationMessageSidelink message to lower layers for transmission.

[TS 38.331, clause 5.3.7.2]

The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure of the MCG and t316 is not configured, in accordance with 5.3.10; or

…

1>	if UE is not configured with attemptCondReconfig:

2>	reset MAC;

2>	release spCellConfig, if configured;

2>	suspend all RBs, and BH RLC channels for IAB-MT, and Uu Relay RLC channels for L2 U2N Relay UE, except SRB0 and broadcast MRBs;

…

1>	release sl-L2RelayUE-Config, if configured;

…

1>	release the SRAP entity, if configured;

1>	if the UE is acting as L2 U2N Remote UE:

…

1> else:

2>	if the UE is capable of L2 U2N Remote UE:

3>	perform either cell selection as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both;

2>	else:

3>	perform cell selection in accordance with the cell selection process as specified in TS 38.304 [20].

[TS 24.554, clause 7.2.6.1]

If the UE receives an indication of radio link failure or an indication of PC5-RRC connection release from the lower layer, the UE shall release the 5G ProSe direct link locally and may delete the KNRP ID associated with this link after an implementation specific time. If the UE acting as 5G ProSe layer-2 UE-to-network relay UE or 5G ProSe layer-2 remote UE receives a request of 5G ProSe direct link release from the lower layer as specified in 3GPP TS 38.331 [13], the UE shall initiate the 5G ProSe direct link release procedure.

12.3.1.2.14.3	Test Description

12.3.1.2.14.3.1	Pre-test conditions

System Simulator:

-	SS-NW

-	2 NR Cells. NR Cell 1 is the serving cell, NR Cell 2 is the intra-frequency neighbour cell.

-	System information combination NR-31 as defined in TS 38.508-1 [4] clause 4.4.3.1 is used.

-	NR-SS-UE

-	NR-SS-UE1 operating as L2 U2N Remote UE.

-	NR-SS-UE1 is synchronised on GNSS.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

- The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.4 and ProSeP using condition Announcing_Restricted_Model_A as per TS 38.508-1 [4] clause 4.7C.1.

- UE is synchronised on GNSS.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

12.3.1.2.14.3.2	Test procedure sequence

Table 12.3.1.2.14.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the NR cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configuration marked "T1" is applied at the points indicated in the Main behaviour description in Table 12.3.1.2.14.3.2-2.

Table 12.3.1.2.14.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | “Off” |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | -88 |  |
| Note: Power level "Off" is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |  |  |  |  |  |

Table 12.3.1.2.14.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The procedure for 5G ProSe Layer-2 U2N Relay initial access / Relay UE side in Table 4.9.41.2.2-1 in TS 38.508-1[4] is performed. | - | - | - | - |
| 2 | The SS-NW adjusts the cell power levels according to row "T1" in Table 12.3.1.2.14.3.2-1. | - | - | - | - |
| - | EXCEPTION: In parallel to events described in steps 3a1 to 3b1 the steps specified in Table 12.3.1.2.14.3.2-3 take place. | - | - | - | - |
| - | EXCEPTION: Steps 3a1 to 3b1 describe a transaction that depends on the UE behaviour; the "lower case letter" identifies a step sequence that takes place if a specific behaviour happens. (Note 1) | - | - | - | - |
| 3a1 | Check: Does the UE transmit a PROSE DIRECT LINK RELEASE REQUEST message to NR-SS-UE1? | --> | PC5-S: PROSE DIRECT LINK RELEASE REQUEST | 1 | P |
| 3a2 | The NR-SS-UE1 sends a PROSE DIRECT LINK RELEASE ACCEPT message. | <-- | PC5-S: PROSE DIRECT LINK RELEASE ACCEPT | - | - |
| 3b1 | Check: Does the UE transmit a NotificationMessageSidelink message to NR-SS-UE1 with the indicationType set as relayUE-Uu-RLF? | --> | PC5 RRC: NotificationMessageSidelink | 1,2 | P |
| NOTE: UE shall either indicate to upper layers (to trigger PC5 unicast link release) or send NotificationMessageSidelink to the connected L2/L3 U2N Remote UE(s). |  |  |  |  |  |

Table 12.3.1.2.14.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE send an RRCReestablishmentRequest message on NR Cell 2? | --> | NR RRC: RRCReestablishmentRequest | 1 | P |
| 2 | The SS transmits an RRCReestablishment message. | <-- | NR RRC: RRCReestablishment | - | - |
| 3 | The UE transmits an RRCReestablishmentComplete message. | --> | NR RRC: RRCReestablishmentComplete | - | - |

12.3.1.2.14.3.3	Specific message contents

Table 12.3.1.2.14.3.3-1: PROSE DIRECT LINK RELEASE REQUEST (Step 3a1, Table 12.3.1.2.14.3.2-2)

Derivation Path: TS 38.508-1 [4], Table 4.7C.3-4 with condition TX

Table 12.3.1.2.14.3.3-2: PROSE DIRECT LINK RELEASE ACCEPT (Step 3a2, Table 12.3.1.2.14.3.2-2)

Derivation Path: TS 38.508-1 [4], Table 4.7C.3-5 with condition RX

Table 12.3.1.2.14.3.3-3: NotificationMessageSidelink (Step 3b1, Table 12.3.1.2.14.3.2-2)

Derivation Path: TS 38.508-1 [4], Table 4.6.1A-2B with condition Uu_RLF

<End of Changed Section>