3GPP TSG-RAN5 Meeting #106	R5-251351

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5421 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.3.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the Table 22.3.1.1.3.2-2, UE transmits the RRC Connection Request at step 8 and receives successful RRCConnectionSetup-NB at step 17B. T300 is configured as ms1000 and this is not sufficient to cover the NTN execution due to the long propagation delay (K-offset set to 550), therefore it is proposed to increase the value to ms25000 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced SIB2 table to updated the value of t300-r13 to ms25000 in SIB2. Updated the table 22.3.1.1.3.3-5 to include the t300-r13 to be set to ms25000 for NTN, and ms1000 for non-NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is final version of R5-250525r1 |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 2.3.1.1	NB-IoT / RACH Procedure / Preamble Selected by MAC / Temporary C-RNTI

##### 22.3.1.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE has need to send a CCCH UL MAC PDU and UE is in enhanced coverage level 0}

then { UE transmits a random access preamble repeated numRepetitionPerPreambleAttempt from Random Access Preambles group and the NPRACH resource corresponding to enhanced coverage level 0}

}

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE has need to send a CCCH UL MAC PDU and UE is in enhanced coverage level 1}

then { UE transmits a random access preamble repeated numRepetitionPerPreambleAttempt from Random Access Preambles group and the NPRACH resource corresponding to enhanced coverage level 1}

}

(3)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE has need to send a CCCH UL MAC PDU and UE is in enhanced coverage level 2}

then { UE transmits a random access preamble repeated numRepetitionPerPreambleAttempt from Random Access Preambles group and the NPRACH resource corresponding to enhanced coverage level 2}

}

(4)

with { UE in E-UTRA RRC_IDLE state after transmission of a NPRACH preamble as per enhanced coverage level 0 or 1}

ensure that {

when { SS does not answer with a matching Random Access Response but only non-matching RAR within ra-ResponseWindowSize and PREAMBLE_TRANSMISSION_COUNTER_CE = maxNumPreambleAttemptCE for the corresponding coverage level + 1}

then { UE transmits a random access preamble repeated numRepetitionPerPreambleAttempt from Random Access Preambles group and the NPRACH resource corresponding to enhanced coverage level 1, 2 respectively}

}

(5)

with { UE in E-UTRA RRC_IDLE state after transmission of a NPRACH preamble }

ensure that {

when { SS does not answer with a matching Random Access Response but only non matching RAR within ra-ResponseWindowSize and PREAMBLE_TRANSMISSION_COUNTER <= preambleTransMax-CE }

then { UE retransmits the NPRACH preamble }

}

(6)

with { UE in E-UTRA RRC_IDLE state and transmitted NPRACH preamble }

ensure that {

when { UE receives during TTI window [RA_WINDOW_BEGIN—RA_WINDOW_END] MAC PDU containing multiple RAR’s but none of the subheaders contains a RAPID corresponding to the UE }

then { UE transmits a random access preamble in the next available Random Access occasion }

}

(7)

with { UE in E-UTRA RRC_IDLE state and transmitted NPRACH preamble }

ensure that {

when { SS transmits during RA Response window MAC PDU containing multiple RAR’s and one of the subheaders contains a RAPID corresponding to the UE }

then { UE transmits MAC PDU containing RRCConnectionRequest-NB and including Data Volume and Power Headroom Report (DPR) MAC control element }

}

(8)

with { UE in E-UTRA RRC_IDLE state and has transmitted Msg3 }

ensure that {

when { SS does not respond before contention resolution timer expiry }

then { UE transmits a random access preamble using a preamble in the same group of random access preambles as used for the previous preamble transmission }

}

(9)

with { UE in E-UTRA RRC_IDLE state and after transmitting a RACH MSG3 containing RRCConnectionRequest-NB message }
ensure that {
  when { SS transmits a valid MAC PDU including 'UE Contention Resolution Identity' MAC control element but with un-matched 'Contention Resolution Identity' } 
    then { UE reinitiates RACH procedure } 
            }

(10)

Void

(11)

with { UE in E-UTRA RRC_IDLE state and after transmitting a RACH MSG3 containing RRCConnectionRequest-NB message } 
ensure that { 
  when { SS transmits a valid MAC PDU containing RRCConnectionSetup-NB, including 'UE Contention Resolution Identity' MAC control element with matched 'Contention Resolution Identity' } 
    then { UE completes RACH procedure } 
            }

(12)

with { UE in E-UTRA RRC_IDLE state and having initiated a random access procedure }

ensure that {

when { The SS transmits a Timing Advance Command in a Random Access Response message }

then {  the UE applies the received Timing Advance value in the next transmitted MAC PDU}

}

##### 22.3.1.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.321, clauses 5.1.1, 5.1.2, 5.1.3, 5.1.4, 5.1.5, 5.4.5a, 6.1.3.10.

[TS 36.321, clause 5.1.1]

The Random Access procedure described in this subclause is initiated by a PDCCH order, by the MAC sublayer itself or by the RRC sublayer. Random Access procedure on an SCell shall only be initiated by a PDCCH order. If a MAC entity receives a PDCCH transmission consistent with a PDCCH order [5] masked with its C-RNTI, and for a specific Serving Cell, the MAC entity shall initiate a Random Access procedure on this Serving Cell. For Random Access on the SpCell a PDCCH order or RRC optionally indicate the ra-PreambleIndex and the ra-PRACH-MaskIndex, except for NB-IoT where the subcarrier index is indicated; and for Random Access on an SCell, the PDCCH order indicates the ra-PreambleIndex with a value different from 000000 and the ra-PRACH-MaskIndex. For the pTAG preamble transmission on PRACH and reception of a PDCCH order are only supported for SpCell. If the UE is an NB-IoT UE and is configured with a non-anchor carrier, perform the Random Access procedure on the anchor carrier.

Before the procedure can be initiated, the following information for related Serving Cell is assumed to be available for UEs other than NB-IoT UEs, BL UEs or UEs in enhanced coverage [8], unless explicitly stated otherwise:

-	the available set of PRACH resources for the transmission of the Random Access Preamble, prach-ConfigIndex.

-	the groups of Random Access Preambles and the set of available Random Access Preambles in each group (SpCell only):

The preambles that are contained in Random Access Preambles group A and Random Access Preambles group B are calculated from the parameters numberOfRA-Preambles and sizeOfRA-PreamblesGroupA:

If sizeOfRA-PreamblesGroupA is equal to numberOfRA-Preambles then there is no Random Access Preambles group B. The preambles in Random Access Preamble group A are the preambles 0 to sizeOfRA-PreamblesGroupA – 1 and, if it exists, the preambles in Random Access Preamble group B are the preambles sizeOfRA-PreamblesGroupA to numberOfRA-Preambles – 1 from the set of 64 preambles as defined in [7].

-	if Random Access Preambles group B exists, the thresholds, messagePowerOffsetGroupB and messageSizeGroupA, the configured UE transmitted power of the Serving Cell performing the Random Access Procedure, PCMAX, c [10], and the offset between the preamble and Msg3, deltaPreambleMsg3, that are required for selecting one of the two groups of Random Access Preambles (SpCell only).

-	the RA response window size ra-ResponseWindowSize.

-	the power-ramping factor powerRampingStep.

-	the maximum number of preamble transmission preambleTransMax.

-	the initial preamble power preambleInitialReceivedTargetPower.

-	the preamble format based offset DELTA_PREAMBLE (see subclause 7.6).

-	the maximum number of Msg3 HARQ transmissions maxHARQ-Msg3Tx (SpCell only).

-	the Contention Resolution Timer mac-ContentionResolutionTimer (SpCell only).

NOTE:	The above parameters may be updated from upper layers before each Random Access procedure is initiated.

The following information for related Serving Cell is assumed to be available before the procedure can be initiated for NB-IoT UEs, BL UEs or UEs in enhanced coverage [8]:

-	…

-	if the UE is a NB-IoT UE:

-	the available set of PRACH resources supported in the Serving Cell, nprach-ParametersList.

-	for random access resource selection and preamble transmission:

-	a PRACH resource is mapped into an enhanced coverage level.

-	each PRACH resource contains a set of nprach-NumSubcarriers subcarriers which can be partitioned into one or two groups for single/multi-tone Msg3 transmission by nprach-SubcarrierMSG3-RangeStart. Each group is referred to as a Random Access Preamble group below in the procedure text.

-	a subcarrier is identified by the subcarrier index in the range:
[nprach-SubcarrierOffset, nprach-SubcarrierOffset + nprach-NumSubcarriers -1]

-	each subcarrier of a Random Access Preamble group corresponds to a Random Access Preamble.

-	when the subcarrier index is explicitly sent from the eNB as part of a PDCCH order ra-PreambleIndex shall be set to the signalled subcarrier index.

-	the mapping of the PRACH resources into enhanced coverage levels is determined according to the following:

-	the number of enhanced coverage levels is equal to one plus the number of RSRP thresholds present in RSRP-ThresholdsPrachInfoList.

-	each enhanced coverage level has one PRACH resource present in nprach-ParametersList.

-	enhanced coverage levels are numbered from 0 and the mapping of PRACH resources to enhanced coverage levels are done in increasing numRepetitionsPerPreambleAttempt order.

-	the criteria to select PRACH resources based on RSRP measurement per enhanced coverage level supported in the Serving Cell rsrp-ThresholdsPrachInfoList.

-	the maximum number of preamble transmission attempts per enhanced coverage level supported in the Serving Cell maxNumPreambleAttemptCE.

-	the number of repetitions required for preamble transmission per attempt for each enhanced coverage level supported in the Serving Cell numRepetitionPerPreambleAttempt.

-	the configured UE transmitted power of the Serving Cell performing the Random Access Procedure, PCMAX, c [10].

-	the RA response window size ra-ResponseWindowSize and the Contention Resolution Timer mac-ContentionResolutionTimer (SpCell only) per enhanced coverage level supported in the Serving Cell.

-	the power-ramping factor powerRampingStep.

-	the maximum number of preamble transmission preambleTransMax-CE.

-	the initial preamble power preambleInitialReceivedTargetPower.

-	the preamble format based offset DELTA_PREAMBLE (see subclause 7.6). For NB-IoT the DELTA_PREAMBLE is set to 0.

The Random Access procedure shall be performed as follows:

-	Flush the Msg3 buffer;

-	set the PREAMBLE_TRANSMISSION_COUNTER to 1;

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	set the PREAMBLE_TRANSMISSION_COUNTER_CE to 1;

-	if the starting enhanced coverage level, or for NB-IoT the initial number of PRACH repetitions, has been indicated in the PDCCH order which initiated the Random Access procedure, or if the starting enhanced coverage level has been provided by upper layers:

-	the MAC entity considers itself to be in that enhanced coverage level regardless of the measured RSRP;

-	else:

-	if the RSRP threshold of enhanced coverage level 3 is configured by upper layers in rsrp-ThresholdsPrachInfoList and the measured RSRP is less than the RSRP threshold of enhanced coverage level 3 and the UE is capable of enhanced coverage level 3 then:

-	the MAC entity considers to be in enhanced coverage level 3;

-	else if the RSRP threshold of enhanced coverage level 2 configured by upper layers in rsrp-ThresholdsPrachInfoList and the measured RSRP is less than the RSRP threshold of enhanced coverage level 2 and the UE is capable of enhanced coverage level 2 then:

-	the MAC entity considers to be in enhanced coverage level 2;

-	else if the measured RSRP is less than the RSRP threshold of enhanced coverage level 1 as configured by upper layers in rsrp-ThresholdsPrachInfoList then:

-	the MAC entity considers to be in enhanced coverage level 1;

-	else:

-	the MAC entity considers to be in enhanced coverage level 0;

-	set the backoff parameter value to 0 ms;

-	for the RN, suspend any RN subframe configuration;

-	proceed to the selection of the Random Access Resource (see subclause 5.1.2).

NOTE:	There is only one Random Access procedure ongoing at any point in time in a MAC entity. If the MAC entity receives a request for a new Random Access procedure while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure.

[TS 36.321, clause 5.1.2]

The Random Access Resource selection procedure shall be performed as follows:

-	…

-	else, for NB-IoT, if ra-PreambleIndex (Random Access Preamble) and PRACH resource have been explicitly signalled:

-	the PRACH resource is that explicitly signalled;

-	if the ra-PreambleIndex signalled is not 000000:

-	the Random Access Preamble is set to nprach-SubcarrierOffset + (ra-PreambleIndex modulo nprach-NumSubcarriers), where nprach-SubcarrierOffset and nprach-NumSubcarriers are parameters in the currently used PRACH resource.

-	else:

-	select the Random Access Preamble group according to the PRACH resource and the support for multi-tone Msg3 transmission.

-	randomly select a Random Access Preamble within the selected group.

-	else the Random Access Preamble shall be selected by the MAC entity as follows:

-	If Msg3 has not yet been transmitted, the MAC entity shall, for NB-IoT UEs, BL UEs or UEs in enhanced coverage:

-	expect for NB-IoT, select the Random Access Preambles group and the PRACH resource corresponding to the selected enhanced coverage level;

-	for NB-IoT, select the PRACH resource corresponding to the selected enhanced coverage level, and select the Random Access Preambles group corresponding to the PRACH resource and the support for multi-tone Msg3 transmission;

-	…

-	except for NB-IoT, set PRACH Mask Index to 0.

-	determine the next available subframe containing PRACH permitted by the restrictions given by the prach-ConfigIndex (except for NB-IoT), the PRACH Mask Index (except for NB-IoT, see subclause 7.3), physical layer timing requirements [2] and in case of NB-IoT, the subframes occupied by PRACH resources related to a higher enhanced coverage level (a MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH subframe);

-	if the transmission mode is TDD and the PRACH Mask Index is equal to zero:

-	…

-	else:

-	determine a PRACH within the determined subframe in accordance with the requirements of the PRACH Mask Index, if any.

-	for NB-IoT UEs, BL UEs or UEs in enhanced coverage, select the ra-ResponseWindowSize and mac-ContentionResolutionTimer corresponding to the selected enhanced coverage level and PRACH.

-	proceed to the transmission of the Random Access Preamble (see subclause 5.1.3).

[TS 36.321, clause 5.1.3]

The random-access procedure shall be performed as follows:

-	set PREAMBLE_RECEIVED_TARGET_POWER to preambleInitialReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_TRANSMISSION_COUNTER – 1) * powerRampingStep;

-	if the UE is a BL UE or a UE in enhanced coverage:

-	the PREAMBLE_RECEIVED_TARGET_POWER is set to:
PREAMBLE_RECEIVED_TARGET_POWER - 10 * log10(numRepetitionPerPreambleAttempt);

-	if NB-IoT:

-	for enhanced coverage level 0, the PREAMBLE_RECEIVED_TARGET_POWER is set to:
 PREAMBLE_RECEIVED_TARGET_POWER - 10 * log10(numRepetitionPerPreambleAttempt)

-	for other enhanced coverage levels, the PREAMBLE_RECEIVED_TARGET_POWER is set corresponding to the max UE output power;

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	instruct the physical layer to transmit a preamble with the number of repetitions required for preamble transmission corresponding to the selected preamble group (i.e., numRepetitionPerPreambleAttempt) using the selected PRACH corresponding to the selected enhanced coverage level, corresponding RA-RNTI, preamble index or for NB-IoT subcarrier index, and PREAMBLE_RECEIVED_TARGET_POWER.

-	else:

-	instruct the physical layer to transmit a preamble using the selected PRACH, corresponding RA-RNTI, preamble index and PREAMBLE_RECEIVED_TARGET_POWER.

[TS 36.321, clause 5.1.4]

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap or a Sidelink Discovery Gap for Transmission or a Sidelink Discovery Gap for Reception, the MAC entity shall monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI defined below, in the RA Response window which starts at the subframe that contains the end of the preamble transmission [7] plus three subframes and has length ra-ResponseWindowSize. If the UE is a BL UE or a UE in enhanced coverage, RA Response window starts at the subframe that contains the end of the last preamble repetition plus three subframes and has length ra-ResponseWindowSize for the corresponding coverage level. If the UE is an NB-IoT UE, in case the number of NPRACH repetitions is greater than or equal to 64, RA Response window starts at the subframe that contains the end of the last preamble repetition plus 41 subframes and has length ra-ResponseWindowSize for the corresponding coverage level, and in case the number of NPRACH repetitions is less than 64, RA Response window starts at the subframe that contains the end of the last preamble repetition plus 4 subframes and has length ra-ResponseWindowSize for the corresponding coverage level.

…

For NB-IoT UEs, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as:

RA-RNTI=1+ floor(SFN_id/4)

where SFN_id is the index of the first radio frame of the specified PRACH.

The MAC entity may stop monitoring for Random Access Response(s) after successful reception of a Random Access Response containing Random Access Preamble identifiers that matches the transmitted Random Access Preamble.

-	If a downlink assignment for this TTI has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded, the MAC entity shall regardless of the possible occurrence of a measurement gap or a Sidelink Discovery Gap for Transmission or a Sidelink Discovery Gap for Reception:

-	if the Random Access Response contains a Backoff Indicator subheader:

-	set the backoff parameter value as indicated by the BI field of the Backoff Indicator subheader and Table 7.2-1, except for NB-IoT where the value from Table 7.2-2 is used.

-	else, set the backoff parameter value to 0 ms.

-	if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble (see subclause 5.1.3), the MAC entity shall:

-	consider this Random Access Response reception successful and apply the following actions for the serving cell where the Random Access Preamble was transmitted:

-	process the received Timing Advance Command (see subclause 5.2);

-	indicate the preambleInitialReceivedTargetPower and the amount of power ramping applied to the latest preamble transmission to lower layers (i.e., (PREAMBLE_TRANSMISSION_COUNTER – 1) * powerRampingStep);

-	process the received UL grant value and indicate it to the lower layers;

-	if ra-PreambleIndex was explicitly signalled and it was not 000000 (i.e., not selected by MAC):

-	consider the Random Access procedure successfully completed.

-	else, if the Random Access Preamble was selected by the MAC entity:

-	set the Temporary C-RNTI to the value received in the Random Access Response message no later than at the time of the first transmission corresponding to the UL grant provided in the Random Access Response message;

-	if this is the first successfully received Random Access Response within this Random Access procedure:

-	if the transmission is not being made for the CCCH logical channel, indicate to the Multiplexing and assembly entity to include a C-RNTI MAC control element in the subsequent uplink transmission;

-	obtain the MAC PDU to transmit from the "Multiplexing and assembly" entity and store it in the Msg3 buffer.

NOTE:	When an uplink transmission is required, e.g., for contention resolution, the eNB should not provide a grant smaller than 56 bits (or 88 bits for NB-IoT) in the Random Access Response.

NOTE:	If within a Random Access procedure, an uplink grant provided in the Random Access Response for the same group of Random Access Preambles has a different size than the first uplink grant allocated during that Random Access procedure, the UE behaviour is not defined.

If no Random Access Response is received within the RA Response window, or if none of all received Random Access Responses contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble, the Random Access Response reception is considered not successful and the MAC entity shall:

-	if the notification of power ramping suspension has not been received from lower layers:

-	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax-CE + 1:

-	if the Random Access Preamble is transmitted on the SpCell:

-	indicate a Random Access problem to upper layers;

-	if NB-IoT:

-	consider the Random Access procedure unsuccessfully completed;

-	else:

-	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

-	if the Random Access Preamble is transmitted on the SpCell:

-	indicate a Random Access problem to upper layers;

-	if the Random Access Preamble is transmitted on an SCell:

-	consider the Random Access procedure unsuccessfully completed.

-	if in this Random Access procedure, the Random Access Preamble was selected by MAC:

-	based on the backoff parameter, select a random backoff time according to a uniform distribution between 0 and the Backoff Parameter Value;

-	delay the subsequent Random Access transmission by the backoff time;

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	increment PREAMBLE_TRANSMISSION_COUNTER_CE by 1;

-	if PREAMBLE_TRANSMISSION_COUNTER_CE = maxNumPreambleAttemptCE for the corresponding enhanced coverage level + 1:

-	reset PREAMBLE_TRANSMISSION_COUNTER_CE;

-	consider to be in the next enhanced coverage level, if it is supported by the Serving Cell and the UE, otherwise stay in the current enhanced coverage level;

-	select the Random Access Preambles group, ra-ResponseWindowSize, mac-ContentionResolutionTimer, and PRACH resource corresponding to the selected enhanced coverage level;

-	if the UE is an NB-IoT UE:

-	if the Random Access Procedure was initiated by a PDCCH order:

-	consider the PRACH resource corresponding to the selected enhanced coverage level as explicitly signalled;

-	proceed to the selection of a Random Access Resource (see subclause 5.1.2).

[TS 36.321, clause 5.1.5]

Contention Resolution is based on either C-RNTI on PDCCH of the SpCell or UE Contention Resolution Identity on DL-SCH. If the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage, the MAC entity shall use the mac-ContentionResolutionTimer for the corresponding enhanced coverage level if it exists.

Once Msg3 is transmitted, the MAC entity shall:

-	start mac-ContentionResolutionTimer and restart mac-ContentionResolutionTimer at each HARQ retransmission;

-	regardless of the possible occurrence of a measurement gap or Sidelink Discovery Gap for Reception, monitor the PDCCH until mac-ContentionResolutionTimer expires or is stopped;

-	if notification of a reception of a PDCCH transmission is received from lower layers, the MAC entity shall:

-	if the C-RNTI MAC control element was included in Msg3:

-	if the Random Access procedure was initiated by the MAC sublayer itself or by the RRC sublayer and the PDCCH transmission is addressed to the C-RNTI and contains an UL grant for a new transmission; or

-	if the Random Access procedure was initiated by a PDCCH order and the PDCCH transmission is addressed to the C-RNTI:

-	consider this Contention Resolution successful;

-	stop mac-ContentionResolutionTimer;

-	discard the Temporary C-RNTI;

-	if the UE is an NB-IoT UE and is configured with a non-anchor carrier:

-	the UL grant or DL assignment contained in the PDCCH transmission on the anchor carrier is valid only for the non-anchor carrier.

-	consider this Random Access procedure successfully completed.

-	else if the CCCH SDU was included in Msg3 and the PDCCH transmission is addressed to its Temporary C-RNTI:

-	if the MAC PDU is successfully decoded:

-	stop mac-ContentionResolutionTimer;

-	if the MAC PDU contains a UE Contention Resolution Identity MAC control element; and

-	if the UE Contention Resolution Identity included in the MAC control element matches the 48 first bits of the CCCH SDU transmitted in Msg3:

-	consider this Contention Resolution successful and finish the disassembly and demultiplexing of the MAC PDU;

-	set the C-RNTI to the value of the Temporary C-RNTI;

-	discard the Temporary C-RNTI;

-	consider this Random Access procedure successfully completed.

-	else

-	discard the Temporary C-RNTI;

-	consider this Contention Resolution not successful and discard the successfully decoded MAC PDU.

-	if mac-ContentionResolutionTimer expires:

-	discard the Temporary C-RNTI;

-	consider the Contention Resolution not successful.

-	if the Contention Resolution is considered not successful the MAC entity shall:

-	flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer;

-	if the notification of power ramping suspension has not been received from lower layers:

-	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax-CE + 1:

-	indicate a Random Access problem to upper layers.

-	if NB-IoT:

-	consider the Random Access procedure unsuccessfully completed;

-	else:

-	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

-	indicate a Random Access problem to upper layers.

-	based on the backoff parameter, select a random backoff time according to a uniform distribution between 0 and the Backoff Parameter Value;

-	delay the subsequent Random Access transmission by the backoff time;

-	proceed to the selection of a Random Access Resource (see subclause 5.1.2).

[TS 36.321, clause 5.4.5a]

The Data Volume and Power Headroom reporting procedure is only applicable for NB-IoT UEs and is used to provide the serving eNB with information about the amount of data available for transmission in the UL buffers associated with the MAC entity, and to provide the serving eNB with information about the difference between the nominal UE maximum transmission power and the estimated transmission power for UL-SCH transmission for the Serving Cell. The reporting is done using the DPR MAC control element, which is sent in Msg3 together with a CCCH SDU.

[TS 36.321, clause 6.1.3.10]

The Data Volume and Power Headroom Report (DPR) MAC control element is identified by the MAC PDU subheader used for the CCCH MAC SDU, as specified in table 6.2.1-2. It does not add any additional subheader and is always placed before the CCCH MAC SDU.

It has a fixed size and consists of a single octet defined as follows (figure 6.1.3.10-1):

-	Data Volume (DV): The Data Volume field identifies the total amount of data available across all logical channels and of data not yet associated with a logical channel after all MAC PDUs for the TTI have been built. The amount of data is indicated in number of bytes. It shall include all data that is available for transmission in the RLC layer, in the PDCP layer, and in the RRC layer; the definition of what data shall be considered as available for transmission is specified in [3], [4] and [8] respectively. The size of the RLC and MAC headers are not considered in the buffer size computation. The length of this field is 4 bits. The values taken by the Data Volume field are shown in Table 6.1.3.10-1;

-	Power Headroom (PH): This field indicates the power headroom level. The length of the field is 2 bits. The reported PH and the corresponding power headroom levels are shown in Table 6.1.3.10-2 below (the corresponding measured values in dB can be found in [9]);

-	R: reserved bit, set to "0".

##### 22.3.1.1.3	Test description

22.3.1.1.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

UE:

None.

Preamble:

-	UE is in state Registered, Idle Mode (state 3-NB) according to [18] in Ncell 1.

22.3.1.1.3.2	Test procedure sequence

Table 22.3.1.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the Ncells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1, T2 and T3" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 22.3.1.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Ncell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | NRS-EPRE | dBm/15kHz | -70 | The power level values are such that enhanced coverage level 0 |
| T1 | NRS-EPRE | dBm/15kHz | -90 | The power level values are such that UE is in enhanced coverage level 1 |
| T2 | NRS-EPRE | dBm/15kHz | -110 | The power level values are such that enhanced coverage level 2 |

Table 22.3.1.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | Exception: Steps 0-17E are repeated once for each time instant specified in table 22.3.1.1.3.2-1 by applying the Ncell 1 power level as per time instance. | - | - | - | - |
| 0 | The SS waits 15s to allow time to the UE to measure Ncell power level. |  |  |  |  |
| 1 | The SS transmits a Paging message including a matched identity. | <-- | MAC PDU (Paging) | - | - |
| - | Exception: Steps 2, 4, 6, 9 & 13 is repeated for numRepetitionsPerPreambleAttempt-r13 times configured for corresponding CE level | - | - | - | - |
| 2 | Check: Does the UE transmit a preamble on NPRACH? NOTE: The RACH preamble is calculated based on same nprach-ParametersList-r13 parameters for corresponding CE level defined in SystemInformationBlockType2-NB | --> | NPRACH Preamble | 1,2,3 | P |
| 3 | The SS transmits Random Access Response with RAPID different to the transmitted Preamble in step 2, including T-CRNTI and not including Back off Indicator sub header. | <-- | Random Access Response | - | - |
| 4 | Check: Does the UE transmit same preamble group as step 2 on NPRACH? | --> | NPRACH Preamble | 1,2,3,5 | P |
| 5 | The SS transmits a MAC PDU addressed to UE RA-RNTI, containing multiple RAR’s but none of the MAC sub headers contains a matching RAPID | <-- | Random Access Response | - | - |
| 6 | Check: Does the UE transmit same preamble group as step 2 on NPRACH? | --> | NPRACH Preamble | 1,2,3,6 | P |
| 7 | The SS transmits a MAC PDU addressed to UE RA-RNTI, containing multiple RAR’s one of the MAC sub headers contains a matching RAPID | <-- | Random Access Response | - | - |
| 8 | The UE transmits an RRCConnectionRequest-NB message including Data Volume and Power Headroom Report (DPR) MAC control element. | --> | MAC PDU (RRCConnectionRequest-NB) | 7 | P |
| 9 | Check: Does the UE transmit same preamble group as step 2 on NPRACH? | --> | NPRACH Preamble | 1,2,3,8 | P |
| 10 | The SS transmits a MAC PDU addressed to UE RA-RNTI, containing multiple RAR’s one of the MAC sub headers contains a matching RAPID | <-- | Random Access Response | - | - |
| 11 | Check: Does the UE transmits an RRCConnectionRequest-NB message including Data Volume and Power Headroom Report (DPR) MAC control element. | --> | MAC PDU (RRCConnectionRequest-NB) | 7 | P |
| 12 | The SS transmits a valid MAC PDU including a ‘UE Contention Resolution Identity’ MAC control element with unmatched ‘Contention Resolution Identity’ and a CCCH message (RRCConnectionSetup-NB). | <-- | MAC PDU (RRCConnectionSetup-NB) | - | - |
| 13 | Check: Does the UE transmit same preamble group as step 2 on NPRACH? | --> | NPRACH Preamble | 9 | P |
| 14 | Void | - | - | - | - |
| - | Exception: Steps 15 & 16 are repeated for numRepetitionsPerPreambleAttempt-r13 times configured for corresponding CE level | - | - | - | - |
| 15 | Check: Does the UE transmit a preamble on NPRACH? NOTE: The RACH preamble is calculated based on same nprach-ParametersList-r13 parameters for corresponding CE level +1 (if CE level is 0,1) or CE level 2 defined in SystemInformationBlockType2-NB | --> | NPRACH Preamble | 4 | P |
| 16 | Check: Does the UE transmit same preamble group as step 15 on NPRACH(PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax-CE (=7)+ 1)? | --> | NPRACH Preamble | 4 | P |
| 17 | The SS transmits a MAC PDU addressed to UE RA-RNTI, containing a matching RAPID. | <-- | Random Access Response |  |  |
| 17A - 17D | Steps 2 to 5 of the 'Generic Test Procedure NB-IoT Control Plane CIoT MT user data transfer non-SMS transport' as described in TS 36.508 [18] clause 8.1.5A.2.3 are performed. | - | - | - | - |
| 17E | SS transmits an RRCConnectionRelease-NB message. | <-- | MAC PDU (RRCConnectionRelease-NB) | - | - |
| 18 | SS sets Ncell 1 power level as per T0 | - | - | - | - |
| 18A | The SS waits 15s to allow time to the UE to measure Ncell power level. | - | - | - | - |
| 19 | The SS transmits a Paging message including a matched identity. | <-- | MAC PDU (Paging) | - | - |
| 20 | Check: Does the UE transmit a preamble on NPRACH? NOTE: The RACH preamble is calculated based on same nprach-ParametersList-r13 parameters for corresponding CE level defined in SystemInformationBlockType2-NB | --> | NPRACH Preamble | 1 | P |
| 21 | SS respond to UE Random Access request by a Random Access Response with TA field within message set to 600 (NOTE 1) | <-- | MAC PDU (Random Access Response (TA=600)) | - | - |
| 22 | Check: Does UE send an RRCConnectionRequest-NB message? NOTE: The UL transmission is using the Timing Advance value sent by the SS in step 21 | --> | MAC PDU (RRCConnectionRequest-NB) | 12 | P |
| 23 | The SS transmits a valid MAC PDU containing a “UE Contention Resolution Identity” MAC control element with matching “Contention Resolution Identity” and a CCCH message (RRCConnectionSetup-NB containing configuration of UE-specific search space). | <-- | MAC PDU (RRCConnectionSetup-NB) | - | - |
| 24 | Void | - | - | - | - |
| 25 | Check: Does UE send an RRCConnectionSetupComplete-NB message? NOTE: The UL transmission is using the Timing Advance value sent by the SS in step 21 | --> | MAC PDU (RRCConnectionSetupComplete-NB) | 11 | P |
| NOTE 1: TA value of 600 has been chosen arbitrarily in the middle of the range 0 to 1282 and corresponds to 0.3125 ms (timing advance in ms = 1000 x NTA x where NTA = TA 16 and seconds according to TS 36.213 and TS 36.211). |  |  |  |  |  |

22.3.1.1.3.3	Specific message contents

Table 22.3.1.1.3.3-1: NPRACH-ConfigSIB-NB-DEFAULT in SystemInformationBlockType2-NB in preamble

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPRACH-ConfigSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| rsrp-ThresholdsPrachInfoList-r13::= SEQUENCE { | 2 entries |  |  |
| RSRP-Range[1] | 61 | -79 dBm |  |
| RSRP-Range[2] | 41 | -99 dBm |  |
| } |  |  |  |
| nprach-ParametersList-r13 ::= SEQUENCE { | 3 entries | 1: CE level 0 2: CE level 1 3: CE level 2 |  |
| { |  |  |  |
| nprach-Periodicity-r13 | ms640 |  |  |
| nprach-StartTime-r13 | ms8 |  |  |
| nprach-SubcarrierOffset-r13 | n12 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | oneThird |  |  |
| maxNumPreambleAttemptCE-r13 | n3 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n1 |  |  |
| npdcch-NumRepetitions-RA-r13 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v4 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| { |  |  |  |
| nprach-Periodicity-r13 | ms640 |  |  |
| nprach-StartTime-r13 | ms32 |  |  |
| nprach-SubcarrierOffset-r13 | n12 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | oneThird |  |  |
| maxNumPreambleAttemptCE-r13 | n3 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n2 |  |  |
| npdcch-NumRepetitions-RA-r13 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v4 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| { |  |  |  |
| nprach-Periodicity-r13 | ms640 |  |  |
| nprach-StartTime-r13 | ms128 |  |  |
| nprach-SubcarrierOffset-r13 | n12 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | oneThird |  |  |
| maxNumPreambleAttemptCE-r13 | n3 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n4 |  |  |
| npdcch-NumRepetitions-RA-r13 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v4 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.1.1.3.3-1A: NPRACH-ConfigSIB-NB-v1530-DEFAULT in SystemInformationBlockType2-NB in preamble

| Derivation Path: 36.508 Table 8.1.6.3-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPRACH-ConfigSIB-NB-v1530-DEFAULT ::= SEQUENCE { |  |  | TDD |
| tdd-Parameters-r15 SEQUENCE { |  |  |  |
| nprach-ParametersListTDD-r15 SEQUENCE (SIZE (1..maxNPRACH-Resources-NB-r13)) OF SEQUENCE { | 3 entries | 1: CE level 0 2: CE level 1 3: CE level 2 |  |
| { |  |  |  |
| nprach-Parameters-r15 SEQUENCE { |  |  |  |
| nprach-Periodicity-r15 | ms640 |  |  |
| nprach-StartTime-r15 | ms10 |  |  |
| nprach-SubcarrierOffset-r15 | n12 |  |  |
| nprach-NumSubcarriers-r15 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r15 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r15 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r15 | v4 |  |  |
| npdcch-Offset-RA-r15 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r15 | n8 |  |  |
| } |  |  |  |
| } |  |  |  |
| { |  |  |  |
| nprach-Parameters-r15 SEQUENCE { |  |  |  |
| nprach-Periodicity-r15 | ms640 |  |  |
| nprach-StartTime-r15 | ms40 |  |  |
| nprach-SubcarrierOffset-r15 | n12 |  |  |
| nprach-NumSubcarriers-r15 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r15 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r15 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r15 | v4 |  |  |
| npdcch-Offset-RA-r15 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r15 | n8 |  |  |
| } |  |  |  |
| } |  |  |  |
| { |  |  |  |
| nprach-Parameters-r15 SEQUENCE { |  |  |  |
| nprach-Periodicity-r15 | ms640 |  |  |
| nprach-StartTime-r15 | ms160 |  |  |
| nprach-SubcarrierOffset-r15 | n12 |  |  |
| nprach-NumSubcarriers-r15 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r15 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r15 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r15 | v4 |  |  |
| npdcch-Offset-RA-r15 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r15 | n8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.1.1.3.3-1B: NPRACH-ConfigSIB-NB-v1550-DEFAULT in SystemInformationBlockType2-NB in preamble

| Derivation Path: 36.508 Table 8.1.6.3-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPRACH-ConfigSIB-NB-v1550-DEFAULT ::= SEQUENCE { |  |  | TDD |
| tdd-Parameters-v1550 SEQUENCE { |  |  |  |
| nprach-ParametersListTDD-v1550 SEQUENCE (SIZE (1..maxNPRACH-Resources-NB-r13)) OF SEQUENCE { | 3 entries | 1: CE level 0 2: CE level 1 3: CE level 2 |  |
| { |  |  |  |
| maxNumPreambleAttemptCE-v1550 | n3 |  |  |
| numRepetitionsPerPreambleAttempt-v1550 | n1 |  |  |
| } |  |  |  |
| { |  |  |  |
| maxNumPreambleAttemptCE-v1550 | n3 |  |  |
| numRepetitionsPerPreambleAttempt-v1550 | n2 |  |  |
| } |  |  |  |
| { |  |  |  |
| maxNumPreambleAttemptCE-v1550 | n3 |  |  |
| numRepetitionsPerPreambleAttempt-v1550 | n4 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.1.1.3.3-2: RACH-ConfigCommon-NB-DEFAULT in SystemInformationBlockType2-NB in preamble

| Derivation Path: 36.508 Table 8.1.6.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| preambleTransMax-CE-r13 | n7 |  |  |
| rach-InfoList-r13 (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF SEQUENCE { | 3 entries | 1: CE level 0 2: CE level 1 3: CE level 2 |  |
| { |  |  |  |
| ra-ResponseWindowSize-r13 | pp10 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| { |  |  |  |
| ra-ResponseWindowSize-r13 | pp10 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| { |  |  |  |
| ra-ResponseWindowSize-r13 | pp10 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.1.1.3.3-3: NPUSCH-ConfigCommon-NB-DEFAULT in SystemInformationBlockType2-NB in preamble

| Derivation Path: 36.508 Table 8.1.6.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPUSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ack-NACK-NumRepetitions-Msg4-r13 (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF SEQUENCE { | 3 entries | 1: CE level 0 2: CE level 1 3: CE level 2 |  |
| ACK-NACK-NumRepetitions-NB-r13 | r8 |  |  |
| ACK-NACK-NumRepetitions-NB-r13 | r8 |  |  |
| ACK-NACK-NumRepetitions-NB-r13 | r8 |  |  |
| } |  |  |  |

Table 22.3.1.1.3.3-4: q-RxLevMin in SystemInformationBlockType1-NB in preamble

| Derivation Path: 36.508 Table 8.1.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-NB ::= SEQUENCE { |  |  |  |
| cellSelectionInfo-r13 SEQUENCE { |  |  |  |
| q-RxLevMin-r13 | -70 (-140 dBm) |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.1.1.3.3-5: SystemInformationBlockType2-NB in preamble

| Derivation Path: 36.508 clause 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants-r13 SEQUENCE { |  |  |  |
| t300-r13 | ms25000 |  | NTN |
| t300-r13 | ms1000 |  |  |
| } |  |  |  |
| } |  |  |  |