3GPP TSG-RAN5 Meeting #106	R5-251360

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4854 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR eCall test case 11.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In this test case, UE is worked on eCall only mode, UE does not perform NAS registration when it switches on. It will perform NAS registration when Test eCall is triggered in 5GMM-DEREGISTERED.eCALL-INACTIVE state. Test eCall belongs to IMS non-emergency call. The 5G NAS core spec does not directly indicate the access category for Test eCall. Therefore, different implementation can be acceptable. One possible implementation is that when eCall is triggered in 5GMM-DEREGISTERED.eCALL-INACTIVE state, the Access attempt is only NAS registration, and after NAS registration success, UE get the NAS registration result, then UE start to perfrom IMS registration, and IMS non-emergency Call. Therefore, the RRCestablishmentcause is ‘mo-signaling’. Another possible implementation is that when Test eCall is triggered in 5GMM-DEREGISTERED.eCALL-INACTIVE state, the Access attempt are NAS registration, IMS registration, and IMS non-emergency Call.NAS registration is triggered by IMS layer. IMS layer may indicate NAS layer “IMS non-emergency Call” and “IMS registration” together. According to the latest NAS spec, Rule#5(MO MMTel voice call)/7.1(MO IMS registration related signalling)/8(UE NAS initiated 5GMM specific procedures) specified in 24.501 Table 4.4.5.2 are matched. UERule#5 is the lowest rule number, so UE select the Access category4 (= MO MMTel voice), and the RRC establishment cause is 'mo-VoiceCall'. [24.501 4.5.2] In order to determine the access category applicable for the access attempt, the NAS shall check the rules in table 4.5.2.2, and use the access category for which there is a match for barring check. If the access attempt matches more than one rule, the access category of the lowest rule number shall be selected. If the access attempt matches more than one operator-defined access category definition, the UE shall select the access category from the operator-defined access category definition with the lowest precedence value (see subclause 4.5.3). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Check in step4A is deleted |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-250581r1. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

11.5.2	eCall Only mode / T3445 / eCall inactivity procedure / Removal of eCall only restriction after a call to URI for test service / 5GS to EPS

11.5.2.1	Test Purpose (TP)

(1)

with { The UE is in the state 5GMM-DEREGISTERED.eCALL-INACTIVE }

ensure that {

when { UE is requested to make an eCall to URI for test service }

then { UE sends REGISTRATION REQUEST message with 5GS registration type set to 'initial registration' }

}

(2)

with { UE receives REGISTRATION ACCEPT message and IMS voice over PS session is supported over 3GPP access }

ensure that {

when { UE is in 5GMM-REGISTERED.NORMAL-SERVICE state and an initial registration is performed }

then { the UE establishes a new PDU session by sending an UL NAS TRANSPORT message with Request type set to "initial request" and a PDU SESSION ESTABLISHMENT REQUEST }

}

(3)

with { UE receives an RRCRelease message and enters RRC_IDLE state }

ensure that {

when { UE receives a Paging message with MT MMTEL voice call }

then {the UE answers the paging request for MT MMTEL voice call }

}

(4)

with { UE receives an RRCRelease message and enters RRC_IDLE state }

ensure that {

when { UE starts T3445 for 12Hours and the periodic registration update timer T3512 expires }

then { the UE initiates the registration procedure for mobility and periodic registration update

and indicates "periodic registration updating" in the 5GS registration type IE }

}

(5)

with { UE in state 5GMM-REGISTERED and 5GMM-IDLE on a 5GC NR cell }

ensure that {

when { UE detects a suitable EPC E-UTRA cell after the serving NGC cell becomes not suitable when eCall Inactivity timer T3445 is running }

then { UE performs a Inter-system change from N1 mode to S1 mode by initiating and successfully
completing a TAU procedure }

}

(6)

with { UE in state EMM-REGISTERED and EMM-IDLE on an E-UTRA cell  }

ensure that {

when { UE detects a suitable NGC cell after the serving E-UTRA cell becomes not suitable when eCall Inactivity timer T3445 is running }

then { UE performs a Inter-system change from S1 mode to N1 mode by initiating and successfully
completing a mobility and periodic registration update procedure }

}

(7)

with { UE is in RRC_IDLE state }

ensure that {

when { eCall Inactivity timer T3445 expires }

then { The UE performs Deregistration procedure }

}

11.5.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in TS 24.501, clauses 5.1.3.2.1.3.8, 5.3.1.3, 5.5.3, TS 24.301, clauses 10.2 and TS 38.331, clauses 5.2.2.4.2, 6.2.2. Unless otherwise stated these are Rel-16 requirements.

[TS 24.501, clause 5.1.3.2.1.3.8]

The substate 5GMM-DEREGISTERED.eCALL-INACTIVE is chosen in the UE when:

a)	the UE is configured for eCall only mode as specified in 3GPP TS 31.102 [22];

b)	timer T3444 and timer T3445 have expired or are not running;

c)	a PLMN has been selected as specified in 3GPP TS 23.122 [5];

d)	the UE does not need to perform an eCall over IMS; and

e)	the UE does not need to perform a call to a non-emergency MSISDN or URI for test or terminal reconfiguration service.

In this substate, the UE shall not initiate any signalling towards the network, except to originate an eCall over IMS, or a call to a non-emergency MSISDN or URI for test or terminal reconfiguration service.

[TS 24.501, clause 5.3.1.3]

The signalling procedure for the release of the N1 NAS signalling connection is initiated by the network.

In N1 mode, upon indication from lower layers that the access stratum connection has been released, the UE shall enter 5GMM-IDLE mode and consider the N1 NAS signalling connection released.

If the UE in 3GPP access is configured for eCall only mode as specified in 3GPP TS 31.102 [22] then:

-	if the N1 NAS signalling connection that was released had been established for eCall over IMS, the UE shall start timer T3444; and

-	if the N1 NAS signalling connection that was released had been established for a call to an HPLMN designated non-emergency MSISDN or URI for test or terminal reconfiguration service, the UE shall start timer T3445.

[TS 24.501, clause 5.5.3]

The eCall inactivity procedure is performed only in 3GPP access and applicable only to a UE configured for eCall only mode as specified in 3GPP TS 31.102 [22]. The procedure shall be started when:

a)	the UE is in any 5GMM-REGISTERED substate except substates 5GMM-REGISTERED.PLMN-SEARCH or 5GMM-REGISTERED.NO-CELL-AVAILABLE;

b)	the UE is in 5GMM-IDLE mode; and

c)	one of the following conditions applies:

1)	timer T3444 expires or is found to have already expired and timer T3445 is not running;

2)	timer T3445 expires or is found to have already expired and timer T3444 is not running; or

3)	timers T3444 and T3445 expire or are found to have already expired.

The UE shall then perform the following actions:

a)	stop other running timers (e.g. T3511, T3512);

b)	if the UE is currently registered to the network for 5GS services, perform a de-registration procedure;

c)	delete any 5G-GUTI, TAI list, last visited registered TAI, list of equivalent PLMNs, and ngKSI; and

d)	enter 5GMM-DEREGISTERED.eCALL-INACTIVE state.

[TS 24.301, clause 10.2]

| T3444 | NOTE 11 | All except EMM-NULL and 5GMM-NULL (defined in 3GPP TS 24.501 [54]) | - UE configured for eCall only mode enters EMM-IDLE mode after an eCall over IMS - UE configured for eCall only mode moves from GERAN/UTRAN to E-UTRAN with timer T3242 (see 3GPP TS 24.008 [13]) running - UE configured for eCall only mode enters 5GMM-IDLE mode (defined in 3GPP TS 24.501 [54]) after an eCall over IMS | - Removal of eCall only restriction - Intersystem change from S1 mode to A/Gb or Iu mode | Perform eCall inactivity procedure in EPS as described in clause 5.5.4. Perform eCall inactivity procedure in 5GS as described in 3GPP TS 24.501 [54]. |
| --- | --- | --- | --- | --- | --- |
| T3445 | NOTE 12 | All except EMM-NULL and 5GMM-NULL (defined in 3GPP TS 24.501 [54]) | - UE configured for eCall only mode enters EMM-IDLE mode after a call to a non-emergency MSISDN or URI for test or terminal reconfiguration service - UE configured for eCall only mode moves from GERAN/UTRAN to E-UTRAN with timer T3243 (see 3GPP TS 24.008 [13]) running - UE configured for eCall only mode enters 5GMM-IDLE mode (defined in 3GPP TS 24.501 [54]) after a call to a non-emergency MSISDN or URI for test or terminal reconfiguration service | Removal of eCall only restriction - Intersystem change from S1 mode to A/Gb or Iu mode | Perform eCall inactivity procedure in EPS as described in clause 5.5.4. Perform eCall inactivity procedure in 5GS as described in 3GPP TS 24.501 [54]. |

[TS 38.331, clause 5.2.2.4.2]

Upon receiving the SIB1 the UE shall:

1>	store the acquired SIB1;

…

4>	forward the eCallOverIMS-Support to upper layers, if present;

[TS 38.331, clause 6.2.2]

eCallOverIMS-Support

Indicates whether the cell supports eCall over IMS services as defined in TS 23.501 [32]. If absent, eCall over IMS is not supported by the network in the cell.

11.5.2.3	Test description

11.5.2.3.1	Pre-test conditions

System Simulator:

-	2 cells

-	NR Cell 1 as defined in TS 38.508-1 [4] Table 4.4.2-3. System information combination NR-6 as defined in TS 38.508-1 [4], sub-clause 4.4.3.1.2.

-	E-UTRA Cell 1 as defined in TS 36.508 [7] Table 4.4.2-2. System information combination 31 as defined in TS 36.508 [7], sub-clause 4.4.3.1.1.

UE:

-	the eCall capable UE is equipped with ‘eCall only’ enabled USIM configured as per TS 38.508-1 [4] Table 6.4.1-24.

Preamble:

-	 The UE is in test state 0-A (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2.

11.5.2.3.2	Test procedure sequence

Table 11.5.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - NR Cell 1 as "Serving cell" - E-UTRA Cell 1 as "Non-suitable "off" cell". | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | Wait for 60s for the UE to enter 5GMM-DEREGISTERED.eCALL-INACTIVE state | - | - | - | - |
| 4 | An eCall to URI for test service is initiated by dialling the eCall Test Number in the EFFDNURI field of the USIM (tel:123456). (Note 1) | - | - | - | - |
| 4A | UE transmits a RRCSetupRequest message . | --> | NR RRC: RRCSetupRequest | - | - |
| 4B | SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 5 | Check: Does the UE send REGISTRATION REQUEST message with 5GS registration type set to 'initial registration'? | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | 1 | P |
| 6-14 | Steps 5-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 15 | The SS transmits a REGISTRATION ACCEPT message and assigns value of 7 hours for the timer T3512. | <-- | 5GMM: REGISTRATION ACCEPT | - | - |
| 16 | The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message. | --> | 5GMM: REGISTRATION COMPLETE | - | - |
| 17 | Check: Does the UE transmit an ULInformationTransfer message with Request type set to "initial request" and a PDU SESSION ESTABLISHMENT REQUEST message? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | 2 | P |
| - | EXCEPTION: In parallel to the events described in steps 17A-17B below the behaviour in table 11.5.2.3.2-2 occurs. | - | - | - | - |
| 17A | The SS transmits an RRCReconfiguration message and a PDU SESSION ESTABLISHMENT ACCEPT | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT ACCEPT | - | - |
| 17B | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 18-31 | Void | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 32 to 34 below steps 2 – 3 of eCall Setup and MSD Update / 5GS as defined in TS 34.229-5 [41] annex A.23 take place | - | - | - | - |
| 32-34 | Steps 11 to 13 of Table 4.9.11.2.2-1 of the test procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 35 | Release eCall for test service using the generic procedure described in TS 38.508-1 [4] subclause 4.9.18. |  |  |  |  |
| 36 | Void | - | - | - | - |
| 37 | The SS releases the RRC connection. (Note 2) | <-- | NR RRC: RRCRelease | - | - |
| 38 | The SS waits 1 minute. | - | - | - | - |
| 39-51 | Steps 1 to 13 of the generic test procedure for IMS MT speech call establishment in 5GC specified in TS 38.508-1 [4] subclause 4.9.16 are performed. | - | - | 3 | P |
| 52 | Generic test procedure for MT release of Voice call in 5GS described in TS 38.508-1 [4] subclause 4.9.18 takes place. | - | - | - | - |
| 53 | Void | - | - | - | - |
| 54 | The SS releases the RRC connection. (Note 3) | <-- | NR RRC: RRCRelease | - | - |
| 55 | Check: Does the UE transmit a REGISTRATION REQUEST message at the expiry of T3512? | --> | 5GMM: REGISTRATION REQUEST | 4 | P |
| 56 | The SS transmits a REGISTRATION ACCEPT message including T3512 value IE. | <-- | 5GMM: REGISTRATION ACCEPT | - | - |
| 57 | The UE transmits a REGISTRATION COMPLETE message | --> | 5GMM: REGISTRATION COMPLETE |  |  |
| 58 | The SS releases the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 59 | The SS configures: - NR Cell 1 as " Non-suitable "off" cell" - E-UTRA Cell 1 as "Serving cell". | - | - | - | - |
| 60 | Check: Does the UE perform on the E-UTRA Cell 1 the TAU procedure for Inter-system change from N1 mode to S1 mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1? | - | - | 5 | P |
| 61 | The SS configures: - NR Cell 1 as "Serving cell" - E-UTRA Cell 1 as "Non-suitable "off" cell". | - | - | - | - |
| 62 | Check: Does the UE perform on the NR Cell 1 the Test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1? | - | - | 6 | P |
| 63 | Check: Does the UE transmit a DEREGISTRATION REQUEST message at expiry of T3445? | --> | 5GMM: DEREGISTRATION REQUEST | 7 | P |
| Note 1: The request to originate a manual eCall may be performed by MMI or AT command. Note 2: Timer T3445 of 12hours starts. Note 3: Timer T3512 of 7hours starts. |  |  |  |  |  |

Table 11.5.2.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour depending on UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - | - |
| 1a1 | Step 5a1 of the procedure of "PDU session establishment " described in TS 38.508-1 [4] Table 4.5A.2.2.2-1 is performed. | - | - | - | - |
| 2-5 | Steps 1-4 of the generic procedure for IMS registration / 5GS as defined in TS 34.229-5 [41] Annex A.2 take place. | - | - | - | - |
| - | EXCEPTION: In parallel to the event described in steps 6-10 below the behaviour in table 11.5.2.3.2-3 occurs. | - | - | - | - |
| 6-10 | Steps 5-8 of the generic procedure for IMS registration as defined in TS 34.229-5 [41] Annex A.2 take place. | - | - | - | - |

Table 11.5.2.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Step 1 of expected sequence from A.23 as defined in TS 34.229-5 [41] is performed. | --> | INVITE | - | - |

11.5.2.3.3	Specific message contents

Table 11.5.2.3.3-1: SIB1 for NR Cell 1 (All steps, Table 11.5.2.3.2-1)

Derivation path: TS 38.508-1 [4] table 4.6.1-28 Condition eCalloverIMSforNR

Table 11.5.2.3.3-2: REGISTRATION ACCEPT (step 15, 56, 62 Table 11.5.2.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| T3512 value |  |  |  |
| Unit | '001'B | value is incremented in multiples of 1 hour |  |
| Timer value | '0 0111'B | 7 hours |  |

Table 11.5.2.3.3-3A: INVITE (Step 1 Table 11.5.2.3.2-3)

| Derivation path: TS 34.229-1 [35] table in annex A.2.1 Condition A25 |  |
| --- | --- |
| Header/param | Value/remark |
| Call-Info url purpose | Any URL EmergencyCallData.eCall.MSD |
| Message-body | The following SDP types and values. Session description: - - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) [Note 2] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Note 1: At least one "c=" field shall be present. Note 2: AMR codec (AMR/8000 and/or AMR-WB/16000) shall be present in the media attributes, optionally including channel number "/1". -boundary value (as provided in SIP hdr Content-Type) Content-Type: application/EmergencyCallData.eCall.MSD Content-ID: same URL as in Call-Info header Content-Disposition: by-reference;handling=optional MSD in ASN.1 PER encoding -boundary value (as provided in SIP hdr Content-Type) |

Table 11.5.2.3.3-3B: 200 OK for INVITE (Step 2 TS 34.229-5 [41] annex A.23, in parallel to steps 32-34 Table 11.5.2.3.2-1)

| Derivation path: TS 34.229-1 [35] table in annex A.3.1 Condition A12 |  |
| --- | --- |
| Header/param | Value/remark |
| Message-body | --boundary1 Content-Type: application/EmergencyCallData.eCall.Control+xml Content-ID: <psap@3gpp.org> Content-Disposition: by-reference <?xml version="1.0" encoding="UTF-8"?> <EmergencyCallData.control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <ack received="true" ref=" addr-spec of the Content-ID of MIME body part containing the MSD sent by the UE in INVITE"/> </EmergencyCallData.control> --boundary1 The following SDP types and values. Session description: - v=0 - o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:37 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 1] - b=AS:37 - b=RS:0 - b=RR:0 Attributes for media: - a=rtpmap: (payload type) AMR/8000/1 or AMR-WB/16000/1 [Note 1] [Note2] - a=fmtp: (format) mode-change-capability=2; max-red=220 - a=ptime:20 - a=maxptime:240 Note 1: The value for fmt, payload type and format is copied from step 1. Note 2: If UE included AMR-WB/16000 in step 1, SS uses AMR-WB/16000/1. Otherwise SS uses AMR/8000/1. |

Table 11.5.2.3.3-3: UL NAS TRANSPORT (step 17, Table 11.5.2.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.7.1-10, condition INITIAL_PDU_REQUEST. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Request type | ‘001’B | initial request |  |
| S-NSSAI | Not Present |  |  |

Table 11.5.2.3.3-4: REGISTRATION REQUEST (step 55, Table 11.5.2.3.2-1)

| Derivation path: TS 38.508-1 [4] table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS registration type |  |  |  |
| 5GS registration type value | ‘011’B |  | PERIODIC |

Table 11.5.2.3.3-5: TRACKING AREA UPDATE REQUEST (Step 60, Table 11.5.2.3.2-1; step 3, TS 38.508-1 [4] Table 4.9.7.2.2-1)

Derivation Path: TS 38.508-1 [4] table 4.9.7.2.3-1 with condition First-N1-to-S1 = TRUE

Table 11.5.2.3.3-6: DEREGISTRATION REQUEST (step 63, Table 11.5.2.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| De-registration type |  |  |  |
| Switch off | ‘0’B | Normal de-registration |  |

<End of modified section 1>