3GPP TSG-RAN5 Meeting #106	R5-251386

# Athens, Greece, 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0650 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test case 10.16 Emergency Call with emergency registration / RTT activated / 5GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test cases for IMS Emergency Calls with acitvated RTT missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test case 10.16 added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage for RTT according to EU directive 2019/882 remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 34.229-2 CR 0360 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | WIC corrected, emergency registration procedure has been removed from test purpose and test steps, location removed from pre-test condition, A.29 removed from test steps, specific message content corrected, Test step for reserving resources (TS 38.508-1) added, Corrections in specific message contents, Steps in main behaviour renumbered, Typo corrected, pre-test conditions modified |  |  |  |  |  |  |  |  |

## <Start of first modified section>

## 10.16	Emergency Call with emergency registration / RTT activated / 5GS

10.16.1	Test Purpose (TP)

(1)

with { UE being registered to IMS }

ensure that {

when { UE is being made to start an emergency call }

then { UE performs IMS emergency registration and initiates the setup of an emergency call. UE sends a correctly composed INVITE request including SDP offer for real-time text }

}

(2)

with { UE having sent INVITE }

ensure that {

when { UE receiving 100 Trying, followed by 180 Ringing, followed by 200 OK indicating that real-time text has been added to the call }

then { UE sends ACK }

}

(3)

with { Emergency call with real-time text being established }

ensure that {

when { UE receives BYE  }

then { UE sends a 200 OK response }

}

10.16.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229 clause 5.1.6.2]:

When the user initiates an emergency call, if emergency registration is needed (including cases described in subclause 5.1.6.2A), the UE shall perform an emergency registration prior to sending the SIP request related to the emergency call.

...

When a UE performs an initial emergency registration the UE shall perform the actions as specified in subclause 5.1.1.2 with the following additions and modifications:

a)	the UE shall include a "sos" SIP URI parameter in the Contact header field as described in subclause 7.2A.13, indicating that this is an emergency registration and that the associated contact address is allowed only for emergency service; and

b)	the UE shall populate the From and To header fields of the REGISTER request with:

-	the first entry in the list of public user identities provisioned in the UE;

-	the default public user identity obtained during the normal registration, if the UE is not provisioned with a list of public user identities, but the UE is currently registered to the IM CN subsystem; and

-	the derived temporary public user identity, in all other cases.

[TS 24.229 clause 5.1.6.3]

Upon receiving the 200 (OK) to the REGISTER request that completes the emergency registration, the UE shall not subscribe to the reg event package of the public user identity specified in the REGISTER request.

[TS 24.229 clause 5.1.6.5]

When a UE performs authentication a UE shall perform the procedures as specified in subclause 5.1.1.5.

[TS 24.229 clause 5.1.6.8.3]

After a successful initial emergency registration, the UE shall apply the procedures as specified in subclause 5.1.2A and 5.1.3 with the following additions:

1)	the UE shall insert in the INVITE request, a From header field that includes the public user identity registered via emergency registration or the tel URI associated with the public user identity registered via emergency registration, as described in subclause 4.2;

2)	the UE shall include a service URN in the Request-URI of the INVITE request in accordance with subclause 5.1.6.8.1;

3)	the UE shall insert in the INVITE request, a To header field with the same emergency service URN as in the Request-URI;

4)	if available to the UE, and if defined for the access type as specified in subclause 7.2A.4, the P-Access-Network-Info header field shall contain a location identifier such as the cell id, line id or the identity of the WLAN access node, which is relevant for routeing the IMS emergency call;

NOTE 1:	The IMS emergency specification in 3GPP TS 23.167 [4B] describes several methods how the UE can get its location information from the access network or from a server. Such methods are not in the scope of this specification.

5)	the UE shall insert in the INVITE request, one or two P-Preferred-Identity header field(s) that include the public user identity registered via emergency registration or the tel URI associated with the public user identity registered via emergency registration as described in subclause 4.2;

NOTE 2:	Providing two P-Preferred-Identity header fields is usually supported by UE acting as enterprise network.

6)	void;

7)	if the UE has its location information available, or a URI that points to the location information, then the UE shall include a Geolocation header field in the INVITE request in the following way:

-	if the UE is aware of the URI that points to where the UE's location is stored, include the URI as the Geolocation header field value, as described in RFC 6442 [89]; or

[Rel-15]

-	if the UE is aware of its location information, include the location information in a PIDF location object, in accordance with RFC 4119 [90], include the location object in a message body with the content type application/pidf+xml, and include a Content ID URL, referring to the message body, as the Geolocation header field value, as described RFC 6442 [89], and include a Content-Disposition header field with a disposition type "render" value and a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

[Rel-16]

-	if the UE is aware of its location information, include the location information in a PIDF location object, in accordance with RFC 4119 [90] and RFC 5491 [267], include the location object in a message body with the content type application/pidf+xml, and include a Content ID URL, referring to the message body, as the Geolocation header field value, as described RFC 6442 [89], and include a Content-Disposition header field with a disposition type "render" value and a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

8)	if the UE includes a Geolocation header field, the UE shall also include a Geolocation-Routing header field with a "yes" header field value, which indicates that the location of the UE can be used by other entities to make routing decisions, as described in RFC 6442 [89];

NOTE 3:	It is suggested that UE's only use the option of providing a URI when the domain part belongs to the current P-CSCF or S-CSCF provider. This is an issue on which the network operator needs to provide guidance to the end user. A URI that is only resolvable to the UE which is making the emergency call is not desirable.

9)	if the UE has neither geographical location information available, nor a URI that points to the location information, the UE shall not insert a Geolocation header field in the INVITE request; and

10)	if support of the current location discovery during an emergency call is allowed in the IP-CAN specific annex and the UE supports the current location discovery during an emergency call, the UE shall include a Recv-Info header field as described in RFC 6086 [25], indicating the g.3gpp.current-location-discovery info package name and shall include an Accept header field indicating the "application/vnd.3gpp.current-location-discovery+xml" MIME type.

NOTE 4:	RFC 3261 [26] provides for the use of the Priority header field with a suggested value of "emergency". It is not precluded that emergency sessions contain this value, but such usage will have no impact on the processing within the IM CN subsystem.

[TS 24.237 clause 7.2]:

When originating an emergency call as specified in 3GPP TS 24.229 [2] and if the SC UE has an IMEI, then the SC UE shall include the sip.instance media feature tag as specified in IETF RFC 5626 [22] with value based on the IMEI as defined in 3GPP TS 23.003 [12] in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [53].

[TS 23.003 clause 13.8]:

An instance-id is a SIP Contact header parameter that uniquely identifies the SIP UA performing a registration.

When an IMEI is available, the instance-id shall take the form of a IMEI URN (see RFC 7254 [79]). The format of the instance-id shall take the form "urn:gsma:imei:<imeival>" where by the imeival shall contain the IMEI encoded as defined in RFC 7254 [79]. The optional <sw-version-param> and <imei-version-param> parameters shall not be included in the instance-id. RFC 7255 [104] specifies additional considerations for using the IMEI as an instance-id. An example of such an instance-id is as follows:

EXAMPLE:	urn:gsma:imei:90420156-025763-0

If no IMEI is available, the instance-id shall take the form of a string representation of a UUID as a URN as defined in IETF RFC 4122 [80]. An example of such an instance-id is as follows:

EXAMPLE:	urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6

For more information on the instance-id and when it is used, see 3GPP TS 24.229 [81].

[TS 26.114 clause 6.2.1]

… An MTSI client shall offer at least one RTP profile for each RTP media stream. Multiple RTP profiles may be offered using SDPCapNeg as described in Clause 6.2.1a. For voice and real-time text, the first SDP offer shall include at least the AVP profile. …

[TS 26.144 clause 7.4.4]

The following RTP payload format shall be used:

-    T.140 text conversation RTP payload format according to RFC 4103 [31] including the updates from RFC 9071 [185] when the negotiation for support of multiparty real-time text is successful.

Real-time text shall be the only payload type in its RTP stream because the RTP sequence numbers are used for loss detection and recovery. The redundant transmission format shall be used for keeping the effect of packet loss low.

10.16.3	Test description

10.16.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to enable RTT and to register for IMS after switch on.

Preamble:

-	The UE is in test state 1N-A (TS 38.508-1 [21]) and registered to IMS.

10.16.3.2	Test procedure sequence

Table 10.16.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is made to make an emergency call with RTT | - | - | - | - |
| 2-11 | Steps 1-10 of test procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 12-15 | Steps 1-4 of annex A.3 (emergency registration) are performed | - | - | - | - |
| - | EXCEPTION: In parallel to steps 2-6 below, steps 11-13 of test procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 [21] takes place. | - | - | - | - |
| 16 | Step 1 of annex A.6 (emergency call) Check: Does the UE send a correctly composed INVITE request? | --> | INVITE | 1 | P |
| 17 | Step 2 of annex A.6 (emergency call) | <-- | 100 Trying | - | - |
| 18 | Step 3 of annex A.6 (emergency call) | <-- | 180 Ringing | - | - |
| 19 | SS reserves resources for real-time text by executing TS 38.508-1 cl FFS | - | - | - | - |
| 20 | Step 4 of annex A.6 (emergency call) | <-- | 200 OK | - | - |
| 21 | Step 5 of annex A.6 (emergency call) Check: Does the UE send ACK? | --> | ACK | 2 | P |
| 22 | Step 1 of annex A.8 (MT Release of Voice Call) | <-- | BYE | - | - |
| 23 | Step 2 of annex A.8 (MT Release of Voice Call) Check: Does the UE send 200 OK for the BYE request and ends the call? | --> | 200 OK | 3 | P |

10.16.3.3	Specific message contents

Table 10.16.3.3-1: INVITE (step 16, table 10.16.3.2-1)

| Derivation Path: Annex A.6, Step 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact |  |  |  |  |
| feature-param |  | containing "text" in addition to what is specified in A.2.1 of TS 34.229-1 [2] |  |  |
| Message-body |  | SDP message as specified for the INVITE at step 1 of annex A.6 with an additional media description for RTT: Media description (real time text): m=text (transport port) RTP/AVP (fmt) c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) b=RS: (bandwidth-value) b=RR: (bandwidth-value) Attributes for media (real time text): a=rtpmap:(payload type) t140/1000 a=rtpmap:(payload type) red/1000 a=fmtp:(format) (format specific parameters) Note 1: The c= line shall be present when there is no c= line in the session description of the SDP message |  | TS 24.229 [7] |

Table 10.16.3.3-2: 200 OK (step 20, table 10.16.3.2-1)

| Derivation Path: Annex A.6 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Message-body |  | SDP message as specified for the 200 OK at step 4 of annex A.6 with an additional media description for RTT: Session description: b=AS:FFS Editor's note: Bandwidth to be specified (37+x?) Media description (real time text): m=text (transport port) RTP/AVP (fmt) [Note 1] b=AS: (bandwidth-value) [Note 1] b=RS: (bandwidth-value) [Note 1] b=RR: (bandwidth-value) [Note 1] Attributes for media (real time text): a=rtpmap:(payload type) t140/1000 [Note 1] a=rtpmap:(payload type) red/1000 [Note 1] a=fmtp:(format) (format specific parameters) [Note 1] Note 1: fmt, bandwidth-values, payload types, format and format specific parameters are copied from the SDP message received at step 2. |  | TS 24.229 [7] |

Table 10.16.3.3-3: BYE (step 22, table 10.16.3.2-1)

Derivation Path: Annex A.8, Step 1, with Condition A9

<End of first modified section>