3GPP TSG-RAN WG5 Meeting # 106	R5-251525

# Athens, Greece, February 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1916 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additions on Ch7 for harmonic mixing MSD for DC_5A_n78A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The REFSENS test points for DC_5A_n78A are not correctly aligned with the minimum requirements. The test points for UL harmonic exception are inaccurate or missing. The test points for harmonic mixing and harmonic mixing avoided are missing and should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update REFSENS test points for UL harmonic exception for DC_5A_n78A. - Add REFSENS test points for harmonic mixing and harmonic mixing avoided for DC_5A_n78A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The REFSENS tests for DC_5A_n78A will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.0.3.2, 7.3B.2.3.4.2.1, 7.3B.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905... CR 0995 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-251094r1: - Undo the new added test case 2a for DC_5A_n78A in Table 7.3B.2.3.4.2.1-6 and Table 7.3B.2.3.5-0 which is duplicated with 1a. - Handle the overlapping issue with R5-250816. |  |  |  |  |  |  |  |  |

<< Start of changes >>

7.3B.2.0.3.2	Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

Sensitivity degradation is allowed for different combinations of UL configurations and DL channel bandwidths if a band if it is impacted by receiver harmonic mixing due to another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (low) and uplink/downlink configurations due to UL harmonic from a PC3 aggressor UL band (high) for either single band uplink or PC3 or PC2 EN-DC are specified in Table 7.3B.2.0.3.2-1.

Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor UL for PC2 EN-DC are specified in Table 7.3B.2.3.2-1a.

For these exceptions, only the listed test points in Table 7.3B.2.3.2-1 and Table 7.3B.2.3.2-1a need to be tested.

Table 7.3B.2.0.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 2 | n71 | 5 | 15 | 25 (RBstart=0) | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| 2 | n71 | 20 | 15 | 50 (RBstart=0) | 20 | 15.6 | NOTE 4 | UL1/DL3 |
| n2 | 71 | 5 | 15 | 25 (RBstart=0) | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| n2 | 71 | 20 | 15 | 50 (RBstart=0) | 20 | 15.6 | NOTE 4 | UL1/DL3 |
| n41 | 26 | 10 | 15 | 25 (RBstart=0) | 5 | 24.3 | NOTE 4 | UL1/DL3 |
| n41 | 26 | 15 | 15 | 75 (RBstart=0) | 15 | 22.5 | NOTE 4 | UL1/DL3 |
| n77 | 2 | 10 | 15 | 25 (RBstart=0) | 5 | 6.1 |  |  |
| n77 | 2 | 20 | 15 | 100 (RBstart=0) | 20 | 3.7 |  |  |
| n77 | 3 | 10 | 15 | 25 (RBstart=0) | 5 | 5.7 |  |  |
| n77 | 3 | 20 | 15 | 100 (RBstart=0) | 20 | 2.7 |  |  |
| n77 | 12 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 2 | UL1/DL5 |
| n77 | 12 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 2 | UL1/DL5 |
| n77 | 13 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 2 | UL1/DL5 |
| n77 | 13 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 2 | UL1/DL5 |
| n77 | 14 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 2 | UL1/DL5 |
| n77 | 14 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 2 | UL1/DL5 |
| n77 | 28 | 10 | 15 | 25 (RBstart=0) | 5 | 28 | NOTE 2 | UL1/DL5 |
| n77 | 28 | 20 | 15 | 100 (RBstart=0) | 20 | 22 | NOTE 2 | UL1/DL5 |
| n77 | 41 | 10 | 15 | 12 (RBstart=0) | 5 | 10.4 | NOTE 8 | UL2/DL3 |
| n77 | 41 | 10 | 15 | 50 (RBstart=0) | 20 | 10.4 | NOTE 8 | UL2/DL3 |
| n78 | 3 | 10 | 15 | 25 (RBstart=0) | 5 | 5.7 |  |  |
| n78 | 3 | 20 | 15 | 100 (RBstart=0) | 20 | 2.7 |  |  |
| n78 | 5 | 10 | 15 | 25 (RBstart=0) | 5 | 5.7 | NOTE 14 | UL1/DL4 |
| n78 | 19 | 10 | 15 | 25 (RBstart=0) | 5 | 7.2 |  |  |
| n78 | 19 | 15 | 15 | 75 (RBstart=0) | 15 | 3.8 |  |  |
| n78 | 41 | 10 | 15 | 12 (RBstart=0) | 5 | 10.4 | NOTE 8 | UL2/DL3 |
| n78 | 41 | 20 | 15 | 50 (RBstart=0) | 20 | 10.4 | NOTE 8 | UL2/DL3 |
| n79 | 11 | 10 | 15 | 25 (RBstart=0) | 5 | 39.3 | NOTE 4 | UL1/DL3 |
| n79 | 11 | 15 | 15 | 75 (RBstart=0) | 15 | 34.5 | NOTE 4 | UL1/DL3 |
| n79 | 19 | 10 | 15 | 25 (RBstart=0) | 5 | 29.5 | NOTE 2 | UL1/DL5 |
| n79 | 19 | 15 | 15 | 75 (RBstart=0) | 15 | 24.7 | NOTE 2 | UL1/DL5 |
| n79 | 21 | 10 | 15 | 25 (RBstart=0) | 5 | 39.3 | NOTE 4 | UL1/DL3 |
| n79 | 21 | 15 | 15 | 75 (RBstart=0) | 15 | 34.5 | NOTE 4 | UL1/DL3 |
| n79 | 26 | 10 | 15 | 25 (RBstart=0) | 5 | 27 | NOTE 2 | UL1/DL5 |
| n79 | 26 | 15 | 15 | 75 (RBstart=0) | 15 | 22.2 | NOTE 2 | UL1/DL5 |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2with the DL carrier frequency in the lower band and fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 9: No requirements apply for the case that there is at least one individual RE within the uplink transmission bandwidth of the relative higher band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the 3 times of the frequency range of the relative lower band’s downlink channel bandwidth. The reference sensitivity is only verified when this is not the case. NOTE 10: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. NOTE 12: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band NOTE 14: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.40.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.2-1a: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | 2 | 10 | 15 | 25 (RBstart=0) | 5 | 9.1 |  |  |
| n77 | 2 | 20 | 15 | 100 (RBstart=0) | 20 | 6.7 |  |  |
| n77 | 3 | 10 | 15 | 25 (RBstart=0) | 5 | 8.1 |  |  |
| n77 | 3 | 20 | 15 | 100 (RBstart=0) | 20 | 5.7 |  |  |
| n77 | 12 | 10 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | 12 | 10 | 15 | 50 (RBstart=0) | 10 | 31 | NOTE 1 | UL1/DL5 |
| n77 | 13 | 10 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | 13 | 10 | 15 | 50 (RBstart=0) | 10 | 31 | NOTE 1 | UL1/DL5 |
| n77 | 14 | 10 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | 14 | 10 | 15 | 50 (RBstart=0) | 10 | 31 | NOTE 1 | UL1/DL5 |
| n77 | 28 | 10 | 15 | 25 (RBstart=0) | 5 | 32 | NOTE 1 | UL1/DL5 |
| n77 | 28 | 20 | 15 | 100 (RBstart=0) | 20 | 25 | NOTE 1 | UL1/DL5 |
| n77 | 41 | 10 | 15 | 12 (RBstart=0) | 5 | 19.4 | NOTE 4 | UL2/DL3 |
| n77 | 41 | 20 | 15 | 50 (RBstart=0) | 20 | 19.4 | NOTE 4 | UL2/DL3 |
| n78 | 19 | 10 | 15 | 25 (RBstart=0) | 5 | 9.8 |  |  |
| n78 | 19 | 15 | 15 | 75 (RBstart=0) | 15 | 5.8 |  |  |
| n79 | 19 | 10 | 15 | 25 (RBstart=0) | 5 | 32.5 | NOTE 1 | UL1/DL5 |
| n79 | 19 | 15 | 15 | 75 (RBstart=0) | 15 | 27.7 | NOTE 1 | UL1/DL5 |
| n79 | 21 | 10 | 15 | 25 (RBstart=0) | 5 | 42.3 | NOTE 3 | UL1/DL3 |
| n79 | 21 | 15 | 15 | 75 (RBstart=0) | 15 | 37.5 | NOTE 3 | UL1/DL3 |
| NOTE 1: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 2: Void. NOTE 3: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency and BWChannelHB the channel bandwidth in the higher band, both in MHz. NOTE 4: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency and BWChannelHB the channel bandwidth in the higher band, both in MHz. NOTE 5: Void. |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.2-2: Void

<< next change >>

7.3B.2.3	Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs)

7.3B.2.3.1	Test purpose

Same as in clause 7.3B.2.1.1.

7.3B.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting inter-band EN-DC within FR1 with 2 DL CCs.

7.3B.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0.

LTE anchor agnostic approach is not applied.

7.3B.2.3.4	Test description

7.3B.2.3.4.1	Void

7.3B.2.3.4.2	Test description

7.3B.2.3.4.2.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths based on EN-DC operating bands specified in clause 5.5B, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in Table 7.3B.2.3.4.2.1-0 to Table 7.3B.2.3.4.2.1-6.

The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS 36.521-1 [10] Annex A.2 and A.3 for E-UTRA RMC, and in TS 38.521-1 [8] Annex A.2 and A.3 for NR RMC respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG, respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG, respectively.

For configurations without any reference sensitivity exception, in a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. For configurations with reference sensitivity exception, in an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected.

Table 7.3B.2.3.4.2.1-0: Test configurations table for all EN-DC configurations for FR1 non-exception requirements

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  | Mid range for E-UTRA and Mid range for NR |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1 |  |  |  |  |  | Refer to "NR NRB "and "E-UTRA NRB " columns |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Lowest supported SCS |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCG -NR |  |  |  |  |
|  | Band | Range | NRB |  | Band |  | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc / UL alloc | UL MOD |  | DL MOD | UL/DL Ch BW | DLalloc / UL alloc |
| Test Settings for a DC_XA_nYA Configuration (Inter-band EN-DC) – Note 2 |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY |  | default |  |  |
|  | N/A | N/A | 5 MHz | N/A | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | Highest NRB | All RBs / REFSENS_NR |
| NOTE 1: REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. NOTE 2: LTE anchor agnostic configuration NOTE 3: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 4: The test configuration table only applies to EN-DC only capable devices and devices not supporting NR/5GC mode in the tested band. NOTE 5: Only one EN-DC combination per FR1 band is tested for each EN-DC configuration as defined in clause 5.5B. |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-1: Void

Table 7.3B.2.3.4.2.1-2_1: Void

Table 7.3B.2.3.4.2.1-2_2: Void

Table 7.3B.2.3.4.2.1-2_3: Void

Table 7.3B.2.3.4.2.1-2_4: Void

Table 7.3B.2.3.4.2.1-2_5: Void

Table 7.3B.2.3.4.2.1-2_6: Void

Table 7.3B.2.3.4.2.1-2_7: Void

Table 7.3B.2.3.4.2.1-2_8: Void

Table 7.3B.2.3.4.2.1-2_9: Void

Table 7.3B.2.3.4.2.1-2_11: Void

Table 7.3B.2.3.4.2.1-2_12: Void

Table 7.3B.2.3.4.2.1-2_13: Void

Table 7.3B.2.3.4.2.1-2_14: Void

Table 7.3B.2.3.4.2.1-2_15: Void

Table 7.3B.2.3.4.2.1-2_16: Void

Table 7.3B.2.3.4.2.1-2_17: Void

Table 7.3B.2.3.4.2.1-2_18: Void

Table 7.3B.2.3.4.2.1-2_19: Void

Table 7.3B.2.3.4.2.1-2_20: Void

Table 7.3B.2.3.4.2.1-2_21: Void

Table 7.3B.2.3.4.2.1-2_22: Void

Table 7.3B.2.3.4.2.1-2_23: Void

Table 7.3B.2.3.4.2.1-4: Void

Table 7.3B.2.3.4.2.1-4a: Void

Table 7.3B.2.3.4.2.1-4b: Void

Table 7.3B.2.3.4.2.1-4c: Void

Table 7.3B.2.3.4.2.1-4d: Void

Table 7.3B.2.3.4.2.1-4e: Void

Table 7.3B.2.3.4.2.1-4f: Void

Table 7.3B.2.3.4.2.1-4g: Void

Table 7.3B.2.3.4.2.1-4h: Void

Table 7.3B.2.3.4.2.1-4i: Void

Table 7.3B.2.3.4.2.1-4j: Void

Table 7.3B.2.3.4.2.1-4k: Void

Table 7.3B.2.3.4.2.1-4l: Void

Table 7.3B.2.3.4.2.1-4m: Void

Table 7.3B.2.3.4.2.1-4n: Void

Table 7.3B.2.3.4.2.1-5: Void

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1. |  |  |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  |  |  |  |  |  |  |  | Lowest supported SCS |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  |  |  |  |  |  | SCG -NR |  |  |  |  |  |  |  |  |
|  | Band | Range |  |  |  |  | NRB |  |  |  |  | Band |  |  | Range |  | NRB |  |  |  |
|  | UL MOD | DL MOD |  |  |  |  | CH BW |  |  | DLalloc / UL alloc |  | UL MOD |  |  | DL MOD |  | UL/DL Ch BW | DLalloc / UL alloc |  |  |
| Test Settings for a DC_1A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12,11 | 1 | Mid |  |  |  |  |  |  |  |  |  | n3 |  |  | Mid |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| 24 | 1 | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  | n3 |  |  | UL 1760 / DL 1855 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 36 | 1 | Low |  |  |  |  |  |  |  |  |  | n3 |  |  | High |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_1A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 | UL 1965 / DL 2155 |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 887.5 / DL 932.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_1A_n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 1 | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 713.3 / DL 768.3 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| 28 | 1 | UL 1970 / DL 2160 |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 708 / DL 763 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| Test Settings for a DC_1A_n41A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 1 | High |  |  |  |  |  |  |  |  |  | n41 |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 26 | 1 | High |  |  |  |  |  |  |  |  |  | n41 |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| Test Settings for a DC_1A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 1 | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3900 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 1 | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3870 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |  |
| 34 | 1 | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4090.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 44 | 1 | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3709.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_1A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3709.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_2A_n41A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 2 | High |  |  |  |  |  |  |  |  |  | n41 |  |  | Low |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_LTE |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| 26 | 2 | DL 1987.5 |  |  |  |  |  |  |  |  |  | n41 |  |  | UL/DL 2546 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| Test Settings for a DC_2A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 2 | UL 1855 / DL 1935 |  |  |  |  |  |  |  |  |  | n66 |  |  | DL 2175 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 2 | UL 1883.3 /DL1963.3 |  |  |  |  |  |  |  |  |  | n66 |  |  | DL 2150 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_2A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 | DL 1980 MHZ |  |  |  |  |  |  |  |  |  | n71 |  |  | UL 665.5 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| 23 | 2 | DL1980 MHZ |  |  |  |  |  |  |  |  |  | n71 |  |  | UL 673 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| 35 | 2 | UL 1881 MHZ |  |  |  |  |  |  |  |  |  | n71 |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_2 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_2A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 | UL 1860 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3720 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 2 | UL 1860 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3690.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |  |
| 3 8 | 2 | UL 1860 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3850.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / 0 |  |  |
| 45 | 2 | DL Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3920.01 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 4a 9 | 2 | DL Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL Mid |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 54 | 2 | UL 1855/DL 1935 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3790.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 64 | 2 | UL 1900/DL 1980 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3720 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 74 | 2 | UL 1885/DL 1965 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3810 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
|  | 2 | DL 1987.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3350 |  |  |  |  |  |
| 86, 16 | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| Test Settings for a DC_2A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 | UL 1870 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3740.01 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 2 | UL 1885 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3739.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |  |
| 34 | 2 | UL 1855/ DL 1935 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3790.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 44 | 2 | UL 1885/ DL 1965 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3690 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_3A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,11 | 3 | Mid |  |  |  |  |  |  |  |  |  | n1 |  |  | Mid |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| 26 | 3 | High |  |  |  |  |  |  |  |  |  | n1 |  |  | Low |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| 34 | 3 | UL 1760 / DL 1855 |  |  |  |  |  |  |  |  |  | n1 |  |  | UL 1950 / DL 2140 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_3A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 | UL 1771 / DL 1866 |  |  |  |  |  |  |  |  |  | n5 |  |  | UL 838 / DL 883 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 3 | UL 1721 / DL 1816 |  |  |  |  |  |  |  |  |  | n5 |  |  | UL 838 / DL 883 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_3A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 | UL 1730 / DL 1825 |  |  |  |  |  |  |  |  |  | n7 |  |  | UL 2535 / DL 2655 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_3A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 | UL 1755 / DL 1850 |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 900 / DL 945 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 3 | UL 1747.5 / DL 1842.5 |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 897.5 / DL 942.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_3A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 3 | High |  |  |  |  |  |  |  |  |  | n41 |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 26 | 3 | High |  |  |  |  |  |  |  |  |  | n41 |  |  | Low |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| 34 | 3 | UL 1740 / DL 1835 |  |  |  |  |  |  |  |  |  | n41 |  |  | UL/DL 2657.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for DC_3A_n77A (PC2 and PC3) and DC_3A_n78A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 3 | Mid |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3495 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 3 | Mid |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3525 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |  |
| 35 | 3 | Mid |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3685.005 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 49 | 3 | Low |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | High |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs/0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 54 | 3 | UL 1740 / DL 1835 |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3574.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 64 | 3 | UL 1765 / DL 1860 |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3435 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_5A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 5 | UL 838/ DL 883 |  |  |  |  |  |  |  |  |  | n66 |  |  | UL 1721/ DL 2121 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_5A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 5 | UL 837.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3350.01 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 5 | UL 829 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4145.01 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 38 | 5 | UL Mid (836.5 MHz) |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL Mid |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 44, 13 | 5 | UL 844/DL 889 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3421.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 54, 13 | 5 | UL 826.5/DL 871.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4177.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for DC_5A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 5 | UL 840 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3360 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 1a3 | 5 | UL 840 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3360 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / 0 |  |  |
| 28 | 5 | UL 840 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3560.01 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 34 | 5 | UL 844/DL 889 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3421.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 45 | 5 | MID |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3525.99 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 59 | 5 | High |  |  |  |  |  |  |  |  |  | n78 |  |  | MID |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_7A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 | DL 2655 |  |  |  |  |  |  |  |  |  | n3 |  |  | DL 1825 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  |  | 10 MHz |  |  | N/A |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_7A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 | DL 2667 |  |  |  |  |  |  |  |  |  | n5 |  |  | DL 879 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  |  | 10 MHz |  |  | N/A |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_7A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 | DL 2655 |  |  |  |  |  |  |  |  |  | n66 |  |  | DL 2130 |  |  |  |  |  |
|  | QPSK |  |  |  |  |  | 10 MHz |  |  | N/A |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_7A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 7 | DL 2672 |  |  |  |  |  |  |  |  |  | n71 |  |  | UL/ 668 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| 28 | 7 | DL 2630 |  |  |  |  |  |  |  |  |  | n71 |  |  | UL 668 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| Test Settings for a DC_7A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 | DL 2660 |  |  |  |  |  |  |  |  |  | N77 |  |  | DL 3870 |  |  |  |  |  |
|  | QPSK |  |  |  |  |  | 5 MHz |  |  | N/A |  | N/A |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_7A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 7 | High |  |  |  |  |  |  |  |  |  | n78 |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_8A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 | DL 932.5 |  |  |  |  |  |  |  |  |  | n1 |  |  | DL 2155 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  |  | 5 MHz |  |  | N/A |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_8A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 | DL 945 |  |  |  |  |  |  |  |  |  | n3 |  |  | DL 1850 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  |  | 5 MHz |  |  | N/A |  | N/A |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 8 | DL 942.5 |  |  |  |  |  |  |  |  |  | n3 |  |  | DL 1842.5 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  |  | 5 MHz |  |  | N/A |  | N/A |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_8A_n20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 | UL 890.5 / DL 935.5 |  |  |  |  |  |  |  |  |  | n20 |  |  | UL 849.5 / DL 808.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 8 | UL 892.5 / DL 937.5 |  |  |  |  |  |  |  |  |  | n20 |  |  | UL 847.5 / DL 806.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_8A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 | UL 885 / DL 942.5 MHz |  |  |  |  |  |  |  |  |  | n41 |  |  | UL / DL 2655 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 60 MHz | All RBs / 0 |  |  |
| 28 | 8 | UL 885 / DL 942.5 MHz |  |  |  |  |  |  |  |  |  | n41 |  |  | UL / DL 2546 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 60 MHz | All RBs / 0 |  |  |
| 34 | 8 | UL 882.5 / DL 927.5 MHz |  |  |  |  |  |  |  |  |  | n41 |  |  | UL / DL 2685MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for DC_8A_n77A and DC_8A_n78A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3590.01 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 2,8 | 8 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3520.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 34 | 8 | UL 897.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3634.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_11A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 11 | Mid |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4457.7 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 2,9 | 11 | Mid |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4512.69 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_12A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 12 | UL 710 |  |  |  |  |  |  |  |  |  | n66 |  |  | UL 1730 /DL 2130 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / 0 |  |  |
| 23 | 12 | UL 710 |  |  |  |  |  |  |  |  |  | n66 |  |  | UL 1730 /DL 2130 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| 38 | 12 | UL 710 |  |  |  |  |  |  |  |  |  | n66 |  |  | UL 1760 /DL 2160 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_12A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 12 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3537.495 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 28 | 12 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3351 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 35 | 12 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3687.495 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 49 | 12 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3750 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 54 | 12 | UL 702/DL 732 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3540 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_12A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 12 | UL 707/DL 737 MHz |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3535 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 28 | 12 | UL 707/DL 737 MHz |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3750 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 34 | 12 | UL 710/DL 740 MHz |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3580 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_13A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 13 | UL Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3909.99 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 28 | 13 | UL Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL Mid |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 35 | 13 | DL Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3755 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 49 | 13 | DL Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3850 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 54 | 13 | UL 784.5/DL 753.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3891.495 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_14A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 14 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3964.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 18 | 14 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3350.1 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 25 | 14 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3814.995 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 29 | 14 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3350.01 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 34 | 14 | UL 795.5/DL 765.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3947.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_18A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 18 | UL 820 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4100 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| 28 | 18 | UL 820 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4000 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| 34,X | 18 | UL 827.5 / DL 872.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3355 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 44,X | 18 | UL 817.5 / DL 862.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4130 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_18A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14,Y | 18 | UL 827.5 / DL 872.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3355 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_19A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 19 | UL 835 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4175 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| 28 | 19 | UL 835 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4100 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| 33 | 19 | UL 840 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL3360 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_19A_n78A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 19 | UL 840 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL3360 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / 0 |  |  |
| 24 | 19 | UL 836.5/ DL 881.5 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL3391 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 35 | 19 | DL 880 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL3520 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_19A_n79A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 19 | DL 884 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4420 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 2,9 | 19 | DL 884 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4445.025 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_20A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 | UL 840 / DL 799 |  |  |  |  |  |  |  |  |  | n3 |  |  | UL 1775 / DL 1870 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 20 | UL 847 / DL 806 |  |  |  |  |  |  |  |  |  | n3 |  |  | UL 1735 / DL 1830 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_20A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 | UL 851 / DL 810 |  |  |  |  |  |  |  |  |  | n7 |  |  | UL 2512 / DL 2632 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_20A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 | UL 849.5 / DL 808.5 |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 892.5 / DL 937.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_20A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,8 | 20 | Mid |  |  |  |  |  |  |  |  |  | n78 |  |  | Mid |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 20 | Mid |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3387.99 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 34 | 20 | UL 850 / DL 809 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3358.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_21A_n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 21 | UL 1450.4/ DL 1498.4 |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 735.5 / DL 790.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_21A_n79A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 21 | DL 1503.4 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4510.2 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 15 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 29 | 21 | DL 1503.4 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4800 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 15 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 34 | 21 | UL 1457.5/ DL 1505.5 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4420.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_21A_n79A Configuration (PC2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 21 | DL 1503.4 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4510.2 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 15 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 29 | 21 | DL 1503.4 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4800 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 15 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 34 | 21 | UL 1457.5/ DL 1505.5 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4420.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_25A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 25 | High |  |  |  |  |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 10 MHz |  |  | All RBs / 0 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| 26 | 25 | High |  |  |  |  |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 15 MHz |  |  | All RBs / 0 |  | N/A |  |  |  | QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| 36 | 25 | High |  |  |  |  |  |  |  |  | n41 |  |  |  | Mid |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 20 MHz |  |  | All RBs / 0 |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_26A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 | High |  |  |  |  |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 1a3 | 26 | High |  |  |  |  |  |  |  |  | n41 |  |  |  | UL/DL 2537 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / 0 |
| 28 | 26 | High |  |  |  |  |  |  |  |  | n41 |  |  |  | High |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | QPSK |  | 10 MHz |  |  | All RBs / 0 |
| 34 | 26 | UL 839 |  |  |  |  |  |  |  |  | n41 |  |  |  | UL/DL 2562 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| 45 | 26 | DL 864 |  |  |  |  |  |  |  |  | n41 |  |  |  | Mid |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 55 | 26 | Low |  |  |  |  |  |  |  |  | n41 |  |  |  | Mid |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 15 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 15 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 68 | 26 | High |  |  |  |  |  |  |  |  | n41 |  |  |  | Mid |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 15 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 15 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_26A_n77A/DC_26A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 | High |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | UL/DL 3366 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 15 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 2,8 | 26 | High |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | UL/DL 3446.01 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 15 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 34 | 26 | UL 836.5 |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | UL/DL 3391.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_26A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 26 | High |  |  |  |  |  |  |  |  | n79 |  |  |  | UL/DL 4432.5 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 15 MHz |  |  | All RBs / 0 |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 60 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 22,9 | 26 | High |  |  |  |  |  |  |  |  | n79 |  |  |  | UL/DL 4470.51 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 15 MHz |  |  | All RBs / 0 |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 60 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_28A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 28 | High |  |  |  |  |  |  |  |  | n5 |  |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 20MHz |  |  | All RBs / REFSENS_ENDC_3 |  | N/A |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / 0 |
| Test Settings for a DC_28A_n77A (PC2 and PC3) /DC_28A_n78A Configuration (Test ID 4 only apply to DC_28A_n77A (PC2 and PC3)) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 28 | Low |  |  |  |  |  |  |  |  |  | n77/n78 |  | UL/DL 3540 |  |  |  |  |  |  |
|  | QPSK | QPSK |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |  |
| 28 | 28 | Low |  |  |  |  |  |  |  |  |  | n77/n78 |  | Low |  |  |  |  |  |  |
|  | QPSK | QPSK |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |  |
| 34 | 28 | UL 705.5 / DL 760.5 |  |  |  |  |  |  |  |  |  | n77/n78 |  | UL/DL 3582.495 |  |  |  |  |  |  |
|  | QPSK | QPSK |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |  |
| 45 | 28 | Low |  |  |  |  |  |  |  |  |  | n77 |  | UL/DL 3815.01 |  |  |  |  |  |  |
|  | N/A | QPSK |  |  | 10 MHz |  |  | All RBs / 0 |  |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_2 |  |
| Test Settings for a DC_30A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 30 | Low |  |  |  |  |  |  |  |  | n66 |  |  |  | High |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 40 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_30A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 30 | UL 2310 |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3487.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_38A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 38 | DL High |  |  |  |  |  |  |  |  | n78 |  |  |  | UL Low |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_39A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 39 | UL 1910 |  |  |  |  |  |  |  |  | n41 |  |  |  | DL 2501 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_ |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / 0 |
| 26 | 39 | DL 1917.5 |  |  |  |  |  |  |  |  | n41 |  |  |  | UL 2546 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  | 5 MHz |  |  | All RBs / 0_ |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_ |
| Test Settings for a DC_40A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 40 | DL 2397.5 |  |  |  |  |  |  |  |  | n41 |  |  |  | UL 2546 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  | 5 MHz |  |  | All RBs / 0_ |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_ |
| Test Settings for a DC_40A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 40 | Mid |  |  |  |  |  |  |  |  | n78 |  |  |  | UL/DL 3525 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 2,9 | 40 | Low |  |  |  |  |  |  |  |  | n78 |  |  |  | Mid |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_2 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_41A_n77A (PC2 and PC3)/DC_41A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 41 | Low |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | UL/DL 3750 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 26 | 41 | High |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | Low |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| 36 | 41 | High |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 20MHz |  |  | All RBs / REFSENS_ENDC_3 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| Test Settings for a DC_48A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 48 |  |  | UL/DL 3555 |  |  |  |  |  |  | n66 |  |  |  | UL 1760 MHZ |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  | 10 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 40 MHz |  |  | All RBs / REFSENS_ENDC_1 |
| 23 | 48 |  |  | UL/DL 3565 |  |  |  |  |  |  | n66 |  |  |  | UL 1770 |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  | 10 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |
| 38 | 48 |  |  | UL/DL 3600 |  |  |  |  |  |  | n66 |  |  |  | UL1760 |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  | 10 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 40 MHz |  |  | All RBs / REFSENS_ENDC_1 |
| 44 | 48 |  |  | UL/DL 3630 |  |  |  |  |  |  | n66 |  |  |  | UL 1715 / DL 2115 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 20 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_66A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 66 |  |  | UL1775/ DL2175 |  |  |  |  |  |  | n2 |  |  |  | UL 1855 / DL 1935 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| 24 | 66 |  |  | UL 1750/ DL 2150 |  |  |  |  |  |  | n2 |  |  |  | UL 1883.3 / DL 1963.3 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_66A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 66 |  |  | UL 1721/ DL 2121 |  |  |  |  |  |  | n5 |  |  |  | UL 838/ DL 883 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_66A_n25A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 66 |  |  | UL 1775/ DL 2175 |  |  |  |  |  |  | n25 |  |  |  | UL 1855/ DL 1935 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| 24 | 66 |  |  | UL 1712.5/ DL 2112.5 |  |  |  |  |  |  | n25 |  |  |  | UL 1912.5/ DL 1992.5 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| 34 | 66 |  |  | UL 1750/ DL 2150 |  |  |  |  |  |  | n25 |  |  |  | UL 1883.3/ DL 1963.3 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_66A_n41A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 66 |  |  | High |  |  |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  | 20 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_66A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 66 |  | UL 1750/ DL 2150 |  |  |  |  |  |  |  | n71 |  |  |  | UL 675/ DL 629 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_66A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 66 |  | UL 1720 |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3440.01 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 23 | 66 |  | UL 1720 |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3410 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / 0 |
| 38 | 66 |  | UL Mid |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL Mid |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 44 | 66 |  | UL 1775/ DL 2175 |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3949.995 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| 54 | 66 |  | UL 1760 / DL 2160 |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3720 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
|  | 66 |  | DL 2197.5 |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3350 |  |  |  |  |  |
| 66, 16 | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_66A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 66 |  | UL 1755 |  |  |  |  |  |  |  | n78 |  |  |  | UL/DL 3510 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 23 | 66 |  | UL 1755 |  |  |  |  |  |  |  | n78 |  |  |  | UL/DL 3480 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / 0 |
| 34 | 66 |  | UL 1730/ DL2130 |  |  |  |  |  |  |  | n78 |  |  |  | UL/DL 3660 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for DC_71A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 71 |  | UL 665.5 |  |  |  |  |  |  |  | n2 |  |  |  | DL 1987.5 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5MHz |  | All RBs / REFSENS_ENDC_1 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / 0 |
| 23 | 71 |  | UL 673 |  |  |  |  |  |  |  | n2 |  |  |  | DL1980 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / 0 |
| 35 | 71 |  | Low |  |  |  |  |  |  |  | n2 |  |  |  | UL 1881 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 49 | 71 |  | High |  |  |  |  |  |  |  | n2 |  |  |  | UL 1881 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for DC_71A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 71 |  | UL 675/ DL 629 |  |  |  |  |  |  |  | n66 |  |  |  | UL1750/ DL 2150 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| NOTE 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-1. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-1. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-1 for PC3 and Table 7.3B.2.0.3.4-1a for PC2 REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 2: Void NOTE 3: Test ID with UL harmonic exception NOTE 4: Test ID with 2UL intermodulation exception. The test point is only applicable to UEs not supporting UE capability singleUL-Transmission. NOTE 5: Test ID with UL receiver harmonic mixing NOTE 6: Test ID with UL cross band isolation NOTE 7: Void NOTE 8: Test ID with UL harmonic exception avoided NOTE 9: Test ID with UL receiving harmonic mixing exception avoided NOTE 10: Void NOTE 11: Test ID with Cross band isolation exception avoided, which is only applicable to DC_1_n3, DC_3_n1 and DC_3_n84. NOTE 12: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 14: If the NR frequency does not match to a valid NR-ARFCN, apply the closest NR frequency with a valid NR-ARFCN. NOTE 15: The test points are executed only when UEs support simultaneous Rx/Tx capability as indicated in Table A.4.3.2B.2.3.1-2 of TS 38.508-2. NOTE 16: This test ID is for UL PC2 only. NOTE 17: For a UE which supports this band combination only when the Band n77 frequency range restriction of 3400 - 4100 MHz in Japan applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 18: For a UE which supports this band combination only when the Band n78 frequency range restriction of 3400 - 3800 MHz in Japan applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, C.2, C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1 of TS 38.521-1 [8].

5.	E-UTRA downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

6.	The DL and UL Reference Measurement channels are set according to Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-6 for E-UTRA CG and NR CG.

7.	NR propagation conditions are set according to Annex B.0 of TS 38.521-1 [8]. E-UTRA propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

8.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause7.3B.2.3.4.2.3.

9.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6. Step 9 only applicable to the test configuration in Table 7.3B.2.3.4.2.1-0.

7.3B.2.3.4.2.2	Test procedure

For test points in Table 7.3B.2.3.4.2.1-0:

1.	NR SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.3.4.2.1-0 on the NR CC. The NR SS sends downlink MAC padding bits on the DL RMC.

2.	NR SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3.4.2.1-0 Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the appropriate REFSENS value defined in TS 38.521-1 [8] Table 7.3.2.5-1 for NR band. Send continuously uplink power control "up" commands in the uplink scheduling information to both carriers to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of the NR carrier for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band.

For test points in Table 7.3B.2.3.4.2.1-6:

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.3.4.2.1-6 on the E-UTRA CC and NR CC. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3.4.2.1-6 on the E-UTRA CC and NR CC. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level for the E-UTRA CC and NR CC to the appropriate REFSENS value defined in clause 7.3B.2.3.5. Send continuously uplink power control "up" commands in the uplink scheduling information to both carriers to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of both NR and E-UTRA for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band, and Annex G.2 of TS 36.521-1 [10] for EUTRA band.

7.3B.2.3.4.2.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

Message contents exceptions for E-UTRA band are according to TS 36.521-1 [10] clause 7.3.4.3 for each network signalling value. Message contents exceptions for NR band are according to TS 38.521-1 [8] clause 7.3.2.4.3 for each network signalling value.

For test points with NOTE 4 in Table 7.3B.2.3.4.2.1-6, the following message exception applies:

Table 7.3B.2.3.4.2.3-1: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 7.3B.2.3.4.2.3-2: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

7.3B.2.3.5	Test requirement

For inter-band EN-DC configurations affected by reference sensitivity exceptions, when test points without NOTE 8, 9 and 11 in Table 7.3B.2.3.4.2.1-6 are tested, the throughput of each CG shall be ≥ 95% of the maximum throughput for the reference receive power level specified in Table 7.3B.2.3.5-0, Table 7.3B.2.3.5-1, Table 7.3B.2.3.5-2, Table 7.3B.2.3.5-3, Table 7.3B.2.3.5-3a, Table 7.3B.2.3.5-4 and Table 7.3B.2.3.5-5 for MSDs due to uplink harmonic, harmonic mixing, cross band isolation for PC3 EN-DC, cross band isolation for PC2 EN-DC, dual uplinks for PC3 EN-DC and dual uplink operation for PC2 EN-DC, respectively. For a given EN-DC combo, if more than one category of MSD applies, UE shall pass all requirement.

For test points with NOTE 8, 9 or 11 in Table 7.3B.2.3.4.2.1-6, reference sensitivity test requirements are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC, and Table 7.3.5-1 in TS 36.521-1 [10] for E-UTRA CC.

Reference sensitivity test requirements for test points in Table 7.3B.2.3.4.2.1-0 are specified in Table 7.3.2.5-1 in TS 38.521-1 [8].

For the UE which supports inter-band EN-DC, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 of TS 38.521-1 [8] for NR band and Table 7.3.5-1 of TS 36.521-1 [10] for EUTRA band, shall be increased by the amount given in ΔRIB,c defined in clause 7.3B.3.3 for the applicable two, three, four and five bands operation.

Table 7.3B.2.3.5-0: Reference sensitivity requirement for PC3 EN-DC

| CA configuration | Test ID | DL band | CBW (MHz) | REFSENS test requirement (dBm) | Exception type |
| --- | --- | --- | --- | --- | --- |
| DC_5A_n78A | 1 | 5 | 5 | REFSENS | UL harmonic |
|  |  | n78 | 100 | -84.1+TT | interference |
|  | 1a | 5 | 5 | REFSENS |  |
|  |  | n78 | 10 | -85.3+TT |  |
|  | 2 | 5 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 100 | REFSENS | avoided |
|  | 3 | 5 | 5 | -89.0 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
|  | 4 | 5 | 5 | -91.6 | Harmonic mixing |
|  |  | n78 | 10 | REFSENS |  |
|  | 5 | 5 | 5 | REFSENS | Harmonic mixing |
|  |  | n78 | 10 | REFSENS | avoiding |
| DC_39A_n41A | 1 | 39 | 20 | REFSENS | Cross band isolation |
|  |  | n41 | 10 | -91.5+TT |  |
|  | 2 | 39 | 5 | -97.7 | Cross band isolation |
|  |  | n41 | 100 | REFSENS |  |
| DC_40A_n41A | 1 | 40 | 5 | -67.9 | Cross band isolation |
|  |  | n41 | 100 | REFSENS | Cross band isolation |
| DC_26A_n41A | 1 | 26 | 5 | REFSENS | UL harmonic |
|  |  | n41 | 100 | -81.8+TT | interference |
|  | 1a | 26 | 5 | REFSENS |  |
|  |  | n41 | 10 | -81.1+TT |  |
|  | 2 | 26 | 5 | REFSENS | UL harmonic exception |
|  |  | n41 | 10 | REFSENS | avoided |
|  | 3 | 26 | 5 | -81.2 | dual uplink operation |
|  |  | n41 | 10 | REFSENS |  |
|  | 4 | 26 | 5 | -72.5 | harmonic mixing |
|  |  | n41 | 10 | REFSENS |  |
|  | 5 | 26 | 15 | -69.5 |  |
|  |  | n41 | 15 | REFSENS |  |
|  | 6 | 26 | 15 | REFSENS | harmonic mixing |
|  |  | n41 | 15 | REFSENS | avoiding |
| NOTE 1: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1a and Table 7.3.2.5-1b for 2 antenna port NR band, Table 7.3.2.5-2a and Table 7.3.2.5-2b for 4 antenna port NR band. |  |  |  |  |  |

Table 7.3B.2.3.5-1: Reference sensitivity due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | SCS (kHz) |  | 5 MHz (dBm) | 10 MHz (dBm) |  | 15 MHz (dBm) |  | 20 MHz (dBm) |  | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 3 | n772 | 15 |  |  | -71.4 +TT |  | -71.4 +TT |  | -71.3 +TT |  |  |  | -71.2 +TT | -71.3 +TT |  |  |  |  |
|  |  | 30 |  |  | -71.7 +TT |  | -71.5 +TT |  | -71.5 +TT |  |  |  | -71.3 +TT | -71.4 +TT | -71.4 +TT | -71.3 +TT | -71.3 +TT | -71.3 +TT |
|  |  | 60 |  |  | -72.1 +TT |  | -71.8 +TT |  | -71.7 +TT |  |  |  | -71.5 +TT | -71.5 +TT | -71.5 +TT | -71.4 +TT | -71.4 +TT | -71.4 +TT |
|  | n773 | 15 |  |  | -94.2 +TT |  | -92.7 +TT |  | -91.9 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.5 +TT |  | -92.8 +TT |  | -92.1 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | -94.9 +TT |  | -93.1 +TT |  | -92.3 +TT |  |  |  |  |  |  |  |  |  |
| 2 | n772, 13 | 15 |  |  | -71.4 +TT |  | -71.4 +TT |  | -71.3 +TT |  |  |  | -71.2 +TT | -71.3 +TT |  |  |  |  |
|  |  | 30 |  |  | -71.7 +TT |  | -71.5 +TT |  | -71.5 +TT |  |  |  | -71.3 +TT | -71.4 +TT | -71.4 +TT | -71.3 +TT | -71.3 +TT | -71.3 +TT |
|  |  | 60 |  |  | -72.1 +TT |  | -71.8 +TT |  | -71.7 +TT |  |  |  | -71.5 +TT | -71.5 +TT | -71.5 +TT | -71.4 +TT | -71.4 +TT | -71.4 +TT |
|  | n773 | 15 |  |  | -94.2 +TT |  | -92.7 +TT |  | -91.9 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.5 +TT |  | -92.8 +TT |  | -92.1 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | -94.9 +TT |  | -93.1 +TT |  | -92.3 +TT |  |  |  |  |  |  |  |  |  |
| 2 | n782 | 15 |  |  | -71.9+TT |  | -71.9 +TT |  | -71.8 +TT |  |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  | 30 |  |  | -72.2 +TT |  | -72.0 +TT |  | -72.0 +TT |  |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  | 60 |  |  | -72.6 +TT |  | -72.3 +TT |  | -72.2 +TT |  |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
| 2 | n783 | 15 |  |  | -94.7 +TT |  | -93.2 +TT |  | -92.4 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.9 +TT |  | -93.3 +TT |  | -92.6 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | --95.4 +TT |  | -93.6 +TT |  | -92.8 +TT |  |  |  |  |  |  |  |  |  |
| 3 | n782 | 15 |  |  | -71.9 +TT |  | -71.9 +TT |  | -71.8 +TT |  |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  | 30 |  |  | -72.2 +TT |  | -72.0 +TT |  | -72.0 +TT |  |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  | 60 |  |  | -72.6 +TT |  | -72.3 +TT |  | -72.2 +TT |  |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
|  | n783 | 15 |  |  | -94.7 +TT |  | -93.2 +TT |  | -92.4 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -95.0 +TT |  | -93.3 +TT |  | -92.6 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | -95.4 +TT |  | -93.6 +TT |  | -92.8 +TT |  |  |  |  |  |  |  |  |  |
| 8 | n776,7 n786,7 | 15 |  |  | -84.5 +TT |  | -84.4 +TT |  | -84.2 +TT |  |  |  | -84.0 +TT | -83.9 +TT |  |  |  |  |
|  |  | 30 |  |  | -84.8 +TT |  | -84.5 +TT |  | -84.4 +TT |  |  |  | -84.1 +TT | -84.0 +TT | -83.9 +TT | -83.8 +TT | -83.5 +TT | -83.7 +TT |
|  |  | 60 |  |  | -85.2 +TT |  | -84.8 +TT |  | -84.6 +TT |  |  |  | -84.3 +TT | -84.1 +TT | -84.0 +TT | -83.9 +TT | -83.6 +TT | -83.8 +TT |
| 8 | n41 | 15 |  |  | -81.8 +TT |  | -81.7 +TT |  | -81.7 +TT |  |  |  | -81.6 +TT | -81.5 +TT |  |  |  |  |
|  |  | 30 |  |  | -82.1 +TT |  | -81.8 +TT |  | -81.9 +TT |  |  |  | -81.7 +TT | -81.6 +TT | -81.4 +TT | -81.3 +TT | -81.2 +TT | -81.2 +TT |
|  |  | 60 |  |  | -82.5 +TT |  | -82.1 +TT |  | -81.1 +TT |  |  |  | -81.9 +TT | -81.7 +TT | -81.6 +TT | -81.3 +TT | -81.2 +TT | -81.2 +TT |
| 12 | n66 | 15 |  | -89.5 +TT | -88.8 +TT |  | -88.3 +TT |  | -87.8 +TT |  |  |  | -87.7 +TT |  |  |  |  |  |
|  |  | 30 |  |  | -89.1 +TT |  | -88.4 +TT |  | -88.0 +TT |  |  |  | -87.8 +TT |  |  |  |  |  |
|  |  | 60 |  |  | -89.5 +TT |  | -88.7 +TT |  | -88.2 +TT |  |  |  | -87.9 +TT |  |  |  |  |  |
| 12 | n774, 5 | 15 |  |  | -84.9 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  |  | -85.2 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  |  | -85.6 +TT |  | -85 +TT |  | -84.8 +TT |  |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 12 | n78 | 15 |  |  | -85.4 +TT |  | -85.1 +TT |  | -84.9 +TT |  | -84.6+TT | -84.4+TT | -84.9 +TT | -84.9 +TT |  |  |  |  |
|  |  | 30 |  |  | -85.7 +TT |  | -85.2 +TT |  | -85.1 +TT |  | -84.7+TT | -84.5+TT | -85.0 +TT | -85.0 +TT | -84.9 +TT | -84.9 +TT | -84.9 +TT | -84.9 +TT |
|  |  | 60 |  |  | -86.1 +TT |  | -85.5 +TT |  | -85.3 +TT |  | -84.9+TT | -84.6+TT | -85.2 +TT | -85.1 +TT | -85.0 +TT | -85.0 +TT | -85.0 +TT | -85.0 +TT |
| 13 | n774, 5 | 15 |  |  | -84.9 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  |  | -85.2 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  |  | -85.6 +TT |  | -85 +TT |  | -84.8 +TT |  |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 14 | n774, 5 | 15 |  |  | -84.9 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  |  | -85.2 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  |  | -85.6 +TT |  | -85 +TT |  | -84.8 +TT |  |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 18 | n774,5 n784,5 | 15 |  |  | -84.9 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  |  | -85.2 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  |  | -85.6 +TT |  | -85.0 +TT |  | -84.8 +TT |  |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 19 | n774,5 |  | 15 |  |  | -84.9 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  |  | 30 |  |  | -85.2 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  |  | 60 |  |  | -85.6 +TT |  | -85.0 +TT |  | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
|  | n776,7 n786,7 |  | 15 |  |  | -84.8 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  | -83.7 +TT | -83.9 +TT |  |  |  |  |
|  |  |  | 30 |  |  | -85.1 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  | -83.8 +TT | -84.0 +TT | -83.9 +TT | -83.8 +TT | -83.5 +TT | -83.7 +TT |
|  |  |  | 60 |  |  | -85.5 +TT |  | -85.0 +TT |  | -84.8 +TT |  |  | -84.0 +TT | -84.1 +TT | -84.0 +TT | -83.9 +TT | -83.6 +TT | -83.8 +TT |
| 21 | n28 | 15 |  | -96.0 +TT | -93.0 +TT |  | -91.0 +TT |  | -88.3 +TT |  |  | -76.0 +TT |  |  |  |  |  |  |
|  |  | 30 |  |  | -93.1 +TT |  | -91.1 +TT |  | -89.5 +TT |  |  | -76.1 +TT |  |  |  |  |  |  |
| 28 | n774,5 n784,5 | 15 |  |  | -84.9 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  |  | -85.2 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  |  | -85.6 +TT |  | -85.0 +TT |  | -84.8 +TT |  |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 20 | n776,7 n786,7 | 15 |  |  | -84.5 +TT |  | -84.4 +TT |  | -84.2 +TT |  |  |  | -83.1 +TT |  |  |  |  |  |
|  |  | 30 |  |  | -84.8 +TT |  | -84.5 +TT |  | -84.4 +TT |  |  |  | -83.2 +TT |  |  |  |  |  |
|  |  | 60 |  |  | -85.2 +TT |  | -84.8 +TT |  | -84.6 +TT |  |  |  | -83.4 +TT |  |  |  |  |  |
| 26 | n776,7 n786,7 | 15 |  |  | -85.0 +TT |  | -84.9 +TT |  | -84.7 +TT |  |  |  | -83.6 +TT | -84.6 +TT |  |  |  |  |
|  |  | 30 |  |  | -85.3 +TT |  | -85.0+TT |  | -84.9 +TT |  |  |  | -83.7 +TT | -84.7 +TT | -84.7 +TT | -84.6 +TT | -84.6 +TT | -84.6 +TT |
|  |  | 60 |  |  | -85.7 +TT |  | --85.3 +TT |  | --85.1 +TT |  |  |  | -83.9 +TT | -84.8 +TT | -84.8 +TT | -84.7 +TT | -84.7 +TT | -84.7 +TT |
| 26 | n774,5 | 15 |  |  | -84.9 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  |  | -85.2 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -85.6 +TT | -84.4 +TT |
|  |  | 60 |  |  | -85.6 +TT |  | -85.0 +TT |  | -84.8 +TT |  |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -85.7 +TT | -84.5 +TT |
| n28 | 18,9,10 | N/A |  | -89.1 | -88.7 |  | -88.3 |  | -88.0 |  |  |  |  |  |  |  |  |  |
| 71 | n211 | 15 |  | -93.4 +TT | -93.8 +TT |  | -92.3 +TT |  | -91.2 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.1 +TT |  | -92.4 +TT |  | -91.4 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | -94.5 +TT |  | -92.7 +TT |  | -91.6 +TT |  |  |  |  |  |  |  |  |  |
| 71 | n212 | 15 |  | -96.3 +TT | -93.8 +TT |  | -92.3 +TT |  | -91.2 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.1 +TT |  | -92.4 +TT |  | -91.4 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | -94.5 +TT |  | -92.7 +TT |  | -91.6 +TT |  |  |  |  |  |  |  |  |  |
| n71 | 211 | N/A |  | -92.7 | -93.3 |  | -91.8 |  | -90.7 |  |  |  |  |  |  |  |  |  |
| n71 | 212 | N/A |  | -95.6 | -93.3 |  | -91.8 |  | -90.7 |  |  |  |  |  |  |  |  |  |
| n71 | 7 | N/A |  | -82.7 | -82.6 |  | -82.4 |  | -82.3 |  |  |  |  |  |  |  |  |  |
| 66 | n772, 13 | 15 |  |  | -71.4 +TT |  | -71.4 +TT |  | -71.3 +TT |  |  |  | -71.2 +TT | -71.3 +TT |  |  |  |  |
|  |  | 30 |  |  | -71.7 +TT |  | -71.5 +TT |  | -71.5 +TT |  |  |  | -71.3 +TT | -71.4 +TT | -71.4 +TT | -71.3 +TT | -71.3 +TT | -71.3 +TT |
|  |  | 60 |  |  | -72.1 +TT |  | -71.8 +TT |  | -71.7 +TT |  |  |  | -71.5 +TT | -71.5 +TT | -71.5 +TT | -71.4 +TT | -71.4 +TT | -71.4 +TT |
|  | n773 | 15 |  |  | -94.2 +TT |  | -92.7 +TT |  | -91.9 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.5 +TT |  | -92.8 +TT |  | -92.1 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | -94.9 +TT |  | -93.1 +TT |  | -92.3 +TT |  |  |  |  |  |  |  |  |  |
| 66 | n782 | 15 |  |  | -71.9 +TT |  | -71.9 +TT |  | -71.8 +TT |  |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  | 30 |  |  | -72.2+TT |  | -72.0 +TT |  | -72.0 +TT |  |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  | 60 |  |  | -72.6 +TT |  | -72.3 +TT |  | -72.2 +TT |  |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
|  | n783 | 15 |  |  | -94.7 +TT |  | -93.2 +TT |  | -92.4 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -95.0 +TT |  | -93.3 +TT |  | -92.6 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | -95.4 +TT |  | -93.6 +TT |  | -92.8 +TT |  |  |  |  |  |  |  |  |  |
| n66 | 48,2, 13 | N/A |  | -70.7 | -70.6 |  | -70.8 |  | -70.8 |  |  |  |  |  |  |  |  |  |
| n66 | 48,3 | N/A |  | -96.1 | -93.6 |  | -92.3 |  | -91.6 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , where andare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 9 The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 10: Void NOTE 11: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 12: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n48, DC_2_n77, DC_42_n3, DC_48_n25, DC_48_n66, DC_66_n48, DC_66_n77, DC_3_n77, DC_3_n78, DC_11_n28 and DC_28_n50, DC_28_n51, DC_66_n78, DC_25_n77, DC_25_n78. NOTE 14: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 15: Void NOTE 16: Void NOTE 17: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-1a: Test Tolerance (TT) for RX sensitivity level

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-2 with uplink configuration specified in Table 7.3B.2.3.4.2.1-3a to Table 7.3B.2.3.4.2.1-3k for each specific EN-DC combination scenario.

Table 7.3B.2.3.5-2: Reference sensitivity due to receiver harmonic mixing for PC3 EN-DC in NR FR1

| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | n714 | 15 | -70.4 +TT | -70.4 +TT | -70.4 +TT | -70.4 +TT |  |  |  |  |  |  |  |
|  |  | 30 |  | -70.7 +TT | -70.7 +TT | -71.8 +TT |  |  |  |  |  |  |  |
|  |  | 60 |  | -72.4 +TT | -72.7 +TT | -77.0 +TT |  |  |  |  |  |  |  |
| n2 | 71 | N/A | -69.7 | -69.9 | -70.1 | -71.2 |  |  |  |  |  |  |  |
| n77 | 2 | N/A | -91.2 | -89.3 | -88.5 | -87.6 |  |  |  |  |  |  |  |
| n77 | 13 | N/A |  | -65.3 | -65.3 |  |  |  |  |  |  |  |  |
| n77 | 12 | N/A | -65.3 | -65.3 |  |  |  |  |  |  |  |  |  |
| n77 | 418 | N/A | -86.9 | -83.9 | -82.1 | -80.9 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n77 | 14 | N/A | -65.3 | -65.3 |  |  |  |  |  |  |  |  |  |
| n77 | 282 | N/A | -69.8 | -69.8 | -69.8 | -68.3 |  |  |  |  |  |  |  |
| n78 | 3 | N/A | -90.6 | -89.3 | -88.5 | -87.6 |  |  |  |  |  |  |  |
| n78 | 19 | N/A | -92.1 | -91.3 | -90.7 |  |  |  |  |  |  |  |  |
| n78 | 40 | N/A | -86.9 | -83.9 | -82.1 | -80.9 |  |  |  |  |  |  |  |
| n78 | 418 | N/A | -86.9 | -83.9 | -82.1 | -80.9 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n79 | 192 | N/A | -69.8 | -69.8 | -69.8 |  |  |  |  |  |  |  |  |
| n79 | 214 | N/A | -60.0 | -60.0 | -60.0 |  |  |  |  |  |  |  |  |
| n79 | 262 | N/A | -69.8 | -69.8 | -69.8 | N/A | N/A | N/A | N/A | N/A | N/A |  | N/A |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band andfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void. NOTE 6: Void. NOTE 7: The requirements should be verified for DL EARFCN of the victim (higher) band (superscript HB) such that fDLHB=50×fULLB0.03 with fDLHB the DL carrier frequency in the higher band and fULLB the UL carrier frequency in the lower band, both in MHz. NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band andfULHBfULHBfDLLB=fULHB/0.150.1 the UL carrier frequency in the higher band, both in MHz. NOTE 9: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 10: Void |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-2a: Reference sensitivity due to receiver harmonic mixing for PC2 EN-DC in NR FR11

| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n77 | 2 | N/A | -88.2 | -86.3 | -85.5 | -84.6 |  |  |  |  |  |  |  |
| n77 | 3 | N/A | -88.2 | -86.3 | -85.5 | -84.6 |  |  |  |  |  |  |  |
| n77 | 132 | N/A |  | -62.3 | -62.3 |  |  |  |  |  |  |  |  |
| n77 | 12 | N/A | -65.3 | -65.3 |  |  |  |  |  |  |  |  |  |
| n77 | 14 | N/A | -65.3 | -65.3 |  |  |  |  |  |  |  |  |  |
| n77 | 282 | N/A | -66.8 | -66.8 | -66.8 | -65.3 |  |  |  |  |  |  |  |
| n78 | 19 | N/A | -89.5 | -89.1 | -88.7 |  |  |  |  |  |  |  |  |
| n79 | 192 | N/A | -66.8 | -66.8 | -66.8 |  |  |  |  |  |  |  |  |
| n79 | 214 | N/A | -57.0 | -57.0 | -57.0 |  |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band andfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void. NOTE 6: Void. NOTE 7: The requirements should be verified for DL EARFCN of the victim (higher) band (superscript HB) such that fDLHB=50×fULLB0.03 with fDLHB the DL carrier frequency in the higher band and fULLB the UL carrier frequency in the lower band, both in MHz. NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band andfULHBfULHBfDLLB=fULHB/0.150.1 the UL carrier frequency in the higher band, both in MHz. NOTE 9: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 10: Void |  |  |  |  |  |  |  |  |  |  |  |  |  |

Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-3 with uplink configuration specified in Table 7.3B.2.3.4.2.1-4a to Table 7.3B.2.3.4.2.1-4n.

Table 7.3B.2.3.5-3: Reference sensitivity exceptions due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| n1 | 3 | N/A | -93.3 | -91 | -89.5 | -88.5 |  |  |  |  |  |  |  |  |  |
| 1 | n3 | 15 | -94 +TT | -91.6 +TT | -90.1 +TT | -89.1 +TT | -88.1 +TT | -87.4 +TT | -86.2 +TT |  |  |  |  |  |  |
| 1 | n41 | 30 |  | -89.0 +TT | -87.0 +TT | -85.9 +TT |  | -83.9 +TT | -82.6 +TT | -81.6 +TT | -80.8 +TT |  | -79.5 +TT | -79.0 +TT | -78.6 +TT |
| 3 | n41 | 30 |  | -94.4 +TT | -92.4 +TT | -91.3 +TT |  | -89.3 +TT | -88.0 +TT | -87 +TT | -86.2 +TT |  | -84.9 +TT | -84.4 +TT | -84.0 +TT |
| n5 | 28 | 15 | -93.3 | -91.8 | -90.8 | -90 |  |  |  |  |  |  |  |  |  |
| n41 | 1 | 15 | -90.2 | -87.2 | -85.4 | -84.2 |  |  |  |  |  |  |  |  |  |
| n41 | 2 | N/A | -96.7 | -93.7 | -91.9 | -90.7 |  |  |  |  |  |  |  |  |  |
| n41 | 3 | N/A | 3 | n41 | 30 | 3 |  |  |  |  |  |  |  |  |  |
| n41 | 25 | N/A | -95.2 | -92.2 | -90.4 | -89.2 |  |  |  |  |  |  |  |  |  |
| n77 | 411 | N/A | -92.8 | -89.8 | -88.0 | -86.8 |  |  |  |  |  |  |  |  |  |
| n41 | 66 | N/A | -95.3 | -92.3 | -90.5 | -89.3 |  |  |  |  |  |  |  |  |  |
| 41 | n773 | 30 |  | -87.3 +TT | -85.3 +TT | -84.1 +TT |  |  | -82.9 +TT | -82.9 +TT | -82.9 +TT |  | -82.1 +TT | -81.7 +TT | -81.3 +TT |
| 30 | n66 | 15 | -91.2 +TT | -88.0 +TT | -86.2 +TT | -85.0 +TT |  |  |  | -81.8 +TT |  |  |  |  |  |
| n78 | 71 | N/A | -93.5 | -90.5 | -88.7 | -87.5 |  |  |  |  |  |  |  |  |  |
| n78 | 38 | N/A | -96.7 | -93.7 | -91.9 | -90.7 |  |  |  |  |  |  |  |  |  |
| n78 | 411 | N/A | -92.8 | -89.8 | -88.0 | -86.8 |  |  |  |  |  |  |  |  |  |
| 41 | n78 | 30 |  | -87.8 +TT | -85.8 +TT | -84.6 +TT |  |  | -83.4 +TT | -83.4 +TT | -83.4 +TT |  | -82.6 +TT | -82.2 +TT | -81.8 +TT |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 3: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 4: Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-3a: Reference sensitivity exceptions due to cross band isolation for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| 3 | n41 | 30 | -94.4 +TT | -92.4 +TT | -91.3 +TT |  | -89.3 +TT | -88.0 +TT | -87.0 +TT | -86.2 +TT |  | -84.9 +TT | -84.4 +TT | -84.0 +TT |  |
| n41 | 1 | 30 | -87.5 | -84.5 | -82.7 | -81.5 |  |  |  |  |  |  |  |  |  |
| n41 | 2 | N/A | -98.0 +1.6 +TT | -95.0 +1.6 +TT | -93.2 +1.6 +TT | -92.0 +1.6 +TT |  |  |  |  |  |  |  |  |  |
| n41 | 3 | N/A | -94.0 | -91.0 | -89.2 | -88.0 |  |  |  |  |  |  |  |  |  |
| n77 | 2 | N/A | -98.0 +1.0 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 411 | 30 |  | -83.3 | -81.5 | -80.3 |  |  |  |  |  |  |  |  |  |
| n77 | 66 | N/A | -99.5 +1.0 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: TT is the same as defined in Table 7.3B.2.3.5-1a. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-4 with uplink configuration specified in Table 7.3B.2.3.4.2.1-6.

Table 7.3B.2.3.5-4: Reference sensitivity exceptions due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | IMD order | Duplex mode |
| DC_1A_n3A | 1 | N/A | [-76.3] | - | - | - | - | IMD3 | FDD |
|  | n3 | 15 | REFSENS |  | - | - | - | N/A | TDD |
| DC_1A_n8A | 1 | N/A | -93.3 | - | - | - | - | IMD4 | FDD |
|  | n8 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_1A_n77A | 1 | N/A | -69.5 | - | - | - | - | IMD23 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n77A | 1 | N/A | -91.3 | - | - | - | - | IMD4- | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n78A | 1 | N/A | -91.3 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n66A | 2 | N/A | -77.3 | - | - | - | - | IMD3 |  |
|  | n66 | 15 | REFSENS | - | - | - | - | N/A |  |
| DC_2A_n66A | 2 | N/A | REFSENS | - | - | - | - | N/A |  |
|  | n66 | 15 | -95.5 +TT | - | - | - | - | IMD5 |  |
| DC_2A_n77A. DC_2A_n77(2A) | 2 | N/A | -71.3 | - | - | - | - | IMD2 | FDD |
| DC_2A-2A_n77A DC_2A_n77(2A) DC_2A-2A_n77(2A) | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 2 | N/A | -89.3 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 2 | N/A | -92.3 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n78A | 2 | N/A | -71.3 | - | - | - | - | IMD23 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | - | TDD |
| DC_2A_n78A | 2 | N/A | -89.3 | - | - | - | - | N/A | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | IMD43 | TDD |
| DC_3A_n1A | 3 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n1 | 15 | -74.0+TT | - | - | - | - | IMD3 | FDD |
| DC_3A_n5A | 3 | N/A |  | -89.3 | - | - | - | IMD4 | FDD |
|  | n5 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 3 | N/A |  | REFSENS | - | - | - | N/A | FDD |
|  | n5 | 15 | -74.0+TT | - | - | - | - | IMD23 | FDD |
| DC_3A_n7A | 3 | N/A | REFSENS | - | - | - | - | - |  |
|  | n7 | 15 | - | -84.6 +TT5 | - | - | - | - |  |
| DC_3A_n8A | 3 | N/A | - | REFSENS | - | - | - | N/A | FDD |
|  | n8 | 15 | -88.3 | - | - | - | - | IMD43 | FDD |
|  | 3 | N/A | - | -86.9 | - | - | - | IMD5 | FDD |
|  | n8 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_3A_n41A | 3 | N/A | -88.1 | - | - | - | - | IMD4 | FDD |
|  | n41 |  | REFSENS | - | - | - | - |  |  |
| DC_3A_n77A, DC_3A_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | N/A | -70.3 | - | - | - | - | IMD23 | FDD |
|  |  |  | -70.67 | - | - | - | - |  |  |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n77A, DC_3A_n77(2A), DC_3C_n77A, DC_3C_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | N/A | -88.3 | - | - | - | - | IMD43 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n66A | 5 | N/A | -67.3 | - | - | - | - | IMD23 | FDD |
|  | n66 |  | REFSENS | - | - | - | - | N/A | FDD |
| DC_5A_n77A8e | 5 | N/A | -89 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 5 | N/A | -91.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_7A_n3A | 7 | N/A | - | -81.3 | - | - | - | IMD4 | FDD |
|  | n3 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_7A_n5A | 7 | N/A |  | REFSENS | - | - | - | N/A | FDD |
|  | n5 | 15 | -86.0+TT | - | - | - | - | IMD33 | FDD |
| DC_7A_n66A | 7 | N/A | - | -79.3 | - | - | - | IMD4 | FDD |
|  | n66 | 15 | REFSENS | - |  | - | - | N/A | TDD |
| DC_7A_n77A | 7 | N/A | -90.2 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_8A_n1A | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n1 | 15 | -94.0 + TT | - | - | -- | - | IMD4 | FDD |
| DC_8A_n3A | 8 | N/A | -88.3 | - | - | - | - | IMD43 | FDD |
|  | n3 | 15 | - | REFSENS | - | - | - | N/A | FDD |
|  | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n3 | 15 | - | -87.4+ TT | - | - | - | IMD5 | FDD |
| DC_8A_n20A | n20 | 15 | -71.3 | - | - | - | - | IMD3 | FDD |
|  | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n20 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 8 | N/A | -71.3 | - | - | - | - | IMD3 | FDD |
| DC_8A_n41A | 8 | N/A | -84.2 | - | - | - | - | IMD33 | FDD |
|  | n41 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_8A_n77A DC_8A_n78A DC_8A-SUL_n78A-n81A | 8 | N/A | -88.0 | - | - | - | - | IMD4 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | H4 | TDD |
| DC_12A_n77A DC_12A_n77(2A) | 12 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_12_n78 | 12 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_13A_n77A | 13 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_14A_n77A DC_14A_n77(2A) | 14 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_18A_n77A | 18 | N/A | -90.9 | - | - | - | - | IMD4X | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 18 | N/A | -94.8 | - | - | - | - | IMD5X | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_18A_n78A | 18 | N/A | -90.9 | - | - | - | - | IMD4Y | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_19A_n78A | 19 | N/A | -85.7 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | FDD |
| DC_20A_n3A | 20 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n3 | 15 | -93.0 +TT | - | - | - | - | IMD4 | FDD |
|  | 20 | N/A | -87.3 | - | - | - | - | IMD4 | FDD |
|  | n3 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_20_n7 | 20 | N/A | -84.3 | - | - | - | - | IMD3 | FDD |
|  | n7 | 15 | - | REFSENS | - | - | - | N/A | FDD |
| DC_20_n8 | 20 | N/A | -71.3 | - | - | - | - | IMD3 | FDD |
|  | n8 | 15 | -71.3 | - | - | - | - | IMD3 | FDD |
| DC_20A_n78A, DC_20A-SUL_n78A-n82A | 20 | N/A | -74.6 | - | - | - | - | IMD44 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_21A_n28A | 21 | N/A | -96.8 | - | - | - | - | IMD5 | FDD |
|  | n28 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_21A_n79A | 21 | N/A | -80.9 | - | - | - | - | IMD3 | FDD |
|  | n79 | 15 | - | - | - | - | REFSENS | N/A | TDD |
| CA_28A_n77A, CA_28A_n78A, DC_28A-SUL_n78A-n83A | 28 | N/A | -92.3 | - | - | - | - | IMD5 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_30A_n77A DC_30A_n77(2A) | 30 | N/A | -90.3 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_48A_n66A | 48 | N/A | - | REFSENS | - | - | - | N/A | TDD |
|  | n66 | 15 | -95.5 +TT | - | - | - | - | IMD5 | FDD |
| DC_66A_n2A | 66 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n2 | 15 | -78.0+TT | - | - | - | - | IMD3 | FDD |
|  | 66 | N/A | -94.8 | - | - | - | - | IMD5 | FDD |
|  | n2 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n5A | 66 | N/A | -68.8 | - | - | - | - | IMD23 | FDD |
|  | n5 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n25A | 66 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n25 | 15 | -76.5+TT | - | - | - | - | IMD3 | FDD |
|  | 66 | N/A | -75.8 | - | - | - | - | IMD3 | FDD |
|  | n25 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 66 | N/A | -94.8 | - | - | - | - | IMD5 | FDD |
|  | n25 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n71A | 66 | N/A | -93.8 | - | - | - | - | IMD4 | FDD |
|  | n71 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n77A DC_66A_n77(2A) DC_66A-66A_n77A DC_66A-66A_n77(2A) | 66 | N/A | -67.8 | - | - | - | - | IMD2 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 66 | N/A | -93.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_66A_n78A | 66 | N/A | -93.8 | - | - | - | - | IMD5 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_71A_n66A | 71 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n66 | 15 | -94.5 +TT | - | - | - | - | IMD4 | FDD |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 5: Void NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 7: Void. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 9: For a UE which supports this band combination only when the Band n77 frequency range restriction of 3400 - 4100 MHz in Japan applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 10: For a UE which supports this band combination only when the Band n78 frequency range restriction of 3400 - 3800 MHz in Japan applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-5: Reference sensitivity exceptions due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm)) | 20 MHz (dBm) | 40 MHz (dBm) | IMD order) | Duplex mode |
| DC_1A_n77A | 1 | N/A | -63.5 | - | - | - | - | IMD23 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 1 | N/A | -81.5 | - | - | - | - | IMD43 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n78A | 1 | N/A | REFSENS-17.8 | - | - | - | - | IMD4 |  |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n77A | 2 | N/A | -65.2 | - | - | - | - | IMD2 | FDD |
| DC_2A-2A_n77A |  |  | -65.157 |  |  |  |  |  |  |
| DC_2A_n77C | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A-2A_n77C | 2 | N/A | -78.2 | - | - | - | - | IMD49 | FDD |
|  |  |  | -78.157 |  |  |  |  |  |  |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n41A | 3 | N/A | -77.9 | - | - | - | - | IMD4 | FDD |
|  |  |  | -80.67 | - | - | - | - |  |  |
|  | n41 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n78A | 3 | N/A | -64.4 | - | - | - | - | IMD2 | FDD |
|  |  |  | -67.17 | - | - | - | - |  |  |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n78A | 3 | N/A | -77.8 | - | - | - | - | IMD4 | FDD |
|  |  |  | -80.57 | - | - | - | - |  |  |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n77A8 | 5 | N/A | -78.7 | - | - | - | - | IMD49 | FDD |
| DC_5A_n77C8 | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_12A_n77A | 12 | N/A | -84.6 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_13A_n77A | 13 | N/A | -80.93 | - | - | - | - | IMD5 | FDD |
| DC_13A_n77C | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_14A_n77A | 14 | N/A | -84.6 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_19A_n78A | 19 | N/A | -74.0 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | FDD |
| DC_21A_n79A | 21 | N/A | -65.9 | - | - | - | - | IMD3 | FDD |
|  | n79 | 15 | - | REFSENS |  | - | - | N/A | TDD |
| DC_28A_n77A | 28 | N/A | -78.6 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_30A_n77A | 30 | N/A | -81.4 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_66A_n77A DC_66A-66A_n77A DC_66A-66A-66A_n77A DC_66A_n77C DC_66A-66A_n77C DC_66A-66A-66A_n77C | 66 | N/A | -64.47 | - | - | - | - | IMD2 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 66 | N/A | -87.53 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| NOTE 1: E-UTRA carrier shall be set to min(+23 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+23 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 5: For UEs only indicating support of Single UL, this requirement is verified with non-simultaneous uplink transmissions on the E-UTRA and NR CGs. NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 7: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 9: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |

==============================================================

<< End of changes >>