3GPP TSG RAN5 Meeting #106		R5-251529

Athens, Greece, 17th - 21st February 2025

Source:		Rohde & Schwarz

Title:	On the MU for 8Rx testing

Agenda Item:	5.3.24.10

Document for:	Discussion and Endorsement

# Introduction

In the last RAN5 meeting several TCs applying interferer signals have been updated for 8Rx testing [1, 2]. However, the impact of 8Rx on the MU has not yet been discussed. In this paper we present the results of our MU assessment and propose to update the MU of the impacted TCs.

# Discussion

In the analysis of the MU of the blocking and ACS test cases the impact of the interferer ACLR and interferer broadband noise is relevant. These MU contributors describe the increase of the wanted DL signals due to the interferer ACLR or due to the interferer broadband noise.

Receiver test cases such as the blocking or ACS test cases use the RefSens requirement for specifying the power levels of the wanted signal. Therefore, the wanted signal power level is significantly lower for 8Rx than for 4Rx.

The interferer power level is specified independent of the RefSens requirement. Thus, the interferer power leaking into the frequency range of the wanted DL signal is the same for 4Rx and 8Rx. Due to the lower distance of the wanted signal to the interferer leakage signal, the impact of interferer broadband noise and interferer ACLR are larger for 8Rx compared to 4Rx.

Since state-of-the-art test equipment has already to operate close to the physical limit of thermal noise in order to comply with the 4Rx MTSU, we propose to update the MU to enable testing of 8Rx.

## Wanted signal power level for 4Rx and 8Rx

### The dependence of the power level on the number of UE Rx antenna connectors is specified by ΔRIB,4R and ΔRIB,8R in Tables 7.3.2-2 and 7.3.2-2a of TS 38.101-1 [3], respectively:

For UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,4R in Table 7.3.2-2 for the applicable operating bands. For operating band frequency range ≤ 1 GHz, the 4Rx operation is primarily for FWA form factor, and when 4Rx operation is supported by handheld UE, ∆RIB,4R as indicated in Table 7.3.2-2 NOTE 2 is applied.

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n5, n8, n13, n26, n28, n71, n85, n105 | -2.71 |
| n5, n8, n28, n71, n20, n26 | -2.42 |
| n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79, n104 | -2.2 |
| NOTE 1: When 4 Rx operation is supported by FWA form factor NOTE 2: When 4Rx operation is supported by handheld UE. |  |

For UE(s) equipped with 8 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,8R in Table 7.3.2-2a for the applicable operating bands.

Table 7.3.2-2a: Eight antenna port reference sensitivity allowance ΔRIB,8R

| Operating band | ΔRIB,8R (dB) |
| --- | --- |
| n7 | -4.5 |
| n41 | -4.3 |
| n77, n78, n79 | -4.0 |
| NOTE 1: 8 Rx operation is targeted for FWA/CPE/Vehicle/Industrial devices form factor. |  |

Thus, the 8Rx RefSens requirement is up to 1.8 dB lower than the 4Rx RefSens requirement.

Observation 1: The 8Rx RefSens requirement is up to 1.8 dB lower than the 4Rx RefSens requirement.

## Derivation of the 8Rx impact of interferer ACLR and interferer broadband noise.

The impact of interferer ACLR and interferer broadband noise in dB is given by:

impact=10⋅log101+10(-SINR/10 ,

where SNR =  wanted signal power – interferer leakage power. This is equivalent to

SNR=-10⋅log1010impact/10-1

With  SNR8Rx= SNR4Rx-1.8 dB, the impact for 8Rx can be calculated based on the impact for 4Rx.

## Adjacent channel selectivity and in-band blocking TCs

For TC 7.5 and TC 7.6.2, RAN5 has assumed a frequency dependent impact of interferer ACLR as stated in Table 1 [4]:

Table 1: Impact of interferer ACLR for ACS TC for 4Rx

|  | f ≤ 3 GHz | 3 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6 GHz |
| --- | --- | --- | --- |
| Impact of interferer ACLR for 4 Rx (dB) | 0.4 | 0.7 | 0.7 |

Based on the formulas in section 2.2, the impact of the interferer ACLR for 8Rx and the MU increase are calculated, and the results are shown in Table 2.

Table 2: Calculation of impact for 8Rx and MU increase

| impact for 4Rx (dB) | SNR for 4Rx (dB) | SNR for 8Rx (dB) | impact for 8Rx (dB) | impact for 8Rx - impact for 4Rx (dB) |
| --- | --- | --- | --- | --- |
| 0.4 | 10.16 | 8.36 | 0.59 | 0.19 |
| 0.7 | 7.57 | 5.77 | 1.02 | 0.32 |

### Therefore, we propose to update MTSU for TC 7.5 as described in Table 3:

Table 3: Updated MTSU for ACS an in-band blocking TCs

| 7.5 Adjacent channel selectivity | ACS value 1Rx, 2Rx, 4Rx: ±1.6 dB, f ≤ 3.0GHz ±2.3 dB, 3.0GHz < f ≤ 4.2GHz ±3.0 dB, 4.2GHz < f ≤ 6.0GHz 8Rx: ±1.8 dB, f ≤ 3.0GHz ±2.7 dB, 3.0GHz < f ≤ 4.2GHz ±3.4 dB, 4.2GHz < f ≤ 6.0GHz |
| --- | --- |
| 7.5F.1 Adjacent channel selectivity for shared spectrum channel access | ACS value1 1Rx, 2Rx, 4Rx: ±3.0 dB, 4.2GHz < f ≤ 7.125GHz 8Rx: ±3.4 dB, 4.2GHz < f ≤ 7.125GHz |
| 7.6.2 Inband Blocking | Blocking 1Rx, 2Rx, 4Rx: ±1.6 dB, f ≤ 3.0GHz ±2.3 dB, 3.0GHz < f ≤ 4.2GHz ±3.0 dB, 4.2GHz < f ≤ 6.0GHz 8Rx: ±1.8 dB, f ≤ 3.0GHz ±2.7 dB, 3.0GHz < f ≤ 4.2GHz ±3.4 dB, 4.2GHz < f ≤ 6.0GHz |
| 7.6E.2.1 In-band blocking for V2X / non-concurrent operation | Blocking 1Rx, 2Rx, 4Rx: ±3.0 dB, 4.2GHz < f ≤ 6.0GHz 8Rx: ±3.4 dB, 4.2GHz < f ≤ 6.0GHz |
| 7.6F.2.1 In-band blocking for shared spectrum channel access | Blocking1 1Rx, 2Rx, 4Rx: ±3.0 dB, 4.2GHz < f ≤ 6.0GHz 8Rx: ±3.4 dB, 4.2GHz < f ≤ 6.0GHz |
| Note 1: In this MU analysis, only the aspect of SNR difference between 4Rx and 8 Rx has been considered in the update of NR-U MU. This does not preclude updates due to other contributions in the future, e.g. different value of impact of interferer ACLR due to different frequency range. |  |

Proposal 1: Update the MTSU for adjacent channel selectivity and in-band blocking as shown in Table 3.

## Out-of-band blocking and narrow band blocking

### As documented in Table F.1.3-1 of TS 38.521-1, RAN5 assumed a value of 0.8 dB for the impact of broadband noise in the derivation of the MTSU for TCs 7.6.3 and 7.6.4.

### Based on the formulas in section 2.2 the impact of the interferer broadband noise for 8Rx and the MU increase is calculated, and the results are shown in Table 4.

Table 4: Calculation of impact for 8Rx and MU increase

| impact for 4Rx (dB) | SNR for 4Rx (dB) | SNR for 8Rx (dB) | impact for 8Rx (dB) | impact for 8Rx - impact for 4Rx (dB) |
| --- | --- | --- | --- | --- |
| 0.8 | 6.94 | 5.14 | 1.16 | 0.36 |

Therefore, we propose to update MTSU for TCs 7.6.3 and 7.6.4 as described in Table 5:

Table 5: Updated MU for OOB and NBB TCs

| 7.6.3 Out-of-band blocking | 1Rx, 2Rx, 4Rx: Wanted signal, f ≤ 3.0GHz ±2.0 dB, Blocking, 1MHz < finterferer ≤ 3GHz ±3.9 dB, Blocking, 3GHz < finterferer ≤ 12.75GHz Wanted signal, 3.0GHz < f ≤ 4.2GHz ±2.2 dB, Blocking, 1MHz < finterferer ≤ 3GHz ±4.0 dB, Blocking, 3GHz < finterferer ≤ 12.75GHz Wanted signal, 4.2GHz < f ≤ 6GHz ±2.6 dB, Blocking, 1MHz < finterferer ≤ 3GHz ±4.2 dB, Blocking, 3GHz < finterferer ≤ 12.75GHz 8Rx: Wanted signal, f ≤ 3.0GHz ±2.4 dB, Blocking, 1MHz < finterferer ≤ 3GHz ±4.2 dB, Blocking, 3GHz < finterferer ≤ 12.75GHz Wanted signal, 3.0GHz < f ≤ 4.2GHz ±2.6 dB, Blocking, 1MHz < finterferer ≤ 3GHz ±4.3 dB, Blocking, 3GHz < finterferer ≤ 12.75GHz Wanted signal, 4.2GHz < f ≤ 6GHz ±3.0 dB, Blocking, 1MHz < finterferer ≤ 3GHz ±4.5 dB, Blocking, 3GHz < finterferer ≤ 12.75GHz |
| --- | --- |
| 7.6.4 Narrow band blocking | Blocking 1Rx, 2Rx, 4Rx: ± 2.0dB, f ≤ 3.0GHz ± 2.4dB, 3.0GHz < f ≤ 4.2GHz ± 3.1dB, 4.2GHz < f ≤ 6.0GHz 8Rx: ± 2.4dB, f ≤ 3.0GHz ± 2.8dB, 3.0GHz < f ≤ 4.2GHz ± 3.5dB, 4.2GHz < f ≤ 6.0GHz |
| 7.6E.3.1 | 1Rx, 2Rx, 4Rx: Wanted signal, 4.2GHz < f ≤ 6GHz ±2.6 dB, Blocking, 1MHz < finterferer ≤ 3GHz ±4.2 dB, Blocking, 3GHz < finterferer ≤ 12.75GHz 8Rx: Wanted signal, 4.2GHz < f ≤ 6GHz ±3.0 dB, Blocking, 1MHz < finterferer ≤ 3GHz ±4.5 dB, Blocking, 3GHz < finterferer ≤ 12.75GHz |

Proposal 2: Update the MTSU for out-of-band Blocking and narrow band blocking as shown in Table 5.

We note that TT does not need to be updated as it is 0 dB for the adjacent channel selectivity and blocking TCs.

Observation 2: An update of TT is not required as TT = 0 dB for the adjacent channel selectivity and blocking TCs.

Accompanying CR in Ref. [5] updates the MTSU in TS 38.521-1.

# Conclusion

In summary, we have assessed the MU for 8 Rx and make the following observations and proposals:

Observation 1: The 8Rx RefSens requirement is up to 1.8 dB lower than the 4Rx RefSens requirement.

Proposal 1: Update the MTSU for adjacent channel selectivity and in-band blocking as shown in Table 3.

Table 3: Updated MTSU for ACS an in-band blocking TCs

| 7.5 Adjacent channel selectivity | ACS value 1Rx, 2Rx, 4Rx: ±1.6 dB, f ≤ 3.0GHz ±2.3 dB, 3.0GHz < f ≤ 4.2GHz ±3.0 dB, 4.2GHz < f ≤ 6.0GHz 8Rx: ±1.8 dB, f ≤ 3.0GHz ±2.7 dB, 3.0GHz < f ≤ 4.2GHz ±3.4 dB, 4.2GHz < f ≤ 6.0GHz |
| --- | --- |
| 7.5F.1 Adjacent channel selectivity for shared spectrum channel access | ACS value1 1Rx, 2Rx, 4Rx: ±3.0 dB, 4.2GHz < f ≤ 7.125GHz 8Rx: ±3.4 dB, 4.2GHz < f ≤ 7.125GHz |
| 7.6.2 Inband Blocking | Blocking 1Rx, 2Rx, 4Rx: ±1.6 dB, f ≤ 3.0GHz ±2.3 dB, 3.0GHz < f ≤ 4.2GHz ±3.0 dB, 4.2GHz < f ≤ 6.0GHz 8Rx: ±1.8 dB, f ≤ 3.0GHz ±2.7 dB, 3.0GHz < f ≤ 4.2GHz ±3.4 dB, 4.2GHz < f ≤ 6.0GHz |
| 7.6E.2.1 In-band blocking for V2X / non-concurrent operation | Blocking 1Rx, 2Rx, 4Rx: ±3.0 dB, 4.2GHz < f ≤ 6.0GHz 8Rx: ±3.4 dB, 4.2GHz < f ≤ 6.0GHz |
| 7.6F.2.1 In-band blocking for shared spectrum channel access | Blocking1 1Rx, 2Rx, 4Rx: ±3.0 dB, 4.2GHz < f ≤ 6.0GHz 8Rx: ±3.4 dB, 4.2GHz < f ≤ 6.0GHz |
| Note 1: In this MU analysis, only the aspect of SNR difference between 4Rx and 8 Rx has been considered in the update of NR-U MU. This does not preclude updates due to other contributions in the future, e.g. different value of impact of interferer ACLR due to different frequency range. |  |

Proposal 2: Update the MTSU for out-of-band Blocking and narrow band blocking as shown in Table 5.

Table 5: Updated MU for OOB and NBB TCs

| 7.6.3 Out-of-band blocking | 1Rx, 2Rx, 4Rx: Wanted signal, f ≤ 3.0GHz ±2.0 dB, Blocking, 1MHz < finterferer ≤ 3GHz ±3.9 dB, Blocking, 3GHz < finterferer ≤ 12.75GHz Wanted signal, 3.0GHz < f ≤ 4.2GHz ±2.2 dB, Blocking, 1MHz < finterferer ≤ 3GHz ±4.0 dB, Blocking, 3GHz < finterferer ≤ 12.75GHz Wanted signal, 4.2GHz < f ≤ 6GHz ±2.6 dB, Blocking, 1MHz < finterferer ≤ 3GHz ±4.2 dB, Blocking, 3GHz < finterferer ≤ 12.75GHz 8Rx: Wanted signal, f ≤ 3.0GHz ±2.4 dB, Blocking, 1MHz < finterferer ≤ 3GHz ±4.2 dB, Blocking, 3GHz < finterferer ≤ 12.75GHz Wanted signal, 3.0GHz < f ≤ 4.2GHz ±2.6 dB, Blocking, 1MHz < finterferer ≤ 3GHz ±4.3 dB, Blocking, 3GHz < finterferer ≤ 12.75GHz Wanted signal, 4.2GHz < f ≤ 6GHz ±3.0 dB, Blocking, 1MHz < finterferer ≤ 3GHz ±4.5 dB, Blocking, 3GHz < finterferer ≤ 12.75GHz |
| --- | --- |
| 7.6.4 Narrow band blocking | Blocking 1Rx, 2Rx, 4Rx: ± 2.0dB, f ≤ 3.0GHz ± 2.4dB, 3.0GHz < f ≤ 4.2GHz ± 3.1dB, 4.2GHz < f ≤ 6.0GHz 8Rx: ± 2.4dB, f ≤ 3.0GHz ± 2.8dB, 3.0GHz < f ≤ 4.2GHz ± 3.5dB, 4.2GHz < f ≤ 6.0GHz |
| 7.6E.3.1 | 1Rx, 2Rx, 4Rx: Wanted signal, 4.2GHz < f ≤ 6GHz ±2.6 dB, Blocking, 1MHz < finterferer ≤ 3GHz ±4.2 dB, Blocking, 3GHz < finterferer ≤ 12.75GHz 8Rx: Wanted signal, 4.2GHz < f ≤ 6GHz ±3.0 dB, Blocking, 1MHz < finterferer ≤ 3GHz ±4.5 dB, Blocking, 3GHz < finterferer ≤ 12.75GHz |

Observation 2: An update of TT is not required as TT = 0 dB for the adjacent channel selectivity and blocking TCs.

# References

- R5-247754, Updating Adjacent channel selectivity test cases for 8Rx UE, Huawei, HiSilicon, 3GPP TSG RAN WG 5 Meeting #105, November 2024

- R5-247021, Updating Blocking test cases for 8Rx UE, Huawei, HiSilicon, 3GPP TSG RAN WG 5 Meeting #105, November 2024

- TS 38.101-1, V18.8.0, 3GPP TSG RAN WG5, December 2024

- R5-183951, Proposal for Maximum Test System Uncertainty for NR FR1 TRx tests, Anritsu, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #79, May 2018

- R5-251528, Update of MU of receiver test cases for 8Rx, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #106, February 2025