3GPP TSG-RAN5 Meeting #106	R5-251538

# Athens, Greece, Feb 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3192 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating PC2 AMPR for UL MIMO testing for band n8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_UL_MIMO_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | PC2 UE is allowed to transmit UL MIMO on band n8. But the PC2 AMPR testing for NS_43, NS_43U and NS_100 are not completed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test configuration table 6.2D.3.4.1-14a for NS_43 and 6.2D.3.4.1-15a for NS_43U for PC2 UE, and adding corresponding test requirement tables for PC2 UE for NS_100 and NS_43 and NS_43U. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AMPR for MIMO can't be tested for PC2 UE on band n8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-250437r1. |  |  |  |  |  |  |  |  |

### <Start of Changes>

### 6.2D.3	UE additional maximum output power reduction for UL MIMO

Editor’s Note:

- Supporting of ULFPTx is only completed for NS_04, NS_03, NS_03U.

6.2D.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated a unique with network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2D.1.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2D.3.2	Test applicability

The requirements of this test apply in test case 6.5D.2.3 Additional Spectrum Emission mask for UL MIMO for network signalling value NS_03, NS_03U, NS_04 to all types of NR UE release 15 and forward that support UL MIMO.

The requirements of this test apply in test case 6.5D.2.3 Additional Spectrum Emission mask for UL MIMO for network signalling value NS_03, NS_03U, NS_04 to all types of NR UE release 16 and forward that support UL full power transmission (ULFPTx).

The requirements of this test apply in test case 6.5D.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, and NS_100 to all types of NR Power Class 1.5, Power Class 2 and 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

The requirements of this test apply in test case 6.5D.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U to all types of NR Power Class 2 and 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

The requirements of this test apply in test case 6.5D.3.3 Additional Spurious Emissions for network signalling value NS_04, NS_47, NS_21, NS_43, NS_43U, NS_56, NS_50 to all types of NR UE release 15 and forward that support UL-MIMO.

The requirements of this test apply in test case 6.5D.3_1.3 Additional Spurious Emissions for network signalling value NS_04 to all types of NR UE release 16 and forward that support UL full power transmission (ULFPTx).

NOTE:	Test execution is not necessary if 6.5D.2.3, 6.5D.2.4.2 and 6.5D.3.3 are executed.

### <Unchanged Skipped>

Table 6.2D.3.4.1-14: Test Configuration table for NS_43 for PC3 UE

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | use channel bandwidth (Ch BW) as specified in test parameters |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |
| A-MPR test parameters for NS_43 (Power Class 3) |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | 902.5 | 5 | Default | N/A | CP-OFDM | QPSK | Outer_Full (A1) |
| 2 | 910 | 10 | Default |  |  | QPSK | Outer_Full (A5) |
| 3 | 907.5 | 15 | Default |  |  | QPSK | Edge_1RB_Left (A6) |
| 4 | 907.5 | 15 | Default |  |  | QPSK | Outer_Full (A6) |
| 5 | 902.5 | 5 | Default |  |  | 16 QAM | Outer_Full (A1) |
| 6 | 910 | 10 | Default |  |  | 16 QAM | Outer_Full (A5) |
| 7 | 907.5 | 15 | Default |  |  | 16 QAM | Edge_1RB_Left (A6) |
| 8 | 907.5 | 15 | Default |  |  | 16 QAM | Outer_Full (A6) |
| 9 | 910 | 10 | Default |  |  | 64 QAM | Outer_Full (A3) |
| 10 | 907.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left (A6) |
| 11 | 907.5 | 15 | Default |  |  | 64 QAM | Outer_Full (A6) |
| 12 | 907.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left (A6) |
| 13 | 907.5 | 15 | Default |  |  | 256 QAM | Outer_Full (A6) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: FFS NOTE 3: Not applicable when SCS=60kHz. |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-14a: Test Configuration table for NS_43 for PC2 UE

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Use channel bandwidth (Ch BW) as specified in test parameters |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest |  |
| A-MPR test parameters for NS_43 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 12 | 902.5 | 5 | Default | N/A for A-MPR | CP-OFDM | QPSK | Outer_Full |
| 2 | 910 | 10 |  |  |  | QPSK | Outer_Full |
| 3 | 907.5 | 15 |  |  |  | QPSK | Outer_Full |
| 4 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left |
| 52 | 902.5 | 5 |  |  |  | 16QAM | Outer_Full |
| 6 | 910 | 10 |  |  |  | 16QAM | Outer_Full |
| 7 | 907.5 | 15 |  |  |  | 16QAM | Outer_Full |
| 8 | 907.5 | 15 |  |  |  | 16QAM | Edge_1RB_Left |
| 92 | 902.5 | 5 |  |  |  | 64QAM | Outer_Full |
| 102 | 910 | 10 |  |  |  | 64QAM | Outer_Full |
| 11 | 907.5 | 15 |  |  |  | 64QAM | Outer_Full |
| 12 | 907.5 | 15 |  |  |  | 64QAM | Edge_1RB_Left |
| 13 | 907.5 | 15 |  |  |  | 256QAM | Outer_Full |
| 14 | 907.5 | 15 |  |  |  | 256QAM | Edge_1RB_Left |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: Not applicable when SCS=60kHz |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-15: Test Configuration table for NS_43U for PC3 UE

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Use channel bandwidth (Ch BW) as specified in test parameters |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | 15 kHz |
| A-MPR test parameters for NS_43U (Power Class 3) |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | 902.5 | 5 | 15 | N/A | CP-OFDM | QPSK | Outer_Full |
| 2 | 910 | 10 | 15 |  |  | QPSK | Outer_Full |
| 3 | 907.5 | 15 | 15 |  |  | QPSK | Edge_1RB_Left |
| 4 | 907.5 | 15 | 15 |  |  | QPSK | Outer_Full |
| 5 | 902.5 | 5 | 15 |  |  | 16 QAM | Outer_Full |
| 6 | 910 | 10 | 15 |  |  | 16 QAM | Outer_Full |
| 7 | 907.5 | 15 | 15 |  |  | 16 QAM | Edge_1RB_Left |
| 8 | 907.5 | 15 | 15 |  |  | 16 QAM | Outer_Full |
| 9 | 902.5 | 5 | 15 |  |  | 64 QAM | Outer_Full |
| 10 | 910 | 10 | 15 |  |  | 64 QAM | Outer_Full |
| 11 | 907.5 | 15 | 15 |  |  | 64 QAM | Edge_1RB_Left |
| 12 | 907.5 | 15 | 15 |  |  | 64 QAM | Outer_Full |
| 13 | 902.5 | 5 | 15 |  |  | 256 QAM | Outer_Full |
| 14 | 910 | 10 | 15 |  |  | 256 QAM | Outer_Full |
| 15 | 907.5 | 15 | 15 |  |  | 256 QAM | Edge_1RB_Left |
| 16 | 907.5 | 15 | 15 |  |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-15a: Test Configuration table for NS_43U for PC2 UE

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Use channel bandwidth (Ch BW) as specified in test parameters |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest |  |
| A-MPR test parameters for NS_43 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 12 | 902.5 | 5 | Default | N/A for A-MPR | CP-OFDM | QPSK | Outer_Full |
| 2 | 910 | 10 |  |  |  | QPSK | Outer_Full |
| 3 | 907.5 | 15 |  |  |  | QPSK | Outer_Full |
| 4 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left |
| 52 | 902.5 | 5 |  |  |  | 16QAM | Outer_Full |
| 6 | 910 | 10 |  |  |  | 16QAM | Outer_Full |
| 7 | 907.5 | 15 |  |  |  | 16QAM | Outer_Full |
| 8 | 907.5 | 15 |  |  |  | 16QAM | Edge_1RB_Left |
| 92 | 902.5 | 5 |  |  |  | 64QAM | Outer_Full |
| 102 | 910 | 10 |  |  |  | 64QAM | Outer_Full |
| 11 | 907.5 | 15 |  |  |  | 64QAM | Outer_Full |
| 12 | 907.5 | 15 |  |  |  | 64QAM | Edge_1RB_Left |
| 13 | 907.5 | 15 |  |  |  | 256QAM | Outer_Full |
| 14 | 907.5 | 15 |  |  |  | 256QAM | Edge_1RB_Left |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: Not applicable when SCS=60kHz |  |  |  |  |  |  |  |

### <Unchanged Skipped>

Table 6.2D.3.5-8: UE Power Class 3 test requirements (NS_100) for band n1

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 3 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 4, 5 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 6 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 7, 8 | 23 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 9 | 23 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 10, 11 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 12 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-9: UE Power Class 3 test requirements (NS_100) for band n2, n3, n8, n25

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 3 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 4, 5 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 6 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 7, 8 | 23 | 3.5 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 9 | 23 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 10, 11 | 23 | 6.5 | 6.5 | 1.5 | 15 | 6 | 3 | 25+TT | 9-TT |
| 12 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-9a: UE Power Class 3 test requirements (NS_100) for band n2, n3, n8, n25 supporting ULFPTx

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_100 | 23 | 0.5 | 2 | 1.5 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 3 | NS_100 | 23 | 0.5 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 4, 5 | NS_100 | 23 | 1 | 2 | 1.5 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 6 | NS_100 | 23 | 1 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 7, 8 | NS_100 | 23 | 2 | 2.5 | 1.5 | 19 | 5 | 3 | 25+TT | 14-TT |
| 9 | NS_100 | 23 | 2 | 2.5 | 0 | 20.5 | 5 | 3 | 25+TT | 15.5-TT |
| 10, 11 | NS_100 | 23 | 2.5 | 3 | 1.5 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 12 | NS_100 | 23 | 2.5 | 3 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 13, 14 | NS_100 | 23 | 4.5 | 4.5 | 1.5 | 17 | 5 | 3 | 25+TT | 12-TT |
| 15 | NS_100 | 23 | 4.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 16, 17 | NS_100 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 18 | NS_100 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 19, 20 | NS_100 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 21 | NS_100 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 22, 23 | NS_100 | 23 | 3.5 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 24 | NS_100 | 23 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 25, 26 | NS_100 | 23 | 6.5 | 6.5 | 1.5 | 15 | 6 | 3 | 25+TT | 9-TT |
| 27 | NS_100 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-9b: UE Power Class 2 test requirements (NS_100) for band n8

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 26 | 3.5 | 4 | 1.5 | 20.5 | 2.5 | 3 | 28+TT | 17.5 |
| 3 | 26 | 1 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19 |
| 4, 5 | 26 | 3.5 | 4 | 1.5 | 20.5 | 2.5 | 3 | 28+TT | 17.5 |
| 6 | 26 | 3 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19 |
| 7, 8 | 26 | 3.5 | 4 | 1.5 | 20.5 | 2.5 | 3 | 28+TT | 17.5 |
| 9 | 26 | 3.5 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19 |
| 10, 11 | 26 | 6.5 | 6.5 | 1.5 | 18 | 4 | 3 | 28+TT | 14 |
| 12 | 26 | 6.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

### <Unchanged Skipped>

Table 6.2D.3.5-18: UE Power Class 3 test requirements (NS_43) for band n8

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 3.5 | 0 |  | 19.5 |  | 5.0 |  | 3 | 25+TT | 14.5-TT |  |
| 2 | 23 | 0 | 3.0 | 4 | 0 |  | 19 |  | 5.0 |  | 3 | 25+TT | 14.0-TT |  |
| 3 | 23 | 0 | 3.0 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 4 | 23 | 0 | 3.0 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 5 | 23 | 0 | 3.0 | 3.5 | 0 |  | 19.5 |  | 5.0 |  | 3 | 25+TT | 14.5-TT |  |
| 6 | 23 | 0 | 3.0 | 4 | 0 |  | 19 |  | 5.0 |  | 3 | 25+TT | 14.0-TT |  |
| 7 | 23 | 0 | 3.0 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 8 | 23 | 0 | 3.0 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 9 | 23 | 0 | 3.5 | 4 | 0 |  | 19 |  | 5.0 |  | 3 | 25+TT | 14.0-TT |  |
| 10 | 23 | 0 | 3.5 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 11 | 23 | 0 | 3.5 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 12 | 23 | 0 | 6.5 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 13 | 23 | 0 | 6.5 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-19: UE Power Class 3 test requirements (NS_43U) for band n8

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 4 | 0 |  | 19 |  | 5.0 |  | 3 | 25+TT | 14.0-TT |  |
| 2 | 23 | 0 | 3.0 | 4 | 0 |  | 19 |  | 5.0 |  | 3 | 25+TT | 14.0-TT |  |
| 3 | 23 | 0 | 3.0 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 4 | 23 | 0 | 3.0 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 5 | 23 | 0 | 3.0 | 4 | 0 |  | 19 |  | 5.0 |  | 3 | 25+TT | 14.0-TT |  |
| 6 | 23 | 0 | 3.0 | 4 | 0 |  | 19 |  | 5.0 |  | 3 | 25+TT | 14.0-TT |  |
| 7 | 23 | 0 | 3.0 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 8 | 23 | 0 | 3.0 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 9 | 23 | 0 | 3.5 | 4 | 0 |  | 19 |  | 5.0 |  | 3 | 25+TT | 14.0-TT |  |
| 10 | 23 | 0 | 3.5 | 4 | 0 |  | 19 |  | 5.0 |  | 3 | 25+TT | 14.0-TT |  |
| 11 | 23 | 0 | 3.5 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 12 | 23 | 0 | 3.5 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 13 | 23 | 0 | 6.5 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25+TT | 11.5-TT |  |
| 14 | 23 | 0 | 6.5 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25+TT | 11.5-TT |  |
| 15 | 23 | 0 | 6.5 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 16 | 23 | 0 | 6.5 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-19a: UE Power Class 2 test requirements (NS_43, NS_43U) for band n8

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 3 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 2 | 26 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 3 | 26 | 3 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 4 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 5 | 26 | 3 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 6 | 26 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 7 | 26 | 3 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 8 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 9 | 26 | 3.5 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 10 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 11 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 12 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 13 | 26 | 6.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 14 | 26 | 6.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

### <End of Changes>