3GPP TSG-RAN5 Meeting #106	R5-251650

# Athens, Greece, Feb.17th – 21st 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0940 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TT analysis for RedCap RRM test cases 16.1.1.1 and 16.1.1.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon,Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Based on current TT analysis (38.903 attachment “38.533 16.1.1.1 TT.zip”), Qoffsets, n is decreased by 2dB to satisfy followings side condition Rn – Rs > 4dB for apply cell reselection requirements. As a result, - At the beginning of T2, Cell 1 is the serving cell and Cell 2 is the target cell. Then we have Rn - Rs = (Cell 2 RSRP – Qoffsets,n) – (Cell 1 RSRP + Qhyst) = 5dB. The side condition is well satisfied. - However, on the other hand, when the UE has reselected to Cell 2 at the end of T2 and before start of T3, the serving cell becomes Cell 2, and the target cell becomes Cell 1.Then we have Rn – Rs = (Cell 1 RSRP – Qoffsets,n) – (Cell 2 RSRP + Qhyst) = -1dB.That is, there is only 1 dB margin before Cell 1 meets the equal-priority reselection criterion. Due to the TE uncertainty on setting cell power levels, the equal-priority reselection criterion may be incorrectly met. As a result, there is a high probability that the DUT will immediately reselect back to Cell 1 after reselecting Cell 2. Then it will fail the test due to failing to initiate a registration update procedure on Cell 2. We have observed this phenomenon in practical testing. Same wrong reselection may also happen when the DUT reselects Cell 1 in T3. Furthermore, the situation is even worse for TC 16.1.1.5 because the margin is only 0.45dB based on current TT analysis. To avoid failing conformant UE, we suggest redo the TT analysis in following alternative way: TC 16.1.1.1: - Change the TT for Qoffset to -1dB. - Raise Cell 2 Es/Noc in T2 and Cell 1 Es/Noc in T3 by 0.45dB, just like 6.1.1.1/16.1.1.2 TC 16.1.1.5: - Change the TT for Qoffset to -1dB. - Change the TT for Cell 2 Es/Noc in T1 and Cell 1 Es/Noc in T2 to 0.45dB, i.e. raise 1dB in comparison of current -0.55dB TT. - Raise SintrasearchP by 2dB to ensure measurement restriction rule doesn’t apply. In this way, the margin after reselecting to target Cell becomes 2.45dB. Based on our practical testing, it can eliminate the probability of wrong reselection |  |  |  |  |  |  |  |  |
| Summary of change: |  | - TT analysis for 16.1.1.1 and 16.1.1.5 are updated. - Grouping table is updated accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8, attachments |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 CR 3739, 3741 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RRM TT |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-250718r1 |  |  |  |  |  |  |  |  |

<Start of Change 1>

Adding following .zip files to TR 38.903 attachments:

“38.533 16.1.1.1 TT_v2.zip”

“38.533 16.1.1.5 TT_v2.zip”

Removing following .zip files from TR 38.903 attachments:

“38.533 16.1.1.1 TT.zip”

“38.533 16.1.1.5 TT.zip”

<End of Change 1>

<Start of Change 2>

8	Grouping of test cases defined in TS 38.533

Table 8-1: Grouping of FR1 test cases defined in Clauses 4, 4A, 6, 8, 16 and 18 of TS 38.533

| Group | Test Case Numbers | .zip file name | Comments |
| --- | --- | --- | --- |
| SCell_Activation_01 | 4.5.3.1 4.5.3.2 4.5.3.3 6.5.3.1 6.5.3.2 6.5.3.3 10.3.3.1 10.3.3.2 10.3.3.3 11.4.3.1 11.4.3.2 11.4.3.3 13.2.2.1 13.2.2.2 13.2.2.3 | “38.533 4.5.3.1+4.5.3.2+6.5.3.1+6.5.3.2 TT v6.zip” | “2 Inter Frequency NR Cells, 3 time periods, Various number of sub-tests, No fading” |
| SCell_Activation_02 | 4.5.3.5 6.5.3.4 | “38.533 4.5.3.5 +6.5.3.4 TT.zip” | “2 Inter Frequency NR Cells, 2 time periods, Various number of sub-tests, No fading” Direct Scell activation configurations |
| <Unchanged cotents are omitted> |  |  |  |
| InterRAT_LTE_Idle_CADC_meas | 8.2.2.1 | “38.533 8.2.2.1 TT.zip” | “1 E-UTRA Cell, 1 NR Cell, 3 time periods, no fading” |
| UE_Rx_Tx_difference_PDC_01 | 6.6.20.1 6.6.21.1 | “38.533 6.6.20+6.6.21 TT.zip” | “1 NR Cell, 2 time periods, no fading” |
| Inter_Reselection_RedCap_1Rx | 16.1.1.1 | “38.533 16.1.1.1 TT_v2.zip” | “2 Intra Frequency NR Cells, 3 time periods, No fading, 1Rx RedCap UE” |
| Inter_Reselection_RedCap_1Rx | 16.1.1.3 | 38.533 16.1.1.3 TT | “2 Inter Frequency NR Cells, 3 time periods, No fading, 1Rx RedCap UE” |
| Intra_Reselection_Low_Mobility_RedCap_1Rx | 16.1.1.5 | “38.533 16.1.1.5 TT_v2.zip” | “2 Intra Frequency NR Cells, 2 time periods, No fading, 1Rx RedCap UE” |
| Inter_Reselection_Low_mobility_RedCap_1Rx | 16.1.1.7 | 38.533 16.1.1.7 TT | “2 Inter Frequency NR Cells, 2 time periods, No fading, 1Rx RedCap UE” |
| <Unchanged cotents are omitted> |  |  |  |
| Inter-LTM_PDCCH_Order | 6.3.2.4.2 | “38.533 6.3.2.4.2 TT.zip” | “2 Inter Frequency NR Cells, 2 time periods, Various number of sub-tests, No fading” |
| InterRAT_re-selection_LTE_Serving_RedCap | 18.1.1.1 | “38.533 18.1.1.1 TT.zip” | “1 NR Cell, 1 LTE serving cell, 3 time periods, no fading, 2Rx RedCap UE” |

<End of Change 2>