3GPP TSG-RAN5 Meeting #106	R5-251672

# Athens, Greece, 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5537 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of RefSens CA test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, LTE_CA_Rel13-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test configuration is not in alignment to the test point analysis for RefSens of CA_14A-66A-66A-66A, CA_2A-66A-66A-66A, CA_2A-14A-66A-66A, and CA_2A-14A-66A-66A-66A. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test configuration has been aligned. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will remain inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.9.4.1, 7.3A.10.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final outcome of one revision of R5-251143. |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 7.3A.9	Reference sensitivity level for 4DL CA

Editor’s note: Following aspects missing or not yet determined:

-	Reference sensitivity QPSK PREFSENS has defined and made for some CA bands, some of are incomplete.

#### 7.3A.9.1	Test purpose

Same as in clause 7.3A.1.1.

#### 7.3A.9.2	Test applicability

This test case applies to all types of E-UTRA UE and forward that support 4DL with CA configurations in Table 7.1-2b.

#### 7.3A.9.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3A.0.

#### 7.3A.9.4	Test description

##### 7.3A.9.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1 through table 5.4.2A.1-2a. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.3A.9.4.1-1 through table 7.3A.9.4.1-2 as appropriate. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

< Unchanged tables omitted >

Table 7.3A.9.4.1-2: Test Configuration Table with Wgap

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  | Highest NRB |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  | Wgap [MHz] | SCC1 |  |  |  | Wgap [MHz] |  | SCC2 |  |  |  | Wgap [MHz] | SCC3 |  |  |  |
|  | UL MOD | DL MOD | ULalloc (Note 2,3,4) | DLalloc |  | UL MOD | DL MOD | ULalloc | DLalloc |  |  | UL MOD | UL MOD | ULalloc | DLalloc |  | UL MOD | DL MOD | ULalloc | DLalloc |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_2A-14A-66A-66A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Mid | 50@50 | ALL RBs | - | 14 | Mid | N/A | ALL RBs | - |  | 66 | CC1 | N/A | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 50 |  |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| 2 | 14 | Mid | 15@0 | ALL RBs | - | 2 | Mid | N/A | ALL RBs | - |  | 66 | CC1 | N/A | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 50 |  | N/A | QPSK |  | 100 |  |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| 3 | 66 | CC1 | 100@0 | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs | - |  | 2 | Mid | N/A | ALL RBs | - | 14 | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 25 |  |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |
| 4 | 66 | CC1 | 100@0 | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs | - |  | 2 | Mid | N/A | ALL RBs | - | 14 | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_2A-66A-66A-66A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Mid | 50@50 | ALL RBs | - | 66 | CC1 | N/A | ALL RBs | Max |  | 66 | CC2 | N/A | ALL RBs | Max | 66 | CC3 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| 2 | 66 | CC1 | 100@0 | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs | Max |  | 66 | CC3 | N/A | ALL RBs | - | 2 | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| 3 | 66 | CC1 | 100@0 | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs | Max |  | 66 | CC3 | N/A | ALL RBs | - | 2 | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  |  | N/A | QPSK |  | 25 |  | N/A | QPSK |  | 100 |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_14A-66A-66A-66A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 14 | Mid | 15@0 | ALL RBs | - | 66 | CC1 | N/A | ALL RBs | Max |  | 66 | CC2 | N/A | ALL RBs | Max | 66 | CC3 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 50 |  | N/A | QPSK |  | 100 |  |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| 2 | 66 | CC1 | 100@0 | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs | Max |  | 66 | CC3 | N/A | ALL RBs | - | 14 | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |
| 3 | 66 | CC1 | 100@0 | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs | Max |  | 66 | CC3 | N/A | ALL RBs | - | 14 | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  |  | N/A | QPSK |  | 25 |  | N/A | QPSK |  | 50 |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Intra-band non-contiguous + Inter-band: Use CA Configuration – specific test points if present in the table. Otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use Default Test Settings test points. Note 3: Intra-band non-contiguous + Intra-band contiguous: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 4: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 5: Intra-band non-contiguous + Inter-band: X, Y, Z correspond to the different bands in the CA Configuration. E.g. for CA_2A-2A-4A-12A,X=2,Y=4,Z=12, for CA_2A-66A-66A-66A, X=2,Y=66. Note 6: Intra-band non-contiguous + Intra-band contiguous: X corresponds to the band of the CA Configuration. E.g. for CA_41C-41A-41A,X=41. Note 7: Intra-band non-contiguous + Intra-band non-contiguous: X,Y correspond to the different bands in the CA Configuration. E.g. for CA_2A-2A-4A-4A,X=2,Y=4. Note 8: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 9: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg, choose the Combination with NRB_CC1 = NRB_CC2 for testing. If no such combination is supported, choose Combination with maximum NRB_CC1 for testing. Note 10: Applicable if only Bandwidth Combination Set 0 is supported by the UE for the CA Configuration. (This note is not applicable for any existing test point and only reserved for future use) Note 11: Not applicable if only Bandwidth Combination Set 0 is supported by the UE for the CA Configuration. Note 12: Applicable if only Bandwidth Combination Set 1 is supported by the UE for the CA Configuration. (This note is not applicable for any existing test point and only reserved for future use) Note 13: Not applicable if only Bandwidth Combination Set 1 is supported by the UE for the CA Configuration. Note 14: Not applicable if operation with 4 antenna ports is supported in the band with carrier aggregation configured. Note 15: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. Note 16: Band 2: fUL = 1868.3MHz (NUL = 18783), fDL = 1948.3MHz (NDL = 783). Band 4: fUL = 1735MHz (NUL = 20200), fDL = 2135MHz (NDL = 2200). Note 17: The orders and numbering of SCCs in this table does not imply any order in test implementation of SCCs. Note 18: Test point that fulfils criteria of Note 16 in Table 7.3A.0-0a. Band 71: fUL = 679.3 MHz (NUL = 133285), fDL = 633.3 MHz (NDL = 68749) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 7.3A.10	Reference sensitivity level for 5DL CA

Editor’s note: Following aspects missing or not yet determined:

-	Test configuration table is under investigating

-	Test requirement for some CA configuration is TBD

-	Test applicability with related information in Table 7.1-2a needs to add to TS36.521-2

#### 7.3A.10.1	Test purpose

Same as in clause 7.3A.1.1.

#### 7.3A.10.2	Test applicability

This test case applies to all types of E-UTRA UE and forward that support 5DL with CA configurations in Table 7.1-2c.

#### 7.3A.10.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3A.0.

#### 7.3A.10.4	Test description

##### 7.3A.10.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1 through table 5.4.2A.1-2a. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.3A.10.4.1-1 through table 7.3A.10.4.1-2 as appropriate. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

< Unchanged tables omitted >

Table 7.3A.10.4.1-2: Test Configuration Table with Wgap

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Highest NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  |  |  |  | Wgap [MHz] |  | SCC1 |  |  |  |  |  |  |  | Wgap [MHz] | SCC2 |  |  |  | Wgap [MHz] |  | SCC3 |  |  |  | Wgap [MHz] | SCC4 |  |  |  |
|  | Band | Range |  | NRB |  |  |  |  |  | Band |  | Range |  | NRB |  |  |  |  | Band | Range | NRB |  |  |  | Band | Range | NRB |  |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD |  | UL alloc (Note 2,3,4) |  | DL alloc |  |  |  | UL MOD |  | DL MOD |  | UL alloc |  | DL alloc |  |  | UL MOD | UL MOD | UL alloc | DL alloc |  |  | UL MOD | DL MOD | UL alloc | DL alloc |  | UL MOD | DL MOD | UL alloc | DL alloc |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_2A-14A-66A-66A-66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 |  | Mid |  | 50@0 |  | ALL RBs |  | - |  | 14 |  | Mid |  | N/A |  | ALL RBs |  | - | 66 | CC1 | N/A | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs | Max | 66 | CC3 | N/A | ALL RBs |
|  | QPSK |  | QPSK |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 50 |  |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 100 |
| 2 | 14 |  | Mid |  | 15@0 |  | ALL RBs |  | - |  | 2 |  | Mid |  | N/A |  | ALL RBs |  | - | 66 | CC1 | N/A | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs | - | 66 | CC3 | N/A | ALL RBs |
|  | QPSK |  | QPSK |  |  |  | 50 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| 3 | 66 |  | CC1 |  | 100@0 |  | ALL RBs |  | Max |  | 66 |  | CC2 |  | N/A |  | ALL RBs |  | Max | 66 | CC3 | N/A | ALL RBs | - | 2 | Mid | N/A | ALL RBs | - | 14 | Mid | N/A | ALL RBs |
|  | QPSK |  | QPSK |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |
| 4 | 66 |  | CC1 |  | 100@0 |  | ALL RBs |  | Max |  | 66 |  | CC2 |  | N/A |  | ALL RBs |  | Max | 66 | CC3 | N/A | ALL RBs | - | 2 | Mid | N/A | ALL RBs | - | 14 | Mid | N/A | ALL RBs |
|  | QPSK |  | QPSK |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |  | N/A | QPSK |  | 25 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

< End of changes >