3GPP TSG-RAN5 Meeting #106	                              R5-251677

Athens, Greece, Feb 17th – Feb 21st, 2025

Agenda item:	5.3.29.9

Source:   Apple Trading, Nokia, QUALCOMM Europe Inc. - Italy

Title:       Discussion on beam lock for multiple beams

Document for:	DISCUSSION AND ENDORSEMENT

# 1	Introduction

At RAN5#104 in August 2024 an action point was established to gather inputs on whether the current UBF conformance test function can support locking two Tx beams from two panels simultaneously.

# 2	Discussion

### 2.1	Additional background on LS from RAN4 on this topic

RAN4 initially raised the query in [1] related to whether the existing UE beamlock test function can be used to lock multiple beams. Additional in a more recent LS in [2], RAN4 clarified that the objective of Rel-19 SI on NR FR2 OTA testing Phase 3 (Acronym: FS_NR_FR2_OTA_Ph3) is to study and define the test methodology for simultaneous transmission with multi-panel (STxMP) testing. The requirements for STxMP have been specified under Rel-18 WI on MIMO Evolution for Downlink and Uplink (Acronym: NR_MIMO_evo_DL_U).

Hence the FR2 UE Beamlock test function that RAN4 asked RAN5 to evaluate is for the testability of requirements defined in Clause 6.2K of TS 38.101-2 in Rel-18.

Observation 1: RAN4 suggests considering the REl-18 WI on MIMO Evolution for Downlink and Uplink to evaluate the testability of requirements defined in Clause 6.2K of TS 38.101-2 in Rel-18.

It is our view that the existing UE beam lock test function that has been in use since Rel-15 has been used for stable FR2 OTA conformance testing for several years. It is critical that the stability of this legacy test function is not impacted.

Observation 2: Some previous proposals on this topic suggested updating the octet values of the legacy UBF to accommodate locking an additional beam. This modification impacts all current UE implementations and test case validations and therefore is critical to avoid such an approach.

There is a possibility that the existing beamlock function without any modifications might be able to lock two beams from two panels simultaneously, but this needs checking by respective UE implementations.

An alternate is to explore creation of a separate test function for locking of two beams.

Proposal 1: Add a NOTE to the UBF definition in TS 38.509 to potentially allow locking of two/multiple Transmit beams.

Example is provided in Summary section

# 3	Summary

Observation 1: RAN4 suggests considering the REl-18 WI on MIMO Evolution for Downlink and Uplink to evaluate the testability of requirements defined in Clause 6.2K of TS 38.101-2 in Rel-18.

Observation 2: Some previous proposals on this topic suggested updating the octet values of the legacy UBF to accommodate locking an additional beam. This modification impacts all current UE implementations and test case validations and therefore is critical to avoid such an approach.

Proposal 1: Add a NOTE to the UBF definition in TS 38.509 to potentially allow locking of two/multiple Transmit beams.

// Example for Activate Beam Lock Procedure

// Example for Deactivate Beam Lock Procedure

# 4	References

R4-2418847– Reply LS on FR2 UE Beamlock test function

R4-2405565 – LS on FR2 UE Beamlock test function

R5-246106 – RF Action Points after RAN5#106

TS 38.521-2 – NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 standalone